SUBMITTED BY:  DONA GUSSOW (FPL)


FGT





6/9/2004


NATGAS QUALITY SPECIFICATIONS LIST
NATGAS PHYSICAL AND CHEMICAL PROPERTIES OR CHARACTERISTICS POTENTIALLY OF INTEREST TO NATGAS USER GROUPS THAT MAY BE MONITORED, TRACKED, OR CALCULATED BY THE INDUSTRY.
	PROPERTY or CHARACTERISTIC
	SYMBOL
	MEASUREMENT

UNIT(S) –or—

CONDITION
	ANALYTICAL
TEST

METHOD
	MINIMUM

VALUE
	MAXIMUM

VALUE

	HIGH HEATING VALUE
	HHV
	Btu/scf
	GPA 2172 or ?
	1000 [R&D]
	

	LOW HEATING VALUE
	LHV
	Btu/scf
	
	
	

	SPECIFIC GRAVITY
	(
	---
	GPA 2172 or ?
	
	

	WOBBE INDEX
	W
	---
	
	
	

	TEMPERATURE
	T
	(F
	
	
	120  [R]

	WATER
	H2O
	lb/MMscf
	
	
	7  [R]

	WATER DEWPOINT
	DH2O
	(F
	
	
	

	HYDROCARBON DEWPOINT
	DHC
	(F
	
	
	

	METHANE
	C1
	%
	
	
	

	ETHANE
	C2
	%
	
	
	

	PROPANE
	C3
	%
	
	
	

	ISOBUTANE
	iC4
	%
	
	
	

	NORMAL BUTANE
	nC4
	%
	
	
	

	ISOPENTANE
	Ic5
	%
	
	
	

	NORMAL PENTANE
	nC5
	%
	
	
	

	HEXANES (and heavier)
	C6 (C6+)
	%
	
	
	

	HEPTANES
	C7
	%
	
	
	

	OCTANES
	C8
	%
	
	
	

	NONANES
	C9
	%
	
	
	

	DECANES
	C10
	%
	
	
	

	NITROGEN
	N2
	%
	
	
	

	CARBON DIOXIDE
	CO2
	%
	
	
	

	OXYGEN
	O2
	% by VOL
	
	
	1/4  [R]

	HYDROGEN
	H2
	%
	
	
	

	HELIUM
	He
	%
	
	
	

	OTHER GASES
	?
	%
	
	
	

	TOTAL SULFUR
	STOT
	Grain/100cf
	
	
	10  [R]

200  [D]

	HYDROGEN SULFIDE
	H2S
	Grain/100cf
	
	
	1/4  [R]

15  [D]

	CARBONYL SULFIDE
	COS
	?
	
	
	

	SULFUR IN MERCAPTANS
	SRSH
	Note 2.
	
	
	

	IRON SULFIDE
	FeS
	lbs
	
	
	

	OTHER SULFUR COMPOUNDS
	?
	?
	
	
	

	MERCURY
	Hg
	ppb
	
	
	

	PARTICULATES
	NOTE 1
	microns
	
	
	

	CARBON DIOXIDE + NITROGEN
	CO2 + N2
	% by VOL
	
	
	3  [R]


NOTES

(1)
Gas…shall be free from objectionable odors, solid matter, dust, gums, etc….
(2)
Gas may contain an odorant at the point of delivery,…
PAGE  
1
K:\EMTCNT\X = NAT GAS QUALITY & MEASUREMENT\BPS TEAM\SPECS FORM\FPL Submittal (FGT).doc

