NATURAL GAS QUALITY MEASUREMENT REPORTS

Transportation Service Providers shall provide website reports under Infomational Postings of daily average gas quality measurements, if available, in a downloadable table that includes the following information:

· Date

· Location of measurement (at representative points entering the market area)

· Heating Value (used to determine natural gas quantities, receipts and deliveries)

· Hydrocarbon Components,  % of C1 – C???, as used in determining Heating Value

· Specific Gravity

· Water (lbs.MMscf)

· Nitrogen, %

· Carbon Dioxide, %

· Oxygen, %

· Hydrogen, %

· Helium, %

· Any other gases being measured

· Total Sulfur, grains/100scf and ppm

· Hydrogen Sulfide, grains/100scf and ppm

· Carbonyl Sulfide, grains/100scf and ppm %

· Mercaptans, grains/100scf and ppm

· Any other sulfur compounds being measured

· Mercury and/or any other contaminants being

· Indicator if above data is preliminary or final

Data shall be posted daily as soon as available for the Gas Day being reported and noted as “preliminary”.  When data is judged to be final, the Indicator will be updated to “final” and classified as historical.

Among the accompanying Notes, provide the following:

· Pipeline’s definition or scope of “total sulfur” measurement (what’s included or excluded).

· Report number of values used in calculating the above “final” daily averages [is it one, two, five or 500] and/or timing and frequency of measurement cycle.

