NATGAS QUALITY SPECIFICATIONS LIST

Specifications in the Tariff ?

NATGAS PHYSICAL AND CHEMICAL PROPERTIES OR CHARACTERISTICS POTENTIALLY OF INTEREST TO NATGAS USER GROUPS THAT MAY BE MONITORED, TRACKED, OR CALCULATED BY THE INDUSTRY.

Name of Company: North Baja Pipeline, LLC

	PROPERTY or CHARACTERISTIC
	SYMBOL
	Specification in the Tariff  

(yes / no / yes- range)

	HIGH HEATING VALUE
	HHV
	No

	LOW HEATING VALUE
	LHV
	Yes, > 995 Btus/sfc, dry basis

	SPECIFIC GRAVITY
	(
	No

	WOBBE INDEX
	W
	No

	TEMPERATURE
	T
	Yes, < 105 F

	WATER
	H2O
	Yes, <7 pounds/1,000,000 scf

	WATER DEWPOINT
	DH2O
	No

	HYDROCARBON DEWPOINT
	DHC
	Yes, <20 F at 600 psig

	METHANE
	C1
	No

	ETHANE
	C2
	No

	PROPANE
	C3
	No

	ISOBUTANE
	iC4
	No

	NORMAL BUTANE
	nC4
	No

	ISOPENTANE
	Ic5
	No

	NORMAL PENTANE
	nC5
	No

	HEXANES (and heavier)
	C6 (C6+)
	No

	HEPTANES
	C7
	No

	OCTANES
	C8
	No

	NONANES
	C9
	No

	DECANES
	C10
	No

	NITROGEN
	N2
	See “Other Gasses”

	CARBON DIOXIDE
	CO2
	Yes, <3% by volume

	OXYGEN
	O2
	Yes, < 0.2% by volume

	HYDROGEN
	H2
	See “Other Gasses”

	HELIUM
	He
	See “Other Gasses”

	OTHER GASES
	?
	Yes, Shall not contain more than 4% total diluents (the total combined carbon dioxide, nitrogen, helium, oxygen, and any other diluent compound) by volume

	TOTAL SULFUR
	STOT
	Yes, < 0.75 grains/100scf

	HYDROGEN SULFIDE
	H2S
	Yes, <1/4 grain/100scf

	CARBONYL SULFIDE
	COS
	No

	SULFUR IN MERCAPTANS
	SRSH
	No

	IRON SULFIDE
	FeS
	No

	OTHER SULFUR COMPOUNDS
	?
	No

	MERCURY
	Hg
	No

	PARTICULATES
	?
	Yes (see Sect 2.1(b) (1))

	
	
	


NOTES:  NBP gas quality standards are located in Section 2, sheet #106 of the NBP approved FERC Gas Tariff

PAGE  

