StSection 2 Standard Terminology for Transmission and Ancillary

Services

Section 2.1 Attribute Values Defining the Period of Service

The data templates of the Phase IA Standards & Communication

Protocols (S&CP) Document have been developed with the use of

standard service attributes in mind. What the Phase IA S&CP

Document does not offer are specific definitions for each

attribute value. This section offers standards for these service

attribute definitions to be used in conjunction with the Phase IA

data templates.

Fixed services are associated with transmission services whose

periods align with calendar periods such as a day, week, or

month. Sliding services are fixed in duration, such as a week

or month, but the start and stop time may slide. For example a

sliding week could start on Tuesday and end on the following

Monday. Extended allows for services in which the start time

may slide and also the duration may be longer than a standard

length. For example an extended week of service could be nine

consecutive days. Various transmission service offerings using

these terms are defined in Standards 2.1.1 through 2.1.13 below.

Table 1-1 identifies the definitions that are proposed as

standard terminology in OASIS Phase IA for the attributes

SERVICE_INCREMENT (Hourly, Daily, Weekly, Monthly, and Yearly)

and WINDOW (Fixed, Sliding, and Extended). . Multiples of the same SERVICE_INCREMENT may be reserved on the same request according to the below stated definitions.  A definition is

 required for each combination of SERVICE_INCREMENT and WINDOW,

except Hourly Sliding and Hourly Extended, which, at the present,

are not sufficiently common in the market to require standard

definitions.

Table 1-1

Standard Service Attribute Definitions Required in Phase IA

1

Fixed Sliding Extended

Hourly X N/A N/A

Daily X X X

Weekly X X X

Monthly X X X

Yearly X X X

1

Included in the Phase IA S&CP Data Dictionary, Version 1.3,

issued September 29, 1998.

The existence of a definition in this table does not imply the

services must be offered by a Transmission Provider.

Requirements as to which services must be offered are defined by

regulation and tariffs.

Each definition assumes a single time zone specified by the

Transmission Provider. It is recognized that daylight time

switches must be accommodated in practice, but they have been

omitted in the definitions for the purpose of simplicity.

Standard 2.1: A Transmission Provider shall use the values and

definitions below for the attributes SERVICE_INCREMENT and WINDOW

for all transmission services offered on OASIS, or shall post

alternative attribute values and associated definitions on the

OASIS Home Page at www.tsin.com, or shall use existing attribute

values and definitions posted by other Transmission Providers.

(See Section 3 for registration requirements.)

Standard 2.1.1: FIXED HOURLY The service starts at the

beginning of a clock hour and stops at the end of a clock

hour. Multi-hour request for service may be made, with the stop time being the last hour of requested service.
Standard 2.1.2: FIXED DAILY The service starts at 00:00

and stops at 24:00 of the same calendar date (same as 00:00

of the next consecutive calendar date). Multi-day request for service may be made, with the stop date being 24:00 of the last day of requested service.
Standard 2.1.3: FIXED WEEKLY The service starts at 00:00

on Monday and stops at 24:00 of the following Sunday (same

as 00:00 of the following Monday). Multi-week request for service may be made, with the stop date being 24:00 on Sunday of the last week of requested service.
Standard 2.1.4: FIXED MONTHLY The service starts at 00:00

on the first date of a calendar month and stops at 24:00 on

the last date of the same calendar month (same as 00:00 of

the first date of the next consecutive month). Multi-month request for service may be made, with the stop date being 24:00 of the last day of the last month of requested service.
Standard 2.1.5: FIXED YEARLY The service starts at 00:00

on the first date of a calendar year and ends at 24:00 on

the last date of the same calendar year (same as 00:00 of

the first date of the next consecutive year).Multi-year request for service may be made, with the stop date being 24:00 of the last day of the last year of requested service.
Standard 2.1.6: SLIDING DAILY The service starts at the

beginning of any hour of the day and stops exactly 24 hours

later at the same time on the next day. Multi-day request for service may be made, with the stop time being the same on the last day of requested service.
Standard 2.1.7: SLIDING WEEKLY The service starts at 00:00

of any date and stops exactly 168 hours later at 00:00 on

the same day of the next week. Multi-week request for service may be made, with the stop date being 24:00 on last day of the last week of requested service.
Standard 2.1.8: SLIDING MONTHLY The service starts at

00:00 of any date and stops at 00:00 on the same date of the

next month (28-31 days later). If there is no corresponding

date in the following month, the service stops at 24:00 on

the last day of the next month. For example: SLIDING MONTHLY starting at 00:00 on January 30 would stop at 24:00 on February 28 (same as 00:00 March Multi-month request for service may be made, with the stop date being 00:00 on the same day of the last month of requested service.


1).

Standard 2.1.9: SLIDING YEARLY The service starts at 00:00

of any date and stops at 00:00 on the same date of the

following year. If there is no corresponding date in the

following year, the service stops at 24:00 on the last day

of the same month in the following year.

For example SLIDING YEARLY service starting on February 29

would stop on February 28 of the following year. Multi-year request for service may be made, with the stop being 00:00 of the same day of the last year of requested service.
Standard 2.1.10: EXTENDED DAILY The service starts at any

hour of a day and stops more than 24 hours later and less

than 168 hours later.

Standard 2.1.11: EXTENDED WEEKLY The service starts at

00:00 of any date and stops at 00:00 more than one week

later, but less than four weeks later.

Standard 2.1.12: EXTENDED MONTHLY The service starts at

00:00 of any date and stops at 00:00 more than one month

later, but less than twelve months later.

Standard 2.1.13: EXTENDED YEARLY The service starts at

00:00 of any date and stops at 00:00 more than one year

later. 
