[image: image1.png]0

I

0




North American Energy Standards Board

1301 Fannin, Suite 2350, Houston, Texas 77002

Phone:  (713) 356-0060, Fax:  (713) 356-0067, E-mail: naesb@naesb.org


Home Page: www.naesb.org

October 20, 2006
TO:
NAESB Wholesale Electric Quadrant (WEQ) Members and Interested Industry Participants
FROM:
WEQ Business Practices Subcommittee (BPS), WEQ Electronic Scheduling and Information Technology Subcommittee (ESS/ITS), and DeDe Kirby, Meeting/Project Manager, NAESB
RE:
Request for Informal Comments on revised draft standards language for R05004A Item 1B - Comments due November 13, 2006 
DATE:
October 20, 2006

Dear NAESB WEQ Members and Interested Industry Participants,

The WEQ BPS and ESS/ITS request informal comments on the revised draft standards language for Request R05004A (Develop transmission service request and scheduling standards using TTC/ATC/AFC and CBM/TRM) Item 1B (granting of partial service by capacity requested, both partial period and partial MW).  Draft standards language for Item 1B was posted for informal comments the first time on August 2, 2006 for a 45 day comment period.  Due to comments received on extended products and multiple requests, the language was revised during the October 10th WEQ BPS and ESS/ITS conference call and is included below (please note that the changes from the August 2nd language are highlighted):
002-4.2.13.2 Partial Service

(Will this be the first paragraph of the section?)
The TP shall offer the maximum available partial long term Firm service in response to a long term Firm request and shall offer the maximum available partial short term Firm service in response to a request for short term Firm service, whenever the full amount of requested service cannot be granted.  The TP is not required to offer short term service in response to a long term Firm request, unless stated in the TP’s tariff and the service requested has a TS_WINDOW of Extended.  

To the extent the TP does not contradict section 19.7 of its approved Pro Forma Tariff, CAPACITY_GRANTED shall be the maximum available  level of capacity within a given SERVICE_INCREMENT, or as defined in the Transmission Provider’s approved Tariff.  For example, partial offers for annual service (FIXED OR SLIDING) may vary in CAPACITY_GRANTED from year to year but not within each year and partial offers for monthly service (FIXED OR SLIDING) may vary in CAPACITY_GRANTED from month to month but not within the month.
In addition, changes were made to NAESB Open Access Same-Time Information Systems (OASIS) Standards WEQ-001 requirement 2.0 for the same reasons as noted above.  Please see the highlighted changes below: 

001-2.0 Attribute Values Defining the Period of Service

The data templates of the most current version of the NAESB Standards and Communications Protocol for Open Access Same-Time Information Systems have been developed with the use of standard service attributes in mind.  What the most current version of the NAESB Standards and Communications Protocol for Open Access Same-Time Information Systems does not offer are specific definitions for each attribute value.  This section offers standards for these services attribute definitions to be used in conjunction with the Phase IA data templates. 

Fixed services are associated with transmission services whose periods align with calendar periods such as a day, week, or month.  Sliding services are fixed in duration, such as a week or month, but the start and stop time may slide. For example a Sliding week could start on Tuesday and end on the following Monday.  Extended allows for services in which the start time may slide and also the duration may be longer than a standard length.  For example an Extended week of service could be nine consecutive days.  Various transmission service offerings using these terms are defined in Standards 2.1.1 through 2.1.14 below.  Next_Increment indicates the next available full Service_Increment, such as the next hour, next day, or next week.  Next_Increment is added at this time to address Next Hour Market Service, but may be used in the future to denote other products.  
Table 2-1 identifies the standard terminology in OASIS Phase IA for the attributes SERVICE_INCREMENT (Hourly, Daily, Weekly, Monthly, and Yearly) and TS_WINDOW (Fixed, Sliding, Extended, and Next_Increment).  Multiples of the same SERVICE_INCREMENT may be reserved on the same request.  Values shown in Table 2-1 as N/A (Not Applicable) are not sufficiently common in the market to require standards.  Next Hour Market Service, a new pro forma service, is denoted as having a Service Increment of Hourly and a TS_WINDOW of Next_Increment.

For further information on the development of this language, please see the September 27, 2006 and October 10, 2006 minutes (posted on the WEQ BPS and ESS/ITS page for ATC/AFC at http://www.naesb.org/weq/weq_atc_afc.asp.)  Comments submitted will be reviewed by the WEQ BPS and ESS/ITS during the November 15, 2006 meeting at the Magnolia Hotel in Dallas, TX.  Please return any comments you have on the document to the NAESB Office (naesb@naesb.org or 713-356-0067 fax) by November 13, 2006.  If you have any questions or need further assistance, feel free to contact the NAESB Office at 713-356-0060.

Best Regards,

DeDe Kirby
NAESB


