[image: image1.wmf]Reservation 12345

(50MW from 6:00 

–

22:00)

Reservation 67890

(50MW from 6:00 

–

22:00)

TIME

06:00

22:00

14:00

CAPACITY

100MW

50MW

0MW

Vertical Stacking

[image: image3.wmf] 


NAESB WEQ FINAL ACTION RATIFIED 06/22/06

                                       For Quadrant: Whole Electric Quadrant                                                

                                       Requesters: NAESB Members of the ESS/ITS


                                       Request No.: R05001 


                                       Request Title: Amend WEQ R03013 to align coordinate interchange transactions with Version 0


004-C
Appendix C
Required and Correctable E-Tag Data

A.
New Interchange Transactions

A new Interchange transaction is an Arranged Interchange (E-Tag) that has not yet been approved or confirmed for implementation.  Such Arranged Interchange must be presented to those Approval Entities (as E-Tag) that are responsible for the implementation of the Interchange transaction in order that they may evaluate the E-Tag and determine whether or not the Interchange can be implemented.  The following information is to be used to describe such an Interchange transaction.  
“Correctable” as used below indicates a field that may be modified by the RFI author prior to a RFI reaching a composite state of confirmed.  The lack of this term below indicates that the field is not correctable.  “Required” as used below indicates a field must be populated with data in order for the RFI to be considered valid.  The lack of this term indicates that the field is not required.
1. Market Information

1.1. (Reserved) 
1.2. Financial Path (Required) ( the description of financially responsible parties for the Interchange in order.  This will typically start with a Purchasing-Selling Entity providing generation (GPE) and finish with a Load Serving Entity (LSE), and where applicable, intermediate Purchasing-Selling Entities between the two.

1.2.1. Energy Title Holder(s) (Required) – the identity of the entities financially responsible to take and/or deliver the energy as described in the physical path.  This will typically be a Purchasing-Selling Entity providing generation, a Load Serving Entity, and where applicable, Intermediate Purchasing-Selling Entities.

1.2.1.1. Energy Product Type (Required) (Correctable) – the type of energy delivered by the Energy Title Holder.  

1.2.1.2. Contract Number(s) (Correctable) – reference to a Transaction entered into by the Energy Title Holder with one or more other participants in the Transaction.

1.2.1.3. Miscellaneous Information (Correctable) – information provided at the Requesting PSE’s option regarding the Transaction.

2. Physical Information
2.1. Physical Path (Required) – the description of physically scheduling parties for the Interchange in order and related to the financially responsible parties described above.  This will always contain a Generation segment, at least one Transmission segment, and a Load segment.

2.1.1. Generation (Required) – set of data describing the physical and contractual characteristics of the energy source.  

2.1.1.1. Source (Required) – the physical point at which the energy is being generated.  This may vary in granularity, dependent on local business practices.

2.1.1.2. Contract Number(s) (Correctable) ( reference to a schedule or agreement entered into by the Purchasing-Selling Entity providing generation and the Generator Operator.

2.1.1.3. Miscellaneous Information (Correctable) – information provided at the Requesting PSE’s option regarding the Interchange Transaction.

2.1.1.4. Energy Profile (Required) – energy to be produced by the Generator Operator for this Transaction.

2.1.2. Transmission (Required) – set of data describing the physical and contractual characteristics of a wheel (import, export, or through).

2.1.2.1. Transmission Service Provider (Required) – the identity of the transmission provider that is wheeling the energy.

2.1.2.2. Point of Receipt (Required) (Correctable) – valid Point of Receipt for scheduled Transmission Reservation.

2.1.2.3. Point of Delivery (Required) (Correctable) – valid Point of Delivery for scheduled Transmission Reservation.

2.1.2.4. Scheduling Entities (Required) (Correctable) – entity that is physically scheduling interchange on behalf of the Transmission Service Provider in order to provide wheeling services.  Typically this is the Balancing Authority for the Transmission Service Provider, but may be several Balancing Authorities supporting a regional transmission service.

2.1.2.5. Loss Provision Information (Required) (Correctable)– Information describing the manner in which losses are accounted when they are not scheduled as in-kind megawatt distributions through the original transaction.  Types may be financial (paid in dollars based on tariff provisions), internal (scheduled in megawatts to the Transmission Service Provider from a resource inside the Transmission Service Provider’s area), or external (scheduled in megawatts to the Transmission Service Provider from a resource outside the Transmission Provider’s area). If internal or external, must specify contract numbers or Transaction IDs.

2.1.2.6. Miscellaneous Information (Correctable) – information provided at the Requesting PSE’s option regarding the transaction.

2.1.2.7. POR and POD Profiles (Required) – schedule of Energy Flow imported at the Point of Receipt and exported at the Point of Delivery.

2.1.2.8. Transmission Reservation Number(s) (Required) (Correctable) – reference to a particular transmission reservation being used to provide transmission capacity to support the transaction being described.

2.1.2.9. Transmission Product (Required) (Correctable) – Specifies the firmness of service associated with the transmission reservation being used.

2.1.2.10. Requesting PSE (Required) (Correctable) – identifies the entity that purchased and holds the transmission reservation being presented for use.

2.1.2.11. 
2.1.2.12. Transmission Allocation Profile  (Required) – profile  of transmission reservation allocated by the Requesting Purchasing-Selling Entity for use for this Transaction.

2.1.3. Load (Required) – set of data describing the physical and contractual characteristics of the energy sink.

2.1.3.1. Sink (Required) – the physical point at which the energy is being consumed.  This may vary in granularity, dependent on local business practices.

2.1.3.2. Contract Number(s) (Correctable) ( reference to a schedule or agreement entered into by the Load Serving Entity and the distribution provider.

2.1.3.3. Miscellaneous Information (Correctable) – information provided at the requesting PSE’s option regarding the Transaction.

2.1.3.4. Energy Profile (Required) – energy to be consumed by the load for this Transaction.

Using Multiple Transmission Reservations to Support a Single Leg of an Interchange Transaction

The use of multiple transmission reservations to support a single leg of an Interchange Transaction is known as transmission stacking.  There are two types of transmission stacking:

· Vertical stacking, in which a Requesting Purchasing-Selling Entity combines multiple reservations to achieve a certain net level of transmission capacity, and

· Horizontal stacking, in which a Requesting Purchasing-Selling Entity combines multiple reservations to achieve a certain transmission capacity coverage over time.

The following diagrams illustrate these concepts more fully.  In both cases, the assumed need is 100 MW of transmission capacity for hours 06:00 through 22:00.

[image: image5.png]\

o

li




[image: image2.wmf]Reservation 12345

(100 MW from 6:00 

–

14:00)

Reservation 67890

(100 MW from 14:00 

–

22:00)

TIME

06:00

22:00

14:00

CAPACITY

100MW

50MW

0MW

Horizontal Stacking


Should a Requesting PSE elect to utilize stacking, including any combination of the two stacking types, to support their Interchange Transaction, they must understand the following requirements:

· Stacks MUST be described through fully qualified profiles for each reservation being used

· At no point may the coverage described by the stack be less than the transmission capacity needed for the Transaction’s energy flow

B.
Curtailments and Reloads (Reliability Related Profile Modifications)

Curtailments and Reloads are special kinds of modifications to an Interchange transaction’s energy profile based on reliability concerns.  Such modifications must be presented to those entities that are responsible for the implementation of the modification in order that they may evaluate the transaction request and determine whether or not the modification can be implemented.  The following information must be used to describe such a modification.  

· The Transaction being curtailed or reloaded

· All necessary profile changes to set the maximum flow allowed for the transaction during the appropriate hours

· A contact person that initiated the curtailment or reload, and

· A description of the necessity for the schedule change.

C.
Market-Related Profile Modifications

Profile Modifications are changes to an Interchange Transaction’s energy profile based on market desires.  Such modifications must be presented to those entities that are responsible for the implementation of the modification in order that they may evaluate the Interchange Transaction request and determine whether or not the modification can be implemented.  The following information must be used to describe such a modification. 

· The Interchange Transaction being modified

· All necessary profile changes to set the transmission capacity or energy flow to the desired levels during the appropriate hours, and

· A contact person that initiated the modification












Copyright © 2006 North American Energy Standards Board, Inc. All Rights Reserved.
Page 1 of 5

[image: image4.wmf] 

_1075620371.doc
[image: image1.png]|

Lo







