Name


A. Introduction

1. Title:
Ten Minute Recallable E-Tag Functionality for Reserves



2. Number:
INT-BPS-011-0
3. Purpose: 
Defines the use of interruptible interchange as an exchange of both an energy product and reserve product.  The terms “interruptible interchange” and “non-firm” are broader than the very narrow definition described within this Business Practice.  This business practice is limited to “Exports that can be interrupted within 10 minutes at the request of the source” whereas “interruptible interchange” and “non-firm” in general may not meet the 10 minute timeframe required for inclusion as a reserve resource by the source and reserve obligation for the sink.  

This Business Practice describes the E-Tagging requirements and Reserve Resource impacts of Interchange that can be decreased within 10 minutes at the request of the source BA and is included in the source BA’s reserve resources and sink BA’s reduction to total reserve resources (product code C-RE).  It defines the impact on reserves for both the source and sink BA and it ensures that these transactions are identified clearly in the tag so that all parties to the transaction are aware of their responsibilities as defined within this Business Practice.

4. Applicability

4.1. Reliability Authority
4.2. Balancing Authorities/Control Areas
4.3. Interchange Authority
4.4. Transmission Service Provider
4.5. Generator Providing Entity
4.6. Purchasing-Selling Entity
4.7. Load-Serving Entity
5. Effective Date:



B. Requirements

R1. The RFI/e-Tag Transaction Type will be set to Normal.

R2. This interchange type will be identified by C-RE in the Product field on the first line in the Market Path.

R3. Activation of reserves is accomplished through a downward adjustment.  

R3.1. Adjustments by the source BA or GPE will be automatically set to the status of IMPLEMENT with no approval process.  (In order for the GPE to adjust the e-Tag, they must be the e-Tag author.) 

R4. E-tags using this product will have a default ramp rate of zero.

R5. The source BA may include the interchange value in their Reserve Resources.

R6. The sink BA must reduce their total Reserve Resource by the interchange value.

C. Measures

M1. Not specified.

D. Compliance

6. Compliance Monitoring Process

6.1. Compliance Monitoring Responsibility

Text

6.2. Compliance Monitoring Period and Reset
Text

6.3. Data Retention

Text

6.4. Additional Compliance Information

Text

7. Levels of Non-Compliance

7.1. Level 1:
Text
7.2. Level 2:
Text
7.3. Level 3:
Text
7.4. Level 4:
Text
E. Regional Differences

8. WECC has both 5 day and 7 day shops.

Version History

	Version
	Date
	Action
	Change Tracking

	0
	???
	Effective Date
	New

	
	
	
	

	
	
	
	

	
	
	
	


Draft
Page 1 of 2
Effective Date

