Standards Authorization Request Form


Standard Authorization Request Form

	Title of Proposed Standard
Revision to INT Standard’s Timing Table for WECC

	Request Date


February 13, 2007


	SAR Requestor Information
	SAR Type (Check a box for each one that applies.)

	Name
WECC Interchange Scheduling and Accounting Subcommittee (WECC ISAS) and the NERC/NAESB Joint Interchange Scheduling Work Group (JISWG)
	 FORMCHECKBOX 

 FORMCHECKBOX 

	New Standard

	Primary Contact
Robert Harshbarger
	 FORMCHECKBOX 

	Revision to existing Standard 

	Telephone
425-462-3348  

Fax
425-462-3049

	 FORMCHECKBOX 

 FORMCHECKBOX 

	Withdrawal of existing Standard 

	E-mail
robert.harshbarger@pse.com
	 FORMCHECKBOX 

 FORMCHECKBOX 

	Urgent Action


	Purpose (Describe the purpose of the standard — what the standard will achieve in support of reliability.)
Modify the Assessment Period for WECC from 5 minutes to 10 minutes for e-Tags submitted between 1 hour and 20 minutes prior to ramp start.  Default ramp start for transactions beginning at the top of the hour is 10 minutes prior to the top of the hour with 20 minute duration.  The effect in most cases would be to increase the assessment period from 5 minutes to 10 minutes for e-Tags submitted between xx:00 and xx:30 that have start times of xx+1:00.  The Timing Table appears in INT-005-1, INT-006-1, and INT-008-1. 

Update the Timing Table to Reflect the Categories (On-time, Late, After-the-fact, and Pre-late) used in the latest E-Tag Specification with respect to receipt of an Arranged Interchange (RFI): 
· Include designation of request status based on start and submittal times.  

· Include assess times for After-The-Fact (ATF) requests. 

· Include a WECC preschedule late (Pre-Late) submittal definition.  



	Industry Need (Provide a detailed statement justifying the need for the proposed standard, along with any supporting documentation.)

The Coordinate Interchange Drafting Team overlooked an issue when they put the timing tables together for WECC.  The new coordinate interchange timing table (INT-005,006,008-001), in row 2, contain a 5 minute assessment period for WECC only, for RFI's submitted < 1 hour prior to ramp start.  This short assessment period was defined in order to allow schedules to be submitted up to xx:40 without being late (maintaining existing scheduling procedures).  Some members of the drafting team were under the impression that the 10 minute duration (total of all columns) was the assessment period, thus reflecting no change from existing practices.  Column B is clearly defined as 5 minutes however.  This short of an assessment window is not necessary until xx:35 for maintaining the existing scheduling timing.  The 5 minute assessment period has been causing a problem within the WECC.  In combination with the removal of passive approval for reliability entities (passive denial instead), the short assessment period is resulting in reliability entities scrambling to more quickly assess and approve e-Tags.  Many entities require that this be done manually.  Most "realtime" e-Tags are submitted between xx:00 and xx:30, a time period in which 10 minute assessments would work well for both marketers and reliability entities.  Unfortunately, with only 5 minutes to assess, many e-Tags were unassessed and transitioned to passive DENIED.  The marketers are frustrated because they need to re-create the e-Tags and the reliability entities are frustrated because they do not have enough time to conduct assessments and also continue their other critical duties.
The timing table currently does not provide for a specification ATF, Late, and on-time requests.  Addition of the initial request status assigned by the Interchange Authority would ensure a common implementation by the various scheduling entities.

The table does include specific timings for After-The-Fact requests.  A clarification to vendors issued by the Joint Interchange Scheduling Work Group (JISWG) asked the vendors to implement a 2 hours assessment window for ATF submittals.

To fully support the WECC interchange tagging process, the tables include a definition for late submittals in the preschedule (i.e., next day, etc.) time frame for WECC, only.  The creation of a Pre-Late status as defined in the attached table, ensures uniform treatment of all late submittals. 




	
	
	A
	B
	C
	D
	

	If

Actual Arranged Interchange (RFI) is Submitted
	IA Assigns Initial Status of
	IA Makes Initial Distribution of Arranged Interchange
	BA and TSP Conduct Reliability Assessments
IA Verifies Reliability Data Complete
	IA Compiles and Distributes Status
	BA Prepares Confirmed Interchange for Implementation
	Minimum Total Reliability Period

(Columns A through D)

	>1 hour after the start time
	ATF
	< 1minute from RFI submission
	< 2 hours from Arranged Interchange receipt from IA
	< 1minute from receipt of all Reliability Assessments
	NA
	NA

	<15 minutes prior to ramp start but <1 hour after the start time
	Late
	< 1minute from RFI submission
	< 10 minutes from Arranged Interchange receipt from IA for all Interconnections except WECC
	< 1minute from receipt of all Reliability Assessments
	< 3 minutes after receipt of confirmed RFI 
	15 minutes 

	<10 minutes prior to ramp start but <1 hour after the start time
	Late
	< 1minute from RFI submission
	< 5 minutes from Arranged Interchange receipt from IA for WECC
	< 1minute from receipt of all Reliability Assessments
	< 3 minutes after receipt of confirmed RFI
	10 minutes 

	<1 hour but > 15 minutes prior to ramp start
	On-time
	< 1minute from RFI submission
	< 10 minutes from Arranged Interchange receipt from IA for all Interconnections except WECC
	<1 minute from receipt of all Reliability Assessments
	>3 minutes prior to ramp start
	15 minutes

	<20 minutes but > 10 minutes prior to ramp start
	On-time
	< 1minute from RFI submission
	< 5 minutes from Arranged Interchange receipt from IA for WECC
	< 1minute from receipt of all Reliability Assessments
	> 3 minutes prior to ramp start
	10 minutes

	<1 hour but > 20 minutes prior to ramp start
	On-time
	< 1minute from RFI submission
	< 10 minutes from Arranged Interchange receipt from IA for WECC
	< 1minute from receipt of all Reliability Assessments
	> 3 minutes prior to ramp start
	15 minutes

	>1 hour but < 4 hours prior to ramp start
	On-time
	< 1minute from RFI submission
	< 20 minutes from Arranged Interchange receipt from IA
	< 1minute from receipt of all Reliability Assessments
	> 39 minutes prior to ramp start
	1 hour plus 1 minute

	> 4 hours prior to ramp start
	On-time*
	< 1minute from RFI submission
	< 2 hours from Arranged Interchange receipt from IA
	< 1minute from receipt of all Reliability Assessments
	> 1 hour 58 minutes prior to ramp start
	4 hours

	Submitted between 1500 and 1700 PPT with start time > 00:00 PPT of following day
	Pre-Late
	< 1minute from RFI submission
	< 2 hours from Arranged Interchange receipt from IA for WECC
	< 1minute from receipt of all Reliability Assessments
	> 1 hour 58 minutes prior to ramp start
	4 hours


Reliability Functions

	The Standard will Apply to the Following Functions (Check box for each one that applies.)

	 FORMCHECKBOX 

	Reliability Coordinator
	Responsible for the real-time operating reliability of its Reliability Coordinator Area in coordination with its neighboring Reliability Coordinator’s wide area view.

	 FORMCHECKBOX 

	Balancing Authority
	Integrates resource plans ahead of time, and maintains load-interchange-resource balance within a Balancing Authority Area and supports Interconnection frequency in real time.

	 FORMCHECKBOX 

	Interchange Coordinator
	Ensures communication of interchange transactions for reliability evaluation purposes and coordinates implementation of valid and balanced interchange schedules between Balancing Authority Areas.

	 FORMCHECKBOX 

	Planning Coordinator 
	Assesses the longer-term reliability of its Planning Coordinator Area.

	 FORMCHECKBOX 

	Resource Planner
	Develops a >one year plan for the resource adequacy of its specific loads within a Planning Coordinator area.

	 FORMCHECKBOX 

	Transmission Planner
	Develops a >one year plan for the reliability of the interconnected Bulk Electric System within its portion of the Planning Coordinator area.

	 FORMCHECKBOX 

	Transmission Service Provider
	Administers the transmission tariff and provides transmission services under applicable transmission service agreements (e.g., the pro forma tariff).

	 FORMCHECKBOX 

	Transmission Owner
	Owns and maintains transmission facilities.

	 FORMCHECKBOX 

	Transmission Operator
	Ensures the real-time operating reliability of the transmission assets within a Transmission Operator Area.

	 FORMCHECKBOX 

	Distribution Provider
	Delivers electrical energy to the End-use customer.

	 FORMCHECKBOX 

	Generator Owner
	Owns and maintains generation facilities.

	 FORMCHECKBOX 

	Generator Operator
	Operates generation unit(s) to provide real and reactive power.

	 FORMCHECKBOX 

	Purchasing-Selling Entity
	Purchases or sells energy, capacity, and necessary reliability-related services as required.

	 FORMCHECKBOX 

	Market Operator
	Interface point for reliability functions with commercial functions.


Reliability and Market Interface Principles

	Applicable Reliability Principles (Check box for all that apply.)

	 FORMCHECKBOX 

	Interconnected bulk electric systems shall be planned and operated in a coordinated manner to perform reliably under normal and abnormal conditions as defined in the NERC Standards.

	 FORMCHECKBOX 

	The frequency and voltage of interconnected bulk electric systems shall be controlled within defined limits through the balancing of real and reactive power supply and demand.

	 FORMCHECKBOX 

	Information necessary for the planning and operation of interconnected bulk electric systems shall be made available to those entities responsible for planning and operating the systems reliably.

	 FORMCHECKBOX 

	Plans for emergency operation and system restoration of interconnected bulk electric systems shall be developed, coordinated, maintained and implemented.

	 FORMCHECKBOX 

	Facilities for communication, monitoring and control shall be provided, used and maintained for the reliability of interconnected bulk electric systems.

	 FORMCHECKBOX 

	Personnel responsible for planning and operating interconnected bulk electric systems shall be trained, qualified, and have the responsibility and authority to implement actions.

	 FORMCHECKBOX 

	The security of the interconnected bulk electric systems shall be assessed, monitored and maintained on a wide area basis.

	Does the proposed Standard comply with all the following Market Interface Principles? (Select “yes” or “no” from the drop-down box.)

	The planning and operation of bulk electric systems shall recognize that reliability is an essential requirement of a robust North American economy.  FORMDROPDOWN 


	An Organization Standard shall not give any market participant an unfair competitive advantage. FORMDROPDOWN 
 

	An Organization Standard shall neither mandate nor prohibit any specific market structure.  FORMDROPDOWN 


	An Organization Standard shall not preclude market solutions to achieving compliance with that Standard.  FORMDROPDOWN 


	An Organization Standard shall not require the public disclosure of commercially sensitive information.  All market participants shall have equal opportunity to access commercially non-sensitive information that is required for compliance with reliability standards.  FORMDROPDOWN 



Related Standards

	Standard No.
	Explanation

	INT-005-1
	Contains timing table

	INT-006-1
	Contains timing table

	INT-008-1
	Contains timing table

	     
	     


Related SARs

	SAR ID
	Explanation

	     
	     

	     
	     

	     
	     

	     
	     

	     
	     

	     
	     

	     
	     

	     
	     


Regional Variances
	Region
	Explanation

	ERCOT
	

	FRCC
	

	MRO
	

	NPCC
	

	SERC
	

	RFC
	

	SPP
	

	WECC
	





Detailed Description 


Modify the Timing Table in INT-005-1, INT-006-1 and INT-008-1 to include the information as shown on the following page.  Proposed additions that go beyond those proposed in the Urgent Action SAR for Emergency Revision to INT Standard’s Timing Table for WECC are highlighted in yellow. 









SAR-3

SAR-2

