IS participant comments:

 

· There is a statement in the request:"This will be of particular interest for PSEs participating in the secondary transmission market conducted off the OASIS to prevent unauthorized uses of their transmission rights."While the sale may be conducted off OASIS there is a FERC requirement the the original TC post a reassignment on OASIS documenting the sale.  I assume when the tag is built the TC with the reassignment will use the reassignment A_Ref which lists him as the PSE with ownership.  As long as the TP checks to see that the MW schedule does not exceed the reassignment A_Ref, doesn't this all work out without needing this change ?

· The only concern would be if the PSE was not actively monitoring the system and there were late tags. Late tags are not encouraged, but there are times they are necessary.  If the PSE was only passively approving the tag, would it go denied if it was late?  If so, that may be an issue.

· Hi.  I had some comments on the PSE E-Tag Approval Rights draft standards request.  I am not opposed to giving PSEs with transmission rights approval rights on tags, I just want to make sure that we don't assume that this gives transmission providers an assurance that may not indeed be provided. In part 4, and other places in the document you mentioned that the transmission provider will be given an active indicator from the rights holder that the tag author has permission to use transmission rights shown on the tag.  But really this benefit is dependent upon the transmission rights holder registering a tag approval service to perform these approvals, and the registered transmission rights holder performing an active approval, rather than allowing tags to be passively approved.  If we compare this to the right of GPEs to approve tags, we see that there are still many GPEs that are not registered to approve tags, and their active approval is not required.  So really, the generating control area or the sink control area can not infer from the implemented tag that the GPE approves the transaction.  And even from this display, if the GPE isn't registered, you have no information about their awareness of the tag, nor approval of it. One might say, if they registered to approve tags, and allow the tags to be passively approved, they have, by their inaction, given up their right to challenge the usage of their transmission reservation.  That is ok with me, but perhaps we should remove the word 'active' from section 4 and 5 where it describes the indication provided to the transmission provider.I just don't want to paint a picture of the TP knowing that the transmission rights are ok for the tag author to use, when in reality, there may or may not be adequate information to determine if the rights holder has approved use of his transmission service. In part 6, does this estimate include modifying the TSIN registry to include transmission rights owners?  Also, you mentioned in section 10 back-posting secondary market transaction information to OASIS from the tag system.  Is this included in your estimate of the cost of implementing this enhancement?  Would the transmission rights owner information be verified on the tag from the OASIS?

· On a high level we support this kind of enhancement to E-Tag.  It enables the PSE to better manage their transmission reservations by being granted E-Tag approval rights over their transmission reservation segments - thereby minimizing the opportunity for another PSE to inadvertently (or intentionally) scheduling on their transmission reservation without authority.  The downside is the potential for additional effort associated with approving tags and the possibility of a delay.

 

