Action Item from July 8, 2005 OASIS 1S Task Force Conference Call:  Ms. Weathers will draft language to clarify the term “all other types” as used in Standard 10.6.

FERC questioned the use of the term “all other types” in their Notice of Proposed Rulemaking, RM05-5-000, dated May 9, 2005.  It appears that Non-Firm service is different from Secondary service, therefore the term “all other types of Non-Firm Secondary Point-to-Point Service” does not make sense and should be changed. I am also recommending that Standard 2.2.3 be added to include Secondary service as a TS_Class as is created by Standard 10.  The suggested redlines are as follows:
Standard 10 – Requirements for dealing with Redirects on a Non-Firm basis

10.6 - For the purposes of curtailment and other capacity reductions, confirmed Redirects on a Non-Firm basis shall be treated comparably to Non-Firm Point-to-Point Service.  

Standard 2 – Standard Terminology for Transmission and Ancillary Services

Standard 2.2:  A Transmission Provider shall use the values and definitions below to describe the service class, TS_CLASS, for transmission services offered on OASIS, or shall post alternative TS_CLASS attribute values and associated definitions on the OASIS Home Page at http://www.tsin.com, or shall use the attribute values and definitions posted by other Transmission Providers.  (See Standard 3 for registration requirements.)

Standard 2.2.1: Firm - Transmission service that always has priority over NONFIRM transmission service and includes Native Load Customers, Network Customers, and any transmission service not classified as non-firm in accordance with the definitions in the pro forma tariff.

Standard 2.2.2:  Non-Firm - Transmission service that is reserved and/or scheduled on an as-available basis and is subject to curtailment or interruption at a lesser priority compared to Firm transmission service, including Native Load Customers and Network Customers, in accordance with the definitions in the pro forma tariff.
Standard 2.2.3:  SECONDARY – Transmission service that is reserved on an alternate, or secondary, Point of Receipt and/or Point of Delivery and is subject to curtailment or interruption at a priority equal to NON-FIRM transmission service.
July 19, 2005

W. Weathers

