Pipeline Segment Work Paper
WGQ BPS - 9/11/06


Proposed Standard 4.3.x1:
For data provided pursuant to NAESB WGQ Standard No. 4.3.90, a Transportation Service Provider (TSP) with tariff based gas quality provisions for the control of interchangeability should calculate a Wobbe Number for location(s) that are representative of mainline gas flow. Where a TSP uses an alternative method to characterize interchangeability, it may substitute or supplement the Wobbe Number with the applicable data.
Proposed Standard 4.3.x2:
For data provided pursuant to NAESB WGQ Standard No. 4.3.90, a Transportation Service Provider (TSP) with tariff-based gas quality provisions for the control of hydrocarbon dropout should measure or calculate a Cricondentherm Hydrocarbon Dew Point (CHDP) for location(s) that are representative of mainline gas flow. Where a TSP uses an alternative method to control hydrocarbon liquid dropout, e.g. the C6+ GPM method, it may substitute or supplement the CHDP with the applicable data.
Proposed modifications to Standard 4.3.90:
The Transportation Service Provider (TSP) should provide on its Informational Postings Web Site daily average gas quality information for prior gas day(s), to the extent available, for location(s) that are representative of mainline gas flow. The information available for the identified location(s) should be provided in a downloadable format. Information should be reported in units as specified in the tariff or general terms and conditions. In any event, all applicable parties’ compliance with gas quality requirements is in accordance with the TSP’s tariff or general terms and conditions.
The following are examples of gas quality attributes that could be included in the posting for the applicable Gas Day(s) and location(s):
Listed below are examples of gas quality attributes:
• Heating Value
• Interchangeability factor(s)
• Hydrocarbon liquid dropout control factor(s)
• Hydrocarbon Components, % of C1 – Cnn, as used in determining Heating Value
• Specific Gravity
• Water
• Nitrogen
• Carbon Dioxide
• Oxygen
• Hydrogen
• Helium
• Total Sulfur
• Hydrogen Sulfide
• Carbonyl Sulfide
• Mercaptans
• Mercury and/or any other contaminants being measured
• Other pertinent gas quality information that is specified in the TSP’s tariff or the general terms and conditions.
