Comments Submitted by E. Davis, Entergy


ENTERGY COMMENTS
WEQ 2008 AP 2.a.iv.4, 2.a.vi.4 and 2.b.ii.1

Existing Transmission Commitments

Due July 21, 2008

The proposed standard in 001-y.3 requires that the Transmission Provider shall set the rollover capacity to zero once the deadline for the Transmission Customer to submit a renewal request has passed.  However, there could be cases when the Transmission Customer has submitted a renewal request for exercising the Rollover Rights and it has not  been either acted upon by the Transmission Provider or confirmed by the Transmission Customer..  In such situations, removing rollover capacity from the ETC will result in inaccurate ATCs that are going to be posted on OASIS.  

EXAMPLE:
A Transmission Customer has Rollover Rights of 500 MW over a path beginning January 1, 2010 with a deadline for a renewal request of September 30th 2009 for exercising these rights.  The Transmission Customer has submitted a renewal request on September  30th 2009, just before the deadline, According to the proposed standard 001-y.3, the Transmission Provider will set the rollover capacity to zero on October 1st 2009 for the period January 1, 2010 on wards.  This action will result in removal of 500 MW of Rollever Rights capacity from the ETC and will result in increase in ATC for the period beginning January 1, 2010 onward by 500 MW.  Therefore, the ATC will be inaccurate as the status of  Rollover Right has actually not changed since the Transmission Customer is in the process of exercising these rights but has not converted this into the actual reservation.   

RECOMMENDATION: 
In order to accurately reflect Rollover Rights in ETC that will result in accurate ATC calculations, Entergy suggests that this standards should be modified to append the language at the end of standard 001-y.3:

 “provided Transmission Customer  has not submitted a renewal request.  If Transmission Customer has submitted a renewal request before the deadline, Transmission Provider shall wait to set the rollover capacity to zero  till the deadline for confirmation of the  renewal requets has passed and the Transmission Customer has not confirmed such request.  If the renewal request is for an amount lesser than the original Rollover Right, the Transmission Provider shall set the Rollover Rights capacity  to the capacity of renewal request in process”.
001-y.3
Once the deadline for the Transmission Customer to submit a renewal request has passed for a Long Term Firm Point-to-Point reservation, the Transmission Provider shall set the rollover capacity to zero, provided Transmission Customer  has not submitted a renewal request.  If Transmission Customer has submitted a renewal request before the deadline, Transmission Provider shall wait to set the rollover capacity to zero  till the deadline for confirmation of the  renewal requets has passed and the Transmission Customer has not confirmed such request.  If the renewal request is for an amount lesser than the original Rollover Right, the Transmission Provider shall set the Rollover Rights capacity  to the capacity of renewal request in process.

