To:
NAESB WEQ Executive Committee:

From:
Kathy York, Jim Busbin co-chairs of the WEQ Executive Committee

Date:
October 16, 2007

RE:
Minor Corrections to the WEQ TLR business practice standards

During the development of the NERC TLR Reliability Standards, several additional minor corrections were identified by the NAESB WEQ BPS and NERC TLR Standards Drafting Team.  These minor corrections can be viewed in the attached redlined NAESB WEQ TLR Business Practice Standard.  The NAESB WEQ Business Practices Subcommittee voted to approve these minor corrections and to post them to the WEQ EC for your consideration.  

These minor corrections are identified below for your convenience:

3.2.5. If an Interchange Transaction is identified as a Dynamic Schedule and the Transmission Service is considered Firm according to the constrained path method, then it will not be held by the IDC during TLR level 4 or lower.  Adjustments to Dynamic Schedules in accordance with NERC INT-004-1 
will not be held under TLR level 4 or lower. 
3.3.1
The Reliability Coordinator shall allow those Interchange Transactions using Firm Transmission Service that have been submitted prior to the NERC-approved tag submission deadline for Reallocation (as found in current version of IRO-006-4
 ),to be initiated as scheduled.

3.3.1.2. Reallocations for Dynamic Schedules are as follows: If an Interchange Transaction is identified as a Dynamic Schedule and the Transmission Service is considered Firm according to the constrained path method, then it will not be held by the IDC during TLR level 4 or lower.  Adjustments to Dynamic Schedules in accordance with current version of NERC INT-004-1
 will not be held under TLR level 4 or lower.

3.4.1.2. Reallocations for Dynamic Schedules are as follows: If an Interchange Transaction is identified as a Dynamic Schedule and the Transmission Service is considered Firm according to the constrained path method, then it will not be held by the IDC during TLR level 4 or lower.  Adjustments to Dynamic Schedules in accordance with current version of NERC INT-004-1
 will not be held under TLR level 4 or lower.
3.5.2.1.If an Interchange Transaction is identified as a Dynamic Schedule and the Transmission Service is considered Firm according to the constrained path method, then it will not be held by the IDC during TLR level 4 or lower.  Adjustments to Dynamic Schedules in accordance with current version of NERC INT-004-1
 will not be held under TLR level 4 or lower.
Thank you,

Kathy York and Jim Busbin
Co-Chairs of the WEQ BPS

�Deleted R5 added -1


�Added -4


�Added -1


�Added -1


�Added -1





