

[image: image1.wmf] 


RECOMMENDATION TO NAESB EXECUTIVE COMMITTEE

                                       For Quadrant: WEQ

                                       Requesters:


                                       Request No.: 


                                       Request Title:
ATC/TTC Narratives

OATI Comments on ATC/TTC Narratives dated 11/07/2007:
1.  RECOMMENDED ACTION:
EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

  X   Accept as requested


   X  Change to Existing Practice

      Accept as modified below


      Status Quo

      Decline

2.  TYPE OF DEVELOPMENT/MAINTENANCE

Per Request:




Per Recommendation:
   X Initiation




   X  Initiation 

      Modification




      Modification

      Interpretation



      Interpretation

      Withdrawal




      Withdrawal

      Principle 




      Principle 

      Definition 




      Definition 

  X  Business Practice Standard 


  X   Business Practice Standard 

      Document 




      Document 

      Data Element 



      Data Element

      Code Value 



      Code Value 

      X12 Implementation Guide


     X12 Implementation Guide

      Business Process Documentation

      Business Process Documentation

3.  RECOMMENDATION

SUMMARY:


In the FERC Order 890 under Docket Nos. RM05-17-000 and RM05-25-000 dated February 16, 2007, Commission made the following conclusions about ATC/TTC Narratives:

369. The Commission adopts the NOPR proposal, with the modifications discussed below, to require that the transmission provider post a brief, but specific, narrative explanation of the reason for a change in monthly and yearly ATC values on a constrained path.  Rather than requiring a narrative when a monthly or yearly ATC value changes as a result of transactions being reserved, service ending, or the TTC estimate for the path changing by more than 10 percent, we will require a narrative when a monthly or yearly ATC value changes only as a result of a 10 percent change in TTC.  This will reduce the number of ATC changes for which a narrative will be required and address concerns that the new requirement unduly burdens transmission providers.  Any remaining burden is justified by the benefit to transmission customers of receiving timely information regarding changes in TTC that result in changes to ATC.  In addition, we adopt NAESB’s suggestion that posted information include the (1) specific events which gave rise to the change and (2) new values for ATC on that path (as opposed to all points on the network).

370. We reject calls for delays prior to posting data.  While commenters allege the possibility of granting others a competitive advantage through the release of “market-sensitive” data, they have proffered no evidence to support the allegation of potential harm.

371. We do require, as suggested in the NOPR, a narrative with regard to monthly or yearly ATC values when ATC remains unchanged at a value of zero for a significant period, and will set that period at six months or longer.  This information will be valuable to customers and regulators in assessing the ability of a transmission provider’s facilities to meet existing service requests.  The information also will provide assurance to customers that the transmission provider is diligent in regularly evaluating ATC on all paths, monitoring persistent constraints and addressing them in its planning processes. 

The ESS/ITS/BPS has reveiwed Order 890 and is recommending adooption of the following standards.  The Subcommittee feels this recommendation complies with the guidance provided by the Commission.
Recommended Standards:

Definitions – The following definitions shall be added to the Business Practice Standards:


Balancing Authority Area – is there a NERC definition for this?
Business Practice Standards for Posting ATC/TTC Narratives
Standard Y:  ATC Narratives

All Transmission Providers shall post on OASIS a narrative with regard to yearly or monthly ATC values when ATC remains unchanged at a value of zero for a six months or longer. 
Standard Y.1:  Frequency and Timing of ATC Narratives

  The Transmission Provider will post an annotation in conjunction with monthly or yearly ATC postings each time one of the following conditions are met:
· A given Monthly ATC value has been posted at a value of zero for the past six months or longer
· Monthly ATC has been posted at a value of zero for a duration of six consecutive months or longer at some time in the future
· Yearly ATC has been posted at a value of zero for one or more future years
Standard Y.2: Posting On OASIS
ATC Narratives shall be posted for each path whose posted Monthly or Yearly ATC meets the criteria specified  in Standard Y.1.
Standard Y.2.1  ATC Narratives shall be posted in accordance with OASIS Implementation Guide (WEQ-013) Version 1.5 and OASIS Standards and Communication Protocols (WEQ-002) Version 1.5 such that they may be queried and viewed on OASIS using the systemdata template.
Standard Y.2.2  ATC Narrative posting shall include in the systemdata template the following information for each path:

· SYSTEM_ELEMENT_TYPE = “PATH”
· SYSTEM_ELEMENT  = Name of the path associated with the posted ATC component value
· SYSTEM_ATTRIBUTE = “ATC_ANNOTATION”

· START_TIME = Date and time that the Monthly Firm ATC initially became zero, which may prior to the date of the posting
· STOP_TIME = Date and time when ATC  is expected to be greater than zero or last date that  Monthly or Yearly Firm ATC is posted
· ATTRIBUTE_VALUE = “0”
· ATTRIBUTE_UNITS = “MW”
· ANNOTATION text shall minimally match one of the Annotation Text options set forth in Standard Y.2.3; Transmission Provider’s may add additional narrative text following the standardized annotation.
Standard Y.2.3  Annotation text shall indicate the status of the path’s ATC.  The following table lists the narrative text options that are permissible:
	Annotation Text
	Explanation

	ATC set to zero for past six months or longer
	Initial instance of given monthly or yearly ATC set to zero for at least the past six months

	ATC remains at zero for longer than six months
	Continued reporting of an instance of monthly or yearly ATC set to zero for six consecutive months or longer.  


	
	


Standard Y.2.4  Once an ATC Narrative has been posted for a given transmission path, and the condition  that initiated the ATC Narrative posting still exists, the Transmission Provider shall only be required to post a new ATC Narrative every six months.

Standard Z:  TTC Narratives

All Transmission Providers shall post on OASIS a brief, but specific, narrative explanation of the reason for a change in monthly or yearly ATC values on a constrained path when a monthly or yearly ATC value changes as a result of a 10 percent or greater change in TTC.  

Standard Z.1: Frequency and Timing of TTC Narratives
TTC Narrative postings shall be made each time TTC is changed by 10 percent or greater.  . 
Standard Z.2: TTC Narrative Posting on OASIS
TTC Narratives shall include the (1) specific events which gave rise to the change in TTC and (2) new values for TTC on that path.
Standard Z.2.1  TTC Narratives shall be posted in accordance with OASIS Implementation Guide (WEQ-013) Version 1.5 and OASIS Standards and Communication Protocols (WEQ-002) Version 1.5 such that they may be queried and viewed on OASIS using the systemdata template.
Standard Z.2.2  TTC Narrative posting shall include in the systemdata template the following information for each path:

· SYSTEM_ELEMENT_TYPE = “PATH”
· SYSTEM_ELEMENT  = Name of the path associated with the posted TTC component value
· SYSTEM_ATTRIBUTE = “ATC_ANNOTATION”
· START_TIME = Date and time that the TTC was changed by at least ten percent (10%)
· STOP_TIME = Date and time for the end of the TTC change posting
· ATTRIBUTE_VALUE =  new TTC value
· ATTRIBUTE_UNITS = “MW”
· 
ANNOTATION text shall comply with Standard Z.2.3
Standard Z.2.3  Annotation text shall include the following:
1. The magnitude of the change to the nearest percent (e.g., TTC increased by 14%)

· 
2. The Balancing Authority Area whose evaluation triggered the change.

3. If the TP’s Balancing Authority Area’s evaluation triggered the change, identify which of the following caused the change (include one or more):

· Load Forecast Change

· Generation Change

· Transmission Change

· Addition or Removal of a Flowgate
· Facility Rating Change
Standard Z.3: Frequency and Timing Associated ATC Postings
Whenever the Transmission Provider is required to post a TTC Narrative as required by standard Z.1, the Transmission Provider shall post any new values for monthly and yearly ATC on all paths which are impacted by the TTC change. 
4.  SUPPORTING DOCUMENTATION
a.  Description of Request:

ORDER 890 WORK PLAN, ESS/ITS ASSIGNMENTS, GROUP 1: POSTING OF ETC; ANNOTATIONS FOR ATC; LOAD FORECAST AND ACTUAL LOAD; RE-BID OF PARTIAL SERVICE; AND PRECONFIRMATION PRIORITY
b.  Description of Recommendation:

c.  Business Purpose:

d.  Commentary/Rationale of Subcommittee(s)/Task Force(s):
�We should standardize on calling these Narratives or Annotations – either one but be consistent throughout.


�Note that the Implementation Guide called for a unique record type for annotations such that the ANNOTATION data element is not required to be carried as part of the time series values for FATC or NFATC.  If this is not the concept envisioned by others, OATI supports SoCo’s recommendation to remove the ANNOTATION from systemdata and move these postings to the security (or other new) template.


�I was not sure if these two “annotations” were to distinguish between a given month set to 0 for the past six month vs. atc of 0 for six consecutive months OR were just for the initial annotation and then any re-posting of the annotation.  Once that’s clear, may revise these annotations to be clearer in intent.


�Could debate on frequency of reposting the condition, but would think no more frequently than once per month at max.


�Note that this is slightly different in nature than posting a narrative when ATC changes due to 10% change in TTC.  One could have a 10% change in TTC that is not a limiting element and hence no change in ATC.  Also, if ATC’s 0 and TTC drops, ATC is still 0 (unchanged).  However, I agree that as written this is the standard we should implement.  


�Intent of the ATC_ANNOTATION record is to not duplicate the actual TTC data postings.  This value should be called out as the first TTC (value @start_time), or Min TTC over start/stop, or other.  May have a TTC change that spans a long period of time but actual TTC value itself changes many times over that interval – it’s the same event so it’s a single narrative even though influeance many TTCs over time.





3
Thursday, January 01, 2004

Page 1

[image: image2.wmf] 

_1075620371.doc
[image: image1.png]|

Lo







