[image: image1.wmf] 


RECOMMENDATION TO NAESB EXECUTIVE COMMITTEE

                                       For Quadrant: 
WEQ

                                       Requesters:
WEQ Joint ESS/ITS and BPS Subcommittee

                                       Request No.: 
2008 WEQ AP Items 2.b.iii.1-3
                                       Request Title:
Capacity Benefit Margin (CBM) 


The following suggested revisions to the Coordinate Interchange Standards WEQ-004 related to the scheduling/tagging of CBM.  These changes will be incorporated into the CBM Standard Recommendation that includes changes to WEQ-001, WEQ-002, WEQ-003, WEQ-013, and 

Modifications to WEQ-004 (New Requirements)

Coordinate Interchange
NOTE: NAESB Staff will assign appropriate enumeration to this standard to replace “n”, with next sequential standard number within WEQ-004
004-n 
All scheduled use of a Transmission Service Provider’s transmission capacity set-aside for Capacity Benefit Margin (CBM) in support of energy imports into a load Balancing Authority served by the Transmission Service Provider shall be uniquely represented in all Requests For Interchange submitted to the IA.
004-n.1 
Until other means for submitting the RFI are adopted by NAESB, the following data fields shall be specified in each e-Tag requesting the use of the Transmission Service Provider’s CBM:

· The e-Tag transaction type shall be EMERGENCY

· The Transmission PSE (TPSE) associated with the Transmission Service Provider whose use of CBM is being requested shall identify the Load Serving Entity requesting use of CBM.  Note that this is not necessarily the PSE submitting the e-Tag.

· The Transmission Product associated with the Transmission Service Provider whose use of CBM is being requested shall be 7-CB 

004-n.2
The Transmission Service Provider may require the specification of a unique Transmission Reservation Number in association with any request for use of CBM.  Such requirement shall be fully documented in the Transmission Service Provider’s Business Practices posted on OASIS.  The TSP reserves the right to deny any RFI requesting use of CBM if the required Transmission Reservation Number is not specified.

July 9, 2008
Page 1

[image: image2.wmf] 

_1075620371.doc
[image: image1.png]|

Lo







