Revised: September 13, 2008

Group Comments on

Options for Providing the Capability in the E-Tag System for

Supporting Multiple Reliability Limits

On behalf of the following group of organizations, I submit these comments for the JISWG’s consideration.

AESO, ERCOT, IESO, ISO-NE, MISO
, PJM and SPP
We generally support the concept of enhancing reliability by inclusion of additional system limitations that may affect transmission service reservations and subsequent implementation of Interchange Transactions. Specific to the questions asked in the proposal, we offer the following comments: 
1. We agree that reliability entities whose limits are shown on an e-Tag must maintain control over the limits they have set. Further, any reliability entity may set different limits on different physical segments that they are associated with to reflect different generation or transmission constraints (or restoration). The proposal calls for “… the Authority service maintain a separate reliability profile. Only the entity issuing the reliability limit may modify it (with the exception of ATF DYNAMIC adjustments).” We are not clear on how this process may be implemented.
If the proposed approach is for the Transmission Service Provider (TSP) to create and maintain this profile based on inputs from the reliability entities such as Transmission Operators (TOPs) and Reliability Coordinators (RCs), then the process would work. Otherwise, the process may not work since neither the TOPs nor the RCs receive tags during the Arrange Interchange stage. The RCs will only be able to view tag information after an Interchange Transaction becomes an Interchange Schedule.

2. We are not clear on the intent of the question: “Should the RC be given the capability of modifying (or eliminating) limits set by other reliability entities?” Does it mean a process that will allow an RC to make changes to a limit on a tag, or does it mean an RC to override the limits set by others on the same transmission segment by other process? If it is the former, the RC does not receive tag information and hence is unable to make any changes. If it is the latter, RCs and TOPs that monitor a transmission facility or path are required by NERC standards to observe the lowest value of multiple limits on that facility or path. We thus do not see the need for any modification for so long as the Transmission Service Provider selects the lowest value on a particular segment that has multiple limits provided by more than one reliability entity.

3. We do not have any additional features to suggest.
4. On the final question, we are unable to offer a preference since we are unclear on who that “Authority” is in approving limit changes. As indicated in the proposal paper, the reliability entity that owns these limits maintains control over these limits and hence it is the provider as well as the inherent approval body for these limits. Hence, updating the limits on the tag does not require an explicit, third party approval, unlike the submission of E-tags for approving Interchange Transactions. We would therefore appreciate the further elaboration on such an approval process as envisaged by the JISWG.
Thank you for your attention to our comments.

Yours truly,

Charles Yeung
Charles Yeung

SPP
� MISO supports all comments except Comment No. 2





