Comments of the National Rural Electric Cooperative Association on the Draft NAESB Standards for Performance Evaluation of Demand Response Resources for Wholesale Electricity Markets – Prepared in Conjunction with GDS Associates



October 22, 2008
Comments of the National Rural Electric Cooperative Association
on the Draft NAESB Standards for Performance Evaluation of Demand Response Resources for Wholesale Electricity Markets-Prepared in Conjunction with GDS Associates
1.0
Summary of NAESB Standards for Demand Response and DSM-EE Programs
1.  These standards were developed by the Independent System Operators and Regional Transmission Organizations (Collectively, System Operators) in North America through the North American Energy Standard Board (NAESB) stakeholder process. The standards reflect business practices applicable to measurement and verification of wholesale market Demand Response (Demand Response or DR) services including the following four product categories: 

Energy Service

A type of Demand Response service in which Demand Resources are 
compensated based solely on Demand reduction performance.

Capacity Service

A type of Demand Response service in which Demand Resources are 
obligated over a defined period of time to be available to provide Demand 
Response upon deployment by the System Operator.

Reserve Service

A type of Demand Response service in which Demand Resources are 
obligated to be available to provide Demand reduction upon deployment 
by the System Operator, based on reserve capacity requirements that are 
established to meet applicable reliability standards.  

Regulation Service

A type of Demand Response service in which a Demand Resource 
increases and decreases Load in response to real-time signals from the 
System Operator. Demand Resources providing Regulation Service are 
subject to dispatch continuously during a commitment period.  Provision of 
Regulation Service does not correlate to Demand Response Event 
timelines, deadlines and durations.

Below is a breakdown of each product category and the framework within each to follow:

[image: image1.wmf]Energy Service

Capacity Service

Reserve Service

Regulation Service

Advance 

Notification

System Operator to 

specify

System Operator to 

specify

System Operator to 

specify

N/A unless otherwise 

specified by the S.O.

Deployment 

Time

System Operator to 

specify

System Operator to 

specify

System Operator to 

specify

N/A unless otherwise 

specified by the S.O.

Reduction 

Deadline

System Operator to 

specify

System Operator to 

specify

System Operator to 

specify

N/A unless otherwise 

specified by the S.O.

Release/Recall

System Operator to 

specify

System Operator to 

specify

System Operator to 

specify

N/A unless otherwise 

specified by the S.O.

Normal 

Operations

System Operator to 

specify

System Operator to 

specify

System Operator to 

specify

N/A unless otherwise 

specified by the S.O.

Demand 

Resource 

Availability 

Measurement

N/A

System Operator to 

specify requirements 

for measuring the 

capability of a DR to 

meet it's obligation

System Operator to 

specify requirements 

for measuring the 

capability of a DR to 

meet it's obligation

N/A unless otherwise 

specified by the S.O.

Aggregation

System Operator to 

specify

System Operator to 

specify

System Operator to 

specify

System Operator to 

specify

Transparency of 

Requirements

System Operator to 

post publicly defined 

requirements

System Operator to 

post publicly defined 

requirements

System Operator to 

post publicly defined 

requirements

System Operator to 

specify

GENERAL


[image: image2.wmf]Energy Service

Capacity Service

Reserve Service

Regulation Service

Telemetry 

Requirement

System Operator to 

specify requirements

System Operator to 

specify requirements

System Operator to 

specify requirements

System Operator to 

specify requirements

On-Site 

Generation 

Telemetry

Real-time data 

required if on-site 

generation is present 

behind the primary 

Telemetry point 

unless otherwise 

specified by S.O.

Real-time data 

required if on-site 

generation is present 

behind the primary 

Telemetry point 

unless otherwise 

specified by S.O.

Real-time data 

required if on-site 

generation is present 

behind the primary 

Telemetry point 

unless otherwise 

specified by S.O.

Real-time data 

required if on-site 

generation is present 

behind the primary 

Telemetry point 

unless otherwise 

specified by S.O.

Telemetry 

Accuracy

To be represented as 

a % of full scale up to 

a max of 3.0%.  

Unless otherwise 

specified by S.O.

To be represented as 

a % of full scale up to 

a max of 3.0%.  

Unless otherwise 

specified by S.O.

To be represented as 

a % of full scale up to 

a max of 3.0%.  

Unless otherwise 

specified by S.O.

To be represented as 

a % of full scale up to 

a max of 3.0%.  

Unless otherwise 

specified by S.O.

Telemetry 

Reporting 

Interval

Shall be up to a max 

of 5 mins unless 

otherwise specified by 

S.O.

Shall be up to a max 

of 5 mins unless 

otherwise specified by 

S.O.

Shall be up to a max 

of 5 mins unless 

otherwise specified by 

S.O.

Shall be up to a max 

of 5 mins unless 

otherwise specified by 

S.O.

Other Telemetry 

Measurements

S.O. to specify

S.O. to specify

S.O. to specify

S.O. to specify

Communication 

Protocol

S.O. to specify

S.O. to specify

S.O. to specify

S.O. to specify

Governor 

Control 

Equivalent

N/A unless otherwise 

specified by S.O.

N/A unless otherwise 

specified by S.O.

N/A unless otherwise 

specified by S.O.

DR's providing 

regulation service 

shall auto respond to 

grid frequency 

deviations unless 

otherwise specified by 

S.O.

TELEMETRY


[image: image3.wmf]Energy Service

Capacity Service

Reserve Service

Regulation Service

After-the-fact 

Metering 

Requirement

Required unless 

otherwise specified by 

S.O.

Required unless 

otherwise specified by 

S.O.

Required unless 

otherwise specified by 

S.O.

Required unless 

otherwise specified by 

S.O.

Meter Accuracy

To be represented as 

a % of full scale up to 

a max of 3.0%.  

Unless otherwise 

specified by S.O.

To be represented as 

a % of full scale up to 

a max of 3.0%.  

Unless otherwise 

specified by S.O.

To be represented as 

a % of full scale up to 

a max of 3.0%.  

Unless otherwise 

specified by S.O.

To be represented as 

a % of full scale up to 

a max of 3.0%.  

Unless otherwise 

specified by S.O.

Details of 

Meter/Equipmen

t Standards

Standards shall meet 

or exceed industry 

standards equivalent 

to ANSI C12 unless 

otherwise specified by 

the S.O.

Standards shall meet 

or exceed industry 

standards equivalent 

to ANSI C12 unless 

otherwise specified by 

the S.O.

Standards shall meet 

or exceed industry 

standards equivalent 

to ANSI C12 unless 

otherwise specified by 

the S.O.

Standards shall meet 

or exceed industry 

standards equivalent 

to ANSI C12 unless 

otherwise specified by 

the S.O.

Meter Data 

Reporting 

Deadline

System Operator shall 

specify 

System Operator shall 

specify 

System Operator shall 

specify 

System Operator shall 

specify 

Meter Data 

Reporting 

Interval

Shall be a maximum 

of 1 hour unless 

otherwise specified by 

the S.O.

Shall be a maximum 

of 1 hour unless 

otherwise specified by 

the S.O.

Shall be a maximum 

of 1 hour unless 

otherwise specified by 

the S.O.

Shall be a maximum 

of 1 hour unless 

otherwise specified by 

the S.O.

Clock/Time 

Accuracy

Standards shall meet 

or exceed industry 

standards equivalent 

to ANSI C12 unless 

otherwise specified by 

the S.O.

Standards shall meet 

or exceed industry 

standards equivalent 

to ANSI C12 unless 

otherwise specified by 

the S.O.

Standards shall meet 

or exceed industry 

standards equivalent 

to ANSI C12 unless 

otherwise specified by 

the S.O.

Standards shall meet 

or exceed industry 

standards equivalent 

to ANSI C12 unless 

otherwise specified by 

the S.O.

Validating, 

Editing and 

Estimating (VEE) 

Method

System Operator shall 

specify 

System Operator shall 

specify 

System Operator shall 

specify 

System Operator shall 

specify 

On-Site 

Generation 

Meter 

Requirement

System Operator shall 

specify additional 

metering 

requirements if on-site 

generation is present 

behind the primary 

metering point

System Operator shall 

specify additional 

metering 

requirements if on-site 

generation is present 

behind the primary 

metering point

System Operator shall 

specify additional 

metering 

requirements if on-site 

generation is present 

behind the primary 

metering point

System Operator shall 

specify additional 

metering 

requirements if on-site 

generation is present 

behind the primary 

metering point

AFTER-THE-FACT-METERING


[image: image4.wmf]Energy Service

Capacity Service

Reserve Service

Regulation Service

Rules for 

Performance 

Evaluation

Shall be evaluated 

through the use of 

one of the following 

methods unless 

otherwise specified by 

the S.O.

• Maximum Base 

Load

• Meter Before / Meter 

After

• Baseline Type-I

• Baseline Type-II

• Metering Generator 

Output



Shall be evaluated 

through the use of 

one of the following 

methods unless 

otherwise specified by 

the S.O.

• Maximum Base 

Load

• Meter Before / Meter 

After

• Baseline Type-I

• Baseline Type-II

• Metering Generator 

Output



Shall be evaluated 

through the use of 

one of the following 

methods unless 

otherwise specified by 

the S.O.

• Maximum Base 

Load

• Meter Before / Meter 

After

• Baseline Type-I

• Baseline Type-II

• Metering Generator 

Output



Shall be evaluated 

using telemetry data 

and additionally 

through use of one of 

the following methods 

unless otherwise 

specified by the S.O.

• Meter Before / Meter 

After

• Metering Generator 

Output



PERFORMANCE EVALUATION


2.0 Comments from NRECA
2.1 Overview

2.  The purpose of this draft proposal is to create compatibility between markets. The National Rural Electric Cooperative Association (NRECA) generally is supportive of these proposed standards as they currently are written.  However, after comparing these standards to other M&V standards (IPMVP in particular), NRECA notes that there are few specific details provided in the definitions and guidelines for each step in the performance evaluation process. This lack of specificity can provide sufficient flexibility to support the current programs already underway in the markets which have been designed to support multiple DR programs for varying industry and operational structures. Nonetheless, only providing general guidance on reaching compatibility across markets, in some cases, may lead to confusion. For example, without further guidance in the recommended standards, as the level of experience in the industry declines in the coming years, confusion eventually could develop as new and inexperienced staff becomes more involved in these programs.  To help minimize potential confusion, NRECA agrees with the suggestion made at the October 3rd NAESB meeting to provide examples of proposed performance evaluation methodologies superimposed on products within each category as an appendix to the document.  

3.  So long as these examples are provided, are reasonable and do not change the draft proposal, NRECA concludes that the draft proposal offers a framework in which smaller entities, such as electric cooperatives, may be able to participate.  However, NRECA is concerned that the draft standards may be amended through the NAESB process in ways that would produce unnecessary and unproductive requirements for cooperatives participating in these markets.  NRECA is also concerned that System Operators could use the flexibility offered by the draft proposal to implement M&V methodologies that may be overly and unnecessarily burdensome on participating cooperatives, or that would prove overly prescriptive with respect to the types of Demand Response programs that cooperatives could implement. The oft-repeated phrase “unless otherwise specified by the system operator” should not be read as a blank check for System Operators to impose, through applicable tariff language, new M&V requirements on market participants. System operators should consider the impact that those requirements might have on cooperatives and other smaller market participants, or on the broad range of potential Demand Response programs that cooperatives tend to offer their member-consumers.
4.  As currently drafted, the proposed wholesale standards appropriately treat each DR product category and evaluation mechanisms the same. NRECA recommends that any wholesale standards NAESB submits to the Federal Energy Regulatory Commission retain that impartiality and do not favor one product, mechanism or industry structure over another.
2.2 Detailed Comments from NRECA
1. 5.  On page 7 of the standards, it is stated that “For purposes of these Measurement and Verification standards, Demand Response does not include Measurement and Verification of energy efficiency or permanent Load reduction.” NRECA supports this approach and suggests that an example be provided here to clarify what is and what is not covered by these standards. For example, non-permanent demand reduction from an interruptible rate tariff is covered by these standards, but permanent demand reduction from a compact fluorescent lighting program would not be covered by these standards. 

2. 6.  Page 6 of the standards lists four product categories of Demand Response services. After consulting with GDS, who has experience working with these four product categories with the ISO New England, PJM, MISO, ERCOT and other independent System Operators, NRECA recommends that concrete examples of each of these product categories be provided in the appendices of the standards. These examples are needed because users of the standards will have varying degrees of knowledge and experience with these four product categories. Having such examples will be very important for electric cooperative staff as they become familiar with the various types of DR resources used in wholesale electric markets. In addition, many of the experienced and knowledgeable electric cooperative staff in the baby boomer generation will be retiring over the next decade, and it will be critical to have examples of the four product categories in order to train the generation of new employees who will just be graduating from college and entering the electric cooperative work force over the next decade. The draft M&V standards as currently written assume that the reader is already very familiar with the four product categories, and this certainly will not be the case over the next decade. 

3. 7.  Pages 9 and 10 of the standards list five acceptable performance evaluations methodologies for assessing the performance of Demand Response resources.  After consulting with GDS, who has experience working with these performance evaluation methodologies across North America, NRECA recommends that concrete examples of each of these evaluation methodologies be provided in the appendices of the standards. These performance evaluation examples are needed because users of the standards, including cooperatives, will have varying degrees of knowledge and experience with these evaluation techniques. As with the product categories, having such examples of performance evaluation methodologies will be very important for electric cooperative staff as they become familiar with the various types of Demand Response resources used in wholesale electric markets and the evaluation methodologies.

8.  As noted above, many of the experienced and knowledgeable electric cooperative staff in the baby boomer generation will be retiring over the next decade, and it will be critical to have examples of the five performance evaluation methodologies in order to train the generation of new employees who will just be graduating from college and entering the electric cooperative force over the next decade. The draft M&V standards as currently written assume that the reader is already very familiar with the five performance evaluation methodologies, and this certainly will not be the case over the next decade.  Thus, having concrete examples of how these methodologies are actually developed and used, including research designs and algorithms is essential. 

Submitted By:

National Rural Electric Cooperative Association
Mary Ann Ralls

Senior Regulatory Counsel

4301 Wilson Boulevard

Arlington, Virginia 22203

Tel: 703.907.5837

Fax: 703.907.5517

Maryann.ralls@nreca.coop
PAGE  
8

