Maximum Base Load: A performance evaluation methodology based solely on a Demand Resource’s ability to reduce to a specified level of electricity demand, regardless of its electricity consumption or demand at Deployment.
Example

[image: image1.emf]1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24

Reduction

8.0 8.0 9.0 10.0 9.0 7.0 8.0 8.0 8.0 6.0 6.0 6.0 6.0 6.0 6.0 6.0 6.0 6.0 9.0 10.0 9.0 3.0 2.0 2.0

Normal Use

8.0 8.0 9.0 10.0 9.0 7.0 8.0 8.0 8.0 9.0 9.0 9.0 9.0 9.0 9.0 9.0 9.0 9.0 9.0 10.0 9.0 3.0 2.0 2.0

0.0

2.0

4.0

6.0

8.0

10.0

12.0

kW

Hour

Maximum Base Load

Customer  required to reduced load by 2kW

for SustainedResponse Period

Deployment


Meter Before / Meter After:  A performance evaluation methodology where electricity consumption or demand over a prescribed period of time prior to Deployment is compared to similar readings during the Sustained Response Period.
Example

[image: image2.emf]1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24

Data

80 80 90 100 90 70 80 80 80 40 40 40 40 40 40 40 40 40 60 90 90 90 90 80

20

30

40

50

60

70

80

90

100

110

kW

Meter Before/Meter After

Demand Response

Meter Before

Meter After

Customer reduced load by ~40 kW during Sustained Response Period

Deployment


Baseline Type-I: A Baseline performance evaluation methodology based on a Demand Resource’s historical interval meter data which may also include other variables such as weather and calendar data.
A performance evaluation methodology that calculates the amount of energy a customer would have consumed absent a request to reduce during a demand response event using that customers historical interval meter data which may also include other variables such as weather and calendar data. Examples can be found in my paper “Evaluating Baselines for Demand Response Programs”.
Baseline Type-II: A Baseline performance evaluation methodology that uses statistical sampling to estimate the electricity consumption of an Aggregated Demand Resource where interval metering is not available on the entire population.
A performance evaluation methodology that estimates the amount of energy reduction for a population during a demand response event using statistical sampling techniques to represent that population.    Will supply example when AEIC White Paper is published.
Metering Generator Output: A performance evaluation methodology, used when a generation asset is located behind the Demand Resource’s revenue meter, in which the Demand Reduction Value is based on the output of the generation asset.
Sheet1

		0.8		80

		0.8		80

		0.9		90

		1		100

		0.9		90

		0.7		70

		0.8		80

		0.8		80

		0.8		80

		0.4		40

		0.2		40

		0.2		40

		0.2		40

		0.2		40

		0.3		40

		0.1		40

		0.2		40

		0.4		40

		0.6		60

		1		90

		0.9		90

		0.3		90

		0.2		90

		0.2		80





Chart1



Meter Before/Meter After

Data	80	80	90	100	90	70	80	80	80	40	40	40	40	40	40	40	40	40	60	90	90	90	90	80	kW



Chart2



Maximum Base Load

Reduction	8	8	9	10	9	7	8	8	8	6	6	6	6	6	6	6	6	6	9	10	9	3	2	2	Normal Use	8	8	9	10	9	7	8	8	8	9	9	9	9	9	9	9	9	9	9	10	9	3	2	2	Hour

kW



Sheet2

		0.8		8.0		8.0

		0.8		8.0		8.0

		0.9		9.0		9.0

		1		10.0		10.0

		0.9		9.0		9.0

		0.7		7.0		7.0

		0.8		8.0		8.0

		0.8		8.0		8.0

		0.8		8.0		8.0

		0.4		6.0		9.0

		0.2		6.0		9.0

		0.2		6.0		9.0

		0.2		6.0		9.0

		0.2		6.0		9.0

		0.3		6.0		9.0

		0.1		6.0		9.0

		0.2		6.0		9.0

		0.4		6.0		9.0

		0.6		9.0		9.0

		1		10.0		10.0

		0.9		9.0		9.0

		0.3		3.0		3.0

		0.2		2.0		2.0

		0.2		2.0		2.0





















































Sheet3






Chart1



Meter Before/Meter After

Data	80	80	90	100	90	70	80	80	80	40	40	40	40	40	40	40	40	40	60	90	90	90	90	80	kW



Sheet1

		0.8		80

		0.8		80

		0.9		90

		1		100

		0.9		90

		0.7		70

		0.8		80

		0.8		80

		0.8		80

		0.4		40

		0.2		40

		0.2		40

		0.2		40

		0.2		40

		0.3		40

		0.1		40

		0.2		40

		0.4		40

		0.6		60

		1		90

		0.9		90

		0.3		90

		0.2		90

		0.2		80





Sheet2





Sheet3






