Reference Document
EM&V for Energy Efficiency Programs

Terms and Definitions Not Used in Model Business Practices

RXQ.0.2.x 
Baseline Period - The period of time selected as representative of the operations of the area of focus before the Energy Efficiency activity takes place.

RXQ.0.2.39
Supplier:  Persons engaged in the competitive sale of energy to end-users.

RXQ.0.2.42
Uniform Electronic Transaction:  Standard data arrangements for trading information, making business requests and exchanging other information, encompassing a number of electronic media and utilizing specific transport protocols.

RXQ.0.2.x 
Accuracy - A concept that refers to the relationship between the true value of a variable and an estimate of the value.   The term can also be used in reference to a model, a set of measured data, or to describe a measuring instrument’s capability.  

RXQ.0.2.x 
Achievable Potential - The amount of energy or demand savings within a defined geographical area or population that can be achieved in response to specific Energy Efficiency program designs, delivery approaches, program funding, and measure incentive levels. Achievable potential studies are sometimes referred to as Market Potential studies. Also see Potential Studies.

RXQ.0.2.x 
Advanced Meter - Also called Smart Meter. An electric meter that is capable of measuring and recording usage data in time differentiated registers, allowing electric consumers, suppliers, and service providers to participate in all types of price-based demand response programs and that provides the additional capabilities to address the electric service (e.g. energy use diagnostics, submetering, detection and documentation of power quality). Also see metering.

RXQ.0.2.x 
Allowances - Allowances represent the amount of an air pollutant that a source is permitted to emit during a specified time in the future under a cap and trade program. Allowances are often confused with credits earned in the context of project-based or offset programs, in which sources trade with other facilities to attain compliance with a conventional air regulatory requirement. Cap and trade program basics are discussed at the following EPA Web site:   http://www.epa.gov/airmarkets/cap-trade/index.html. 

RXQ.0.2.x 
Annualized Energy Savings - The savings associated with an energy saving measure, project, or program calculated based on a full year’s installation and operation.

RXQ.0.2.x 
ANSI (American National Standards Institute) - The American National Standards Institute is the national organization that coordinates development and maintenance of consensus standards and sets rules for fairness in their development for the United States’ federated national standards system. The ANSI federation consists of nine hundred companies, large and small, and some two hundred trade, technical, professional, labor, and consumer organizations. ANSI also represents the United States in developing international standards. 

RXQ.0.2.x 
Avoided Costs - In the context of Energy Efficiency, these are the costs that are avoided by the implementation of an Energy Efficiency measure, program, or practice. Such costs are used in benefit cost analyses of Energy Efficiency measures and programs. Because efficiency activity reduces the need for electric generation, these costs include those associated with the cost of electric generation, transmission, distribution, and reliability. Typically, costs associated with avoided energy and capacity are calculated. Other costs avoided by the efficiency activity can also be included, among them the value of avoided emissions not already embedded in the generation cost, impact of the demand reduction on the overall market price for electricity, avoided fuel or water, etc. For natural gas efficiency programs, avoided costs include components of the production, transportation, storage, and service that are variable to the amount of natural gas delivered to customers. 

 RXQ.0.2.x 
Ballast - A device required by electric-discharge light sources such as fluorescent or HID lamps to regulate voltage and current supplied to the lamp during start and throughout operation.
RXQ.0.2.x 
Barrier - Any factor that discourages or limits decisions or actions related to implementation of Energy Efficiency projects or strategies.

RXQ.0.2.x 
Baseline Data - The baseline conditions of the facilities, market segment, generating equipment, or other area of focus of the subject project or program.

RXQ.0.2.x 
BCF (Billion Cubic Feet) - Gas measurement approximately equal to one trillion Btus.

RXQ.0.2.x 
Benchmarking - A process that compares the energy, emissions, and other resource-related conditions of a facility against industry best practices.

RXQ.0.2.x 
Benefits - Energy - See Avoided Cost and Co-Benefits.

RXQ.0.2.x 
Benefits - Non-Energy - See Non-Energy Benefits.

RXQ.0.2.x 
Benefit-Cost Ratio - The mathematical relationship between the benefits and costs associated with the implementation of Energy Efficiency measures, programs, practices, or emissions reductions. The benefits and costs are typically expressed in dollars. Also see Benefit Cost Test and Avoided Cost.

RXQ.0.2.x 
Benefit Cost Test - Also called Cost-Effectiveness Test. The methodology used to compare the benefits of an investment with the costs. Five key benefit-cost tests have, with minor updates, been used for over 20 years as the principal approaches for Energy Efficiency program evaluation. These five cost-effectiveness tests are the participant cost test (PCT), the utility/program administrator cost test (PACT), the ratepayer impact measure test (RIM), the total resource cost test (TRC), and the societal cost test (SCT).

RXQ.0.2.x 
Billing Data - Data obtained from the electric or gas meter that are used to bill the customer for energy used in a particular billing period. In an evaluation context, billing data also refers to the customer billing records over time that are used to conduct analyses of energy use before and after implementation of Energy Efficiency measures.

RXQ.0.2.x 
Breakage - A factor representing the ratio between the number of rebate or mail-in coupons taken by participants who purchase or install an Energy Efficiency measure and the number of such coupons actually redeemed for refund.

RXQ.0.2.x 
British Thermal Unit (Btu) - The standard measure of heat energy. It takes one Btu to raise the temperature of one pound of water one degree Fahrenheit from 58.5 to 59.5 degrees Fahrenheit under standard pressure of 30 inches of mercury at or near its point of maximum density. For example, it takes about 1,000 Btus to make a pot of coffee.

RXQ.0.2.x 
Building Commissioning - Building commissioning, often abbreviated as “Cx,” is a systematic quality assurance process associated with new construction that spans the entire design and construction process, helping ensure that a new building’s performance meets owner expectations. Also see Retro-Commissioning.

RXQ.0.2.x 
Building Energy Simulation Model - Computer models based on physical engineering principals and/or standards used to estimate energy usage and/or savings. These models usually incorporate site-specific data on customers and physical systems such as square footage, weather, surface orientations, elevations, space volumes, construction materials, equipment use, lighting, and building occupancy. Building simulation models can usually account for interactive effects between end uses (e.g. lighting and HVAC), part-load efficiencies, and changes in external and internal heat gains/losses. Examples of building simulation models include DOE-2, EnergyPlus, and Carrier HAP.

RXQ.0.2.x 
Calibration - In economic, planning, or engineering modeling, the process of adjusting the components of the model to reflect reality as best as possible, in order to prepare for the model’s use in future applications. The term also applies to the process whereby metering and measurement equipment is periodically adjusted to maintain industry measurement standards.

RXQ.0.2.x 
California Measurement Advisory Council (CALMAC) - An informal committee made up of representatives of the California utilities, state agencies, and other interested parties. CALMAC provides a forum for the development, implementation, presentation, discussion, and review of regional and statewide market assessment and evaluation studies for California Energy Efficiency programs conducted by member organizations. See www.calmac.org.

RXQ.0.2.x 
California Demand-Side Management Measurement Advisory Council (CADMAC) - CADMAC is the predecessor to CALMAC. It covers market assessment and evaluation on programs conducted under the "Protocols And Procedures For The Verification Of Costs, Benefits, And Shareholder Earnings From Demand-Side Management Programs" (Protocols). Programs evaluated under the Protocols generally were fielded during 1994 through 1997, but evaluations of those programs (and carryover applications) continued to occur through 2007. See www.calmac.org/cadmac.asp.

RXQ.0.2.x 
Capacity - The amount of electric power for which a generating unit, generating station, or other electrical apparatus is rated either by the user or manufacturer. The term is also used for the total volume of natural gas that can flow through a pipeline over a given amount of time, considering such factors as compression and pipeline size. 

RXQ.0.2.x 
Cap & Trade - A market-based policy tool for protecting human health and the environment. A cap and trade program first sets an aggressive cap, or maximum limit, on emissions. Sources covered by the program then receive authorizations to emit in the form of emissions allowances with the total amount of allowances limited by the cap. Each source can design its own compliance strategy to meet the overall reduction requirement including sale or purchase of allowances, installation of pollution controls, implementation of efficiency measures, among other options. Individual control requirements are not specified under a cap and trade program but, each emissions source must surrender allowances equal to its actual emissions in order to comply. Sources must also completely and accurately measure and report all emissions in a timely manner to guarantee that the overall cap is achieved. A well-designed program provides:


strict limits on emissions yielding dramatic pollution reductions; high levels of compliance, transparency, and complete accountability;  regulatory certainty and flexibility for sources;  incentives for early pollution reduction and innovations in control technologies; compatibility with state and local programs; significant, widespread, and guaranteed human health and environmental benefits; and efficient use of government resources.

RXQ.0.2.x 
Capacity Factor - A percentage that indicates how much of a power plant’s capacity is used over a twelve month period. The term is also used for the total volume of natural gas that can flow through a pipeline over a given amount of time, considering such factors as compression and pipeline size.  

 RXQ.0.2.x 
Central Air Conditioner (CAC) - An air conditioning system that provides cooling to an entire building through the use of a central cooling system that delivers the cooling to rooms through ducts.

RXQ.0.2.x 
Citygate - A location at which custody of natural gas passes from a gas pipeline company to a local distribution company.

RXQ.0.2.x 
Coefficient of Variation (CV) - The mean (average) of a sample, divided by its standard error.  
RXQ.0.2.x 
Coincident Demand - The demand of a device, circuit, or building that occurs at the same time as the peak demand of a utility’s system load or at the same time as some other peak of interest, such as building or facility peak demand. The peak of interest should be specified (e.g. “demand coincident with the utility system peak”). 

RXQ.0.2.x 
Coincidence Factor - The ratio, expressed as a numerical value or as a percentage, of the simultaneous maximum demand within a specified period of a group of electrical appliances or consumers within a specified period, to the sum of their individual maximum demands within the same period.

RXQ.0.2.x 
Commissioning - A process for achieving, verifying and documenting the performance of equipment to meet the operational needs of a facility within the capabilities of the design, and to meet the design documentation and the owner’s functional criteria, including preparation of operator personnel.  Also see Building Commissioning.

RXQ.0.2.x 
Comparison group - Also called Control Group.  A selected group of individuals or organizations that have not had the opportunity to receive program benefits and that has been selected because its characteristics match those of another group of individuals or organizations that have had the opportunity to receive program benefits. The characteristics used to match the two groups should be associated with the action or behavior that the evaluation is trying to examine. The comparison group is used to isolate program effects from other factors that affect energy use. 

RXQ.0.2.x 
Computer Simulation of System Performance - The use of computer models to predict the energy use of systems. (e.g. DOE-2 for buildings). These models can be calibrated with actual performance data. 

RXQ.0.2.x 
Control Group - See Comparison Group.

RXQ.0.2.x 
Cooling Degree Days - The cumulative number of degrees in a month or year by which the mean temperature is above 18.3°C/65°F.  Also see Degree Days.

RXQ.0.2.x 
Cooling Load - The rate at which heat must be extracted from a space in order to maintain the desired temperature within the space.  Measured in tons, the Cooling Load is the amount of heat removed by an air conditioning system that would melt 1 ton of ice in 24 hours. 1 refrigeration ton = 12,000 Btu/hr.

RXQ.0.2.x 
Correlation - for a set of observations, such as for participants in an Energy Efficiency program, the extent to which high values for one variable are associated with high values of another variable for the same participant.  For example, facility size and energy consumption usually have a high positive correlation.

RXQ.0.2.x 
Cost-Benefit and Cost-Effectiveness Analysis - Analysis that compares the benefits associated with a program or measure’s outputs or outcomes with the costs (resources expended) to produce them. Cost-benefit analysis is typically conducted to determine the relationship of the program’s benefits and costs, as a ratio, once the decision has been made to implement or design the program; programs with benefit-cost ratios greater than 1.0 provide overall ratepayer benefits. Cost-effectiveness analysis is generally undertaken to compare one program or program approach to other approaches, or options for the use of funds, to determine the relationship among the options. The terms are often interchanged in evaluation discussions. 

RXQ.0.2.x 
Cost-Effectiveness - An indicator of the relative performance or economic attractiveness of any Energy Efficiency investment or practice. In the Energy Efficiency field, the present value of the estimated benefits produced by an Energy Efficiency program is compared to the estimated total costs to determine if the proposed investment or measure is desirable from a variety of perspectives (e.g. whether the estimated benefits exceed the estimated costs from a societal perspective). 

RXQ.0.2.x 
Cost-Effectiveness Test - See Benefit-Cost Test.

RXQ.0.2.x 
Cross-Sectional Data - Observations collected on subjects or events during a single period of time. 

RXQ.0.2.x 
Cubic Feet per Minute (CFM) - This measurement indicates how many cubic feet of air pass by a stationary point in one minute. 

RXQ.0.2.x 
Cubic Foot - The most common unit of measurement of natural gas volume. It equals the amount of gas required to fill a volume of one cubic foot under stated conditions of temperature, pressure and water vapor. One cubic foot of natural gas has an energy content of approximately 1,000 s. One hundred (100) cubic feet equals one therm (100 ft3 = 1 therm). 

RXQ.0.2.x 
Cumulative Energy Savings - The summation of energy savings from multiple projects or programs over a specified period of time, incorporating the multi-year energy savings that each project or program produces.

RXQ.0.2.x 
Custom Program - An Energy Efficiency program intended to provide efficiency solutions to unique situations not amenable to common or prescriptive solutions. Each custom project is examined for its individual characteristics, savings opportunities, efficiency solutions, and often, customer incentives.

See Retail Customer under Business Definitions

RXQ.0.2.x
Database for Energy-Efficient Resources (DEER) - A California database designed to provide well-documented estimates of energy and peak demand savings values, measure costs, and effective useful life.  See www.deeresources.com.

RXQ.0.2.x 
Defensibility - The ability of evaluation results to stand up to scientific scrutiny.  Defensibility is based on assessments by experts of the evaluation’s validity, reliability, and accuracy.  

RXQ.0.2.x 
Degree Days - For any individual day, degree days indicate how far that day's average temperature departed from 65°F. Heating Degree Days measure heating energy demand. It is a measure to indicate how far the average temperature fell below 65°F. Similarly, Cooling Degree Days, which measure cooling energy demand, indicate how far the temperature averaged above 65°F. In both cases, smaller values represent less fuel demand, but values below 0 are set equal to 0, because energy demand cannot be negative. Furthermore, since energy demand is cumulative, degree day totals for periods exceeding 1 day are simply the sum of each individual day's degree day total. For example, if a location has a mean temperature of 60°F on day 1 and 80°F on day 2, there would be 5 HDD's for day 1 (65 minus 60) and 0 for day 2 (65 minus 80, set to 0). For the day 1 + day 2 period, the HDD total would be 5 + 0 = 5. In contrast, there would be 0 CDD's for day 1 (60 minus 65, reset to 0), 15 CDD's for day 2 (80 minus 65), resulting in a 2-day CDD total of 0 + 15 = 15. 

RXQ.0.2.x 
Delta Watts - The difference in the wattage between existing or baseline equipment and its more efficient replacement or installation at a specific time, expressed in watts or kilowatts.

RXQ.0.2.x 
Dependent Variable - Term used in regression analysis or other analyses seeking to explain the relationship among variables to quantify the variable that is being explained by the other (independent) variables.

RXQ.0.2.x 
Direct Emissions - Emissions from sources within an entity’s organizational boundaries that are owned or controlled by the entity, including stationary combustion emissions, mobile combustion emissions, process emissions, and fugitive emissions. Direct emissions are the source of avoided emissions for thermal Energy Efficiency measures (e.g. avoided emissions from burning natural gas in a water heater).

RXQ.0.2.x 
Diversity - That characteristic of a variety of electric loads whereby individual maximum demands usually occur at different times.
RXQ.0.2.x 
Diversity Factor - 1) The percent of maximum demand savings from Energy Efficiency measures available at the time of the company’s peak demand; 2) the ratio of the sum of the demands of a group of users to their coincident maximum demand; 3) the percent of time available that a machine, piece of equipment, or facility has its maximum or nominal load or demand.
RXQ.0.2.x 
Economic Potential - The amount of savings opportunity that can be acquired cost-effectively. 
Also see Achievable Potential, Technical Potential, and Potential Studies

RXQ.0.2.x 
Effective useful life (EUL) - An estimate of the median number of years that efficiency measures installed under a program are still in place and operable.

RXQ.0.2.x 
Efficacy, Lighting - The ratio of light from a lamp to the electrical power consumed, including ballast losses, expressed as lumens per watt.

RXQ.0.2.x 
End-Use - General categories of Energy Efficiency measures, usually including lighting, HVAC, motors, and refrigeration. 

RXQ.0.2.x 
End-Use Metering - The direct measuring of energy consumption or demand by specific end-use equipment, typically as part of load research studies or to measure the impacts of DSM programs.

RXQ.0.2.x 
Energy Adjustment Factor - Applied to gross gas and electric savings, a factor made up of one or more evaluation impact parameters applied to gross savings in the calculation of net savings.
RXQ.0.2.x 
Energy Audit - A review of a customer's energy usage, often including recommendations to alter the customer's demand or reduce energy usage. An audit typically involves a visit to the customer's facility.

RXQ.0.2.x 
Energy Conservation - Term used to reflect doing with less of a service in order to save energy. The term is often unintentionally used instead of Energy Efficiency.

RXQ.0.2.x 
Energy Efficiency Ratio (EER) - The ratio of cooling capacity of an air conditioning unit in Btus per hour to the total electrical input in watts under specified test conditions.

RXQ.0.2.x 
Energy Management System (EMS) - A control system (often computerized) designed to regulate the energy consumption of a building by controlling the operation of energy consuming systems, such as the heating, ventilation and air conditioning (HVAC), lighting, and water heating systems.

RXQ.0.2.x 
Energy Performance Contract - A contract between two or more parties where payment is based on achieving specified results, which are typically guaranteed reductions in energy consumption and/or operating costs. Payments are often based on the cost savings associated with the anticipated results.

RXQ.0.2.x 
Energy Star® - A joint program of the U.S. Environmental Protection Agency and the U.S. Department of Energy designed to reduce energy use and the impact on the environment. The Energy Star label is awarded to products that meet applicable Energy Efficiency guidelines and to homes and commercial buildings that meet specified Energy Efficiency standards. The program provides a range of energy management tools, primarily computer-based, for businesses.

RXQ.0.2.x 
Engineering Methods - The use of standard formulas or models based on those formulas, typically accepted by ASHRAE, as the basis for calculating energy use.

RXQ.0.2.x 
Engineering Model - Engineering equations used to calculate energy usage and savings. These models are usually based on a quantitative description of physical processes that transform delivered energy into useful work such as heat, lighting, or motor drive. In practice, these models may be reduced to simple equations in spreadsheets that calculate energy usage or savings as a function of measurable attributes of customers, facilities, or equipment (e.g. lighting use = watts × hours of use).

RXQ.0.2.x 
Equipment Life - The number of years that a measure is installed and operates until failure. 
RXQ.0.2.x 
Ex Ante Savings Estimate - Forecasted savings used for program and portfolio planning purposes.

RXQ.0.2.x 
Ex Post Savings Estimate - Savings estimate reported by an evaluator after the energy impact evaluation has been completed.

RXQ.0.2.x 
Footcandle - A unit of illuminance on a surface that is one foot from a uniform point source of light of one candle and is equal to one lumen per square foot.

RXQ.0.2.x 
Free Driver - A program non-participant who has adopted a particular efficiency measure or practice as a result of the evaluated program. Also see Spillover.

RXQ.0.2.x 
Free Ridership Rate - The percent of savings attributable to free riders.

RXQ.0.2.x 
Gross kW - Expected demand reduction based on a comparison of standard or replaced equipment, and equipment installed through an Energy Efficiency program.

RXQ.0.2.x 
Gross kWh - Expected kWh reduction based on a comparison of standard or replaced equipment, and equipment installed through an Energy Efficiency program.

RXQ.0.2.x 
Heating Degree Days - The cumulative number of degrees in a month or year by which the mean temperature falls below 18.3°C/65°F. Also see Degree Days.

RXQ.0.2.x 
Heating Seasonal Performance Factor (HSPF) - A measure of heat pump’s Energy Efficiency over one heating season. It represents the total heating output of a heat pump (including supplementary electric heat) during the normal heating season (in Btus) compared to the total electricity consumed (in watt-hours) during the same period. The higher the rating, the more efficient the heat pump. 

RXQ.0.2.x 
Heat Pump - A heating and cooling unit that draws heat from an outdoor source and transports it to an indoor space for heating purposes; or inversely, for cooling purposes. There are various types of heat pumps defined by the content or location of the heat transfer material - air source, water source, and ground source.

RXQ.0.2.x 
High Electric Demand Days (HEDD) - Days of high electricity demand, which can dramatically increase ozone-forming air pollution from electric generation and often result in nitrogen oxide (NOx) emissions that can be greater than two times their average levels. Days of high electrical use often coincide with days with high ozone levels.

RXQ.0.2.x 
Home Energy Rating System (HERS) - Associated with Energy Star, HERS is an indexing system used in residential new construction to rate the pre- and post- construction of new homes to highlight and indicate the degree of Energy Efficiency embedded in the construction. The HERS Index is a scoring system established by the Residential Energy Services Network (RESNET) in which a home built to the specifications of the HERS Reference Home (based on the 2006 International Energy Conservation Code) scores a HERS Index of 100, while a net zero energy home scores a HERS Index of 0. The lower a home’s HERS Index, the more energy efficient it is in comparison to the HERS Reference Home. Each 1-point decrease in the HERS Index corresponds to a 1% reduction in energy consumption compared to the HERS Reference Home.

RXQ.0.2.x 
Horsepower (hp) - A unit for measuring the rate of doing work. One horsepower

equals about three-fourths of a kilowatt (745.7 watts).

RXQ.0.2.x 
Incremental Cost - The difference between the cost of existing or baseline equipment or service and the cost of alternative energy efficient equipment or service.

RXQ.0.2.x 
Incremental Energy Savings - The difference between the amount of energy savings acquired in a project or a program in one period and the amount of energy savings acquired by that project or program in a prior period.

RXQ.0.2.x 
Independent variables - The explanatory factors in a regression model that are assumed to affect the variable under study (e.g. energy use).

RXQ.0.2.x 
Indirect Emissions - Emissions that are a consequence of activities that take place within the organizational boundaries of an entity, but that occur at sources owned or controlled by another entity. For example, emissions of electricity used by a manufacturing entity that occur at a power plant represent the manufacturer’s indirect emissions. Indirect emissions are the source of avoided emissions for electric Energy Efficiency measures.

RXQ.0.2.x 
Inspections - Site visits to facilities treated under an efficiency program that document the existence, characteristics, and operation of baseline or reporting period equipment and systems as well as factors that affect energy use.

RXQ.0.2.x 
Interactive Effects - The influence in energy use between one technology application and the energy required to operate another application. An example is the reduced heat in a facility as a result of replacing incandescent lights with CFLs, and the resulting need to increase space heating from another source, usually oil or gas fired.

RXQ.0.2.x 
Kilowatt (kW) - A measure of the rate of power used during a preset time period (e.g. minutes, hours, days or months) equal to 1,000 watts. In the abbreviation, the W is capitalized because the unit was named to honor one of Scotland’s great inventors, James Watt, who coined the term “horsepower”. 

RXQ.0.2.x 
Kilowatt-Hour (kWh) - A common unit of electric energy; one kilowatt-hour is numerically equal to 1,000 watts used for one hour.

RXQ.0.2.x 
Latent Cooling Load - The load created by moisture in the air, including from outside air infiltration and that from indoor sources such as occupants, plants, cooking, and showering. 

RXQ.0.2.x 
Leakage - In the context of a cap-and-trade program (e.g. RGGI), the concept that there could be a shift of electricity generation from sources subject to the cap-and-trade program to higher emitting sources not subject to the program that results in a net increase in carbon dioxide (CO2) emissions. More broadly, in the context of an inventory of emissions sources, the situation that occurs as a result of an emissions reduction by one source, the emission is accounted for in the inventory, which leads to an increase in emissions from another source, but no equivalent accounting in the inventory for the increasing source.

RXQ.0.2.x 
Lifetime kW - The expected demand savings over the lifetime of an installed measure, calculated by multiplying the annual peak kW reduction associated with a measure by the expected lifetime of that measure. It is expressed in units of kW-years.

RXQ.0.2.x 
Lifetime MWh - The expected electrical energy savings over the lifetime of an installed measure, calculated by multiplying the annual MWh reduction associated with a measure by the expected lifetime of that measure.

RXQ.0.2.x 
Lifetime Therms - The expected gas energy savings over the lifetime of an installed measure, calculated by multiplying the annual reduction in therms associated with a measure by the expected lifetime of that measure.

RXQ.0.2.x 
Light Power Density (LPD) - Sometimes referred to as power density. A measurement of the ratio of light output in an area and the electric power used to produce that light. LPD is determined by dividing the total light output by the total wattage consumed and is measured in lumens per watt. 

RXQ.0.2.x 
Load Factor - A percentage indicating the difference between the amount of electricity or natural gas a consumer used during a given time span and the amount that would have been used if the usage had stayed at the consumer’s highest demand level during the whole time. The term also is used to mean the percentage of capacity of an energy facility, such as a power plant or gas pipeline that is utilized in a given period of time.

RXQ.0.2.x 
Load Impact Regression Model (LIRM) - A statistical model that produces estimates of the load impacts of energy conservation programs. Depending on the particular approach and the statistical issues encountered, it may involve more than one regression model and technique: (1) the load impact estimation model typically is a linear or non-linear regression model that uses billing data to estimate gross and/or net load impacts. Data from program non-participants, in addition to participant data, can be used to derive net impacts directly or to affect other statistical control. (2) The participant/decision model typically is a discrete choice model used in conjunction with the load impact estimation model to isolate free ridership effects, generate self-selection correction terms, and/or net-to-gross ratios as needed. When this model is used to estimate a net-to-gross ratio, the resulting estimate is multiplied by an estimate of gross load impact to yield an estimate of net load impact.

RXQ.0.2.x 
Load Management - Steps taken to reduce power demand at peak load times or to shift some of it to off-peak times. Load management may coincide with peak hours, peak days or peak seasons. Load management may be pursued by persuading consumers to modify behavior or by using equipment that regulates some electric consumption. This may lead to complete elimination of electric use during the period of interest (load shedding) and/or to an increase in electric demand in the off-peak hours as a result of shifting electric usage to that period (load shifting).

RXQ.0.2.x 
Load Shapes - Representations such as graphs, tables, and databases that show the time-of-use pattern of customer or equipment energy use. These are typically shown over a 24 hour or whole year (8760 hours) period.

RXQ.0.2.x 
Logic Model - The graphical representation of a program theory showing the connection among activities, their outputs, and subsequent short-term, intermediate, and long-term outcomes. Often the logic model is displayed with these elements in boxes and the causal flow is shown by arrows from one to the others in the program logic. It can also be displayed as a table with the linear relationship presented by the rows in the table.

RXQ.0.2.x 
Lost Opportunity Program - A program that captures Energy Efficiency opportunities at the time of a naturally-occurring market event, such as when a customer constructs, expands, renovates, or remodels a home or a building or makes an initial purchase of equipment, or replaces failed equipment.

RXQ.0.2.x 
Lumen - A measure of the amount of light available from a light source equivalent to the light emitted by one candle.

RXQ.0.2.x 
Lumens/Watt - A measure of the efficacy of a light fixture; the number of lumens output per watt of power consumed.

RXQ.0.2.x 
Luminaire - A complete lighting unit consisting of a lamp or lamps together with the parts designed to distribute the light, to position and protect the lamps and to connect the lamps to the power supply.

RXQ.0.2.x 
Market Assessment - An analysis that provides an assessment of how and how well a specific market or market segment is functioning with respect to the definition of well-functioning markets or with respect to other specific policy objectives. Generally includes a characterization or description of the specific market or market segments, including a description of the types and number of buyers and sellers in the market, the key actors that influence the market, the type and number of transactions that occur on an annual basis, and the extent to which market participants consider Energy Efficiency as an important part of these transactions. This analysis may also include an assessment of whether a market has been sufficiently transformed to justify a reduction or elimination of specific program interventions. Market assessment can be blended with strategic planning analysis to produce recommended program designs or budgets. One particular kind of market assessment effort is a baseline study, or the characterization of a market before the commencement of a specific intervention in the market, for the purpose of guiding the intervention and/or assessing its effectiveness later.

RXQ.0.2.x 
Market effect evaluation - An evaluation of the change in the structure or functioning of a market, or the behavior of participants in a market, that results from one or more program efforts. Typically the resultant market or behavior change leads to an increase in the adoption of energy-efficient products, services, or practices.

RXQ.0.2.x 
Market penetration rate - A measure of the diffusion of a technology, product, or practice in a defined market, as represented by the percentage of annual sales for a product or practice, or as a percentage of the existing installed stock for a product or category of products, or as the percentage of existing installed stock that uses a practice.

RXQ.0.2.x 
Market Saturation - A percentage indicating the proportion of a specified end-user market that contains a particular product. An example would be the percentage of all households in a given geographical area that have a certain appliance. Studies conducted to obtain this information within the residential sector are referred to as residential appliance saturation studies (RASS).

RXQ.0.2.x 
Market Theory - A theoretical description of how a market operates relative to a specific program or set of programs designed to influence that market. Market theories typically include the identification of key market actors, information flows, and product flows through the market, relative to a program designed to change the way the market operates. Market theories are typically grounded upon the information provided from a market assessment but can also be based on other information. Market theories often describe how a program intervention can take advantage of the structure and function of a market to transform the market. Market theories can also describe the key barriers and benefits associated with a market and describe how a program can exploit the benefits and overcome the barriers.

RXQ.0.2.x 
Market Transformation Program - An energy program strategy that leads to a reduction in market barriers resulting from a market intervention, as evidenced by market effects that last after the intervention has been withdrawn, reduced, or changed.

RXQ.0.2.x 
Mcf - The quantity of natural gas occupying a volume of one thousand cubic feet at a temperature of sixty degrees Fahrenheit and at a pressure of fourteen and seventy-three hundredths pounds per square inch absolute. One Mcf has an energy value of one million Btus. 

RXQ.0.2.x 
Measure life - The life of an energy consuming measure, including its equipment life and measure persistence (not savings persistence). 

RXQ.0.2.x 
Measurement Error - In the evaluation context, a reflection of the extent to which the observations conducted in the study deviate from the true value of the variable being observed. The error can be random (equal around the mean) or systematic (indicating bias).

RXQ.0.2.x 
Measure persistence - The duration of an energy consuming measure, taking into account business turnover, early retirement of installed equipment, and other reasons measures might be removed or discontinued.

RXQ.0.2.x 
Measure Retention Study - An assessment of (a) the length of time the measure(s) installed during the program year are maintained in operating condition; and (b) the extent to which there has been a significant reduction in the effectiveness of the measure(s).

RXQ.0.2.x 
Megawatt (MW) - A unit for measuring electricity equal to 1,000 kilowatts or one million watts. Utility companies, power generating plants and very large users of electricity are the primary users of the term.

RXQ.0.2.x 
Megawatt-Hour (MWh) - A unit of electric energy; a Megawatt-hour is numerically equal to 1,000,000 watts used for one hour.

RXQ.0.2.x 
Metered Data - Data collected over time through a meter for a specific end use, energy-using system (e.g. lighting and HVAC), or location (e.g. floors of a building or a whole premise). Metered data may be collected over a variety of time intervals. Usually refers to electricity or gas data.

RXQ.0.2.x 
Metering - The collection of energy consumption data over time through the use of meters. These meters may collect information about an end-use, a circuit, a piece of equipment, or a whole building (or facility). Short-term metering generally refers to data collection for no more than a few weeks. End-use metering refers specifically to separate data collection for one or more end-uses in a facility, such as lighting, air conditioning or refrigeration. Spot metering is an instantaneous measurement (rather than over time) to determine equipment size or power draw.

RXQ.0.2.x 
MMBtu - A thermal unit of energy equal to 1,000,000 Btus, the equivalent of 1,000 cubic feet of gas having a heating content of 1,000 Btus per cubic foot.

RXQ.0.2.x 
Monitoring - The collection of relevant measurement data over time at a facility, including but not limited to energy consumption or emissions data (e.g. energy and water consumption, temperature, humidity, volume of emissions, hours of operation, etc.),  for the purpose of savings analysis or to evaluate equipment or system performance.

RXQ.0.2.x 
Naturally Occurring Efficiency - The effects of energy-related decisions that would have been made in the absence of the program administrator programs by both program participants and non-participants.

RXQ.0.2.x 
Non-Energy Effects or Non-Energy Benefits (NEB) - Also referred to as Non-Energy Impacts (NEI). The identifiable and sometimes quantifiable non-energy results associated with program implementation or participation. Some examples of NEBs include: reduced emissions and environmental benefits, productivity improvements, jobs created, reduced program administrator debt and disconnects, and higher comfort and convenience level of participant. The effects of an Energy Efficiency or resource acquisition program that are other than energy saved. The value is most often positive, but may also be negative (e.g. the cost of additional heating required to replace the residual heat no longer available from incandescent lamps that have been replaced by CFLs).

RXQ.0.2.x 
Normalized Annual Consumption (NAC) Analysis - A regression-based method that analyzes monthly energy consumption data and adjusts the consumption data to eliminate annual or other periodic fluctuations in an influencing factor (such as weather on heating and cooling needs) based on a historical normal or average pattern of the influencing factor. 

RXQ.0.2.x 
Off-Peak Energy kWh Savings - The kWh reduction that occurs during a specified period of off‑peak hours for energy savings. (e.g. Monday-Friday, 9 p.m. to 8 a.m. and all day on weekends and holidays).

RXQ.0.2.x 
Offset - Program mechanism that allows an entity to neutralize the amount of its greenhouse gas contribution by orchestrating or funding projects offsite that should cause an equal reduction of emissions.

RXQ.0.2.x 
On-Peak Energy kWh Savings - The kWh reduction that occurs during a specified period of on‑peak hours for energy savings. (e.g. Monday-Friday, 8 a.m. to 9 p.m. and except holidays).

RXQ.0.2.x 
Other Demand Resources (ODR) - Term used by ISO-New England in its Market Rules to mean installations undertaken as part of merchant, utility, or state sponsored program, and may include Energy Efficiency, load management, and distributed generation projects that are installed after June 16, 2006, and that result in additional and verifiable reductions in end-use customer demand on the electricity network in the New England Control Area during ODR Performance Hours (which may include Critical Peak Hours), as described in Section III.8.3.6.2 of Market Rule 1.

RXQ.0.2.x 
Participant Cost Test (PCT) - A cost-effectiveness test that measures the economic impact to the participating customer of adopting an Energy Efficiency measure.

RXQ.0.2.x 
Peak Load - The highest electrical demand within a particular period of time. Daily electric peaks on weekdays typically occur in late afternoon and early evening. Annual peaks typically occur on hot summer days.

RXQ.0.2.x 
Performance Contracts - See Energy Performance Contracts. 

RXQ.0.2.x 
Potential Studies - Studies conducted to assess market baselines and future savings that may be expected for different technologies and customer markets over a specified time horizon. Potential is typically defined in terms of 1) technical potential - savings estimate based solely on currently and anticipated available technology; 2) achievable potential - savings estimate based on market forces, codes and standards, equipment efficiency, and Energy Efficiency programs; and 3) economic potential - estimate of savings limited by only those found to be cost-effective.

RXQ.0.2.x 
Practice Retention Study - An assessment of the length of time a customer continues the Energy Efficiency or conservation behavioral changes after adoption of these changes.

RXQ.0.2.x 
Prescriptive Program - An Energy Efficiency program focused on measures that are one-for-one replacements of the existing equipment and for which fixed customer incentives can be developed based on the anticipated similar savings that will accrue from their installation.

RXQ.0.2.x 
Primary Effects - Effects that the project or program are intended to achieve. For efficiency programs, this is predominantly a reduction in energy use per unit of output.

RXQ.0.2.x 
Program Administrator Cost Test (PACT) - See Utility/Program Administrator Cost Test
RXQ.0.2.x 
Program Incentive - An incentive, generally monetary, that is offered to a customer through an Energy Efficiency program to encourage the customer to participate in the program. The incentive is intended to overcome one or more barriers that keep the customer from taking the Energy Efficiency activity on his own.

RXQ.0.2.x 
Program Manager - The individual who manages an Energy Efficiency program as it is implemented in the field.

RXQ.0.2.x 
Program Theory - A presentation of the goals of a program, incorporated with a detailed presentation of the activities that the program will use to accomplish those goals and the identification of the causal relationships between the activities and the program’s effects. 

RXQ.0.2.x 
Proxy Variable - In program Evaluation or project modeling, a proxy variable is used to estimate energy savings, and is intended to represent a variable that is more directly related to the energy savings activity but that cannot itself be directly measured. 

RXQ.0.2.x 
Qualitative Data - Information expressed in the form of words.

RXQ.0.2.x 
Quantitative Data - Information expressed in the form of numbers. 

RXQ.0.2.x 
R-Value - A measure of thermal resistance of a material, equal to the reciprocal of the U-Value. The R-Value is expressed in terms of degrees Fahrenheit multiplied by hours, multiplied by square feet per Btu.

RXQ.0.2.x 
Ratepayer Impact Measure Test (RIM) - A cost-effectiveness test that measures the impact on utility operating margin and whether rates would have to increase to maintain the current levels of margin if a customer installed energy efficient measures. The Ratepayer Impact Measure (RIM) test measures what happens to customer bills or rates due to changes in utility revenues and operating costs caused by the program.

RXQ.0.2.x 
Realization Rate - The term is used in several contexts in the development of reported program savings. The primary applications include the ratio of project tracking system savings data (e.g. initial estimates of project savings) to savings 1) adjusted for data errors, 2) that incorporate evaluated or verified results of the tracked savings, and 3) that account for free ridership and/or spillover.  

RXQ.0.2.x 
Rebate Program - An Energy Efficiency program in which the program administrator offers a financial incentive for the installation of energy-efficient equipment.

RXQ.0.2.x 
Rebound Effect - Also called Snap Back.   A change in energy-using behavior that yields an increased level of service that is accompanied by an increase in energy use and occurs as a result of taking an Energy Efficiency action. The result of this effect is that the savings associated with the direct Energy Efficiency action is reduced by the resulting behavioral change. 

RXQ.0.2.x 
Re-commissioning (ReCx) - The process of commissioning a building several years after it has been commissioned to help keep it operating optimally.

RXQ.0.2.x 
Regression Analysis - Analysis of the relationship between a dependent variable (response variable) to specified independent variables (explanatory variables). The mathematical model of their relationship is the regression equation.

RXQ.0.2.x 
Regression Model - A mathematical model based on statistical analysis where the dependent variable is quantified based on its relationship to the independent variables which are said to determine its value. In so doing, the relationship between the variables is estimated statistically from the data used.

RXQ.0.2.x 
Reliability - The quality of a measurement process that would produce similar results on: (1) repeated observations of the same condition or event; or (2) multiple observations of the same condition or event by different observers.
RXQ.0.2.x 
Renewable Energy - Energy derived from resources that are naturally replenishing but flow-limited. They are virtually inexhaustible in duration but limited in the amount of energy that is available per unit of time. Renewable energy resources include: biomass, hydro, geothermal, solar, wind, ocean thermal, wave action, and tidal action.

RXQ.0.2.x 
Reporting Period - The time following implementation of an Energy Efficiency activity during which results are to be determined.

RXQ.0.2.x 
Representative Sample - A sample that has approximately the same distribution of characteristics as the population from which it was drawn.

RXQ.0.2.x 
Resource Acquisition Program - A program designed to achieve directly energy and or demand savings. Such a program generally involves encouraging customers to replace existing equipment with more efficient equipment. Also known as a retrofit program.

RXQ.0.2.x 
Retro-commissioning - The application of the commissioning process to existing buildings. Retro-commissioning is a process that seeks to improve how building equipment and systems function together. Depending on the age of the building, retro-commissioning can often resolve problems that occurred during design or construction, or address problems that have developed throughout the building’s life. In all, retro-commissioning improves a building’s operations and maintenance (O&M) procedures to enhance overall building performance. 

RXQ.0.2.x 
Retrofit Program - An Energy Efficiency program that provides incentives, information and technical support to customers in an effort to encourage the replacement of existing and operating equipment with more efficient equipment that provides the same function. 

RXQ.0.2.x 
Rigor - The level of effort expended to minimize uncertainty due to factors such as sampling error and bias. The higher the level of rigor, the more confident one is that the results of the Evaluation are both accurate and precise.

RXQ.0.2.x 
Sample - In program Evaluation, a portion of the population selected to represent the whole. Differing Evaluation approaches rely on simple or stratified (based on some characteristic of the population) samples.

RXQ.0.2.x 
Sample Design - The approach used to select the sample units. 

RXQ.0.2.x 
Sampling Error - The error in estimating a parameter caused by the fact that in the sample at hand all the disturbances are not zero. 

RXQ.0.2.x 
Savings Persistence Rate - Percentage of first year energy or demand savings expected to persist over the life of the installed Energy Efficiency equipment; developed by conducting surveys of installed equipment several years after installation to determine presence and operational capability of the equipment. 
RXQ.0.2.x 
Seasonal Energy Efficiency Ratio (SEER) - The total cooling output of a central air conditioning unit in Btus during its normal usage period for cooling divided by the total electrical energy input in watt-hours during the same period, as determined using specified federal test procedures.

RXQ.0.2.x 
Seasonal Performance Factor (SPF) - Ratio of useful energy output of a device to the energy input, averaged over an entire heating season.

RXQ.0.2.x 
Sensible Cooling Load - The interior heat gain due to heat conduction, convection, and radiation from the exterior into the interior, and from occupants and appliances. 

RXQ.0.2.x 
SIC Code (Standard Industrial Classification code) - Four digit numerical codes assigned by the U.S. government to business establishments to identify the primary business of the establishment.

RXQ.0.2.x 
Simple Random Sample - A method for drawing a sample from a population such that all samples of a given size have equal probability of being drawn.

RXQ.0.2.x 
Smart Meter - See Advanced Meter.

RXQ.0.2.x 
Snap Back - See Rebound Effect.
RXQ.0.2.x 
Simulation Model - An assembly of algorithms that calculates energy use based on engineering equations and user-defined parameters.

RXQ.0.2.x 
Societal Cost Test (SCT) - A cost-effectiveness test that measures the net economic benefit to the utility service territory, state, or region, as measured by the total resource cost test, plus indirect benefits such as environmental benefits.

RXQ.0.2.x 
Spillover rate - Estimate of energy savings attributable to spillover effects expressed as a percent of savings installed by participants through an Energy Efficiency program.
RXQ.0.2.x 
Split System - An HVAC system in which some components are located inside the structure of the house and some are located outside. 

RXQ.0.2.x 
Standard error - a measure of the variability in a data sample, how far a “typical” data point is from the mean of a sample.  In a large sample, about 2/3 of observations lie within one standard error of the mean, and 95 percent of observations lie within two standard errors.

RXQ.0.2.x 
Statistically Adjusted Engineering (SAE) models - A category of statistical analysis models that incorporates the engineering estimate of savings as a dependent variable. The regression coefficient in these models is the percentage of the engineering estimate of savings observed in changes in energy usage. For example, if the coefficient on the SAE term is 0.8, this means that the customers are on average realizing 80% of the savings from their engineering estimates.

RXQ.0.2.x 
Stipulated values- See Deemed Savings. 

RXQ.0.2.x 
Stratified Random Sampling - The population is divided into X units of subpopulations, called strata, that are non-overlapping and together comprise the entire population. A simple random sample is taken of each strata to create a sample based upon stratified random sampling.

RXQ.0.2.x 
Stratified Ratio Estimation - A sampling method that combines a stratified sample design with a ratio estimator to reduce the coefficient of variation by using the correlation of a known measure for the unit (e.g. expected energy savings) to stratify the population and allocate sample from strata for optimal sampling.

RXQ.0.2.x 
Structured Interview - An interview in which the questions to be asked, their sequence, and the detailed information to be gathered are all predetermined. These are used where maximum consistency across interviews and interviewees is needed.

RXQ.0.2.x 
Sustainability - The ability to meet the needs of the present without compromising the ability of future generations to meet their needs. In the context of Energy Efficiency, sustainability refers to the likelihood that observed program-induced market changes would continue in the absence of the program.

RXQ.0.2.x 
Takeback effect - See Rebound Effect.

RXQ.0.2.x 
Technical Potential - An estimate of energy savings based on the assumption that all existing equipment or measures will be replaced with the most efficient equipment or measure that is technically feasible over a defined time horizon, without regard to cost or market acceptance.

RXQ.0.2.x 
Therm - One hundred thousand (100,000) British thermal units (1 therm = 100,000 Btus).

RXQ.0.2.x 
Time Series Analysis - An analysis of an ordered sequence of values of a variable at equally spaced time intervals to obtain an understanding of the underlying forces and structure that produced the observed data.

RXQ.0.2.x 
Ton - Unit of measure for determining cooling capacity. One ton equals 12,000 Btus heat removed per hour.

RXQ.0.2.x 
Total Resource Cost Test (TRC) - A cost-effectiveness test that measures the net direct economic impact to the utility service territory, state, or region

RXQ.0.2.x 
U-Value - The quantity of heat transmitted per hour through one square foot of a building section (wall, roof, window, etc.) for each degree Fahrenheit of temperature difference between the air on the warm side and the air on the cold side of the building section.

RXQ.0.2.x 
Upstream Program - A program that provides information and/or financial assistance to entities in the delivery chain of high-efficiency products at the retail, wholesale, or manufacturing level. Such a program is intended to yield lower retail prices for the products.

RXQ.0.2.x 
Utility/Program Administrator Cost Test - Also called Program Administrator Cost Test (PACT) and also known as the utility cost test.  A cost-effectiveness test that measures the change in the amount the utility must collect from the customers every year to meet an earnings target—e.g. a change in revenue requirement. In a number of states, this test is referred to as the program administrator cost test. In those cases, the definition of the “utility” is expanded to program administrators (utility or third party).

RXQ.0.2.x 
Watt - A unit of measure of electric power at a point in time, as capacity or demand. One watt of power maintained over time is equal to one joule per second. The watt is named after Scottish inventor James Watt and is capitalized when shortened to W and used with other abbreviations, as in kWh.

RXQ.0.2.x 
Watt-Hour - One watt of power expended for one hour. One thousandth of a kilowatt-hour.

RXQ.0.2.x 
Wet-Bulb Temperature - The temperature at which water, by evaporating into air, can bring the air to saturation at the same temperature. Wet-bulb temperature is measured by a wet-bulb psychrometer.

RXQ.0.2.x 
Whole-Building Calibrated Simulation Approach - A savings measurement approach (defined in IPMVP Option D and ASHREA Guideline 14) that involves the use of an approved computer simulation program to develop a physical model of the building in order to determine energy and demand savings. The simulation program is used to model the energy used by the facility before and after the retrofit. The pre or post-retrofit models are developed by calibration with measured energy use and demand data and weather data.

RXQ.0.2.x 
Whole-Building Metered Approach - A savings measurement approach (defined in the IPMVP Option C and ASHRAE Guideline 14) that determines energy and demand savings through the use of whole-facility energy (end use) data, which may be measured by utility meters or data loggers. This approach may involve the use of monthly utility billing data or data gathered more frequently from a main meter.

<<  retail_EE-UnusedTerms  >>

Page 1

