1.  RECOMMENDED ACTION:	EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

| x | Accept as requested | x | Change to Existing Practice |
| --- | --- | --- | --- |
|  | Accept as modified below |  | Status Quo |
|  | Decline |  |  |

2.  TYPE OF DEVELOPMENT/MAINTENANCE

| Per Request: |  | Per Recommendation: |  |
| --- | --- | --- | --- |
| x | Initiation | x | Initiation |
|  | Modification |  | Modification |
|  | Interpretation |  | Interpretation |
|  | Withdrawal |  | Withdrawal |
| x | Principle | x | Principle |
| x | Definition | x | Definition |
| x | Business Practice Standard | x | Business Practice Standard |
|  | Document |  | Document |
|  | Data Element |  | Data Element |
|  | Code Value |  | Code Value |
|  | X12 Implementation Guide |  | X12 Implementation Guide |
|  | Business Process Documentation |  | Business Process Documentation |

Note: This does not include Technical data

3.  RECOMMENDATION

SUMMARY:

The Retail Electric Quadrant (REQ) DSM-EE Subcommittee submits this Recommendation for 2011 Retail Annual Plan Item No. 3(b) Develop business practice standards used to measure and verify reductions in energy and demand from energy efficiency in wholesale and retail markets.  This includes developing business practice standards to measure and verify energy reductions for energy efficiency or a stand-alone Energy Efficiency Portfolio Standard.

Energy Efficiency.

DISCLAIMER:  This document contains Model Business Practices for the evaluation of incremental annual and lifetime electrical energy and demand impacts of End-Use Energy Efficiency Programs implemented by retail Electric Distribution Companies or designated state Energy Efficiency Program Administrators.  The information contained within this document is not intended to replace applicable tariff, market rules, operating procedures, protocols or manuals, for retail Energy Efficiency (“Governing Documents”), and in the event of a conflict, the latter documents should have precedence over these Model Business Practices.

Energy Efficiency

Recommended Standards:

REQ.19 Evaluation, Measurement & Verification of Energy Efficiency Programs

TABLE OF CONTENTS

EXECUTIVE SUMMARY	7

VERSION NOTES	7

INTRODUCTION AND BACKGROUND	7

Scope of the Model Business Practices	7

REQ.19.2.	Definitions	8

REQ.19.2.1.	Business Definitions	8

REQ.19.2.2.	Technical Definitions	8

REQ.19.3.	Model Business Practices	14

REQ.19.3.1.	EM&V OBJECTIVES	14

REQ.19.3.2.	HIERARCHY OF DOCUMENTS	15

REQ.19.3.2.1.	EM&V Framework	15

REQ.19.3.2.2.	Portfolio EM&V Plan	15

REQ.19.3.2.3.	Impact Evaluation Plans	15

REQ.19.3.2.4.	Site Specific M&V (SSMV) Plans	15

REQ.19.3.2.5.	Site Specific M&V (SSMV) Report	15

REQ.19.3.2.6.	Impact Evaluation Report	16

REQ.19.3.2.7.	Portfolio Evaluation Report	16

REQ.19.3.3.	EM&V FRAMEWORK	17

REQ.19.3.3.1.	Development Process	17

REQ.19.3.3.2.	Regulatory and Legal Context	18

REQ.19.3.3.3.	Coverage	18

REQ.19.3.3.4.	EM&V Principles	18

REQ.19.3.3.5.	Objectives	18

REQ.19.3.3.6.	Reporting Energy Efficiency Savings Impacts 	18

REQ.19.3.3.7.	Definition of EM&V Cycle	18

REQ.19.3.3.8.	Who Conducts EM&V	19

REQ.19.3.3.9.	EM&V Schedule	19

REQ.19.3.3.10.	Requirements for Uncertainty Analysis	19

REQ.19.3.3.11.	Best Practices	19

REQ.19.3.3.12.	EM&V Methods and Key Assumptions	19

REQ.19.3.3.13.	Data Management Strategies	20

REQ.19.3.3.14.	Dispute Resolution	20

REQ.19.3.3.15.	Required Documents	Error! Bookmark not defined.

REQ.19.3.4.	PORTFOLIO EM&V PLAN	20

REQ.19.3.4.1.	Coverage	20

REQ.19.3.4.2.	Portfolio EM&V Objectives and Metrics	20

REQ.19.3.4.3.	EM&V Budget	20

REQ.19.3.4.4.	Summary and Schedule of EM&V Activities	21

REQ.19.3.5.	IMPACT EVALUATION PLANS	21

REQ.19.3.5.1.	Program Description	21

REQ.19.3.5.2.	Program Logic and Market Factors	21

REQ.19.3.5.3.	Ex-ante forecasts	21

REQ.19.3.5.4.	Policy Context	21

REQ.19.3.5.5.	Evaluability Assessment	21

REQ.19.3.5.6.	Potential Bias	21

REQ.19.3.5.7.	Sources of Uncertainty	22

REQ.19.3.5.8.	EM&V Methodologies	22

REQ.19.3.5.9.	Deemed Savings Estimates 	Error! Bookmark not defined.

REQ.19.3.5.10.	Large-scale Billing Analysis	Error! Bookmark not defined.

REQ.19.3.5.11.	M&V Approaches	22

REQ.19.3.6.	SITE SPECIFIC M&V PLAN	23

REQ.19.3.6.1.	SSMVP Required Elements	23

REQ.19.3.7.	SITE SPECIFIC M&V REPORTS	24

REQ.19.3.7.1.	Compliance with SSMVP	24

REQ.19.3.7.2.	TBD – from IPMVP	25

REQ.19.3.7.3.	TBD	Error! Bookmark not defined.

REQ.19.3.8.	IMPACT EVALUATION REPORTS	25

REQ.19.3.8.1.	Schedule	25

REQ.19.3.8.2.	Compliance with the Impact Evaluation Plan	25

REQ.19.3.8.3.	Key performance metrics	25

REQ.19.3.8.4.	Adjusted Gross Savings	25

REQ.19.3.8.5.	Reporting Impact Evaluation Results	25

REQ.19.3.9.	PORTFOLIO EVALUATION REPORTS	26

REQ.19.3.9.1.	Schedule	26

REQ.19.3.9.2.	Compliance with the Impact Evaluation Plans	26

REQ.19.3.9.3.	Common Reporting Formats	26

REQ.19.3.9.4.	Key Performance Metrics	26

REQ.19.3.9.5.	TBD	26

REQ.19.3.9.6.	TBD	26

REQ.19.3.10.	BASELINES AND USEFUL LIFE	26

REQ.19.3.10.1.	State Underlying Assumptions	27

REQ.19.3.10.2.	Early Retirement	27

REQ.19.3.10.3.	Standard Useful Lives	27

REQ.19.3.11.	STATISTICAL SIGNIFICANCE, SAMPLING AND OTHER SOURCES OF UNCERTAINTY	27

REQ.19.3.11.1.	General Requirements	27

REQ.19.3.11.2.	Statistical Significance	28

REQ.19.3.11.3.	Identify Statistical Confidence/Precision Requirements	28

REQ.19.3.11.4.	Establish Unique Precision Targets and Dimensions	28

REQ.19.3.11.5.	Pursue the Most Challenging Target	28

REQ.19.3.11.6.	Other Sources of Uncertainty and Threats to Validity:	28

4.  SUPPORTING DOCUMENTATION 	34

A.	Description of Request	34

B.	Description of Recommendation	35

C.	Business Purpose	35

D.	Commentary/Rationale of Subcommittee(s)/Task Forces(s):	35

Appendix A – EM&V Principles	36

Appendix B.  IPMVP Summary of Options	38

Appendix C – References	39

Additional Thoughts – Parking Lot	40

# EXECUTIVE SUMMARY

This document contains the Model Business Practices for the Evaluation, Measurement & Verification (EM&V) of End-Use Energy Efficiency Programs.  These Model Business Practices are applicable to the evaluation of incremental annual and life cycle electrical Energy and Demand impacts of Energy Efficiency programs implemented by and for electric Distribution Companies or other designated state Energy Efficiency program administrator (e.g., Energy Efficiency or sustainable energy utility).

Energy Efficiency programs encompass a variety of interactions between Distribution Companies (or Energy Efficiency/sustainable utilities), Retail Customers and Energy Services Providers.  These Model Business Practices may be applied within the context of regulatory or other market requirements and agreements. The information contained in this document does not replace applicable tariff, market rules, operating procedures, protocols or manuals for retail Energy Efficiency (“Governing Documents”).  In the event of a conflict, the Governing Documents should have precedence over these Model Business Practices.

# VERSION NOTES

(Insert publication date)

# INTRODUCTION AND BACKGROUND

The North American Energy Standards Board (NAESB) is a voluntary, non-profit organization comprised of members from all aspects of the natural gas and electric industries.  Within NAESB, the Retail Electric Quadrant (REQ) and the Retail Gas Quadrant (RGQ) focus on issues impacting the retail sale of energy to Retail Customers.  REQ / RGQ Model Business Practices are intended to provide guidance to Distribution Companies and other Market Participants involved in providing energy services to Retail Customers.

These Model Business Practices are voluntary and do not address many policy issues that are the subject of state legislation or regulatory decisions.  These Model Business Practices have been adopted with the realization that as the industry evolves, additional and amended Model Business Practices may be necessary.  These Model Business Practices may also reference other relevant standards, protocols or best practices.  Any industry participant seeking additional or amended Model Business Practices (including principles, definitions, data elements, process descriptions, and technical implementation instructions) should submit a request to the NAESB office, detailing the change, so that the appropriate process may take place to amend the Model Business Practice.

# Scope of the Model Business Practices

These Model Business Practices contain guidance for the Evaluation, Measurement and Verification (EM&V) of the impacts of retail electric end-use Energy Efficiency programs administered and/or sponsored by Distribution Companies or other designated Program Administrators (‘Program Administrators’).  They include only guidance for which consensus could be generated among the Market Participants.  They are not intended to be comprehensive in scope, nor does it contain the level of detail necessary to be a stand-alone guidance document.  Model Business Practices require that EM&V be conducted by qualified and experienced EM&V practitioners familiar with industry practices.

Specific limitations of the scope include:

- These Model Business Practices do not provide guidance related to the compensation, design, operation, or use of Energy Efficiency programs.

- These Model Business Practices do not provide guidance related to how the Impact Evaluation results are used.

- These Model Business Practices focus on conducting Impact Evaluations, and development of supporting plans and reports. They do not include guidance related to the evaluation of program design and implementation (i.e., process evaluation) or market assessments (market evaluations).

- These Model Business Practices focus on guidance for conducting Impact Evaluations to support determination of gross energy and demand savings.  They do not provide guidance on Net Savings determination (e.g., free-ridership or spillover). Though outside the scope of these Model Business Practices, determining Net Savings and Net-to-Gross Ratios may be desirable or required for program EM&V. Should these metrics be desired, it should be indicated in the EM&V Framework and subsequent planning documents.

### REQ.19.2	Definitions

#### REQ.19.2.A	Business Definitions NEEDS TO BE UPDATED

###### RXQ.0.2.xx	Applicable Regulatory Authority:

The state regulatory agency or other local governing body that provides oversight, policy guidance, and direction to any parties involved in the process of providing energy to Retail Customers through regulations and orders.

###### RXQ.0.2.xx	Baseline:

A method of estimating the electricity that would have been used by a Customer or a Demand Resource in the absence of a Demand Response Event.  It may be calculated using interval metering and/or statistical sampling techniques.  (NOTE: Ratified Definition)

###### RXQ.0.2.xx	Baseline Window:

A period of time preceding and optionally following a Demand Response Event over which the electricity usage data is collected for the purpose of establishing a Baseline.  (NOTE: Ratified Definition)

- RXQ.0.2.xx	Demand:

The rate at which electric energy is delivered to or by a system, generally expressed in kilowatts or megawatts, at a given instant or averaged over any designated interval of time; and the rate at which energy is being used by the Retail Customer.  (NOTE: Ratified Definition)

###### RXQ.0.2.xx	Demand Response:

A temporary change in electricity usage by a Demand Resource in response to market or reliability conditions.  For purposes of these Model Business Practices, Demand response does not include Energy Efficiency or permanent Load reductions.  (NOTE: Ratified Definition)

###### RXQ.0.2.xx	Distribution Company:

A regulated Entity which provides distribution services and may provide energy and/or transmission/transportation services in a given area.

###### RXQ.0.2.xx	Evaluation –

The conduct of any of a wide range of assessment studies and other activities aimed at determining the effects of a program, understanding or documenting program performance, program or program-related markets and market operations, program-induced changes in Energy Efficiency markets, levels of demand or energy savings, or program cost-effectiveness. Market assessment, monitoring and evaluation (M&E), and measurement and verification (M&V) are aspects of evaluation.

###### RXQ.0.2.xx	Evaluation:

The process used by the Registration Agent and other Market Participants to determine which Enriollment will be effectuated in the event that there are multiple Pending Enrollments within a defined period of time.  (NOTE: Ratified Definition)

###### RXQ.0.2.xx	Governing Documents:

Documents that determine the interactions among parties, including, but not limited to, applicable law, regulatory documents (e.g. tariffs, rules, regulations), contractual agreements, Distribution Company Operational Manuals, and other relevant models and operational procedures.

###### RXQ.0.2.xx	Market Participant:

A Distribution Company, Supplier, registration Agent, settlement agent, meter reading Entity or other party engaged in the process of providing competitive retail energy or another specific energy-related program to retail Customers.

###### RXQ.0.2.xx	Model Business Practices:

Electric and gas industry processes and procedures developed by interested parties representing the NAESB Retail Gas and Electric Quadrants’ segments and ratified by the NAESB Retail Gas and Electric Quadrants’ members.

###### RXQ.0.2.xx	Program Administrator:

An investor-owned, governmental or cooperative Distribution Company with the responsibility for developing and operating specific programs.  (NOTE: a Ratified Definition)

###### RXQ.0.2.xx	Retail Customer:

Any Entity that takes gas and/or electric service for its own use.

#### REQ.19.B	Technical Definitions

###### REQ.19.2.1t		Bias –

The extent to which a measurement or a sampling or analytic method systematically underestimates or overestimates a value.

- REQ.19.2.2t		Co-benefits –

The impacts of an Energy Efficiency program other than the direct purpose for which it was designed (i.e. Energy and Demand Savings). Examples include savings in other resources (gas, fossil fuel, and water), emissions reductions, and hazardous waste reduction.  Also see Non-electric benefits and Non-energy benefits.

###### REQ.19.2.3t		Confidence –

An indication of how close, expressed as a probability, the true value of the quantity in question is within a specified distance to the estimate of the value. Confidence is the likelihood that the Evaluation has captured the true value of a variable within a certain estimated range. Also see Precision.

- REQ.19.2.4t		Deemed Savings –

An estimate of Energy or Demand Savings for a single unit of an installed Energy Efficiency measure that (a) has been developed from data sources and analytical methods that are widely considered acceptable for the measure and purpose and (b) is applicable to the situation being evaluated. Individual parameters or calculation methods can also be deemed.

- REQ.19.2.5t		Demand Savings –

The reduction in electric or gas Demand from the Baseline to the Demand associated with the higher efficiency equipment or installation. This term is usually applied to billing Demand to calculate cost savings or to peak Demand for equipment sizing purposes.

###### REQ.19.2.6t		Demand Side Management (DSM) –

Strategies used to manage energy and demand including Energy Efficiency, load management, fuel substitution and load building.  (PRP Question:  Should this be submitted to Glosaary to become a Business Definition ??)

- REQ.19.2.7t		Energy –

The quantity characterizing the ability of a physical system to produce external activity. Energy exists in different forms transformable one into the other; examples are mechanical, electromagnetic, chemical, thermal, and nuclear energy. (PRP Question:  Should this be submitted to Glosaary to become a Business Definition ??)

Energy is power produced in the form of electricity, measured in kilowatthours or megawatthours.

- REQ.19.2.8t		Energy Efficiency –

The use of less energy to provide the same or an improved level of service to the Retail Customer; or the use of less energy to perform the same function. (PRP Question:  Should this be submitted to Glosaary to become a Business Definition ??)

Energy Efficiency is installed measures (e.g., products, equipment, systems, services, practices and/or strategies) on end-use customer facilities that reduce the total amount of electrical energy needed, while  delivering a comparable or improved level of end-use service. Such measures include, but are not limited to, the installation of more energy efficient lighting, motors, refrigeration, HVAC equipment and control systems, envelope measures, operations and maintenance procedures, and industrial process equipment.

###### REQ.19.2.9t		Energy Efficiency Measure –

An installed piece of equipment or system, or modification of equipment, systems, or operations on Retail Customer facilities that reduce the total amount of electrical or gas energy and capacity that would otherwise have been needed to deliver an equivalent or improved level of service.

Installed measures on Retail Customer facilities that achieve a permanent reduction in electric energy usage while maintaining a comparable quality of service. (WEQ definition)

- REQ.19.2.10t	Energy Savings –

Reduction in electricity use (kWh) or in fossil fuel use in thermal unit(s).

###### REQ.19.2.11t	Energy Services Company (ESCO) –

A firm that provides a range of Energy Efficiency and financing services and guarantees that specified results will be achieved under an energy performance contract.

###### REQ.19.2.12t	Energy Services Provider:

- REQ.19.2.13t	Free Rider –

A program participant who would have implemented the program measure or practice in the absence of the program. Free riders can be 1) total, in which the participant’s activity would have completely replicated the program measure; 2) partial, in which the participant’s activity would have partially replicated the program measure; or 3) deferred, in which the participant’s activity would have completely replicated the program measure, but at a future time than the program’s timeframe.

###### REQ.19.2.14t	Gross Savings

The change in energy usage and/or Demand that results directly from program-related actions taken by participants in an Energy Efficiency program, regardless of why they participated.

###### REQ.19.2.15t	 Impact Evaluation –

An Evaluation of the program-specific directly induced quantitative changes (e.g. kWh, kW, and therms) attributable to an Energy Efficiency program.

- REQ.19.2.16t	Measurement and Verification (M&V) –

.

The process of determining reductions in usage and/or demand resulting from Demand Response or Energy Efficiency. (WEQ Definition)

###### REQ.19.2.17t	Net Savings –

The total change in load that is attributable to an Energy Efficiency program. This change in Load may include, implicitly or explicitly, the effects of free drivers, Free Riders, Energy Efficiency standards, changes in the level of energy service, and other causes of changes in energy usage or Demand.

###### REQ.19.2.19t	Non-Participant –

Any Retail Customer who was eligible but did not participate in the subject Energy Efficiency program in a given program year.

###### REQ.19,2,22t	Portfolio –

A collection of similar programs addressing the same market (e.g. a Portfolio of residential programs), technology (e.g. motor efficiency programs), or mechanisms (e.g. loan programs). (b) The set of all programs conducted by one or more organizations, such as a Program Administrator (and which could include programs that cover multiple markets, technologies, etc.).

###### REQ.19.2.23t	Precision –

The indication of the closeness of agreement among repeated measurements of the same physical quantity. It is also used to represent the degree to which an estimated result in social science (e.g. energy savings) would be replicated with repeated studies.

###### REQ.19.2.25t	Program Participant –

A Retail Customer that received a service offered through an Energy Efficiency program in a given program year. The term “service” can be one or more of a wide variety of services, including financial rebates, technical assistance, product installations, training, Energy Efficiency information or other services, items, or conditions.

###### REQ.19.2.26t	Project –

An activity or course of action involving one or multiple Energy Efficiency measures, at a single facility or site.

###### REQ.19.2.27t	Sampling Error –

The error in estimating a parameter caused by the fact that in the sample at hand all the disturbances are not zero.

###### REQ.19.2.28t	Spillover –

Reductions in energy usage and/or Demand caused by the presence of an Energy Efficiency program, beyond the program-related Gross Savings of the participants and without financial or technical assistance from the program. There can be Participant and/or Non-Participant Spillover. Participant Spillover is the additional energy savings that occur when a program Participant independently installs Energy Efficiency measures or applies energy saving practices after having participated in the Energy Efficiency program as a result of the program’s influence. Non-Participant Spillover refers to energy savings that occur when a program Non-Participant installs Energy Efficiency measures or applies energy savings practices as a result as a result of a program’s influence.

###### REQ.19.2.29t	Uncertainty –

The range or interval of doubt surrounding a measured or calculated value within which the true value is expected to fall with some degree of confidence.

###### REQ.19.2.30t	Verification –

An independent assessment of the reliability (considering completeness and accuracy) of claimed energy savings or an emissions source inventory.

### REQ.19.3	Model Business Practices

#### REQ.19.3.1	EM&V OBJECTIVES

There are three key objectives of evaluations:

- To document and measure the effects of Energy Efficiency activities and determine whether these activities met their goals with respect to being a reliable energy resource.

- To provide data for energy resource planning.

- To help understand why the Energy Efficiency activities’ effects occurred, identify ways to improve or discontinue current activities, and select future activities.

Energy Efficiency evaluations should develop estimates of energy savings from Energy Efficiency programs in a manner that is defensible in regulatory proceedings that are conducted to ensure that funds are properly and effectively spent to support relevant or applicable policies. In addition to serving as basis for documenting savings, evaluation results should be used to improve programs and to provide a basis for future savings estimates.

#### REQ.19.3.2	HIERARCHY OF DOCUMENTS

Each jurisdiction that adopts these Model Business Practices should document relevant Energy Efficiency policy decisions, set forth jurisdiction-specific EM&V objectives and requirements, plan and budget for EM&V activities, conduct EM&V activities, and report results. The following hierarchy of documents provides a useful framework for EM&V activities. It is not a requirement of these Model Business Practices that a jurisdiction follow the exact structure shown below. Jurisdictions may combine documents or otherwise modify the structure. The order of the section follows the chronological order in which the documents are commonly prepared.

##### REQ.19.3.2.1	EM&V Framework

A framework is a primary document that lays out EM&V principles, metrics, allowable approaches, net versus gross savings issues, reporting requirements, schedules, and the roles and responsibilities of various entities. An EM&V framework is established [describe timeframe], but can be updated periodically.  The EM&V framework sets the expectations for the content and scope of other EM&V documents.

##### REQ.19.3.2.2	Portfolio EM&V Plan

A Portfolio EM&V Plan indicates the major evaluation activities that will be conducted during the evaluation cycle (typically one, two or three years), including budget and allocation between programs/measures/market sectors, as applicable. These plans can be prepared, or updated, annually or at the start of each planning cycle.

##### REQ.19.3.2.3	Impact Evaluation Plans

Specific research plans are created for the major EM&V activities planned in a given cycle prior to the time each effort is launched. Such documents are frequently referred to as program impact evaluation plans.

##### REQ.19.3.2.4	Site Specific M&V (SSMV) Plans

Site-specific measurement and verification plans may be required for custom project sites that are analyzed and/or inspected. SSMV Plans are not typically required for all measures or participants in a program.

##### REQ.19.3.2.5	Site Specific M&V (SSMV) Report

Site-specific measurement and verification reports are required for projects for which SSMV Plans were developed.  They document the impacts determined for a specific site or measure as well as the methods used to determine the impacts.

##### REQ.19.3.2.6	Impact Evaluation Report

The results of carrying out the evaluation activities described in each Impact Evaluation Plan are documented in an Impact Evaluation Report.  The report documents the impacts of a program or bundle of programs as well as the methods used to determine the impacts.

##### REQ.193.2.7	Portfolio Evaluation Report

The results of carrying out the evaluation activities described in Portfolio Plan are documented in a Portfolio Evaluation Report. It documents the impacts associated with the program portfolio as well as the methods used to determine the impacts. This report often discusses portfolio-level goals and the Program Administrator’s progress toward meeting those goals.

- EM&V Framework

  - Portfolio EM&V Plan for each period

    - Detailed Research Plans

      - Program Site Specific M&V plans

  - Reporting for each Planning Period

    - Impact Evaluation Reports

      - Program M&V Reports

  - Portfolio Evaluation Report for each Period

#### Figure 1. EM&V Framework

#### REQ.19.3.3	EM&V FRAMEWORK

The overarching planning document is the EM&V Framework. The purpose of this document is to describe the process by which the Program Evaluator will document the energy and demand savings and other metrics associated with the implementer’s Demand Side Management activities and to indicate a range of methods to be used as well as the process for continuous improvement and third-party review.  The following sections should be considered for inclusion in the EM&V Framework document:

##### REQ.19.3.3.1	Development Process

The EM&V Framework is typically developed with oversight and approval by the Regulatory Agency and should ideally be developed with input from a wide range of key stakeholders including evaluators, regulated utilities, ratepayer advocates, environmental advocates, and program administrators.

##### REQ.19.3.3.2	Regulatory and Legal Context

The EM&V Framework document should summarize the appropriate regulations and enabling legislation.

##### REQ.19.3.3.3	Coverage

The EM&V Framework document should define the Energy Efficiency Programs or Portfolios covered by the Portfolio EM&V Plan.

##### REQ.19.3.3.4	EM&V Principles

The EM&V Framework document should define the EM&V principles, such as independent evaluation and how a verification process or other means will be used to ensure evaluation credibility and accuracy.

##### REQ.19.3.3.5	Objectives

The EM&V Framework should include the following evaluation objectives:

- Determining cost effectiveness

- Determining if compliance with savings targets have been met

- Program feedback and enhancement

- Resource planning input

- Compensation/cost recovery for administrators

- Determining non-energy benefits, such as avoided emissions

##### REQ.19.3.3.6	Reporting Energy Efficiency Savings Impacts

The EM&V Framework should provide guidance about transparency and reporting of Energy Efficiency impacts. Parameters reported at the customer sector level (Residential, Low Income, Commercial and Industrial) and by specific Program Administrator Energy Efficiency programs should, at a minimum, include:

- Annual Incremental Energy Savings

- Lifetime Energy Savings

- Demand Savings (with options for basis of peak period)

- Avoided Emissions (with basis/explanation for calculation method)

- For the four bullets above, report whether whether energy savings, demand savings or other impacts should be reported as gross, adjusted gross, and/or net impacts.

##### REQ.19.3.3.6.1	Other Reporting Items:

##### In addition to the items listed in REQ.19.3.3.6, the following items should also be considered:

- Program funding sources

- Program expenditures

- Leveled Cost of Saved Energy

##### REQ.19.3.3.7	Definition of EM&V Cycle

The EM&V Framework should describe the EM&V cycle with respect to the EM&V activities and reporting as follows:

- Time frame of analyses

- Hierarchy of planning steps for each cycle

- The plan should describe any deviations from the documents recommended by this Model Business Practice.

##### REQ.19.3.3.8	Who Conducts EM&V

The EM&V Framework should include a discussion of the entity(ies) that will conduct EM&V, including:

- How is independent EM&V defined

- By whom they will be retained e.g., statewide evaluator reporting to utilities with independent verifier reporting to commission, evaluator reporting to commission, etc.

- Process for 3rd party consultant selection, as applicable

##### REQ.19.3.3.9	EM&V Schedule

The EM&V Framework should provide a clear schedule of activities, taking into consideration the needs and constraints of the regulatory framework and should be developed in collaboration with EM&V contractor, Applicable Regulatory Authority,legislators and other stakeholders, as appropriate. In general, EM&V results should be available for incorporation into subsequent program planning.

##### REQ.19.3.3.10	Requirements for Uncertainty Analysis

The EM&V Framework should include key expectations for the level of uncertainty of savings and other impacts to be evaluated.  Specific requirements should be provided in the Portfolio EM&V Plan.  The Framework should:

- Include a requirement that evaluators take steps to control for systematic error through best practices, trained experts, etc.

- Control for random Sampling Error by defining a Confidence and Precision level for any sampling to be done.

- Define how the certainty of evaluation impact estimates will be reported.

##### REQ.19.3.3.11	EM&V Methods and Key Assumptions

The EM&V Framework should provide guidance on what EM&V approaches and key assumptions will be used and how they will be selected.  Specific guidance may include:

- What Baselines are used, general definitions of Baseline, factors or implications to consider in developing Baselines

- Deemed Savings and deemed calculated savings “values,” and how and when will sources of values be updated

- Level of detail required for Impact Evaluation research

##### REQ.3.3.12	Data Management Strategies

The EM&V Framework should provide guidance on:

- Data to be collected

- Tracking system requirements to be used

- Design schema and structure of data flow

- Specification of standardized data sets

- How quality assurance and control will be enforced

##### REQ.19.3.3.13	Reporting and Dispute Resolution

The EM&V Framework should provide guidance on the reporting process and schedule as well as dispute resolution process. The EM&V Framework should include a list and general description of required documents and reports.

#### REQ.,19.3.4	PORTFOLIO EM&V PLAN

The Portfolio EM&V Plan should outline the major EM&V activities that will be conducted during the planning cycle, including budget and allocation between programs/measures/market sectors, as applicable, and guided by the EM&V Framework.  The purpose of this document is to describe the major EM&V activities that will be conducted during the year, including budget and allocation between programs/measures/market sectors, EM&V schedules, and type, if not identify, of Entities that will be evaluators.

##### REQ.19.3.4.1	Coverage

The Portfolio EM&V Plan should define the Energy Efficiency programs within the portfolio of programs (if there are multiple programs) covered by the EM&V activities indicated and the period of time covered by the plan, typically one or two years but sometimes three years.

##### REQ.19.3.4.2	Portfolio EM&V Objectives and Metrics

In conformance with the EM&V Framework, the Portfolio EM&V Plan should define objectives of the programs covered by the EM&V activities indicated and the metrics that will be reported. Objectives may include some or all of the following and are not limited to the following:

- Determining cost effectiveness

- Determining if compliance with savings targets have been met

- Program feedback and enhancement

- Resource planning input

- Compensation/cost recovery for administrators

- Determining non-energy benefits, such as avoided emissions

Metrics to be reported may include some or all of the following and are not limited to the following:

- Annual Incremental Energy Savings

- Lifetime Energy Savings

- Demand Savings (with options for basis of peak period)

- Program funding sources

- Program expenditures

- Leveled Cost of Saved Energy

- Avoided Emissions (with basis/explanation for calculation method)

Savings and other impacts may be reported on gross, adjusted gross, and/or net bases.

##### REQ.19.3.4.3	EM&V Budget

An overall EM&V Budget should be indicated with conditions under which changes might be adopted as necessary. Budget allocations should be presented indicating allocation between the EM&V activities and programs, and rationale for allocation.  Trade-offs between cost and certainty should be discussed.

##### REQ.19.3.4.4	Summary and Schedule of EM&V Activities

The Portfolio EM&V Plan should provide a summary of, and associated schedule for key EM&V activities including:

- Define baseline assumptions and any studies, or other activities, which will be conducted to define program and measure baselines.

- Technical reference manuals indicating deemed savings values and/or deemed savings algorithms to be used in programs (if any)

- Process EM&V

- Market Evaluations

#### REQ.19.3.5	IMPACT EVALUATION PLANS

EM&V activity specific detailed research plans, or Impact Evaluation Plans, are developed for each study in the Portfolio Plan. They should address the following:

##### REQ.19.3.5.1	Program Description

The Impact Evaluation Plans should include descriptions of the portfolio or program to be evaluated. Portfolio and program descriptions should include savings goals and other program objectives and the associated metrics on which they will be evaluated,, targeted program participants, key stakeholders and their roles (e.g, trade allies, retailers, contractors), outreach strategies, participation incentives, and other important program components.

##### REQ.19.3.5.2	Program Logic and Market Factors

The Impact Evaluation Plans should include a clear description and understanding of the logic associated with each program.  It should also include discussion of market barriers, external influences and agents (including other programs), and performance indicators. Creation of program logic diagrams is strongly encouraged.

##### REQ.19.3.5.3	Ex-ante forecasts

The Impact Evaluation Plan should include forecasts of numbers of participants, energy efficiency measures undertaken (e.g., equipment installed, homes weatherized, audits conducted),  estimated energy savings, and other metrics (e.g., demand savings, avoided emissions) consistent with program objectives and metrics.

##### REQ.19.3.5.4	Policy Context

Policy context should demonstrate understanding of EM&V policy objectives and stakeholders.  Potentially conflicting EM&V requirements – e.g., EM&V requirements for bids into forward capacity markets – should be acknowledged.

##### REQ.19.3.5.5	Data Quality Assessment

Existing Program Administrator tracking systems should include preliminary assessment of program data quality and transferability.

##### REQ.19.3.5.6	Potential Bias

The Impact Evaluation Plans should include a section that discusses the various sources of bias and the implications for the EM&V of each program, and the Portfolio, as a whole.  Plans to eliminate sources of potential bias should also be discussed.

##### REQ.19.3.5.7	Sources of Uncertainty

The Impact Evaluation Plan should include a section on various sources of uncertainty, and a discussion of the implications for the EM&V of each program, and the Portfolio, as a whole.  The trade-offs between cost and uncertainty should be discussed and general principles that the evaluator is should use to ensure value-maximization for the client.

##### REQ.19.3.5.8	Baselines

The Impact Evaluation Plan should define and describe the baseline.  A major impact evaluation decision is defining the baseline. The baseline defines the conditions, including energy consumption and related emissions, that would have occurred without the subject program. Baseline definitions consist of site-specific issues and broader, policy-oriented considerations.  Baseline approaches and definitions are generally defined in the Program Evaluation Plan.

##### REQ.19.3.5.9	M&V Approaches (directly from NAPEE Guide, reference)

M&V approach(es) used to determine gross impact savings of the subject programs should use one or more of the following approaches:

- Measurement and verification (M&V). A representative sample of projects in the program is selected and the savings from those selected projects are determined and applied to the entire population of projects, i.e. the program. The individual project savings are determined using one or more of the four M&V options defined in the IPMVP, as noted in REQ.19.3.6 Site Specific M&V Plan and Appendix B. This is the most common approach used for programs involving non-residential facilities, retrofit, or new construction, in which a wide variety of factors determine savings and when individual facility savings values are desired.

- Deemed savings. Savings are based on stipulated values, which come from historical savings values of typical projects. As with the M&V approach, the savings determined for a sample of projects are applied to all the projects in the program. However, with the use of deemed savings there are no or very limited measurement activities and only the installation and operation of measures is verified. This approach is only valid for projects with fixed operating conditions and well-known, documented stipulation values (e.g., energy-efficient appliances such as washing machines, computer equipment and refrigerators, lighting retrofit projects with well understood operating hours). This approach involves multiplying the number of installed measures by the estimated (or deemed) savings per measure. Deemed savings values are only valid when they are derived from documented and validated sources, such as historical evaluations, and only apply to the most common efficiency measures. Deemed savings are the per- unit energy savings values that can be claimed from installing specific measures under specific operating situations. Examples include agreed-upon savings per fixture for lighting retrofits in office buildings, with specific values for lights in private offices, common areas, hallways, etc.

- Large-scale data analysis. Statistical analyses are conducted on the energy usage data (typically collected from the meter data reported on utility bills) for all or most of the participants and possibly non-participants in the program. This approach is primarily used for residential programs with relatively homogenous participants and measures, when project-specific analyses are not required or practical.

#### REQ.19.3.6	SITE SPECIFIC M&V PLAN

Many Impact Evaluation approaches rely in part on the use of metering and onsite verification to determine the savings achieved for certain Energy Efficiency installations. For each project selected to be in the Impact Evaluation sample, a Site Specific M&V Plan should be prepared. Measurement and Verification methodologies should be consistent with the International Performance Measurement and Verification Protocol (IPMVP), 2010 or current version.  IPMVP offers four options or approaches to conducting M&V depending on the type of project or project objectives and budget.  The options include:

- Option A: Partially Measured Retrofit Isolation – measurement of key parameters

###### Option B: Retrofit Isolation/Metered Equipment – measurement of all parameters

###### Option C: Whole facility/Regression Analysis

###### Option D: Calibrated Simulation

More information on the IPMVP options is provided in Appendix B.

##### REQ.19.3.6.1	SSMVP Required Elements

An SSMVP should include the following:

###### REQ.19.3.6.1.1	Project Description

The SSMVP should include a project description, not limited to: a general description of the project location, facility types, measure types, number and type of project participants,  ex ante estimated energy savings and other project objective metrics (e.g., demand savings) and estimated cost of project (capital, operation and M&V).

###### REQ.19.3.6.1.2	Schedule

Plan should include a schedule describing the timeline of the project, with key milestones for commencement, implementation, M&V, and reporting.  Plans should include a detailed schedule of M&V activity during project implementation and following installation of energy efficiency measures or other project activities.

###### REQ.19.3.6.1.3	Measure Description

Plan should include measure level characteristics, including but not limited to:

###### REQ.19.3.6.1.4	Equipment/Process Specification and Documentation

Plan should include a detailed description of existing and new equipment and process characteristics sufficient to establish Energy Efficiency Baseline calculations.

###### REQ.19.3.6.1.5	Measurement and Verification Approach

Plan should describe and  justify the M&V approach taken.  The approach may be, but is not necessarily limited to,  the four IPMVP options, deemed savings, and large-scale data analysis methodologies at the measure level.

###### REQ.19.3.6.1.6	Methodological Assumptions

Plan should include a detailed description of underlying assumptions for Energy Efficiency Baseline and post installation energy use.

###### REQ.19.3.6.1.7	M&V Activities: Energy Efficiency Baseline Period

Plan should include a description of the period of time under which the Energy Efficiency Baseline will be quantified.

###### REQ.19.3.6.1.8	M&V Activities: Post Installation Period

Plan should include a description of activities conducted post installation to document actual energy savings and, if among the project’s objectives, demand reduction.

###### REQ.19.3.6.1.9	Metering Plan

If the M&V approach includes metering, the plan should include a detailed description of meter management, not limited to maintenance, meter data collection and processing.

#### REQ.19.3.7	SITE SPECIFIC M&V REPORTS

The following should be included in a Site Specific M&V Report:

##### REQ.19.3.7.1	Compliance with SSMVP

The Site Specific M&V Report should describe any deviations in approach or algorithms between those used to determine the verified savings and those which the SSMVP anticipated would be used.

##### REQ.19.3.7.2	TBD – either copy from or refer to report description in IPMVP

#### REQ.19.3.8	IMPACT EVALUATION REPORTS

The following should be included in an Impact Evaluation Report:

##### REQ.19.3.8.1	Schedule

The Impact Evaluation reports should all be submitted in accordance with the schedule set forth in the Portfolio Evaluation Plan.

##### REQ.19.3.8.2	Compliance with the Impact Evaluation Plan

The Impact Evaluation Report should describe any deviations in approach or algorithms between those used to determine the verified savings and those which the Impact Evaluation Plan anticipated would be used.

##### REQ.19.3.8.3	Key performance metrics

The key performance metrics should be submitted in accordance with the schedule set forth in the Impact Evaluation Plan.  It is recommended that the following parameters be reported, but are not limited to:

- Annual Incremental Energy Savings

- Lifetime Energy Savings

- Demand Savings (with options for basis of peak period)

- Program funding sources

- Program expenditures

- Leveled Cost of Saved Energy

- Avoided Emissions (with basis/explanation for calculation method)

Savings and other impacts may be reported on gross, adjusted gross, and/or net bases as required by the Impact Evaluation Plan.

##### REQ.19.3.8.4	Adjusted Gross Savings and Net Savings

The Impact Evaluation Report  should include a summary that indicates the types of adjustments that were made to Gross Energy Savings (or that are included in Realization Rate) to derive Adjusted Gross Savings or, if required, Net Savings. The report should also indicate if adjustments are made to some or all measures. Adjustments to derive Adjusted Gross Savings should include but are not limited to:

- Data Errors

- Measure Persistence Factor

- Savings Persistence Factor

- In-Service Rate

- Interactive Effects

Additional adjustments, such as for Free Riders and Spillover, may be made if Net Savings estimates are required by the Impact Evaluation Plan. (Guidance Net Savings estimation is not covered by this Model Business Practice).

##### REQ.19.3.8.5	Reporting Impact Evaluation Results

The evaluation results should include reporting parameters consistent with the EM&V Framework Plan, as provided in REQ 019.3.3.6.  Additional tables will likely be included at the discretion of the evaluator, and additional tables may be requested by the appropriate regulatory agency or the utility.

#### REQ.19.3.9	PORTFOLIO EVALUATION REPORTS

The Portfolio evaluation results should include reporting parameters consistent with the Portfolio Evaluation Plan, REQ…. The following should be included in a Portfolio Evaluation Report:

##### REQ.19.3.9.1	Schedule

The Portfolio Evaluation reports should be submitted in accordance with the schedule set forth in the Portfolio Evaluation Plan.

##### REQ.19.3.9.2	Compliance with the Impact Evaluation Plans

The Portfolio Evaluation Report should describe any deviations in approach or algorithms between those used in the Impact Evaluations to determine the verified savings and those which the Portfolio Evaluation Plan anticipated would be used.

##### REQ.19.3.9.3	Common Reporting Formats

The evaluation results should include the summary reports defined in the NEEP reporting guidelines. Additional tables will likely be included at the discretion of the evaluator and additional tables can be requested by the appropriate regulatory agency or the utility.

##### REQ.19.3.9.4	Key Performance Metrics

The key performance metrics should be submitted in accordance with the schedule set forth in the portfolio evaluation plan and it is recommended that the following parameters be reported:

- Annual Incremental Energy Savings

- Lifetime Energy Savings

- Demand Savings (with options for basis of peak period)

- Program funding sources

- Program expenditures

- Leveled Cost of Saved Energy

- Avoided Emissions (with basis/explanation for calculation method)

Savings and other impacts may be reported on gross, adjusted gross, and/or net bases as required by the Portfolio Evaluation Plan.

#### REQ.19.3.10	BASELINES AND USEFUL LIFE

A key aspect of determining energy and demand savings is defining baseline conditions.  The following is guidance that, while not required for compliance with this MBP, describes considerations and typical approaches to addressing baseline issues.

##### REQ.19.3.10.1	State Underlying Assumptions

Baseline definitions should include a description of underlying assumptions used for establishing the baseline conditions that would have occurred in the absence of the program (i.e. the counterfactual)

##### REQ.19.3.10.2	Early Retirement

For programs that promote the early retirement of functioning appliances or technology, evaluators can use as the Baseline, the estimated energy use of the existing, in place equipment.  This baseline would be applied for a period of time not to exceed the remaining useful life of the equipment being replaced, with possible consideration of equipment repairs and possible use of equipment beyond useful life.

Where practical, data about the existing equipment should be collected by Program Administrators who verify the working condition of the existing equipment, collect the nameplate data and provide estimates of the remaining useful life of equipment. After the remaining useful life of the baseline equipment expires, the baseline, for the remaining useful life of the new high-efficiency equipment, can be new equipment that can be assumed to have been installed that meets codes/standards or is common practice.

##### REQ.19.3.10.3	Standard Useful Lives

In lieu of determining useful life and energy savings on a case-by-case basis, the standard useful lives of various types of appliances and equipment may be stipulated.  Stipulated useful life assumptions should be clearly documented and readily available for review by the Applicable Regulatory Authority and other stakeholders.

#### REQ.19.3.11	STATISTICAL SIGNIFICANCE, SAMPLING AND OTHER SOURCES OF UNCERTAINTY

##### REQ.19.3.11.1	General Requirements

Means for control of systematic error and uncertainty should be considered, along with determining acceptable levels of confidence and precision. Both systematic (due to methodological flaws or bias) and random error should be addressed. Error and uncertainty should be controlled in the modeling and measurement of savings; representative sampling should be managed in such a way as to mitigate uncertainty.  Quality assurance and control systems should be established and maintained.  Results should be reported with an indication of the reliability, e.g. precision and confidence, of results

##### REQ.19.3.11.2	Statistical Significance

Statistical significance of sampled data should include the following considerations:

##### REQ.19.3.11.2.1	Identify Statistical Confidence/Precision Requirements

These should include key regulatory and/or other requirements (e.g. capacity market standards) and/or legacy objectives.  The domain for applying statistical significance may vary depending on the requirement, be it the portfolio, program, state, load-zone, etc.

##### REQ.19.3.11.2.2	Establish Unique Precision Targets and Dimensions

Regulatory and market requirements may offer Program Administrators either a threshold or a range of confidence intervals and precision. In either case, Program Administrators may make an independent assessment of the precision targets that are necessary for their particular needs relative to the domain of the EM&V (i.e. sector, program, end use,), their intended use and audience for the EM&V results, and considerations of expected variability and the financial or system impact of varying degrees of uncertainty.

##### REQ.19.3.11.2.3	Pursue the Most Appropriate Target

Statistical objectives are typically multi-pronged, with different confidence intervals and precision levels (e.g., 80/10 for summer kW, 80/10 for winter kW, and 90/10 for energy kWh.  Designing a single sample to meet all objectives can be difficult and is dependent upon the unique population characteristics and expected variability for each parameter.  In practice, one often can achieve all objectives by pursuing the element with the greatest variability (e.g., large commercial and industrial programs can be the winter coincident demand impact).

##### REQ.19.3.11..2.4	Other Sources of Uncertainty and Threats to Validity:

Confidence/precision requirements are for statistical sampling alone and do not reflect other important sources of uncertainty such as measurement error, equipment accuracy, and parameter bias.  Statistical precision can be misleading if there is bias or non-statistical error in the underlying data.  Bias can be hard to identify and extremely difficult to quantify, but should not be ignored or dismissed.  Evaluators should remain vigilant for sources of error such as response bias, hand-picked (or excluded) sample projects, and measurement error.  See NAESB Wholesale M&V Standards and/or ISO New England and PJM Interconnection Energy Efficiency M&V Manuals for more detailed information on types of bias to address for different EM&V methodologies.

# SUPPORTING DOCUMENTATION

(STILL TO BE DRAFTED – CURRENTLY USES WHOLESALE LANGUAGE)

## Description of Request

WEQ 2010 Annual Plan Item 4 (d) (from WEQ doc)

Develop business practice standards used to measure and verify reductions in energy and demand from Energy Efficiency in wholesale and retail markets.   This includes developing business practice standards to measure and verify energy reductions that are made to comply with a Renewable Portfolio Standard that included Energy Efficiency or a stand-alone Energy Efficiency Portfolio Standard.

## Description of Recommendation

(from WEQ doc) The recommended Business Practice Standards support the measurement and verification characteristics of Energy Efficiency products and services administered for application in the wholesale market and may be the subject of individual tariffs filed with and approved by the Federal Energy Regulatory Commission.

## Business Purpose

## Commentary/Rationale of Subcommittee(s)/Task Forces(s):

Energy Efficiency programs are an increasingly important component of retail electric supply, offsetting the need for new generation and distribution capacity, reducing the overall cost of energy services, and reducing greenhouse gas emissions and other pollutants associated with the generation of electricity.  The accurate estimation of impacts from Energy Efficiency programs is necessary to guide investments in Energy Efficiency programs, improve program design and execution, and enable Energy Efficiency programs to be credited as resources in electric utility system planning.  Consistent methods and practices within political jurisdictions can facilitate comparison of Energy Efficiency programs, help reveal best practices, and enhance credibility with some stakeholders.  Consistent methods across political jurisdictions could facilitate policies such as Energy Efficiency resource standards, programmatic carbon offsets, and mechanisms intended to compensate program administrators and Distribution Companies for superior program performance.

An extensive body of Energy Efficiency program Evaluation, Measurement and Verification research and a substantial and diverse professional capacity is available to evaluate Energy Efficiency programs.

The intent of these Model Business Practices is to:

- Provide Distribution Companies, Applicable Regulatory Authorities, and other Market Participants with a greater understanding of how Energy Efficiency programs are evaluated, highlight key issues that need to be addressed as part of an Energy Efficiency program Evaluation, suggest minimum acceptable practices for addressing some of those issues, and suggest sources for additional information.

- Help Distribution Companies and Applicable Regulatory Authorities understand the various tradeoffs between Evaluation cost and uncertainty, know what to expect and ask of program evaluators, ensure Evaluations meet accepted industry standards if not best practices, and structure an Evaluation oversight process that will maximize credibility with stakeholders.

- Ensure the highest quality and integrity of Energy Efficiency program Evaluations given inevitable resource constraints.

In doing so, it is expected that these Model Business Practices for the Evaluation of retail Energy Efficiency programs will broaden the implementation and acceptance of energy reduction measures and practices.

# Appendix A – EM&V Principles

Evaluations should be complete, transparent, relevant, consistent, and balanced in risk management between certainty of results and costs to achieve the results. They should also follow the guiding principles defined by the American Evaluation Association http://www.eval.org.  In addition, several documents list principles that can be adopted as guiding evaluation activities.  One such set of principles is maintained by the Consortium for Energy Efficiency and are listed below, effective as of Spring 2011:

Variables that are measured tend to drive outcomes. Consider the possible unintended consequences of required goals and related progress indicators or metrics.

- As much as possible, plan for evaluation in conjunction with program planning and implementation, and look to evaluation to serve both program and broader organizational goals. Planning in advance for evaluation-related activities to occur at appropriate points in the cycle of program planning and implementation, rather than planning for evaluation after program launch, results in more efficient and effective evaluation expenditures.

- Agree on baseline considerations for measuring savings, including measurement methodology, at the beginning of program and evaluation planning. “Baseline” refers to initial measurements of energy use and other key variables that are expected to change in response to an Energy Efficiency program.

- To ensure EM&V results are timely, consider the rate of change of programs, the markets they serve, or regulations, and take this into account in planning for evaluation.

- In planning for evaluation, consider and disclose causal factors other than the program activities being evaluated that could contribute to observed outcomes, and the possible impacts of these factors on the outcomes.  Such factors can include (but are not limited to): other programs, tax incentives, government standards, building codes, and changes in technology or markets.

- Consider the choice of cost-effectiveness test, and inputs to the test, carefully in light of program goals. Measurement of cost-effectiveness is sensitive both to the type of cost-effectiveness test selected and to the associated input assumptions, such as discount rate, avoided costs, how program administrative costs are allocated among programs, whether program start up costs are amortized over multiple years , whether externality costs and benefits are included etc.

- EM&V should consider interactive effects of various measures, where measures are installed, and how people will interact with measures.

- Programs aimed at altering behavior, manufacturing and stocking practices, and other lasting structural change tend to require a different evaluation approach from traditional rebate programs. Lasting structural change takes time, so longer measurement time horizons are needed.

- In determining precision requirements, consider how the results are to be used and what is practical and affordable relative to the undertaking or goal. There is no single rule or confidence interval for evaluation estimates that is attainable or useful for all evaluations. None of the key elements of utility resource planning, such as outage rates or fossil price forecasts, are precise, yet the process is designed to manage with reasonable imprecision. In the same spirit, a good evaluation plan allocates an appropriate level of resources toward enough precision for the things we need to know most.

- Disclose the level of precision associated with reported measurements. To avoid a misleading impression of an estimate's precision, provide evaluation estimates of a program’s impacts with a range, usually including a central estimate, rather than just a single figure.

- Recognize, address and report potential sources of bias in evaluation methods or data that could produce inaccurate results, no matter what the level of precision. Identifying and correcting sources of bias should be a top priority of any evaluation.

- Consider evaluation as a routine part of a continuous program improvement process.  Ongoing feedback from process evaluations, market assessments and impact evaluations allows for program assumptions to be routinely tested and helps focus programs.

- Where program administrators have mature data tracking systems and reporting capability, internal capability for data cleaning and quality control, and collect necessary evaluation data along the way, evaluation is more affordable, timely and accurate.

- The timing of data collection is important to good evaluation. Depending on the design and objectives of particular projects in a program, data may need to be collected at one or more of the following times: pre-program intervention to establish baselines, soon after an intervention to verify measure installation or other program activity while memories are fresh, and one or more later intervals to assess impacts over time. Generally, evaluations should adhere to planned data collection schedules to assure accuracy, comparability, and consistency of analyses and to provide feedback for program improvement.

- Allow adequate time to pass before calculating cost-effectiveness. Program costs effectiveness often improves over time as programs mature, participation increases, and start up costs are spread across  start up costs are spread over several years of program activity.

- Multiple EM&V methods can be used to evaluate the same program.  Combining multiple methods with lower precision may provide more accurate results at lower cost.

- As programs and markets evolve, the evaluation methods may need to change. Ensure that the evaluation method(s) being used continue to be appropriate.

- Evaluation results should be communicated in a manner that allows those with limited background in the subject matter to understand the key findings and conclusions.

- Allocate evaluation resources in relation to the expected program savings, level of uncertainty of savings estimates, cost or difficulty of measurement, and the importance of the undertaking or goal.

# Appendix B.  IPMVP Summary of Options

| IPMVP Summary of Options | Option A | Option B | Option C | Option D |
| --- | --- | --- | --- | --- |
| Financial objective | Retrofit isolation systems: key parameter measurement | Retrofit isolation systems: all parameter measurement | Whole facility | Calibrated simulation |
| Description | Savings are calculated using data from the main performance parameter(s) defining energy consumption for the system involved in the Energy Efficiency solution. Estimates are used for parameters not chosen for actual measurements. | Savings are calculated using actual energy consumption data for the system involved in the Energy Efficiency solution | Savings are established using actual energy consumption data for the facility or a section of it. Data for energy use within the facility as a whole is gathered on an ongoing basis throughout the reporting period. | Savings are established by simulating energy consumption for the facility or a section of it. There must be evidence that the simulation procedures are providing an adequate model of the facility’s actual energy performance. |
| Savings calculation | An engineering calculation is performed for the energy consumed during the baseline period and the reporting period based on: - Ongoing or short-term measurements of the main performance parameter(s), - And estimated values. | Ongoing or short-term measurements of the energy consumed during the baseline period and the reporting period | An analysis of data on the energy consumed during the baseline period and the reporting period for the whole facility. Routine adjustments are required, using techniques such as simple comparison or regression analysis. | Energy use simulation, calibrated with hourly or monthly utility billing data |
| When to use option | The results obtained using this option are rather equivocal given that some parameters are estimated. However, this option is a much less expensive method than Option B. | Option B is more expensive than Option A, as all parameters are measured. It is the better option, however, for customers who require a high level of accuracy. | For complex energy management programs affecting many systems within a facility, Option C supports savings and helps to simplify the processes involved. | Option D is only used when there is no baseline data available. This may be the case where a site did not have a meter before the solution was implemented or where acquiring baseline data would involve too much time or expense |

# Appendix C – References

National Action Plan for Energy Efficiency (2007). Model Energy Efficiency Program Impact Evaluation Guide. Prepared by Steven R. Schiller, Schiller Consulting, Inc

EVO. International Performance Measurement and Verification Protocol: Concepts and Options for Determining Energy and Water Savings Volume 1. September 2009. 21-38. See also EVO / Cowain and Sain. M&V Fundamentals and the International Performance Measurement and Verification Protocol (IPMVP) For Energy Managers and Emission Traders. The Association of Energy Engineers. N.d.

For further reference regarding error mitigation, see NAPEE. Model Energy Efficiency Program Impact Evaluation Guide. November 2007. D-5 – D-11. Also EVO. International Performance Measurement and Verification Protocol: Concepts and Options for Determining Energy and Water Savings, Volume 1. September 2009. 46-47, 85-104.

EVO. International Performance Measurement and Verification Protocol: Concepts and Options for Determining Energy and Water Savings Volume 1. September 2009. 85.

# Additional Thoughts – Parking Lot

The following text was moved from earlier versions of the document. Some of this should perhaps be reinserted. If anything is to be reinserted, consider if it is relevant to one the hierarchy documents or if it belongs in one of the cross-cutting sections.

- Establish performance, service quality, existing standards, and weather, if applicable.

- Analyze extant use and demand

- Estimate projected costs and energy savings, particularly if cost-effectiveness is a program criteria or requirement.

- Measure efficiency should include a description of how Energy Efficiency measure eligibility is defined by each program and whether the efficiency of measures is assumed or actual.

- Verification of measure installation and characteristics should include plans for verifying that measures are installed and operating as reported/assumed by Distribution Companies.

- Primary data collection activities frequently include onsite metering of equipment power usage, logging hours of use, phone or in-person surveys of program participants and non-participants, interviews with trade allies and/or Program Administrators.

- Primary data collection frequently involves sampling.  Sampling plans should include target confidence and precision levels, stratification methods, methods and assumptions (e.g., covariance) used to determine the required sample size.  Plans should discuss measures taken to minimize sources of sampling bias, including (a) construct validity; (b) sampling frame versus population; (c) selection bias (for a sample and for a census attempt where not all sites within the census received usable data); (d) non-response bias; (e) error in measuring variables; (f) sample homogeneity relative to project (external validity); (g) outlier data points; and (h) missing data.

- Metering plans should include a description of plans for meter/logger installation, retrievals, training, data processing and quality control.

- Survey instrument design should include a discussion of existing survey instruments (if any) that will be used and how they will be adapted for the Evaluation

- In many cases, primary data specifically applicable to a jurisdiction will not be available and ex ante values and algorithms will be necessary.  Where using ex ante values and algorithms, should specify technical reference manual and studies to be used and how/whether values and algorithms will be reviewed and tested for applicability to local conditions.

- Modeling should include purpose, type, data availability and reference other similar modeling efforts.

- Secondary data sources should include a description of types and sources of secondary data that might be used in the EvaluationEM&V.

- Data Validation Procedures

- Updating of Evaluation EM&V Results

- Evaluation EM&V Reporting and Communications

- Management Reporting

- Evaluation EM&V Schedule

- Select Gross Savings Evaluation EM&V Approaches

- Draw Measurement Boundary - The boundary or scope of project or program measurement must be established.  Measurement may be done on discrete equipment installed or on buildings or facilities or on portions of buildings or facilities

- Define Methodology and Performance Metrics - The metrics will be used to Measure and Verify program performance, including data collection approaches.  Approaches may include direct measurement of savings, deemed savings, energy bill analysis, various types of self-reporting, surveys, or economic modeling

- Deemed Savings

- Comparison Groups

- Execute Program

- Install monitoring infrastructure, if applicable, at onset of program implementation.

  - Such infrastructure should include both physical and technological infrastructure (e.g. data loggers and other monitoring equipment) as well as the human and knowledge capital to operate and maintain a program monitoring regime.

  - Depending on the structure and type of program being implemented, monitoring infrastructure may need to be installed before the program begins, or it may simply be more efficient to do so.

- Calculate Gross Savings. Gross savings are those directly attributable to the program itself, exclusive of co-benefits and ignoring issues of free-ridership, etc.  Including, if applicable:

  - Annual average demand savings

  - Peak demand reductions

  - Coincident peak demand reduction

  - Demand response peak demand reduction

- Calculate net savings, if applicable (though out of the scope of this process document, some notes on net savings are included at the end see REQ.19.3.4.44)

- Calculate uncertainty and evaluate error

- Uncertainty should be reviewed in instrumentation, modeling, sampling, interactive effects, and estimation.

- Systematic Error: the extent to which results are biased by systemic issues to over- or underreport results.

- Random Error: sampling error resulting from the fact that only a portion of total participants may have been included in measurement.

- Evaluate Program Based on EM&V Findings - Compare objectives and actual, including EM&V objectives vs. actual

- Report Evaluation EM&V Results

- Evaluate and Respond to Feedback

- Identify possible program and operational improvements for future implementation, if applicable - Coordinate and collaborate with the Program Administrators to implement program improvements, if applicable

- Evaluate Savings Persistence, If Within Project Scope

- Savings persistence is determined by evaluating energy savings over a period of time after implementation, relative to savings determined initially. This would normally require repeated EM&V activities over some periodic interval, e.g. annually. Methodology for persistence could mirror original project EM&V, either exactly or based on smaller representative sampling. Savings persistence is measured as a percentage of first year savings.

- Ongoing commissioning-type activities could be undertaken to maintain program savings, adapted to evaluation of savings persistence EM&V.

- About EM&V principles under EM&V framework - I don’t see anything about program objectives and other contextual info that could guide evaluators – e.g, “Legislature established EERS, Commission concerned about cost program effectiveness, particularly the impact on ratepayers” of “Utilities have no experience with EE programs.”

- About EM&V principles under EM&V framework (Energy Numbers) - Since we are focused on electric, not sure what this is.  Is this asking for clarity about what other energy resources will be in evaluation of scope (e.g., gas, fuel oil)?  Or is this a waste heat factor?  If latter, I think it should move down in this list and t raises question whether we should include additional similar issues here – e.g., in service rates, waste heat factors (cooling bonus and/or heating penalty), partial use factors (a big issue for appliance retirement programs).  By the way, I am not saying all these issues should addressed in the framework, just that the framework should say that these issues should be addressed in the "Annual Portfolio Plan."

- About EMV principles (cost data) - I would emphasize linkage to objectives, not necessarily parameters – could be data collection methods or sources that will be used.  Also, could be sensitivity cases as opposed to single NTGR for each program/measure.

Energy efficiency may be a wholesale product, such as capacity. Energy efficiency in retail markets may be from individual energy efficiency measures at the project level or a portfolio of projects that make up an energy efficiency program.

National Action Plan for Energy Efficiency Model Energy Efficiency Program Impact Evaluation Guide (add full reference).