# 1.  RECOMMENDED ACTION:	EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

| X | Accept as requested | X | Change to Existing Practice |
| --- | --- | --- | --- |
|  | Accept as modified below |  | Status Quo |
|  | Decline |  |  |

2.  TYPE OF DEVELOPMENT/MAINTENANCE

| Per Request: |  | Per Recommendation: |  |
| --- | --- | --- | --- |
| X | Initiation | X | Initiation |
|  | Modification |  | Modification |
|  | Interpretation |  | Interpretation |
|  | Withdrawal |  | Withdrawal |
| X | Principle | X | Principle |
| X | Definition | X | Definition |
| X | Business Practice Standard | X | Business Practice Standard |
|  | Document |  | Document |
|  | Data Element |  | Data Element |
|  | Code Value |  | Code Value |
|  | X12 Implementation Guide |  | X12 Implementation Guide |
|  | Business Process Documentation |  | Business Process Documentation |

# 3.  RECOMMENDATION

# SUMMARY:

The UCAIug OpenADE Task Force submitted a request for the initiation of NAESB Model Business Practices on July 29, 2010 (R10008) to standardize the interface which allows for the exchange of Energy Usage Information between designated parties. The UCAIug OpenADE Task Force provided the artifacts on which these Model Business Practices were based.

These Model Business Practices will build on the NAESB Energy Usage Information Model and, subject to the Governing Documents and Applicable Regulatory Authority, will help enable Retail Customers to share energy usage information with Third Parties who have acquired the right to act in this role. This Energy Services Provider Interface will provide a consistent method for Retail Customers to authorize a Third Party to gain access to energy usage data.  Doing so will help enable Retail Customers to choose Third Party products to assist them to better understand their energy usage and to make more economical decisions about their usage.  This Energy Services Provider Interface will contribute to the development of an open and interoperable method for Third Party authorization and machine-to-machine exchange of Retail Customer usage information.

# Recommended Standards:

# REQ.21	ENERGY SERVICES PROVIDER INTERFACE

## EXECUTIVE SUMMARY

This document establishes the Model Business Practices for the Energy Services Provider Interface.  For Retail Customers to better realize the benefits of the Smart Grid, Retail Customer related data (e.g. usage information, etc.) should be made available in a timely manner to the Retail Customer and to the Authorized Third Parties chosen by the Retail Customer.

The Energy Services Provider Interface encompasses a variety of interactions between Retail Customers, Distribution Companies, and Third Parties. In a business environment where best practices are voluntary, Model Business Practices should be applied within the context of regulatory requirements and agreements. These Model Business Practices define an Energy Services Provider Interface, but any obligation to use it would be established by Governing Documents and Applicable Regulatory Authority rules and regulations not these Model Business Practices

## INTRODUCTION

The North American Energy Standards Board (NAESB) is a voluntary non-profit organization comprised of members from all aspects of the natural gas and electric industries. Within NAESB, the Retail Electric Quadrant (REQ) and the Retail Gas Quadrant (RGQ) focus on issues impacting the retail sale of energy to Retail Customers. REQ / RGQ Model Business Practices are intended to provide guidance to Distribution Companies, and other Market Participants involved in providing energy service to Retail Customers. The focus of these Model Business Practices is the Energy Service Provider Interface.

These Model Business Practices are voluntary and do not address policy issues that are the subject of state legislation or regulatory decisions. These Model Business Practices have been adopted with the realization that as the industry evolves, additional and amended Model Business Practices may be necessary. Any industry participant seeking additional or amended Model Business Practices (including principles, definitions, data elements, process descriptions, and technical implementation instructions) should submit a request to the NAESB office, detailing the change, so that the appropriate process may take place to amend the Model Business Practice.

## BUSINESS PROCESSES AND PRACTICES

## Overview

## REQ.21.1 Principles

REQ.21.1.1	The processes for ESPI should minimize the complexity associated with authorizing Third Parties to access Retail Customers energy usage data.

REQ.21.1.2	The processes associated with the ESPI should be consistent with any related requirements established by the Governing Documents and Applicable Regulatory Authority.

## REQ.21.2 Definitions

### REQ.21.2.A Business Definitions [Definitions identified with an asterisk are works in process to be modified based on the actors identified in the use cases]

TASK FORCE NOTE:  The REQ working glossary can be found through the following link: http://www.naesb.org/pdf3/req_rgq_glossary_defined_terms.xls

Terms used from the glossary

Applicable Regulatory Authority: The state regulatory agency or other local governing body that provides oversight, policy guidance, and direction to any parties involved in the process of providing energy to Retail Customers through regulations and orders.

Distribution Company: A regulated Entity which provides distribution services and may provide energy and/or transmission/transportation services in a given area.

Enrollment: The process of initiating a Retail Customer’s participation in specific energy-related products and services offered by a Market Participant.  In competitive energy markets it includes designating a Market Participant that has the responsibility for providing energy to a Retail Customer.

Entity: A person or organization with sufficient legal standing to enter into a contract or arrangement with another such person or organization (as such legal standing may be determined by those parties) for the purpose of conducting and/or coordinating energy transactions.

Governing Documents: Documents that determine the interactions among parties, including but not limited to: applicable law, regulatory documents (e.g., tariffs, rules, regulations), contractual agreements, Distribution Company Operational Manuals, and other relevant models and operational procedures.

Retail Customer: Any Entity that takes gas and/or electric service for its own consumption.

### REQ.21.2.B Technical Definitions

### [Definitions identified with an asterisk are works in process to be modified based on the actors identified in the use cases]

REQ.21.2.B.x	Authorizing Entity: An Entity (e.g. PUC, Distribution Company, bonding agent, etc.) who approves a Third Party to utilize the Energy Services Provider Interface. (return to this and add specifics)

REQ.21.2.B.x	Authorized Third Party:  A Third Party that has been approved by the Authorizing Entity to utilize the Energy Services Provider Interface.  Third Party must provide proof that they meet the governing documents’ requirements for data security and data privacy protection in order to become an Authorized Third Party..

*REQ.21.2.B.x	Energy Service Provider Interface:  A standardized machine-to machine interface that permits a Data Custodian to share, at the Retail Customer’s request and under the Retail Customer’s direction, a broad set of that Retail Customer’s Data Custodian data with Authorized Third Parties.

REQ.21.2.B.x 	Personally Identifiable Information: any information about an individual maintained by an agency, including (1) any information that can be used to distinguish or trace an individual‘s identity, such as name, social security number, date and place of birth, mother‘s maiden name, or biometric records; and (2) any other information that is linked or linkable to an individual, such as medical, educational, financial, and employment information.

REQ.21.2.B.x

Data Custodian:  A Data Custodian holds Retail Customer resource information and will share this information with Third Parties only in accordance with the Governing Documents, Applicable Regulatory Authority and the direction of the Retail Customer. A Data Custodian typically has direct access to the pertinent information (e.g., by directly acquiring electricity usage data from a meter).  A Data Custodian may be a Distribution Company.

REQ.21.2.B.x 	Third Party:  An Entity which provides some service to a Retail Customer based on information it does not have direct access to or direct authority over.  A Third Party relies on a Data Custodian to provide access to Retail Customer information.

*REQ.21.2.B.x	EnergyUsage Information: Any measurement of energy  consumption by a Retail Customer.

### REQ.21.2.C	Acronyms

| Abbreviation / Acronym | Meaning |
| --- | --- |
| ADE | Automatic Data Exchange |
| ESPI | Energy Services Provider Interface |
| EUI | Energy Usage Information |
| NISTIR | National Institute of Standards and Technology Interagency Report |
| PII | PII |

## REQ.21.3	Model Business Practices

### REQ.21.3.1	General Practices for Energy Services Provider Interface (ESPI)

REQ.21.3.1.1 	To the extent required by the Applicable Regulatory Authority, Authorized Third Parties and Data Custodians should exchange Retail Customer’s energy usage information at the Retail Customer’s request pursuant to the requirements as set forth in NAESB REQ.21, subject to the Governing Documents.

REQ.21.3.1.2	The ESPI relationship requires a set of agreements between Retail Customer-Authorized Third Party, Retail Customer-Data Custodian, and Authorized Third Party-Data Custodian to ensure that the appropriate information is provided as needed and other information access is restricted.

REQ.21.3.1.3	A Third Party cannot access PII from a Data Custodian.  PII may only be provided to a Third Party by the Retail Customer.

REQ.21.3.1.4	Subject to the Governing Documents and Applicable Regulatory Authority, the ESPI should enable a Retail Customer to share energy usage information for such Retail Customer with Authorized Third Parties who have acquired the right to act in this role.

REQ.21.1.5	The ESPI should allow exchange of usage information without requiring access to PII.

REQ.21.1.6	All information exchanged by the ESPI should be secure in accordance with the security requirements stated herein.  These may be further constrained by Governing Documents and Applicable Regulatory Authority.

REQ.21.1.7	A Retail Customer should have the ability to authorize the Data Custodian to release energy usage information for such Retail Customer to an Authorized Third Party who has acquired the right to act in this role, subject to the Governing Documents and Applicable Regulatory Authority.

REQ.21.1.8	Subject to the Governing Documents and Applicable Regulatory Authority, a Retail Customer should have the ability to authorize multiple Authorized Third Parties to have limited time based read only access to specified EUI or other types of information for such Retail Customer, with any default expiration for such access established by such Governing Documents or Applicable Regulatory Authority.

REQ.21.1.9 	Subject to the Governing Documents and Applicable Regulatory Authority, a Retail Customer should have the ability to designate a specific expiration date, extend any specific expiration date, or indicate an open-ended access timeframe other than the default access period.

REQ.21.1.10	The ESPI should have the capability to support the Retail Customers’ ability to select / revoke which Authorized Third Parties are authorized for read-only access to EUI.

REQ.21.1.11	The ESPI should have the capability to notify the relevant Authorized Third Parties, Data Custodian and Retail Customers when access has been granted, access has been changed, or access has been revoked for a Service Delivery Point.

REQ.21.1.12	Subject to the Governing Documents and Applicable Regulatory Authority, the ESPIshould be consistent with the applicable guidelines around security and authorization for Third Party data access as set forth in the NISTIR 7628.

REQ.21.1.13	The ESPI should provide the ability for any party to terminate any information sharing relationship in which that party is involved.

REQ.21.1.14	Future versions of the ESPI should be backwards compatible, including provisions for exchanging versioning information and negotiating interface capabilities.

REQ.21.1.16	Any Third Party wishing to access energy usage information via the ESPI must establish and maintain a trusted relationship with each Data Custodian who provides a ESPI. Subject to the Governing Documents and Applicable Regulatory Authority, both the Data Custodian and the Authorized Third Party should disallow requests from Entities who are not Authorized Third Parties.

REQ.21.1.17	Subject to the Governing Documents and Applicable Regulatory Authority during communications of any information confidentiality will be maintained.

REQ.21.1.18	Subject to the Governing Documents and Applicable Regulatory Authority, Third Parties must be authorized by the Authorizing Entity and/or the Data Custodian to be an Authorized Third Party and utilize the ESPI and must maintain their status as an Authorized Third Party.

REQ.21.1.19	If an Authorizing Entity exists within a jurisdiction, Retail Customers should be able to view a list of Third Parties who have been authorized to use the ESPI.

REQ.21.1.20	Subject to the Governing Documents and Applicable Regulatory Authority, energy usage information (EUI) should be made available to Authorized Third Parties (as directed by the Retail Customer) in a reasonable and timely fashion.

REQ.21.1.21	When any of the required authorized relationships described in this recommendation are terminated, access to the ESPI shall not be granted.

REQ.21.1.22	It is necessary in this model business practice to identify participants and their relationships with globally unique identifiers.  The identifiers should conform to IETF RFC 4122.

REQ.21.1.23	The creation and dissolution of trusted relationships between any two parties are outside the scope of this Model Business Practice and are necessary preconditions for the use of the ESPI.

REQ.21.1.24	Upon dissolution of any of the required trusted relationships any ESPI relationships must be terminated and parties notified.

REQ.21.1.25	If and when the relationships or criteria change all effected parties should be notified.

REQ.21.1.26	The most interoperable and widely supported technologies should be used to ensure adoption regardless of development and deployment platforms used

REQ.21.1.27	The technologies chosen should be well specified, with active communities and tools and/or frameworks available.

REQ.21.1.28	Technologies chosen should be compatible and interoperable with technologies specified for access to HAN resources.

## REQ.21.4	Models

### REQ.21.4.1		Profile of REQ.18 Energy Usage Information Model

The following model represents the NAESB PAP10 EUI model, with extensions and restrictions aligned with SEP 2.0, further restricted and extended to target the required ESPI elements.

[Additional NAESB extensions may be added to allow bulk transfer using Web Services without requiring hrefs.]

[Additional work will be needed to harmonize this model further with both SEP 2.0 and IEC CIM.]

Figure 1: ESPI Usage

Figure 2: ESPI Types

IntervalBlockListLink

A Link to a List of IntervalReading instances.

| Name | Type | Description |
| --- | --- | --- |
| href | anyURI | A URI reference. |

MeterReadingListLink

A Link to a List of MeterReading instances.

| Name | Type | Description |
| --- | --- | --- |
| href | anyURI | A URI reference. |

ReadingTypeLink

A Link to an instance of ReadingType.

| Name | Type | Description |
| --- | --- | --- |
| href | anyURI | A URI reference. |

IntervalBlockList

A List element to hold IntervalBlock objects.

| Name | Type | Description |
| --- | --- | --- |
| href | anyURI | A reference to the resource address (URI). Required in return from GET, ignored otherwise. |

MeterReadingList

A List element to hold MeterReading objects.

| Name | Type | Description |
| --- | --- | --- |
| href | anyURI | A reference to the resource address (URI). Required in return from GET, ignored otherwise. |

UsagePointList

A List element to hold UsagePoint objects.

| Name | Type | Description |
| --- | --- | --- |
| href | anyURI | A reference to the resource address (URI). Required in return from GET, ignored otherwise. |

ServiceCategory

Category of service provided to the customer.

| Name | Type | Description |
| --- | --- | --- |
| kind | ServiceKind | Service kind |

UsagePoint

Logical point on a network at which consumption or production is either physically measured (e.g. metered) or estimated (e.g. unmetered street lights).

| Name | Type | Description |
| --- | --- | --- |
| href | anyURI | A reference to the resource address (URI). Required in return from GET, ignored otherwise. |
| mRID | HexBinary128 | A Model Authority issues mRIDs. Given that each Model Authority has a unique id and this id is part of the mRID, then the mRID is globally unique. For ESPI (and SEP 2.0), the IANA PEN provider ID shall be specified in the first 32 bits, and objects created by that provider shall be assigned unique IDs with the remaining 96 bits. |
| status | UInt8 | Specifies the current status of this usage point. 0 = off 1 = on |
| description | String32 | The description is a human readable text describing or naming the object. |

HexBinary128  «XSDsimpleType»

A 128-bit field encoded as a hex string (32 characters / 16 octets)

String32  «XSDsimpleType»

Character string of max length 32

UInt32  «XSDsimpleType»

Unsigned integer, max inclusive 4294967296 (2^32)

UInt48  «XSDsimpleType»

Unsigned integer, max inclusive 281474976710656 (2^48)

UInt8  «XSDsimpleType»

Unsigned integer, max inclusive 256 (2^8)

AccumulationBehaviourType

0 = Not Applicable

1 = BulkQuantity

3 = Cumulative

4 = DeltaData

6 = Indicating

9 = Summation

12 = Instantaneous

CommodityType

0 = Not Applicable

1 = Electricity secondary metered value (a premise meter is typically a secondary meter)

2 = Electricity primary metered value

4 = Air

7 = NaturalGas

8 = Propane

9 = PotableWater

10 = Steam

11 = WasteWater

12 = HeatingFluid

13 = CoolingFluid

ConsumptionTierType

0 = Not Applicable

1 = Block Tier 1

2 = Block Tier 2

3 = Block Tier 3

4 = Block Tier 4

5 = Block Tier 5

6 = Block Tier 6

7 = Block Tier 7

8 = Block Tier 8

9 = Block Tier 9

10 = Block Tier 10

11 = Block Tier 11

12 = Block Tier 12

13 = Block Tier 13

14 = Block Tier 14

15 = Block Tier 15

16 = Block Tier 16

CurrencyCode

Follows codes defined in ISO 4217. Full list at tiny.cc/4217.

0 - Not Applicable

36 - Australian Dollar

124 - Canadian Dollar

840 - US Dollar

978 - Euro

DataQualifierType

0 = Not Applicable

2 = Average

8 = Maximum

9 = Minimum

12 = Normal

? = Standard Deviation

DateTimeInterval  «Compound»

Interval of date and time.

| Name | Type | Description |
| --- | --- | --- |
| start | TimeType | Date and time that this interval started. |
| duration | UInt32 | Duration of the interval, in seconds, to be used when start or end are not specified. |

FlowDirectionType

0 = Not Applicable

1 = Forward

19 = Reverse

KindType

0 = Not Applicable

3 = Currency

8 = Demand

12 = Energy

37 = Power

PhaseCode

0 = Not Applicable

129 = Phase AN

128 = Phase A

132 = Phase AB

64 = Phase BN

64 = Phase B

32 = Phase CN

32 = Phase C

224 = Phase ABC

66 = Phase BC

40 = Phase CA

PowerOfTenMultiplierType  «XSDsimpleType»

0 = None

1 = deca=x10

2 = hecto=x100

-3 = mili=x10-3

3 = kilo=x1000

6 = Mega=x106

-6 = micro=x10-3

9 = Giga=x109

ServiceKind

Service kind

0 - electricity

1 - gas

2 - water

4 - pressure

5 - heat

6 - cold

7 - communication

8 - time

TOUType

0 = NotApplicable

1 = TOU A

2 = TOU B

3 = TOU C

4 = TOU D

5 = TOU E

6 = TOU F

7 = TOU G

8 = TOU H

9 = TOU I

10 = TOU J

11 = TOU K

12 = TOU L

13 = TOU M

14 = TOU N

15 = TOU O

TimeAttributeType

0 = Not Applicable

1 = 10-minute

2 = 15-minute

4 = 24-hour

5 = 30-minute

7 = 60-minute

11 = Daily

13 = Monthly

15 = Present

16 = Previous

24 = Weekly

32 = ForTheSpecifiedPeriod

79 = Daily30minuteFixedBlock

TimeType  «XSDsimpleType»

Time is an signed 64 bit value representing the number of seconds since 0 hours, 0 minutes, 0 seconds, on the 1st of January, 1970.

UomType

0 = Not Applicable

5 = A (Current)

29 = Voltage

31 = J (Energy joule)

33 = Hz (Frequency)

38 = Real power (Watts)

42 = m3 (Cubic Meter)

61 = VA (Apparent power)

63 = VAr (Reactive power)

65 = Cos? (Power factor)

67 = V² (Volts squared)

69 = A² (Amp squared)

71 = VAh (Apparent energy)

72 = Real energy (Watt-hours)

73 = VArh (Reactive energy)

106 = Ah (Ampere-hours / Available Charge)

119 = ft3 (Cubic Feet)

122 = ft3/h (Cubic Feet per Hour)

125 = m3/h (Cubic Meter per Hour)

128 = US gl (US Gallons)

129 = US gl/h (US Gallons per Hour)

130 = IMP gl (Imperial Gallons)

131 = IMP gl/h (Imperial Gallons per Hour)

132 = BTU

133 = BTU/h

134 = Liter

137 = L/h (Liters per Hour)

140 = PA(gauge)

155 = PA(absolute)

169 = Therm

IntervalBlock

Time sequence of Readings of the same ReadingType. Contained IntervalReadings may need conversion through the application of an offset and a scalar defined in associated Pending.

| Name | Type | Description |
| --- | --- | --- |
| href | anyURI | A reference to the resource address (URI). Required in return from GET, ignored otherwise. |
| interval | DateTimeInterval | Specifies the time range during which the contained readings were taken. |
| mRID | HexBinary128 | A Model Authority issues mRIDs. Given that each Model Authority has a unique id and this id is part of the mRID, then the mRID is globally unique. For ESPI (and SEP 2.0), the IANA PEN provider ID shall be specified in the first 32 bits, and objects created by that provider shall be assigned unique IDs with the remaining 96 bits. |
| description | String32 | The description is a human readable text describing or naming the object. |

MeterReading

Set of values obtained from the meter.

| Name | Type | Description |
| --- | --- | --- |
| href | anyURI | A reference to the resource address (URI). Required in return from GET, ignored otherwise. |
| mRID | HexBinary128 | A Model Authority issues mRIDs. Given that each Model Authority has a unique id and this id is part of the mRID, then the mRID is globally unique. For ESPI (and SEP 2.0), the IANA PEN provider ID shall be specified in the first 32 bits, and objects created by that provider shall be assigned unique IDs with the remaining 96 bits. |
| description | String32 | The description is a human readable text describing or naming the object. |

QualityOfReading

List of codes indicating the quality of the reading, using specification:

<system> "." <categorization> "." <index>

where <system> is one of:

0 - not applicable

1 - end device

2 - metering collection system

3 - meter data management system

<categorization> "." <index> is one of:

0.0 - valid

0.1 - validated

7.0 - manually edited

8.0 - estimated

10.0 - questionable

11.0 - derived deterministically

11.1 - derived with inference

12.0 - projected

Reading

Specific value measured by a meter or other asset. Each Reading is associated with a specific ReadingType.

| Name | Type | Description |
| --- | --- | --- |
| cost | UInt48 | Specifies a cost associated with this reading, in millionths of the currency specified in the ReadingType for this reading. (e.g. 840 = USD, US dollar) |
| timePeriod | DateTimeInterval | The date time and duration of a reading. If not specified, readings for each "intervalLength" in ReadingType are present. |
| value | UInt48 | Value in units specified by ReadingType |

ReadingQuality

Quality of a specific reading value or interval reading value. Note that more than one Quality may be applicable to a given Reading. Typically not used unless problems or unusual conditions occur (i.e., quality for each Reading is assumed to be 'Good' (valid) unless stated otherwise in associated ReadingQuality).

| Name | Type | Description |
| --- | --- | --- |
| quality | QualityOfReading | Quality, to be specified if different than 'raw'. |

ReadingType

Type of data conveyed by a specific Reading.

| Name | Type | Description |
| --- | --- | --- |
| href | anyURI | A reference to the resource address (URI). Required in return from GET, ignored otherwise. |
| mRID | HexBinary128 | A Model Authority issues mRIDs. Given that each Model Authority has a unique id and this id is part of the mRID, then the mRID is globally unique. For ESPI (and SEP 2.0), the IANA PEN provider ID shall be specified in the first 32 bits, and objects created by that provider shall be assigned unique IDs with the remaining 96 bits. |
| description | String32 | The description is a human readable text describing or naming the object. |
| accumulationBehaviour | AccumulationBehaviourType | The “accumulation behaviour” indicates how the value is represented to accumulate over time. |
| commodity | CommodityType | Commodity |
| consumptionTier | ConsumptionTierType | Consumption tier |
| currency | CurrencyCode | The currency code indicating the currency for monetary values (kind="3"). |
| dataQualifier | DataQualifierType | The data type can be used to describe a salient attribute of the data. Possible values are average, absolute, and etc. |
| defaultQuality | QualityOfReading | Characteristics of a data value conveyed by a specific Reading, which allow an application to understand how a specific Reading is to be interpreted. |
| flowDirection | FlowDirectionType | Anything involving current might have a flow direction. Possible values include forward and reverse. |
| kind | KindType | Compound class that contains kindCategory and kindIndex |
| intervalLength | UInt32 | Specifies the default interval length for this ReadingType, in seconds |
| phase | PhaseCode | Contains phase information associated with the type. |
| powerOfTenMultiplier | PowerOfTenMultiplierType | Power of ten multipliers |
| timeAttribute | TimeAttributeType | Time interval |
| tou | TOUType | TOU type |
| uom | UomType | Compound class that contains metricPrefix, uomCategory, and uomIndex |

### REQ.21.4.2		Additional Models in Support of Services

[These objects are presented in logical form currently, and are reflected in the Logical Information Models section in the Appendix, but will be modeled explicitly for the final specification.]

REQ.21.4.2.1	ServiceStatus

ServiceStatus is used to obtain the status of an information service. Attributes include the following.

- Current status (normal, unavailable)

REQ.21.4.2.2	Authorization

An Authorization is a Customer grant of 3rd Party access to specific resources. The attributes of this object are listed below.

- Information consumer (Third Party) identifier

- Resource subject 
Specifies the resource to which access from the information provider is granted by the customer to the information consumer

- Authentication Token
One or more token / secret pairs proving the identity of the requester to be the Customer associated with the resource. Different methods may be defined against which token / secret pairs are created and verified.

- Authorized period

- Status (requested, valid, invalid, error, unavailable)

## REQ.21.5	Related Model Business Practices

### REQ.21.5.1		Conformance to REQ.18 Energy Usage Information Model

The ESPI services conform to REQ.18 Energy Usage Information Model (PAP10), due to the ability to directly transform between the models without loss of precision, and without external inputs. The ESPI model uses the same concept names and structures as PAP10 EUI.

[ESPI model will be aligned with IEC CIM model and Part 9 (Meter Reading and Control) as well as Smart Energy Profile 2.0 as much as possible.]

## REQ.21.6	Technical Implementation

### REQ.21.6.1	Common

The following specifications apply to both Web Services and REST implementations.

#### REQ.21.6.1.1		Security

Providers of ESPI services, including both data custodian and third party, are responsible for protecting their systems, networks, and interface endpoints against threats, as recommended in NIST-IR and SECURITY PROFILE FOR THIRD PARTY DATA ACCESS.

#### REQ.21.6.1.1.1		Encryption

Establishment of mutually-authenticated encrypted channels shall be performed using HTTP/S, as documented in IETF RFC 2818, over which data may be securely transferred between data custodian and third party.

#### REQ.21.6.1.1.2		User Authorization

OAuth, as documented in IETF RFC 5849, shall be used for authorization grant and access by third parties to shared data custodian resources. This protocol results in access tokens that can be used to subscribe to specific user data, or to request it immediately, if supported.

### REQ.21.6.2	Web Services

#### REQ.21.6.2.1	Communication Specifications

#### REQ.21.6.2.2	Technical Considerations

#### REQ.21.6.2.3	Conformance

### REQ.21.6.3	REST

#### REQ.21.6.3.1	Communication Specifications

#### REQ.21.6.3.2	Technical Considerations

#### REQ.21.6.3.3	Conformance

# Appendices

This section contains informative descriptions and requirements derived by the committee for these service definitions.

# A. Overview

The scope of these Model Business Practices includes authorization by the Retail Customer and the Automatic Data Exchange of the energy usage information to the Authorized Third Party in accordance with parameters (e.g. term of access, type of data, quantity of data, etc.) determined by the Retail Customer subject to the Governing Documents and Applicable Regulatory Authority. Figure 3 shows the logical components involved in this authorization and data exchange process. Note that while the authorization process shown in this figure is made using a web browser, the services provided by the Authorized Third Party are not required to use a web browser to deliver such services.

Figure 3: Overview of Logical Components

# B. Use Cases

This section presents a superset of the use cases that are informative of the third party data access relationship. Alternative use cases are presented for certain activities that can be achieved in different ways, such as the delivery of shared resource information by push or by pull.

The concept of a Shared Resource Key is used throughout these use cases. A Shared Resource Key is a token used to uniquely identify an instance of a Third Party data access relationship (i.e., each Retail Customer-Data Custodian-Third Party combination for a particular resource will have a unique Shared Resource Key). A Shared Resource Key contains no PII regarding the Retail Customer and so can be freely shared among all three roles without unnecessary disclosure of sensitive information. Once the relationship is established, inclusion of a Shared Resource Key in an interaction is sufficient to identify a specific Third Party data access relationship.

Each use case contains the following sections:

- Use Case Description: This is a summary of the use case, describing the overallpurpose.

- Pre-Conditions: These are conditions that must be true for the use case to be successfully executed.

- Invariant Conditions: These are properties that will be true any time the use case is initiated, regardless of whether it terminates successfully.

- Post-Conditions: These are properties that will be true only if the use case terminates successfully. This requires that all preconditions and all condition checks (e.g., for validity of a request) be satisfied during execution of the use case.

- Basic Path Scenario: This defines the series of steps undertaken by each role during successful execution of the use case. The scenario is depicted graphically in a Unified Modeling Language (UML) sequencediagram and each step is summarized in text.

The following use cases are informative and not normative.

Figure 4: ESPI Use Case Diagram

1: Third Party Establishes Relationship With Data Custodian

Figure 5: Third Party Establishes Relationship With Data Custodian

Description

A Third Party service provider wants to register with a Data Custodian to provide services to Retail Customers with data stored at the Data Custodian.

Pre-Condition: Third Party had demonstrated that it meets security and privacy requirements as specified by governing documents.

Invariant Constraint: No resource data or personal data is provided to the Third Party by the Data Custodian as part of this interaction.

Post-Condition: A Shared Identity Key is generated to allow the Third Party to identify its identity to Data Custodian.

Post-Condition: The Third Party has permission to get specified resource data from the Data Custodian with permission of a Retail Customer.

Scenario: Basic Path

- The Third Party wishes to provide value added services to Retail Customers with data stored by the Data Custodian.

- Third Party requests that the Data Custodian establish relationship.

- Third Party provides proof that they meet the governing documents’ requirements for data security and privacy protection.

- Third Party provides description of the services it wishes provide for Retail Customers.

- The Data Custodian generates an Identity Key for the Third Party.  Third Party will use this key to identify itself during Use Cases 2 through 12.

- The Data Custodian adds the Third Party to its list of available services it presents to Retail Customers in Use Case 2.

- Third Party adds Data Custodian to its list of Data Custodians it presents in Use Case 2.

- Third Party persists the Identity Key.

- As needed, Third Party checks their ability to connect to the service, and obtains the current status of the service.

2: Retail Customer Authorizes Third Party Resource Access via Data Custodian

Figure 6: Retail Customer Authorizes Third Party Resource Access via Data Custodian

Description

A Retail Customer wants to grant permission for a Data Custodian to share a particular data resource with a Third Party. The Retail Customer initiates the process through the Data Custodian.

Pre-Condition: Retail Customer has established accounts with Data Custodian and Third Party.

Pre-Condition: Third Party has an established account with Data Custodian.

Pre-Condition: Data Custodian and Third Party have published and made Retail Customer aware of their privacy policy related to collection and handling of customer information.

Invariant Constraint: No resource data or personal data is provided to the Third Party by the Data Custodian as part of this interaction.

Post-Condition: A Shared Resource Key is generated to allow all three roles to refer to the same shared resource without disclosing PII. This key is known to all three roles.

Post-Condition: The Third Party has the Retail Customer's permission to get the specified resource data from the Data Custodian.

Post-Condition: The Data Custodian sends the Retail Customer confirmation of establishment of the Third Party data access relationship.

Scenario: Basic Path

- The Retail Customer decides to grant permission for the Data Custodian to share their resource data with the Third Party

- (Optional) Retail Customer finds their appropriate Data Custodian from Third Party, and navigates to the appropriate place to begin establishment of sharing relationship.

- Retail Customer requests that the Data Custodian establish a new data access relationship.

- Data Custodian presents the Retail Customer with a list of resources that can be shared with Third Parties. Any additional attributes (e.g., duration for which permission should be granted) that can be selected are also presented.

- Retail Customer selects a resource to share, sets any available attributes for the relationship, and specifies a Third Party that is known to the Data Custodian. Selecting these parameters and completing the interaction indicates permission for the Data Custodian to grant the specified Third Party access to the specified shared resource.

- The relationship will only be created if the Data Custodian accepts the selections for the Third Party (e.g., a Data Custodian may constrain access to certain resource attributes depending on resource sensitivity).

- Data Custodian generates a Shared Resource Key (Request Token) to begin creation of this relationship and provides it to the Third Party. Each Shared Resource Key is unique to the relationship between a Retail Customer, Data Custodian, Third Party, and specific data resource.

- Third Party requests authorization of the token by the Retail Customer, via the Data Custodian.

- Retail Customer authenticates with Data Custodian and authorizes the Request Token.

- Third Party exchanges the authorized Request Token for an Access Token from the Data Custodian.

- Third Party and Data Custodian persist the Authorization, associating it with its identity of the Retail Customer.

3: Retail Customer Modifies Resource Authorization

Figure 7: Retail Customer Modifies Resource Authorization

Description

The Retail Customer has an existing third party data access relationship with a particular Data Custodian and Third Party and wants to either extend or restrict the permissions associated with that relationship.

Pre-Condition: Retail Customer has established a Third Party data access relationship with the Data Custodian and the Third Party with respect to a particular resource, resulting in a unique Shared Resource Key identifying the relationship.

Invariant Constraint: No resource data or personal data is provided to the Third Party by the Data Custodian as part of this interaction.

Post-Condition: Future interactions between the Data Custodian and the Third Party with respect to the specified resource are governed by the modified permissions.

Post-Condition: The Third Party handles any data not allowed by the termination of the relationship in the manner specified in any service agreements among the parties in the relationship (e.g., all instances of the data in control of the Third Party are deleted within

Post-Condition: The Data Custodian sends the Retail Customer confirmation of modification of the permissions of the Third Party data access relationship.

Scenario: Basic Path

- Retail Customer chooses to modify relationship permissions with the Data Custodian.

- Data Custodian presents the Retail Customer with a list of resources that are shared with Third Parties. If the Retail Customer may only grant access to one resource, S2 and S3 may be skipped.

- Retail Customer chooses particular resource whose permissions he/she wishes to modify.

- Data Custodian provides available resource attributes and current settings to Retail Customer.

- Retail Customer chooses new settings.

- The new permissions governing the relationship will apply only if the Data Custodian accepts the selections for the Third Party (e.g., a Data Custodian may constrain access to certain resource attributes depending on resource sensitivity).

- Data Custodian persists the new permissions, which will be used from this point forward to govern the relationship (until further changed or the relationship is terminated).

- Data Custodian notifies Third Party that permissions have changed (identifying the resource by its Shared Resource Key). This notification may be immediate or deferred (e.g., as part of a resource push from Use Case 8, perhaps as part of a header).

- Data Custodian notifies Retail Customer that permissions have been changed.

- The Third Party handles any data not allowed by the modification of the resource authorization in the manner specified in any service agreements among the parties in the relationship.

4: Retail Customer Revokes Resource Authorization

Figure 8: Retail Customer Revokes Resource Authorization

Description

The Retail Customer has an existing third party data access relationship with a particular Data Custodian and Third Party and wants to terminate that relationship

Pre-Condition: Third Party has an established account with Data Custodian.

Pre-Condition: Retail Customer has established a Third Party data access relationship with the Data Custodian and the Third Party with respect to a particular resource, resulting in a unique Shared Resource Key identifying the relationship.

Invariant Constraint: No resource data or personal data is provided to the Third Party by the Data Custodian as part of this interaction.

Post-Condition: Both the Third Party and the Data Custodian delete the Shared Resource Key for the relationship and no future interactions are permitted for that relationship.

Post-Condition: The Third Party handles any data not allowed by the termination of the relationship in the manner specified in any service agreements among the parties in the relationship (e.g., all instances of the data in control of the Third Party are deleted within

Post-Condition: The Data Custodian sends the Retail Customer confirmation of termination of the Third Party data access relationship.

Scenario: Basic Path

- Retail Customer requests that Data Custodian terminate the data access relationship.

- Data Custodian presents the Retail Customer with a list of resources for which there are valid relationships with Third Parties. If the Retail Customer only has one valid relationship, S2 and S3 may be skipped.

- Retail Customer chooses a resource whose relationship is to be terminated.

- Data Custodian terminates the relationship, deleting the appropriate Shared Resource Key from its list of valid relationships.

- Data Custodian notifies Third Party that the relationship has been terminated (identifying the relationship by its Shared Resource Key).

- Data Custodian notifies Retail Customer that the relationship has been terminated.

- The Third Party handles any data not allowed by the termination of the relationship, in the manner specified in any service agreements among the parties in the relationship.

5: Data Custodian Revokes Resource Authorization

Figure 9: Data Custodian Revokes Resource Authorization

Description

The Retail Customer has an existing third party data access relationship with a particular Data Custodian and Third Party. The Data Custodian wants to terminate the relationship (for whatever reason).

Pre-Condition: Third Party has an established account with Data Custodian.

Pre-Condition: Retail Customer has established a Third Party data access relationship with the Data Custodian and the Third Party with respect to a particular resource, resulting in a unique Shared Resource Key identifying the relationship.

Invariant Constraint: No resource data or personal data is provided to the Third Party by the Data Custodian as part of this interaction.

Post-Condition: Both the Third Party and the Data Custodian delete the Shared Resource Key for the relationship and no future interactions are permitted for that relationship.

Post-Condition: The Third Party handles any data not allowed by the termination of the relationship, in the manner specified in any service agreements among the parties in the relationship (e.g., all instances of the data in control of the Third Party are deleted within

Post-Condition: The Data Custodian sends the Retail Customer notification of termination of the Third Party data access relationship.

Scenario: Basic Path

- Data Custodian decides to terminate relationship with Third Party.

- Data Custodian notifies Retail Customer of termination decision; no acknowledgement or confirmation is required.

- Data Custodian notifies Third Party of termination of the relationship, identifying the relationship by a Shared Resource Key.

- The Third Party handles any data not allowed by the termination of the relationship, in the manner specified in any service agreements among the parties in the relationship.

6: Third Party Terminates Relationship

Figure 10: Third Party Terminates Relationship

Description

The Retail Customer has an existing third party data access relationship with a particular Data Custodian and Third Party. The Third Party determines that it no longer wants to provide services to the Retail Customer and terminates the relationship.

Pre-Condition: Third Party has an established account with Data Custodian.

Pre-Condition: Retail Customer has established a Third Party data access relationship with the Data Custodian and the Third Party with respect to a particular resource, resulting in a unique Shared Resource Key identifying the relationship.

Invariant Constraint: No resource data or personal data is provided to the Third Party by the Data Custodian as part of this interaction.

Post-Condition: Both the Third Party and the Data Custodian delete the Shared Resource Key for the relationship and no future interactions are permitted for that relationship.

Post-Condition: The Third Party handles any data not allowed by the termination of the relationship, in the manner specified in any service agreements among the parties in the relationship (e.g., all instances of the data in control of the Third Party are deleted within

Post-Condition: The Data Custodian sends the Retail Customer notification of termination of the Third Party data access relationship.

Scenario: Basic Path

- Third Party decides to terminate a third party data access relationship.

- Third Party notifies Data Custodian of termination of relationship, identifying the relationship by the Shared Resource Key.

- An invalid request (e.g., specification of a Shared Resource Key not associated with the Third Party) will not be accepted.

- Data Custodian deletes Shared Resource Key, terminating the relationship.

- Data Custodian notifies the Retail Customer of termination of the relationship. No acknowledgement or confirmation is required.

- The Third Party handles any data not allowed by the termination of the relationship, in the manner specified in any service agreements among the parties in the relationship.

7: Third Party Establishes Subscription with Data Custodian - Asynchronous

Figure 11: Third Party Establishes Subscription with Data Custodian - Asynchronous

Description

The Retail Customer has an existing third party data access relationship with a particular Data Custodian and Third Party. The Third Party establishes a subscription indicating the circumstances (i.e., an agreed-upon schedule and/or specification of special events) under which the Data Custodian should provide the Third Party with the relevant resource data.

Depending on the services offered by a Data Custodian, the subscription may indicate the circumstances under which the Data Custodian will send resource data or only notification that resource data is available (i.e., whether the Data Custodian supports a push or pull model). Subscriptions may be parameterized, if supported by the Data Custodian, to define preferred delivery criteria (e.g., new data whenever available or only once per day).

Pre-Condition: Third Party has an established account with Data Custodian.

Pre-Condition: Retail Customer has established a Third Party data access relationship with the Data Custodian and the Third Party with respect to a particular resource, resulting in a unique Shared Resource Key identifying the relationship.

Invariant Constraint: No resource data or personal data is provided to the Third Party by the Data Custodian as part of this interaction.

Post-Condition: The Data Custodian records a valid subscription on behalf of the Third Party. Future data availability triggers satisfying the subscription will result in the appropriate information being sent to the Third Party.

Post-Condition: Data Custodian sends the Third Party confirmation of its subscription request

Post-Condition: Data Custodian sends the Retail Customer notification of the Third Party's subscription request

Scenario: Basic Path

- Third Party requests that the Data Custodian establish a new subscription.

- Third Party provides Data Custodian with information defining the subscription request. At a minimum, this information includes a Shared Resource Key identifying the resource whose data is to be shared. The information may include additional subscription parameters, as supported by the Data Custodian.

- The subscription will not be accepted if the Shared Resource Key is invalid.

- The Data Custodian saves the subscription information, associating the subscription with the Shared Resource Key and the Third Party.

- The Data Custodian notifies the Third Party that the subscription request was successful. No acknowledgement or confirmation is required.

- The Data Custodian notifies the Retail Customer that the Third Party has completed a subscription for their resource data. No confirmation is required, as the Third Party already has permissions as indicated by the valid Shared Resource Key. If the subscription is not acceptable to the Retail Customer, Use Case 3 can be exercised to modify permissions for the Third Party.

8: Third Party Requests Data from Data Custodian - Asynchronous

Figure 12: Third Party Requests Data from Data Custodian - Asynchronous

Description

The Retail Customer has an existing third party data access relationship with a particular Data Custodian and Third Party. The Third Party requests specific resource data to be delivered with next transfer.

Pre-Condition: Third Party has an established account with Data Custodian

Pre-Condition: Retail Customer has established a Third Party data access relationship with the Data Custodian and the Third Party with respect to a particular resource, resulting in a unique Shared Resource Key identifying the relationship

Invariant Constraint: No resource data or personal data is provided to the Third Party by the Data Custodian as part of this interaction.

Post-Condition: The Data Custodian records the request on behalf of the Third Party. Future data availability triggers will result in the appropriate information being sent to the Third Party.

Post-Condition: Data Custodian sends the Third Party confirmation of its data request.

Scenario: Basic Path

- Third Party decides to request resource data from the Data Custodian.

- Data Custodian Checks validity of request.

- Data Custodian queues request for next asynchronous transfer.

- Data Custodian sends confirmation to Third Party.

9: Data Custodian Sends (Pushes) Data to Third Party - Asynchronous

Figure 13: Data Custodian Sends (Pushes) Data to Third Party - Asynchronous

Description

The Retail Customer has an existing third party data access relationship with a particular Data Custodian and Third Party. The Third Party has established a subscription for receiving the relevant resource data from the Data Custodian. This resource data is sent (pushed) to the subscribed Third Party by the Data Custodian when an event triggers indicates a need to push new resource data.

Conditions observable to the Data Custodian change, causing a data availability trigger to be checked to see if there is a need to push resource data to the Third Party. Such triggers can be caused by any of the following observable changes

- New resource data is received by the Data Custodian

- A new subscription is received by the Data Custodian

- A pre-defined interval has elapsed

- A request for resource data has been received from a Third Party

Pre-Condition: Third Party has an established account with Data Custodian.

Pre-Condition: Retail Customer has established a Third Party data access relationship with the Data Custodian and the Third Party with respect to a particular resource, resulting in a unique Shared Resource Key identifying the relationship.

Pre-Condition: A subscription by the Third Party to receive resource data from the Data Custodian has been established.

Invariant Constraint: No personal information is provided to the Third Party by the Data Custodian.

Post-Condition: The Data Custodian sends resource data to the subscribed Third Party.

Post-Condition: Only data specifically requested or modified and in a subscription is sent to the Third Party.

Scenario: Basic Path

- A data availability trigger is received by the Data Custodian.

- Data Custodian determines the Shared Resource Keys associated with the data availability trigger. It then determines if there are any subscriptions associated with the Shared Resource Key and whether the conditions of the subscription are satisfied (i.e., if it is time to send out resource data). If so, it proceeds to S3.

- Data Custodian determines the Third Party associated with the subscriptions. This includes a check that the Third Party is still in a valid relationship with the Data Custodian and any other relevant checks prior to releasing resource data to that Third Party.

- Data Custodian provides data resources to Third Party.

- Third party persists data for the period specified by data retention requirements.

10: Data Custodian Notifies Third Party of Data Availability - Asynchronous

Figure 14: Data Custodian Notifies Third Party of Data Availability - Asynchronous

Description

The Retail Customer has an existing third party data access relationship with a particular Data Custodian and Third Party. The Third Party has established a subscription for receiving the relevant resource data from the Data Custodian.  A Third Party is notified when new data satisfying its subscription parameters is available.

Conditions observable to the Data Custodian change, causing a data availability trigger to be checked to see if there is a need to notify a Third Party of resource data availability. Such triggers can be caused by any of the following observable changes

- New resource data is received by the Data Custodian

- A new subscription is received by the Data Custodian

- A pre-defined interval has elapsed

- A request for resource data has been received from a Third Party

Pre-Condition: Third Party has an established account with Data Custodian

Pre-Condition: Retail Customer has established a Third Party data access relationship with the Data Custodian and the Third Party with respect to a particular resource, resulting in a unique Shared Resource Key identifying the relationship.

Pre-Condition: Data Custodian has resource data relevant to the Third Party

Invariant Constraint: No personal information is provided to the Third Party by the Data Custodian

Post-Condition: The Data Custodian has resource data (e.g., electricity usage data) that is available for access by the Third Party

Post-Condition: The Data Custodian sends the Third Party notification of availability of resource data

Scenario: Basic Path

- A data availability trigger event is received by the Data Custodian.

- Data Custodian determines the Shared Resource Keys associated with the data availability trigger. The Data Custodian then determines if there are any subscriptions associated with the Shared Resource Key and whether the conditions of the subscription are satisfied (i.e., if it is time to notify a Third Party). If so, it proceeds to S3.

- Data Custodian determines the Third Party associated with subscriptions. This includes a check that the Third Party is still in a valid relationship with the Data Custodian and any other relevant checks prior to determining that it is appropriate to send resource data to that Third Party

- Data Custodian notifies the Third Party of the availability of resource data associated with the Shared Resource Key. Note that notification can take different forms. Notification could be sent asynchronously as soon as the trigger is evaluated. Notification for several resources could be bundled for delivery to a common Third Party. Notification could be queued, awaiting the next scheduled interaction with the Third Party (e.g., as part of a response to a regular pull from the Third Party). The essence of the use case is that a notification is prepared and delivered at some point; the specific mechanism and timing is not restricted.

11: Third Party Receives (Pulls) Requested Data from Data Custodian - Asynchronous

Figure 15: Third Party Receives (Pulls) Requested Data from Data Custodian - Asynchronous

Description

The Retail Customer has an existing third party data access relationship with a particular Data Custodian and Third Party. The Third Party requests the relevant subscribed and requested resource data from the Data Custodian, who replies with the data if the request is valid.

Pre-Condition: Third Party has an established account with Data Custodian

Pre-Condition: Retail Customer has established a Third Party data access relationship with the Data Custodian and the Third Party with respect to a particular resource, resulting in a unique Shared Resource Key identifying the relationship

Pre-Condition: Data Custodian has resource data relevant to the Third Party

Invariant Constraint: No personal data is provided to Third Parties by the Data Custodian.

Post-Condition: The Data Custodian replies with the requested data

Post-Condition: Only the requested resource data is provided by the Data Custodian

Scenario: Basic Path

- Third Party receives notification or periodically attempts to pull resource data from the Data Custodian.

- Data Custodian checks validity of request.

- Data Custodian replies with requested and subscribed resource data to Third Party.

- Third Party persists resource data for use in performing services for Retail Customer.

12: Third Party Requests Data from Data Custodian - Synchronous

Figure 16: Third Party Requests Data from Data Custodian - Synchronous

Description

The Retail Customer has an existing third party data access relationship with a particular Data Custodian and Third Party. The Third Party directly requests specific resource data from the Data Custodian, who replies with the requested data synchronously if the request is valid.

Pre-Condition: Third Party has an established account with Data Custodian

Pre-Condition: Retail Customer has established a Third Party data access relationship with the Data Custodian and the Third Party with respect to a particular resource, resulting in a unique Shared Resource Key identifying the relationship

Pre-Condition: Third Party requests authorized resource data

Invariant Constraint: No personal data is provided to Third Parties by the Data Custodian.

Post-Condition: The Data Custodian replies with the requested data.

Post-Condition: Only the requested resource data is provided by the Data Custodian.

Scenario: Basic Path

- Third Party decides to pull resource data from the Data Custodian.

- Third Party specifies the resource data being requested. The request must contain the Shared Resource Key. It may also contain parameters (e.g., the period over which the specified data is requested), if permitted by Data Custodian.

- Data Custodian checks validity of request (e.g., Shared Resource Key is still valid and registered with this Third Party or validity of any additional parameters).

- Data Custodian sends requested resource data to Third Party.

- Third Party persists resource data for use in performing services for Retail Customer.

# C. ESPI Abstract Services

This section provides definition of the abstract services used in the use cases. These are the services that will be specified fully for the ESPI model business practice specification. The services are named using the following conventions, since not all are intended to be fully standardized.

- Underscore before the method name means “must be done, but is not standardized”

- Underscore after the method name means “optional”

Figure: 17 - Logical Service Interfaces

DataCustodian

The DataCustodian service interface contains methods to be called by Third Party in order to authorize and receive data.

Operations

| Method | Notes | Parameters |
| --- | --- | --- |
| _CreateThirdPartyId() ThirdPartyId | Allows Data Custodian to obtain agreement to terms of service, contact information and application details about a Third Party application. Provides Third Party with service key and consumer secret. | ApplicationInformation [in] ApplicationInformation |
| ReadServiceStatus() ServiceStatus | Allows Third Parties to check their ability to access the Data Custodian service, and its current status. |  |
| CreateRequestToken() RequestToken | Allows Third Party to request an unauthorized request token. | Authorization [in] authRequest |
| Authorize() Authorization | Provides ability for Retail Customer to authenticate and verify desire to authorize a Third Party request token. This results in a verifier to be used with CreateAccessToken. | Authorization [in] authorization |
| CreateAccessToken() AccessToken | Allows Third Party to exchange an authorized request token for an access token. | Authorization [in] authRequest |
| _ReadAuthorizationList() AuthorizationList | Allows Retail Customer to choose an existing Authorization. Is not standardized, since the method involves user input. | RetailCustomerId [in] customerID |
| UpdateAuthorization() boolean | Provides ability to update an existing Authorization. | Authorization [in] authorization |
| CreateSubscription() boolean | Allows Third Party to request ongoing updates to the data resources associated with the specified Authorization, to be delivered asynchronously. | Authorization [in] authorization |
| DeleteSubscription() boolean | Removes the data resources associated with the specified Authorization from the subscriptions. | Authorization [in] authorization |
| RequestData() boolean | Allows Third Parties to request initial transfer of existing authorized data, or re-transfer of same. Results are delivered asynchronously. | Authorization [in] authorization |
| ReadData() Batch | Allows Third Party request ("pull") of asynchronously requested and subscribed data. | BatchLocation [in] batch |
| ReadData_() DataResource | Allows "on demand" (synchronous) access to authorized data. Some providers may choose not to make this method available. | Authorization [in] authorization DateTimeInterval [in] requestedInterval |

RetailCustomer

The Retail Customer service interface represents methods used to make requests of the Retail Customer.

Operations

| Method | Notes | Parameters |
| --- | --- | --- |
| _UpdateAuthorizationNotification() boolean | This method is not standardized, but provides notification to the Retail Customer that an Authorization was updated. May be optional, based on policies of Data Custodian. | Authorization [in] authorization |
| _RequestAuthorization() boolean | This method represents the delivery of the request to authorize an access grant. The normal flow implements this using a URL redirect, but other methods may be possible. | Authorization [in] authorization |

ThirdParty

The Third Party service interface contains methods to be called by the Data Custodian.

Operations

| Method | Notes | Parameters |
| --- | --- | --- |
| _ReadDataCustodianList() DataCustodianList | This is a required, but not standardized, method to allow the Retail Customer to find their Data Custodian. | RetailCustomerId [in] reatilCustomerID |
| _ProvideAuthorization() boolean | This method represents the callback after authorization of a request token. | Authorization [in] authorization |
| NotifyUpdateAuthorization_() boolean | This method allows for a Data Custodian to notify a Third Party when an authorization has been modified, so that timely requests to extend authorizations can be provided to Retail Customer, and so that the Third Party can differentiate between problems and lack of authorization. | Authorization [in] authorization |
| NotifyData_() boolean | This optional method can be implemented in order to avoid having to poll for new data. It is called by the Data Custodian to indicate that requested authorized data is available via ReadData. | BatchList [in] batchList |
| UpdateData_() boolean | This method can be implemented by the Third Party to allow asynchronous transfers to use the "push" model for delivery. If used, notify is not used, and polling is avoided. | Batch [in] data |

## Logical Information Model

This section contains descriptions of the data elements used in the abstract services.

Figure 18: ESPI Logical Information Model

AccessToken

AccessToken is a shared key representing the relationship between a RetailCustomer, DataCustodian, and ThirdParty for a particular data resource.

ApplicationInformation

Includes (non-standardized) information about the Third Party Application requesting access to the DataCustodian services. Information requested may include items such as Organization Name, Website, Contact Info, Application Name, Description, Icon, Type, default Notification and Callback endpoints, and may also include agreement with terms of service.

Authorization

Description of an authorization. Includes the information constraining and defining access to the Data. May include additional security elements, such as signature, timestamp, nonce, etc. as well as callback to allow redirection of the user agent.

| Name | Type | Description |
| --- | --- | --- |
| thirdPartyID | ThirdPartyId | The identifier issued to the Third Party by the Data Custodian. |
| requestToken | Token | An unauthorized token, issued by Data Custodian. |
| accessToken | Token | An access token that has been authorized by the user at the Data Custodian. |
| data | DataResource | The data resource to which the authorization was granted. |
| validityInterval | DateTimeInterval | The date time interval during which this access is authorized. |

Batch

This object is simply a container to hold a number of data resources.

BatchList

BatchList is a container to refer to a multiple batches of data.

BatchLocation

Specifies the location of a specific batch of data.

| Name | Type | Description |
| --- | --- | --- |
| reference | String |  |

CurrentStatus  «enumeration»

Valid values for current status.

| Name | Type | Description |
| --- | --- | --- |
| normal |  |  |
| unavailable |  |  |
| terminated |  |  |

DataCustodianId

An identifier for the Data Custodian.

DataResource

Generalization of any data object that can be exchanged.

| Name | Type | Description |
| --- | --- | --- |
| operation | byte | Create, Update, Delete |

RequestToken

RequestToken is the first step toward obtaining an authorized AccessToken.

RequestorID

Generic superclass for identifiers.

RetailCustomerId

An identifier for the Retail Customer.

ServiceStatus

Contains the current status of the service.

| Name | Type | Description |
| --- | --- | --- |
| currentStatus | CurrentStatus |  |

Subscription

Defines the parameters of a subscription between third party and data custodian

ThirdPartyId

An identifier for the Third Party.

Token

A Token is a key associated with a secret used for securing exchanges.

UsagePointList

Container to hold multiple UsagePoint objects.

# D. Goals for the Capability of the Standard

### Sample Business Process Guidelines

- ESPI Discovery

  - Authorized Third Party and Data Custodian agree to service level agreements (SLAs) and configure ESPIs services (REQ 21.1.3, REQ 21.1.16, REQ 21.1.18)

  - Authorized Third Party retrieves listing of supported operations with extensions and versions (REQ 21.1.14)

  - Data Custodian retrieves listing of supported operations with extensions and versions (REQ 21.1.14)

  - Authorized Third Party and Data Custodian subscribe to notifications

- Retail Customer Authorization

  - Retail Customer Grants Permission (REQ.21.1.4, REQ.21.1.7, REQ.21.1.8, REQ.21.1.9, REQ.21.1.10, REQ.21.1.15)

  - Retail Customer Extends Permission (REQ.21.1.9)

  - Retail Customer Terminates Permission (REQ.21.1.10)

- ADE Publication

  - Authorized Third Party Requests or Subscribes to Retail Customer energy usage information

  - Data Custodian Provides Retail Customer energy usage information to Authorized Third Party (REQ.21.1.20)

E. Cyber Security Requirements

[These will be provided by CSWG/ASAP as contribution assignee: MJB Coordinator]

F. Model Conformance Information

The following table provides information about the elements included in ESPI and their relation to the NAESB PAP10 Energy Usage Information Model as well as the IEC TC57 CIM Model. Harmonization across these models is a goal of this recommendation, as is aligning with other usage information interfaces, including Smart Energy Profile 2.0.

| PAP10 EUI Model Element | Type | ESPI Model Element | Type | CIM Notes |
| --- | --- | --- | --- | --- |
| CustomerAuthorisation.name | String | (OAuth) access_token |  | N/A |
| CustomerAuthorisation.validityInterval | DateTimeInterval | (OAuth 2.0) expires_in |  | N/A |
| UsagePoint.name | String | UsagePoint.mRID | HexBinary128 | Same |
| UsagePoint.description | String | UsagePoint.description | String32 | Same |
|  |  | UsagePoint.status | UInt8 | connectionState |
| ServiceCategory.kind | ServiceKind | ServiceCategory.kind | ServiceKind | Same |
| ServiceKind.electricity |  | ServiceKind 0 |  | (encoded) |
| ServiceKind.gas |  | ServiceKind 1 |  | (encoded) |
| ServiceKind.water |  | ServiceKind 2 |  | (encoded) |
| MeterReading.name | String | MeterReading.mRID | HexBinary128 | Same |
|  |  | MeterReading.description | String32 | Same |
| ReadingType.name | String | ReadingType.mRID | HexBinary128 | Same |
|  |  | ReadingType.description | String32 | Same |
| ReadingType.defaultQuality | QualityOfReading | ReadingType.defaultQuality | QualityOfReading | Recommended extension |
| ReadingType.direction | ReadingDirection | ReadingType.flowDirection | FlowDirectionType | Same |
| ReadingType.intervalLength | Duration | ReadingType.intervalLength | UInt32 | Recommended extension |
| ReadingType.kind | ReadingKind | ReadingType.kind | KindType | measurementKind |
| ReadingType.multiplier | UnitMultiplier | ReadingType.powerOfTenMultiplier | PowerOfTenMultiplierType | Recommended extension |
| ReadingType.unit | UnitSymbol | ReadingType.uom | UomType | unit |
|  |  | ReadingType.accumulationBehaviour | AccumulationBehaviourType | accumulation |
|  |  | ReadingType.dataQualifier | DataQualifierType | Recommended extension |
|  |  | ReadingType.tou | TOUType | Same |
|  |  | ReadingType.currency | CurrencyCode | Same |
|  |  | ReadingType.commodity | CommodityType | Same |
|  |  | ReadingType.consumptionTier | ConsumptionTierType | Same |
|  |  | ReadingType.phase | PhaseCode | phases |
|  |  | IntervalBlock.mRID | HexBinary128 | Recommended extension |
|  |  | IntervalBlock.description | String32 | Recommended extension |
|  |  | IntervalBlock.interval | DateTimeInterval | Recommended extension |
| Reading.cost | Float | Reading.cost | UInt48 | Recommended extension |
| Reading.timeStamp | AbsoluteDateTime | Reading.timePeriod | DateTimeInterval | Same |
| Reading.value | Float | Reading.value | UInt48 | Same |
| ReadingQuality.quality | QualityOfReading | ReadingQuality.quality | QualityOfReading | Recommended extension |
| DateTimeInterval.start | AbsoluteDateTime | DateTimeInterval.start | TimeType | Same |
| DateTimeInterval.duration | Duration | DateTimeInterval.duration | UInt32 | Uses "end" instead of "duration" |
| QualityOfReading.estimated |  | QualityOfReading *.8.0 |  | (encoded) |
| QualityOfReading.raw |  | QualityOfReading 1.* |  | (encoded) |
| QualityOfReading.validated |  | QualityOfReading *.0.1 |  | (encoded) |
| ReadingDirection.forward |  | FlowDirectionType 1 |  | (encoded) |
| ReadingDirection.reverse |  | FlowDirectionType 19 |  | (encoded) |
| ReadingKind.energy |  | FlowDirectionType 12 |  | (encoded) |
| ReadingKind.power |  | FlowDirectionType 37 |  | (encoded) |
| ReadingKind.demand |  | FlowDirectionType 8 |  | (encoded) |
| UnitMultiplier.micro |  | PowerOfTenMultiplierType -6 |  | (encoded) |
| UnitMultiplier.m |  | PowerOfTenMultiplierType -3 |  | (encoded) |
| UnitMultiplier.c |  | PowerOfTenMultiplierType -2 |  | (encoded) |
| UnitMultiplier.d |  | PowerOfTenMultiplierType -1 |  | (encoded) |
| UnitMultiplier.k |  | PowerOfTenMultiplierType 3 |  | (encoded) |
| UnitMultiplier.M |  | PowerOfTenMultiplierType 6 |  | (encoded) |
| UnitMultiplier.G |  | PowerOfTenMultiplierType 9 |  | (encoded) |
| UnitMultiplier.T |  | PowerOfTenMultiplierType 12 |  | (encoded) |
| UnitMultiplier.none |  | PowerOfTenMultiplierType 0 |  | (encoded) |

# 4.  SUPPORTING DOCUMENTATION

a.  Description of Request:

b.  Description of Recommendation:

c.  Business Purpose:

d.  Commentary/Rationale of Subcommittee(s)/Task Force(s):

NIST Special Publication 800-122, Guide to Protecting the Confidentiality of Personally Identifiable Information (PII) April 2010, page 2-1.