AMI-ENT Task Force
AMI-ENT Functional Requirements and Use Case Document
AMI-ENT Demand Response  
Functional Requirements and Use Case Document  
 Version 1.0  
	Version:
	1.0

	Created:
	June 30, 2009

	Last Update:
	

	Print Date:
	

	By:
	AMI ENT DR Use Case Team

	Distribution:
	Public


Acknowledgements

The following individuals and their companies are members of the UCAiug OpenSG and have contributed and/or provided support to the work of the AMI ENT Demand Response Functional Requirements and Use Case Document:

Albert Chiu, Pacific Gas and Electric

Reggie Cole, Lockheed Martin

Kent Dickson, Tendril  

Tray Fleming, SAIC Global Utility Sector

Gerald R. Gray, Consumers Energy

Erich Gunther, EnerNex

Greg Hinchman, Lockheed Martin

Doug Houseman, Capgemini

Alex Levinson, Lockheed Martin 

Wayne Longcore, Consumers Energy

Randy Lowe, American Electric Power

John Mani, Converge

Stuart McCafferty, EnerNex

Jerry Melcher, EnerNex

Terry Mohn, Sempra Energy Utilities

Dave Owens, Lockheed Martin

Phil Montell, American Electric Power

Greg Robinson, Xtensible Solutions 

Craig Rodine, Grid Net

Daniel J. Rogier, American Electric Power 

Marc Rosson, Snohomish County PUD

Bob Roth, Sparx Systems

Jack. Shih, Southern California Edison

Deb Smith, Digi International

Kay Stefferud, Lockheed Martin

Eva Thomas, Corporate Systems Engineering, Inc

Bon Truong, San Diego Gas & Electric

Mark Van Den Broek, Lockheed Martin 

Bud Vos, Converge

Xiaofeng Wang, GE Energy

Josh Wepman, SAIC

Joe Zhou, Xtensible Solutions

Table of Contents

11.0
INTRODUCTION


11.1
Purpose of Document


22.0
Application Overview


52.1
Scope


62.2
Context


62.3
Technical Environment


62.4
Terms and Definitions


112.5
Function Decomposition


143.0
DRMS Model


143.1
Business Process Models


143.1.1
Business Functions


153.1.2
Demand Response Business Process Model


163.1.3
Analyze Demand Response Scenario


173.1.4
Capture and Store Behavior Information on Demand Resources


173.1.5
Manage DR Program


183.1.6
Monitor and Store Real Time Network Information


183.1.7.1
Add DR Device


193.1.7.2
Remove DR Device


193.1.7.3
Manage Demand through Direct Load Control


203.1.8
Non-Functional Requirements


203.1.8.1
Performance Requirements


203.1.8.2
Performance


213.1.8.3
Scalability


213.1.8.3.1
Scalable for current markets


213.1.8.3.2
Scalable for future markets


213.1.8.4
Security


213.1.8.5
Transport


223.1.8.5.1
Compliant with IEC TC57 standards


223.1.9
Actors


233.1.9.1
Billing Agent


233.1.9.2
Customer


233.1.9.3
Demand Response Provider


233.1.9.4
Distributor


233.1.9.5
ISO


233.1.9.6
ISO or Grid Operator


243.1.9.7
Large C/I Customer and Co-Generator


243.1.9.8
Metering Agent


243.1.9.9
Regulator


243.1.9.10
Scheduling Agent


243.1.9.11
Settlement Agent


243.1.9.12
Small-Scale Merchant Generator


263.2
Manage Energy Resource


263.3
Manage Demand


263.4
Manage Demand for Economic Effect


263.5
Manage Demand through Direct Load Control


273.6
Manage Demand for Maintenance Purpose


273.7
Manage Demand in Respond to Pricing Signal


273.8
Manage DR Programs


283.8.1
Administer Program


283.8.2
Create DR Program


293.8.3
Delete DR Program


293.8.4
Enroll Customer


293.8.5
Dis-enroll Customer


303.8.6
Execute Program


303.8.7
Manage Program Customer


303.8.8
Measurement and Verification


313.8.9
Read DR Program


313.8.10
Supply Interruptible Resource


323.8.11
Supply Non-Interruptible Resource


333.8.12
Update DR Program


333.8.13
Manage Statistics


343.9
Manage DR Customer


343.10
Provision Demand Response Equipment


343.10.1
Add DR Device


353.10.2
Remove DR Device


353.10.3
Read DR Device Information


353.10.4
Update Active DR Device


353.11
Manage Supply


363.12
Manage Supplier


363.13
Manage Supply through Price Signal


363.14
Manage Supply through Direct Control


363.15
Provision Supply Response Equipment


374.0
DR Event Class




List of Figures

2Figure 2-1.  Timing of a Demand Response Event


4Figure 2-2.  Illustration of Baseline Concept


5Figure 2-3.  Demand Response Service Types


11Figure 2-4.  DRMS High-Level Functional Decomposition


12Figure 2-5.  Manage Customer Programs Functional Decomposition


12Figure 2-6.  DR Measurement and Verification Functional Decomposition


13Figure 2-7.  Execution Event Resopnse Funcational Decomposition


14Figure 3-1.  IEG-61968 IRM Business Functions


15Figure 3-2.  Demand Response Business Process Model


16Figure 3-3.  Analyze Demand Response Scenario


17Figure 3-4.  Capture and Store Behavior Information on Demand Resources


17Figure 3-5.  Manage DR Program


18Figure 3-6.  Monitor and Store Real Time Network Information


18Figure 3-7.  Add DR Device to Data Repository


19Figure 3-8.  Remove DR Device From Data Repository


19Figure 3-9.  Manage Demand through Direct Load Control


20Figure 3-10.  Non-Functional Requirements


20Figure 3-11.  Performance


21Figure 3-12.  Scalability


21Figure 3-13.  Transport


22Figure 3-14.  Reference Architecture


23Figure 3-15.  Actors


25Figure 3-16.  Primary Use Case Diagram


27Figure 3-17.  Manage DR Programs Use Case Diagram


32Figure 3-18.  Supply Interruptible DR Resource


33Figure 3-19.  Supply Non-Interruptible DR Resource


34Figure 3-20.  Provision Demand Response Equipment


37Figure 4-1.  DR Event Class Diagram




1.0
INTRODUCTION
Demand Response is the proactive management of electric and gas utility loads in order to more efficiently and reliably market, produce, transmit and deliver energy. Applications of demand response are as simple as the Utility interrupting load in response to severe grid transients or supply shortages (direct load control or active demand-side management), or as complex as millions of customers voluntarily reducing their consumption/load in response to price signals (passive demand-side management). With the exception of having to address emergencies, DR is generally used to flatten the demand peaks. In any case, the Utility must have a communications gateway to either directly control the consumer's loads, or provide a pricing signal to allow the consumer to manage their consumption directly by:

· Making the decision when to use appliances/equipment

· As input to a home/premise energy management system
Large Commercial and Industrial Customer DR Programs are not new. They have been in-place for more than 20 years. Demand Response systems have traditionally been utilized with large commercial and industrial customers because the individual loads are larger, requiring fewer controls and automation, in achieving the desired load reduction/shedding. However, as demand has continued to grow, there has been a noticeable shift in the overall makeup and magnitude of the energy demand peak. Residential consumers now make up about 60% of the peak, with unprecedented growth occurring, such as 17% growth in the last three years in the U.S. Mid‑Atlantic states. Additional DR will have to come primarily from residential consumers. There currently are successful, residential DR programs – Florida Power and Light (FP&L) Company has about 750,000 residential customers enrolled with the capability to shed ~1,000 Mw of load.
To clarify terms, this document describes:

· Energy Efficiency – Reduce total kilowatt of load with permanent and efficient technologies

· Demand Response – Temporary reduction of peak energy usage for a defined duration. Curtailment events are triggered either by reliability events or pricing signals.

· Load Shifting – Flattening the peak by using off-peak power in place of on-peak power. This is often a permanent peak shift driven by combining technologies and time-of-use rates. An example includes thermal energy storage.

1.1
Purpose of Document
The Purpose of this Document is to define the Use Cases and functional requirements for a Demand Response Management System (DRMS) deployable anywhere in the world. The desired system will be developed using open non-proprietary standards, will be scalable to any geographic area, and will be designed to be upwardly compatible with future enhancements,
2.0
Application Overview
Demand Response systems curtail load to maintain grid reliability and to reduce demand during peak load periods.  Demand Response systems manage load by issueing Demand Response Events.
The illustration below (Figure 2-1) from the Recommendation to the NAESB Executive Committee represents the terms for timing events and time durations applicable to a Demand Response Event.  The definitions of the ten elements in the illustration are the basis for describing the Timing of a Demand Response Event.  The applicablity of these elements to a Demand Response Service is dependent on the Service type.  The Grid Operator shall specify whether any or all of the elements illustrated in the Timing Demand Response Event figure are applicable.  In some cases, some elements will not be applicable; the inclusion of the elements establish a requirement for said elements.
[image: image1.emf]DEPLOYMENT PERIOD

RECOVERY

PERIOD

R

E

D

U

C

T

I

O

N

 

D

E

A

D

L

I

N

E

D

E

P

L

O

Y

M

E

N

T

RAMP

PERIOD

SUSTAINED RESPONSE

PERIOD

R

E

L

E

A

S

E

/

R

E

C

A

L

L

N

O

R

M

A

L

 

O

P

E

R

A

T

I

O

N

S

DEMAND RESPONSE EVENT

A

D

V

A

N

C

E

 

N

O

T

I

F

I

C

A

T

I

O

N

(

S

)

 


Source: Recommendation to the NAESB Executive Committee DSM-EE Subcommittee dated December 2, 2008
Figure 2-1.  Timing of a Demand Response Event

The following terms refer to the above Figure 2-1.

	Term
	Definition

	Advance Notification(s)
	One or more communications to Demand Resources of an impending Demand Response Event in advance of the actual event.

	Deployment
	The time at which a Demand Resource begins reducing Demand on the system in response to an instruction.

	Deployment Period
	The time in a Demand Response Event beginning with the Deployment and ending with the Release/Recall.

	Normal Operations
	The time following Release/Recall at which a System Operator may require a Demand Resource to have returned its Load consumption to normal levels, and to be available again for Deployment.

	Ramp Period
	The time between Deployment and Reduction Deadline, representing the period of time over which a Demand Resource is expected to achieve its change in Demand.

	Recovery Period
	The time between Release/Recall and Normal Operations, representing the window over which Demand Resources are required to return to their normal Load.

	Reduction Deadline
	The time at the end of the Ramp Period when a Demand Resource is required to have met its Demand Reduction Value obligation.

	Release/Recall
	The time when a System Operator or Demand Response Provider notifies a Demand Resource that the Deployment Period has ended or will end.

	Sustained Response Period
	The time between Reduction Deadline and Release/Recall, representing the window over which a Demand Resource is required to maintain its reduced net consumption of electricity. 


A Baseline is an estimate of the electricity that would have been consumed by a Demand Resource in the absence of a Demand Response Event. The Baseline is compared to the actual metered electricity consumption during the Demand Response Event to determine the Demand Reduction Value.  Depending on the type of Demand Response product or service, Baseline calculations may be performed in real-time or after-the-fact.  The Grid Operator may offer multiple Baseline models and may assign a Demand Resource to a model based on the characteristics of the Demand Resource's Load or allow the Demand Resource to choose a performance evaluation model consistent with its load characteristics from a predefined list.  A baseline model is the simple or complex mathematical relationship found to exist between Baseline Window demand readings and Independent Variables.  A baseline model is used to derive the Baseline Adjustments which are part of the Baseline, which in turn is used to compute the Demand Reduction Value.  Independent variable is a parameter that is expected to change regularly and have a measureable impact on demand.  Figure 2-2 illustrates the concept of Baseline relative to a Demand Response Event.

[image: image28.png]Expected or
N Contracted Load
Baseline Reduction Measured
Load

ELECTRICITY
USAGE

/ \ T T TIME

Deployment Reduction Release Normal
instruction deadline operations



Source: Recommendation to the NAESB Executive Committee DSM-EE Subcommittee dated December 2, 2008
Figure 2-2.  Illustration of Baseline Concept
DR Services define the typical services that a Grid Operator can request or correspondingly that a Demand Response Provider can supply.
[image: image2.wmf]
Figure 2-3.  Demand Response Service Types
2.1
Scope
Demand Response (DR) systems are currently being considered for adoption and expansion in a number of markets.  Existing DR systems have been successfully deployed by organizations including ISO New England, co-ops and others using AMR meters, and through numerous programs controlling air conditioners with wireless signals.

Newer DR systems will ideally build upon features of existing systems while providing for future enhancements via open standards which anticipate technology advancements including smart meters, increased local generation, e.g., microgrids, and other SmartGrid infrastructure enhancements.
The specific DR system covered by this document is anticipated to be used by public utilities, coops, government-owned utilities and direct access providers.  In order to leverage both existing and future vendor investments in DR system development, the intent of the DR functionality described here is to be compatible with existing technology, and to be compliant with IEC electrical standards, specifically with IEC 61968.

2.2
Context
This document is targeted to utilities, regional transmission operators, third party aggregators and large demand response providers.  A Demand Response Management Systems (DRMS) is envisioned that will evolve over time to accomplish many tasks including:

· DR program design, operations, and enrollment

· DR event execution including forecasting, bidding and scheduling

· DR performance evaluation including measurement and validation based on baseline methodology

· DR event billing and settlement

All these DR activities result in a need for implementation of business processes that are derived from a solid set of functional requirements. These activities will result in integration with utilities' internal systems as well as external clients.  Information exchange will be a fundamental requirement to successful deployment of a DRMS.

2.3
Technical Environment
We envision this new system to integrate with at least the following existing systems: Customer Billing, Outage Management, Meter Data Management, Customer Relationship Management and Financial.
2.4
Terms and Definitions

This subsection provides the definition of terms in general use:

	Term
	Definition

	Adjustment Window
	The period of time prior to a Demand Response Event used for calculating a Baseline adjustment.

	Advanced Metering
	Technology which allows two way communications between the utility and the meter. This communication enables the ability to analyze energy consumption resulting in more efficient demand response systems.

	Advanced Metering Infrastructure (AMI)
	The infrastructure built around advanced metering allowing the utility and consumer to communicate in real time with respect to energy consumption. Based on the information collected the utility is able to obtain an accurate reading of demands, while consumers are able to modify their usage to save energy. 

	Aggregated Demand Resource
	A group of independent Load facilities that provide Demand Response services as a single Demand Resource.

	After-the-Fact Metering
	Interval meter data separate from Telemetry that is used to measure Demand Response.  May not apply to Demand Resources under Baseline Type II (Non‑Interval Meter).

	Automated Meter Reading
	Automated meter reading is a subcategory of AMI which allows for communication devices to transfer data from a meter to the utility or from a meter to the data management provider.

	Baseline
	A Baseline is an estimate of the electricity that would have been consumed by a Demand Resource in the absence of a Demand Response Event. The Baseline is compared to the actual metered electricity consumption during the Demand Response Event to determine the Demand Reduction Value.  Depending on the type of Demand Response product or service, Baseline calculations may be performed in real-time or after-the-fact.  The Grid Operator may offer multiple Baseline models and may assign a Demand Resource to a model based on the characteristics of the Demand Resource’s Load or allow the Demand Resource to choose a performance evaluation model consistent with its load characteristics from a predefined list.  A baseline model is the simple or complex mathematical relationship found to exist between Baseline Window demand readings and Independent Variables.  A baseline model is used to derive the Baseline Adjustments which are part of the Baseline, which in turn is used to compute the Demand Reduction Value.  Independent variable is a parameter that is expected to change regularly and have a measureable impact on demand.

	Baseline Adjustment
	An adjustment that modifies the Baseline to reflect actual conditions immediately prior to or during a Demand Response Event to provide a better estimate of the energy the Demand Resource would have consumed but for the Demand Response Event. The adjustments may include but are not limited to weather conditions, near real time event facility Load, current Demand Resource operational information, or other parameters based on the System Operator’s requirements.

	Baseline Type-I (Interval Metered)
	A Baseline performance evaluation methodology based on a Demand Resource’s historical interval meter data which may also include other variables such as weather and calendar data.

	Baseline Type-II (Non-Interval Metered)
	A Baseline performance evaluation methodology that uses statistical sampling to estimate the electricity consumption of an Aggregated Demand Resource where interval metering is not available on the entire population.

	Baseline Window
	The window of time preceding and optionally following, a Demand Response Event over which the electricity consumption data is collected for the purpose of establishing a Baseline.  The applicability of this term is limited to Meter Before/Meter After, and Baseline Type-I and Type-II.

	Business Intelligence
	A term describing the extraction and presentation of data to provide business value. 

	Business Service Provider
	Software delivered over the internet as web services. The platform for integrating these web services is the enterprise service bus. 

	Capacity Service
	A type of Demand Response service in which Demand Resources are obligated over a defined period of time to be available to provide Demand Response upon deployment by the System Operator.

	Communicate
	Interact and cooperate with people or organizations (groups of people).  

	Control
	Monitor and regulate the supply (of electricity or other commodity)

	Daily Consumption
	The amount of energy a customer uses in a 24 hour period. This information is used to drive business intelligence solutions.

	Demand
	The rate at which electric energy is delivered to or by a system or part of a system, generally expressed in kilowatts or megawatts, at a given instant or averaged over any designated interval of time; and the rate at which energy is being used by the customer (NERC Definition).

	Demand Billing
	The energy demand of a customer upon which billing is calculated. This is often based on peak demand or some other demand related measurement.

	Demand Interval
	The interval of time between demand queries to the meter. This is typically in 15, 30, and 60 minute intervals.

	Demand Reduction Value
	Quantity of reduced electrical consumption by a Demand Resource, expressed as MW or MWh.

	Demand Resource
	A Load or aggregation of Loads capable of measurably and verifiably providing Demand Response.

	Demand Response
	An agreement between customer and utility that states that the customer agrees to allow the utility to manage their energy consumption when the utility deems necessary. Often times this result in the utility increasing or reducing energy distribution based on supply based metrics. Demand response mechanisms typical operate in on or off whereas dynamic response mechanisms may passively curtail energy usage as the mechanism senses stress on the grid. 

	Demand Response Provider
	The entity that is responsible for delivering Demand reductions from Demand Resources and is compensated for providing such Demand Response products in accordance as specified by the System Operator.

	Distributed Generation
	Electricity generation from small energy sources allowing for more efficient energy distribution. This approach allows for energy to be generated closer to the source of the consumption which reduces the distance the generated energy has to travel.

	Energy Data Acquisition 
	Obtaining meter data by way of handheld devices. Essentially a non automated meter reading typically administered by a utility worker. 

	Energy Data Management
	Analyzing meter data for consumption by backend systems. Often times these back end systems will measure load, calculate demand response, billing intervals, etc.

	Energy Service
	A type of Demand Response service in which Demand Resources are compensated solely based on their performance during a Demand Response Event.

	Enterprise Resource Planning
	Integrating all back and front office data and process into one unified enterprise system.

	ESB
	Enterprise Service Bus. The ESB provides the features necessary for a service oriented architecture implementation by providing a place to host all of the web services.

	Highly-Variable Load
	A Load with a fluctuating or unpredictable electricity consumption pattern.

	IEC
	The International Electrotechnical Commission (IEC). The IEC TC57 maintains an electric utility focused information model called CIM (Common information model).

	IEC 61968
	International standards for Energy Distribution Managements Systems, respectively, specify a Common Information Model (CIM) for utility data exchange, Applications Programming Interfaces (API) for application integration (GID), and XML messaging standards.

	Load
	An end-use device or customer that receives power from the electric system (NERC Definition).

	Load Shedding
	Reducing a customer's demand in order to maintain integrity of the grid. Load shedding in utility operations, is monitoring electric usage continuously (usually by automated instrumentation) and shutting down certain pre-arranged electric loads or devices if a certain upper threshold of electric usage is approached.

	Logical Data Model
	A representation of an organization's data based upon entities and attributes of those entities. A logical data model is often a logical representation of a business' integration or business requirements.

	Maintain
	Keep up to date information on, typically associated with data storage.

	Manage
	Control and effectively handle devices or processes. 

	Maximum Base Load
	A performance evaluation methodology based solely on a Demand Resource’s ability to reduce to a specified level of electricity Demand, regardless of its electricity consumption or Demand at Deployment.

	Meter Before/ Meter After
	A performance evaluation methodology where electricity Demand over a prescribed period of time prior to Deployment is compared to similar readings during the Sustained Response Period.

	Meter Bus (M‑bus)
	Allows for the interconnecting of many different utility measuring units (i.e. gas, electric, water, etc.) The M-Bus acts as the central station for these different utilities to communicate with.

	Meter Data Management
	A system for storing, processing, consuming and analyzing large quantities of meter data.

	Meter Data Recording Interval
	The time between electricity meter consumption recordings.

	Metering Date Reporting Deadline
	The maximum allowed time from the end of a Demand Response Event (Normal Operations) to the time when meter data is required to be submitted for performance evaluation and settlement.  The Meter Data Reporting Deadline may be either relative (a number of hours/days after Normal Operations) or fixed (a fixed calendar time, such as end-of-month).

	Metering Generator Output
	A performance evaluation methodology, used when a generation asset is located behind the Demand Resource’s revenue meter, in which the Demand Reduction Value is based on the output of the generation asset.

	Performance Window
	The period of time in a Demand Response Event analyzed by the System Operator to measure and verify the Demand Reduction Value for a Demand Resource.

	Price Signals
	Messages sent to consumers which specify a commodity price.

	Provide
	Feature/possess the capability to.  System has the ability to perform the specified function.

	Ramp Rate
	The rate, expressed in megawatts per minute, that a generator changes its output. (NERC Definition)   Demand Resource ramp rate is the rate, expressed in megawatts per minute, that a Demand Resource changes its Load.

	Real Time Metering
	Meter readings taken almost in real time to allow for adjustments to be made as the energy market fluctuates.

	Regulation Service
	A type of Demand Response service in which a Demand Resource increases and decreases Load in response to real-time signals from the System Operator.  Demand Resources providing Regulation Service are subject to dispatch continuously during a commitment period.  Provision of Regulation Service does not correlate to Demand Response Event timelines, deadlines and durations.

	Receive
	Accept messages from devices.

	Reserve Service
	A type of Demand Response service in which Demand Resources are obligated to be available to provide Demand reduction upon deployment by the System Operator, based on reserve capacity requirements that are established to meet applicable reliability standards.

	SLA 
	Service Level Agreement: the part of a service contract where the level of the services are agreed upon between two systems.

	Smart Grid
	The term smart grid represents the digital upgrade of our distribution and long distance transmission grid allowing for increased energy efficiency as well as a boost in optimization of current systems.

	Smart Meters
	Meters with extra functionality that allow for more accurate and useful meter readings.  This extra functionality allows the meter to collect usage data and transmit this data back to the utility over a network.

	SOA
	Service oriented architecture - The concept of grouping business functionality around business processes. These services are than packaged as interoperable services.  A SOA architecture allows for the transmission of data between multiple systems as they participate in multiple business processes.

	SOAP
	Simple Object Access Protocol (XML protocol) - A protocol for exchanging xml messages for web services in a service oriented architecture implementation. 

	Supervisory Control and Data Acquisition (SCADA)
	SCADA systems monitor and control the electric power generation, transmission, and distribution.

	Supply Resources
	Devices which have the capability to supply electricity.

	System Operator
	A System Operator is a Balancing Authority, Transmission Operator, or Reliability Coordinator whose responsibility is to monitor and control an electric system in real time (based on NERC definition).  The System Operator is responsible for initiating Advance Notifications, Deployment, and Release/Recall instructions.

	Telemetry
	Real-time continuous communication between a Demand Resource or Demand Response Provider and the System Operator.

	Telemetry Interval
	The time unit between communications between a Demand Resource or Demand Response Provider and a System Operator.

	Transmit
	Send messages to devices.

	UML
	Unified Modeling Language is a general purpose modeling language commonly used for object/data modeling.

	Utility Sub Metering
	An implementation that allows for a multi tenant property to bill tenants for individual energy usage. This is most commonly implemented in apartments and condominiums.

	Validation, Editing and Estimation
	The process of taking raw meter data and performing validation and, as necessary, editing and estimation of corrupt or missing data, to create validated data.  (VEE guidelines are published in the Edison Electric Institute’s Uniform Business Practices for Unbundled Electricity Metering, Volume Two, Published 12/05/00, http://www.naesb.org/REQ/req_form.asp).

	WSDL
	Web Services Description Language is a xml format used to describe web services and the messages that interface with the web services. 

	XML
	Extensible Markup Language – general purpose markup language for creating custom mark-up languages.

	XSD
	A description describing a specific xml document focusing primarily on the restraints and structure of that xml document. 


2.5
Function Decomposition
Functional decompositions define a system in terms of its main activities. For the DRMS, the main activities have been defined in  three main areas:
· Manage Customer Programs

· Execute DR Event Response and

· DR Measurement and Verification.

The three main functional areas were then further broken down into subfunctions as shown in Figures 2-4, 2-5, 2-6 and 2-7.

[image: image3.wmf]Demand 

Response 

Management 

System 

(

DRMS

)

Manage 

Customer 

Programs

Execute DR 

Event Response

DR Measurement 

and Verification


Figure 2-4.  DRMS High-Level Functional Decomposition
[image: image4.wmf]Manage 

Customer 

Programs

Administer 

Customer 

Programs

Create Customer 

Programs

Modify Customer 

Programs

Remove 

Customer 

Programs

Manage 

Customer 

Enrollment

Manage 

Customer Web 

Portal Enrollment

Manage 

Customer Call 

Center Enrollment

Provision DR 

Equipment

Add DR Device

Modify DR Device

Remove DR 

Device


Figure 2-5.  Manage Customer Programs Functional Decomposition

[image: image5.emf]Measure DR 

Response

DR Measurement 

and Verification

Verify DR 

Response

Post Results


Figure 2-6.  DR Measurement and Verification Functional Decomposition

[image: image6.emf]Execute DR 

Event Response

Forecast DR 

Resource 

Availability

Monitor and Store 

Event Data

Create DR

Event Resource 

DR Resource 

Behavior

DR Resource 

Baseline 

Select DR 

Resources/

Custom 

Aggregation 

Group

Select DR 

Resource Service 

Type


Figure 2-7.  Execution Event Resopnse Funcational Decomposition
3.0
DRMS Model
3.1
Business Process Models
3.1.1
Business Functions
[image: image7.emf]object Business Functions

 IEC-61968 IRM

 Distribution 

Mamagement

 External to 

Distribution 

Management

 Electric Distribution Network 

Planning, Construction, 

Maintenance, and Ops

 Generation and Transmission 

Management, ERP, Supply Chain, and 

General Corporate Services

 NO- Network Ops

 AM - Records and 

Asset Management

 OP - Operational 

Planning and 

Optimization

 MC - Maintenance 

and Construction

 EMS - Energy 

Management and 

Trading

 RET- Retail

 SC - Supply Chain 

and Logistics

 NE - Network 

Extension Planning

 CS - Customer 

Support

 MR - Meter 

Reading and 

Control

 ACT - Customer 

Account 

Management

 FIN - Financial

 PRM - Premises

 HR - Human 

Resources


Figure 3-1.  IEG-61968 IRM Business Functions
3.1.2
Demand Response Business Process Model
[image: image8.emf]class Demand Response Business Process Model

 Manage Demand for Grid Reliability

 Customer

 Energy 

Shortage/Congestion/Equipment 

Failure

 «goal»

Maintain Reliability of 

the Grid

 Load Control 

Transaction

 ISO or Grid Operator

(from Actors)

 Supply Profile

 Compliance

 Distributor

(from Actors)

 Demand Response 

Provider

(from Actors)

 Manage Demand for Economic Dispatch

 «goal»

Achieve Least Cost 

Dispatch

 Automatic 

Generation Control 

(AGC)

 Spinning Reserves

 Non-spinning 

Reserves

 Replacement 

Reserves

 Market condition that forces either buying 

more energy at significant cost or reduce 

demand to avoid the buy

 Load Control Transaction Sufficient 

to Avoid Buying Energy

«uses»

«supply»

«signal»

«load»

«uses»

«signal»

«load»

«signal»

«flow» «flow» «flow» «flow»

«signal»


Figure 3-2.  Demand Response Business Process Model
3.1.3
Analyze Demand Response Scenario
[image: image9.emf]BPMN Analyze Demand Response Scenario

DR Analyzer

«Pool»

DRMS

«Pool»

 ProcessStart

  Create DR scenario

analysis request

 «Message»

AnalyzeDRScenarioRequest

  Receive DR scenario

for analysis

 «BusinessProcess»

Perform DR scenario

analysis

  Send DR scenario

results

 «Message»

DRScenarioResult

  Receive DR scenario

results

 ProcessEnd

«SequenceFlow»

«SequenceFlow»

«flow»

«flow»

«flow»

«flow»

 


Figure 3-3.  Analyze Demand Response Scenario
3.1.4
Capture and Store Behavior Information on Demand Resources
[image: image10.emf]BPMN Capture and Store behavior information on Demand Resources

MDMS

«Pool»

DRMS

«Pool»

DR Anayzer

«Pool»

 «Message»

DemandResourceLoadProfileDataRequest

 ProcessStart

  Request demand

resource load profile

data

  Process data request

  Send demand

resource load profile

data

 «Message»

DemandResourceLoadProfileData

 «datastore»

Data Historian

  Build demand

resource model

  Request demand

resource service

model

 «Message»

DemandResouceServiceModelRequest

  Process data

request

  Send demand

resource service

model

 «Message»

DemandResourceServiceModel

  Compute demand

resource baseline

 ProcessEnd

 «Message»

DemandResourceEventResponseDataRequest

  Request Demand

Resource event

response data

     «flow»

«flow»

«flow»

«flow»

«flow»

«flow»

«flow»

«flow»


Figure 3-4.  Capture and Store Behavior Information on Demand Resources
3.1.5
Manage DR Program
[image: image11.emf]BPMN Manage DR Program

 «BusinessProcess»

DR Program Creation &

Maintenance

 «BusinessProcess»

DR Program Enrollment

 «BusinessProcess»

DR Program

Dis-enrollment


Figure 3-5.  Manage DR Program
3.1.6
Monitor and Store Real Time Network Information
[image: image12.emf]BPMN DRACS Monitors and Stores Real Time Network Information

SCADA/EMS

«Pool»

DRACS «Pool»

 ProcessStart

  Request system

update

 «Message»

NetworkSnapshotRequest

  Create system

snapshot

  Send system

snapshot

 «Message»

NetworkSnapshot

 «datastore»

Data Historian

  Process system

snapshot

  Create graphical

view

 ProcessEnd

«flow»

«flow»

«flow»

«flow»

   


Figure 3-6.  Monitor and Store Real Time Network Information
3.1.7.1
Add DR Device
[image: image13.emf]BPMN Add DR Device to Data Repository

 Request for Adding New DR

Device

  Receive Request

  Review Attributes of

DR Device

  Is set of attributes complete?

  Add DR Device 

 End

Any industry standard 

to be adhered to?

  Handle Incomplete

Attributes

    No

   yes


Figure 3-7.  Add DR Device to Data Repository
3.1.7.2
Remove DR Device
[image: image14.emf]BPMN Remove DR Device From Data Repository

 Request for Removing DR

Device

  Receives Request for

Removing DR

Device

  Review Request

If DR device peforms 

the next scheduled 

direct load control, can 

it still be removed?

  Approved?

  Remove DR Device

from Data Repository

 End

  Handle Incomplete

Attributes

     yes

 no


Figure 3-8.  Remove DR Device From Data Repository
3.1.7.3
Manage Demand through Direct Load Control
[image: image15.emf]class Manage Demand through Direct Load Control

Customer

«Lane»

DRMS

«Lane»

  Receive Emergency

Signal to reduce

demand

 ISO

(from Primary Use Cases)

  Make Decision on

how to adjust load

  Advance Notify

Customer of DR

curtailment

  Present Notificiation

of DR Event

  Send Curtailment

Notice

  Optional 

Opt Out

  Present Load

Curtailment Event to

Customer

  Curtail Load

  Customer has energy management system?

  Do not curtail load

  Capture Compliance

  Load Goal Achieved?

 Stop

  Do Alternative

Course of Action

 Distributor

(from Actors)

  «flow»

    «flow»

«flow»

 No

 Yes

  No

 Yes

 


Figure 3-9.  Manage Demand through Direct Load Control
3.1.8
Non-Functional Requirements
[image: image16.emf]custom Non-Functional Requirements

 Performance

 Scalability

 Security

 Persistence

 Transport

These packages contain non-functional 

requirements specified for the new system. These 

typically describe performance criteria, reliability, 

security and other operational parameters.

The Transport package defines constraints 

and requirements affecting the 

transmission of information between nodes.

Networks, relays, protocols, quality of 

service and even transmission of physical 

media are included here.

The Persistence package details 

operational and performance criteria 

relating to the storage of information, 

including where relevant, redundancy, 

back-up, database system, files and other 

persistent storage mechanisms.

The Security package details 

requirements with respect to both data 

access (information security) and physical 

security (access to servers and other 

critical hardware).

Scalability requirements define the 

operational parameters with respect to 

system size, number of transactions, 

capacity, number of users and node 

distribution. 

Performance requirements define 

parameters such as transactions per 

second, network latency, form load times

and other measurable aspects of the 

system that govern overall speed and 

responsiveness.


Figure 3-10.  Non-Functional Requirements
3.1.8.1
Performance Requirements
MERGEFIELD Element.NotesPerformance requirements define parameters such as transactions per second, network latency, form load times and other measurable aspects of the system that govern overall speed and responsiveness.

3.1.8.2
Performance
[image: image17.emf]custom Performance


Figure 3-11.  Performance
3.1.8.3
Scalability
[image: image18.emf]custom Scalability

 Scalable for all 

markets

 Scalable for 

current markets 

 Scalable for 

future markets


Figure 3-12.  Scalability
3.1.8.3.1
Scalable for current markets
The DR solution shall scale to up to MERGEFIELD Element.NotesCurrent market size includes known programs with 10,000,000 meters with up to 50,000,000 devices.

3.1.8.3.2
Scalable for future markets
MERGEFIELD Element.NotesFuture markets may contain entities with up to a 1000 number of participants in a single geographic location.
3.1.8.4
Security
The DR solution shall be in compliance with the AMI System Security Requirements and Guidelines, version 1.0, approved at the January 8, 2009 AMI-SEC meeting held in Palo Alto, California.
3.1.8.5
Transport
[image: image19.emf]custom Transport

 Meets 

international IEC 

standards

 Compliant with 

IEC TC57 

standards


Figure 3-13.  Transport
3.1.8.5.1
Compliant with IEC TC57 standards
MERGEFIELD Element.NotesThe DR solution shall be compliant with the latest IEC TC57 Reference Architecture.  See the reference architecture in Figure 14.
[image: image20.emf] 

Market Operation    Apps  

60870 - 6 - 503   App Services  

SCADA Apps  

EMS Apps  DMS Apps   Engineering &    Maintenance Apps  

External   IT Apps  

Data Acquisition and Control Front - End / Gateway / Proxy Server / Mapping Services / Role - based Access  Control  

61850 - 8 - 1   Mapping to MMS  

 

TC13  WG14    Meter  Standards  

 

61334  

60870 - 5   101   &    104  

61970 Component Interface Specification (CIS) and 61968 SIDMS for Enterprise Application Integration (EAI, EII, and ETL)  

61970 / 61968 Common  Information Model (CIM)  

Inter - Application Messaging Middleware (specified in XML; mapped to appropriate protocols)  

61850  Substation    Devices  61850 IED    Field Devices  & Distribution  Feeders     60870 - 6   TASE.2   Other Control    Centers   60870 - 5   RTUs   or   Substation   Systems  

IEDs, Relays, Meters, Switchgear, CTs, VTs  

End - to - End Security Standards and Recommendations (work in  progress)  

IEC TC57 Reference Architecture  

External Systems   (Symmetric client/server    protocols)  

Specific Communication    Services Mappings  

Specific Object    Mappings  

Application   Interfaces  

Equipment And   System Interfaces  

Telecontrol    Communications    Media and Services  

Communication Industry Standard Protocol Stacks   (ISO/TCP/IP/Ethernet)  

XML   Messaging   (work in   progress)  

Protocol Profiles  

XML    Messaging   External    Systems  (e.g.,    Substations)  

WAN   Communications    Media and Services  

Field   Devices  

Utility Customers   Energy   Market  Participants   Other  Businesses   Utility    Service  Providers  

Network, System, and Data Management  (future)  

TC13  WG14  

*Notes: 1) Solid colors correlate different parts of protocols within the architecture.                 2) Non - solid patterns represent areas that are future work, or work in progress, or related work provided by another IEC TC.  

61850 - 7 - 2    ACSI  

61850 - 7 - 3, 7 - 4    Object Models  

Customer   Meters  

Peer - to - Peer 61850 over   Substation bus and  Process bus  

60870 - 6 - 802   Object Models  

60870 - 6 - 703   Protocols  

Field Object Models  

Application To Application (A2A)     and Business To Business   (B2B) Communications  


Figure 3-14.  Reference Architecture

3.1.9
Actors
MERGEFIELD Element.NotesActors are the users of the system being modeled. Each Actor will have a well-defined role, and in the context of that role have useful interactions with the system.

A person may perform the role of more than one Actor, although they will only assume one role during one use case interaction.

An Actor role may be performed by a non-human system, such as another computer program.

[image: image21.emf]uc Actors

 Metering Agent

 Settlement Agent

 Billing Agent

 ISO or Grid Operator

 Distributor

 Small-Scale Merchant 

Generator

 Large C/I Customer and 

Co-Generator

 Demand Response 

Provider

 Customer Residential

 Customer Commercial

 Customer Industrial

 Customer

A system that collects detailed information about customer 

loads and customer response patterns. It also maintains 

information regarding the number of times a customer has 

complied in a given time period vs the compliance 

requirements of the tariff applicable to that customer. 

This information is brought together for the user so that the 

user can see what probable load is available to be curtailed

in total and at various points in the network. 

The system will also receive and process requests for 

curtailment and will balance the requests across subscribers 

based on load, and how recently they have been curtailed.

Demand Response Management

System (DRMS)

 Regulator

 Scheduling Agent

«role»

«role»

«role»

«role»

«generalization»

«generalization»

«generalization»

«role»

«role»

«role»

«role»

«role»

«role»

«role»


Figure 3-15.  Actors
3.1.9.1
Billing Agent
MERGEFIELD Element.NotesThe billing agent determines the cost including applicable credits for customers.  Typically costs vary among different classes of customers.
3.1.9.2
Customer
MERGEFIELD Element.NotesA customer of a utility, including customers who provide more power than they consume.
3.1.9.3
Demand Response Provider
An entity responsible for delivering demand reductions which is compensated in accordance with policy.
3.1.9.4
Distributor
MERGEFIELD Element.NotesA system which transfers energy from generation systems to consumers.

3.1.9.5
ISO
MERGEFIELD Element.NotesAn Independent System Operator (ISO).

3.1.9.6
ISO or Grid Operator
MERGEFIELD Element.NotesAn organization which manages the grid.

3.1.9.7
Large C/I Customer and Co-Generator
MERGEFIELD Element.NotesA customer whose usage typically exceeds a threshold and which is capable of providing electricity to the grid.
3.1.9.8
Metering Agent
MERGEFIELD Element.NotesRecords the DR meter information during the time of the DR event onto persistent media.  Provides DR event meter data to the MDMS. Typically a smart meter.

3.1.9.9
Regulator
Public commission that approves demand response programs.
3.1.9.10
Scheduling Agent
MERGEFIELD Element.NotesSometimes is an IOU and sometimes is independent operator who provides an estimate of system demand.

3.1.9.11
Settlement Agent
MERGEFIELD Element.NotesA system which provides the accounting services necessary to determine payments and bills for customers and energy providers including 3rd party aggregators.
3.1.9.12
Small-Scale Merchant Generator
MERGEFIELD Element.NotesA customer whose usage typically exceeds a threshold and which is capable of proving electricity to the grid.
 SHAPE  \* MERGEFORMAT 



Figure 3-16.  Primary Use Case Diagram
3.2
Manage Energy Resource
3.2-1
MERGEFIELD Element.NotesThe DR solution shall manage demand resources.

3.2-2
The DR solution shall manage supply resources.
3.2-3
The DR solution shall manage resources which combine characteristics of supply and demand resources.  Resources which combine characteristics of both supply and demand include devices which can be controlled to either take or discharge load such as energy storage, pumped storage hydroelectricity, PEV batteries, compressed air systems, and new systems not yet developed. 

3.2-4
The DR solution shall communicate with organizations that manage the grid. Organizations which manage the grid include ISOs, RTOs, and corporations such as TVA, BPA, and others.
3.2-5
The DR solution shall provide for load control provided by independent service providers.
3.2-6
The DR solution shall provide for load control provided by third party Aggregator of Retail Customers (ARC).
3.3
Manage Demand
3.3-1
MERGEFIELD Element.NotesThe DR solution shall provide the capability to manage demand for grid reliability purposes.
3.3-2
The DR solution shall provide the capability to manage demand for economic reasons.
3.3-3
The DR solution shall provide the capability to manage demand for maintenance purposes.
3.3-4
The DR solution shall provide the capability to manage demand in response to price signals.
3.3-5
The DR solution shall provide the capability to specify the availability of resources for demand management on a daily basis.

3.4
Manage Demand for Economic Effect
3.4-1
MERGEFIELD Element.Notes The DR solution shall provide the ability to manage economic effect programs. Economic effect is defined as the ability to reduce overall energy cost for example by trading load generation with load reduction.
3.4-2
The DR solution shall manage the transmission of economic cost information to third party aggregators.  
3.4-3
The DR solution shall interact with customers to convey carbon usage information.

3.5
Manage Demand through Direct Load Control
3.5-1
MERGEFIELD Element.NotesThe DR solution shall provide the ability to manage direct load control programs.
3.5-2
The DR solution shall manage the transmission of direct load control actions to direct-load-control-enabled devices.
3.5-3
The DR solution shall interact with customers to provide direct load control event notification. Studies indicate that customers want to know when direct load control measures are in effect.
3.5-4
The DR solution shall interact with customers to convey direct load control information.
3.6
Manage Demand for Maintenance Purpose
3.6-1
The DR solution shall provide the ability to manage load reduction to support maintenance programs. Load reduction to support maintenance programs reduces demand when resources, typically a circuit, are taken off-line and demand needs to be reduced to facilitate maintenance in the field.   MERGEFIELD Element.Notes
3.7
Manage Demand in Respond to Pricing Signal
3.7-1
MERGEFIELD Element.Notes The DR solution shall provide the ability to manage pricing signal programs designed to reduce load. Pricing signals are any type of dynamic pricing information including time-of-use (TOU), real-time pricing (RTP), and critical peak pricing (CPP) data distributed to customers either a day ahead or periodically during the day.
3.7-2
The DR solution shall manage the transmission of price signal information to DR-enabled devices.
3.7-3
The DR solution shall provide interact with customers to convey price signal information. Studies indicate that customers who understand the cost of electricity reduce their usage, especially when prices are high.
3.8
Manage DR Programs
[image: image23.emf]uc Manage DR Programs

 Create DR Program

 Read DR Program

 Update DR Program 

 Delete DR Program

 DR Program Adminstrator

 Enroll Customer

 Dis-enroll Customer

 Administer Program

 Manage Program 

Customer

 Execute Program

 Supply Interruptible 

Resource

 Supply 

Non-Interruptible 

Resource

 Customer

(from Actors)

 Measurement and 

Verification

«include»

«include»

«include»

«include»

«include» «include»

«include»

«include»

«include»

«include»


Figure 3-17.  Manage DR Programs Use Case Diagram
3.8-1
MERGEFIELD Element.NotesThe DR solution shall provide the ability to manage multiple DR programs.
3.8-2
The DR solution shall provide the ability to support customizable programs.
3.8-3
The DR solution shall provide the ability to manage programs which control demand.

3.8-4
The DR solution shall provide the ability to manage programs which control supply.

3.8-5
The DR solution shall communicate with the DR program administrator(s).
3.8-6
The DR solution shall manage existing demand response programs.

3.8-7
The DR solution shall manage new demand response programs.
3.8-8
The DR solution shall manage existing supply response programs.

3.8-9
The DR solution shall manage new supply response programs.
3.8-10
The DR solution shall manage existing load shifting programs. An example of a load shifting program includes pre-cooling air conditioning system. e DR solution shall manage existing load shifting programs.

3.8-11
The DR solution shall manage new load shifting response programs.
3.8.1
Administer Program
3.8.1-1
MERGEFIELD Element.NotesThe DR solution shall manage multiple programs.

3.8.1-2
The DR solution shall allow customers to be in multiple programs.
3.8.1-3
The DR solution shall allow a program to be applied for all customers.

3.8.1-4
The DR solution shall allow a program to be applied to all members of a class of customers.
3.8.1-5
The DR solution shall allow a program to be applied for some but not all classes of customers.
3.8.1-6
The DR solution shall not force enrollment in any program.
3.8.1-7
The DR solution shall allow a program to be the default program for all customers.

3.8.1-8
The DR solution shall allow a program to be the default program for some but not all classes of customers.

3.8.1-9
The DR solution shall allow a program to be the default program for a class of customers.

3.8.2
Create DR Program
3.8.2.1
MERGEFIELD Element.NotesThe DR solution shall support the creation of new DR programs. The DR program administer typically performs these steps to create a new program:
1. Determine the need for program

2. Do an analysis of the current load

3. Consider effects of regulations/coop rules /city board issues

4. Determine goals of program

5. Design DR program

6. Simulate the effect of the new program
7. Brief Senior Management & Acquire DR Prog. Approval
8. Brief regulators (PUC) /City board (city) /Owner board & USDA (coops) to receive approval

3.8.2.3
The DR solution shall simulate the predicted effect of a new program.   A typical sequence of steps used to simulate predicted effects of a new program is:
	1. Determine need for program

2. Do analysis of current load

3. Consider effects of regulations/coop rules/city board issues

4. Determine goals of program

5. Design DR program

6. Simulate effect of program
7. Brief Senior Management & Acquire DR Prog. Approval
8. Brief Regulators/City Board (city)/Owner board & USDA for Coops & receive approval


3.8.3
Delete DR Program
3.8.3-1
MERGEFIELD Element.NotesThe DR solution shall support the removal of a DR program.
3.8.4
Enroll Customer
3.8.4-1
MERGEFIELD Element.NotesThe DR Solution shall provide the capability to enroll customers in DR programs.

3.8.4-2
The DR Solution shall provide the ability to communicate with customers via multiple methods to enroll them in DR programs.  Typically, customers communicate with a customer service representative or a web portal interface to enroll in a program.
3.8.4-3
The DR solution shall communicate with the DR Program Administration to verify program enrollment eligibility. 


3.8.4-4
The DR solution shall communicate with the DR Program Administration to request program enrollment eligibility.


3.8.4-5
The DR solution shall communicate with the DR Program Administration to check program enrollment eligibility.
3.8.5
Dis-enroll Customer
3.8.5-1
MERGEFIELD Element.NotesThe DR Solution shall provide the capability to dis-enroll customers in DR programs.

3.8.5-2
The DR Solution shall provide the ability to communicate with customers via multiple methods to dis-enroll them in DR programs.  Typically, customers communicate with a customer service representative or a web portal interface to dis-enroll in a program.
3.8.5-3
The DR solution shall communicate with the DR Program Administration to verify program dis-enrollment eligibility. 

3.8.5-4
The DR solution shall communicate with the DR Program Administration to request program dis-enrollment eligibility.
3.8.5-5
The DR solution shall communicate with the DR Program Administration to check program dis-enrollment eligibility.
3.8.6
Execute Program
3.8.6-1
MERGEFIELD Element.NotesThe DR solution shall manage load reduction program execution.

3.8.6-2
The DR solution shall manage load shifting program execution. An example of load shifting includes thermal energy storage.

3.8.6-3
The DR solution shall manage programs which increase energy supply. Examples of programs which increase supply include auxiliary generators and discharging PEV batteries.
3.8.6-4
The DR solution shall manage programs which decrease the energy supply.
3.8.7
Manage Program Customer
3.8.7-1
MERGEFIELD Element.NotesThe DR solution shall provide the capability to manage multiple customers.

3.8.7-2
The DR solution shall provide the capability to manage classes of customers.  Typical customer classes include industrial, commercial, agricultural and residential.

3.8.7-3
The DR solution shall provide the capability to add new classes of customers.

3.8.7-4
The DR solution shall provide the capability to delete a class of customers.

3.8.7-5
The DR solution shall provide the capability for customers in different customer classes to enroll in the same program.

3.8.7-6
The DR solution shall provide the capability to restrict programs to particular customer classes.
3.8.8
Measurement and Verification
3.8.8-1
MERGEFIELD Element.NotesThe DR solution shall provide the ability to measure the load reduction from DR program execution.
3.8.8-2
The DR solution shall be able to estimate the predicted amount of load reduction for a DR program event.

3.8.8-3
The DR solution shall be able to calculate baseline usage for DR program participants.


3.8.8-4
During DR event execution, the DR solution shall be able to calculate the actual load reduction for some programs on a customer by customer basis. 

3.8.8-5
During DR event execution, the DR solution shall be able to calculate the difference between the actual load reduction and the predicted load reduction for a geographic area. 

3.8.8-6
During DR event execution, the DR solution shall be able to calculate the difference between the actual load reduction and the predicted load reduction for some individual customers.

3.8.8-7
During DR event execution, the DR solution shall be able to calculate the difference between the actual load reduction and the predicted load reduction at step down transformers.

3.8.8-8
During DR event execution, the DR solution shall be able to calculate the difference between the actual load reduction and the predicted load reduction at branch circuits.

3.8.8-9
During DR event execution, the DR solution shall be able to calculate the difference between the actual load reduction and the predicted load reduction at distribution feeders.


3.8.8-10
During DR event execution, the DR solution shall be able to calculate the difference between the actual load reduction and the predicted load reduction at substations. 
3.8.8-11
The DR solution shall validate settlement in non-real time to correctly bill customers.
3.8.8-12
The DR solution shall provide the ability to validate the actual load reduction from DR program execution.
3.8.8-13
The DR solution shall provide the ability to store the load reduction from DR program execution on a customer by customer basis.
3.8.8-14
The DR solution shall provide the ability to determine the credit earned by customers enrolled in DR programs.

3.8.8-15
The DR solution shall provide the ability to determine penalties for customers who fail to meet specified levels of load reduction.
3.8.8-16
The DR solution shall provide the ability to determine the credit earned by customers who are by default covered by programs not requiring enrollment.
3.8.9
Read DR Program
3.8.9-1
MERGEFIELD Element.NotesThe DR solution shall support reading of the DR program parameters.
3.8.10
Supply Interruptible Resource
3.8.10-1
MERGEFIELD Element.NotesThe DR solution shall be capable of reducing load by providing interruptible power. Interruptible power is typically produced by generators.

A typical sequence of events to supply interruptible resources is:
1. ISO/Grid Operator releases event

2. ISO/Grid Operator selects DR resource to activate

3. ISO/Grid Operator submits to Scheduling Agent selected DR resource

4. Scheduling Agent posts schedule for DR resource

5. DR resource Provider executes DR event (provides)
6. DR resource Provider notifies customer of DR event

7. Customer acknowledges DR event 
8. Customer provides DR resource

9. Customer reports DR resource status to DR resource provider

10. DR event ends (timeout or DR resource provider terminates event)

[image: image24.emf]sd Supply Interruptible DR Resource

 Demand Response Provider

(from Actors)

 Scheduling Agent

 ISO or Grid

Operator

(from Actors)

 Customer

(from Actors)

     Sequence of Events



1 ISO/Grid Operator releases event

2. ISO/Grid Operator selects DR resource to activate

3. ISO/Grid Operator submits to Scheduling Agent selected DR 

resource

4. Scheduling Agent posts schedule for DR resource

5. DR resource Provider executes DR event (provides) 

6. DR resource Provider notifies customer of DR event

7. Customer acknowledges DR event

8. Customer provides DR resource

9. Customer reports DR resource status to DR resource provider

10. DR event ends (timeout or DR resource provider terminates 

event)

1.0 Releases Event()

1.1 Selects DR resource to activate()

1.2 Submits Selected Dr Resource()

1.3 Posts Resource()

1.4  Executes DR Event()

1.5  Notify Customer of DR

Event()

1.6 Acknowledge DR

Event()

1.7  Provide DR Resource()

1.8 Report DR resource

status()


Figure 3-18.  Supply Interruptible DR Resource
3.8.11
Supply Non-Interruptible Resource
3.8.11-1
MERGEFIELD Element.NotesMERGEFIELD Element.NotesThe DR solution shall have the capability to reduce load using non-interruptible resources.  Non-interruptible resources include geothermal, solar and wind. Non-interruptible resources are also called non-dispatchable resources.



A typical sequence of events to supply non-interruptible resources is:
1. DR provider submits DR bid to Scheduling Coordinator

2. Scheduling Coordinator validates DR bid

3. Scheduling Coordinator posts DR bid to ISO/grid operator

4. ISO awards and schedules DR resource  
5. Scheduling Coordinator notified DR resource provider of award and schedule

6. DR resource Provider executes DR event (provides)
7. DR resource Provider notifies customer of DR event

8. Customer acknowledges DR event

9. Customer provides DR resource

10. Customer reports DR resource status to DR resource provider

11. DR event ends (timeout or DR resource provider terminates event)

[image: image25.emf]sd Supply Non-Interruptible DR Resource

 Demand Response Provider

(from Actors)

 Scheduling Agent

 ISO or Grid

Operator

(from Actors)

 Customer

(from Actors)

1. DR provider submits DR bid to Scheduling Coordinator

2. Scheduling Coordinator validates DR bid

3. Scheduling Coordinator posts DR bid to ISO/grid operator

4. ISO awards and schedules DR resource

5. Scheduling Coordinator notified DR resource provider of award 

and schedule

6. DR resource Provider executes DR event (provides) 

7. DR resource Provider notifies customer of DR event

8. Customer acknowledges DR event

9. Customer provides DR resource

10. Customer reports DR resource status to DR resource provider

11. DR event ends (timeout or DR resource provider terminates 

event)

1.0 Submit DR Bid()

1.1 Validates DR Bid()

1.2  Posts DR

Bid()

1.3 Awards DR

Resource()

1.4 Award and Schedule()

1.5  Executes DR Event()

1.6  Notify Customer of DR

Event()

1.7 Acknowledge DR

Event()

1.8  Provide DR Resource()

1.9 Report DR resource

status()


Figure 3-19.  Supply Non-Interruptible DR Resource
3.8.12
Update DR Program 
3.8.12-1
MERGEFIELD Element.NotesThe DR solution shall support modifying of the DR program parameters.

3.8.12-2
The DR solution shall support common program parameters such as TOD, % reduction, amount paid to customers, maximum number of events which can be called, kWh to be reduced, credits earned per unit of reduction, etc.  


A typical sequence of events to update a DR program is:

	1. Determine need for changes

2. Do analysis of current load

3. Consider effects of regulations/coop rules /city board issues

4. Determine goals of program

5. Design DR program

6. Simulate effect of program

7. Brief Senior Management & Acquire DR Prog. Approval

8. Brief Regulators/City Board (city)/Owner board & USDA for Coops & receive approval



3.8.13
Manage Statistics 

3.8.13-1
The DR solution shall manage the collection of the percent of active devices which responded to a DR event.


3.8.13-2
The DR solution shall manage the collection of the percent of customers who responded to a DR event.


3.8.13-3
The DR solution shall manage the collection of the percent of customers who opted out of a DR event.
3.8.13-4
The DR solution shall manage the collection of the average response latency to a DR event. 


3.8.13-5
The DR solution shall calculate statistics for the collected data.  


3.8.13-6
The DR solution shall calculate statistics for the collected data including standard deviations.
3.9
Manage DR Customer
3.9-1
MERGEFIELD Element.NotesThe DR solution shall manage customers for DR purposes. 
3.9-2
The DR solution shall communicate with customers.

3.9-3
The DR solution shall manage existing customers.

3.9-4
The DR solution shall manage new customers.
3.10
Provision Demand Response Equipment
3.10-1
MERGEFIELD Element.NotesThe DR solution shall manage the provisioning of active DR equipment.

[image: image26.emf]uc Provision Demand Response Equipment

 Add DR Device

 Remove DR Device

 Read DR Device 

Information

 Update DR Device

 CS - Customer 

Support

(from Business Functions)


Figure 3-20.  Provision Demand Response Equipment
3.10.1
Add DR Device
3.10.1-1
MERGEFIELD Element.NotesThe DR solution shall manage the provisioning of new active DR devices.
3.10.1-2
If a DR device is capable of self-configuring on a network, then the DR solution shall be capable of commissioning the device into the DR program.
3.10.1-3
The DR solution shall maintain a database of all active DR devices.
3.10.1-4
The DR solution shall manage the provisioning of existing active DR devices. 

3.10.1-5
The DR Solution shall validate the location of DR devices.
3.10.1-6
The DR solution shall manage the provisioning of new types of future DR devices.

3.10.1-7
The DR Solution shall be capable of adding DR device types.
3.10.2
Remove DR Device
3.10.2-1
MERGEFIELD Element.Notes The DR solution shall maintain records of all active DR devices removed for a time period as defined by a regulatory agency.
3.10.2-2
The DR solution shall manage the decommissioning of existing DR devices.
3.10.2-3
The DR solution shall manage the removal of existing DR devices from a program.

3.10.2-4
The DR solution shall manage the removal of existing DR devices from a customer.

3.10.2-5
The DR solution shall manage the removal of existing DR devices from a list of premise equipment. 

3.10.2-6
The DR Solution shall be capable of removing obsolete DR device types.
3.10.3
Read DR Device Information
3.10.3-1
MERGEFIELD Element.Notes The DR solution shall read information from active DR devices.
3.10.3-2
The DR solution shall maintain the information read from active DR devices.
3.10.3-3
The DR solution shall manage changing the recording rate of selected meters on selected programs.

3.10.3-4
The DR solution shall manage changing the reporting rate of selected meters on selected programs.
3.10.4
Update Active DR Device
3.10.4-1
MERGEFIELD Element.NotesThe DR solution shall be capable of updating active DR device information. 

The DR solution shall maintain up-to-date information for all active DR devices.
3.10.4-2
The DR solution shall manage the updating of existing active DR devices.

3.10.4-3
The DR solution shall be capable of remotely updating active DR devices from the utility.
3.10.4-4
The DR solution shall be capable of remotely updating DR devices.

3.10.4-5
The DR solution shall validate updates of remote devices.

3.10.4-6
The DR solution shall be capable of updating the attached load.

3.11
Manage Supply
3.11-1
MERGEFIELD Element.NotesThe DR solution shall manage the available supply of energy.  Attributes of energy supply include type of supply energy capacity, reliability, regulation amount of supply in mW, duration, number of times per day that it can be supplied.

 3.11-2
The DR solution shall provide the capability to manage supply through price signals.
3.11-3
The DR solution shall provide the capability to manage supply through direct load control.
3.12
Manage Supplier
3.12-1
MERGEFIELD Element.NotesThe DR Solution shall allow commercial customers to supply electricity to the grid.
3.12-2
The DR solution shall allow residential customers to supply energy to the grid.

3.13
Manage Supply through Price Signal
3.13-1
MERGEFIELD Element.NotesThe DR solution shall provide the ability to manage pricing signal programs designed to increase supply.
3.13-2
The DR solution shall manage the transmission of price signal information to enabled devices.

3.13-3
The DR solution shall interact with customers to convey price signal information.
3.14
Manage Supply through Direct Control
3.14-1
MERGEFIELD Element.NotesThe DR solution shall provide the ability to manage direct supply control programs.
3.14-2
The DR solution shall manage the transmission of direct supply control actions to direct-control-enabled devices.

3.14-3
The DR solution shall provide interact with customers to convey direct supply control information.
3.14-4
The DR solution shall interact with customers to provide supply event notification.
3.15
Provision Supply Response Equipment
3.15-1
MERGEFIELD Element.NotesMERGEFIELD Element.NotesThe DR solution provisions equipment designed to increase supply. An example of equipment designed to increase supply is an additional meter which measures the output from a customer’s system which provides electricity to the grid. Another example would be a backup generator.
3.15-2
The DR solution shall be capable of adding a supply resource to a DR program. 

3.15-3
The DR solution shall be capable of managing a supply resource. 


3.15-4
The DR solution shall be capable of measuring a supply resource response to a DR event.


3.15-5
The DR solution shall be capable of verifying a supply resource response to a DR event.
3.15-6
The DR solution shall have the capability to manage multiple meters on a single customer site.
4.0
DR Event Class

This DR Event Class diagram shows the association between the main classes in one potential DRMS system decomposition.
[image: image27.emf]class DR Event

 DResourceServiceType

 DRProgram

 DRCustomer

 DRDevice

 DRResourceCustomAggregationGroup

 DREvent


Figure 4-1.  DR Event Class Diagram























































































































































































































































































































































































Version 1.0, June 30, 2009

© Copyright 2009, AMI, All Rights Reserved


[image: image29.emf]uc Primary Use Cases

 Manage Demand

 Provision Demand 

Response Equipment

 Manage DR Customer

 Manage Energy 

Resource

 Manage Supply

 Manage DR Programs

 Manage Demand for 

Economic Effect

 Manage Demand 

through Direct Load 

Control

 Manage Supply 

through Price 

Signal

 Manage Supply 

through Direct 

Control

 Manage Supplier

 ISO or Grid Operator

 Customer

 Distributor

 Provision  Supply 

Response Equipment

 Small-Scale Merchant 

Generator

 Large C/I Customer and 

Co-Generator

 Demand Response 

Provider

 Add DR Device

 Remove DR Device

 Manage Demand for 

Maintenance Purpose

 Manage Demand in 

Respond to Pricing 

Signal

 Read DR Device 

Information

 Update DR Device

«precedes»

«precedes»

«include»

«precedes»

«precedes»

«include»

«include»

«precedes»

«include»

«include»

«include»

«include»

«include»

«include»

«include»

«precedes»

«include»

«include»

_1300978284.vsd
Measure DR Response


DR Measurement and Verification


Verify DR Response


Post Results



_1300978283.vsd
Execute DR Event Response


Forecast DR Resource Availability


Monitor and Store Event Data


Create DR Event Resource 


DR Resource Behavior


DR Resource Baseline 


Select DR Resources/Custom Aggregation Group


Select DR Resource Service Type



