# Retail DR Use Cases

| Release | 0.1 |
| --- | --- |
| Version: | DRAFT 1.1 |
| Created: | Friday, July 31, 2026 |
| Last Update: |  |
| Print Date: |  |
| By: | NAESB Smart Grid Task Force |
| Distribution: | Public |

Table of Contents

Use Case Document	1

1    Introduction	6

1.1    Purpose of Document	6

1.2    Glossary	6

2    Application Overview	6

2.1    Define the Scope	6

2.2    Context	6

2.3    Technical Environment	6

Use Cases	7

3    Model	7

3.1    NAESB Demand Response Use Cases for NIST Smart Grid	7

<anonymous>	8

3.1.1    Actors	9

Commercial Customer	9

Curtailment Service Provider	9

DMS	10

DR Aggregator	10

DR Asset	10

DR Asset/Resource Owner	10

DR Controlling Entity	10

DR Program Participant	11

DR Resource	11

DRMS	11

Demand Response Program	11

Distributed Energy Resource	12

Distribution Company	12

Distribution Operator	12

Electricity Consumer	12

Energy Service Provider	12

Industrial Customer	13

LMS	13

Load Serving Entity	13

Merchant Operator	13

Metering Agent	13

Regulator	13

Residential Customer	14

System and Market Operator	14

3.1.2    DR Use Cases	14

3.1.2.1    1.0 Administrate DR Program	15

1.0 Administrate DR Program	16

1.1 Create DR Program	16

Sequence Diagram	17

1.2 Update DR Program	18

Sequence Diagram	18

1.3 Remove DR Program	19

Sequence Diagram	20

3.1.2.2    2.0 Administrate Customer for DR	20

2.0 Administrate Customer for DR	21

2.1 Register / Enroll Customer for DR Program	21

Sequence Diagram	22

2.2 Update Customer Identity	23

Sequence Diagram	23

2.3 Remove Customer from DR Program	24

Sequence Diagram	25

3.1.2.3    3.0 Administrate DR Resource	25

3.0 Administrate DR Resource	26

3.1 Administrate Bulk Power DR Resource	26

3.2 Administrate Distribution DR Resource	27

3.2.1 Declare DR Resource	27

Sequence Diagram	28

3.2.2 Register DR Resource	29

Sequence Diagram	31

3.2.3 Remove DR Resource	31

Sequence Diagram	31

3.2.4 Administrate DR Asset (Direct)	32

3.2.4.1 Register DR Asset	32

Sequence Diagram	33

3.2.4.2 Update DR Asset	33

Sequence Diagram	34

3.4.2.3 Remove DR Asset	34

Sequence Diagram	35

3.2.5 Administrate DER for DR Purpose	35

3.3 DR Bidding	35

3.3.1 DR Bid to Supply (Offers)	36

Sequence Diagram	37

3.3.2 DR Bid to Buy	37

Sequence Diagram	38

3.1.2.4    4.0 Execute DR Event	39

4.0 Execute DR Event	39

4.1 Notify DR Event (Bulk Power)	40

4.1.1 Advanced Notification for DR (Bulk Power)	40

Sequence Diagram	41

4.1.2 Update a DR Event (Bulk Power) 	41

4.1.3 Cancel a DR Event (Bulk Power)	42

Sequence Diagram	42

4.1.4 DR Resource Confirmation (Bulk Power)	43

4.2 Notify DR Event (Distribution)	43

4.2.1 Advanced Notification for DR (Distribution)	43

Sequence Diagram	45

4.2.2 Update a DR Event (Distribution)	46

Sequence Diagram	46

4.2.3 Cancel a DR Event (Distribution)	46

Sequence Diagram	46

4.2.4 DR Resource Confirmation (Distribution)	47

Sequence Diagram	48

4.3 Broadcast Pricing (Wholesale) for DR Purpose	48

Sequence Diagram	50

4.4 Broadcast Pricing (Retail) for DR Purpose	50

Sequence Diagram	51

4.5 Dispatch DR Instructions (Bulk Power)	51

Sequence Diagram	53

4.5.1 Monitor DR Event (Bulk) 	53

4.6 Dispatch DR Instructions (Distribution)	53

Sequence Diagram	54

4.7 DR Direct Load Control (Distribution)	55

Sequence Diagram	55

4.7.1 Monitor DR Event (DR Resource)	55

Sequence Diagram	56

4.7.2 Monitor DR Event (DR Asset)	56

Sequence Diagram	57

4.8 DR Event Execution	57

4.8.1 DR Execution - Dynamic Price-Based	57

Sequence Diagram	57

4.8.2 DR Execution - Notification Based	58

Sequence Diagram	58

4.8.2 DR Exedcution - Direct Load Control	58

Sequence Diagram	59

4.9 Operational Coordination between Transmission, Distribution and DR Resources	59

Sequence Diagram	59

Adjust Schedule	60

Apply Retail Uplift Charges	61

Assess Congestion Uplift Charges	61

Impacted Curcuit Analysis	61

Load Forecast by Circuit	61

Specify Load Profile	61

3.1.2.5    5.0 Post DR Event Management	61

5.0 Post DR Event Management	62

5.1 Post DR Event M&V / Settlement (Open Wholesale, No Open Retail)	62

Sequence Diagram	63

5.2 Post DR Event M&V / Settlement (No Open Wholesale, No Open Retail)	63

Sequence Diagram	63

5.3 Post DR Event M&V / Settlement (Open or Not Open Wholesale, Open Retail)	64

Sequence Diagram	64

# Introduction

This document is not a final product, but it does contain a great deal of information which the NAESB Smart Grid Task Force (SGTF) believes is beneficial to distribute to the community at large for their review.  Nonetheless, it is a work in progress.  The Retail DR Use Cases are primarily focused on retail use cases, but can easily be expanded to show the connection to the wholesale market place.  Within the retail sector, there are two basic situations: retail competition or no retail competition.  The no retail competition case could have been shown as the most simple retail competition case where there is only a single retail entity.  At this time, there may still be some overlap between some of the wholesale actors in the retail use cases, but these use cases will be cleaned up before the recommendations from the SGTF are finalized.  The NAESB SGTF co-chairs encourage all stakeholders to join or continue their participation in the efforts of the SGTF.

## Purpose of Document

This is to be a standard paragraph to lay out the Business Logic and the elements within the package that describe the Use Cases for the package under development.  This text is definable as a template by the user and stored locally.  The system variables are filled in by EA via the RTF Document Generator.

## Glossary

| ##### Term | ##### Acronym | ##### Definition | ##### Source |
| --- | --- | --- | --- |
| ##### Aggregation of Retail Customers | ##### ARC | ##### See Resource |  |
| ##### Demand Management System | ##### DMS | ##### A system that manages the distribution network operations. |  |
| ##### Demand Response | ##### DR | ##### Abbreviated DR |  |
| ##### Demand Response Program | ##### DRP | ##### A defined set of procedures and rules that enable Electricity Consumers to temporarily change electricity consumption of a DR Asset or DR Resource in response to market or reliability conditions. For purposes of these standards, Demand Response does not include energy efficiency or permanent Load reduction. |  |
| ##### Demand Response Management System | ##### DRMS | ##### A system that manages the demand response programs from utility point of view. It includes load control, integration with DMS, and DR program management, and uses historical and externally input data to make predictions and what-if analysis for DR purposes. |  |
| ##### Demand Side Management | ##### DMS | ##### The process utilized by a utility distribution company to control the grow of their peak load either through energy efficiency or demand response. |  |
| ##### Load Management System | ##### LMS | ##### Load management systems that include load forecast. |  |
| ##### Market Participant | ##### MP | ##### An organization registered with the System Operator that may take on roles such as DRP, LSE, UDC, SE and/or MA in accordance with the System Operator’s market rules. |  |
| ##### Regulator |  | ##### Regulators are state public service commissioners who regulate essential utility services, such as electricity, gas, telecommunications, water, and transportation throughout the country. As regulators, they are charged with protecting the public and ensuring that rates charged by regulated utilities are fair, just, and reasonable. The term regulator can also apply at the federal level to include the Federal Energy Regulatory Commission (FERC). |  |

# Application Overview

## Define the Scope

## Context

The purpose of this section is is to give a brief definition of the context in which the application will be implemented, specifying the relationship this system will have to existing systems within the environment.

## Technical Environment

The purpose of this section is to give a brief definition of any applications relevant to the system being implemented.  The relationship between these, as well as any aspects upon which this system is reliant, must be defined.

# Use Cases

# Model

## NAESB Demand Response Use Cases for NIST Smart Grid

NAESB Demand Response Use Cases for NIST Smart Grid

This package contains the set of Use Cases for Demand Response scenarios which may be implemented within a NIST standard Smart Grid.

Figure 1: NAESB-NIST-SMART-GRID-DR-USE-CASES

Actors are the users of the system being modeled. Each Actor will have a well-defined role, and, in the context of that role, have useful interactions with the system.

A person may perform the role of more than one Actor, although they will only assume one role during one use case interaction.

An Actor role may be performed by a non-human system, such as another computer program.

### Actors

Actors

Figure 2: Actors

| ##### Actor | ##### Acronym | ##### Definition | ##### Source |
| --- | --- | --- | --- |
| ##### Commercial Customer |  | ##### A commercial business entity that consumes electricity, typically with demand between 20 – 200 kW. |  |
| ##### Demand Response Asset |  | ##### An energy resource that is capable of shedding load in response to Demand Response Events, Electricity Price Signals or other system events (e.g. under frequency detection). ##### Examples of Demand Response Assets are: Smart Appliances, entire buildings that are under the control of an Energy Management System, Electric Vehicles. |  |
| ##### Demand Response Asset Owner |  | ##### The entity that is responsible for the DR Resource or Asset in the retail market. |  |
| ##### Demand Response Controlling Entity |  | ##### This is a generalized actor class and represents all the different entities that may need to manage and interact with wholesale and/or retail DR resources. It includes the following actors: ISO/RTO, Distribution Company, Load Serving Entity, DR Aggregator. |  |
| ##### Demand Response Program Participant | ##### DRPP | ##### This may be a DRP, LSE, UDC or Electricity Consumer. |  |
| ##### Demand Response Provider | ##### DRP | ##### An entity or role with the responsibility to coordinate demand resources to deliver demand response services. A DRP includes entities that have acquired curtailment rights from electricity consumers, and manage the aggregation of capacity that is curtailable as an eligible energy or capacity resource for participating in Demand Response programs. |  |
| ##### Demand Response Resource |  | ##### A DR resource is a virtual representation of one or more DR assets. ##### It is similar to a DR Asset in that it is capable of shedding load in response to a triggering event. Unlike a DR Asset, which is autonomous, a DR Resource may consist of multiple DR Assets that have been aggregated to form a larger capacity or energy resource. ##### An apartment building with multiple electricity consumers, each one having one or more DR Assets may be considered one large DR Resource by aggregating the total load shedding capacity of all the DR Assets in the apartment building and representing the sum total of this capacity as one DR Resource. ##### A DR Resource may also consist of different types of Assets (e.g., a wind Turbine and an electric motor that work in combination to meet DR program obligations). |  |
| ##### Designated Dispatch Entity | ##### DDE | ##### An entity or role which carries the responsibility of receiving and processing demand resource dispatch instructions or market information and (optionally) providing demand response information. |  |
| ##### Distributed Energy Resource | ##### DER | ##### Distributed energy resources are small, modular, energy generation and storage technologies that provide electric capacity or energy where it is needed.* ##### A DER may be implemented by an Electricity Consumer to generate electricity for its own use during DR Events. In order to meet load shedding obligations, it may have within a Demand Response Program. ##### *Definition of DER provided by the Department of Energy, http://www1.eere.energy.gov/femp/pdfs/31570.pdf | ##### Department of Energy* |
| ##### Electricity Consumer |  | ##### Any entity that consumes electricity. This term includes Commercial Customer, Industrial Customer and Residential Customer. |  |
| ##### Energy Service Provider | ##### ESP | ##### A business entity that provides energy services to end users in a deregulated retail market. |  |
| ##### Industrial Customer |  | ##### An industrial business entity that consumes electricity, typically with demand higher than 200 kW. |  |
| ##### Load Serving Entity | ##### LSE | ##### The entity that is responsible for serving the Electricity Customers’ electricity needs. An LSE may also perform additional roles such as DRP and DDE. |  |
| ##### Metering Authority | ##### MA | ##### An entity or role that carries the responsibility of providing the data necessary to determine the performance of a Resource. |  |
| ##### Residential Customer |  | ##### A residential entity that consumes electricity, typically with demand lower than 20 KW, although this could change with more PHEV adoption at home. |  |
| ##### Scheduling Entity | ##### SE | ##### An entity or role that carries the responsibility of submitting bids/offers and receives Schedules and Awards. |  |
| ##### System and Market Operator | ##### SO or MO | ##### A System Operator is a Balancing Authority, Transmission Operator, or Reliability Coordinator whose responsibility is to monitor and control an electric system in real time (based on NERC definition). The System Operator is responsible for initiating Demand Response Events (e.g., Advance Notifications, Deployment, and Release/Recall instructions). ##### The Market Operator manages the bulk electricity market and produces prices for various products. ##### ISO New England and PJM Interconnection are examples of Market and System Operators that perform this function. |  |
| ##### Utility Distribution Company | ##### UDC | ##### An entity or role that carries the responsibility of operating an electricity distribution system. A Company that delivers electricity to end use customers and can accept energy delivery from end use customers that have some form of generation (DER). |  |
| ##### Utility Distribution Operator | ##### UDO | ##### The operation center of a UDC that is responsible for the reliable delivery of electricity to end users. |  |

### DR Use Cases

DR Use Cases

Figure 3: DR Use Case Overview Diagram

#### 1.0 Administrate DR Program

1.0 Administrate DR Program

Figure 4: 1.0 Administrate DR Program

###### 1.0 Administrate DR Program

This is a high level use case.  See its sub-use cases for details.

###### 1.1 Create DR Program

A DR program is created to meet one of two, or both, situations.  A DR program is created to meet a reliability need, i.e., a projected insufficient energy supply, and/or for economic purposes, i.e., when market prices are peaking.

A properly designed DR program will have a clearly stated goal or objective, System Operator or Program Administrator, activation and operational protocols, and responsibilities and obligations.  The items that should be considered include, but are not limited to:

·    The type and size of the target Customer;

·    The type of program to be implemented,, e.g., Direct Load Control, Firm Service Level, Guaranteed Load Drop, etc.;

·    The type of metering to be used/required;

·    Any regulatory requirements;

·    Method of communication;

·    Activation protocol;

·    Marketing and Sales activities;

·    Customer Education activities;

·    Duration;

·    Delivery Period;

·    LSE Compensation;

·    Data Exchange Protocols;

·    Customer Authorization;

·    Third Party Participation;

·    Registration / Enrollment Process;

·    Approval Process;

·    Baseline Calculations;

·    Settlement Process;

·    Payment Process;

·    M&V Methodology;

·    Contractual Agreements;

·    Dispute Resolution Process;

·    Phase-in or Roll-out Plan.

Once the DR program is developed, it should be tested prior it implementation.  Such testing may be accomplished through a pilot program. The results of the test/pilot should then be compared against the goal/objective of the program and any needed modifications made.  Upon satisfactory results, the DR program should be filed with and approved by the Applicable Regulatory Authority, if any.  Depending on which Entity is creating the DR program, it must also be submitted to and approved by the System Operator.  Finally, the DR program must be implemented.

Upon activation of a DR program, an observable demand reduction should occur.

| Customer Size |  | Functional |
| --- | --- | --- |
| Customer Type |  | Functional |
| Metering Type |  | Functional |
| Program Type |  | Functional |
| Regulatory Authority | State or federal | Functional |
| Tariff Indicator | Is it a tariff based program? | Functional |

Sequence Diagram

###### 1.2 Update DR Program

Once a DR program is initiated, changes in various elements of the program may be required or desired.  After indentifying the specific modification required, each of the considerations identified in the “Create” step should be investigated to insure that no unintended impacts occur as a result of the modification. If such unintended impacts occur, further investigation of the modification is required prior to implementation.

A regulated Entity must then present the modifications to the Applicable Regulatory Authority for approval and, then to the System Operator for approval. Once approved, the modifications must be communicated to Customers.

(Same data requirements as Use Case 1.1)

Sequence Diagram

###### 1.3 Remove DR Program

Termination of a DR program must take into account any DR resource long term commitments in System and Market Operator programs and contractual obligations with DR Aggregators or Load Serving Entities.  In addition, State and or Federal regulatory agency filings and approvals must be considered prior to terminating a DR program.  Termination of a DR program may involve the removal of DR infrastructure, such as Direct Load Control Devices and advanced metering and pulse devices used in the M+V of the program. All outstanding payments to the DR resources should be completed prior to the termination of the DR program.

Data requirements to remove the DR program include, but are not limited to, the following:

·    Contractual Agreements;

·    Any regulatory requirements;

·    Customer Education activities;

·    Payment Process;

·    Method of communication;

·    Third Party Participation;

·    Registration / Enrollment Process.

Sequence Diagram

#### 2.0 Administrate Customer for DR

2.0 Administrate Customer for DR

Figure 5: 2.0 Administrate Customer for DR

###### 2.0 Administrate Customer for DR

This is a high level use case.  See its sub-use cases for details.

###### 2.1 Register / Enroll Customer for DR Program

The registration/enrollment process must capture the key identifiers in order to enable accurate accounting of DR resources.  A requirement for a registration process to track registrations of System and Market Operators or facilitating agencies and the movement of DR resources from one DR aggregator to another as may occur from time to time is necessary. An internet-based application for the registration/enrollment process is ideal for this type of program administration.

| Approval Process |  |  |
| --- | --- | --- |
| Baseline Calculations |  |  |
| Contractual Agreements |  |  |
| Customer Authorization |  |  |
| Delivery Period |  |  |
| DR Administrator | · System and Operator · Utility · DR Aggregators · Energy Consultants |  |
| DR Resources |  |  |
| M&V Methodology |  |  |
| Metering Type | The type of metering to be used/required. |  |
| Program Type | The type of program to be implemented, e.g., Direct Load Control, Firm Service Level, Guaranteed Load Drop, etc. |  |
| Registration / Enrollment Process |  |  |
| Third Party Participation |  |  |

Sequence Diagram

###### 2.2 Update Customer Identity

Once a DR registration is completed, changes in various elements of the registration may be required or desired.  After identifying the specific modification required, each of the considerations identified in the register/enroll step should be investigated to ensure that it is accurate and complete.

All parties (Utility, Aggregators, Energy Consultants, DR resources, Regulatory Agencies) affected by the update should be given reasonable time to review, dispute, and approve/disapprove the update(s).

(Same Data Requirements as Use Case 2.1)

Sequence Diagram

###### 2.3 Remove Customer from DR Program

Removing a Customer Entity from a DR Program enrollment must take into account any DR resource long term commitments in System and Market Operator programs, and contractual obligations with DR Aggregators or Load Serving Entities.  In addition, State and/or Federal regulatory agency filings and approvals must be considered prior to removing a customer enrollment from a DR program.

| Approval Process |  |  |
| --- | --- | --- |
| Contractual Agreements |  |  |
| Customer ID | Unique identifier of the customer |  |
| DR Administrator | · System and Operator · Utility · DR Aggregators · Energy Consultants |  |
| DR Asset ID | DR asset(s) identifier(s) |  |
| DR Resource ID | DR resource identifier. |  |
| Third Party Participation |  |  |

Sequence Diagram

#### 3.0 Administrate DR Resource

3.0 Administrate DR Resource

Figure 6: 3.0 Administrate DR Resource

###### 3.0 Administrate DR Resource

This is a high level use case.  See its sub-use cases for details.

###### 3.1 Administrate Bulk Power DR Resource

###### 3.2 Administrate Distribution DR Resource

These activities are used to administer all of the DR Resources and Assets, including a variety of activities surrounding the registration of DR Resources and the management of information surrounding the DR Resources that a Controlling entity may call upon during a DR event. Before a DR Resource may be called upon, it must be registered with the Controlling entity.  The administration of a DR Resource covers the following requirements:

- The need of the Controlling entity to identify a DR Resource for communications purposes;

- The need of the Controlling entity to establish DR Resource accounts for accounting purposes;

- The need of the Controlling entity to collect information from DR Resources prior to DR events for the purposes of determining which DR Resources to call upon during a DR Event; and

- The need of the Controlling entity to collect information from DR Resources prior to DR events for the purposes of determining the nature of the DR signals to send to the DR Resources.

3.2.1 Declare DR Resource

| Asset-ID | This uniquely identifies a DR Asset |  |
| --- | --- | --- |
| DR Resorurce Operational Constraints | These are constraints that define the amount load that can be made available during a DR event, including the following: - Minimum load - Maximum load. |  |
| DR Resource Bid | This is a bid from the DR Resource. See the DR Bidding use case for a description of the information requirements. |  |
| DR Resource Schedule Constraints | These are a set of constraints that specify when the DR Resource will be available. It may contain such information as: - Time of day schedule constraints - Black out dates - Maximum consecutive days of participation - Maximum duration of DR event participation - Minimum duration of DR event participation - Max number of times per day the DR Resource may be called - Minimum advanced notification necessary. |  |
| DR Resrouce Identifer | This identifies the DR Resource. |  |
| Effective-Start-Date | The date and time when the declared capabilities will be in effect. |  |
| Hourly-Capacity-Availability | The amount of capacity available for interruption by hour for the period defined by the effective start/end date, expressed in Megawatts in decimal notation with 3 digit precision. |  |
| Message-Exchange-Pattern | The means by which price information is communicated to any party that is interested in this information. The following are valid options: - In-Only - Robust In-Only - In-Out. Refer to http://www.w3.org/TR/wsdl20-adjuncts/#patterns for complete descriptions. |  |
| Monthy-Capacity-Availability | The average capacity available for interruption by month for the period defined by the effective start/end date, expressed in Megawatts in decimal notation with 3 digit precision. |  |
| Source-Authentication | The recipient must have some assurance that the message was produced by a reliable, authoritative party. This requires some ability to validate the authenticity of the source and integrity of the data (e.g., digital signature applied to a price message). |  |
| Temporary Opt-Out | This is used to temporarily opt out of DR Events and to override the normal operational constraints. The opt-out can be specified using the following criteria: - All events in a program indefinitely - Specific DR Event - All events in a specific time period. |  |

Sequence Diagram

3.2.2 Register DR Resource

Description:

A party with ownership, controlling interest, or administrative responsibilities for a DR Asset or DR Resource that communicates status-related operational information about the DR Asset or DR Resource to a controlling entity.

For example, the owner of a DR  Resource or Asset may wish to declare their inability to shed load (an outage) due to Summer shutdown or may wish to reduce the available capacity of a resource/asset due to equipment maintenance or other causes.

| Asset-ID | This uniquely identifies a DR Asset. |  |
| --- | --- | --- |
| Available-Capacity | The total amount of power (megawatts) available from the asset/resource, expressed in integer format representing the amount of kilowatts available. |  |
| Customer Identifier | This is a customer identifier (e.g., account number) that signifies the owner of the DR Resource to the Utility. |  |
| DR Program Identifier | This identifies the DR program in which a DR Resource is participating. |  |
| DR Resource Identifier | This identifies the DR Resource that is being registered. |  |
| DR Resource Operational Constraints | These are constraints that define the amount load that can be made available during a DR event and includes the following: - Minimum load - Maximum load. |  |
| DR Resource Schedule Constraints | These are a set of constraints that specify when the DR Resource will be available. It may contain such information as: - Time of day schedule constraints - Black out dates - Maximum consecutive days of participation - Maximum duration of DR event participation - Minimum duration of DR event participation - Max number of times per day the DR Resource may be called - Minimum advanced notification necessary. |  |
| Effective-Start-Date-Time | The start date and time which an asset/resource is available. |  |
| Location | An identifier to indicate the location of the asset/resource. |  |
| Location-type | A value used to interpret the value contained in the Location element. Examples of Location-type include: - Zone - Node - Zip-code. |  |
| Maximum-Duration | The maximum amount of time the asset/resource is capable of delivering power (megawatts), expressed in hours using decimal notation. For example, a value of 0.5 indicates a maximum duration of 30 minutes which the asset/resource can reduce power consumption by the level indicated in Available Capacity. |  |
| Message-Exchange-Pattern | The means by which price information is communicated to any party that is interested in receiving this information. The following are valid options: - In-Only - Robust In-Only - In-Out. Refer to http://www.w3.org/TR/wsdl20-adjuncts/#patterns for complete descriptions. |  |
| Minimum-Duration | The minimum amount of time that an Asset/Resource Owner is willing to allow the resource/asset to be utilized during a DR event. It is expressed in decimal format representing hours. For example, a value of 1.5 indicates that an asset/resource must be utilized for no less than one hour and 30 minutes during any DR event. |  |
| Parent-Resource ID | This is used by an Asset to indicate that it "belongs to" a particular Resource. Assets may be aggregated into a Resource, but are not required to be so aggregated. Resources are autonomous and may not be aggregated. |  |
| Report-type | This indicates the type of report being issued by the Asset or Resource Owner. This is an enumerated value containing one of the following: - REGISTRATION (to register a new asset/resource) - CHANGE (refers to permanent changes) - RETIREMENT. |  |
| Response-time | The amount of time before an asset/resource is capable of meeting its full performance, in response to a request by a controlling entity to shed load, expressed as minutes in decimal format. |  |
| Source-Authentication | The recipient must have some assurance that the message was produced by a reliable, authoritative party. This will require some ability to validate the authenticity of the source and integrity of the data (e.g., digital signature applied to a price message). |  |

Sequence Diagram

3.2.3 Remove DR Resource

| DR Resource Identifier | This identifies the DR Resource that is being removed. |  |
| --- | --- | --- |

Sequence Diagram

3.2.4 Administrate DR Asset (Direct)

3.2.4.1 Register DR Asset

The DR Asset registration process must capture key identifiers to enable accurate tracking of DR assets and their capabilities. A requirement for a System and Market Operator, Load Serving Entity, DR Aggregator, or other entity facilitating the registration process (hereinafter referred to as DR Asset Administrator) is to track assets. This is done through DR asset registration and association of physical DR assets to DR Resources to recognize the asset and its potential contribution as part of a DR Resource.  The DR Asset Administrator ultimately administers the DR Asset registration process to recognize DR Assets that can serve as part of a DR Resource, although the Customer may be party to the registration process.

| Asset Operator |  |  |
| --- | --- | --- |
| Asset Owner |  |  |
| Asset Physical Location |  |  |
| Authorized Asset Servicers |  |  |
| Date of Registration and Last Update |  |  |
| DR Asset Administrator | - System and Operator - Distribution Utility - Load Serving Entity - DR Aggregators |  |
| DR Asset Availability and Status | Run Status, Setpoint, Override status, etc. |  |
| DR Asset Identifier | Asset Name, ID |  |
| DR Asset Physical Capabilities | - Ramp Up/Down Rate, Maximum Capacity |  |
| DR Asset Product | Manufacturer, Model, Version, Date of Manufacturer |  |
| DR Asset Type | (DG, renewable, storage, curtailable or interruptible load) |  |
| DR Resources |  |  |
| State of Registration Process |  |  |

Sequence Diagram

3.2.4.2 Update DR Asset

Once a DR asset registration is completed, changes to elements of the registration may be required or desired. After identifying the specific required modification and verifying the authorization to perform the update, each of the considerations identified in the DR Asset registration step should be evaluated and utilized, where necessary, during the update of the DR Asset information. The ability to update DR Asset information helps to ensure the asset information on record is current and complete.

Parties that are privy to and rely on the update should be notified of updates as soon as practicable after they are made.  Additionally, these parties should have access to review and, in some cases, to approve/disapprove the update(s).

| Asset Operator |  |  |
| --- | --- | --- |
| Asset Owner |  |  |
| Asset Physical Location |  |  |
| Authorized Asset Servicers |  |  |
| Date of Registration and Last Update |  |  |
| DR Asset Administrator | - System and Operator - Distribution Utility - Load Serving Entity - DR Aggregators |  |
| DR Asset Availability and Status | Run Status, Setpoint, Override status, etc. |  |
| DR Asset Identifier | Asset Name, ID |  |
| DR Asset Physical Capabilities | Ramp Up/Down Rate, Maximum Capacity |  |
| DR Asset Product Origin | Manufacturer, Model, Version, Date of Manufacturer |  |
| DR Asset Type | DG, renewable, storage, curtailable or interruptible load |  |
| DR Resources |  |  |
| State of Registration Process |  |  |

Sequence Diagram

3.4.2.3 Remove DR Asset

Removing a DR Asset must take into account any DR Resource long term commitments in System and Market Operator programs and contractual obligations with DR Aggregators or Load Serving Entities.  In addition, State and or Federal regulatory agency filings and approvals must be considered prior to removing a DR Asset from a DR program.

| Approval Process |  |  |
| --- | --- | --- |
| Contractual Agreements |  |  |
| DR Administrator | - System and Operator - Utility - DR Aggregators - Energy Consultants |  |
| DR Resources |  |  |
| Third Party Participation |  |  |

Sequence Diagram

3.2.5 Administrate DER for DR Purpose

###### 3.3 DR Bidding

Demand Response Bidding occurs after the DR Assets and Resources that are to comprise the desired bid have been registered and a bid is to be formed. The DR Bidding use case has two sub-use cases, depending on the type of bid to be developed for submission: a bid to supply (i.e., an offer), or a bid to buy. The bidder may be a Customer bidding with an Aggregator or LSE, a DR Aggregator bidding up to a LSE or regional market operator, or a LSE bidding up to a regional market operator.

Before the start of the bidding process, it is assumed that DR Resources available for  selection are already registered to provide the physical resource participating in the DR Bidding program. It is also assumed that the bidder has already registered to bid with the bid receiver and has been informed on the appropriate timing and parameters for bid participation. The DR bidding process starts with the decision of the bidder to develop a bid for submission. The bidder submits its bid to the bid receiver within the appropriate time window and following set guidelines for bidding. The bidder receives information on the outcome of the submitted bid from the bid receiver, who notifies of the bid outcome (e.g., a resource award or other DR commitment).

Examples of some types of products that may be (or become) eligible for DR bidding include:

* Retail Emergency DR;

* Retail Price-responsive DR;

* Installed Capacity;

* Day-ahead Energy;

* Real-time Imbalance Energy;

* Ancillary Service Operating Reserve;

* Regulation;

* Frequency Control.

| DR Resource Identifier | This identifies the DR Resource for which the bid is being submitted. |  |
| --- | --- | --- |
| Load Amount | This is the amount of load that the DR Resource can make available during the scheduled period. |  |
| Load Profile Schedule | This is the schedule during which the DR Resource's load profile will be available. |  |
| Offer Price | This is the price at which the DR Resource is offering to shed their resource. |  |

3.3.1 RETAIL DR Bid to Supply (Offers)

The Bid Receiver opens the forum/venue for receiving a bid. (This may include initiating the bidding process by sending initial information to Bidders indicating parameters of acceptability for bids to be made). The Bidder communicates an offer to supply a specified product/service and the pricing or other incentive structure the bidder is willing to accept for DR. (This may be as simple as the customer sending a response back to the Bid Receiver that accepts or rejects the initial bid sought, or may involve more complex bid structuring).

Each bid will have a clearly specified product or service that is being bid and the pricing/incentive structure being offered. The bidder also nominates the amount of and nature of supply being offered, the DR Resource to physically provide the service, and other physical parameters affecting resource availability and capability, in addition to the identity (or ID) of the bidder and resource(s) that comprise the bid.

The Bid Receiver reviews bids received and clears them, notifying the bidders of the outcome.

The Bidder receives the notification of awards or resource commitments and uses the information internally in preparation for DR Resource Dispatch.

| Bidder Identity |  |  |
| --- | --- | --- |
| Identity of Resource(s) |  |  |
| Pricing structure |  |  |
| Product/Service type | The Product/Service type offered to supply. |  |
| Quantity of offer |  |  |
| Resource Availability |  |  |
| Resource Capability (Max Capacity) |  |  |
| Resource Physical Operating Capability | (Ramp up/Ramp down, and type specific) |  |
| Resource Type |  |  |
| Timestamp of Bid |  |  |
| Trade Date or Period | The timeframe of the offer. |  |

Sequence Diagram

3.3.2 DR Bid to Buy

The Bid Receiver opens the forum/venue for receiving a bid. (This may include initiating the bidding process by sending initial information to Bidders indicating parameters of acceptability for bids to be made). The Bidder communicates a bid to buy a specified product/service and the pricing or other incentive structure the bidder is willing to pay to avoid DR. (This may be as simple as the customer sending a response back to the Bid Receiver that accepts or rejects the initial bid sought, or may involve more complex bid structuring).

Each bid will have a clearly specified product or service that is being bid for purchase and the corresponding pricing structure that is acceptable to the bidder. The bidder also includes the quantity of product/service in the bid to buy, and specifies any DR Resource(s) that can physically substitute for the product/service in the event the bid to buy is not cleared (i.e., the bidder is unwilling to pay more than the going/market outcome price for the product/service). Other information submitted include physical parameters affecting resource availability and capability, in addition to the identity (or ID) of the bidder and resource(s) that comprise the bid.

The Bid Receiver reviews bids received and clears all bids, notifying the bidders of the outcome.

The Bidder receives the notification of resource commitments and uses the information internally in preparation for DR Resource Dispatch.

| Bidder Identity |  |  |
| --- | --- | --- |
| Identity of Resource(s) |  |  |
| Product/Service type | The Product/Service type offered to supply |  |
| Quantity of bid |  |  |
| Resource Availability |  |  |
| Resource Capability (Max Capacity) |  |  |
| Resource Physical Operating Capability | (Ramp up/Ramp down, and type specific) |  |
| Resource Type |  |  |
| Timestamp of Bid |  |  |
| Trade Date or Period | The timeframe of the offer. |  |

Sequence Diagram

#### 4.0 Execute DR Event

4.0 Execute DR Event

Figure 7: Execute DR Event

###### 4.0 Execute DR Event

This is a high level use case.  See its sub-use cases for details.

###### 4.1 Notify DR Event (Bulk Power)

4.1.1 Advanced Notification for DR (Bulk Power)

Description:

One or more communication to DR Program Participants that a DR event is expected to be implemented by the System Operator. DR Events may be called for reliability, emergency, or other situations as determined by the System Operator.

A DR Event refers to the time periods, deadlines, and transitions during which Demand Resources perform. The System Operator shall specify the estimated duration and applicability of a Demand Response Event.  All deadlines, time periods, and transitions may not be applicable to all Demand Response products or services.

| Effective-Start-Date-Time | The starting date and time when DR Resources are expected to comply with their obligations under a DR program. | Functional |
| --- | --- | --- |
| Estimated-Duration | A decimal representation of the expected duration of the event with integers representing hours and decimal positions representing fractions of hours. For example: - 1.5 indicates duration of 1 hour and 30 minutes from the Effective-Start-Date-Time. | Functional |
| Event-Description | A description of the resource or program event that is being announced by the System Operator. | Functional |
| Event-Identifier | A unique identifier for the event, expressed as a URI in accordance with http://www.w3.org/TR/uri-clarification/#uri-schemes. | Functional |
| Location | An identifier used to indicate the location affected by an event. This is mutually exclusive from Target-Identifier. An event MUST pertain to either a Location or Target-Identifier. | Functional |
| Location-type | A value used to interpret the value contained in the Location element. Examples of Location-type include: - Zone - Node - Zip-code - USNG. | Functional |
| Message-Exchange-Pattern | The means by which DR event information is communicated between parties. The following are valid options: - In-Only - Robust In-Only - In-Out. Refer to http://www.w3.org/TR/wsdl20-adjuncts/#patterns for complete descriptions. | Functional |
| Source-Authentication | The recipient must have some assurance that the message was produced by a reliable, authoritative party. This will require some ability to validate the authenticity of the source and integrity of the data (e.g., digital signature applied to a price message). | Functional |
| Target-Identifier | This is the specific Asset or Resource affected by the event. It is mutually exclusive from Location. An event MUST pertain to either a Location or Target-Identifier. | Functional |
| TimeStamp | The date and time when the event was announced. This MUST be in ISO 8601 standard format. | Functional |

Sequence Diagram

4.1.2 Update a DR Event (Bulk Power)

4.1.3 Cancel a DR Event (Bulk Power)

Description:

Cancellation of a DR Event occurs when a System Operator decides that a DR Event which was previously announced will not be implemented.

| Effective-End-Date-Time | The date and time when the cancellation is effective. | Functional |
| --- | --- | --- |
| Event-Identifier | A unique identifier that identifies the previously announced notification of DR Event which is being cancelled. It MUST refer to an existing Advanced Notification of DR Event. | Functional |
| Message-Exchange-Pattern | The means by which DR event information is communicated between parties. The following are valid options: - In-Only - Robust In-Only - In-Out. Refer to http://www.w3.org/TR/wsdl20-adjuncts/#patterns for complete descriptions. | Functional |
| Source-Authentication | The recipient must have some assurance that the message was produced by a reliable, authoritative party. This will require some ability to validate the authenticity of the source and integrity of the data (e.g., digital signature applied to a price message). | Functional |
| TimeStamp | The date and time when the "cancellation notice" was announced. This MUST be in ISO 8601 standard format. | Functional |

Sequence Diagram

MUST-REFER-TO-ACTIVE-ADVANCED-NOTIFICATION

This can only be issued for an active Advanced Notification of DR event.

4.1.4 DR Resource Confirmation (Bulk Power)

###### 4.2 Notify DR Event (Distribution)

These sets of interactions are used to notify a DR Resource of a DR event.

4.2.1 Advanced Notification for DR (Distribution)

This is used by the Controlling entity to send the DR Event notification (i.e., DR signal) to the DR Resource.

| Controlling Entity ID | An identifier for the controlling entity issuing the DR event. |  |
| --- | --- | --- |
| DR Program ID | An identifier of the program for which a DR event was issued. | Functional |
| DR Resource Group ID | A group to which a DR Resource belongs. This can be used for multi-cast notifications. | Functional |
| DR Resource ID | An identifier of the DR Resource for which an Event is intended. | Functional |
| DR Resource Operation Mode (Schedule) | Operation modes are representations of how the DR Resource should respond to the Event Data that is part of an event. Operation modes allow DR logic to be implemented within the DR Controlling entity and allow the DR Resource to respond to a much simpler representation of the DR Event data. Operation modes are simple enumerations of the values that may be scheduled during the DR Event active period. The possible values are the following: NORMAL - This value is used to signify that no response to the DR Event is necessary by the DR Resource. MODERATE - This value is typically used to signify a moderate level of response by the DR Resource. HIGH- This value is typically used to signify a high level of response by the DR Resource. SPECIAL- This value is typically used to signify a circumstance special to the DR Resource. | Functional |
| DR Resource Specifications | This specifies which DR Assets within a DR Resource should be used, and can be specified in the following fashion: - Geographic location - Grid location - DR Asset group ID. |  |
| DR Resource Type ID | A program specific field that can be used to identify a type of DR Resource within a DR program. | Functional |
| Event Data | A collection of information that is associated with a DR event. An instance of such information represents a specific type of information that was defined to exist for DR events when the program was first developed and implemented. For example, a DR program may have been developed and implemented to use real-time pricing as the values that are sent when DR events are issued. In that case, the information would be prices. Furthermore, each type of information may have a schedule of values that are valid across the entire period for which the DR event is active. Therefore, a single type of information may have multiple values, each of which are valid during a different time period during the event. Each type of information has the following attributes when it is defined: - name - data type. The following data types should be supported: - PRICE_ABSOLUTE - Price number - PRICE_RELATIVE - Change in price - PRICE_MULTIPLE - Multiple of current price - LOAD_LEVEL - Amount of load based on an enumeration, i.e. moderate, high, etc - LOAD_AMOUNT - Fixed amount of load to shed in KW - LOAD_PERCENTAGE - Percentage of load to shed, i.e. 10% - GRID_RELIABILITY - Number signifying the reliability of the grid. | Functional |
| Event ID | An identifier for the DR event that was created when the DR event was first issued. This should be a globally unique identifier for each Controlling entity. | Functional |
| Event Schedule | A Schedule for the DR Event periods that includes the following: - Notification date/time - Event start date/time - Ramp time (potentially randomized) - Event end date/time (possibly specified as the duration). | Functional |
| Test Event Flag | This attribute signifies whether this is a test event or not. Test events may be issued by the Utility/ISO like other DR Events. | Functional |

Sequence Diagram

4.2.2 Update a DR Event (Distribution)

This is used to update a DR event that has been previously notified.  The requirements for this use case are the same as those for the "Advanced Notification for DR" use case.

| Event | See the "Advanced Notification for DR" use case. |  |
| --- | --- | --- |
| Event Modification Number | This is Modification number of the DR event. It is used to indicate that the DR Event has been modified by the Utility. Each time a DR Event is modified, this number is incremented. |  |
| Modification reason code | The reason the event was modified. |  |

Sequence Diagram

4.2.3 Cancel a DR Event (Distribution)

This is used by the Controlling entity to notify the DR Resource that a DR Event is being cancelled. It should only be executed if a DR Resource has previously received notification of a DR Event and that event has subsequently been cancelled.

| Cancellation reason code | The reason the event is being cancelled. |  |
| --- | --- | --- |
| DR Event Identifier | An iIdentifier for the event that is being cancelled. |  |
| Effective date/time | The date and time a cancellation takes effect. |  |

Sequence Diagram

4.2.4 DR Resource Confirmation (Distribution)

This is a confirmation message that is sent by a DR Resource to the Controlling entity as a result of receiving a DR Signal form the Controlling entity. It may be used as an acknowledgement of the receipt of the DR signal, but it may also contain various information used to signify how the DR Resource will respond to the DR Signal.

| DR Assets Characterisitics | The expected response characteristics of each DR Asset that is part of the DR Resource. These may be needed for aggregated DR Assets and may be as simple as whether a DR Asset is participating or its actual load profile. For each DR Asset, the following should be given: - Location of Asset, either geographic or grid location - Asset schedule of participating - Asset load profile (schedule). |  |
| --- | --- | --- |
| DR Resouce ID | The identifier of the DR Resource. |  |
| DR Resource Load Profile Response | The load profile response characterization of the DR Resource in response to getting the DR signal. |  |
| Notification Received Acknolwledgement | This is an acknowledgement of the receipt of a DR notification or dispatch. It should include any necessary provisions for non-repudiation. |  |
| Opt in/out | This is used to temporarily opt in/out of DR Events and to override the normal operational constraints. The opt-in/out can be specified using the following criteria: - All events in a program indefinitely - Specific DR Event - All events in a specific time period. In addition, there may be a schedule associated with the opt in/out state. |  |

Sequence Diagram

###### 4.3 Broadcast Price (Wholesale) for DR Purpose

Description:

The System Operator calculates the price of products per location and communicates the price to interested parties

| Currency | An identifier used to interpret the price element. It MUST follow ISO 4217 standard. | Functional |
| --- | --- | --- |
| Duration | The amount of time for which this price is valid, commencing at the Effective-Date-Time specified. A value of zero means price is valid until next price broadcast override. This is specified in decimal notation where integers represent minutes and decimals represent fractions of minutes. For example: - 5 indicates that prices will be published every 5 minutes - .5 indicates that prices will be published every 30 seconds. | Functional |
| Effective-Date-Time | The date and time which the price is in effect in ISO 8601 standard format. | Functional |
| Location | An identifier used to indicate an area in which this price is in effect. A value of "null" indicates that the price is in effect for all areas. | Functional |
| Location-type | A value used to interpret the value contained in the Location element. Examples of Location-type include: - Zone - Node - Zip-code - USNG. | Functional |
| Message-Exchange-Pattern | The means by which price information is communicated to any party that is interested in receiving this information. The following are valid options: - In-Only - Robust In-Only - In-Out. Refer to http://www.w3.org/TR/wsdl20-adjuncts/#patterns for complete descriptions. | Functional |
| Price | Prices MAY be either positive or negative and should be expressed in decimal notation with a precision up to 6 decimal places. | Functional |
| Product-Identifier | This identifies the type of product (e.g., energy, regulation, etc.) to which this price pertains. It also contains an enumeration of various products that may be offered. Extensibility MUST be supported in order to accommodate multiple jurisdictions and markets. | Functional |
| Publishing-Interval | This indicates the frequency with which price information is expected to be published and should be specified in decimal notation where integers represent minutes and decimals represent fractions of minutes. For example: - 5 indicates that prices will be published every 5 minutes - .5 indicates that prices will be published every 30 seconds. | Functional |
| Source-Authentication | The recipient of a "Price Message" must have some assurance that the message was produced by a reliable, authoritative party. This will require some ability to validate the authenticity of the source and integrity of the data (e.g., digital signature applied to a price message). | Functional |
| TimeStamp | This indicates the Date and Time when this information was published and MUST follow ISO 8601 standard representation. | Functional |
| Unique-Identifier | An identifier used to identify the message that should be expressed as a URI in accordance with http://www.w3.org/TR/uri-clarification/#uri-schemes | Functional |
| Unit-of-Measure | This indicates the unit of measure to which the price pertains. It MUST be compliant with the International System of Units as defined by NIST SP 330, ref: http://physics.nist.gov/Pubs/SP330/sp330.pdf. Examples of NIST compliant units of measure include: - kWh - MWh. | Functional |

Sequence Diagram

MUST-SUPPORT-IN-ONLY-PATTERN

At a minimum, the "IN-only" MUST be supported. This will ensure that any party, DR Asset or DR Resource that wishes to receive prices may access this information.

###### 4.4 Broadcast Price (Retail) for DR Purpose

Demand Response programs are designed to address different power system concerns caused by the imbalance between generation, transmission and consumption, or environmental concerns. Some DR programs are conducted on a voluntary basis, where the customer can opt to maintain the level or load.  Some DR programs are mandatory, where either the customer loads will be curtailed under certain conditions or the customer will incur financial penalties for noncompliance.

In retail market, certain “DR Messages” can be broadcasted to DR resources either directly or through Service Providers.

Based on the OpenADR discussion, the broadcasting function is mainly used for volunteer or non-mission critical DR messages. Therefore, message acknowledgement or confirmation is optional.

DR Source of Power/Generation

“DR Message” also can be source of power or generation. ANSI C12.19-2008 defined the following source of power/generation in Standard Table 12.

Precondition:

The DR controlling entity sends the DR message to service providers. In the case of DR pricing, the DR controlling entity calculates the wholesale price per location and communicates the appropriate price associated with the DR product to service providers.

Post Condition:

DR Resources received the DR pricing information, and may or may not act based on the DR agreement or contract.

| Same as the wholesale pricing use case |  |  |
| --- | --- | --- |

Sequence Diagram

###### 4.5 Dispatch DR Instructions (Bulk Power)

Description:

This instructs a DR Asset or Resource to change its consumption level (reduction and restore purposes).

| Effective-Start-Date-Time | The starting date and time when a DR Asset or Resource is expected to comply with the dispatch instruction. This is referred to as the "Reduction Deadline" in the NAESB M&V standard. | Functional |
| --- | --- | --- |
| Load-Reduction-Amount | The amount of electric power to be shed (reduced). This must be interpreted within the context of the Unit-of-Measure value. For example, a value of 1 in this element combined with a Unit-of-Measure value of MW would indicate a that a reduction of 1 megawatt is expected from a DR Asset or Resource. A value of zero indicates that no load reductions are needed and consumption may return to normal levels. This is equivalent to the "release/recall" instruction in the NAESB M&V standard. | Functional |
| Measurement | Information provided by a DR Participant indicating consumption levels over time. A Measurement Message contains all required information needed by a System Operator to assess a DR Asset or Resource Performance. | Functional |
| Message-Exchange-Pattern | The means by which Dispatch information is communicated between parties. The following are valid options: - In-Only - Robust In-Only - In-Out. Refer to http://www.w3.org/TR/wsdl20-adjuncts/#patterns for complete descriptions. | Functional |
| Source-Authentication | The recipient must have some assurance that the message was produced by a reliable, authoritative party. This will require some ability to validate the authenticity of the source and integrity of the data (e.g., digital signature applied to a price message). | Functional |
| Target-Identifier | A unique identifier for the DR Asset or DR Resource, as determined by the System or Market Operator, that is the intended recipient of a dispatch instruction. | Functional |
| TimeStamp | This indicates the Date and Time when the dispatch instruction was issued and MUST follow ISO 8601 standard representation. | Functional |
| Unit-of-Measure | This indicates the unit of measure to which the Load-Reduction-Amount pertains. It MUST be compliant with the International System of Units as defined by NIST SP 330, ref: http://physics.nist.gov/Pubs/SP330/sp330.pdf. Examples of NIST compliant units of measure include: - kW - MW. | Functional |

Sequence Diagram

4.5.1 Monitor DR Event (Bulk)

###### 4.6 Dispatch DR Instructions (Distribution)

This interaction is used to dispatch DR Resources.  This type of interaction is used when the Controlling entity needs to achieve specific objectives from the DR Resources during DR events. This means that specific instructions will be given to the DR Resource with objectives (e.g. shed 100KW) for the load profile of the DR Resource.

Note that for the retail use cases, a Dispatch is considered a special type of DR Notification, and thus, it is very similar in nature to the requirements given for that interaction.

| Dispatch Instructions | A collection of dispatch instructions that is associated with a DR event. An instance of such instructions represents a specific type of instruction that was defined specifically for the DR program. Each type of instruction may have a schedule of values that are valid across the entire period for which the DR event is active. Therefore, a single type of instruction may have multiple values, each of which are valid during a different time period during the event. Each type of instruction has the following attributes when it is defined: - name - instruction type. The following instruction types should be supported: - LOAD_LEVEL - Amount of load based on an enumeration, i.e., moderate, high, etc - LOAD_AMOUNT - Fixed amount of load to shed in KW - LOAD_PERCENTAGE - Percentage of load to shed, i.e., 10%. |  |
| --- | --- | --- |
| DR Program Name | An identifier of the program for which a DR event was issued. |  |
| DR Resource ID | An identifier of the DR Resource for which a dispatch is intended. |  |
| Event ID | An identifier for the DR event that was created when the DR event was first issued. |  |
| Event Modification Number | A modification number for the DR event. This is used to indicate if the DR Event has been modified by the Utility. Each time it is modified, this number is incremented. |  |
| Event Schedule | Schedule for the DR Event periods, which includes the following: - Notification date/time - Event start date/time - Event end date/time. |  |
| Test Event Flag | This attribute signifies whether this is a test event or not. Test events may be issued by the Utility/ISO like other DR Events. |  |

Sequence Diagram

###### 4.7 DR Direct Load Control (Distribution)

This use case covers direct interactions between the Controlling entity and a specific DR Asset for the purposes of putting that asset into a specific load control state (e.g., to turn it on or off). In this case, the DR Asset is a DR Resource with only one DR Asset, and thus, the interaction is directly with the DR Asset.

| Load Address/Identifier | This is an identifier for the load which is being controlled. Note: Is this an ID or and address? What else is needed? |  |
| --- | --- | --- |
| Load Control Command | This is a command to a specific load. |  |

Sequence Diagram

4.7.1 Monitor DR Event (DR Resource)

This interaction is used to monitor a DR Resource's behavior. It may be executed as a result of the DR Resource receiving a DR signal or it may be conducted continuously.

| Operational Constraints | This is used to supply information about when and how a DR Resource may participate in a DR Event. The following items may be supplied: - Limits on load profile performance - Schedule of participation. Note that this information may also be supplied as part of the administration of the DR Resource, and thus, it may not be necessary that it be part of this interaction. |  |
| --- | --- | --- |
| Opt in/out state | This is used to temporarily opt in/out of DR Events and to override the normal operational constraints. The opt-in/out can be specified using the following criteria: - All events in a program indefinitely - Specific DR Event - All events in a specific time period. In addition, there may be a schedule associated the opt in/out state. |  |
| Usgage Information | Information about the current electricity usage of the DR Resource. |  |

Sequence Diagram

4.7.2 Monitor DR Event (DR Asset)

This interaction is used to monitor the state of a DR Asset.  It may be executed as a result of the DR Asset receiving a DR signal or it may be conducted continuously.

| Exception Conditions | This is used to report that the load controller may not behave as commanded because of a variety of conditions including: - Faults in device - Customer override. |  |
| --- | --- | --- |
| Load Control State | The state of the load, which includes both commanded states and user settings. This may include a schedule of future states if a particular control algorithm for the load controller is being executed. |  |
| Operational Constraints | Constraints on how the load may be controlled. This may include limits on the state of the load controller as well as schedules upon those constraints. |  |

Sequence Diagram

###### 4.8 DR Event Execution

4.8.1 DR Execution - Dynamic Price-Based

Sequence Diagram

4.8.2 DR Execution - Notification Based

Sequence Diagram

4.8.2 DR Exedcution - Direct Load Control

Sequence Diagram

###### 4.9 Operational Coordination between Transmission, Distribution and DR Resources

Sequence Diagram

Adjust Schedule

Apply Retail Uplift Charges

Assess Congestion Uplift Charges

Impacted Curcuit Analysis

This analysis identifies impacted circuits, checks circuit operating limits, and checks equipment asset management contsraints.

Load Forecast by Circuit

Specify Load Profile

#### 5.0 Post DR Event Management

5.0 Post DR Event Management

Figure 8: 5.0 Post DR Event Management

###### 5.0 Post DR Event Management

###### 5.1 Post DR Event M&V / Settlement (Open Wholesale, No Open Retail)

After DR Event has been completed, then Measurement and Verification (M&V) can confirm whether action has been taken or to measure the impact of the response.  After the M&V, then the Financial Settlement is calculated and sent to the Electricity Customer.

This Use Case covers the scenarios with an open Wholesale, but no Open Retail Markets.

Sequence Diagram

RELEASE-RECALL

This indicates when DR has finished its Deployment Period and calculations for M&V and Settlement can take place.

###### 5.2 Post DR Event M&V / Settlement (No Open Wholesale, No Open Retail)

After a DR Event has been completed, then Measurement and Verification (M&V) can confirm action has been taken or to measure the impact of the response.  After the M&V, the Financial Settlement is calculated and sent to the Electricity Customer.

This Use Case covers the scenarios with no Open Wholesale or Retail Markets.

Sequence Diagram

RELEASE-RECALL

This indicates when DR has finished its Deployment Period and calculations for M&V and Settlement can take place.

###### 5.3 Post DR Event M&V / Settlement (Open or Not Open Wholesale, Open Retail)

After DR Event has been completed, Measurement and Verification (M&V) can confirm action has been taken or to measure the impact of the response.  After the M&V, the Financial Settlement is calculated and sent to the Electricity Customer.

This Use Case covers the scenarios with no Open Retail Markets, and includes either Open or Closed Wholesale Markets.

Sequence Diagram

RELEASE-RECALL

This indicates when DR has finished its Deployment Period and calculations for M&V and Settlement can take place.