PARALLEL FLOW VISUALIZATION OPTIONS
1.  (a) Tag all non-firm transactions.
· Requires all non-firm (inter/intra, point-to-point/network) transactions to be tagged.

· Requires change order 310 in order to recognize tags.

· By default consider all else firm.
 (b) Tag all firm and non-firm transactions. 

· Requires all firm and non-firm (inter/intra, point-to-point/network) transactions to be tagged.

· Requires change order 310 in order to recognize tags.

Status: 


Work Group Lead:
Jim Busbin

Work Group Participants: 
Alan Pritchard, Barry Green, Narinder Saini, Carlos Gonzalez-Perez (or Nelson Muller), Emilie Nelson
2. Consider all generation output from DNR to be firm and all non-DNR output to be non-firm.  
· Prioritize based on generation output instead of tag. 

· Requires change order 310 to recognize tags.
· All point-to-point (inter/intra, firm/non-firm) must be tagged.

· Discuss identification of firm and non-firm gen output (DNR or non-DNR and recognition of slice of system DNR)

· Examples

· Generation serving load in the same BA (internal gen to internal load)

· Rule 1 - Internal firm generation (e.g., DNR) that serves load in its own BA.  Internal DNR, 100% of the unit, serving load in the BA (intra-BA load). – firm

· Rule 2 - Internal non-DNR non-firm (secondary network), 100% of unit, serving load intra-BA – non-firm 6-NN

· Rule 3 - Internal generation where all or part of the generator is delivered intra-BA using firm point-to-point service. Not a DNR (energy only, non-firm).  The transaction must be tagged to be considered firm. The unit output minus firm tag (the untagged portion) is non-firm.  The Tagged amount is firm. 
· Rule 4 - Internal generation where all or part of the generator is delivered intra-BA using non-firm point-to-point service. Not a DNR (energy only, non-firm).  The unit output minus firm tag (the untagged portion) is non-firm point-to-point.  The Tagged amount is non-firm 6-NN. 
· Interchange
The tagged amount of the unit output is prioritized based on the tag with no change from today.  The amount of the unit that remains in the GCA is prioritized based on the generation priority (e.g., DNR or non-DNR)

· Rule 5 - Inter-BA, interchange 500 MW export from DNR (which must be undesignated), tagged firm GCA to the LCA – firm.  And decrease the reported unit output for internal flow by 500.

· Rule 6 - Inter-BA, interchange 500 MW export from DNR, non-firm to LCA.  Use tag priority for the amount exported.  Remaining output is reported to IDC per Rules 1 and 2.

Status: 


Work Group Lead: 
Ed Skiba 
Work Group Participants: 
Deral Danis, Marjorie Parsons, Chris Advena, Cathy Wesley, Emilie Nelson, Carlos Gonzalez-Perez (or Nelson Muller) Tom Mallinger, Blaine Erhardt, Jim Busbin
3. Flowgate Allocations. 

Status: 


Work Group Lead: 
Tom Mallinger
Work Group Participants: 
Chris Advena, Cathy Wesley, Emilie Nelson, Carlos Gonzalez-Perez (or Nelson Muller), Steve Sanders, Blaine Erhardt, Alan Johnson, Ed Skiba
Last Edit Date: April 1, 2010
Page 1 of 2

