NAESB WEQ BPS Workpaper
Jim Busbin


Non-Firm Transmission Presently Excluded or Mis-Identified for TLR Priority

1. Non-Firm Usage (6-NN)

2. For Non-Market Entities - Non-Designated Resources

3. Intra- Balancing Authority (BA) Transactions

Non-Firm Usage (6-NN) / For Non-Market Entities – Non-Designated Resources:
(1) and (2) refer to the excess of generation output beyond that which has been designated as a Network Resource.  That generation could presently be mis-identified as generation utilizing Firm Network (7-F) transmission service.  For example;, if there exists a generating plant with a total generation capability of 1,000 MWs and 800 MW has been designated as a Network Resource, any generation output beyond 800 MW utilized for native load serving should utilize Secondary Network transmission service (6-NN).  Presently, there is not a methodology in place to make this form of evaluation.
The identification of network resources for use in a large scale visualization process is further complicated by the lack of standardization in posting those designations.  Designations can be made with regard to generating units or total plant.  Most entities post their network resource designations on their OASIS sites under a General Information heading.
Intra-Balancing Authority (BA) Transactions:

(3) refers to the present inability of the congestion management process to assess the effect of intra-BA transactions.  Effects of transactions are measured by something called transfer distribution factors (TDFs).  In the congestion management process TDFs are calculated for all inter-BA possibilities.  Any intra-BA calculation will have a calculated value of zero and, thus, the transaction does not register any effect related to the congestion management process.  The result is a transaction utilizing a form of Non-Firm transmission service in an intra-BA scenario will essentially be treated as “firmer than Firm.”
