1.  RECOMMENDED ACTION:
EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

  X   Accept as requested


  X Change to Existing Practice

      Accept as modified below


      Status Quo

      Decline

2.  TYPE OF DEVELOPMENT/MAINTENANCE

Per Request:




Per Recommendation:
  X    Initiation




  X    Initiation 

  X  Modification




 X   Modification

      Interpretation



      Interpretation

      Withdrawal




      Withdrawal

  X    Principle 




  X    Principle 

   X   Definition 




   X   Definition 

  X  Business Practice Standard 


  X  Business Practice Standard 

    X  Document 




   X   Document 

    X  Data Element 



   X   Data Element

   X   Code Value 



  X    Code Value 

      X12 Implementation Guide


     X12 Implementation Guide

   X   Business Process Documentation

  X    Business Process Documentation

3.  RECOMMENDATION

SUMMARY:


The Commission issued Orders 890, 890-A, 890-B and 890-C directing Transmission Providers to develop through NAESB, Business Practice Standards, including OASIS functionality related to Network Integration Transmission Service (NITS).

Recommended Standards:
The following modifications to WEQ-000, WEQ-001, WEQ-002, WEQ-003 and WEQ-013 along with additions of WEQ-001-100 through WEQ-001-107, inclusive, WEQ-002-n, and WEQ-013-n are recommended to implement the Commission’s findings in Orders 890, 890-A, 890-B and 890-C.

Add Recommendation Here
WEQ-000 revisions

WEQ-000 additions

Additions to WEQ-000-1 (Acronym)

NSR – Network Scheduling Right
Additions to WEQ-000-2 (Glossary)

Application – Shall have the same meaning as defined in the FERC Pro Forma OATT. 
Concomitant Evaluation - The evaluation of a request for new PTP or Network transmission service submitted in conjunction with a request for undesignation of a Network Resource.
Designated Agent – Shall have the same meaning as defined in the FERC Pro Forma OATT.
Network Customer –Shall have the same meaning as defined in the FERC Pro Forma OATT. 
Network Resource – Shall have the same meaning as defined in the FERC Pro Forma OATT.
Network Load – Shall have the same meaning as defined in the FERC Pro Forma OATT.
Network Scheduling Right (NSR) – Optional mechanism on OASIS, enabled at the Transmission Provider’s discretion, to facilitate e-Tag validation. When used, the mechanism shall either 1) allow requests for network transmission capacity or 2) allow Transmission Provider posting of the scheduling rights associated with NITS. 
Pre-submittal Workspace – Functionality or facility provided by OASIS wherein Eligible Customers may enter, save, and revise information in preparation for submittal of a NITS Application or NITS modification of service.
Request Interval – Time interval associated with NITS modifications of service requests, including DNR, DNL, and NSR requests, as well as with all requests for Secondary Network Transmission Service.   
WEQ-001 revisions

WEQ-001 additions

001-100
INDEX OF NITS REQUIREMENTS

001-101
NITS – GENERAL REQUIREMENTS

001-101.12
Designation Network Resource Information Requirements

001-101.13
Designated Network Load Information Requirements
001-102
NITS – APPLICATION

001-102.1 
Submission of NITS Application 

001-102.2
Initial Review of NITS Application

001-102.3
Invalid NITS Application

001-102.4
Deficient NITS Application

001-102.5
Completed NITS Application

001-103
NITS – EVALUATION OF NITS APPLICATION

001-103.1
Review of  Completed NITS Application

001-103.2
Study of NITS Application

001-103.3
Declining a NITS Application

001-103.4
Refusal of Completed NITS Application

001-103.5
Counteroffer to NITS Application

001-103.6
Acceptance of NITS Application

001-103.7
Changes to NITS Application Prior To Confirmation of the NITS Application

001-103.8
Customer Actions

001-103.9
Final Disposition of NITS Application

001-103-10
Changes to a CONFIRMED NITS Application

001-104
NITS – ADMINISTRATIVE CHANGES

001-105
NITS – MODIFICATION OF SERVICE

001-105.1
General Requirements 
001-105.2
Designation of New Network Resource 
001-105.2.1
Submission of Request for Designation of New Network Resource

001-105.2.2
Initial Review of Request for Designation of New Network Resource

001-105.2.3
Invalid Request for Designation of New Network Resource

001-105.2.4
Deficient Request for Designation of New Network Resource
001-105.2.5
Completed Request for Designation of New Network Resource

001-105.2.6
Review of Completed Request for Designation of New Network Resource

001-105.2.7
Study of Request for Designation of New Network Resource

001-105.2.8
Decline of Request for Designation of New Network Resource

001-105.2.9
Refusal of Request for Designation of New Network Resource

001-105.2.10
Counteroffer to Request for Designation of New Network Resource

001-105.2.11
Acceptance of Request for Designation of New Network Resource

001-105.2.12
Customer Actions

001-105.2.13
Final Disposition of Request for Designation of New Network Resource

001-105.3     Termination of Network Resource

001-105.3.2
Temporary Termination of Network Resource
001-105.3.3
Submission of Request for Temporary Termination of Network Resource 

001-105.3.4
Initial Review of Request for Temporary Termination of Network Resource 

001-105.3.5
Invalid Request for Temporary Termination of Network Resource 

001-105.3.6
Deficient Request for Temporary Termination of Network Resource 

001-105.3.7
Completed Request for Temporary Termination of Network Resource 

001-105.3.8
Acceptance of Request for Temporary Termination of Network Resource 

001-105.3.9
Customer Actions

001-105.3.10
Final Disposition of Request for Temporary Termination of Network Resource 

001-105.3.11
Indefinite Termination of Network Resource

001-105.3.12
Submission of Request for Indefinite Termination of Network Resource

001-105.3.13
Initial Review of Request for Indefinite Termination of Network Resource

001-105.3.14
Invalid Request for Indefinite Termination of Network Resource

001-105.3.15
Completed Request for Indefinite Termination of Network Resource

001-105.3.16
Acceptance of Request for Indefinite Termination of Network Resource

001-105.3.17
Customer Actions

001-105.3.18
Final Disposition of Request for Indefinite Termination of Network Resource

001-105.4
Designation of New Network Load or Undesignation of Network Load

001-105.4.2
Submission of Request for Designation of New Network Load or Undesignation of Network Load

001-105.5
NSR

001-105.5.2
Submission of Request for New NSR

001-1065.10
SECONDARY NETWORK TRANSMISSION SERVICE 
001-106.1
Request for Secondary Network Transmission Service

001-106.2
Releasing Secondary Network Tranmission Service Capacity

001-107
NITS – CONCOMITANT EVALUATION 

001-107.8
Concomitant Evaluation Final Disposition

001-108
NITS – CHANGE TO NITS APPLICATION STOP_TIME  

001-101
NITS – GENERAL REQUIREMENTS

The following practices are defined to enhance consistency of processing NITS requests.

001-101.1
OASIS shall support the queuing and processing of all requests for NITS, including the NITS Application and any subsequent modifications of service, as specified in the technical requirements of Business Practice Standards WEQ-002, WEQ-003 and WEQ-013. 
001-101.2
OASIS shall support capability that allows the Eligible Transmission Customer to submit information required for the arrangement of NITS through multiple requests that together represent the service being requested.  This shall include multiple requests submitted as part of a single NITS Application, as well as multiple requests submitted as modifications of service (including Concomitant Evaluations).

001-101.2.1
A Designated Agent may act on behalf of an Eligible Transmission Customer or a Network Customer within the authorization period as authorized by the customer.

001-101.2.2
Any request submitted by a Designated Agent on behalf of an Eligible Transmission Customer or Network Customer must be within the Designated Agent authorization period.
001-101.3
All requests for NITS and the current status of those requests shall be posted on OASIS using the defined values for STATUS as set forth in Business Practice Standards WEQ-013 for NITS.  

001-101.4
The Transmission Provider may opt to input information into OASIS on behalf of a Network Customer at the request of the Network Customer and shall specify the circumstances as required in Business Practice Standards WEQ-001-13.  
001- 101.5
The Transmission Provider shall provide a Pre-submittal Workspace. 
001-101.5.1
OASIS shall restrict access to and viewing of all information associated with the Pre-submittal Workspace to only the Transmission Provider, the Eligible Transmission Customer and its Designated Agent.  

001-101.5.2
OASIS shall have the ability to allow an Eligible Transmission Customer to replicate all or some of an existing NITS Application into a Pre-submittal Workspace for use in a subsequent NITS Application or NITS modification of service.

001-101.5.3
Audit and data retention requirements set forth in Title 18 CFR 37.6 and 37.7 are not applicable to the information in the Pre-submittal Workspace. 

001-101.5.3.1
OASIS shall have the ability to submit all information in the Pre-submittal Workspace as the NITS Application.

001-101.5.3.1.1
The Transmission Provider may remove information from the Pre-Submittal Workspace once the NITS Application has been submitted. 

001-101.5.3.2
If information is entered into the Pre-submittal Workspace on OASIS by the Eligible Transmission Customer and a NITS Application is not submitted within 6 months of the time of the last update to the Pre-submittal Workspace, then the Transmission Provider may remove all information from the Pre-submittal Workspace.  The Eligible Transmission Customer may request and the Transmission Provider may agree to a later removal date.

001-101.5.3.3
The Eligible Transmission Customer may remove the information contained in the Pre-submittal Workspace at any time.

001- 101.6
As an alternative to entering the information directly on OASIS, OASIS shall provide the ability for an Eligible Transmission Customer to complete the Application off OASIS and upload the NITS Application to OASIS for additional modifications in the Pre-submittal Workspace or for submittal of the request(s) to the Transmission Provider. 
001-101.7
The Eligible Transmission Customer may set the status of NITS-related request(s) (whether uploaded to or entered directly on OASIS) to PRESUBMITTED to indicate that the Eligible Transmission Customer desires to save the information entered and continue working on the request(s) in the Pre-submittal Workspace.  Alternatively, the Eligible Transmission Customer may submit its NITS-related request(s) (whether uploaded to or entered directly on OASIS) to indicate the Eligible Transmission Customer’s desire to formally submit the request(s).

001- 101.8
Any required information, including deposits, which is not submitted electronically through OASIS shall be provided to the Transmission Provider by some other method, as specified in Business Practice Standards WEQ-001-13. 

001-101.9
Consistent with the requirements in Business Practice Standards WEQ-013-x, Transmission Providers are required to post a reason for certain STATUS changes.

001-101.10
The Eligible Transmission Customer shall have the ability to remove a NITS Application or request for modification of service from further consideration by setting the status of the request to WITHDRAWN at any time prior to confirmation.

001-101.11
Consistent with the requirements in Business Practice Standards WEQ-001-13, the Transmission Provider shall specify in their posted Business Practice the method the Eligible Transmission Customer shall use to identify the provision of ancillary services associated with the NITS Application. 

001-101.12
Designated Network Resource Information Requirements:
001-101.12.1
For Network Customer-owned or -leased generation, the Network Customer shall use the TSIN-registered Source name if available. 

001-101.12.2
For Network Customer-owned or –leased generation, the electrical location will be identified by the BAA where the generation is electrically located and the geographical location will be identified by the county or parish and state. 

001-101.12.3
For purchased power, the electrical location is the point of receipt (on the Transmission Provider’s system referenced in the Customer’s Application or request for modification of service) and the geographical location is “not applicable”. 

001-101.13
Designated Network Load Information Requirements:

001-101.13.1
For all requests for designation of Network Load, the Network Customer shall use the TSIN-registered Sink name if available. 

001-102
NITS – APPLICATION

001- 102.1 
Submission of NITS Application 

001- 102.1.1 
An Eligible Transmission Customer shall submit, or arrange for the submittal of, the NITS Application on the Transmission Provider’s OASIS in accordance with the technical requirements of Business Practice Standards WEQ-002, WEQ-003 and WEQ-013. 

001-102.1.1.1
OASIS shall assign a unique identifier to each NITS Application.  

001-102.1.1.2
An Eligible Transmission Customer may enter and update information on OASIS for a NITS Application prior to submission by setting the status of the NITS Application to PRESUBMITTED. 

001-102.1.1.2.1
Information (e.g., load forecasts, resource designations, etc.) associated with such a PRESUBMITTED NITS Application (whether provided electronically or by other means) shall not be considered by the Transmission Provider until the NITS Application is submitted.  

001- 102.1.2
The Eligible Transmission Customer shall indicate to the Transmission Provider that the NITS Application is ready for Transmission Provider review for consistency with the Transmission Provider’s minimum information requirements which are posted as specified in Business Practice Standards WEQ-001-13, by submitting the NITS Application on OASIS.

001-102.1.2.1
OASIS shall set the queue time for the NITS Application to the time that the Eligible Transmission Customer submits the NITS Application and shall set the status of the NITS Application to QUEUED.

001-102.1.2.1.1
OASIS shall set the queue time of all associated portions submitted as part of a NITS Application,, which require a queue time (e.g., DNRs, DNLs, NSRs, etc.), to the queue time of the NITS Application and shall set the status of such  portions to QUEUED.
001-102.1.2.2
The Transmission Provider shall acknowledge the queued NITS Application  by setting the status of the NITS Application to RECEIVED.

001-102.1.2.2.1
The Transmission Provider shall acknowledge the queued associated portions of the NITS Application as specified in Business Practice Standard WEQ-001-102.1.2.1.1 by setting the status of such portions to RECEIVED.
001-102.2
Initial Review of NITS Application

001- 102.2.1
The Transmission Provider shall respond to a NITS Application by setting the status to one of the following:

I.
INVALID 

II.
DEFICIENT 

III.
COMPLETED 

001-102.3
Invalid NITS Application

001-102.3.1
If the required information, as specified in the Transmission Provider’s minimum information requirements that are posted  in accordance with Business Practice Standards WEQ-001-13, has not been provided in the NITS Application, the Transmission Provider shall set the status of the NITS Application to INVALID. The NITS Application shall be deemed withdrawn and will receive no further consideration.

001-102.3.1.1
If any portion of a NITS Application is set to a status of INVALID, the entire NITS Application will be set to a status of INVALID.

001-102.4
Deficient NITS Application

001-102.4.1
The Transmission Provider shall determine if each portion of information required in a NITS Application can be deemed sufficient (i.e. consistent with the Transmission Provider’s OATT requirements for a Completed Application).

001-102.4.1.1
If deemed sufficient, the Transmission Provider shall set the status of such portion to COMPLETED.  

001-102.4.1.2
If not deemed sufficient, the Transmission Provider shall set the status of such portion to DEFICIENT
001-102.4.2
If the Transmission Provider determines that any portion of the NITS Application is deficient, the Transmission Provider shall set the status of the overall NITS Application to DEFICIENT and may begin informal communications with the Eligible Transmission Customer to resolve those deficiencies. 

001-102.4.3
The Eligible Transmission Customer shall remedy the deficiency and shall set that portion’s status to REEVALUATE. Once all deficiencies have been remedied, the Eligible Transmission Customer shall set the status of the overall NITS Application to REEVALUATE.

001-102.4.4
Upon notification that the Eligible Transmission Customer has provided the necessary information, either on or off OASIS as appropriate, to remedy an identified deficiency, the Transmission Provider shall review the provided information and take one of the following actions:

001-102.4.4.1
If the information provided for any portion identified as deficient fails to remedy that deficiency, the Transmission Provider shall set the status for that portion of the NITS Application to DEFICIENT.

001-102.4.4.2
If the information provided for any portion identified as deficient remedies that deficiency, the Transmission Provider shall set the status for that portion of the NITS Application to COMPLETED.

001-102.4.4.3
If the information provided for any portion of the NITS Application is deemed by the Transmission Provider to be a significant change to the overall NITS Application, the Transmission Provider shall set the status of the overall NITS Application to DECLINED.

001-102.4.4.3.1
Once the status of the overall NITS Application has been set to DECLINED, the Transmission Provider shall set the status of all portions of the NITS Application to DECLINED.
001-102.4.5
If the Eligible Transmission Customer fails to respond to the Transmission Provider’s notice of a deficient NITS Application, or fails to remedy the deficiency to the Transmission Provider’s satisfaction in a timely manner, the Transmission Provider shall set the status of the overall NITS Application to DECLINED.

001-102.4.5.1
Once the status of the overall NITS Application has been set to DECLINED, the Transmission Provider shall set the status of all portions of the NITS Application to DECLINED.

001-102.4.6
If the Eligible Transmission Customer fails to provide the necessary credit or deposit information in a timely manner, the Transmission Provider may respond by setting the status of the overall NITS Application to DECLINED.

001-102.4.6.1
Once the status of the overall NITS Application has been set to DECLINED, the Transmission Provider shall set the status of all portions of the NITS Application to DECLINED.

001-102.4.8
When the Transmission Provider determines that all deficiencies have been remedied and all portions of the NITS Application have been identified as COMPLETED, the Transmission Provider shall continue processing the NITS Application in accordance with the Business Practice Standards for a Completed NITS Application.

001-102.5
Completed NITS Application

001-102.5.1
When the Transmission Provider has reviewed the entire NITS Application for completeness and all portions associated with the NITS Application have been deemed COMPLETED, the Transmission Provider shall indicate that the NITS Application is deemed complete by setting the status of the overall NITS Application to COMPLETED.

001-102.5.2
Consistent with the requirements of Business Practice Standard WEQ-001-102.1.2.1.1, the Transmission Provider shall ensure that the queue times of all COMPLETED portions associated with the COMPLETED NITS Application are set to the same queue time established on the initial submission of the NITS Application.

001-102.5.3
The COMPLETED NITS Application and set of associated COMPLETED portions shall constitute the entire initial NITS Application to be considered and evaluated further for granting of service by the Transmission Provider.

001-103
NITS – EVALUATION OF NITS APPLICATION

001-103.1
Review of  Completed NITS Application

001- 103.1.1
Once the Transmission Provider has set the status of the NITS Application to COMPLETED, the Transmission Provider shall change the status of the overall NITS Application to one of the following to indicate its evaluation of the NITS Application:

I.
STUDY

II.
DECLINED

III.
REFUSED 

IV.
COUNTEROFFER

V.
ACCEPTED 

001-103.2
Study of NITS Application

001-103.2.1 
If the Transmission Provider determines that some level of study is required or is being performed, the Transmission Provider shall set the status of the overall NITS Application to STUDY.

001-103.2.2
From the status of STUDY, the Transmission Provider shall set the status of the overall NITS Application to DECLINED, REFUSED, COUNTEROFFER, or ACCEPTED.

001-103.3
Declining a NITS Application

001-103.3.1
If at any time in the evaluation of the NITS Application or conduct of any associated study(ies) the Transmission Provider determines that the Eligible Transmission Customer has failed to meet any of the terms and conditions for the provision of service (e.g., failure to provide a required deposit, execute a study agreement, etc.), the Transmission Provider shall set the status of the overall NITS Application to DECLINED.

001-103.3.1.1
Once the status of the overall NITS Application has been set to DECLINED, the Transmission Provider shall set the status of all portions of the NITS Application to DECLINED.
001-103.4
Refusal of Completed NITS Application

001-103.4.1
If at any time in the evaluation of the NITS Application or conduct of any associated study(ies) the Transmission Provider determines that the requested service cannot be provided due to insufficient transfer capability consistent with the Transmission Provider’s tariff requirements, the Transmission Provider may set the status of the overall NITS Application to REFUSED.

001-103.5
Counteroffer to NITS Application

001-103.5.1
When, in the evaluation of the NITS Application or on completion of any applicable study(ies), and, where appropriate, through informal communication with the Eligible Transmission Customer, the Transmission Provider determines that the requested transmission service may only be granted in part, the Transmission Provider shall set the status of the overall NITS Application to COUNTEROFFER.

001-103.5.2
Prior to setting the status of the overall NITS Application to COUNTEROFFER, the Transmission Provider shall update the status associated with each portion of the Completed Application as follows:

001-103.5.2.1
The status of any portion of the NITS Application that will not be granted shall be set to the appropriate final state.

001-103.5.2.2
The status of any portion of the NITS Application that is to be granted as specified in the NITS Application shall be set to ACCEPTED.

001-103.5.2.3
The status of any portion of the NITS Application that is to be granted at a reduced capacity shall be set to COUNTEROFFER.

001-103.6
Acceptance of NITS Application

001-103.6.1
When, in the evaluation of the NITS Application or on completion of any applicable study(ies), and, where appropriate, through informal communication with the Eligible Transmission Customer, the Transmission Provider determines that the requested transmission service may be granted, the Transmission Provider shall set the status of the overall NITS Application to ACCEPTED.

001-103.6.2
Prior to setting the status of the overall NITS Application to ACCEPTED, the Transmission Provider shall update the status associated with each portion of the NITS Application to ACCEPTED.

001-103.7
Changes to NITS Application Prior To Confirmation of the NITS Application

001-103.7.1
A request for a change to a NITS Application which is an administrative change in accordance with Business Practice Standards WEQ 001-104 may be made at any time.

001-103.7.2
The status of a request for a change to a NITS Application in any final state other than CONFIRMED which is a request for modification of service in accordance with Business Practice Standards WEQ 001-105 shall be set to INVALID.

001-103.7.3
If, during informal communications, it is determined that a change to the NITS Application is necessary, the Eligible Transmission Customer shall use the process for modification of service in accordance with Business Practice Standards WEQ 001-105 to submit a request for such change.  Such a request for change will be deemed by the Transmission Provider to be part of the NITS Application as initially submitted.

001-103.7.3.1
If the request is outside the scope of informal communications, the Transmission Provider may deny the request by setting the status of the request to INVALID.

001-103.8
Customer Actions

001-103.8.1
After the status of the overall NITS Application has been set to ACCEPTED or COUNTEROFFER by the Transmission Provider, the Eligible Transmission Customer may confirm or withdraw the overall NITS Application by setting the status to CONFIRMED or WITHDRAWN. 

001-103.8.1.1
Confirming the overall NITS Application shall constitute the Network Customer’s agreement to receive service as specified in all portions of the NITS Application that are not already in a final state. Confirming the overall NITS Application shall set the status of all portions of the NITS Application not already in a final state to CONFIRMED. The Network Customer is not permitted to CONFIRM only a portion of the NITS Application.

001-103.8.1.1.1
If the Eligible Transmission Customer requests the Transmission Provider to file an unexecuted service agreement at FERC, the Eligible Transmission Customer, at the time of setting the status of the overall NITS Application to CONFIRMED, shall also set the indicator to “Unexecuted” (language in green to be determined by tech team).
001-103.8.1.2
Withdrawal of the overall NITS Application shall constitute the Eligible Transmission Customer’s withdrawal of all portions of the NITS Application including those portions already in a final state.  Withdrawal of the overall NITS Application shall set the status of all portions of the NITS Application to WITHDRAWN.

001-103.8.1.3
The Eligible Transmission Customer may not act on the individual portions of the NITS Application.  Once the overall NITS Application is in a final state of CONFIRMED, any subsequent changes must be made through a request for modification of service as provided for in the Business Practice Standards WEQ 001-105.  

001-103.9
Final Disposition of NITS Application

001-103.9.1
Once CONFIRMED the overall NITS Application shall remain CONFIRMED throughout its term of service regardless of subsequent modification of service unless otherwise mutually agreed; in which case the Transmission Provider shall set the status of the overall NITS Application and all portions of the NITS Application that are not already in a final state to ANNULLED.

001-103.9.1.1
If a Commission order related to a NITS Application is issued, the Eligible Transmission Customer shall use the modifications of service process as provided for in the Business Practice Standards WEQ-001-105 to modify the overall NITS Application to reflect the Commission order.

001-103.9.1.1.2
If the Eligible Transmission Customer determines not to accept the terms and conditions ordered by the Commission, the Eligible Transmission Customer may request that the Transmission Provider set the status of the overall NITS Application and all portions of the NITS Application that are not already in a final state to ANNULLED.

001-103.9.2
If, within the time limits specified in the Transmission Provider’s tariff for execution of a service agreement or for requesting filing of an unexecuted service agreement, the Eligible Transmission Customer fails to set the status to CONFIRMED or WITHDRAWN or does not request the Transmission Provider to file at FERC an unexecuted service agreement, the Transmission Provider may set the status of the overall NITS Application to RETRACTED. Retraction of the overall NITS Application shall set the status of the portions not already in a final state to RETRACTED.

001-103.10            Changes to a CONFIRMED NITS Application 

001-103.10.1
Any request submitted by the Eligible Transmission Customer associated with a NITS Application that is deemed by the Transmission Provider to be a request for modification of service shall be processed in accordance with the applicable Business Practice Standards WEQ 001-105 when the overall NITS Application status is CONFIRMED.

001-104 
NITS – ADMINISTRATIVE CHANGES

001-104.1
The Network Customer shall have the right to request administrative changes pursuant to the NITS Application. 
001-104.2
A request for administrative changes shall be submitted to the Transmission Provider on OASIS and assigned a unique identifier in accordance with the Business Practice Standards WEQ-002 and WEQ-013 and with explicit reference to the associated NITS Application.
001-104.3
A request for administrative changes shall be submitted as pre-confirmed. 

001-104.4
The Transmission Provider will accept or deny the request within 7 calendar days of receipt of the request. The Transmission Provider will provide a reason for denial of a request for administrative change.

001-104.5
The types of requests considered administrative changes will be in accordance with Business Practice Standards WEQ-002 and WEQ-013.
NOTE: 08/31/2010 – JT to put something in the template structures to address the effective date of the change as well as the time frame for the TC to act.  
001-105
NITS – MODIFICATION OF SERVICE

001-105.1
General Requirements 
001-105.1.1
The Network Customer shall have the right to request modification of service pursuant to the NITS Application. 

001-105.1.2
Any CONFIRMED request for modification of service shall be incorporated into the associated NITS Application.
001-105.1.3
A request for modification of service shall be submitted to the Transmission Provider on OASIS in accordance with the Business Practice Standards WEQ-002 and WEQ-013 and with explicit reference to the associated NITS Application.
001-105.1.4
A request for modification of service shall be queued, assigned a unique identifier by OASIS, and evaluated in the same manner as any other request, subject to the other requirements of the Business Practice Standards. 
001-105.1.5
The following timing requirements shall apply to all requests for modifications of service requests, including DNR, DNL, and NSR requests, as well as all requests for Secondary Network Transmission Service:

Table 105-a

NITS ReqUest Timing Requirements

Information in this table is used solely for determining timing requirements associated with evaluation of NITS modification of service requests.

	Request
	Request Interval1

	Time Queued Prior to Start


	Provider Evaluation Time Limit2
	Customer Confirmation Time Limit3 After ACCEPTED 

or COUNTEROFFER4
	Provider Counter Time Limit after REBID5

	Secondary Network Transmission Service/ Release of Secondary Network Transmission Service 
	Hour

Hour

Hour

Hour

Day

Week

Month
	< 1 hour

 1 to 24 hours

> 24 to 72 hours

> 3 days

N/A

N/A

N/A
	Best effort

30 minutes

60 minutes

2 days (add footnote similar to footnote 7)

30 minutes

4 hours

2 days6
	5 minutes

5 minutes

30 minutes

60 minutes

2 hours

24 hours

24 hours
	5 minutes

5 minutes

10 minutes

10 minutes

10 minutes

4 hours

4 hours

	DNR/

DNL/

NSR
	Day

Day

Week

Month

Year
	<24 hours

> 1 day

N/A

N/A

60 days8
	Best effort

30 days7
30 days7
30 days7
30 days7
	2 hours

24 hours

48 hours

4 days

15 days
	30 minutes

4 hours

4 hours

4 hours

4 hours

	Termination of DNR/ Termination of NSR9 
	N/A
	< 1 hour

1 to 24 hours
> 24 hours
	Best effort7
30 minutes  

2 hours
	5 minutes

30 minutes
24 hours
	N/A


Notes for Table 105-A:

1
Request Interval is used to for purposes of documenting response times.  Request Interval does not define or modify the product being requested.  The following Request Intervals shall be associated with this table:

a. Hour: Request Interval of less than one day

b. Day: Request Interval comprising at least one day but less than one week 

c. Week: Request Interval comprising at least one week but less than one month 

d. Month: Request Interval comprising at least one month but less than one year 

e. Year: Request Interval comprising at least one year
2
For DNR, DNL and NSR, measurement starts at the time the request is COMPLETED.  For all other requests, measurement starts at the time the request is QUEUED.
3
Confirmation time limits are not to be interpreted to extend scheduling deadlines.

4
Measurement starts at the time the request is first moved to either ACCEPTED or COUNTEROFFER.  The time limit does not reset on subsequent changes of state.

5
Measurement starts at the time the Network Customer changes the state to REBID.  The measurement resets each time the request is changed to REBID.

6
Days are defined as calendar days.

7

Transmission Providers shall make best efforts to respond within 72 hours or prior to the scheduling deadline, whichever is earlier.

8
Whenever feasible and on a nondiscriminatory basis, Transmission Providers should accommodate requests made with less than 60 days notice.

9 
When the Concomitant Evaluation flag has been set, the timing requirements of the associated request for PTP transmission service, designation of Network Resource or Secondary Network Transmission shall apply.
001-105.1.6
The following requests are considered modification of service:
· Request for designation of a new Network Resource

· Request for temporary termination of Network Resource

· Request for indefinite termination of Network Resource

· Request for designation of new Network Load

· Request for undesignation of Network Load

· Request for new NSR

· Request for Concomitant Evaluation

· Request to change to NITS Application STOP_TIME
001-105.2
Designation of New Network Resource 
001-105.2.1
Submission of Request for Designation of New Network Resource

001-105.2.1.1
The Network Customer may enter and update information on OASIS to support a request for designation of new Network Resource prior to submission to the Transmission Provider by setting the status of the request to PRESUBMITTED.

001- 105.2.1.2
The Eligible Transmission Customer shall indicate to the Transmission Provider that a request for designation of new Network Resource is ready for Transmission Provider review for consistency with the Transmission Provider’s minimum information requirements which are posted as specified in Business Practice Standards WEQ-001-13, by submitting the request for designation of new Network Resource on OASIS..  

001- 105.2.1.3
The Network Customer shall indicate to the Transmission Provider that the request for designation of new Network Resource is ready for Transmission Provider review by submitting the request on OASIS.  

001-105.2.1.3.1
OASIS shall set the queue time for the request for designation of new Network Resource to the time that the Network Customer submits the request, and shall set the status of the request to QUEUED.

001-105.2.1.3.2
The Transmission Provider shall acknowledge the queued request for designation of new Network Resource by setting the status of the request to RECEIVED.

001-105.2.2
Initial Review of Request for Designation of New Network Resource

001- 105.2.2.1
The Transmission Provider shall respond to the request by setting the status to one of the following:

I.
INVALID 

II.
DEFICIENT 

III.
COMPLETED 

001-105.2.3
Invalid Request for Designation of New Network Resource

001-105.2.3.1
If the required information, as specified in the Transmission Provider’s minimum information requirements that are posted in accordance with Business Practice Standards WEQ-001-13, has not been provided in the request for designation of new Network Resource, the Transmission Provider shall set the status of the request for designation of new Network Resource to INVALID. The request for designation of new Network Resource shall be deemed withdrawn and will receive no further consideration.

001-105.2.4
Deficient Request for Designation of New Network Resource
001-105.2.4.1
For each valid request for designation of new Network Resource, the Transmission Provider shall determine if the request can be deemed sufficient (i.e. consistent with the Transmission Provider’s OATT requirements for the request for designation of new Network Resource).  

001-105.2.4.1.1
If deemed sufficient, the Transmission Provider shall set the status of such request to COMPLETED.  

001-105.2.4.1.2
If deemed insufficient, the Transmission Provider shall set the status of such request to DEFICIENT. 

001-105.2.4.2
A request for designation of a new Network Resource shall be accompanied by an attestation. 
001-105.2.4.3
If the Transmission Provider determines that the request for designation of new Network Resource is deficient, the Transmission Provider shall set the request to DEFICIENT and begin informal communications with the Network Customer to resolve the deficiencies. 

001-105.2.4.4
The Network Customer shall remedy the deficiency and shall set the status of the request for designation of new Network Resource to REEVALUATE.

001-105.2.4.5
Upon notification that the Network Customer has provided the necessary information, either on or off OASIS as appropriate, to remedy an identified deficiency, the Transmission Provider shall review the provided information and take one of the following actions:

001-105.2.4.5.1
If the information provided fails to remedy the deficiency, the Transmission Provider shall set the status of the request for designation of new Network Resource to DEFICIENT.

001-105.2.4.5.2
If the information provided remedies the deficiency, the Transmission Provider shall set the status of the request for designation of new Network Resource to COMPLETED.

001-105.2.4.5.3
If the information provided is deemed by the Transmission Provider to be a significant change to the request for designation of new Network Resource, the Transmission Provider shall set the status of the request to DECLINED.

001-105.2.4.5.3.1
Once the status of the request for designation of new Network Resource has been set to DECLINED, the Transmission Provider shall set the status of all portions of the request for designation of new Network Resource to DECLINED.

001-105.2.4.6
If the Network Customer fails to respond to the Transmission Provider’s notice of a deficient request for designation of new Network Resource, or fails to remedy the deficiency to the Transmission Provider’s satisfaction, the Transmission Provider shall set the status of the request to DECLINED.

001-105.2.4.6.1
Once the status of the request for designation of new Network Resource has been set to DECLINED, the Transmission Provider shall set the status of all portions of the request for designation of new Network Resource to DECLINED.

001-105.2.5
Completed Request for Designation of New Network Resource

001-105.2.5.1
Once the Transmission Provider has set the status of the request for designation of new Network Resource to COMPLETED, the Transmission Provider shall continue processing the request in accordance with Business Practice Standard WEQ-001-105.2.6.

001-105.2.6
Review of Completed Request for Designation of New Network Resource

001- 105.2.6.1
Once the Transmission Provider has set the status of the request for designation of new Network Resource to COMPLETED, the Transmission Provider shall respond to the request, consistent with filed tariffs, within the Provider Response Time Limit defined in Table 105-A, NITS Request Timing Requirements. To indicate its evaluation of the request, a Transmission Provider may respond by setting the state of the request to one of the following:

I.
STUDY

II.
DECLINED

III.
REFUSED 

IV.
COUNTEROFFER

VI.
ACCEPTED 

001-105.2.7
Study of Request for Designation of New Network Resource

001-105.2.7.1 
If the Transmission Provider determines that some level of study is required or is being performed, the Transmission Provider shall set the status of the request for designation of new Network Resource to STUDY.

001-105.2.7.2
From the status of STUDY, the Transmission Provider shall set the status of the request for designation of new Network Resource to DECLINED, REFUSED, COUNTEROFFER, or ACCEPTED.

001-105.2.8
Decline of Request for Designation of New Network Resource

001-105.2.8.1
If, at any time in the evaluation of the request for designation of new Network Resource or conduct of any associated study(ies), the Transmission Provider determines that the Network Customer has failed to meet any of the terms and conditions for the designation of a new Network Resource (e.g., failure to execute a study agreement, etc.), the Transmission Provider shall set the status of the request to DECLINED. 
001-105.2.8.1.1
Once the status of the request for designation of a new Network Resource has been set to DECLINED, the Transmission Provider shall set the status of all portions of the request for designation of a new Network Resource to DECLINED.

001-105.2.9
Refusal of Request for Designation of New Network Resource

001-105.2.9.1
If, at any time in the evaluation of the request for designation of new Network Resource or conduct of any associated study(ies), the Transmission Provider determines that the requested service cannot be provided due to lack of availability of transfer capability consistent with the Transmission Provider’s tariff requirements, the Transmission Provider may set the status of the request to REFUSED.

001-105.2.10
Counteroffer to Request for Designation of New Network Resource

001-105.2.10.1
When, in the evaluation of a request for designation of new Network Resource or on completion of any applicable study(ies), the Transmission Provider determines that the request may be granted in part, the Transmission Provider may make a partial offer.  

001-105.2.10.1.1
If the Transmission Provider makes a partial offer, the Transmission Provider shall notify the Network Customer on OASIS by setting the amount of Network Resource capacity granted over time in the response to the request for designation of new Network Resource and setting the status of such request to COUNTEROFFER.

001-105.2.11
Acceptance of Request for Designation of New Network Resource

001-105.2.11.1
When, in the evaluation of the request for designation of new Network Resource or on completion of any applicable study(ies), the Transmission Provider determines that the request can be approved, the Transmission Provider shall set the status of the request to ACCEPTED.

001-105.2.12
Customer Actions

001-105.2.12.1
From ACCEPTED or COUNTEROFFER, the Network Customer may change the status of the request for designation of new Network Resource to CONFIRMED, REBID or WITHDRAWN.  The Network Customer has the amount of time designated as Customer Confirmation Time Limit in Table 105-A NITS Request Timing Requirements to change the state of the request.  The Customer Confirmation Time Limit is measured from the first time the request is moved to ACCEPTED or COUNTEROFFER, and is not reset with subsequent iterations of negotiation. 

001-105.2.12.1.1
The Network Customer shall use the request status of REBID to initiate negotiation of designated Network Resource capacity or engage in continued negotiation of the designated Network Resource capacity with the Transmission Provider.

001-105.2.12.1.2
The Network Customer may negotiate for a lower amount of designated Network Resource capacity by updating the capacity requested over time and changing the request status from ACCEPTED or COUNTEROFFER to REBID.  For example, the Network Customer may choose to coordinate the level of transmission service purchased across multiple Transmission Providers.  Negotiation for a higher amount of designated Network Resource capacity shall not be allowed.  A Network Customer seeking additional designated Network Resource capacity must submit a new request, which will be assigned a new queue time.

001-105.2.12.1.3
Once the status of the request for designation of a new Network Resource has been set to ACCEPTED or COUNTEROFFER, the Network Customer may change the status to CONFIRMED after positively acknowledging agreement by setting the requested designated Network Resource capacity equal to the Transmission Provider’s granted designated Network Resource capacity.  Alternatively, the Network Customer may withdraw the request or the Network Customer may continue negotiations consistent with Business Practice Standard WEQ-001-105.2.12.1.2.

001-105.2.13
Final Disposition of Request for Designation of New Network Resource

001-105.2.13.1
Should the Network Customer elect to initiate or continue the negotiation of designation of new Network Resource capacity by moving the request to REBID, the Transmission Provider shall respond by setting the status of the request to DECLINED, ACCEPTED, or COUNTEROFFER within the Provider Counter Time Limit as specified in Table 105-A NITS Request Timing Requirements.  
001-105.2.13.1.1
The Transmission Provider shall positively acknowledge agreement to the designation of new Network Resource capacity over time by setting the granted new Network Resource capacity equal to the Network Customer’s requested new Network Resource capacity prior to acceptance of the request.   

001-105.2.13.2
If the Network Customer fails to set the status of the ACCEPTED request for designation of new Network Resource to CONFIRMED or WITHDRAWN within the time limits specified in Table 105-A NITS Request Timing Requirements, the Transmission Provider may set the status of the request to RETRACTED. 

001-105.3
Termination
 of Network Resource

001-105.3.1
A Network Customer may terminate the designation of a Network Resource on a temporary or indefinite basis for all or a portion of the unscheduled capacity of the Network Resource.

001-105.3.1.1
If a Network Customer submits a request for termination of Network Resource with a term that does not extend to the end of the term of the designated Network Resource, such request shall be a request for temporary termination of Network Resource.  
001-105.3.1.2
If a Network Customer submits a request for termination of Network Resource with term that extends to the end of the term of the designated Network Resource, such request shall be a request for indefinite termination of Network Resource. 

001-105.3.1.3
The Network Customer shall ensure any necessary adjustments of the schedule/e-Tag prior to submitting the request for termination of Network Resource.

001-105.3.1.3
The Transmission Provider shall not deny a valid and complete request for termination of Network Resource.

001-105.3.2
Temporary Termination of Network Resource

001-105.3.3
Submission of Request for Temporary Termination of Network Resource 

001-105.3.3.1
A request for temporary termination of Network Resource must include redesignation of the same resource in accordance with Business Practice Standards WEQ 001-105-xxx.
001-105.3.3.2
Prior to submission to the Transmission Provider, the Network Customer may enter and update information on OASIS to request temporary termination of Network Resource by setting the status of the request to PRESUBMITTED.  

001- 105.3.3.3
The Network Customer shall indicate to the Transmission Provider that the request for temporary termination of Network Resource is ready for Transmission Provider review by submitting the request on OASIS.  

001-105.3.3.4
OASIS shall set the queue time for the request for temporary termination of Network Resource to the time that the Network Customer submits the request and shall set the request to QUEUED.

001-105.3.3.5
The Transmission Provider shall acknowledge the queued request for temporary termination of a Network Resource by setting the status of the request to RECEIVED.

001-105.3.4
Initial Review of Request for Temporary Termination of Network Resource 

001- 105.3.4.1
The Transmission Provider shall respond to a request by setting the status to one of the following:

I.
INVALID 

II.
DEFICIENT 

III.
COMPLETED 

001-105.3..5
Invalid Request for Temporary Termination of Network Resource 

001-105.3.5.1
If the required information, as specified in the Transmission Provider’s minimum information requirements that are posted  in accordance with Business Practice Standards WEQ-001-13, has not been provided in the request for temporary termination of Network Resource, the Transmission Provider shall set the status of the request for temporary termination of Network Resource to INVALID. The request for temporary termination of Network Resource shall be deemed withdrawn and will receive no further consideration.

001-105.3.6
Deficient Request for Temporary Termination of Network Resource 

001-105.3.6.1
For each valid request for temporary termination of Network Resource, the Transmission Provider shall determine if the request can be deemed sufficient (i.e. consistent with the Transmission Provider’s OATT requirements for the request for temporary termination of Network Resource).  

001-105.3.6.1.1
If deemed sufficient, the Transmission Provider shall set the status of such request to COMPLETED.  

001-105.3.6.1.2
If deemed insufficient, the Transmission Provider shall set the status of such request to DEFICIENT
001-105.3.6.2
Redesignation of the Network Resource shall meet the following requirements:

001-105.3.6.2.1
Any redesignation shall be accompanied by an attestation. 

001-105.3.6.2.2
The STOP_TIME for the temporary termination of Network Resource shall also be the effective date and time that the same Network Resource is redesignated.

001-105.3.6.2.3
The request shall include all data necessary consistent with the requirements for designation of a new Network Resource.  Previously submitted data may be incorporated by reference with the exception of the attestation required in Business Practice Standard WEQ 001-105.3.5.2.

001-105.3.6.3
If the Transmission Provider determines that the request for temporary termination of Network Resource is deficient, the Transmission Provider shall set the request to DEFICIENT and begin informal communications with the Eligible Transmission Customer to resolve the deficiencies. 

001-105.3.6.4
The Network Customer shall remedy the deficiency and shall set the status of the request for temporary termination of Network Resource to REEVALUATE.

001-105.3.6.5
Upon notification that the Network Customer has provided the necessary information, either on or off OASIS as appropriate, to remedy an identified deficiency, the Transmission Provider shall review the provided information and take one of the following actions:

001-105.3.6.5.1
If the information provided fails to remedy the deficiency, the Transmission Provider shall set the status of the request for temporary termination of Network Resource to DEFICIENT.

001-105.3.6.5.2
If the information provided remedies the deficiency, the Transmission Provider shall set the status of the request for temporary termination of Network Resource to COMPLETED.

001-105.3.6.5.3
If the information provided is deemed by the Transmission Provider to be a significant change to the request for temporary termination of Network Resource, the Transmission Provider shall set the status of the request to DECLINED.

001-105.3.6.5.3.1
Once the status of the request for temporary termination of Network Resource has been set to DECLINED, the Transmission Provider shall set the status of all portions of the request for temporary termination of Network Resource to DECLINED.

001-105.3.6.6
If the Network Customer fails to respond to the Transmission Provider’s notice of a deficient request for temporary termination of Network Resource, or fails to remedy the deficiency to the Transmission Provider’s satisfaction in a timely manner, the Transmission Provider shall set the status of the request to DECLINED.

001-105.3.6.6.1
Once the status of the request for temporary termination of Network Resource has been set to DECLINED, the Transmission Provider shall set the status of all portions of the request for temporary termination of Network Resource to DECLINED.

001-105.3.6.7
In response to a state of DECLINED, the Network Customer may submit a new request for temporary termination of Network Resource, which will be assigned a new queue time.  

001-105.3.7
Completed Request for Temporary Termination of Network Resource 

001-105.3.7.1
Once the Transmission Provider has set the status of the request for temporary termination of Network Resource to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these Business Practice Standards.

001-105.3.8
Acceptance of Request for Temporary Termination of Network Resource 

001-105.3.8.1
Once the status of the request for temporary termination of Network Resource is set to COMPLETED, OASIS shall set the status of the request to ACCEPTED.

001-105.3.9
Customer Actions

001-105.3.9.1
After the status of the request for temporary termination of Network Resource has been set to ACCEPTED by the Transmission Provider, the Network Customer may confirm or withdraw the request by setting the status to CONFIRMED or WITHDRAWN. 

001-105.3.9.2
The Network Customer has the amount of time designated as Customer Confirmation Time Limit in Table 105-A NITS Request Timing Requirements to change the state of the request.  
001-105.3.10
Final Disposition of Request for Temporary Termination of Network Resource 

001-105.3.10.1
If the Network Customer fails to set the status of the ACCEPTED request for temporary termination of Network Resource to CONFIRMED or WITHDRAWN within the time limits specified in Table 105-A NITS Request Timing Requirements, the Transmission Provider may set the status of the request to RETRACTED. 

001-105.3.11
Indefinite Termination of Network Resource

001-105.3.12
Submission of Request for Indefinite Termination of Network Resource

001-105.3.12.1
Prior to submission to the Transmission Provider, the Network Customer may enter and update information on OASIS to request indefinite termination of Network Resource by setting the status of the request to PRESUBMITTED. 

001- 105.3.12.2
The Network Customer shall indicate to the Transmission Provider that the request for indefinite termination of Network Resource is ready for Transmission Provider review by submitting the request on OASIS.  

001-105.3.12.2.1
OASIS shall set the queue time for the request for indefinite termination of Network Resource to the time that the Network Customer submits the request and shall set the request to QUEUED.

001-105.3.12.2.2
The Transmission Provider shall acknowledge the queued request for indefinite termination of Network Resource by setting the status of the request to RECEIVED.

001-105.3.13
Initial Review of Request for Indefinite Termination of Network Resource

001-105.3.13.1
The Transmission Provider shall respond to a request for indefinite termination of Network Resource by setting the status to one of the following:

I.
INVALID 

II.
COMPLETED

001-105.3.14
Invalid Request for Indefinite Termination of Network Resource

001-105.3.14.1
If the required information, as specified in the Transmission Provider’s minimum information requirements that are posted  in accordance with Business Practice Standards WEQ-001-13, has not been provided in the request for indefinite termination of Network Resource, the Transmission Provider shall set the status of the request for indefinite termination of Network Resource to INVALID. The request for indefinite termination of Network Resource shall be deemed withdrawn and will receive no further consideration.

001-105.3.15
Completed Request for Indefinite Termination of Network Resource 

001-105.3.15.1
Once the Transmission Provider has set the status of the request for indefinite termination of Network Resource to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these Business Practice Standards.

001-105.3.16
Acceptance of Request for Indefinite Termination of Network Resource

001-105.3.16.1
Once the request for indefinite termination of Network Resource is deemed to be valid, OASIS shall set the status of the request to ACCEPTED.

001-105.3.17
Customer Actions

001-105.3.17.1
After the status of the request for indefinite termination of Network Resource has been set to ACCEPTED by the Transmission Provider, the Network Customer may confirm or withdraw the request by setting the status to CONFIRMED or WITHDRAWN. 

001-105.3.17.2
The Network Customer has the amount of time designated as Customer Confirmation Time Limit in Table 105-A NITS Request Timing Requirements to change the state of the request.  

001-105.3.18
Final Disposition of Request for Indefinite Termination of Network Resource

001-105.3.18.1
If the Network Customer fails to set the status of the ACCEPTED request for indefinite termination of Network Resource to CONFIRMED or WITHDRAWN within the time limits specified in Table 105-A NITS Request Timing Requirements, the Transmission Provider may set the status of the request to RETRACTED.
001-105.4
Designation of New Network Load or Undesignation of Network Load
001-105.4.1
The Network Customer shall submit a request for designation of new Network Load or undesignation of Network Load on OASIS, including the information specified in Business Practice Standard WEQ-001-101.13. 
001-105.4.1.1
A request for designation of new Network Load may be a request for either (1) a load addition to a Network Customer’s existing Network Load(s) exclusive of projected load growth or (2) a load addition that is separate from a Network Customer’s existing Network Load(s).

001-105.4.1.2
A request for designation of new Network Load or undesignation of Network Load shall be submitted and processed in accordance with Business Practice Standards WEQ-002, WEQ-003, and WEQ-013.
001-105.4.2
Submission of Request for Designation of New Network Load or Undesignation of Network Load

001-105.4.2.1
The Network Customer may enter and update information on OASIS in association with a request for designation of new Network Load or undesignation of Network Load prior to submission to the Transmission Provider by setting the status of the request to PRESUBMITTED.

001- 105.4.2.2
The Network Customer shall indicate to the Transmission Provider that the request for designation of new Network Load or undesignation of Network Load is ready for Transmission Provider review by submitting the request on OASIS.  

001-105.5
NSR
001-105.5.1
Enabling the NSR is at the Transmission Provider’s discretion.  If enabled, the request for new NSR shall, at the Transmission Provider’s discretion, allow either:


(i) the Network Customer to request network transmission capacity, by submitting a request for new NSR on OASIS, or 


(ii) the Transmission Provider to post on OASIS the scheduling rights associated with NITS by completing a request for new NSR and setting the status of the request to ACCEPTED.   

001-105.5.1.1
If the request for new NSR is used, the Transmission Provider shall specify, in accordance with Business Practice Standards WEQ-001-13, the minimum information required.

001-105.5.1.2
If the request for termination of NSR is used, the Transmission Provider shall specify, in accordance with Business Practice Standards WEQ-001-13, the minimum information required. 
001-105.5.1.3
A request for new NSR or termination of NSR shall be submitted and processed in accordance with Business Practice Standards WEQ-002, WEQ-003, and WEQ-013
001-105.5.2
Submission of Request for New NSR

001-105.5.2.1
The Network Customer may enter and update information on OASIS to support a request for new NSR prior to submission to the Transmission Provider by setting the status of the request to PRESUBMITTED.

001- 105.5.2.2
The Network Customer shall indicate to the Transmission Provider that the request for new NSR is ready for Transmission Provider review by submitting the request on OASIS.  

001-106
SECONDARY NETWORK TRANSMISSION SERVICE

001-106.1
Request for Secondary Network Transmission Service 

001-106.1.1
The Network Customer shall indicate to the Transmission Provider that the request for Secondary Network Transmission Service is ready for Transmission Provider review by submitting the request on OASIS

001-106.1.2
OASIS shall set the queue time for the request for Secondary Network Transmission Service to the time that the Network Customer submits the request and shall set the status of the request to QUEUED.  

001-106.1.3
The Transmission Provider shall acknowledge the queued request for Secondary Network Transmission Service by setting the status of the request to RECEIVED.

001- 106.1.4
The Transmission Provider shall respond to the request, consistent with filed tariffs, within the Provider Response Time Limit defined in Table 105-A NITS Request Timing Requirements. The time limit is measured from the time the request is QUEUED.  To indicate its evaluation of the request, a Transmission Provider may respond by setting the state of the request to one of the following:

I.       INVALID

II.
DECLINED

III.
REFUSED 

IV.
SUPERSEDED 

V.
COUNTEROFFER

VI.
ACCEPTED

001-106.1.5
From ACCEPTED or COUNTEROFFER, the Network Customer may change the status of the request for Secondary Network Transmission Service to CONFIRMED, REBID or WITHDRAWN. The Network Customer has the amount of time designated as Customer Confirmation Time Limit in Table 105-A NITS Request Timing Requirements to change the state of the request. The Customer Confirmation Time Limit is measured from the first time the request is moved to ACCEPTED or COUNTEROFFER, and is not reset with subsequent iterations of negotiation. 

001-106.1.6
The Network Customer shall use the request status of REBID to initiate or continue the negotiation of Secondary Network Transmission Service capacity.  

001-106.1.7
Should the Network Customer elect to initiate or continue the negotiation of Secondary Network Transmission Service capacity by changing the status of the request for Secondary Network Transmission Service to REBID, the Transmission Provider shall respond by changing the status of the request to DECLINED, SUPERSEDED, ACCEPTED, or COUNTEROFFER within the Provider Counter Time Limit, specified in Table 105-A NITS Request Timing Requirements. The Transmission Provider response time is measured from the most recent REBID time.

001-106.1.8
The Network Customer may negotiate for a lower amount of Secondary Network Transmission Service capacity by updating the capacity requested over time and changing the request status from ACCEPTED or COUNTEROFFER to REBID. Negotiation for a higher amount of Secondary Network Transmission Service capacity shall not be allowed. A Network Customer seeking additional Secondary Network Transmission Service capacity must submit a new request, which will be assigned a new unique identifier and a new queue time.

001-106.1.9
Once the status of the request for Secondary Network Transmission Service has been set to ACCEPTED or COUNTEROFFER, the Network Customer may change the status to CONFIRMED after positively acknowledging agreement by setting the requested Secondary Network Transmission Service capacity equal to the Transmission Provider’s granted Secondary Network Transmission Service capacity.  Alternatively, the Network Customer may withdraw the request or the Network Customer may continue negotiations consistent with Business Practice Standard WEQ-001-105.1.1.5.

001-106.1.10
If, within the time limits specified in Table 105-A NITS Request Timing Requirements, the Network Customer fails to set the status of the ACCEPTED request for Secondary Network Transmission Service to CONFIRMED or WITHDRAWN, the Transmission Provider may set the status of the request to RETRACTED.

001-106.2
Releasing Secondary Network Transmission Service Capacity

001-106.2.1
The Network Customer shall have the right to request the Transmission Provider to release all or part of unscheduled capacity associated with a confirmed request for Secondary Network Transmission Service capacity by using the request type RELINQUISH. 
001-106.2.2
Requests for releasing Secondary Network Transmission Service capacity shall be submitted pre-confirmed.
001-106.2.3
Requests for releasing Secondary Network Transmission Service capacity shall be bound by the start/stop times of the Secondary Network Transmission Service reservation  and may not be requested for past hours.

001-106.2.4
The Network Customer shall ensure any necessary adjustments of the schedule/e-Tag prior to submitting the request for releasing Secondary Network Transmission Service capacity.

001-106.2.5
The Transmission Provider shall have the right to refuse the request for releasing Secondary Network Transmission Service capacity if the capacity requested for release is not available for the duration of the release request.

001-106.2.6
Upon confirmation of the request for releasing Secondary Network Transmission Service capacity, the available non-firm transfer capability shall be increased by the amount released and for the time period of the release.

001-106.2.7
Upon confirmation of the requests for releasing Secondary Network Transmission Service capacity, the capacity available on the Secondary Network Transmission Service shall be decreased by the amount released and for the time period of the release.

Note:  Need implementation plan 

001-107
NITS –CONCOMITANT EVALUATION

001-107.1
A Concomitant Evaluation only applies to the combination of a request for temporary or indefinite termination of a single Network Resource and a single request for NITS or PTP transmission service on a single transmission provider system.

001-107.1.1
To be eligible for Concomitant Evaluation, each of the two single requests must be submitted simultaneously with the Concomitant Evaluation flag set. 

001-107.1.2
The Transmission Provider shall evaluate the two single requests concomitantly.  Such Concomitant Evaluation must take into account the termination of those Network Resources, as well as all competing requests of higher priority.

001-107.1.2.1
If one of the requests is for PTP transmission service, then the reservation priority for both requests shall be consistent with reservation priority of the PTP request as specified in Business Practice Standard WEQ-001-4.16 Table 4-3.

001-107.1.3
If the Network Customer sets the pre-confirmation flag on one of the requests, the Network Customer shall also set the pre-confirmation flag on the other request.
001-107.1.4
In circumstances where the request for new transmission service cannot be granted in its entirety, and an increase in MW value of the termination of a single Network Resource would allow granting of the request for new transmission service in its entirety, the Transmission Provider may, at its sole discretion and on a non-discriminatory basis, counteroffer the request for termination of DNR at a MW value greater than that of the original request for termination of DNR.

001-107.1.5
The withdrawal of one of the single requests submitted for Concomitant Evaluation will result in the withdrawal of both single requests, including any preconfirmed requests.
001-107.1.6
All requests for Concomitant Evaluation shall be assigned the same queue time..  Such requests shall have the same status during the Concomitant Evaluation process, including any final status.

001-107.2
A single request that does not have the Concomitant Evaluation flag set will be considered a separate request and will be processed according to the appropriate Business Practice Standards for such request.

001-107.3
Denial of one of the single requests shall result in denial of both single requests. 

001- 107.4
When the Concomitant Evaluation flag has been set for a PTP transmission service request, then the request timing requirements contained in Business Practice Standard WEQ-001-4.13 Table 4-2 shall apply. 

001- 107.5
When the Concomitant Evaluation flag has been set for a request for designation of Network Resource or Secondary Network Transmission Service request, then the request timing requirements for the request for  designation of Network Resource or Secondary Network Transmission Service contained in Business Practice Standard WEQ-001-105.1.5 Table 5-A shall apply. 

001-107.6
If the Transmission Provider counteroffers either of the single requests, the status of both requests shall be set to COUNTEROFFER.

001-107.6.1
Should the Network Customer elect to initiate or continue the negotiation of the two single requests by changing the status of the two requests to REBID, the Transmission Provider shall respond by changing the status of the request to DECLINED, ACCEPTED, or COUNTEROFFER. All responses shall be within time limits specified in the applicable timing requirements table.
001-107.7
After the status of both single requests has been set to ACCEPTED by the Transmission Provider, the Network Customer may confirm or withdraw the requests by setting the status of both single requests to CONFIRMED or WITHDRAWN. 

001-107.8
Concomitant Evaluation Final Disposition

001-107.8.1
If the Network Customer fails to set the status of both single ACCEPTED requests to CONFIRMED or WITHDRAWN within the time limits specified in the applicable timing requirements table, the Transmission Provider may set the status of both single requests to RETRACTED.

001-107.8.1.1
In response to a state of RETRACTED, the Network Customer may submit new requests consisting of two single requests both with the Concomitant Evaluation flag set, which will be assigned a new queue time.
001-107.8.2
After the status of the two single requests has been set to CONFIRMED, the Concomitant Evaluation process shall be terminated. Any subsequent change in the status of one, shall have no impact on the status of the other.

001-108
NITS – CHANGE TO NITS APPLICATION STOP_TIME 

001-108.1
The Network Customer shall submit a request to change a NITS Application STOP_TIME in accordance with Business Practice Standards WEQ-002 and WEQ-013.

001-108.2
If the Network Customer fails to provide timely notice of its intent to exercise its rights to continue transmission service, the NITS Application STOP_TIME shall not change.

001-108.2.1
Any portions of the NITS Application that have a STOP_TIME that is later than the NITS Application STOP_TIME, shall be set by the Transmission Provider to the same STOP_TIME as the NITS Application.
In WEQ-013 include in text of standard

State Diagram for NITS Application

State Diagram for Request for Modification of Service

State Diagram for Secondary Network Transmission Service

Will need to put in an amended state diagram for the point to point standard

APPENDIX XXX

	  STATE
	PRESUBMITTED
	QUEUED
	RECEIVED
	DEFICIENT
	REEVALUATE
	COMPLETED
	STUDY
	COUNTER

-OFFER
	REBID
	ACCEPTED
	CONFIRMED

	NITS Application
	Y
	Y
	Y
	Y
	Y
	Y
	Y
	Y
	N
	Y
	Y

	Request to Designate Network Resource
	Y
	Y
	Y
	Y
	Y
	Y
	Y
	Y
	Y
	Y
	Y

	Temporary Termination  of Network Resource (with redesignation of the same Network Resource)
	Y
	Y
	Y
	Y
	Y
	Y
	N
	N
	N
	Y
	Y

	Designate Network Load
	Y
	Y
	Y
	Y
	Y
	Y
	Y
	N
	N
	Y
	Y

	Indefinite Termination of Network Resource
	Y
	Y
	Y
	N
	N
	Y
	N
	N
	N
	Y
	Y

	Indefinite Termination Of Network Load
	Y
	Y
	Y
	N
	N


	Y
	N
	N
	N
	Y
	Y

	Request NSR
	Y
	Y
	Y
	N
	N
	Y
	Y
	Y
	Y
	Y
	Y

	Temporary Termination NSR
	Y
	Y
	Y
	N
	N
	Y
	N
	N
	N
	Y
	Y

	Indefinite Termination NSR
	Y
	Y
	Y
	N
	N
	Y
	N
	N
	N
	Y
	Y

	Request Secondary Network Resource
	Y
	Y
	Y
	N
	N
	Y
	N
	Y
	Y
	Y
	Y

	Terminate Secondary Network Resource
	Y
	Y
	Y
	N
	N
	Y
	N
	N
	N
	Y
	Y


WEQ-002 revisions

WEQ-002 additions

WEQ-003 revisions and WEQ-003 additions

WEQ-013 revisions

WEQ-013 additions

4.  SUPPORTING DOCUMENTATION
a.  Description of Request:

NORTH AMERICAN ENERGY STANDARDS BOARD

2011 ANNUAL PLAN for the WHOLESALE ELECTRIC QUADRANT

	2
	Develop business practice standards in support of the FERC RM05-25-000 and RM05-17-000 (OATT Reform)


	
	a)
	Develop version 2 business practice standards to better coordinate the use of the transmission system among neighboring transmission providers.  

Status: Underway

Request R050004 was expanded to include the Order No. 890 (Docket Nos.RM05-17-000 and RM02-25-000),  (Order No. 890-A (Docket Nos. RM05-17-001, 002 and RM05-25-001, 002), and Order No. 890-B (Docket Nos. RM05-17-03 and RM05-25-03) “Preventing Undue Discrimination and Preference in Transmission Services”

	
	
	i)
	Group 3: Network Service On OASIS

	
	
	
	1) Use of OASIS to Make Electronic Requests to Designate and Terminate  Network Resource

Status:  Underway
	4th Q, 2010
	OASIS

	
	
	
	2) Ability to Query Requests to Designate and Terminate Network Resources and Allow for Queries of All Information Provided with Designation Requests

Status:  Underway
	4th Q, 2010
	OASIS

	
	
	
	3) Masking of Designated Network Resource Operating Restrictions and Generating Cost Information

Status:  Underway
	4th Q, 2010
	OASIS

	
	
	
	4) Procedural Requirements for Submitting Designations over new OASIS Functionality

Status:  Underway
	4th Q, 2010
	OASIS

	
	
	
	5) Specify How Designated Network Service Informational Postings are Posted on OASIS

Status:  Underway
	4th Q, 2010
	OASIS

	
	
	
	6) Develop standards for the treatment of OASIS Requests when the Customer Fails to Provide the Necessary Attestation

Status:  Underway
	4th Q, 2010
	OASIS

	
	
	
	7) Procedural Requirements for Submitting Both Temporary and Indefinite Terminations of Network Resources

Status:  Underway
	4th Q, 2010
	OASIS

	
	
	
	8) Procedures for Submitting and Processing Requests for Concomitant Evaluations of Transmission Requests and Temporary Terminations

Status:  Underway
	4th Q, 2010
	OASIS

	b)
	Develop the needed business practices to support existing Request No. R05004: The  processing of transmission service requests, which use  TTC/ATC/AFC, in coordination with NERC changes to MOD 001 where the allocation of flowgate capability based on historical Network Native Load impacts the evaluation of transmission service requests, requiring the posting of those allocation values in conjunction with queries of service offerings on OASIS

Status:  Underway

	
	i)
	

	
	
	

	3
	Develop business practices standards to improve the current operation of the wholesale electric market and develop and maintain business practice and communication standards for OASIS and Electronic Scheduling

	
	a)
	Develop and/or maintain business practice standards as needed for OASIS and electronic scheduling. Specific items to address include:

	
	
	i)
	Network Services: Determine and develop needed business practice standards or other support is needed to support use of OASIS for Network Service transactions (R04006E). (Related to AP 2(a)(iii))

Status: Underway 


FERC Orders

In FERC Order 890 under Docket Nos. RM05-17-000 and RM05-25-000 issued February 16, 2007, the Commission made the following conclusions regarding Network Integration Service on OASIS:

385.  The Commission adopts the NOPR proposal and requires transmission providers and network customers to use OASIS to request designation of new network resources and to terminate designation of network resources.  This information shall be posted on OASIS for 90 days and available for audit for a five-year period.  Transmission customers thus shall be able to query requests to designate and terminate a network resource.  This requirement adds valuable transparency without undue burden, since it is nothing more than maintaining a database of designation requests made and responded to electronically.  The Commission orders public utilities, working through NAESB, to develop appropriate templates for OASIS. [footnote omitted]

1477.  We direct transmission providers to develop OASIS functionality to (1) allow all of the information required for a request to designate network resources to be provided electronically, (2) mask information about operating restrictions and generating cost on OASIS
, and (3) allow for queries of all information provided with designation requests in accordance with section 37.6 of the Commission’s regulations.  As provided in paragraph 385, we also direct transmission providers to work in conjunction with NAESB to develop business practice standards describing procedural requirements for submitting designations over any new OASIS functionality.  Transmission providers need not implement this new OASIS functionality and any related business practices until NAESB develops appropriate standards.  Prior to implementation of this new OASIS functionality, any information that cannot be provided electronically may be submitted by transmitting the information to the transmission provider by telefax or providing the information by telephone over the transmission provider’s time recorded telephone line. [footnote omitted]

1377. The Commission agrees that transmission requests across multiple transmission systems should be coordinated by the relevant transmission providers. We will not, however, amend the pro forma OATT to require such coordination. Rather, we require transmission providers working through NAESB to develop business practice standards related to coordination of requests across multiple transmission systems. In order to provide guidance to NAESB, we will articulate the principles that should govern processing across multiple systems. All the transmission providers involved in a request across multiple systems should consider a request that requires studies across multiple systems to be a single application for purposes of establishing the deadlines for renderingan agreement for service, revising queue status, eliciting deposits and commencing service. In order to preserve the rights of other transmission customers with studies in the queue, the priority for the single application should be based on the latest priority across the transmission providers involved in the multiple system request. We note that regional entities like wesTTrans are already coordinating requests across multiple transmission systems and we believe such coordination is an acceptable solution to this issue.

1504.  In response to South Carolina E&G’s request, we reiterate that not all of the information required by section 29.2 of the pro forma OATT for designation of a network resource will be made publicly available on OASIS.  As discussed above, information about operating restrictions and generating cost will be masked to protect commercially sensitive information.  South Carolina E&G has also requested clarification of the Commission’s intent with respect to how designated network resource information is posted.  Our existing regulations specify the view, download, and query requirements for information posted regarding network resource designations.  The details of how those informational postings are accomplished are best left to be determined as part of the NAESB standards development process. [footnote omitted]

1532.  In response to TranServ’s request that the exact nature of how the customer would make an attestation should be determined in the NAESB forum, we note that the contents and the specific information that is required to be provided with the attestation are specified in the pro forma OATT, and we are requiring that the attestation be submitted through OASIS with each request to designate a new network resource.  The appropriate subject for transmission providers to coordinate with NAESB to resolve is limited to the appropriate formatting of such information to be provided in OASIS.  In response to TranServ's request that NAESB should also determine the treatment of OASIS requests where the customer fails to provide the necessary attestation, we point out that we have already directed that such requests are to be found deficient by the transmission provider and treated in accordance with the procedures in section 29.2 of the pro forma OATT.
1541.  We direct transmission providers to develop OASIS functionality and, working through NAESB, business practice standards describing the procedural requirements for submitting both temporary and indefinite terminations of network resources, to allow network customers to provide all required information for such terminations.  Such OASIS functionality should allow for electronic submittal of the type of termination (temporary or indefinite), the effective date and time of the termination, and identification and capacity of resource(s) or portions thereof to be terminated. For temporary terminations, such OASIS functionality should also allow for electronic submittal of (1) effective date and time of redesignation, following the period of temporary termination; (2) information and attestation for redesignating the network resource following the temporary termination, in accordance with section 30.2 of the pro forma OATT; and (3) identification of any related transmission service requests to be evaluated concomitantly with the request for temporary termination.  In response to TranServ’s request, we clarify that the request for temporary termination of the resource and the requests for the related transmission service identified in item (3), if any, should be evaluated as a single request, and approved or disapproved as such.  We specifically direct transmission providers, working through NAESB, to develop business standards describing the procedures for submitting and processing requests for concomitant evaluations of transmission requests and temporary terminations.  When processing such requests, the evaluation of the transmission service requests identified in item (3) should take into account the undesignation of the network resources identified in the request for termination.  However, the evaluation of the transmission service requests in item (3) should be processed taking proper account of all competing transmission service requests of higher priority.
In FERC Order 890-A under Docket Nos. RM05-17-001, 002 and RM05-25-001, 002 issued December 28, 2007, the Commission made the following conclusions regarding Network Integration Service on OASIS:

919.  The Commission clarifies, in response to South Carolina E&G’s request, that the language in paragraph 1521 of Order No. 890 is only meant to be a paraphrase of the more detailed attestation to be provided in the pro forma OATT itself.  A network customer designating network resources should submit an attestation using the language set forth in sections 29.2(viii) and 30.2 of the pro forma OATT, as amended in Order No. 890, not the language of the preamble.  A network customer is not permitted to merely reference the applicable section of the pro forma OATT when completing the attestation requirement.  If the OASIS customer comment section does not currently allow enough space for a network customer to provide its attestation, transmission providers should modify, in coordination with NAESB, OASIS functionality to accommodate the full attestation.  In the interim, the transmission provider should identify alternate means, such as by telefax or e-mail, for the network customer to provide the attestation. 

In FERC Order 890-B under Docket Nos. RM05-17-001, 002 and RM05-25-001, 002 issued June 23, 2008, the Commission made the following conclusions Network Integration Service on OASIS:

182.  The Commission grants rehearing to more accurately state the requirement to provide an attestation supporting the designation of network resources pursuant to sections 29.2(viii) and 30.2 of the pro forma OATT.  In order to designate a network resource, section 30.7 of the Order No. 888 pro forma OATT required each network customer to demonstrate that (i) it owns or has committed to purchase generation pursuant to an executed contract or (ii) execution of a contract is contingent upon the availability of transmission service in order to designate a generating resource.  In Order No. 890, the Commission adopted the attestation requirement as the means by which the network customer can make this demonstration, revising sections 29.2 and 30.2 accordingly.  We affirm this requirement, consistent with the network customer’s obligations under section 30.7, and grant rehearing of the Commission’s statements in this proceeding indicating that the attestation can instead be submitted at the time a resource designation is confirmed, rather than requested.

188.  In Order No. 890, the Commission directed transmission providers to evaluate as a single request a request for temporary undesignation and related requests for transmission service.  Transmission providers were therefore directed to develop, working through NAESB, business practices allowing for electronic identification of related transmission service requests to be evaluated concomitantly with the request for temporary undesignation.  This was appropriate in light of the Commission’s decision to allow network customers to temporarily undesignate their network resources without forfeiting the right to use the resource at a specified point in the future, provided they pair the temporary undesignation with a request to redesignate the resource.  

209.  We also conclude that concerns regarding the ability to verify or monitor the buyer’s decision to designate a purchase of system power as a network resource are overstated in light of the clarification that the buyer and seller must be on the same transmission system.  In Order No. 890, the Commission directed transmission providers, working through NERC, to develop OASIS functionality for the designation of network resources and for queries of information provided with designation requests.  Parties to a sale of system power on the same transmission system will therefore have ready access to the treatment of the resource.  Sellers also may rely on commitments made by the buyer to designate the purchase as a network resource.  [footnote omitted]

In FERC Order 890-C under Docket Nos. RM05-17-001, 002 and RM05-25-001, 002 issued March 19, 2009, the Commission made the following conclusions regarding Network Integration Service on OASIS:

17. The Commission affirms the requirement that network resources used to supply sales of system power to off-system buyers must first be undesignated. As we explained in Order No. 890, transactions in which a buyer and seller are both network customers located on the same transmission system are distinct from transactions involving sales of energy from a network customer to an off-system buyer. In the latter circumstance, the off-system buyer will not be using network service to take delivery from the host transmission provider and, instead, must identify the points of receipt and delivery for the transaction on the host transmission provider’s system, i.e., the points where capacity and energy will be received from the seller and delivered to the buyer. The point-to point transmission reservation and the corresponding resource-specific undesignation provide the transmission provider with the information it needs regarding the location of particular resources being used by the seller to source the transaction in order to model the effect of the transaction on its transmission system and set aside ATC accordingly. 

b.  Description of Recommendation:

See Section 3 Summary
c.  Business Purpose:


Implementation of FERC Orders 890, 890-A, 890-B and 890-C.
d.  Commentary/Rationale of Subcommittee(s)/Task Force(s):
NAESB will need to add appropriate links to this section.
· NITS Reference Document: 

· NITS Scope Document

· NITS Motions Document 

· Meeting Minutes
· February 12-13, 2007
· February 26, 2007
· March 15-16, 2007
· May 30-31, 2007
· October 9, 2007
· November 1, 2007
· November 8, 2007
· November 20, 2007
· January 31, 2008
· February 25, 2008
· March 3, 2008
· March 17, 2008
· March 24, 2008
· July 15-16, 2008
· September 17-19, 2008
· October 1, 2008
· October 15-16, 2008
· October 29, 2008
· November 12-14, 2008
· December 4, 2008
· December 15-16, 2008
· January 7, 2009
· January 20-22, 2009
· April 22, 2009
· May 6, 2009
· June 3, 2009
· June 17-19, 2009
· July 7-9, 2009
· July 22, 2009
· August 5, 2009
· August 19-20, 2009
· September 1, 2009
· September 29-30 & October 1, 2009
· October 28-29, 2009
· November 9, 2009
· November 17, 2009
· December 1-3, 2009
· December 7, 2009
· December 16, 2009
· January 12-14, 2010
· January 26, 2010
· February 3-4, 2010
· February 17, 2010
· March 2-4, 2010
· March 23, 2010
· April 13-15, 2010
· April 28, 2010
· May 5-6, 2010
· May 26-27, 2010
· June 2, 2010
· June 8-10, 2010
· June 17, 2010
· June 21, 2010 (draft)
· July 20-22, 2010
· August 4, 2010
· August 18-19, 2010
· August 24, 2010
· August 30-31, 2010
· September 8-10, 2010
· September 28-29, 2010
· October 12-13, 2010
· October 21, 2010
· October 27-29, 2010
· November 8-9, 2010
· November 15-17, 2010
· December 6, 2010
· December 14-16, 2010
· January 5, 2011
· January 13, 2011
· January 19-21, 2011
· Add New meeting ….. etc.
� Termination of a Network Resource is also known as “Undesignation” of a Network Resource.


� FERC Order No. 890, issued February 16, 2007 can be accessed from the following link - � HYPERLINK "http://www.naesb.org/doc_view4.asp?doc=ferc021607.doc" ��http://www.naesb.org/doc_view4.asp?doc=ferc021607.doc� (insert a hyperlink)


� NAESB staff has spoken to the FERC staff liaison regarding the reference to NERC in paragraph 209. The FERC staff liaison indicated that the reference to NERC was an error.





�Need to deceide where this appendix will reside.


�JTW to change all references to the 2011 annual plan   DONE 011411


�B. Rehman comment: are we going beyond this for masking?






Page 30 of 43
Revised 01-20-2011


[image: image1.wmf] 

[image: image2.wmf] 

_1075620371.doc
[image: image1.png]|

Lo







