Additions to WEQ-000-1 (Acronym)

SAMTS – Service Across Multiple Transmission Systems
Additions to WEQ-000-2 (Glossary)

Coordinated Group – Two or more requests that are linked together for the purpose of procuring service across multiple transmission systems.
Coordinated Request – A request that is part of a Coordinated Group.
Revisions to WEQ-000-2 (Glossary)

Additions to WEQ-001 
001-xx
COORDINATION OF REQUESTS FOR SERVICE ACROSS MULTIPLE TRANSMISSION SYSTEMS (SAMTS)
The following requirements provide consistency in processing SAMTS requests.
001-xx.1
General Requirements

001-xx.1.1
OASIS shall support the queuing and processing of all Coordinated Groups, as specified in the technical requirements of Business Practice Standards WEQ-002, WEQ-003 and WEQ-013. 

001-xx.1.2
In the event that all Transmission Providers cannot accommodate the full amount requested, a Transmission Customer or Network Customer will not be required to reserve the full requested capacity on the remaining Coordinated Request(s).
001-xx.1.3
Queue submission should be subject to the same requirements as if the Coordinated Request were not part of a Coordinated Group (timing tables do not need to be changed)

001-xx.1.4
In addition to the routine validation done for each Coordinated Request, each Transmission Provider will be limited to performing validation that the product requested on the Transmission Provider’s system is an allowable “linked” product.  
001-xx.1.4.1
There should be no requirement for a Transmission Provider to validate consistency of product names and/or attributes across Coordinated Requests which are part of a Coordinated Group.

001-xx.1.4.2
The Coordinated Group(linked group) shall be made up of contiguous requests and/or reservations such that there should be no gaps between the earliest start time of any Coordinated Request in the Coordinated Group and latest stop time of any Coordinated Request in the Coordinated Group.

001-xx.1.5
The true-up process shall not require a Transmission Customer to ultimately reserve Transmission Service in excess of the lowest capacity granted by any Transmission Provider.
 
001-xx.1.6
The coordination procedure will extend only for evaluation and processing of Coordinated Requests in a Coordinated Group.  After coordination is completed (final true-up), each of the resulting reservations shall be treated as any other reservation. Once all Coordinated Requests are in a final state, the coordination process is complete and the Coordinated Group is no longer linked for any further processing.
001-xx.1.7
The Business Practice Standard shall permit regional alliances to establish a mechanism wherein a single request is processed for Transmission Service on multiple Transmission Providers.  Such a mechanism must allow Coordinated Requests that extend beyond  the boundaries of the alliance.

001-xx.1.8
This new NAESB Business Practice Standard should not require offering of additional products in order to establish an identical hour-by-hour match of reservations on OASIS systems that serve different time zones.  
001-xx.1.9
This new NAESB Business Practice Standard should not require the Transmission Customer to procure additional products in order to establish an identical hour-by-hour match of reservations on OASIS systems that serve different time zones.
001-xx.2
SAMTS Submission of Requests for Transmission Service

001-xx.2.1
Eligibility

001-xx.2.1.1
The Coordinated Group(linked group) shall be made up of contiguous requests and/or reservations such that there should be no gaps between the earliest start time of any Coordinated Request in the Coordinated Group and latest stop time of any Coordinated Request in the Coordinated Group.

001-xx.2.1.2
Motion 1, when referring to contiguous, shall permit time zone differences to exist. For example, a start time of midnight EST shall be deemed contiguous with a start time of midnight CST. This is so that we will consistent with the EC approved scope, Additional Guidance 2.g.i.

001-xx.2.1.3
The customer attest that sufficient firm reservations are under the TC control to meet the contiguity requirements.
001-xx.2.1.4
Unless otherwise prohibited, Transmission Provider(s) will be required to allow any combination of the following Transmission Services in a Coordinated Group:

· Yearly Firm PTP
· Monthly Firm PTP
· Monthly Non-Firm PTP

· Firm Network Service with a minimum duration of one month

· Monthly Secondary Network Service
001-xx.2.1.5
Pre-confirmation is required for all Coordinated Requests

001-xx.2.1.5.1
These pre-confirmed requests shall be treated as any other pre-confirmed Transmission Service request with respect to queue position and preemption (Table 4-3)

001-xx.2.1.5.2
Final true-up will permit changes in capacity on pre-confirmed Monthly  PTP requests
001-xx.2.2
General
001-xx.2.2.1
A Transmission Customer will be permitted to submit a set of requests on multiple transmission systems and group them together
001-xx.2.2.2
The Transmission Customer must provide the following for each Coordinated Request:

001-xx.2.2.2.1
A unique identifier that is assigned to the Coordinated Group (similar to the way a unique identifier is established for each e-tag)

001-xx.2.2.2.2
The following information about each of the other Coordinated Requests for transmission service that are part of the Coordinated Group shall include:

· Transmission Provider

· OASIS number of the Coordinated Request for Transmission Service
· Product (Yearly Firm PTP, Monthly Firm PTP, Monthly Non-Firm PTP, Firm Network Service with a minimum duration of one month, Monthly Secondary Network Service)

001-xx.2.2.3
After the initial submission the TC shall be permitted to amend the request to permit adding the information as described in WEQ-001-xx1.2.4.2. Submission window for all such information is 24 hours from the queue time of the request.
001-xx.2.2.4
The Transmission Customer must be permitted to include any number of Coordinated Requests in a Coordinated Group:
001-xx.2.2.4.1
Permit single or multiple Coordinated Requests on one Transmission system to be linked with single or multiple Coordinated Requests on other Transmission systems

001-xx.2.2.4.2
Permit one or more Coordinated Requests to be a concomitant request 

001-xx.2.2.4.3
Permit Coordinated Requests of different product types

001-xx.2.2.4.4
Permit Coordinated Request with different durations (start/stop times)

001-xx.2.2.4.5.Permit Coordinated Requests which are not on adjoining transmission systems

001-xx.2.2.5
Each Transmission Provider will evaluate and respond to the Coordinated Request on its OASIS as if it were not part of a Coordinated Group with the following exceptions:

001-xx.2.2.5.1
A new state shall be defined for use in acceptance of a Coordinated Request
001-xx.2.2.5.2
A new state shall be defined for use in counteroffering of a Coordinated Request
001-xx.2.2.5.3
The Transmission Customer’s response time requirement shall be extended and shall be defined as outlined in the true-up phase. 
001-xx.2.2.6
Transmission Providers may not deny a request for any of the following reasons:

· A Coordinated Request should not be denied because it has a different time zone than another Coordinated Request in the Coordinated Group.  

· A Coordinated Request should not be denied because it has a different Coordinated Requested MW capacity than another request in the Coordinated Group.  

· A Coordinated Request should not be denied because it has a different product type than another Coordinated Request in the Coordinated Group.  

· A Coordinated Request should not be denied because it has different start/stop time(s) than another Coordinated Request in the Coordinated Group 

· A Coordinated Request should not be denied because Coordinated Requests in the Coordinated Group are not contiguous.  (Coordinated Groups are not required to be comprised of Coordinated Requests on adjoining systems.)
001-xx.3
SAMTS Initial Processing of Transmission Service Requests (Pre-true-up)

001-xx.3.1
Each Transmission Provider that receives a Coordinated Request that is part of a Coordinated Group shall process that Coordinated Request in the order queued on the Transmission Provider’s system as it would for any other request for Transmission Service.

001-xx.3.2
For each PTP request that is part of a Coordinated Group, the Transmission Provider shall meet the timing requirements presently outlined in Table 4-2. 

001-xx.3.3
For each NITS request that is part of a Coordinated Group, the Transmission Provider shall meet the timing requirements presently outlined in Table 105.
001-xx.3.4
The customer response time is a function of the response time (PTP table 4.2, NITS table 105) of the longest Coordinated Request (monthly/yearly) in the Coordinated Group (all Coordinated Requests will have the same response time of the longest Coordinated Request)
001-xx.3.5
Upon completion of this initial processing, each Coordinated Request will proceed to  either a CR_ACCEPTED, CR_COUNTEROFFER, SUPERSEDED or REFUSED state..
001-xx.4
SAMTS Final Processing of Transmission Service Requests (True-up)

001-xx.4.1
When each of the Coordinated Requests of the Coordinated Group is in a  CR_ACCEPTED, CR_COUNTEROFFER, SUPERSEDED or REFUSED state, the Transmission Customer shall determine the final requested capacity of each Coordinated Request, as follows:

001-xx.4.1.1
Nothing shall prevent a customer from confirming an accepted or counteroffered Coordinated Request prior to knowing the final outcome of all Coordinated Requests in a Coordinated Group.  Once confirmed and finalized, no adjustment in capacity shall be permitted.

001-xx.4.1.2
If one or more Coordinated Request was placed in a CR_COUNTEROFFER, SUPERSEDED or REFUSED state  the Transmission Customer is permitted, but not required, to lower the amount of requested capacity on any and all of the Coordinated Requests in the Coordinated Group through a rebid or a withdraw.
001-xx.4.1.2.1
Rebids will apply only to capacity

001-xx.4.1.2.2
Prior to rebidding or withdrawing a Coordinated Request, the TC must indicate on the Coordinated Request the capacity requested and the capacity granted for each Coordinated Request that was not accommodated in full.

001-xx.4.1.2.3
The Transmission Customer must communicate its true-up decisions to each Transmission Provider after all Coordinated Requests are in a pre-true-up state. The OASIS subcommittee will determine the proper timing and communication requirements.  
The TC must respond to each Coordinated Request within the assigned response window per weq-xxxxxx.
001-xx.4.1.2.4
Upon deciding the final capacity for each of the Coordinated Requests in the Coordinated Group, the Transmission Customer is required to respond (confirm, rebid, or withdraw) to each of the outstanding Coordinated Requests reflecting  the desired amount of true-up capacity for each.
001-xx.4.2
If all Coordinated Requests were accepted with the requested capacity The Transmission Customer must confirm all Coordinated Requests 
001-xx.4.3
Each Transmission Provider shall accept any rebid capacity for such Coordinated Request(s).  At this point all Coordinated Requests are in a final state
Additional requirements:

b) 02/09/2011 Subcommittee discussed and decided that the deadline for withdrawals of all other requests is 24 hours from the time that the state change occurs for the ANNULLED, INVALID or DECLINED state if the replacement cannot be made in the information window. 

c) 02/09/2011 Subcommittee discussed and decided that the customer who withdraws must indicate which request was annulled, invalid or declined.

d) 02/09/2011 Subcommittee discussed and decided that theTP is not required to monitor these states but if observed that the customer has not responded within the deadline then the TP may set the state to RETRACTED. 
a)02/09/2011 Subcommittee discussed and decided that TP negotiation of price, via a CR_COUNTEROFFER, will not trigger negotiation of capacity on other requests as part of the Coordinated Request.

Additions to WEQ-002

Additions to WEQ-003

Additions to WEQ-013

013-2.2 
TRANSACTION STATUS

CR_COUNTEROFFER = applies to a Coordinated Request in a Coordinated Group when assigned by the Transmission Provider or Reseller to indicate that a new value for OFFER_PRICE and/or CAPACITY_GRANTED over time is being proposed in the negotiation of requested Transmission Service, (i.e., offering of Partial Service or negotiation of price).
CR_ACCEPTED = applies to a Coordinated Request in a Coordinated Group when assigned by the Transmission Provider or Reseller to indicate the Transmission Service request at the designated BID_PRICE and CAPACITY_REQUESTED has been approved/accepted. Depending upon the type of ancillary services required, the Seller may or may not require all ancillary service reservations to be completed before accepting a Transmission Service request.
�This will be the place to add the TC response deadline requirements.





Revised 02-09-2011


