	WEQ-003-0
Definition of Data Element, Version 1.5

	OASIS Data Dictionary
Element Name
	Alias
	Field Format :
minimum characters  
{type of ASCII} 
maximum characters
	Restricted Values
	Definition of Data Element

	ACCUMULATED_HOURS
	ACCUMHRS
	0{NUMERIC}12
	Integer number
	The number of hours a CCO Reservation has actually been curtailed when in a Conditional Curtailment Priority Level under the Number-of-Hours Criteria within a specified time interval.

	ADDITIONAL_INFO
	ADDCOM
	0{ALPHANUMERIC}255
	Free form text
	Informative text. Additional information to be communicated by the Transmission Customer and Seller.

	AFFILIATE_FLAG
	AFFLAG
	2{ALPHANUMERIC}3
	Valid Values:

YES

NO
	Set to YES if Transmission Customer is an Affiliate of the Transmission Provider.

	AGENT_AUTHORIZATION
	AGENTAUTH
	2{ALPHA}3
	Valid Values:

YES

NO
	Set to YES if the Eligible Transmission Customer or Transmission Customer authorizes the agent to assume responsibility for the designated functions.

	AGENT_CODE
	AGENTCODE
	1{ALPHANUMERIC}6
	Unique value, registered on www.tsin.com
	Any entity (or its designated agent) that is eligible to view OASIS information, to execute a service agreement, and/or to receive Transmission Service.

	AGENT_DUNS
	AGENTDUNS
	9{NUMERIC}9
	Unique DUNS number
	Unique DUNS number for a Transmission Customer or Seller that assumes the agent role

	AGENT_EMAIL
	AGENTEMAIL
	1{ALPHANUMERIC}25
	Valid Internet E-mail address
	Internet E-mail address of Transmission Customer or Seller contact person that assumes the agent role.

	AGENT_FAX
	AGENTFAX
	14{ALPHANUMERIC}20
	Area code and telephone number, plus any extensions

(aaa)-nnn-nnnn xnnnn
	FAX phone number of Transmission Customer or Seller contact person that assumes the agent role.

	AGENT_NAME
	AGENTNAME
	1{ALPHANUMERIC}25
	Free form text
	Name of Transmission Customer or Seller contact person that assumes the agent role.

	AGENT_PHONE
	AGENTPHONE
	14{ALPHANUMERIC}20
	Area code and telephone number, plus any extensions

(aaa)-nnn-nnnn xnnnn
	Telephone of Transmission Customer or Seller contact person that assumes the agent role

	ANC_SERVICE_POINT
	ANCPOINT
	0{ALPHANUMERIC}12
	Free form text
, null can be used if there is no ancillary service point other than the BA
	Name of ancillary service point within a BA, such as a POR/POD/SOURCE/SINK from which the ancillary service is provided.

	ANNOTATION
	ANNOTATE
	0{ALPHANUMERIC}1000
	Free-form text
	Additional notes, comments or other descriptive information with the posted event.

	AS_TYPE
	ASTYPE
	1{ALPHANUMERIC}20
	Valid Values:
SC
RV
RF

EI

SP

SU

GI

DT

TL

BS

{Registered}
	SC –
Scheduling, system Control and Dispatch
RV –
Reactive Supply and Voltage Control
RF –
Regulation and Frequency Response
EI –
Energy Imbalance

SP –
Spinning Reserve

SU –
Supplemental Reserve

GI –
Generator Imbalance

DT –
Dynamic Transfer

TL –
Real power Transmission Loss

BS –
System Black Start Capability

{Registered} – must be registered with www.tsin.com and listed in the ancserv Template

	ANC_SVC_LINK
	ANCSVCLINK
	0{ALPHANUMERIC}300
	Formatted string as follows:

SC:(AA[:xxx[:yyy[:nnn] ]]); RV:(AA[:xxx[:yyy[:nnn ]]]); RF:(AA[:xxx[:yyy[:nnn] ]]); EI:(AA[:xxx[:yyy[:nnn]] ]); 

SP:(AA[:xxx[:yyy[:nnn] ]]); 
SU:(AA[:xxx[:yyy[:nnn] ]]);
GI:(AA[:xxx[:yyy[:nnn] ]]);
DT:(AA[:xxx[:yyy[:nnn] ]]);
TL:(AA[:xxx[:yyy[:nnn] ]]);
BS:(AA[:xxx[:yyy[:nnn] ]]);

{Registered}:(AA[:xxx[ :yyy[:nnn]]])
	The method for linking ancillary service types to a Transmission Service request. The Transmission Provider and capacity of each ancillary service is identified using the formatted string:

SC:(AA[:xxx[:yyy[:nnn]]]);
RV:(AA[:xxx[:yyy[:nnn]]]);
RF:(AA[:xxx[:yyy[:nnn]]]);
EI:(AA[:xxx[:yyy[:nnn]]]); 

SP:(AA[:xxx[:yyy[:nnn]]]);
SU:(AA[:xxx[:yyy[:nnn]]]);
GI:(AA[:xxx[:yyy[:nnn]]]);
DT:(AA[:xxx[:yyy[:nnn]]]);
TL:(AA[:xxx[:yyy[:nnn]]]);
BS:(AA[:xxx[:yyy[:nnn]]]);

{Registered}:(AA[:xxx[:yyy[:nnn]]])

where AA is the appropriate PRIMARY_PROVIDER_CODE, SELLER_CODE, or CUSTOMER_CODE, and represents the company providing the ancillary services. “AA” may be unspecified for “xxx” type identical to “FT”, in which case the “:” character must be present and precede the “FT” type.

If multiple “AA” terms are necessary, then each “AA” grouping will be enclosed within parenthesis, with the overall group subordinate to the AS_TYPE specified within parenthesis and where xxx represents either:

“FT” –
to indicate that the Transmission Customer will determine ancillary services at a future time, or
“SP” –
to indicate that the Transmission Customer will self-provide the ancillary services, or

“RQ” –
to indicate that the Transmission Customer is asking the OASIS Node to initiate the process for making an ancillary services reservation with the indicated Transmission Provider or Reseller on behalf of the Transmission Customer. The Transmission Customer must then continue the reservation process with the Transmission Provider or Reseller. If the transmission services request is for preconfirmed service, then the ancillary services shall also be preconfirmed, or

	ANC_SVC_LINK (cont.)
	
	
	
	“AR” –
to indicate an Assignment Reference Number sequence follows. The terms “yyy” and “nnn” are subordinate to the xxx type of “AR”. yyy represents the ancillary services reservation number (ASSIGNMENT_REF) and nnn represents the capacity of the reserved ancillary services. Square brackets are used to indicate optional elements and are not used in the actual linkage itself. Specifically, the :yyy is applicable to only the “AR” term and the :nnn may optionally be left off if the capacity of ancillary services is the same as for the Transmission Services, and optionally multiple ancillary reservations may be indicated by additional (xxx[:yyy[:nnn]]) enclosed within parenthesis. If no capacity amount is indicated, the required capacity is assumed to come from the ancillary reservations in the order indicated in the codes, on an "as-needed" basis.

	ANC_SVC_REQ 
	ANCSVCREQ
	0{ALPHANUMERIC}100
	SC:{M,R,O,U};

RV:{M,R,O,U}; RF:{M,R,O,U}; EI:{M,R,O,U}; SP:{M,R,O,U}; SU:{M,R,O,U}; GI:{M,R,O,U}; DT:{M,R,O,U}; TL:{M,R,O,U};

BS:{M,R,O, U};
{registered}:{M,R,O,U }  
	Ancillary services required for a Transmission Services offering. The appropriate letter {M,R,O,U} will be assigned to each of the six Proforma FERC ancillary service types (see AS_TYPE), where the letters mean the following:

·
(M) Mandatory, which implies that the Transmission Provider must provide the ancillary service 

·
(R) Required, which implies that the ancillary service is required, but not necessarily from the Transmission Provider

·
(O) Optional, which implies that the ancillary service is not necessarily required, but could be provided

·
(U) Unknown, which implies that the requirements for the ancillary service are not known at this time

	APPLICATION_STATUS
	APPSTATUS
	5{ALPHANUMERIC}25
	Valid Values:
QUEUED
INVALID
RECEIVED
STUDY
COUNTEROFFER
DECLINED
ACCEPTED
REFUSED
CONFIRMED
WITHDRAWN
ANNULLED
RETRACTED

PRESUBMITTED

DEFICIENT

COMPLETED

REEVALUATE
	QUEUED – initial status assigned by the TSIP on receipt of "Transmission Customer services purchase request" or “Transmission Customer NITS Application request”.

INVALID – assigned by the TSIP or Transmission Provider indicating an invalid field in the request, such as improper POR, POD, SOURCE, SINK, etc. or that the request does not meet minimum information or data requirements (Final state).

RECEIVED – assigned by the Transmission Provider or Reseller (PTP only) to acknowledge QUEUED requests and indicate the service request is being evaluated, including for completing the required ancillary services.

STUDY – assigned by the Transmission Provider or Reseller (PTP only) to indicate some level of study is required or being performed to evaluate service request.

REFUSED – assigned by the Transmission Provider or Reseller (PTP only) to indicate service request has been denied due to lack of ATC. (Final state). 

COUNTEROFFER – assigned by Transmission Provider or Reseller (PTP only) to indicate that a new OFFER_PRICE and/or CAPACITY_GRANTED over time is being proposed in the negotiation of requested service (i.e., offering of Partial Service or negotiation of price).



	APPLICATION_STATUS (cont.)
	
	
	
	ACCEPTED – assigned by the Transmission Provider or Reseller (PTP only) to indicate the service request at the designated BID_PRICE and/or CAPACITY_REQUESTED has been approved/accepted. Depending upon the type of ancillary services required, the Seller may or may not require all ancillary service reservations to be completed before accepting a request.

DECLINED – assigned by the Transmission Provider or Reseller (PTP only) to indicate that the terms and conditions of the request, such as the BID_PRICE, are unacceptable and that negotiations are terminated or that contractual terms have not been met or the Transmission Customer has failed to respond to a deficiency within an established time frame. (Final state).

RETRACTED - assigned by the Transmission Provider or Reseller (PTP only) when the Transmission Customer fails to confirm or withdraw the request within the required time period. (Final state).

WITHDRAWN – assigned by the Transmission Customer during a request evaluation to withdraw the request from any further action. (Final state).

CONFIRMED – assigned by the Transmission Customer in response to the Transmission Provider or Reseller (PTP only) posting "ACCEPTED" or “COUNTEROFFER” status, to confirm service or the NITS Application. Once a request has been "CONFIRMED", a Transmission Service reservation exists or the NITS Application is completed. (Final state, unless overridden by DISPLACED or ANNULLED state).

	APPLICATION_STATUS (cont.)
	
	
	
	ANNULLED – assigned by the Seller when, by mutual agreement with the Transmission Customer, a confirmed reservation, NITS Application, or pre-confirmed request is to be voided or assigned unilaterally by the Transmission Provider when a Resale reservation or NITS Application is to be voided. (Final state).

PRESUBMITTED – assigned by OASIS prior to submission by the Eligible Network Customer or Network Customer in a workspace to be saved while developing a NITS Application or request for modification of service such that the NITS Application or request for modification of service can be submitted at a later date. 

DEFICIENT – assigned by the Transmission Provider indicating a correctable deficiency in the NITS Application or request for modification of service, such as missing, incomplete, or inconsistent information. 

COMPLETED – assigned by the Transmission Provider indicating that are no deficiencies or all deficiencies in the NITS Application or request for modification of service have been met. 

REEVALUATE – assigned by the Eligible Transmission Customer or Network Customer during the NITS Application or request for modification of service process indicating that deficiencies have been corrected, and the revised NITS Application or request for modification of service is ready for review. 

	ASSIGNMENT_REF
	AREF
	1{ALPHANUMERIC}12
	Unique value
	A unique reference number assigned by a TSIP to provide a unique record for each Transmission Service, NITS Application Descriptions, or ancillary service request. A single Transmission Service, NITS Application Description, or ancillary service request will be over a contiguous time period, i.e. from a START_TIME to an STOP_TIME.

	ATTRIBUTE_UNITS
	ATTRUNITS
	1{ALPHANUMERIC}20
	Free form text
	System data attribute units.

	ATTRIBUTE_VALUE
	ATTRVALUE
	1{NUMERIC}12
	Real number
	System data attribute value.

	BID_PRICE
	BIDPR
	1{NUMERIC}5 + "."

+ 2{NUMERIC}4
	Positive number with 2 to 4 decimals
	The current bid price of a Transmission Service in dollars and cents. Used by Transmission Customers to designate a price being bid.

	CAPACITY
	CAP
	0{NUMERIC}12
	Number in units of MW
	Transfer capability is the measure of the ability of the interconnected electric system to readily move or transfer power from one area to another over all transmission lines (or Posted Paths) between those areas under specified system conditions. In this context "area" may be an individual electric system, powerpool, BA, subregion, or NERC region or portion thereof.

	CAPACITY_AVAILABLE
	CAPAVAIL
	0{NUMERIC}12
	Non-negative number in units of MW
	Amount of transmission capacity available after all the reductions are applied to CAPACITY_GRANTED over the time interval.

	CAPACITY_CURTAILED
	CAPCUR
	1{NUMERIC}12
	Non-negative number in units of MW
	The amount of transfer capability curtailed by the Transmission provider for emergency reasons.

	CAPACITY_GRANTED
	CAPGRNT
	0{NUMERIC}12
	Non-negative number in units of MW
	The amount of capacity offered/granted by the Seller equal to or less than CAPACITY_REQUESTED by the Transmission Customer.

	CAPACITY_REDUCED
	CAPREDU
	0{NUMERIC}12
	Negative number in units of MW
	Amount of transmission capacity reduced.

	CAPACITY_REQUESTED
	CAPREQ
	0{NUMERIC}12
	Non-negative number in units of MW
	Transmission capacity requested by the Transmission Customer.

	CAPACITY_USED
	CAPUSED
	0{NUMERIC}12
	Non-negative number in units of MW
	Reflects the peak MW amount of the reservation used to support the scheduled transaction.

	CATEGORY
	CAT
	0{ALPHANUMERIC}25
	Valid name from CATEGORY in list Template
	A name to be used to categorize messages. Valid names would include:  want-ad, Curtailment, Outage, OASIS_Maintenance_Outage.

	CEILING_PRICE
	CEILPR
	1{NUMERIC}5 + "." 

+ 2{NUMERIC}4 
	Positive number with 2 to 4 decimals
	Ceiling price of the Transmission Service as entered by the Transmission Provider.

	CG_CONTIGUITY
	CGCONT
	1{ALPHANUMERIC}1
	Valid values:
Y
N
	If set to Y indicates that the Transmission Customer or Network Customer has attested that the Coordinated Group meets the contiguity requirements established in Business Practice Standard WEQ-001-xx.2.1

	CG_FLAG
	CGFLAG
	1{ALPHANUMERIC}1
	Valid values:
Y
N
	If set to Y indicates this request will be a Coordinated Request asociated with a Coordinated Group.

	COLUMN_HEADERS
	HEADERS
	1{ALPHANUMERIC} Limited to all the Data Elements names in one Template
	Headers separated by commas. Limited to valid Template Data Element names. Must use full Data Element name and not alias.
	Example:

COLUMN_HEADERS=PATH_NAME,POI NT_OF_RECEIPT,POINT_OF_DELIVERY, SOURCE,SINK

	COMPETING_REQUEST_FLAG
	COMPREQ
	1{ALPHANUMERIC}1
	Valid values:
Y
N
	If "Y", indicates there is one or more competing requests for this reservation. The competing request AREFs are listed in the SELLER_COMMENTS.

	CONDITIONAL_CURTAILMENT_OPTION
	CCOPT
	1{ALPHANUMERIC}10
	Valid values

HOURS

CONDITIONS

{registered}
	In association with a CCO Reservation, indicates whether the Number-of-Hours Criteria or System-Conditions Criteria is applicable.

	CONDITIONAL_START_TIME
	CONDSTTIME
	0,16{ALPHANUMERIC}16 
	Valid date and time to seconds:

yyyy+mo+dd+hh

+mm+ss+tz
	Start of the interval designating the number of ACCUMULATED_HOURS and MAXIMUM_HOURS available.

	CONDITIONAL_STOP_TIME
	CONDSPTIME
	0,16{ALPHANUMERIC}16 
	Valid date and time to seconds:

yyyy+mo+dd+hh

+mm+ss+tz
	End of the interval designating the number of ACCUMULATED_HOURS and MAXIMUM_HOURS available.

	CONTINUATION_FLAG
	CONT
	1{ALPHANUMERIC}1
	Valid values:
Y
N
	Indicates whether or not this record is a continuation from the previous record.

	CONTROL_AREA
	AREA
	1{ALPHANUMERIC}20
	Valid name of a BA
	A part of the power system with metered tie lines and capable of matching generation and load while meeting scheduled Interchange. Location of ancillary services is my CONTROL_AREA.

	CR_ACCOMMODATED
	CRACC
	1{ALPHANUMERIC}1
	Valid values:
Y
N
	If set to N Indicates that for at least one Coordinated Request in the same Coordinated Group that the requested capacity was not available.

	CR_ASSIGNMENT_REF
	CRAREF
	1{ALPHANUMERIC}12
	Unique value
	The ASSIGNMENT_REF assigned by a TSIP for each of the other Transmission Service, NITS Application Descriptions, or ancillary service request used in Coordinated Requests in the Coordinated Group.

	CR_CONFIRMATION_LIMIT
	CRCONF
	16{ALPHANUMERIC}16
	Valid date and time to seconds:

yyyy+mo+dd+hh

+mm+ss+tz
	Date and time to seconds by when a response must be received from a Transmission Customer or NITS Customer used in Coordinated Requests in the Coordinated Group..

	CR_CAPACITY_GRANTED
	CRCAPGRNT
	0{NUMERIC}12
	Non-negative number in units of MW
	The CAPACITY_GRANTED for each of the amount of capacity offered/granted by the Seller used in Coordinated Requests in the Coordinated Group.

	CR_INTERVAL
	CRINT
	1{ALPHANUMERIC}8
	Valid Values:

MONTH

YEAR

{Registered}
	The SERVICE_INCREMENT or REQUEST_INTERVAL for each of the other NITS or PTP transmission service increments provided used in Coordinated Requests in the Coordinated Group.

	CR_PRIMARY_PROVIDER_CODE_
	CRPROVIDER
	1{ALPHANUMERIC}4
	Unique code
	The PRIMARY_PROVIDER_CODE for each of the other Transmission Provider used in Coordinated Requests in the Coordinated Group.

	CR_SUBMIT_DEADLINE
	CRSUB
	0,16{ALPHANUMERIC}16 
	Valid date and time to seconds:

yyyy+mo+dd+hh

+mm+ss+tz
	End of the interval designating the deadline for providing or modifying the information used in the Coordinated Requests in the Coordinated Group.

	CR_CAPACITY_REQUESTED
	CRCAPREQ
	0{NUMERIC}12
	Non-negative number in units of MW
	The CAPACITY REQUESTED for each of the Transmission capacity requested by the Transmission Customer used in the Coordinated Requests in the Coordinated Group.

	CR_TS_CLASS
	CRTSCLASS
	1{ALPHANUMERIC}20
	Valid Values:

FIRM

NON-FIRM

SECONDARY

{Registered}
	The TS_CLASS for each of the other Transmission Service classes provided used in Coordinated Requests in the Coordinated Group.

	CURTAILMENT_OPTIONS
	CUROPT
	0{ALPHANUMERIC}80
	Free form text
	Transmission Customer options, if any, to avoid Curtailment

	CUSTOMER_CODE
	CUST
	1{ALPHANUMERIC}6 
	Unique value, registered on www.tsin.com
	Any entity (or its designated agent) that is eligible to view OASIS information, to execute a service agreement, and/or to receive Transmission Service.

	CUSTOMER_COMMENTS
	CUSTCOM
	0{ALPHANUMERIC}255
	Free form text
	Informative text. For information to be communicated between the Transmission Customer and Seller.

	CUSTOMER_CREDIT_WORTH
	CUSTCRED
	0{ALPHA}1
	Valid values:
Y
N
	If set to Y indicates that indicates whether or not the Customer is credit worthy

	CUSTOMER_DEPOSIT
	CUSTDEP
	0{ALPHA}1
	Valid values:
Y
N
	If set to Y indicates that indicates whether or not the Customer has put a deposit down

	CUSTOMER_DUNS
	CUSTDUNS
	9{NUMERIC}9
	Unique DUNS number
	Unique DUNS number for a Transmission Customer or Eligible Transmission Customer

	CUSTOMER_EMAIL
	CUSTEMAIL
	1{ALPHANUMERIC}25
	Valid Internet E-mail address
	Internet E-mail address of Transmission Customer contact person.

	CUSTOMER_FAX
	CUSTFAX
	14{ALPHANUMERIC}20
	Area code and telephone number, plus any extensions

(aaa)-nnn-nnnn xnnnn
	FAX phone number of Transmission Customer contact person.

	CUSTOMER_NAME
	CUSTNAME
	1{ALPHANUMERIC}25
	Free form text
	Name of Transmission Customer contact person.

	CUSTOMER_PHONE
	CUSTPHON
	14{ALPHANUMERIC}20
	Area code and telephone number, plus any extensions

(aaa)-nnn-nnnn xnnnn
	Telephone of Transmission Customer contact person.

	CUSTOMER_STATEMENT
	CUSTSTATE
	0{ALPHA}1
	Valid values:
Y
N
	If set to Y, the requestor is attesting to be eligible for taking network service. 

	CUSTOMER_SYSTEM_CONTRACT_RESTRICTIONS
	CUSTSYSCONRESTR
	0{ALPHA}1
	Valid values:
Y
N
	If set to Y, the contractual restrictions or committed uses of the Eligible Customer’s transmission system, other than the Eligible Customer’s Network Loads and Resources, is on file with the Transmission Provider.

	CUSTOMER_SYSTEM_LOAD_FLOW
	CUSTSYSLDFL
	0{ALPHA}1
	Valid values:
Y
N
	If set to Y, the load flow and stability data, such as real and reactive parts of the load, lines, transformers, reactive devices and load type, including normal and emergency ratings of all transmission equipment in a load flow format compatible with that used by the Transmission Provider is on file with the Transmission Provider.

	CUSTOMER_SYSTEM_MAPS
	CUSTSYSMAP
	0{ALPHA}1
	Valid values:
Y
N
	If set to Y, the transmission system maps that include any proposed expansions or upgrades is on file with the Transmission Provider.

	CUSTOMER_SYSTEM_NAME
	CUSTSYSNAME
	1{ALPHANUMERIC}55
	Free form text
	Name of Transmission Customer system name.

	CUSTOMER_SYSTEM_OP_GUIDES
	CUSTSYSOPGUIDE
	0{ALPHA}1
	Valid values:
Y
N
	If set to Y, the operating guides employed by system operators is on file with the Transmission Provider.

	CUSTOMER_SYSTEM_OP_RESTRICTIONS
	CUSTSYSOPRESTR
	0{ALPHA}1
	Valid values:
Y
N
	If set to Y, the operating restrictions needed for reliability is on file with the Transmission Provider.

	CUSTOMER_SYSTEM_PROJECTION
	CUSTSYSPROJ
	0{ALPHA}1
	Valid values:
Y
N
	If set to Y, the ten (10) year projection of system expansions or upgrades is on file with the Transmission Provider.

	CUSTOMER_SYSTEM_REF
	CUSTSYSREF
	1{ALPHANUMERIC}12
	Unique value
	A unique reference number assigned by a TSIP to provide a unique record for identifying a single customer system information submittal. 

	CUSTOMER_SYSTEM_TIE_THERMAL_RATING
	CUSTSYSTIERATE
	0{ALPHA}1
	Valid values:
Y
N
	If set to Y, the thermal ratings of Eligible Customer’s Control Area ties with other Control Areas is on file with the Transmission Provider.

	DATA_ROWS
	ROWS
	1{NUMERIC}unlimited
	Positive number 
	Number of records (rows) of data exclusive of header information that are to be uploaded or downloaded in a file.

	DATE_TIME_EFFECTIVE
	TIMEEFCT
	16{ALPHANUMERIC}16 
	Valid date and time in seconds

yyyy+mo+dd+hh

+mm+ss+tz
	Date and time a message or service offer is in effect.

	DEAL_REF 
	DREF
	0{ALPHANUMERIC}12 
	Unique value, assigned by Transmission Customer
	The unique reference assigned by a Transmission Customer to two or more service purchases to identify each of them as related to others in the same power service deal. These requests may be related to each other in time sequence through a single Transmission Provider, or as a series of wheels through multiple Transmission Providers, or a combination of both time and wheels. The user uses the DEAL_REF to uniquely identify a combination of requests relating to a particular deal. 

	DISCRETION_DESCRIPTION
	DISCDESC
	0{ALPHANUMERIC}1000
	Free form text
	A detailed description of the waiver being.  The discretion template and the related DISCRETION_DESCRIPTION data element are an optional implementation at the Transmission Provider’s discretion.

	DISPATCH_MERIT_ORDER
	
	1{INTEGER}10
	Integer
	Priority level of the Transmission Customer generation

	DNR_ATTESTATION
	DESRESATT
	0{ALPHA}1
	Valid Values:
Y
N
	For new NITS Applications, if set to Y, the valid value would indicate that the signed statement by an authorized officer or agent of an Eligible/Network Customer is on file with the Transmission Provider.

For modification of service, if set to Y, the valid value would indicate that the Eligible/Network Customer attests to ownership of the new Network Resource.

	DNR_CAPACITY_GRANTED
	DESCAPGRNT
	0{NUMERIC}12
	Non-negative number in units of MW
	Amount of DNR capacity granted by the Seller equal to or less than CAPACITY_REQUESTED by the Eligible/Network Customer.

	DNR_CAPACITY_REQUESTED
	DESCAPREQ
	0{NUMERIC}12
	Non-negative number in units of MW
	Amount of DNR capacity requested by the Eligible/Network Customer.

	DNR_EXTERNAL_TP_AREF
	DNREXTTPAFRE
	1{ALPHANUMERIC}12
	Unique value
	A unique reference number assigned by a TSIP to provide a unique record for identifying the external transmission assignment ref. 

	DNR_EXTERNAL_TP_CODE
	DNREXTTPCD
	1{ALPHANUMERIC}4
	Unique code
	Unique code for each Transmission Provider. Used by PATH_NAME and in URL. Registered as part of URL at www.tsin.com.

	DNR_NAME
	DNRNAME
	0{ALPHANUMERIC}14
	Unique value, registered on www.tsin.com
	Any resource that is defined on OASIS by the Transmission Customer as a valid network resource.

	DNR_PROJECTION_CAPACITY
	DNRPROJCAP
	0{ALPHA}1
	Valid values:
Y
N
	If set to Y, the ten (10) year projection of DNR expansions or upgrades is on file with the Transmission Provider.

	DNR_REF
	DNRREF
	1{ALPHANUMERIC}12
	Unique value
	A unique reference number assigned by a TSIP to provide a unique record for identifying the external DNR assignment ref. 

	DNR_TYPE
	DNRTYPE
	1{ALPHA}30
	Valid Values:

ONSYSTEM

OFFSYTEM

PPA

CUSTOMEROWN

SLICEOFSYS

RESOURCE
	ONSYSTEM – 

OFFSYSTEM – 

PPA – 

CUSTEROWN – 

SLICEOFSYS – 

RESOURCE –



	ELEMENT_NAME
	ELEMENT
	1{ALPHANUMERIC}40
	Valid Template element name
	Template element name as indicated in OASIS Data Dictionary.

	EMPLOYEE_NAME
	EMPNAME
	1{ALPHANUMERIC}25
	Free form text
	Name of person who is transferring from one position to another.  The personnel template and the related EMPLOYEE_NAME data element are an optional implementation at the Transmission Provider’s discretion.

	ERROR_MESSAGE
	ERROR
	1{ALPHANUMERIC}250
	Free form text
	Error message related to a RECORD_STATUS or REQUEST_STATUS.

	EVENT_ID
	EVENTID
	0{ALPHANUMERIC}25
	Free form text
	The EVENT_ID Data Element is any Regional or Interconnection-wide recognized security event identifier for events that are of greater scope than those administered locally by the Transmission Provider (e.g., a NERC Reliability Coordinator assigned identifier corresponding to a particular implementation of the NERC TLR procedure).   

	FACILITY_CLASS
	FACLASS
	0{ALPHANUMERIC}25
	Free form text, for example:

TRANSFORMER,

LINE,

FLOWGATE

or as defined in the list Template
	Type of limiting device such as ‘transformer’, ‘line’  or ‘Flowgate’

	FACILITY_LIMIT_TYPE
	FACLIMTYP
	0{ALPHANUMERIC}25
	Free form text, for  example: thermal, stability, voltage or as defined in list Template
	For example: thermal, stability, voltage

	FACILITY_LOCATION
	FACLOC
	0{ALPHANUMERIC}8
	Free form text, for example:

INTERNAL 

EXTERNAL

Or as defined in the LIST Template
	Location of facility that caused the interruption, either internal to the TP or external to the TP grid.

	FACILITY_NAME
	FACNAME
	0{ALPHANUMERIC}100
	Free form text
	Name of facility, such as name of Posted Path or name of Flowgate.

	FORMER_COMPANY
	FORMCO
	1{ALPHANUMERIC}25
	Free form text
	Former company of the person who is transferring.  The personnel template and the related FORMER_COMPANY data element are an optional implementation at the Transmission Provider’s discretion.

	FORMER_DEPARTMENT
	FORMDEPT
	1{ALPHANUMERIC}52
	Free form text
	Former department of the person who is transferring.  The personnel template and the related FORMER_DEPARTMENT data element are an optional implementation at the Transmission Provider’s discretion.

	FORMER_POSITION
	FORMPOS
	1{ALPHANUMERIC}25
	Free form text
	Former position held by the person who is transferring.  The personnel template and the related FORMER_POSITION data element are an optional implementation at the Transmission Provider’s discretion.

	GCA_CODE
	GCA
	1{ALPHANUMERIC}4
	Registered BA company code
	GCA Code. Information from Tag.

	GEN_ATTR_LAGVAR
	GENLAGVAR
	1{ALPHANUMERIC}12
	LAGVAR
	LAGVAR – Lagging  VAR capability for the generating unit



	GEN_ATTR_LEDVAR
	GENLEDVAR
	1{ALPHANUMERIC}12
	LEDVAR
	LEDVAR – Leading VAR capability for the generating unit



	GEN_ATTR_MWCAPACITY
	GENMWCAP
	0{NUMERIC}12
	Non-negative number
	MWCAPACITY – Name plate MW capacity of the generator



	GEN_ATTR_MWDESIGNATED
	GENMWDES
	1{ALPHANUMERIC}12
	MWDESIGNATED
	MWDESIGNATED – MW designated for the resource usage

	GEN_ATTRIBUTE_ATTACHMENT
	GENATTATT
	1{ALPHANUMERIC}255
	Free form text
	Informative text. For attachment information that is required by the Transmission Provider to be on file for generation information.

	GEN_ATTRIBUTE_DESC
	GENATTDESC
	1{ALPHANUMERIC}255
	Free form text
	Informative text. For generation attribute information that is required by the Transmission Provider to be communicated between the Transmission Customer and Seller.

	GEN_DELIVERY_POINT
	GENDELPT
	1{ALPHANUMERIC}12, Only non-numeric and non-alpha character allowed is ".".
	Unique value within Transmission Provider. Only special character allowed is ".", for example, ab.cde.123
	

	GEN_ELECTRICAL_LOCATION
	GENELELOC
	1{ALPHANUMERIC}255
	Free form text
	For Network Customer-owned or –leased generation, the electrical location will be identified by the BAA where the generation is electrically located, for purchased power, the electrical location is the point of receipt (on the Transmission Provider’s system referenced in the Customer’s Application or request for modification of service)

	GEN_GEOGRAPHICAL_LOCATION
	GENGEOLOC
	1{ALPHANUMERIC}255
	Free form text
	For Network Customer-owned or –leased generation, the geographical location will be identified by the county or parish and state, for purchased power, the geographical location is “not applicable”.

	GEN_INFO_USE
	GENINFOUSE
	1{ALPHANUMERIC}255
	Free form text
	Informative text. For generation information that is required by the Transmission Provider to be communicated between the Transmission Customer and Seller.

	GEN_JOINT_OWNERSHIP
	GENJOINTOWN
	2{ALPHA}3
	Valid Values:
YES
NO
	Unique code to identify ownership of generation that is held jointly with another entity

	GEN_NAME
	GENNAME
	1{ALPHANUMERIC}14
	Valid  generation  name
	Name of the generation facility as registered on www.tsin.com

	GEN_OPERATOR
	GENOPER
	1{ALPHANUMERIC}4
	Unique code
	Unique code for each Generator Operator. Registered  at www.tsin.com.

	GEN_REF
	GENREF
	1{ALPHANUMERIC}12
	Unique reference
	Contains the ASSIGNMENT_REF of any generation information associated with the DNR.

	GEN_RESTR_MAINTSCHED
	GENRESMAINT
	1{ALPHANUMERIC}12
	MAINTSCHED
	MAINTSCHED – Maintenance schedule throughout the year

	GEN_RESTR_MIN
	GENRESMIN
	1{ALPHANUMERIC}12
	MIN
	MIN – Minimum loading level of unit

	GEN_RESTR_MUSTRUN
	GENRESMUST
	1{ALPHANUMERIC}12
	MUSTRUN
	MUSTRUN – Any must-run unit designations required for system

reliability or contract reasons

	GEN_RESTR_NORM
	GENRESNORM
	1{ALPHANUMERIC}12
	NORM
	NORM – Normal operating level of unit


	GEN_RESTR_RESTROPER
	GENRESRES
	1{ALPHANUMERIC}12
	RESTROPER
	RESTROPER – Any periods of restricted operations throughout the year

	GEN_RESTRICTION_ATTACHMENT
	GENRESTRATT
	1{ALPHANUMERIC}255
	Free form text
	Informative text. For generation restriction information that is required by the Transmission Provider to be on file for generation information.

	GEN_RESRICTION_DESC
	GENRESTRDESC
	1{ALPHANUMERIC}255
	Free form text
	Informative text. For generation restriction information that is required by the Transmission Provider to be communicated between the Transmission Customer and Seller.

	GEN_SOURCE_CONTROL_AREA
	GENSOURCECA
	1{ALPHANUMERIC}20
	Valid name of a BA
	

	GEN_THIRDPARTY_SALES
	GENTHRDPSALES
	1{ALPHANUMERIC}255
	Free form text
	Informative text. For third party sales information that is required by the Transmission Provider to be communicated between the Transmission Customer and Seller.

	IMPACTED
	IMPACTED
	0{NUMERIC}4
	Number
	Indicates whether the reservation has been impacted by another reservation.  For an original reservation this counter is 0. This counter is incremented by 1 by TSIP on the Parent Request when its ASSIGNMENT_REF is entered in any other reservation’s REASSIGNED_REF or RELATED_REF or in entered in any reduction. 

	IMPACTING_REF
	IMPACTRE F
	0{ALPHANUMERIC}12
	Unique reference
	References the ASSIGNMENT_REF of the associated Transmission Service reservation (if applicable) that caused the reduction in capacity.

	INITIATING_PARTY
	INITPARTY
	0{ALPHANUMERIC}4
	Registered company code for a Transmission Provider (TP), Security Coordinator (SC) or Control Area (CA)
	Company code for company responsible for initiating execution of a transmission security procedure.

	INTERFACE_TYPE
	INTERFACE
	0{ALPHANUMERIC}1
	Valid Values:
I
E
	Type of interface define by Posted Path: Internal (I) to a BA or External (E) to a BA.

	LCA_CODE
	LCACODE
	0{ALPHANUMERIC}4
	Valid registered BA code
	Load Control Area registered code. Information comes from Tag.

	LIST_ITEM
	ITEM
	1{ALPHANUMERIC}50
	Free form text
	Item from LIST, such as list of SELLER, list of PATH_NAME, list of POINT_OF_RECEIPT, list of POINT_OF_DELIVERY, list of SERVICE_INCREMENT,  list of TS_CLASS, list of TS_TYPE, list of TS_PERIOD, list of TS_WINDOW, list of TS_SUBCLASS, list of AS_TYPE, list of REQUEST_TYPE,  list of ANC_SERVICE_POINT,  list of FACILITY_CLASS, list of FACILITY_LIMIT_TYPE, list of PROCEDURE_NAME, list of SYSTEM_ATTIBUTE,  list of SECURITY_TYPE, list of FACILITY_LOCATION, list of  NERC_CURTAILMENT_PRIORITY, list of OTHER_CURTAILMENT_PRIORITY, list of CATEGORY, list of TEMPLATE, list of LIST

	LIST_ITEM_DESCRIPTION
	ITEMDESC
	0{ALPHANUMERIC}100
	Free form text
	A detailed description of the LIST_ITEM.

	LIST_NAME
	LIST
	1{ALPHANUMERIC}50
	Valid Values:
LIST
SELLER
PATH
POR
POD
SERVICE_INCREMENT
TS_CLASS 

TS_TYPE

TS_PERIOD

TS_SUBCLASS

AS_TYPE

NERC_CURTAILMENT_PRIORITY

REQUEST_TYPE 

ANC_SERVICE_POINT 

FACILITY_CLASS 

FACILITY_LIMIT_TYPE 

PROCEDURE_NAME 

SYSTEM_ATTRIBUTE 

SECURITY_TYPE 

FACILITY_LOCATION

OTHER_CURTAILMENT_PRIORITY

CATEGORY

TEMPLATE
	List of valid names for each of the types of lists. The minimum set of lists defined must be implemented.

	MAXIMUM_HOURS
	MAXHRS
	0{NUMERIC}12
	Integer number
	The maximum number of hours a  CCO Reservation may be curtailed under the Number-of-Hours Criteria within a specified time interval.

	MESSAGE
	MSG
	1{ALPHANUMERIC}200
	Free form text
	An informative text message.

	MODIFYING_COMPANY_CODE
	MODCODE
	1{ALPHANUMERIC}6
	Registered company code for a TP, SC or CA
	Contains the registered company code that modified the transaction, used in the audit Templates.

	MODIFYING_NAME
	MODNAME
	0{ALPHANUMERIC}25
	Free form text
	Contain the name of the person that modified the transaction, used in the audit Templates.

	MODIFICATION_REF
	MODREF
	1{ALPHANUMERIC}12
	Valid ASSIGNMENT_REF
	Forward pointer. Pointing to next reservation that replaces the current reservation.

	NEGOTIATED_PRICE_FLAG
	NGPRIFLG
	0{ALPHANUMERIC}1
	Valid Values:
H
L
null
	Set to H if OFFER_PRICE is higher than the currently posted price;
Set to L if OFFER_PRICE is lower than the currently posted price

	NERC_CURTAILMENT_PRIORITY
	NERCURT
	1{INTEGER}1
	Integer
	One of the NERC Curtailment priorities, documented in list Template.

	NEW_COMPANY
	NEWCO
	1{ALPHANUMERIC}25
	Free form text
	New company of the person who is transferring.  The personnel template and the related NEW_COMPANY data element are an optional implementation at the Transmission Provider’s discretion.

	NEW_DEPARTMENT
	NEWDEPT
	1{ALPHANUMERIC}52
	Free form text
	New department of the person who is transferring.  The personnel template and the related NEW_DEPARTMENT data element are an optional implementation at the Transmission Provider’s discretion.

	NEW_POSITION
	NEWPOS
	1{ALPHANUMERIC}25
	Free form text
	New position held by the person who is transferring.  The personnel template and the related NEW_POSITION data element are an optional implementation at the Transmission Provider’s discretion.

	NITS_LOAD_REF
	NITSLDREF
	1{ALPHANUMERIC}12
	Unique reference
	Contains the ASSIGNMENT_REF of any original load that is affected by this request.

	OFFER_PRICE
	OFFPR
	1{NUMERIC}5  + "." 

+ 2{NUMERIC}4 
	Positive number with 2 to 4 decimals
	The current offered price of a Transmission Service in dollars and cents. Used by the Seller to indicate the offering price.

	OFFER_START_TIME
	OFFSTIME
	0,16{ALPHANUMERIC}16 
	Valid date and time to seconds:

yyyy+mo+dd+hh

+mm+ss+tz
	Start time of the window during which a Transmission Customer may request the service posted in the service offering, i.e., opening time of an offer. If null, no restrictions on the start of the offering time is implied (other than tariff requirements).

	OFFER_STOP_TIME
	OFFSPTIME
	0,16{ALPHANUMERIC}16 
	Valid date and time to seconds:

yyyy+mo+dd+hh

+mm+ss+tz
	Stop time of the window during which a Transmission Customer may request the service posted in the service offering, i.e., expiration time of an offer. If null, no restrictions on the end of the offering time is implied (other than tariff requirements).

	OPTIONAL_CODE
	N/A
	0{ALPHANUMERIC}25
	Unique Posted Path name within region
	Unique for Posted Path. If used for directionality, then the first 12 characters shall represent POR, followed by "-", followed by 12 characters which shall represent POD. Used by PATH_NAME.

	OTHER_CURTAILMENT_PRIORITY
	OTHCUR
	0{ALPHANUMERIC}8
	Valid Values:

{Registered}
	Other than NERC Curtailment priorities, such as regional Curtailment priorities. Suggested format region+number, for example MRO4, WECC7. Documented in list Template and registered with central EIR.

	OUTPUT_FORMAT
	FMT
	4{ALPHANUMERIC}4
	Valid Values:
HTML
DATA
	Format of response:

HTML– Hyper Text Markup Language for presentation using a web browser

DATA – text for use in a downloaded file.

	PATH_CODE
	N/A
	0{ALPHANUMERIC}12
	Unique code for each Posted Path as defined by Transmission Provider
	Unique code within a region for each Posted Path. Used by PATH_NAME.

	PATH_NAME
	PATH
	5{ALPHANUMERIC}50
	Unique value
	The unique name assigned to a single transmission line or the set of one or more parallel transmission lines whose power transfer capabilities are strongly interrelated and must be determined in aggregate. These lines are typically described as being on a Posted Path, corridor or Interconnection in some regions, or as crossing an interface or cut-plane in other regions. Multiple lines may be owned by different parties and require prorating of capability shares.

The name is constructed from the following codes, with each code separated by a "/". Trailing "/" may be omitted, if there are no values for OPTION_CODE and SPARE_CODE:

REGION_CODE - 2 chars, unique to OASIS system 

PRIMARY_PROVIDER_CODE - 4 chars, unique within region

PATH_CODE - 12 chars, unique for Transmission Provider

OPTIONAL_CODE - 25 chars, unique for Posted Path. If used for directionality, then the first 12 characters shall represent POR, followed by "-", followed by 12 characters which shall represent POD

SPARE_CODE - 3 chars. 

	POINT_OF_DELIVERY
	POD
	1{ALPHANUMERIC}12, Only non-numeric and non-alpha character allowed is ".".
	Unique value within Transmission Provider. Only special character allowed is ".", for example, ab.cde.123
	One or more point(s) of Interconnection on the Transmission Provider's transmission system where capacity and/or energy transmitted by the Transmission Provider will be made available to the receiving party. This is used along with POR to define a Posted Path and direction of flow on that path.  For internal Posted Paths, this would be a specific location(s) in the area.  For an external Posted Path, this may be an area-to-area interface.

	POINT_OF_RECEIPT
	POR
	1{ALPHANUMERIC}12

Only non-numeric and non-alpha character allowed is ".".
	Unique value within Transmission Provider. Only special character allowed is ".", for example, ab.cde.123
	One or more point(s) of Interconnection on the Transmission Provider's transmission system where capacity and/or energy transmitted will be made available to the Transmission Provider by the delivering party. This is used along with POD to define a Posted Path and direction of flow on that path.  For internal Posted Paths, this would be a specific location(s) in the area.  For an external Posted Path, this may be an area-to-area interface.

	POINT_OF_TITLE
	POT
	1{ALPHANUMERIC}12

Only non-numeric and non-alpha character allowed is ".".
	Unique value within Transmission Provider. Only special character allowed is ".", for example, ab.cde.123
	Point where the Transmission Customer takes title to the output of the generation that the Transmission Customer does not own. 

	POSTING_NAME
	POSTNAME
	1{ALPHANUMERIC}25
	Free form text
	Name of person who is posting the information on the OASIS Node.

	POSTING_REF
	POSTREF
	1{ALPHANUMERIC}12
	Unique Value
	Assigned by TSIP when a Transmission Service Request or Informal Message Posting Request is received by TSIP. Unique reference can be used by the user to modify or delete the posting.

	PRECONFIRMED
	PRECONF
	2{ALPHA}3
	Valid Values:
YES
NO
	Used by Transmission Customer to preconfirm sale in Template transrequest or ancrequest. If Transmission Customer indicates sale is preconfirmed, then the response is YES and the Transmission Customer does not need to confirm the sale.

	PRICE_UNITS
	UNITS
	0{ALPHA}20
	Free form text
	The units used for CEILING_PRICE, OFFER_PRICE, and BID_PRICE. 

Examples: $/MWhr, $/MWmonth, etc. for Transmission Providers. $MW – Hour for Resellers

	PRIMARY_PROVIDER_APPROVAL
	PPAPP
	0{ALPHA}1
	Valid Values:
Y

N
	To indicate Transmission Provider approval of a Transfer.

Y- Yes, Transmission Provider has approved Transfer

N - No, Transmission Provider does not approve the Transfer

	PRIMARY_PROVIDER
_CODE
	PROVIDER
	1{ALPHANUMERIC}4
	Unique code
	Unique code for each Transmission Provider. Used by PATH_NAME and in URL. Registered as part of URL at www.tsin.com.

	PRIMARY_PROVIDER_COMMENTS
	PPROVCOM
	0{ALPHANUMERIC}255
	Free form text
	Informative text. Usually entered by the Transmission Provider through a back end system. For information communicated between  Transmission Provider and all other parties.

	PRIMARY_PROVIDER_DUNS
	PPROVDUNS
	9{NUMERIC}9
	Valid DUNS number
	Unique code for each Transmission Provider. Provided by Dun and Bradstreet.

	PRIMARY_PROVIDER_PROVISIONS
	PPPROV
	0{ALPHA}1
	Valid Values

Y

Null
	To indicate certain Transmission Provider provisions apply to the Transmission Service reservation, e.g., rollover rights, CCO, etc.

Y = Yes, additional  Transmission Provider provisions are associated with the Transmission Service reservation

Null = No additional Transmission Provider provisions are associated with the Transmission Service reservation 

	PROCEDURE_NAME
	PROCNAME
	0{ALPHANUMERIC}25
	Valid Values:

NERC TLR

WECC USF

{Registered}


	Name of a transmission security procedure:

- NERC TLR as defined in NERC Standard IRO-006
- WECC USF as defined in WECC Policy

-
Local procedure as registered by Transmission Providers

	PROCEDURE_LEVEL
	PROCLVL
	1{ALPHANUMERIC}25
	Valid Values:

(NERC TLR Levels}

{WECC USF Levels}

{Registered}
	Levels or stages associated with actions to be taken in implementation of a transmission security procedure as defined in:

- NERC Standard IRO-006 for the NERC TLR procedure

- WECC Policy for the WECC USF procedure

-
Local procedures as registered by Transmission Providers

	PROVIDER_ACTION
	PROVACT
	1{ALPHANUMERIC}25
	Free form text, for example:

DENIED

CURTAILED

INTERRUPTED
	Indicates the particular action taken by the Transmission Provider with respect to the scheduled transaction; specific values to be returned are, DENIED if the schedule was not started as requested, CURTAILED if the scheduled MW was limited for reliability reasons, or INTERRUPTED if the scheduled MW was limited for economic reasons.  

	REASSESSMENT_END_TIME 
	RASMTTIME
	0,16{ALPHANUMERIC}16  
	Valid date and time to seconds:

yyyy+mo+dd+hh+tz 
	Time marking the end of the current Biennial Reassessment interval, if applicable, when the conditions associated with a reservation subject to a CCO may be subject to change.

	REASSIGNED_CAPACITY
	RASCAP
	1{NUMERIC}12
	Positive number, cannot exceed previous assigned capacity 
	The amount of transfer capability that was reassigned from one entity to another.

	REASSIGNED_REF
	REREF
	1{ALPHANUMERIC}12
	Unique value
	Contains the ASSIGNMENT_REF of any preceding (Parent) Reservations that are affected by this request. Used only for secondary market sales. 

	REASSIGNED_START_TIME
	RESSTIME
	16{ALPHANUMERIC}16  
	Valid date and time to seconds:

yyyy+mo+dd+hh+tz 
	Beginning date and time of the reassigned Transmission Service.

	REASSIGNED_STOP_TIME
	RESSPTIME
	16{ALPHANUMERIC}16  
	Valid date and time to hour:

yyyy+mo+dd+hh+tz 
	Date and time of the end of the Transmission Service that is reassigned to another user.

	RECORD_STATUS
	RECSTATUS
	1{NUMERIC}3
	Error number
	Record status indicating record was successful or error code if unsuccessful. 

200 = Successful.

	RECORD_TYPE
	RECTYPE
	1{ALPHA}1
	Valid Values:

I

U

D
	Indicates the type of information reported in a response record generated by an audit Template.  "I" designates information as it was initially inserted (posted) on OASIS; "U" designates information updated (modified) on OASIS; "D" designates deleted information as it appeared on OASIS just prior to being deleted (as appropriate).

	REDUCTION_REASON
	REDREAS
	1{ALPHANUMERIC}50
	Free form text
	Reason for the reduction

	REDUCTION_TYPE
	REDTYPE
	1{ALPHANUMERIC}25
	Valid values:

CONDITIONAL_EVENT

CONDITIONAL_HOURS

RECALL

REDIRECT

RESALE

TRANSFER

{Registered}


	Type of reduction in reserve capacity or service Curtailment priority

	REGION_CODE
	N/A
	1{ALPHANUMERIC}2
	Unique within OASIS system 
	Defined for NERC regions, with the following defined:

S –
SERC

T –
ERCOT

A –
MRO
P –
SPP

N –
NPCC

W –
WECC

F –
FRCC
R –
Reliability First Corporation
Second character or digit reserved for subregion id as defined by each region.

	RELATED_REF
	RELREF
	1{ALPHANUMERIC}12
	Unique reference
	Contains the ASSIGNMENT_REF of any preceding (Parent) Reservations or NITS Application that are affected by this request.

	RENEWAL_DUE_TIME
	RENEWTIME
	0,16{ALPHANUMERIC}16  
	Valid date and time to seconds:

yyyy+mo+dd+hh+tz 
	The deadline for Transmission Customer submission of a renewal request to exercise their on-going rollover rights.

	REQUEST_INTERVAL
	RINT
	1{ALPHANUMERIC}8
	Valid Values:

HOUR

DAY

WEEK

MONTH

YEAR

{Registered}
	Request Interval is used to for purposes of documenting response times.  Request Interval does not define or modify the product being requested.  The following Request Intervals shall be associated with this table:

a. Hour: Request Interval of less than one day

b. Day: Request Interval comprising at least one day but less than one week 

c. Week: Request Interval comprising at least one week but less than one month 

d. Month: Request Interval comprising at least one month but less than one year 

e. Year: Request Interval comprising at least one year

while additional intervals can be used if they are registered on www.tsin.com and shown in the Transmission Provider's list Template.

	REQUEST_REF
	RREF
	0{ALPHANUMERIC}12
	Unique value
	A reference uniquely assigned by a Transmission Customer to a request for service from a Transmission Provider.

	REQUEST_STATUS
	RSTATUS
	1{NUMERIC}3
	Error number
	Message status indicating message was successful (if all RECORD_STATUS show success) or error code if any RECORD_STATUS showed unsuccessful. 

200 = Successful

	REQUEST_TYPE
	REQTYPE
	1{ALPHA}30
	Valid Values:

ORIGINAL

RESALE

RENEWAL

MATCHING

DEFERRAL

REDIRECT

RELINQUISH
FULL_TRANSFER

PART_TRANSFER

RECALL {Registered}
	ORIGINAL – typical reservation requests submitted to the Transmission Provider (as the Seller of the Transmission Service or ancillary service).

RESALE – secondary market requests submitted to a Transmission Customer as Reseller.

RENEWAL – request to exercise rollover rights.

MATCHING – request to meet or exceed a competing request to retain Transmission Service (right of first refusal).

DEFERRAL – request to defer or apply for extension on start of Transmission Service

REDIRECT – request to Redirect all or portion of a Transmission Service reservation to an alternate POR/POD and/or make other changes to the terms of service as permitted.

RELINQUISH – request to release all or a portion of the capacity of a Redirect on a non-firm basis to the firm Parent Reservation.
FULL_TRANSFER = request to Transfer all capacity, rights, encumbrances and obligations, including financial liability to the Transmission Provider, from one Transmission Customer to another.

PART_TRANSFER = request to Transfer a portion,  but not all, capacity, and all rights, and obligations, including financial liability associated with the transferred capacity to the Transmission Provider, from one Transmission Customer to another.  No encumbrances (Resales, etc) may be Transferred with a PART_TRANSFER.

RECALL – request submitted by the Seller (Reseller or Transmission Provider) to take back all or a portion of the capacity of a Transmission Service reservation

{registered} – Primary Provider's may register values for REQUEST_TYPE to implement specific provisions of their Tariffs.

	RESPONSE_TIME_LIMIT
	RESPTL
	16{ALPHANUMERIC}16
	Valid date and time to seconds:

yyyy+mo+dd+hh

+mm+ss+tz
	Date and time to seconds by when a response must be received from a Transmission Customer.

	RESPONSIBLE_PARTY
	PARTY
	1{ALPHANUMERIC}4
	Registered company code for a Transmission Provider (TP), Security Coordinator (SC) or Control Area (CA)
	The company code of the entity responsible for administering a transmission security procedure.

	RESPONSIBLE_PARTY
_NAME
	PARTNAME
	1{ALPHANUMERIC}25
	Free form text 
	The name of the person responsible for granting the waiver.  The discretion template and the related RESPONSIBLE_PARTY_NAME data element are an optional implementation at the Transmission Provider’s discretion.

	RETURN_TZ
	TZ
	2{ALPHANUMERIC}2
	Valid Values:
AD
AS
PD
PS
ED
ES
MD
MS
CD
CS
UT
	A time zone code, indicating the base time zone, and whether daylight saving time is to be used. This field may be set by a Transmission Customer in a query. Returned date and time data is converted to this time zone.


Time zones:

Atlantic time = AD, AS

Eastern time = ED, ES

Central time = CD, CS

Mountain time = MD, MS

Pacific time = PD, PS

Universal time = UT

	ROLLOVER_CAPACITY
	ROLLCAP
	0{NUMERIC}12
	Non-negative number in units of MW
	The amount of reserved capacity eligible for rollover over the interval of ROLLOVER_START_TIME to ROLLOVER_STOP_TIME.

	ROLLOVER_START_TIME
	ROLLSTTIME
	0,16{ALPHANUMERIC}16 
	Valid date and time to seconds:

yyyy+mo+dd+hh

+mm+ss+tz
	Start of the interval designating the amount of ROLLOVER_CAPACITY available.

	ROLLOVER_ELIGIBLE
	ROLLELG
	0{NUMERIC}12
	Non-negative number in units of MW
	The maximum amount of reserved capacity that is to be considered eligible for granting of on-going rollover rights.  The actual amount available to be renewed over time will be the lower of ROLLOVER_ELIGIBLE and ROLLOVER_CAPACITY. 

	ROLLOVER_STOP_TIME
	ROLLSPTIME
	0,16{ALPHANUMERIC}16 
	Valid date and time to seconds:

yyyy+mo+dd+hh

+mm+ss+tz
	End of the interval designating the amount of ROLLOVER_CAPACITY available.

	ROLLOVER_WAIVED
	ROLLWV
	0{ALPHA}1
	Valid Values:
Y
Null
	Used by Transmission Customer to indicate their waiving (ROLLOVER_WAIVED=’Y’) of any consideration for on-going rollover rights that may be applicable to the service requested at the time of submittal. 

	SALE_REF
	SREF
	0{ALPHANUMERIC}12
	Unique value 
	Identifier which is set by Seller (including Transmission Provider) when posting a service for sale.

	SCHEDULE_GRANTED
	SCHEDGRNTED
	0{NUMERIC}12
	Non-negative number in units of MW
	Reflects the MW value of energy actually scheduled by the Transmission Provider at either the POR or POD, whichever is larger, over the START_TIME/STOP_TIME time interval.

	SCHEDULE_LIMIT
	SCHEDULELIM
	0{NUMERIC}12
	Non-negative number in units of MW
	Reflects the maximum MW value over the START_TIME/STOP_TIME interval that the Transmission Provider has determined can be scheduled.

	SCHEDULE_PRIORITY
	SPRIORITY
	0{ALPHANUMERIC}8
	Registered e-Tag transmission product code
	Identifies the relative priority of this particular Interchange Transaction as compared to all other scheduled transactions with respect to the application of Curtailments or interruptions as well as the type of transmission service schedule.  SCHEDULE_PRIORITY would typically reflect the lowest priority (if multiple reservations are used to support the schedule) registered transmission product code used to support the schedule on the Transmission Provider’s system.  Transmission product codes are registered values established by the Version 1.8.1 Electronic Tagging Functional Specifications.  If the schedule is not associated with an electronic tag, this should represent the NERC_CURTAILMENT_PRIORITY or OTHER_CURTAILMENT_PRIORITY associated with the transmission service scheduled. 

	SCHEDULE_REF
	SCHDREF
	0{ALPHANUMERIC}20
	Unique reference
	Unique reference assigned by Transmission Provider to a posting of a schedule information.

	SCHEDULE_REQUESTED
	SCHEDULEREQ
	0{NUMERIC}12
	Non-negative number in units of MW
	Scheduled energy requested by the Transmission Customer.

	SECURITY_REF
	SECREF
	1{ALPHANUMERIC}10
	Unique value
	Unique value generated by company initiating the security for each security event in the security Template.

	SECURITY_TYPE
	SECTYPE
	1{ALPHANUMERIC}6
	Valid Values:

OUTAGE

LIMIT
{Registered}
	Identifies the type of information posted for the event; restricted values are:
OUTAGE – for postings reflecting the state of critical transmission facilities
LIMIT – for postings reflecting the implementation of security procedures to limit or reduce scheduled transactions.

	SELLER_CODE
	SELLER
	1{ALPHANUMERIC}6
	Unique value
	Organization name of Transmission Provider or Reseller. 

	SELLER_COMMENTS
	SELCOM
	0{ALPHANUMERIC}255
	Free form text
	Informative text provided by the Seller. For information communicated between the Seller (either Transmission Provider or Reseller) to the Transmission Customer of the services. 

	SELLER_DUNS
	SELDUNS
	9{NUMERIC}9
	Valid DUNS number
	Unique Data Universal Numbering System (DUNS) provided by Dun and Bradstreet. Code for a  Seller.

	SELLER_EMAIL
	SELEMAIL
	5{ALPHANUMERIC}60
	Valid network reference
	E-mail address of Seller contact person.

	SELLER_FAX 
	SELFAX
	14{ALPHANUMERIC}20
	Area code and telephone number, plus any extensions

Example: (aaa)-nnn-nnnn xnnnn
	The fax telephone number for contact person at Seller.

	SELLER_NAME 
	SELNAME
	1{ALPHANUMERIC}25
	Free form text 
	The name of an individual contact person at the Seller.

	SELLER_PHONE 
	SELPHONE
	14{ALPHANUMERIC}20
	Area code and telephone number, plus any extensions

(aaa)-nnn-nnnn xnnnn
	The telephone number of a contact person as a Seller.

	SELLER_REF
	SELREF
	0{ALPHANUMERIC}12
	Free form text
	Identifier which is set by Seller (including Transmission Provider) to uniquely identify reservation requests for Seller’s own internal use.

	SERVICE_DESCRIPTION 
	SVCDESC
	0{ALPHANUMERIC}200 
	Free form text
	Information regarding a service.

	SERVICE_INCREMENT
	SRVINCR
	1{ALPHANUMERIC}8
	Valid Values:

HOURLY

DAILY

WEEKLY

MONTHLY

YEARLY

{Registered}
	The Transmission Service increments provided. Five are pre-defined, while additional increments can be used if they are registered on www.tsin.com and shown in the Transmission Provider's list Template.

	SERVICE_NAME 
	SVCNAME
	1{ALPHANUMERIC}25
	Free form text
	Name of service affected by the discretionary action.

	SERVICE_TYPE
	SVCTYPE
	1{ALPHANUMERIC}25
	Free form text
	Type of service affected by the discretionary action.

	SINK
	SINK
	0{ALPHANUMERIC}14
	Valid  area  name
	The area in which the SINK is located.

	SOURCE
	SOURCE
	0{ALPHANUMERIC}14
	Valid  area  name
	The area in which the SOURCE is located.

	SOURCE_BA
	SOURCEBA
	0{ALPHANUMERIC}14
	Valid  area  name
	The control area (BA) from which the power will originate.

	SPARE_CODE
	N/A
	0{ALPHANUMERIC}3
	Defined by region
	Spare code to be used at a later time. Used by PATH_NAME.

	STANDARDS_OF_CONDUCT_ISSUES
	STDISSUE
	0{ALPHANUMERIC}800
	Free form text
	Information disclosures that were in violation of the FERC Standards of Conduct described in 18 CFR, Part 358.  This text may include a reference pointer to a more detailed description.  The stdconduct Template and the related STANDARDS_OF_CONDUCT_ISSUES data element are an optional implementation at the Transmission Provider’s discretion.

	START_TIME
	STIME
	16{ALPHANUMERIC}16 
	Valid date and time to seconds:

yyyy+mo+dd+hh

+mm+ss+tz
	Start date and clock time of a service. When used as a Query Variable, it requires the return of all items whose Stop time is after the Start time. 

Note that for some Templates when used as a Query Variable the time may be only valid up to the hour, day or month. If more data is given than is valid, the hour, day or month will be used to make the date and time inclusive, i.e. date or time will be truncated to valid hour, day or month.

	START_TIME_POSTED
	STIMEP
	16{ALPHANUMERIC}16 
	Valid date and time to seconds:

yyyy+mo+dd+hh

+mm+ss+tz
	Query parameter to indicate all the records are to be retrieved that were posted on or after this time.

	START_TIME_QUEUED
	STIMEQ
	16{ALPHANUMERIC}16 
	Valid date and time to seconds:

yyyy+mo+dd+hh

+mm+ss+tz
	Start date and clock time of a service, used for requesting transactions queued after this time.

	STATUS

	STATUS
	5{ALPHANUMERIC}25
	Valid Values:
QUEUED
INVALID
RECEIVED
STUDY
REBID
COUNTEROFFER
CR_COUNTEROFFER
DECLINED
SUPERSEDED
ACCEPTED
CR_ACCEPTED
REFUSED
CONFIRMED
WITHDRAWN
DISPLACED
ANNULLED
RETRACTED
PRESUBMITTED

DEFICIENT

COMPLETED

REEVALUATE
	QUEUED – initial status assigned by the TSIP on receipt of "Transmission Customer services purchase request or “Transmission Customer NITS Application request”".

INVALID – assigned by the TSIP or Transmission Provider indicating an invalid field in the request, such as improper POR, POD, SOURCE, SINK, etc. or that the request does not meet minimum information or data requirements (Final state).

RECEIVED – assigned by the Transmission Provider or Reseller (PTP only) to acknowledge QUEUED requests and indicate the service request is being evaluated, including for completing the required ancillary services.

STUDY – assigned by the Transmission Provider or Reseller (PTP only) to indicate some level of study is required or being performed to evaluate service request.

REFUSED – assigned by the Transmission Provider or Reseller (PTP only) to indicate service request has been denied due to lack of ATC. (Final state).

COUNTEROFFER – assigned by Transmission Provider or Reseller (PTP only) to indicate that a new OFFER_PRICE and/or CAPACITY_GRANTED over time is being proposed in the negotiation of requested service (i.e., offering of Partial Service or negotiation of price).
CR_COUNTEROFFER – assigned by Transmission Provider or Reseller to indicate that a new OFFER_PRICE and/or CAPACITY_GRANTED over time is being proposed in the negotiation of requested service for Coordinated Request (i.e., offering of Partial Service or negotiation of price

REBID – assigned by the Transmission Customer to indicate that a new value for BID_PRICE (PTP only) and/or CAPACITY_REQUESTED over time is being proposed.
SUPERSEDED – assigned by the Transmission Provider or Reseller (PTP only) when a request which has not yet been confirmed is preempted by another reservation request. (Final state).

	STATUS (cont.)
	
	
	
	ACCEPTED – assigned by the Transmission Provider or Reseller (PTP only) to indicate the service request at the designated BID_PRICE and CAPACITY_REQUESTED has been approved/accepted. Depending upon the type of ancillary services required, the Seller may or may not require all ancillary service reservations to be completed before accepting a request.
CR_ACCEPTED - assigned by the Transmission Provider or Reseller to indicate the service Coordinated Request at the designated BID_PRICE and CAPACITY_REQUESTED has been approved/accepted. Depending upon the type of ancillary services required, the Seller may or may not require all ancillary service reservations to be completed before accepting a Coordinated Request.

DECLINED – assigned by the Transmission Provider or Reseller (PTP only) to indicate that the terms and conditions of the request, such as the BID_PRICE, are unacceptable and that negotiations are terminated or that contractual terms have not been met or the Transmission Customer has failed to respond to a deficiency within an established time frame. (Final state).

RETRACTED - assigned by the Transmission Provider or Reseller (PTP only) when the Transmission Customer fails to confirm or withdraw the request within the required time period. (Final state).
WITHDRAWN – assigned by the Transmission Customer during a request evaluation to withdraw the request from any further action. (Final state).

CONFIRMED – assigned by the Transmission Customer in response to the Transmission Provider or Reseller (PTP only) posting "ACCEPTED" or “COUNTEROFFER” status, to confirm service or the NITS Application. Once a request has been "CONFIRMED", a Transmission Service reservation exits or the NITS Application is completed. (Final state, unless overridden by DISPLACED or ANNULLED state).

	STATUS (cont.)
	
	
	
	DISPLACED – (PTP and Secondary Network Transmission Service only) - assigned by the Transmission Provider or Reseller when a "CONFIRMED" reservation from a Transmission Customer is displaced by a higher priority reservation, and the Transmission Customer is not offered or has not exercised right of first refusal (i.e. refused to match terms of new request). (Final state).

ANNULLED – assigned by the Seller when, by mutual agreement with the Transmission Customer, a confirmed reservation, NITS Application, or pre-confirmed request is to be voided or assigned unilaterally by the Transmission Provider when a Resale reservation is to be voided or NITS Application is to be voided. (Final state).
PRESUBMITTED – assigned by OASIS prior to submission by the Eligible Network Customer or Network Customer in a workspace to be saved while developing a NITS Application or request for modification of service such that the NITS Application or request for modification of service can be submitted at a later date. 

DEFICIENT – assigned by the Transmission Provider indicating a correctable deficiency in the NITS Application or request for modification of service, such as missing, incomplete, or inconsistent information. 

COMPLETED – assigned by the Transmission Provider indicating that are no deficiencies or all deficiencies in the NITS Application or request for modification of service have been met.
REEVALUATE – assigned by the Eligible Transmission Customer or Network Customer during the NITS Application or request for modification of service process indicating that deficiencies have been corrected, and the revised NITS Application or request for modification of service is ready for review.

	STATUS_COMMENTS
	STACOM
	0{ALPHANUMERIC}255
	Free form text
	Informative: For information to be communicated by any party to all other parties.

	STATUS_NOTIFICATION
	STATNOT
	0{ALPHANUMERIC}200
	http://URL:portnumber/direcotry/cgi script/query parameters 

or 

Mailto: <E-mail address>
	Shall contain the protocol field "http:", which designates the notification method/protocol to be used, followed by all resource location information required; the target domain name and port designations shall be inserted into the notification URL based on the Transmission Customer's Company registration information.  The resource location information may include directory information, CGI script identifiers and URL encoded query string name/value pairs as required by the Transmission Customer's application.

or

Mailto and E-mail address for the status information the Transmission Customer wants to receive upon a change in STATUS of transstatus, or ancstatus
.

	STOP_TIME
	SPTIME
	16{ALPHANUMERIC}16
	Valid date and time

yyyy+mo+dd+hh

+mm+ss+tz
	Stop date and clock time. When used as a Query Variable, it requires the return of all items which start before the stop time. 

Note that for some Templates when used as a Query Variable the time may be only valid up to the hour, day or month. If more data is given than is valid, the hour, day or month will be used to make the date and time inclusive, i.e. date or time will be increased to include STOP_TIME.

	STOP_TIME_POSTED
	STPTIMEP
	16{ALPHANUMERIC}16 
	Valid date and time to seconds:

yyyy+mo+dd+hh

+mm+ss+tz
	Query parameter to indicate all the records are to be retrieved that were posted on or before this time.

	STOP_TIME_QUEUED
	SPTIMEQ
	16{ALPHANUMERIC}16 
	Valid date and time to seconds:

yyyy+mo+dd+hh

+mm+ss+tz
	Stop date and clock time, used for requesting transactions queued before this time.

	SUMMER_INTERRUPT_FREQUENCY
	SUMINTFREQ
	0{ALPHA}1
	Valid Values:
Y

N
	If set to Y, the acceptable frequency of interruptions of summer load is on file with the Transmission Provider.

	SUMMER_INTERRUPT_FREQUENCY_PERIOD
	SUMINTFREQPD
	0{ALPHA}1
	Valid Values:
Y

N
	If set to Y, the time frame during which summer load is subject to interruption is on file with the Transmission Provider.

	SUMMER_INTERRUPT_LIMITATIONS
	SUMINTLIM
	0{ALPHA}1
	Valid Values:
Y

N
	If set to Y, the limitations of the frequency and amount of interruptions of summer load is on file with the Transmission Provider.

	SUMMER_INTERRUPT_LOAD_AMOUNT
	SUMINTLDAMOUNT
	0{ALPHA}1
	Valid Values:
Y

N
	If set to Y, the MW amount of acceptable interruptions of summer load is on file with the Transmission Provider.

	SUMMER_INTERRUPT_LOAD_CONDITIONS
	SUMINTLDCOND
	0{ALPHA}1
	Valid Values:
Y

N
	If set to Y, the conditions under which the summer load is subject to interruption is on file with the Transmission Provider.

	SUMMER_INTERRUPT_LOAD_PORTION
	SUMINTLDPORT
	0{ALPHA}1
	Valid Values:
Y

N
	If set to Y, the portion of summer load subject to interruption is on file with the Transmission Provider.

	SUMMER_LOAD_FORECAST
	SUMLDFORE
	0{ALPHANUMERIC}20
	Valid Values:

SUMMER_FORECASTED_LOAD

{Registered}
	A ten year forecast of winter load requirements, beginning with the first year after the service is scheduled to commence.

	SUBJECT
	SUBJ
	0{ALPHANUMERIC}80
	Free form text
	Informative text used to summarize a topic in a message.

	SYSTEM_ATTRIBUTE
	SYSATTR
	0{ALPHANUMERIC}20
	Valid Values:

CBM
FTRM

NFTRM

TTC

FATC

NFATC

FGF

NFGF

ATC_ANNOTATION
ZERO_FATC_NARRATIVE

ZERO_NFATC_NARRATIVE

FATC_CHANGE_NARRATIVE

NFATC_CHANGE_NARRATIVE

ZONE_FORECASTED_LOAD
SYSTEM_FORECASTED_LOAD
NATIVE_FORECASTED_LOAD
ZONE_ACTUAL_LOAD
SYSTEM_ACTUAL_LOAD
NATIVE_ACTUAL_LOAD
{Registered}
	Type of system data viewed by systemdata template:

CBM

FTRM – TRM for use in FATC

NFTRM – TRM for use in NFATC

TTC – Total Transmission Capability

FATC – Firm Available Transmission Capability

NFATC – Non-firm Available Transmission Capability

FGF – Firm Grandfathered firm Transmission Service

NFGF – Non-firm Grandfathered Transmission Service

ATC_ANNOTATION – Annotation for a change in monthly or yearly posted ATC (no longer used replaced by ZERO_FATC_NARRATIVE, ZERO_NFATC_NARRATIVE, FATC_CHANGE_NARRATIVE, NFATC_CHANGE_NARRATIVE)
ZERO_FATC_NARRATIVE – Narrative when a posted firm monthly or yearly ATC value remains unchanged at a value of zero

ZERO_NFATC_NARRATIVE – Narrative when a posted non-firm monthly or yearly ATC value remains unchanged at a value of zero

FATC_CHANGE_NARRATIVE – Narrative when a posted firm monthly or yearly ATC changes as a result of a 10 percent change in TTC/TFC
NFATC_CHANGE_NARRATIVE – Narrative when a posted non-firm monthly or yearly ATC changes as a result of a 10 percent change in TTC/TFC
ZONE_FORECASTED_LOAD – anticipated Forecasted Daily Zonal Load
SYSTEM_FORECASTED_LOAD – anticipated Forecasted Daily System-Wide Peak Load

	SYSTEM_ATTRIBUTE (cont.)
	
	
	
	NATIVE_FORECASTED_LOAD – Anticipated Forecasted Daily Native Peak Load portion of the Forecasted Daily System-Wide Peak Load for the NL
ZONE_ACTUAL_LOAD – Actual Daily Zonal Load

SYSTEM_ACTUAL_LOAD – Actual Daily System-Wide Peak Load

NATIVE_ACTUAL_LOAD – Actual Daily Native Peak Load portion of Actual Daily System-Wide Peak Load for the NL

{registered} – Transmission Provider specific registered name for the data posted

	SYSTEM_ELEMENT
	SYSELM
	1{ALPHANUMERIC}255
	Free form text
	The name of a transmission system element associated with the SYSTEM_ATTRIBUTE being posted.  For example, the value of PATH_NAME for records with SYSTEM_ELEMENT_TYPE = PATH.  See Business Practice Standard WEQ-013 for further information.

	SYSTEM_ELEMENT_TYPE
	SYSELMTY
	1{ALPHA}30
	Valid Values:

PATH

LOAD_ZONE

{Registered}
	PATH - Designates that the value of SYSTEM_ELEMENT is the name of a Posted Path (PATH_NAME).

LOAD_ZONE - Designates that the value of SYSTEM_ELEMENT is the name of the metered zone whose forecast and actual load data is being posted.

{registered} - Transmission Provider's may register values for SYSTEM_ELEMENT_TYPE to implement specific provisions of their tariffs.

	TARIFF_REFERENCE
	TARIFF
	0{ALPHANUMERIC}150
	Free form text
	Tariffs approved by FERC
Name and description of tariff

	TEMPLATE
	TEMPL
	1{ALPHANUMERIC}20
	Valid name of Template from Business Practice Standard WEQ-002-4.3.1 or from list Template
	The name of a logical collection of DATA_ELEMENTS in a user's interaction with an OASIS Node.

	THIRD_PARTY_POWER_ARRAGEMENTS
	
	1{ALPHANUMERIC}255
	Free form text
	Arrangements governing sale and delivery of power to third parties from generating facilities located in the Transmission Provider Control Area, where only a portion of unit output is designated as a Network Resource.

	TIME_OF_LAST_UPDATE
	TLUPDATE
	16{ALPHANUMERIC}16 
	Valid date and time to seconds:

yyyy+mo+dd+hh

+mm+ss+tz
	Date and time to seconds that data was last updated. May be used to search data updated since a specific point in time. 

	TIME_POSTED
	TIMEPST
	16{ALPHANUMERIC}16 
	Valid date and time to seconds:

yyyy+mo+dd+hh

+mm+ss+tz
	Date and time a message is posted.

	TIME_QUEUED
	TIMEQ
	16{ALPHANUMERIC}16 
	Valid date and time to seconds:

yyyy+mo+dd+hh

+mm+ss+tz
	Date and time that the request was queued.

	TIME_STAMP
	TSTAMP
	16{ALPHANUMERIC}16  
	Valid date and time to seconds

yyyy+mo+dd+hh+mm+ss+tz
	Time data is created.

	TRANSACTION_ID
	TRANSID
	1{ALPHANUMERIC}30
	Free form text
	Identifier associated with an Interchange Transaction that may span multiple SCHEDULE_REF records. May be the NERC Tag id as specified in the Version 1.8.1 Electronic Tagging Functional Specification.

	TRANSACTION_TYPE
	TRANST
	0{ALPHANUMERIC}25
	Registered e-Tag transaction type
	Registered transaction type identifier as specified in the Version 1.8.1 Electronic Tagging Functional Specification.  If the transaction is not based on an electronic tag, this may take on any appropriate value established by the Transmission Provider.

	TPSE_CODE
	TPSE
	1{ALPHANUMERIC}8
	Registered e-Tag company code
	This is the registered code associated with the Transmission Customer whose Transmission Service has been scheduled.

	TS_CLASS
	TSCLASS
	1{ALPHANUMERIC}20
	Valid Values:

FIRM

NON-FIRM

TTC

SECONDARY

{Registered}
	The Transmission Service classes provided. Four are pre-defined, while additional classes can be used if they are registered on www.tsin.com and shown in the Transmission Provider's list Template page. SECONDARY is defined as alternate PORs or PODs for POINT_TO_POINT, or as non-designated resources for NETWORK service.
TTC is retained as a Valid Value for historical purposes only.
For Network Service the valid value will be defined  as FIRM except non-designated resources which will be defined as SECONDARY.

	TS_PERIOD
	TSPER
	1{ALPHANUMERIC}20
	Valid Values:

ON_PEAK

OFF_PEAK

FULL_PERIOD

{Registered}
	The Transmission Service periods provided. Three are pre-defined, while additional periods can be used if they are registered on www.tsin.com and shown in the Transmission Provider's list Template.

	TS_SUBCLASS
	TSSUBC
	0{ALPHANUMERIC}20
	Free form text
	The Transmission Service subclasses provided. These are free form.

	TS_TYPE
	TSTYPE
	1{ALPHANUMERIC}20
	Valid Values:

POINT_TO_POIN T

NETWORK

ATC

{Registered}
	The Transmission Service types provided. Three are pre- defined, while additional types can be used if they are registered on www.tsin.com and shown in the Transmission Provider's list Template.
ATC is retained as a Valid Value for historical purposes only.
For Network Service the valid value will be defined as NETWORK.

	TS_WINDOW
	TSWIND
	1{ALPHANUMERIC}20
	Valid Values:

FIXED

SLIDING

EXTENDED

NEXT_INCREME NT

{Registered }
	The Transmission Service windows provided. Four are pre-defined, while additional windows can be used if they are registered on www.tsin.com and shown in the Transmission Provider's list Template.

	TZ
	TZ
	2{ALPHANUMERIC}2
	Valid Values:
AD
AS
PD
PS
ED
ES
MD
MS
CD
CS
UT
	Valid time zone and indication whether daylight savings time is to be used

Time zones:

Atlantic time = AD, AS

Eastern time = ED, ES

Central time = CD, CS

Mountain time = MD, MS

Pacific time = PD, PS

Universal time = UT

	URL_EXTERNAL_AREF
	URLEXTAREF
	1{ALPHANUMERIC}12
	Unique value
	A unique reference number assigned by a TSIP to provide a unique record for identifying the URL external transmission assignment ref.

	VALID_FROM_TIME
	VALFTIME
	16{ALPHANUMERIC}16
	Valid date and time

yyyy+mo+dd+hh

+mm+ss+tz
	Date and time after which the message is valid.

	VALID_TO_TIME
	VALTTIME
	16{ALPHANUMERIC}16
	Valid date and time

yyyy+mo+dd+hh

+mm+ss+tz
	Date and time before which the message is valid.

	VARIABLE_GEN_COST
	
	1{NUMERIC}5 + "." 

+ 2{NUMERIC}4 
	Positive number with 2 to 4 decimals
	Approximate variable generating cost ($/MWH) for redispatch computations

	VERSION
	VER
	1{REAL NUMBER}6
	Range of 1.0 to 9999.9
	Specifies which version of the Business Practice Standard WEQ-002 to use when interpreting the request.

	VOLTAGE_LEVEL
	VOLTLEV
	0{ALPHANUMERIC}25
	Free form text, for  example: thermal, stability, voltage or as defined in list Template
	The voltage level at which the load is connected to the network (point of interconnection/delivery).

	WINTER_INTERRUPT_FREQUENCY
	WININTFREQ
	0{ALPHA}1
	Valid Values:
Y

N
	If set to Y, the acceptable frequency of interruptions of winter load is on file with the Transmission Provider.

	WINTER_INTERRUPT_FREQUENCY_PERIOD
	WININTFREQPD
	0{ALPHA}1
	Valid Values:
Y

N
	If set to Y, the time frame during which winter load is subject to interruption is on file with the Transmission Provider.

	WINTER_INTERRUPT_LIMITATIONS
	WININTLIM
	0{ALPHA}1
	Valid Values:
Y

N
	If set to Y, the limitations of the frequency and amount of interruptions of winter load is on file with the Transmission Provider.

	WINTER_INTERRUPT_LOAD_AMOUNT
	WININTLDAMOUNT
	0{ALPHA}1
	Valid Values:
Y

N
	If set to Y, the MW amount of acceptable interruptions of winter load is on file with the Transmission Provider.

	WINTER_INTERRUPT_LOAD_CONDITIONS
	WININTLDACOND
	0{ALPHA}1
	Valid Values:
Y

N
	If set to Y, the conditions under which the winter load is subject to interruption is on file with the Transmission Provider.

	WINTER_INTERRUPT_LOAD_PORTION
	WININTLD
	0{ALPHA}1
	Valid Values:
Y

N
	If set to Y, the portion of winter load subject to interruption is on file with the Transmission Provider.

	WINTER_LOAD_FORECAST
	WINTERFORE
	0{ALPHANUMERIC}20
	Valid Values:

WINTER_FORECASTED_LOAD

{Registered}
	A ten year forecast of winter load requirements, beginning with the first year after the service is scheduled to commence.


� For all Data Elements indicated as being free form text, the Field Format designation of ALPHANUMERIC shall represent any valid printable character in the ASCII character set.





�Need to review the definitions in terms of modification of service.


�Need to add NITS-related template name(s) when we know them





