013-2
OASIS TRANSACTION PROCESSING
The basic OASIS transaction process is described below.  This OASIS Implementation Guide also provides additional requirements and guidance for processing specific types of business transactions in the implementation of OASIS.  Note that the Transmission Provider may, but is not limited to, interacting with OASIS using the Transmission Customer template or user interface.  Transmission Providers may also implement OASIS functions on back-end systems and are not required to perform all transaction processing on an OASIS Node proper, provided that the results of all transaction processing are correctly posted on OASIS as required by the tariff, regulation, or other established Business Practices.
The following is a summary of the templates used and actions that may be taken by the Transmission Customer and Seller to execute a transaction on OASIS.  Note that the OASIS Business Practice Standards require all template functionality to be provided through a user interface.  While this discussion focuses on template execution, all actions must be supported through a browser-based user interface. Detailed examples of the transaction process and description of the business logic envisioned to be implemented as part of the Transmission Provider’s OASIS or other back-end transaction support services are provided in subsequent sections of this OASIS Implementation Guide.

a.
The transrequest and ancrequest templates shall be used by the Transmission Customer to enter a transaction request for specific Transmission Services or ancillary services from a specified Seller.  All pertinent transaction-specific information must be provided in the template Data Elements.
b.
The transstatus and ancstatus templates shall be used by both Transmission Customer and Seller to query for the current transaction information (e.g., STATUS).  Alternatively, the Transmission Customer may request dynamic notification per Business Practice Standard WEQ-002-4.2.10.3 whenever the transaction data is changed.
c.
The transsell and ancsell templates shall be used by the Seller to indicate to the Transmission Customer whether the request is acceptable or not by setting the transaction STATUS to one of RECEIVED, INVALID, STUDY, COUNTEROFFER, CR_COUNTEROFFER, ACCEPTED, CR_ACCEPTED, REFUSED, SUPERSEDED, DECLINED, DISPLACED, ANNULLED, or RETRACTED. A Transmission Provider as the Seller may use the transsell and ancsell templates to act on requests or may use proprietary software solutions to perform this function in a similar manner.
d.
The transcust and anccust templates shall be used by the Transmission Customer to indicate to the Seller whether they wish to negotiate, confirm or withdraw the transaction by setting the transaction STATUS to one of REBID, CONFIRMED, or WITHDRAWN.
e.
The transassign and ancassign templates shall be used by the Seller to notify the Transmission Provider of the transfer of rights from the Seller to the Transmission Customer consummated off the OASIS Node. 

f.
The source of all Transmission Customer and Seller contact information shall be provided under Business Practice Standard WEQ-002-3.1. Therefore, it shall not be input as part of uploads, but shall be provided as part of all transaction downloads.

g.
OASIS Nodes shall accept a Seller-initiated change in STATUS to ACCEPTED or CR_ACCEPTED only when OFFER_PRICE matches BID_PRICE. 

h.
OASIS Nodes shall accept a Transmission Customer-initiated change in STATUS to CONFIRMED only when BID_PRICE matches OFFER_PRICE.
i.
If CAPACITY_GRANTED is null when STATUS is being changed to ACCEPTED, CR_ACCEPTED or CONFIRMED, the OASIS Node shall set it equal to CAPACITY_REQUESTED.

j.
OASIS Nodes shall set the initial transaction STATUS of the request to QUEUED and assign a unique ASSIGNMENT_REF identifier for the transaction.

k.
If the Transmission Customer has identified the transaction as PRECONFIRMED and the Seller has set the transaction STATUS to ACCEPTED, OASIS Nodes shall automatically set the transaction’s STATUS to CONFIRMED without any Transmission Customer interaction required.  For Transfers (PART_TRANSFER and FULL_TRANSFER), the Seller must have posted ACCEPTED in the STATUS and the Transmission Provider must have posted Y in the   PRIMARY_PROVIDER_APPROVAL Data Elements respectively before the OASIS Nodes will automatically set the transaction’s STATUS to CONFIRMED.

l.
If the Transmission Customer has identified the transaction as PRECONFIRMED and the Seller has set the transaction STATUS to COUNTEROFFER, OASIS Nodes shall take no automatic confirmation action on the transaction and require explicit confirmation by the Transmission Customer.
m.
 If the Transmission Customer has identified the transaction as PRECONFIRMED and the Seller has set the transaction STATUS to CR_ACCEPTED, OASIS Nodes shall wait to set the transaction’s STATUS to CONFIRMED until after expiration of the Transmission Customer confirmation time limit, specified in Business Practice Standard WEQ-001 Table 4-2 Request Timing Requirements, without any Transmission Customer interaction required.

n.
If the Transmission Customer has identified the transaction as PRECONFIRMED and the Seller has set the transaction STATUS to CR_COUNTEROFFER, OASIS Nodes shall wait to set the transaction’s STATUS to RETRACTED until after expiration of the Transmission Customer confirmation time limit, specified in Business Practice Standard WEQ-001 Table 4-2 Request Timing Requirements, without any Transmission Customer interaction required.

013-2.2
Transaction Status
The following are the defined values that may appear in the STATUS Data Element associated with a given OASIS transaction:

QUEUED = initial status assigned by the TSIP on receipt of "Transmission Customer services purchase request."  

INVALID = assigned by the TSIP, or Transmission Provider indicating an invalid field in the request, such as improper POR, POD, SOURCE, SINK, etc. (Final state)

RECEIVED = assigned by the Transmission Provider or Reseller to acknowledge QUEUED requests and indicate the Transmission Service request is being evaluated, including for completing the required ancillary services.

STUDY= assigned by the Transmission Provider or Reseller to indicate some level of study is required or being performed to evaluate Transmission Service request.

REFUSED = assigned by the Transmission Provider or Reseller to indicate Transmission Service request has been denied due to lack of ATC. (Final state)

COUNTEROFFER = assigned by the Transmission Provider or Reseller to indicate that a new value for OFFER_PRICE and/or CAPACITY_GRANTED over time is being proposed in the negotiation of requested Transmission Service, (i.e., offering of Partial Service or negotiation of price).
CR_COUNTEROFFER = assigned by Transmission Provider or Reseller to indicate that a new OFFER_PRICE and/or CAPACITY_GRANTED over time is being proposed in the negotiation of requested service for Coordinated Request (i.e., offering of Partial Service or negotiation of price).
REBID = assigned by the Transmission Customer to indicate that a new value for BID_PRICE and/or CAPACITY_REQUESTED over time is being proposed.

SUPERSEDED = assigned by the Transmission Provider or Reseller when a request which has not yet been confirmed is preempted by another Transmission Service request. (Final state)

ACCEPTED = assigned by the Transmission Provider or Reseller to indicate the Transmission Service request at the designated BID_PRICE and CAPACITY_REQUESTED has been approved/accepted. Depending upon the type of ancillary services required, the Seller may or may not require all ancillary service reservations to be completed before accepting a Transmission Service request.
CR_ACCEPTED = assigned by the Transmission Provider or Reseller to indicate the Coordinated Request at the designated BID_PRICE and CAPACITY_REQUESTED has been approved/accepted. Depending upon the type of ancillary services required, the Seller may or may not require all ancillary service reservations to be completed before accepting a Coordinated Request
DECLINED = assigned by the Transmission Provider or Reseller to indicate that the terms and conditions of the Transmission Service request, such as the BID_PRICE, are unacceptable and that negotiations are terminated or that contractual terms and conditions have not been met. (Final state)

CONFIRMED = assigned by the Transmission Customer in response to the Transmission Provider or Reseller posting ACCEPTED or COUNTEROFFER STATUS, to confirm service. Once a request has been set to CONFIRMED, STATUS a Transmission Service reservation exists. (Final state, unless overridden by DISPLACED or ANNULLED state)

WITHDRAWN = assigned by the Transmission Customer during an Transmission Service request evaluation to withdraw the Transmission Service request from any further action. (Final state)

DISPLACED = assigned by the Transmission Provider or Reseller when a confirmed Transmission Service reservation from a Transmission Customer is displaced by a higher priority Transmission Service reservation and the Transmission Customer is not offered or has not exercised right of first refusal (i.e. refused to match terms of new request). (Final state)

ANNULLED = assigned by the Seller when, by mutual agreement with the Transmission Customer, a confirmed Transmission Service reservation or pre-confirmed Transmission Service request is to be voided, or assigned unilaterally by the Transmission Provider when a Resale Transmission Service reservation is to be voided. (Final state)

RETRACTED = assigned by the Transmission Provider or Reseller when the Transmission Customer fails to confirm or withdraw the Transmission Service request within the required time period. (Final state)
[image: image1.emf]
Exhibit 2 - State Diagram of Purchase Transactions
Pre-confirmed Transmission Service requests for short-term firm and non-firm PTP may not be withdrawn by the Transmission Customer unless the request has been counter offered by the Seller.  Pre-confirmed Coordinated Requests may be withdrawn as specified in Business Practice Standard WEQ-001-4.9 At the Transmission Customer’s request, however, the Transmission Provider may use its discretion and move the request from any STATUS value to ANNULLED to void the transaction. For clarity, this is not shown in the above exhibit. Pre-confirmed request for long-term firm PTP, NITS, and Redirects on a non-firm basis may be withdrawn by the Transmission Customer at any time prior to confirmation with the exception of pre-confirmed Coordinated Requests. 
013-2.2.1
ACCEPTED and CR_ACCEPTED Status Restrictions
OASIS shall block a request from being set to a STATUS of ACCEPTED or CR_ACCEPTED by the Seller if the values for OFFER_PRICE and CAPACITY_GRANTED, do not exactly match the corresponding values for BID_PRICE and CAPACITY_REQUESTED over every START_TIME/STOP_TIME interval.  This ensures that both parties to the transaction are in agreement to the terms of the request.

If the request is flagged as PRECONFIRMED=YES and CG_FLAG=N, OASIS shall immediately set the request STATUS to CONFIRMED when it is accepted, with the exception of the PART_TRANSFER and FULL_TRANSFER requests.  For these two REQUEST_TYPEs, OASIS shall require that STATUS has been set to ACCEPTED and PRIMARY_PROVIDER_APPROVAL has been set to Y before moving the request STATUS to CONFIRMED. 
If the request is flagged as PRECONFIRMED=YES and CG_FLAG=Y, OASIS shall wait to set the request STATUS to CONFIRMED until after expiration of the Transmission Customer confirmation time limit, specified in Business Practice Standard WEQ-001 Table 4-2 Request Timing Requirements,
013-2.3
Basic OASIS Transaction Handling

Requests to reserve or purchase Transmission Service or ancillary service shall be submitted to OASIS by the Transmission Customer via the transrequest or ancrequest templates.

The Seller specified in the request must be the Transmission Provider for REQUEST_TYPE of ORIGINAL, REDIRECT, RELINQUISH, RENEWAL, or DEFERRAL.  The Seller specified in the request must be a registered entity other than the Transmission Provider for REQUEST_TYPE of RESALE, FULL_TRANSFER or PART_TRANSFER.  The Seller may be either the Transmission Provider or another registered entity for REQUEST_TYPE of MATCHING.  

OASIS should screen submitted requests to validate proper use of REQUEST_TYPE.  Additional restrictions based on specific REQUEST_TYPEs are detailed in subsequent Business Practice Standards.  Validations on the Transmission Service requested, Transmission Service start time and duration, submission time, etc., are established by Transmission Provider’s Business Practice.

Once successfully submitted on OASIS, the Seller may take any of the following actions via the transsell/ancsell template:

· Acknowledge receipt by setting STATUS to RECEIVED or STUDY

· Deny the request by setting STATUS to INVALID, DECLINED, or REFUSED

· Approve the request by setting STATUS to ACCEPTED, CR_ACCEPTED, COUNTEROFFER or CR_COUNTEROFFER
At any time during the processing of a request, the Transmission Customer may set STATUS to WITHDRAWN to remove the request from further consideration by the Seller with the exception of certain pre-confirmed requests as specified in Business Practice Standard WEQ-013-2.2.

Once the Seller approves the request, the Transmission Customer may take any of the following actions via the transcust/anccust template:

· Withdraw the request

· Continue negotiation of the request by setting STATUS to REBID

· Complete the request by setting STATUS to CONFIRMED

Prior to final confirmation by the Transmission Customer, the Seller may override their approval of the request with the following actions:

· Retract approval based on exceeding of Transmission Customer confirmation time limits and/or scheduling deadlines or other criteria established by Business Practice by setting the STATUS to RETRACTED

· Retract approval based on receipt of a higher priority competing request by setting the STATUS to SUPERSEDED

Once confirmed on OASIS by the Transmission Customer a Transmission Service reservation shall be deemed to exist.  The Reseller or the Transmission Provider makes the following changes to a confirmed Transmission Service reservation:

· Nullify the reservation for cause by setting the STATUS to ANNULLED

· Displace the reservation in-whole to accommodate a higher priority competing request by setting the STATUS to DISPLACED

Seller’s shall provide a reason in the SELLER_COMMENTS whenever a service request is set to the STATUS of INVALID, REFUSED, DECLINED, RETRACTED, SUPERSEDED, ANULLED or DISPLACED.  The Transmission Provider, when not acting as the Seller, shall provide a reason in the PROVIDER_COMMENTS whenever a service request is set to the STATUS of ANNULLED or DISPLACED.
013-2.6.1
ORIGINAL Requests
Original requests shall be submitted by the Transmission Customer to arrange for new Transmission Service or ancillary service with the Transmission Provider.

The following are specific restrictions or requirements for OASIS Transmission Service requests with REQUEST_TYPE of ORIGINAL.  If REQUEST_TYPE is not specified by the Transmission Customer and SELLER_CODE and SELLER_DUNS are the same as PRIMARY_PROVIDER_CODE and PRIMARY_PROVIDER_DUNS, OASIS shall default REQUEST_TYPE to ORIGINAL.

	Data Element
	Restriction/Requirement

	REQUEST_TYPE
	Must be ORIGINAL

	RELATED_REF
	Must be null

	SELLER_CODE
	Must match PRIMARY_PROVIDER_CODE

	SELLER_DUNS
	Must match PRIMARY_PROVIDER_DUNS

	ROLLOVER_WAIVED
	If the Transmission Customer does not wish a Long-Term Firm Point-to-Point Transmission Service request to be evaluated for conveyance of rollover rights, the Transmission Customer  must submit the request with the ROLLOVER_WAIVED data element set to ‘Y’ in the transrequest  template.

	PRECONFIRMED
	Must be ‘YES’ for a Coordinated Request associated with a Coordinated Group

	CG_FLAG
	If the Transmission Customer wishes this request to be a Coordinated Request associated with a Coordinated Group, the Transmission Customer must submit the request with the CG_FLAG data element set to ‘Y’ in the transrequest template.


Note: Data Elements are listed on basis of importance, which may be different from the order required in the template.

Additional requirements related to the specification of service points, attributes, pricing, and timing are subject to the Transmission Provider’s Business Practices.

The Transmission Customer may submit a time varying profile of capacity as allowed by the Transmission Provider’s Business Practice by repeating the template Data Elements of BID_PRICE, CAPACITY_REQUESTED, START_TIME and STOP_TIME in template continuation records.  The segments of any submitted profile must not overlap in time.
Once submitted by the Transmission Customer the earliest START_TIME and latest STOP_TIME defines the overall term of service.  Any further negotiation or offer of Partial Service may modify the segments/profiles times, but any such modifications must span the entire period from the original earliest START_TIME to latest STOP_TIME.

If the Transmission Customer does not wish a long-term firm PTP Transmission Service request to be evaluated for conveyance of rollover rights, the Transmission Customer must submit the request with the ROLLOVER_WAIVED data element set to Y in the transrequest  template.

If the Transmission Customer wishes this request to be a Coordinated Request associated with a Coordinated Group, the Transmission Customer must submit the request with the CG_FLAG data element set to ‘Y” in the transrequest template.
013-2.6.1.1
Offering of Partial Service

If in the evaluation of a Transmission Service request, the Transmission Provider determines that only a portion of the Transmission Customer's requested capacity (CAPACITY_REQUESTED Data Element) can be accommodated and that the Transmission Provider is obligated or elects to offer the Transmission Customer only a portion of the requested capacity, the Transmission Provider shall set the values for START_TIME, STOP_TIME, CAPACITY_GRANTED, and OFFER_PRICE as appropriate, and set the request STATUS to COUNTEROFFER or CR_COUNTEROFFER. 

If the CAPACITY_REQUESTED and/or CAPACITY_GRANTED are not constant over time, continuation records shall be used to convey the time varying profile of MW capacity associated with the Transmission Service request.   The profile of CAPACITY_GRANTED and OFFER_PRICE must span the entire START_TIME to STOP_TIME interval initially requested by the Transmission Customer even if CAPACITY_GRANTED is zero.

The Transmission Customer shall recognize the offer of Partial Service by CAPACITY_GRANTED not being equal to CAPACITY_REQUESTED and the request STATUS of COUNTEROFFER or CR_COUNTEROFFER. The Transmission Customer may elect to set the request STATUS to CONFIRM, WITHDRAW, or REBID for the Partial Service using the transcust template.   To rebid for Partial Service the Transmission Customer shall specify the revised START_TIME, STOP_TIME, CAPACITY_REQUESTED and BID_PRICE values and set the request STATUS to REBID using the transcust template.  OASIS shall restrict CAPACITY_REQUESTED on a rebid to not exceed the Seller’s most recent CAPACITY_GRANTED over time.

If the Transmission Service reservation request was marked preconfirmed by the Transmission Customer and an offer of Partial Service is extended, the reservation request must be explicitly confirmed by the Transmission Customer.  The OASIS Node shall not automatically confirm a request where CAPACITY_REQUESTED and/or BID_PRICE does not equal CAPACITY_GRANTED and/or OFFER_PRICE over time when/if an attempt is made to set STATUS to ACCEPTED or CR_ACCEPTED.

The Transmission Provider shall use this same process in handling the deferral to the start of Transmission Service due to delays in completing the necessary transmission system studies associated with the request.  In these cases, the Transmission Provider shall document the deferral by setting an initial Transmission Service profile record with CAPACITY set to zero MWs and START_TIME set to the originally requested START_TIME and STOP_TIME coincident with the delayed start of service.  The Transmission Provider shall then specify the capacity to be made available to the Transmission Customer in one or more subsequent Transmission Service profile continuation records by defining START_TIME, STOP_TIME, CAPACITY_GRANTED and OFFER_PRICE as appropriate.

Note that OASIS must verify that the values for CAPACITY_REQUESTED and BID_PRICE match the values for CAPACITY_GRANTED and OFFER_PRICE over each START_TIME/STOP_TIME interval, and block any attempt to set request status to ACCEPTED, CR_ACCEPTED, or CONFIRMED if these Data Elements are not equal.  This ensures that both parties to the transaction agree to the final term and price of service.
013-2.6.1.2
Negotiation of Price

Negotiation of price is initiated by the Transmission Customer submitting a service request (via transrequest/ancrequest templates) with a BID_PRICE that is different (higher or lower) from the currently posted offer price, or the tariff rate, for that service.  The following negotiation process is required where the Seller is the Transmission Provider.  Resales or Transfers between Transmission Customers may use this process, but there is no obligation on the (Re)Seller to offer a negotiated rate to other Transmission Customers.

If the Seller determines that the BID_PRICE is acceptable, the following actions must be taken (via transsell/ancsell  templates):

· Update the currently posted offer price for the service requested and all other applicable services offered as dictated by current discounting policy (e.g., all unconstrained paths to the same POD) to match BID_PRICE;

· Update the request’s NEGOTIATED_PRICE_FLAG to L or H if the BID_PRICE was lower than or higher than, respectively, the posted price when the request was submitted;

· Set the OFFER_PRICE equal to the BID_PRICE;

· Set the CAPACITY_GRANTED appropriately (if left null or undefined, OASIS shall set CAPACITY_GRANTED equal to CAPACITY_REQUESTED when STATUS is set to ACCEPTED or CR_ACCEPTED);

· Set the request STATUS to ACCEPTED or CR_ACCEPTED (or COUNTEROFFER or CR_COUNTEROFFER if offering Partial Service)

The Transmission Customer may then confirm the purchase or withdraw the request by updating the request STATUS (via transcust/anccust templates).

If the Seller determines that the BID_PRICE is unacceptable, and negotiation of price is not going to be entertained, the Seller shall set the request STATUS to DECLINED (via transsell/ancsell templates):

If the Seller elects to enter into price negotiation, the following actions must be taken (via transsell/ancsell templates):

· If the price to be counter offered by the Transmission Provider to the Transmission Customer is different than the currently posted offer price:

· Update the currently posted offer price for the service requested and all other applicable services offered as dictated by current discounting policy (e.g., all unconstrained paths to the same POD) to match the price to be counteroffered;

· Update the request’s NEGOTIATED_PRICE_FLAG to L or H if the price to be counter offered is lower than or higher than, respectively, the posted price when the request was submitted;

· Set the OFFER_PRICE and CAPACITY_GRANTED appropriately;

· Set the request STATUS to COUNTEROFFER or CR_COUNTEROFFER.

The Transmission Customer may then confirm the purchase, withdraw the request, or propose a new BID_PRICE by performing the following (via transcust/anccust templates):

· Update the request BID_PRICE appropriately;

· Set the request STATUS to REBID or CONFIRMED.

If the Transmission Customer has set the status to REBID, the Seller may then act on the new BID_PRICE and/or CAPACITY_REQUESTED over time by declining the request, accepting the BID_PRICE/CAPACITY_REQUESTED, or counter offering a new OFFER_PRICE and/or CAPACITY_GRANTED over time using the same sequence of actions as stated above in Business Practice Standard WEQ-013-2.6.1.2.

Negotiation of price may also be initiated on receipt of a request for similar service submitted with a higher BID_PRICE.  If required by Business Practice, the Seller (Transmission Provider) may update any ACCEPTED or CR_ACCEPTED but unconfirmed requests to COUNTEROFFER or CR_COUNTEROFFER with the associated OFFER_PRICE set to meet the higher received BID_PRICE, and the negotiation of price can proceed as described above in Business Practice Standard WEQ-013-2.6.1.2.

Note that OASIS must verify that the values for CAPACITY_REQUESTED and BID_PRICE match the values for CAPACITY_GRANTED and OFFER_PRICE over each START_TIME/STOP_TIME interval, and block any attempt to set request status to ACCEPTED, CR_ACCEPTED, or CONFIRMED if these Data Elements are not equal.  This ensures that both parties to the transaction agree to the final term and price of service.
013-2.6.2
RENEWAL Requests
Transmission Customers shall use the REQUEST_TYPE of RENEWAL only to exercise rollover rights.  renewal requests must always specify the Transmission Provider as Seller.

The following are specific restrictions or requirements for OASIS Transmission Service requests with REQUEST_TYPE of RENEWAL.

	Data Element
	Restriction/Requirement

	REQUEST_TYPE
	Must be RENEWAL

	RELATED_REF
	Must specify an ASSIGNMENT_REF that is associated with an existing confirmed firm Transmission Service reservation or a pending Redirect on a firm basis held by the Transmission Customer that 1) has rollover rights or may be conveyed rollover rights, and 2) has not had the deadline for the exercise of rollover rights pass. 

	SELLER_CODE
	Must match PRIMARY_PROVIDER_CODE

	SELLER_DUNS
	Must match PRIMARY_PROVIDER_DUNS

	PATH
	Must represent the same corresponding service points in the reservation/request specified in RELATED_REF

	POINT_OF_RECEIPT
	

	POINT_OF_DELIVERY
	

	SOURCE
	

	SINK
	

	SERVICE_INCREMENT
	Must specify a set of valid Transmission Service attributes recognized by the Transmission Provider as a valid service designation eligible for exercising of rollover rights held by the reservation/request specified in RELATED_REF

	TS_CLASS
	

	TS_TYPE
	

	TS_PERIOD
	

	TS_WINDOW
	

	TS_SUBCLASS
	

	START_TIME
	Must match the STOP_TIME of the reservation/request specified in RELATED_REF 

	STOP_TIME
	With START_TIME, must specify a valid interval of service eligible for exercising of renewal/rollover rights held by the reservation/request specified in RELATED_REF

	CAPACITY_REQUESTED
	Must be less than or equal to the amount of capacity eligible for renewal/rollover over the interval of service

	BID_PRICE
	Must specify the price to be paid for the service requested

	ROLLOVER_WAIVED
	If the Transmission Customer does not wish a Long-Term Firm Point-to-Point Transmission Service request to be evaluated for conveyance of rollover rights, the Transmission Customer  must submit the request with the ROLLOVER_WAIVED data element set to ‘Y’ in the transrequest  template.

	PRECONFIRMED
	Must be ‘YES’ for a Coordinated Request associated with a Coordinated Group

	CG_FLAG
	If the Transmission Customer wishes this request to be a Coordinated Request associated with a Coordinated Group, the Transmission Customer must submit the request with the CG_FLAG data element set to ‘Y’ in the transrequest template.


Note: Data Elements are listed on basis of importance, which may be different from the order required in the template.

Renewal requests must be submitted on OASIS prior to the dedline for exercising rollover rights as established by the Transmission Provider’s tariff or Business Practice.

CUSTOMER_CODE and CUSTOMER_DUNS in the renewal request should correspond to the CUSTOMER_CODE and CUSTOMER_DUNS in the RELATED_REF reservation.  If not, the Transmission Provider should verify that the submitting Transmission Customer has a valid agency agreement with the original  service agreement holder and is authorized to submit such a request on behalf of that entity.

The Transmission Service attributes, e.g., TS_CLASS, etc., should match the corresponding attributes in the reservation specified in RELATED_REF.  However, changes may be made to these attributes over time such that some differences are necessary to accommodate changes in the Transmission Provider’s Business Practices.  This also applies to changes in service points, (e.g., PATH, etc., over time).

Renewal requests may be subject to offering of Partial Service as a result of competition and/or failure to exercise a right of first refusal.

If the Transmission Customer does not wish a long-term firm PTP Transmission Service request to be evaluated for conveyance of rollover rights, the Transmission Customer  must submit the request with the ROLLOVER_WAIVED Data Element set to ‘Y’ in the transrequest  template.
If the Transmission Customer wishes this request to be a Coordinated Request associated with a Coordinated Group, the Transmission Customer must submit the request with the CG_FLAG data element set to ‘Y’ in the transrequest template.
013-2.6.3
MATCHING Requests
Transmission Customers shall use the REQUEST_TYPE of MATCHING to exercise right of first refusal to avoid being displaced by a higher priority competing request.

The following are specific restrictions or requirements for OASIS Transmission Service requests with REQUEST_TYPE of MATCHING.

	Data Element
	Restriction/Requirement

	REQUEST_TYPE
	Must be MATCHING

	RELATED_REF
	Must specify the ASSIGNMENT_REF associated with an existing confirmed Transmission Service reservation held by the Transmission Customer that is subject to displacement. 

	PATH
	Must represent the same corresponding service points in the reservation specified in RELATED_REF

	POINT_OF_RECEIPT
	

	POINT_OF_DELIVERY
	

	SOURCE
	

	SINK
	

	SERVICE_INCREMENT
	Must specify a set of valid Transmission Service attributes that meet the requirements to exercise right of first refusal

	TS_CLASS
	

	TS_TYPE
	

	TS_PERIOD
	

	TS_WINDOW
	

	TS_SUBCLASS
	

	START_TIME
	Must specify the new requested time for the start of Transmission Service

	STOP_TIME
	With START_TIME, must specify a valid interval of service based on the Transmission Service attributes

	CAPACITY_REQUESTED
	Must be equivalent to the amount of capacity required to retain the Transmission Customer’s rights originally held on RELATED_REF

	BID_PRICE
	Must specify the price to be paid for the service requested

	PRECONFIRMED
	Must be ‘YES’ for a Coordinated Request associated with a Coordinated Group

	CG_FLAG
	If the Transmission Customer wishes this request to be a Coordinated Request associated with a Coordinated Group, the Transmission Customer must submit the request with the CG_FLAG data element set to ‘Y’ in the transrequest template.


Note: Data Elements are listed on basis of importance, which may be different from the order required in the template.
If the Transmission Customer wishes this request to be a Coordinated Request associated with a Coordinated Group, the Transmission Customer must submit the request with the CG_FLAG data element set to ‘Y’ in the transrequest template.
013-2.6.5
REDIRECT Requests 

The Redirect request is submitted by an existing firm PTP Transmission Customer to request the use of alternate PORs and/or PODs from the Transmission Provider.  By definition, the Seller in a Redirect request must be the Transmission Provider even if those rights being redirected were acquired from another Transmission Customer via Resale or Transfer.

The following Business Practice Standards WEQ-013-2.6.5.1 through WEQ-013-2.6.5.2 set forth the requirements for submission of Redirect requests on either a firm or non-firm basis.

013-2.6.5.1
Redirect on a Firm Basis
A Transmission Customer holding confirmed firm PTP rights may request the use of those rights on alternate PORs and/or PODs on a firm basis by submission of a Redirect request to the Transmission Provider as Seller.  The following information must be submitted by the Transmission Customer in the Redirect request via the transrequest template.

	Data Element
	Restriction/Requirement

	REQUEST_TYPE
	Must be REDIRECT

	RELATED_REF
	Must identify by ASSIGNMENT_REF a confirmed Transmission Service reservation for firm PTP held by the submitting Transmission Customer

	SELLER_CODE
	Must match PRIMARY_PROVIDER_CODE

	SELLER_DUNS
	Must match PRIMARY_PROVIDER_DUNS

	PATH
	Must represent the new Transmission Service points being requested

	POINT_OF_RECEIPT
	

	POINT_OF_DELIVERY
	

	SOURCE
	

	SINK
	

	SERVICE_INCREMENT
	Must represent a set of valid Transmission Service attributes for firm PTP offered by the Transmission Provider and being requested on the new service points by the Transmission Customer

	TS_CLASS
	

	TS_TYPE
	

	TS_PERIOD
	

	TS_WINDOW
	

	TS_SUBCLASS
	

	START_TIME
	Must specify the requested start of Transmission Service and must be within the bounds of START_TIME specified in the reservation identified in RELATED_REF

	STOP_TIME
	Must specify the requested stop/end of Transmission Service and must be within the bounds of STOP_TIME specified in the reservation identified in RELATED_REF, and with START_TIME must represent a valid interval of service for the firm PTP being requested

	CAPACITY_REQUESTED
	Must specify the amount of transmission capacity being requested

	BID_PRICE
	Should specify the price for the service being requested; may be null

	ROLLOVER_WAIVED
	May specify the desire to waive conveyance of rollover to the Redirect by specifying ‘Y’ at time of submission of the request to Redirect on a Firm basis request (retains rollover rights on the Parent Reservation)

	PRECONFIRMED
	Must be ‘YES’ for a Coordinated Request associated with a Coordinated Group

	CG_FLAG
	If the Transmission Customer wishes this request to be a Coordinated Request associated with a Coordinated Group, the Transmission Customer must submit the request with the CG_FLAG data element set to ‘Y’ in the transrequest template.


Note: Data Elements are listed on basis of importance, which may be different from the order required in the template.

The Transmission Provider shall evaluate each Redirect on a firm basis as any other new request for firm PTP.  Transmission Provider Business Practices establish the requirements for service duration, submission time, evaluation time, confirmation time, etc.

OASIS or Transmission Provider procedures should verify that the Transmission Service reservation identified in RELATED_REF meets all the requirements to support the redirect of Transmission Service rights to the new service points.  This should include the validation that the current rights, the Capacity Available for Redirect, held on that reservation in the amount of the redirect over time have not been encumbered by any other confirmed Redirects, Resales, schedules, CCO conditions, etc.  This capacity validation may occur at any point in the request process, but shall always be performed prior to setting the Redirect STATUS to CONFIRMED.

Once confirmed, the Transmission Service rights held on the RELATED_REF reservation in the amount of the Redirect shall be permanently released by the Transmission Provider and conveyed to the Redirect reservation. The only mechanism for the Transmission Customer to return to the original PORs and/or PODs is to submit another Redirect request.

The impact on ATC for the reservation identified by RELATED_REF shall be released and the impact of the Redirect transaction on ATC shall be accounted for in the amount and over time of the Redirect simultaneously.

The impact of the Redirect transaction on the reservation(s) identified by RELATED_REF shall be posted and viewable using the reduction  template.

OASIS or Transmission Provider procedures shall also apply any outstanding conditions that may exist from the Parent CCO Reservation to the Redirect reservation, if applicable, (e.g., Number-of-Hours Criteria or System-Conditions Criteria) in accordance with Business Practice Standard WEQ-001-21.5.2.2.1 and WEQ-013-2.6.1.4. and any outstanding conditions shall be viewable using the cco template.

If the Redirect is eligible for the conveyance of rollover/renewal rights, these rights shall be communicated through the rollover template Data Elements RENEWAL_DUE_TIME, ROLLOVER_ELIGIBLE, ROLLOVER_START_TIME, ROLLOVER_STOP_TIME, and ROLLOVER_CAPACITY.  Conveyance of rollover rights to the Redirect request/reservation may have an impact on those rights held on the RELATED_REF reservation.  These impacts on the RELATED_REF reservation shall be documented through an update to the rollover template Data Elements ROLLOVER_ELIGIBLE, ROLLOVER_START_TIME, ROLLOVER_STOP_TIME, and ROLLOVER_CAPACITY associated with the RELATED_REF.

If the Transmission Customer does not wish a firm request to Redirect on a firm basis to be evaluated for conveyance of rollover rights, the Transmission Customer  must submit the request with the ROLLOVER_WAIVED Data Element set to ‘Y’ in the transrequest  template.
If the Transmission Customer wishes this request to be a Coordinated Request associated with a Coordinated Group, the Transmission Customer must submit the request with the CG_FLAG data element set to ‘Y’ in the transrequest template.
013-2.6.5.2
Redirect on a Non-Firm Basis

A Transmission Customer holding confirmed firm PTP rights may request the use of those rights on alternate PORs and/or PODs on a non-firm basis by submission of a Redirect request to the Transmission Provider as Seller.  The following information must be submitted by the Transmission Customer in the Redirect request via the transrequest template.

	Data Element
	Restriction/Requirement

	REQUEST_TYPE
	Must be REDIRECT

	RELATED_REF
	Must identify by ASSIGNMENT_REF a confirmed Transmission Service reservation for firm PTP held by the submitting Transmission Customer

	SELLER_CODE
	Must match PRIMARY_PROVIDER_CODE

	SELLER_DUNS
	Must match PRIMARY_PROVIDER_DUNS

	PATH
	Must represent the new Transmission Service points being requested

	POINT_OF_RECEIPT
	

	POINT_OF_DELIVERY
	

	SOURCE
	

	SINK
	

	SERVICE_INCREMENT
	Must be HOURLY

	TS_CLASS
	Must be SECONDARY

	TS_TYPE
	Must be POINT_TO_POINT

	TS_PERIOD
	Must be FULL_PERIOD

	TS_WINDOW
	Must be FIXED

	TS_SUBCLASS
	Must be null

	START_TIME
	Must specify the requested start of Transmission Service and must be within the bounds of BID_START_TIME specified in the reservation identified in RELATED_REF

	STOP_TIME
	Must specify the requested stop/end of Transmission Service and must be within the bounds of BID_STOP_TIME specified in the reservation identified in RELATED_REF, and with BID_START_TIME must represent a valid interval of service for the firm PTP being requested

	CAPACITY_REQUESTED
	Must specify the amount of transmission capacity being requested

	BID_PRICE
	Should specify the price of $0.00

	PRECONFIRMED
	Must be ‘YES’ for both PTP and a Coordinated Request associated with a Coordinated Group

	CG_FLAG
	If the Transmission Customer wishes this request to be a Coordinated Request associated with a Coordinated Group, the Transmission Customer must submit the request with the CG_FLAG data element set to ‘Y’ in the transrequest template.


Note: Data Elements are listed on basis of importance, which may be different from the order required in the template.

Transmission Provider Business Practices establish the requirements for service duration, submission time, evaluation time, confirmation time, etc.

OASIS or Transmission Provider procedures should verify that the Transmission Service reservation identified in RELATED_REF meets all the requirements to support the Redirect of Transmission Service rights to the new service points.  This should include the validation that the current rights, the Capacity Available for Redirect, held on that reservation in the amount of the Redirect over time have not been encumbered by any other confirmed Redirects, Resales, schedules, etc.  This capacity validation may occur at any point in the request process, but shall always be performed prior to setting the Redirect STATUS to CONFIRMED.

Once confirmed, the Transmission Service rights held on the RELATED_REF reservation in the amount of the Redirect shall be removed from the Capacity Available for Redirect on that reservation. Should the Transmission Customer wish to restore the rights to schedule (or Redirect, resell, etc.) under the RELATED_REF reservation, they must submit a Relinquish request to the Transmission Provider to release the capacity held under the Redirect request and reinstate that capacity on the original service points. The Transmission Provider must accommodate this release of capacity back to the original service points, and shall through OASIS or Transmission Provider procedures insure that honoring this release of capacity and subsequent scheduling under the Transmission Customer’s original firm reservation will not unduly cause a reliability problem.

The impact on ATC for the reservation identified by RELATED_REF and the impact of the Redirect transaction on ATC shall be accounted for to both preserve the ability of the Transmission Customer to return to the original firm path and reflect the fact that the redirected capacity on the RELATED_REF and Redirect reservations cannot be scheduled simultaneously.

The impact of the Redirect transaction on the reservation(s) identified by RELATED_REF shall be posted and viewable using the reduction template.
If the Transmission Customer wishes this request to be a Coordinated Request associated with a Coordinated Group, the Transmission Customer must submit the request with the CG_FLAG data element set to ‘Y’ in the transrequest template.
013-2.6.7
Resale Requests
Resale requests may be submitted by the Transmission Customer (Assignee) to arrange for the sale or assignment of scheduling rights from another Transmission Customer (Reseller). Resellers and Assignees are not required to use OASIS to arrange for such sale or assignment of scheduling rights.  However, the Reseller is required to notify the Transmission Provider and post information documenting the (re)sale on the Transmission Provider’s OASIS where the rights were originally acquired.

The following are specific requirements for OASIS service requests with REQUEST_TYPE of RESALE.  If REQUEST_TYPE is not specified by the Transmission Customer and SELLER_CODE and SELLER_DUNS are not the same as PRIMARY_PROVIDER_CODE and PRIMARY_PROVIDER_DUNS, OASIS shall default REQUEST_TYPE to RESALE.

013-2.6.7.1
Resale on OASIS

Resale transactions conducted on OASIS shall adhere to the basic OASIS request processing requirements where the Reseller is identified as the Seller and the Assignee identified as the Transmission Customer.  

The Assignee (Transmission Customer) initiates the Resale of scheduling rights by submitting the following required information on OASIS via the transrequest template.  Data Elements not listed are optional. There shall be no requirement imposed by OASIS that the Reseller post any corresponding offer of Transmission Service for sale on that OASIS.

	Data Element
	Restriction/Requirement

	REQUEST_TYPE
	Must be RESALE

	SELLER_CODE
	Must identify the Reseller (registered entity other than the Transmission Provider)

	SELLER_DUNS
	Must identify the Reseller

	PATH
	Must represent the Transmission Service points being requested

	POINT_OF_RECEIPT
	

	POINT_OF_DELIVERY
	

	SOURCE
	

	SINK
	

	SERVICE_INCREMENT
	Must represent a set of valid Transmission Service attributes offered by the Transmission Provider and being requested from the Reseller

	TS_CLASS
	

	TS_TYPE
	

	TS_PERIOD
	

	TS_WINDOW
	

	TS_SUBCLASS
	

	START_TIME
	Must specify the requested start of Transmission Service 

	STOP_TIME
	Must specify the requested stop/end of Transmission Service 

	CAPACITY_REQUESTED
	Must specify the amount of transmission capacity being requested

	BID_PRICE
	Must specify the price being requested in $/MW-Hour

	PRECONFIRMED
	Must be ‘YES’ for a Coordinated Request associated with a Coordinated Group

	CG_FLAG
	If the Transmission Customer wishes this request to be a Coordinated Request associated with a Coordinated Group, the Transmission Customer must submit the request with the CG_FLAG data element set to ‘Y’ in the transrequest template.


Note: Data Elements are listed on basis of importance, which may be different from the order required in the template.

If the Resale meets all OASIS validation requirements, the Resale request shall be posted on OASIS as a QUEUED Transmission Service request.  It is the Reseller’s obligation to respond to Resale requests submitted to them on OASIS.  Resellers may register for dynamic notification of these events if the Transmission Provider’s OASIS implementation provides for such functionality.

The Reseller may act on the Resale request using any of the processes defined in Business Practice Standards WEQ-013-2.3, WEQ-013-2.6.1.1 and/or WEQ-013-2.6.1.2.  At some point in this process, the Reseller must identify those Transmission Service reservations whose scheduling rights are to be conferred to the Assignee as follows.

	Data Element
	Restriction/Requirement

	REASSIGNED_REF
	Collectively represent those confirmed Transmission Service reservations held by the Reseller whose rights to schedule transmission capacity over time are to be conferred to the Assignee.  Each reservation’s service points and Transmission Service attributes must correspond with those specified in the  Resale request

	REASSIGNED_CAPACITY
	

	REASSIGNED_START_TIME
	

	REASSIGNED_STOP_TIME
	


Note: Data Elements are listed on basis of importance, which may be different from the order required in the template.

Transmission Provider Business Practices may dictate that this information be supplied and validated at the time the Resale request STATUS is set to either ACCEPTED, CR_ACCEPTED, COUNTEROFFER, or CR_COUNTEROFFER.  Transmission Providers Business Practices may also dictate the types of services that may be resold and/or aggregated in support of the Resale as referenced by the REASSIGNED_REF Data Element.

Once the Resale request STATUS is finally set to CONFIRMED by the Assignee, OASIS or Transmission Provider procedures should (again) insure that every reservation represented in the REASSIGNED_REF Data Element meet the following requirements:

· Represent valid confirmed Transmission Service reservations held by the Reseller

· There is sufficient unscheduled or otherwise encumbered (e.g., other Resale, Redirect, etc., transactions)  transmission capacity to meet REASSIGNED_CAPACITY over REASSIGNED_START_TIME to REASSIGNED_STOP_TIME

· Service points, (e.g., PATH, etc.), correspond correctly with those specified in the Resale
· Transmission Service attributes, (e.g., TS_CLASS, etc.), correspond correctly with those specified in the Resale
· The sum of REASSIGNED_CAPACITY over time for all REASSIGNED_REF reservations satisfies the Resale requirements as specified in CAPACITY_GRANTED, START_TIME and STOP_TIME.

· The Assignee has executed a service agreement with the Transmission Provider that covers the terms of the Resale.

If all of these requirements are met, OASIS or Transmission Provider procedures shall convey the specified Transmission Service rights from the Reseller’s reservation to the Assignee’s reservation.  The transmission capacity resold shall not be available to the Reseller to schedule, resell, redirect, or otherwise use.

The impact of the Resale transaction on the reservation(s) identified by REASSIGNED_REF shall be posted and viewable using the reduction template.

OASIS or Transmission Provider procedures shall also apply any outstanding conditions that may exist from the Reseller’s reservation to the Assignee’s reservation (e.g., Number-of-Hours Criteria or System-Conditions Criteria) in accordance with Business Practice Standard WEQ-013-2.6.1.4 and any outstanding conditions shall be viewable using the cco template.

If the Transmission Provider determines the posted Resale does not comply with their Business Practices for such transactions, they shall have the right to reset the Resale STATUS to ANNULLED.  OASIS or Transmission Provider procedures shall then reinstate all reassignments of capacity over time that were designated by the Assignee to the Parent (REASSIGNED_REF) Reservation(s) as appropriate.  The Reseller setting the Resale STATUS to DISPLACED or ANNULLED shall also reinstate all reassignments of capacity over time that were designated by the Assignee to the Parent (REASSIGNED_REF) Reservation(s) as appropriate.
If the Transmission Customer wishes this request to be a Coordinated Request associated with a Coordinated Group, the Transmission Customer must submit the request with the CG_FLAG data element set to ‘Y’ in the transrequest template.
013-4.1.5
File Examples of the Use of Continuation Records
a.
Basic Continuation Records

The first example of the use of Continuation Records is for the transrequest template submitted by a Transmission Customer, “MOP” for purchase of a Transmission Service reservation spanning 16 hours from 06:00 to 22:00 with "ramped" demand at beginning and end of time period. Two additional reservation requests are also submitted in this request prior to and following the profile to demonstrate the handling of ASSIGNMENT_REF by the OASIS Node.  The last request is for a purchase from a Reseller, “EFG”.

The OASIS S&CP identifies which Data Elements may appear in continuation records.  For profiled request of capacity, the Data Elements START_TIME, STOP_TIME, CAPACITY_REQUESTED, and BID_PRICE are repeated in continuation records to define each segment of the profiled requests.  Specification of any values corresponding to COLUMN_HEADERs that are not specified as being allowed in continuation records will be ignored, however commas must be included to properly align the Data Elements associated with each continuation record.

Input:

VERSION=1.5(
TEMPLATE=transrequest(
OUTPUT_FORMAT=DATA(
PRIMARY_PROVIDER_CODE=AAA(
PRIMARY_PROVIDER_DUNS=123456789(
RETURN_TZ=ES(
DATA_ROWS=7(
COLUMN_HEADERS=CONTINUATION_FLAG, SELLER_CODE,  SELLER_DUNS, PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, SOURCE, SINK,CAPACITY_REQUESTED, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD, TS_WINDOW, TS_SUBCLASS, STATUS_NOTIFICATION, START_TIME, STOP_TIME, BID_PRICE, PRECONFIRMED, ANC_SVC_LINK, POSTING_REF, SALE_REF, REQUEST_REF, DEAL_REF, CUSTOMER_COMMENTS,REQUEST_TYPE,RELATED_REF, ROLLOVER_WAIVED,CG_FLAG,CG_CONTIGUITY, CR_PRIMARY_PROVIDER_CODE,CR_ASSIGNMENT_REF,CR_TS_CLASS,CR_INTERVAL,CR_REQUESTED,CR_GRANTED,CR_ACCOMMODATED(
N, AAA,123456789, X/AEF/CEF-ECS//, CEF, ECS,,,35, DAILY, FIRM, POINT_TO_POINT, OFF_PEAK, FIXED,, /AAA/incoming?ref=R765, 20070423000000ES, 20070424000000ES,24.5, NO,,,, R765, D123, Standard daily reservation, ORIGINAL,, N,,,,,,,, (
N, AAA,123456789,, AEF, MPO,,,5, HOURLY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, /AAA/incoming?ref=R766, 20070423060000ES, 20070423070000ES,2.5, YES,,,, R766, D123, First piece of profile spanning 5 records, ORIGINAL,, N,,,,,,,, (
Y,,,,,,,,10,,,,,,,, 20070423070000ES, 20070423080000ES,2.5,,,,,,,,,, N,,,,,,,,(
Y,,,,,,,,15,,,,,,,, 20070423080000ES, 20070423200000ES,2.5,,,,,,,,,, N,,,,,,,,(
Y,,,,,,,,10,,,,,,,, 20070423200000ES, 20070423210000ES,2.5,,,,,,,,,, N,,,,,,,,(
Y,,,,,,,,5,,,,,,,, 20070423210000ES, 20070423220000ES,2.5,,,,,,,,,, N,,,,,,,, (
N, EFG,678912345, X/AEF/CEF-ECS//, CEF, ECS,,,20,DAILY, FIRM, POINT_TO_POINT,FULL_PERIOD, FIXED,, /AAA/incoming?ref=R767, 20070423040000ES, 20070423160000ES,2, YES,,,, R767, D123, Resale hourly reservation after profiled reservation, RESALE,,N,,,,,,,,(
Response:

REQUEST_STATUS=200(
ERROR_MESSAGE=Success (
TIME_STAMP=20070422160523ES (
VERSION=1.5(
TEMPLATE=transrequest(
OUTPUT_FORMAT=DATA(
PRIMARY_PROVIDER_CODE=AAA(
PRIMARY_PROVIDER_DUNS=123456789(
RETURN_TZ=ES(
DATA_ROWS=7(
COLUMN_HEADERS=RECORD_STATUS, CONTINUATION_FLAG, ASSIGNMENT_REF, SELLER_CODE,  SELLER_DUNS, PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, SOURCE, SINK, CAPACITY_REQUESTED, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD, TS_WINDOW, TS_SUBCLASS, STATUS_NOTIFICATION, START_TIME, STOP_TIME, BID_PRICE, PRECONFIRMED, ANC_SVC_LINK, POSTING_REF, SALE_REF, REQUEST_REF, DEAL_REF, CUSTOMER_COMMENTS, REQUEST_TYPE, RELATED_REF, ROLLOVER_WAIVED,CG_FLAG,CG_CONTIGUITY, CR_PRIMARY_PROVIDER_CODE,CR_ASSIGNMENT_REF,CR_TS_CLASS,CR_INTERVAL,CR_REQUESTED,CR_GRANTED,CR_ACCOMMODATED, ERROR_MESSAGE(
200,N,23879032, AAA,123456789, X/AEF/CEF-ECS//, CEF, ECS,,,35, DAILY, FIRM, POINT_TO_POINT, OFF_PEAK, FIXED,, /AAA/incoming?ref=R765, 20070423000000ES, 20070424000000ES,24.5, NO,,,, R765, D123, Standard daily reservation, ORIGINAL, ,, N,,,,,,,,No Error(
200,N,23879037, AAA,123456789,, AEF, MPO,,,5, HOURLY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,, /AAA/incoming?ref=R766, 20070423060000ES, 20070423070000ES,2.5, YES,,,, R766, D123, First piece of profile spanning 5 records, ORIGINAL, ,, N,,,,,,,, No Error(
200,Y,23879037,,,,,,,,10,,,,,,,, 20070423070000ES, 20070423080000ES,2.5,,,,,,,,, ,, N,,,,,,,,No Error(
200,Y,23879037,,,,,,,,15,,,,,,,, 20070423080000ES, 20070423200000ES,2.5,,,,,,,,, ,, N,,,,,,,,No Error(
200,Y,23879037,,,,,,,,10,,,,,,,, 20070423200000ES, 20070423210000ES,2.5,,,,,,,,, ,, N,,,,,,,,No Error(
200,Y,23879037,,,,,,,,5,,,,,,,, 20070423210000ES, 20070423220000ES,2.5,,,,,,,,,,, N,,,,,,,,No Error(
200,N,23879040, EFG,678912345, X/AEF/CEF-ECS//, CEF, ECS,,,20,DAILY, FIRM, POINT_TO_POINT,FULL_PERIOD, FIXED,, /AAA/incoming?ref=R767, 20070423040000ES, 20070423160000ES,2, YES,,,, R767, D123, Resale hourly reservation after profiled reservation, RESALE, ,N,,,,,,,,, No Error(
d.
Query of Transmission Service Reservation Status:

The following is a hypothetical response to a transstatus query that might be delivered for reservations starting on April 23, 2007. 
Input:

<appropriate query name/value pairs to return reservations for April 23, 2007>

Response:

REQUEST_STATUS=200(
ERROR_MESSAGE=No error. (
TIME_STAMP=20000423160523ES(
VERSION=1.5(
TEMPLATE=transstatus(
OUTPUT_FORMAT=DATA(
PRIMARY_PROVIDER_CODE=AAA(
PRIMARY_PROVIDER_DUNS=123456789(
RETURN_TZ=ES(
DATA_ROWS=10(
COLUMN_HEADERS=CONTINUATION_FLAG, ASSIGNMENT_REF, SELLER_CODE, SELLER_DUNS, CUSTOMER_CODE, CUSTOMER_DUNS, AFFILIATE_FLAG, PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, SOURCE, SINK,CAPACITY_REQUESTED,CAPACITY_GRANTED, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD,TS_WINDOW, TS_SUBCLASS, NERC_CURTAILMENT_PRIORITY, OTHER_CURTAILMENT_PRIORITY, START_TIME, STOP_TIME, CEILING_PRICE, OFFER_PRICE, BID_PRICE, PRICE_UNITS, PRECONFIRMED, ANC_SVC_LINK, ANC_SVC_REQ, POSTING_REF, SALE_REF, REQUEST_REF, DEAL_REF, IMPACTED, COMPETING_REQUEST_FLAG, REQUEST_TYPE, RELATED_REF, NEGOTIATED_PRICE_FLAG, STATUS, STATUS_NOTIFICATION, STATUS_COMMENTS, TIME_QUEUED, RESPONSE_TIME_LIMIT, TIME_OF_LAST_UPDATE, PRIMARY_PROVIDER_COMMENTS, SELLER_REF, SELLER_COMMENTS, CUSTOMER_COMMENTS, SELLER_NAME, SELLER_PHONE, SELLER_FAX, SELLER_EMAIL, CUSTOMER_NAME, CUSTOMER_PHONE, CUSTOMER_FAX, CUSTOMER_EMAIL, REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME , REASSIGNED_STOP_TIME, PRIMARY_PROVIDER_APPROVAL, PRIMARY_PROVIDER_PROVISIONS, ROLLOVER_WAIVED,CG_FLAG, CG_CONTIGUITY, CR_PRIMARY_PROVIDER_CODE,CR_ASSIGNMENT_REF,CR_TS_CLASS,CR_INTERVAL ,CR_REQUESTED,CR_GRANTED,CR_ACCOMMODATED (
N,8207, RSELLR,234567890, ACSTMR,987654321, N, , CE, VP, ,,10,10, HOURLY, FIRM, POINT_TO_POINT, OFF_PEAK, FIXED,,7,, 20070423000000ES, 20070423060000ES,,2,2, $/MW-Hour, NO, SC:(AEP:AR:121);RV:(AEP:AR:122);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, SC:M;RV:M;RF:U;EI:U;SP:U;SU:U;, , S1235, , ,0,, RESALE,, L, CONFIRMED,, , 20070422121354ES,, 20070422123054ES, Transmission Provider Comments go here,, Seller comments go here, Transmission Customer comments go here, Joe Smith, (777)-312-7456, (777)-312-7450, jsmith@xyz.com, John Dealer, (534)-223-4567,,,7019,10, 20070423000000ES, 20070423060000ES,,, N,,,,,,,, (
Y,8207,,,,,,,,,,,10,10,,,,,,,,, 20070423220070ES, 20070424000000ES,,2,2,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,7019,10, 20070423220070ES, 20070424000000ES,,, N,,,,,,,, (
N,8234, AAA,123456789, CUSTMR,345678912, N, , CE, MECS, ,,35,35, DAILY, FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,,7,, 20070423000000ES, 20070424000000ES,42,24.5,24.5, $/MW-Day, NO, SC:(AEP:AR:123);RV:(AEP:AR:124);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, SC:M;RV:M;RF:U;EI:U;SP:U;SU:U;, P0123 , S123, R765, D123,0,, ORIGINAL,, L, CONFIRMED, pub/AAA/incoming,, 20070422131354ES, 20070422173354ES, 20070422133354ES, Standard daily reservation,, System Operator, Transmission Customer comments go here, Frank Orth, (999)-123-4567, (888)-123-1231, jsmith@xyz.com, Jane Doe, (999)-123-4567, (999)-123-8823,,,,,,,, N,,,,,,,, (
N,8235, AAA,123456789, CUSTMR,345678912, N, , CE, AMPO, ,,5,5, HOURLY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,,2,, 20070423060000ES, 20070423070000ES,2,2,2, $/MW-Hour, NO, SC:(AEP:AR:125);RV:(AEP:AR:126);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, SC:M;RV:M;RF:U;EI:U;SP:U;SU:U;, P0123 , S123, R765, D123,0,, ORIGINAL,,, CONFIRMED, pub/AAA/incoming,, 20070422160523ES, 20070422171203ES, 20070422170523ES, Profile verified,, First piece, Transmission Customer comments go here, System Operator, (888)-123-4567, (888)-123-1231, jsmith@xyz.com, Jane Doe, (999)-123-4567, (999)-123-8823,,,,,,,, N,,,,,,,, (
Y,8235,,,,,,,,,,,10,10,,,,,,,,, 20070423070000ES, 20070423080000ES,2.5,2.5,2.5,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, N,,,,,,,, (
Y,8235,,,,,,,,,,,15,15,,,,,,,,, 20070423080000ES, 20070423200700ES,2.5,2.5,2.5,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, N,,,,,,,, (
Y,8235,,,,,,,,,,,10,10,,,,,,,,, 20070423200700ES, 20070423210000ES,2.5,2.5,2.5,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, N,,,,,,,, (
Y,8235,,,,,,,,,,,5,5,,,,,,,,, 20070423210000ES, 20070423220070ES,2.5,2.5,2.5,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, N,,,,,,,, (
N,8242, AAA,123456789, CUSTMR,345678912, N, , CE, VP, ,,20,20,WEEKLY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED,,4,, 20070423000000ES, 20070430000000ES,123,123,123,$/MW-Week, NO, SC:(AEP:AR:127);RV:(AEP:AR:128);RF:(FT);EI:(FT);SP:(FT);SU:(FT);, SC:M;RV:M;RF:U;EI:U;SP:U;SU:U;, P0123 , S123, R765, D123,0,, ORIGINAL,,, CONFIRMED, pub/AAA/incoming,, 20070422160723ES, 20070422223024ES, 20070422171523ES, Bid price refused,, Negotiated OFFER_PRICE accepted,, Joe Smith, (888)-123-4567, (888)-123-1231, jsmith@xyz.com, Jane Doe, (999)-123-4567, (999)-123-8823,,,,,,,, N,,,,,,,, (
Y,8242,,,,,,,,,,,18,18,,,,,,,,, 20070430000000ES, 20070507000000ES,123,123,123,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, N,,,,,,,, (
013-4.1.6
Examples of Negotiation of Price and Partial Service Offer
013-4.1.6.1
Negotiation with Preconfirmation

a.
The Transmission Customer submits a preconfirmed Transmission Service request using the transrequest template. Initially, the STATUS is set to QUEUED by the OASIS Node. 

b.
The Seller has the option of setting STATUS via the transsell template to one of the following: INVALID, RECEIVED, STUDY, COUNTEROFFER, CR_COUNTEROFFER, ACCEPTED, CR_ACCEPTED, DECLINED, or REFUSED.
c.
The Seller has the option of entering a CAPACITY_GRANTED and setting the STATUS to COUNTEROFFER or CR_COUNTEROFFER via the transell template if the Seller can only provide Partial Service.
d.
If the Seller sets STATUS to ACCEPTED or CR_ACCEPTED (and, as required by Business Practice Standard WEQ-013-2.2, the OASIS Node forces the Seller to set OFFER_PRICE equal to BID_PRICE as a condition to setting STATUS to ACCEPTED or CR_ACCEPTED) and CAPACITY_GRANTED is equal to CAPACITY_REQUESTED, the OASIS Node will immediately set STATUS to CONFIRMED, except where the STATUS is CR_ACCEPTED the OASIS Node shall wait to set the transaction’s STATUS to CONFIRMED. (Business Practice Standard WEQ-013-2.2 requires the OASIS Node to set a null CAPACITY_GRANTED equal to CAPACITY_REQUESTED when STATUS is set to ACCEPTED or CR_ACCEPTED.)
e.
The Transmission Customer may withdraw request via transcust template at any time up to point where the Seller sets STATUS to ACCEPTED or CR_ACCEPTED. 
f.
Once the STATUS is CONFIRMED, the OFFER_PRICE and CAPACITY_GRANTED officially becomes the terms of the reservation.
013-4.1.6.2
Negotiations without Preconfirmation

a.
The Transmission Customer submits a Transmission Service reservation request with the BID_PRICE less than the CEILING_PRICE via the transrequest template. Initially the STATUS is set to QUEUED by the OASIS Node.

b.
The Seller has the option of setting the STATUS via the transsell template to one of the following: INVALID, RECEIVED, STUDY, ACCEPTED, DECLINED, COUNTEROFFER, or REFUSED. If the STATUS is set to INVALID (due to invalid entries in the request), DECLINED (due to the Seller determining that the proposed price is not acceptable and further negotiations are not desired), or REFUSED (due to the unavailability of the requested service), the Transmission Service reservation request is terminated.
c.
The Seller has the option of entering a CAPACITY_GRANTED and setting the STATUS to COUNTEROFFER via the transell template if the Seller can only provide Partial Service.
d.
If the Seller set the STATUS to RECEIVED or STUDY, and determines that the BID_PRICE is too low, the Seller sets the OFFER_PRICE to the price desired, and sets the STATUS to COUNTEROFFER via the transsell template. 
e.
The Transmission Customer agrees to the OFFER_PRICE, sets the BID_PRICE equal to the OFFER_PRICE, and sets the STATUS to CONFIRMED via the transcust template.
f.
The OFFER_PRICE and CAPACITY_GRANTED with the STATUS of CONFIRMED locks in the terms of the reservation.
013-4.1.6.3
Multiple Step Negotiations

a.
The Transmission Customer submits a Transmission Service reservation request with the BID_PRICE less than the CEILING_PRICE via the transrequest template. Initially the STATUS is set to QUEUED by the OASIS Node.

b.
The Seller has the option of setting the STATUS via the transsell template to one of the following: INVALID, RECEIVED, STUDY, ACCEPTED, CR_ACCEPTED, DECLINED, COUNTEROFFER, CR_COUNTEROFFER, or REFUSED. If the STATUS is set to INVALID, DECLINED, or REFUSED, the Transmission Service reservation request is terminated.
c.
The Seller has the option of entering a CAPACITY_GRANTED and setting the STATUS to COUNTEROFFER or CR_COUNTEROFFER via the transell template if the Seller can only provide Partial Service. If ATC changes before the request reaches the STATUS of CONFIRMED, Seller may change the CAPACITY_GRANTED.
d.
The Seller determines that the BID_PRICE is too low, sets the OFFER_PRICE to the desired value, and sets the STATUS to COUNTEROFFER or CR_COUNTEROFFER via the transsell template. 
e.
The Transmission Customer responds to the new OFFER_PRICE with an updated BID_PRICE and sets the STATUS to REBID for re-evaluation by the Seller.
f.
The Seller determines that the BID_PRICE now is acceptable, and sets the STATUS to ACCEPTED or CR_ACCEPTED via the transsell template. The transition to ACCEPTED or CR_ACCEPTED state requires the OFFER_PRICE to be set to the BID_PRICE: accepting a reservation with an OFFER_PRICE different from BID_PRICE would require the STATUS be set to COUNTEROFFER or CR_COUNTEROFFER rather than ACCEPTED or CR_ACCEPTED (see Business Practice Standard WEQ-013-4.1.6.3.c).
g.
The Transmission Customer agrees to the OFFER_PRICE and sets the STATUS to CONFIRM via the transcust template.
h.
The OFFER_PRICE and CAPACITY_GRANTED with the STATUS as CONFIRMED locks in the terms of the reservation.
013-4.1.6.4
Negotiations Declined by Seller

a.
The Transmission Customer submits a Transmission Service reservation request with the BID_PRICE less than the CEILING_PRICE via the transrequest template. Initially the STATUS is set to QUEUED by the OASIS Node.

b.
The Seller has the option of setting the STATUS via the transsell template to one of the following: INVALID, RECEIVED, STUDY, ACCEPTED, CR_ACCEPTED, DECLINED, COUNTEROFFER, CR_COUNTEROFFER, or REFUSED. If the STATUS is set to INVALID, DECLINED, or REFUSED, the Transmission Service reservation request is terminated.

c.
The Seller determines that the BID_PRICE is too low, sets OFFER_PRICE to his desired value, and sets STATUS to COUNTEROFFER or CR_COUNTEROFFER via the transsell template. 

d.
The Transmission Customer responds to OFFER_PRICE with updated BID_PRICE and sets the STATUS to REBID via the transcust template for re-evaluation by Seller.

e.
The Seller breaks off all further negotiations by setting the STATUS to DECLINED, indicating that the price is unacceptable and that he does not wish to continue negotiations.
013-4.1.6.5
Negotiations Withdrawn by Transmission Customer

a.
The Transmission Customer submits a Transmission Service reservation request with the BID_PRICE less than the CEILING_PRICE via the transrequest. Initially the STATUS is set to QUEUED by the OASIS Node.

b.
The Seller has the option of setting the STATUS via the transsell template to one of the following: INVALID, RECEIVED, STUDY, ACCEPTED, CR_ACCEPTED, DECLINED, COUNTEROFFER, CR_COUNTEROFFER, or REFUSED. If the STATUS is set to INVALID, DECLINED, or REFUSED, the Transmission Service reservation request is terminated.

c.
The Seller has the option of entering a CAPACITY_GRANTED and setting the STATUS to COUNTEROFFER or CR_COUNTEROFFER via the transell template if the Seller can only provide Partial Service.

d.
The Seller determines that the BID_PRICE is too low, sets the OFFER_PRICE to his desired value, and sets the STATUS to COUNTEROFFER or CR_COUNTEROFFER via the transsell template.

e.
The Transmission Customer responds to the OFFER_PRICE with an updated BID_PRICE and sets the STATUS to REBID for re-evaluation by Seller.

f.
The Seller determines that the BID_PRICE is still too low, sets the OFFER_PRICE to another value, and sets STATUS to COUNTEROFFER or CR_COUNTEROFFER via the transsell template.

g.
The Transmission Customer breaks off all further negotiations, either because the OFFER_PRICE or CAPACITY_GRANTED are unacceptable, by setting STATUS to WITHDRAWN (or the Transmission Customer/Seller could go through additional iterations of REBID/COUNTEROFFER/CR_COUNTEROFFER until negotiations are broken off or the reservation is confirmed).
013-4.1.6.6
Negotiations Superseded by Higher Priority Reservation

a.
The Transmission Customer submits a Transmission Service reservation request with the BID_PRICE less than the CEILING_PRICE via the transrequest template. Initially the STATUS is set to QUEUED by the OASIS Node.

b.
The Seller has the option of setting the STATUS via the transsell template to one of the following: INVALID, RECEIVED, STUDY, ACCEPTED, CR_ACCEPTED, DECLINED, COUNTEROFFER, CR_COUNTEROFFER, or REFUSED. If the STATUS is set to INVALID, DECLINED, or REFUSED, the Transmission Service reservation request is terminated.

c.
If the Seller determines that another reservation has higher priority and must displace this request, he sets the STATUS of this request to SUPERSEDED and the negotiations are terminated.

d.
However, if desired and permitted by the tariff, the Seller may set the STATUS of a request in any of these previous states (including COUNTEROFFER, CR_COUNTEROFFER, ACCEPTED, and CR_ACCEPTED) to COUNTEROFFER or CR_COUNTEROFFER with an OFFER_PRICE which could avoid the request being superseded, thus allowing the Transmission Customer the choice of being SUPERSEDED or accepting the proposed OFFER_PRICE.
013-4.1.7.2
Reservations

The following is an example of a hypothetical audit query for a specific Transmission Service reservation (line breaks and indentations added to improve readability):

Response:

REQUEST_STATUS=200(
ERROR_MESSAGE=(
TIME_STAMP=19980821092048ES(
VERSION=1.5(
TEMPLATE=transstatusaudit(
OUTPUT_FORMAT=DATA(
PRIMARY_PROVIDER_CODE=WXYZ(
PRIMARY_PROVIDER_DUNS=78912345(
RETURN_TZ=ES(
DATA_ROWS=15(
COLUMN_HEADERS=RECORD_TYPE, TIME_OF_UPDATE, MODIFYING_COMPANY_CODE, MODIFYING_NAME, CONTINUATION_FLAG, ASSIGNMENT_REF, SELLER_CODE, SELLER_DUNS, CUSTOMER_CODE, CUSTOMER_DUNS, AFFILIATE_FLAG, PATH_NAME, POINT_OF_RECEIPT, POINT_OF_DELIVERY, SOURCE, SINK, CAPACITY_REQUESTED, CAPACITY_GRANTED, SERVICE_INCREMENT, TS_CLASS, TS_TYPE, TS_PERIOD, TS_WINDOW, TS_SUBCLASS, NERC_CURTAILMENT_PRIORITY, OTHER_CURTAILMENT_PRIORITY, START_TIME, STOP_TIME, CEILING_PRICE, OFFER_PRICE, BID_PRICE, PRICE_UNITS, PRECONFIRMED, ANC_SVC_LINK, ANC _SVC_REQ, POSTING_REF, SALE_REF, REQUEST_REF, DEAL_REF, IMPACTED, COMPETING_REQUEST_FLAG, REQUEST_TYPE, RELATED_REF, NEGOTIATED_PRICE_FLAG, STATUS, STATUS_NOTIFICATION, STATUS_COMMENTS, TIME_QUEUED, RESPONSE_TIME_LIMIT, TIME_OF_LAST_UPDATE, PRIMARY_PROVIDER_COMMENTS, SELLER_REF, SELLER_COMMENTS, CUSTOMER_COMMENTS, SELLER_NAME, SELLER_PHONE, SELLER_FAX, SELLER_EMAIL, CUSTOMER_NAME, CUSTOMER_PHONE, CUSTOMER_FAX, CUSTOMER_EMAIL, REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME, REASSIGNED_STOP_TIME, PRIMARY_PROVIDER_APPROVAL, 
PRIMARY_PROVIDER_PROVISIONS, ROLLOVER_WAIVED,CG_FLAG,CG_CONTIGUITY,CR_PRIMARY_PROVIDER_CODE,CR_ASSIGNMENT_REF,CR_TS_CLASS,CR_INTERVAL,CR_REQUESTED,CR_GRANTED,CR_ACCOMMODATED (
U, 20070815131629ES, DEFPM, Alan Trader, N,104392, WXYZ,78912345, DEFPM,912876543, N, X/WXYZ/AAA-DDD//, AAA , DDD, AAA, ZZZ,50,50, DAILY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED, ,3, , 20070817000000ES, 20070818000000ES,102,85,85, $/MW-Day, N, , SC:M;RF:M, , , , ,,, ORIGINAL,, L, CONFIRMED, , , 20070815121510ES, 20070815144100ES, 20070815131629ES, ,, , , Jane Doe, 123-456-7813, 123-456-7801, doej@wxyz.com, Alan Trader, 312-678-9104, 312-678-9100, a .trader@defmarketing.com, , , , ,,, N,,,,,,,, (
U,,,, Y,104392,,,,,,,,,,,75,75,,,,,,,,, 20070818000000ES, 20070819000000ES,,85,85,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, N,,,,,,,, (
U,,,, Y,104392,,,,,,,,,,,100,100,,,,,,,,, 20070819000000ES, 20070820000000ES,,85,85,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, N,,,,,,,, (
U, 20070815125042ES, WXYZ, Jane Doe, N,104392, WXYZ,78912345, DEFPM,912876543, N, X/WXYZ/AAA-DDD//, AAA , DDD, AAA, ZZZ,50,50, DAILY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED, ,3, , 20070817000000ES, 20070818000000ES,102,85,82, $/MW-Day, N, , SC:M;RF:M, , , , ,,, ORIGINAL,, L, COUNTEROFFER, , , 20070815121510ES, 20070815144100 ES, 20070815125042ES, ,, , , Jane Doe, 123-456-7813, 123-456-7801, doej@wxyz.com, Alan Trader, 312-678-9104, 312-678-910 0, a.trader@defmarketing.com, , , , ,,, N,,,,,,,, (
U,,,, Y,104392,,,,,,,,,,,75,75,,,,,,,,, 20070818000000ES, 20070819000000ES,,85,82,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, N,,,,,,,, (
U,,,, Y,104392,,,,,,,,,,,100,100,,,,,,,,, 20070819000000ES, 20070820000000ES,,85,82,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, N,,,,,,,, (
U, 20070815124811ES, DEFPM, Alan Trader, N,104392, WXYZ,78912345, DEFPM,912876543, N, X/WXYZ/AAA-DDD//, AAA , DDD, AAA, ZZZ,50,50, DAILY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED, ,3, , 20070817000000ES, 20070818000000ES,102,90,82, $/MW-Day, N, , SC:M;RF:M, , , , ,,, ORIGINAL,, , REBID, , , 20070815121510ES, 20070815144100ES, 20070 815124811ES, ,, , , Jane Doe, 123-456-7813, 123-456-7801, doej@wxyz.com, Alan Trader, 312-678-9104, 312-678-9100, a.trader @defmarketing.com,,,,,,, N,,,,,,,, (
U,,,, Y,104392,,,,,,,,,,,75,75,,,,,,,,, 20070818000000ES, 20070819000000ES,,90,82,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, N,,,,,,,, (
U,,,, Y,104392,,,,,,,,,,,100,100,,,,,,,,, 20070819000000ES, 20070820000000ES,,90,82,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, N,,,,,,,, (
U, 20070815124100ES, WXYZ, Jane Doe, N,104392, WXYZ,78912345, DEFPM,912876543, N, X/WXYZ/AAA-DDD//, AAA , DDD, AAA, ZZZ,50,50, DAILY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED, ,3, , 20070817000000ES, 20070818000000ES,102,90,80, $/MW-Day, N, , SC:M;RF:M, , , , ,,, ORIGINAL,, , COUNTEROFFER, , , 20070815121510ES, 20070815144100 ES, 20070815124100ES, ,, , , Jane Doe, 123-456-7813, 123-456-7801, doej@wxyz.com, Alan Trader, 312-678-9104, 312-678-910 0, a.trader@defmarketing.com,,,,,,, N,,,,,,,, (
U,,,, Y,104392,,,,,,,,,,,75,75,,,,,,,,, 20070818000000ES, 20070819000000ES,,90,80,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, N,,,,,,,, (
U,,,, Y,104392,,,,,,,,,,,100,100,,,,,,,,, 20070819000000ES, 20070820000000ES,,90,80,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, N,,,,,,,, (
I, 20070815121510ES, DEFPM, Alan Trader, N,104392, WXYZ,78912345, DEFPM,912876543, N, X/WXYZ/AAA-DDD//, AAA , DDD, AAA, ZZZ,50,, DAILY, NON-FIRM, POINT_TO_POINT, FULL_PERIOD, FIXED, ,3, , 20070817000000ES, 20070818000000ES,102,,80, $/MW-Day, N, , SC:M;RF:M, , , , ,,, ORIGINAL,, , QUEUED, , , 20070815121510ES, , 20070815121510ES, ,, , , Company Default, 123-456-7800, 123-456-7801, , Alan Trader, 312-678-9104, 312-678-9100, a.trader@defmarketing.com,,,,,,, N,,,,,,,, (
I,,,, Y,104392,,,,,,,,,,,75,,,,,,,,,, 20070818000000ES, 20070819000000ES,,,80,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, N,,,,,,,, (
I,,,, Y,104392,,,,,,,,,,,100,,,,,,,,,, 20070819000000ES, 20070820000000ES,,,80,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,,, N,,,,,,,, (
First, this example shows the handling of continuation records which conveyed a time varying demand of 50 MW on August 1, 2007, 75 MW on August 18, 2007, and 100 MW on August 19, 2007. This demand profile was initially entered with the original reservation request (transrequest template) at 12:15 on August 15, 2007, by Alan Trader.
As part of the original reservation, Alan Trader attempted to negotiate a price for service of $80./MWday. Jane Doe responded to this request with a counter offer at the non-negotiated rate of $90./mwday at 12:41 on August 15, 2007. The RESPONSE_TIME_LIMIT Data Element has been updated to reflect the time by which the Transmission Customer must confirm service 
At 12:48, Alan Trader attempted to negotiate further for a rate of $82.00 /MWday and the reservation status was set to REBID.  Jane Doe responded at 12:50 with a second counter offer restating a negotiated rate of $85.00/MWday, which Alan Trader finally agreed to at 13:16 on August 15, 2007. The current posted information on OASIS would show this final confirmed reservation.

�









7
Revised 4/15/2011


