NAESB OASIS Standards and Communication Protocol (S&CP), Version 1.6 – WEQ-002

002-5.10
IMPLEMENTATION PLAN

 All template interactions with OASIS under the latest set of ratified NAESB Standards as set forth in the OASIS Standards and Communication Protocols are to use the following version identifier:

VERSION=1.6(
Until adopted as regulation, OASIS nodes shall be required to respond to template queries and uploads in the VERSION=1.54 format for backward compatibility.  

OASIS implementations electing to support the NAESB standards prior to adoption as regulation must respond correctly to template queries and uploads in both the VERSION=1.5 format and VERSION=1.6 format.

Once adopted as regulation, OASIS nodes shall be required to support the VERSION 1.6 template format within six months of such adoption, and shall maintain support for VERSION 1.5 queries and uploads for the full six months implementation period.

Primary Providers may maintain support for VERSION 1.5 queries and uploads as required to meet regulatory data retention requirements or may elect to convert historical data to be compliant with the VERSION 1.6 template for data retention purposes.

002-5.10.1
OASIS DATA ELEMENTS  

The following table identifies the Data Elements 
required for NITS transactions on OASIS associated with existing Transmission Customers
	Template
	Current PTP Data Element
	New Data Element
	Conversion Requirements

	TBD (NITS Aplication)
	
	SELLER_CODE
	

	TBD (NITS Aplication)
	
	SELLER_DUNS
	For each Network Customer which has completed the application process and is a current customer, the TP will populate this data element of the template.  The TP will also populate this data element for any Native Load Customer.

	TBD (NITS Aplication)
	
	Customer Name
	For each Network Customer which has completed the application process and is a current customer, the TP will populate this data element of the template.  The TP will also populate this data element for any Native Load customer.

	TBD (NITS Aplication)
	
	Customer Code
	For each Network Customer which has completed the application process and is a current customer, the TP will populate this data element of the template.  The TP will also populate this data element for any Native Load customer.

	TBD (NITS Aplication)
	
	START_TIME
	For each Network Customer which has completed the application process and is a current customer, the TP will populate this data element of the template.  The TP will also populate this data element for any  Native Load customer. If  a customer for more than 5 years prior to the implementation date START_TIME may be set equal to a date 5 years prior to 
the implemention date. 

	TBD (NITS Aplication)
	
	STOP_TIME
	For each Network Customer which has completed the application process and is a current customer, the TP will populate this data element of the template.  The TP will also populate this data element for any Native Load customer.   For Native Load customers and Network Customers with evergreen NITS agreements, the value for this Data Element shall be set equal to null.

	TBD (NITS Aplication)
	
	Master_Ref
	For each Network Customer which has completed the application process and is a current customer, the TP will populate this data element of the template.  The TP will also populate this data element for any Native Load customer.

	TBD (NITS Customer)
	
	Master_Ref
	For each Network Customer which has completed the application process and is a current customer, the TP will populate this data element of the template.  The TP will also populate this data element for any Native Load customer.

	CustomerDesc 
	
	NITS_Transaction_Ref
	For each Network Customer which has completed the application process and is a current customer, the TP will populate this data element of the template.  The TP will also populate this data element for any Native Load customer.

	CustomerDesc 
	
	Customer Name


	For each Network Customer which has completed the application process and is a current customer, the TP will populate this data element of the template.  The TP will also populate this data element for any Native Load customer.

	CustomerDesc 
	
	Customer Code
	For each Network Customer which has completed the application process and is a current customer, the TP will populate this data element of the template.  The TP will also populate this data element for any Native Load customer.

	CustomerDesc 
	
	REQUEST_TYPE
	For each Network Customer which has completed the application process and is a current customer, the TP will populate this data element with a value “Application”.  The TP will also populate this data element for any Native Load customer with a value of “Application”).

	AgentDesc 
	
	Master_Ref
	For each Network Customer which has completed the application process and has established an active agent, the TP may populate this data element.  

	AgentDesc 
	
	NITS_Transaction_Ref
	For each Network Customer which has completed the application process and has established  an active agent, the TP may populate this data element.  

	AgentDesc 
	
	Customer Name
	For each Network Customer which has completed the application process and has established an active agent, the TP may populate this data element.  

	AgentDesc 
	
	Customer Code
	For each Network Customer which has completed the application process and has established an active agent, the TP may populate this data element.  

	AgentDesc 
	
	Authorized Agent Name
	For each Network Customer which has completed the application process and has established an active agent, the TP may populate this data element.  

	AgentDesc 
	
	Authorized Agent Code
	For each Network Customer which has completed the application process and has established an active agent, the TP may populate this data element.  

	AgentDesc 
	
	Transaction_Start_Time
	For each Network Customer which has completed the application process and has established an active agent, the TP may populate this data element.  

	AgentDesc 
	
	Transaction_Stop_time
	For each Network Customer which has completed the application process and has established an active agent, the TP may populate this data element.  

	AgentDesc 
	
	Authorization_Start_Time
	For each Network Customer which has completed the application process and has established an active agent, the TP may populate this data element.  

	AgentDesc 
	
	Authorization_Stop_Time
	For each Network Customer which has completed the application process and has established an active agent, the TP may populate this data element.  

	LoadDesc 
	
	Master_Ref
	For each Network Customer which has completed the application process and is a current customer, the TP will populate this data element of the template.  The TP will also populate this data element for any Native Load customer.

	LoadDesc 
	
	DNL_REF
	For each Network Customer which has completed the application process and is a current customer, the TP will populate this data element of the template.  The TP will also populate this data element for any Native Load customer.

	LoadDesc 
	
	Customer Name
	For each Network Customer which has completed the application process and is a current customer, the TP will populate this data element of the template.  The TP will also populate this data element for any Native Load customer.

	LoadDesc 
	
	Customer Code
	For each Network Customer which has completed the application process and is a current customer, the TP will populate this data element of the template.  The TP will also populate this data element for any Native Load customer.

	LoadDesc 
	
	SINK
	For each Network Customer which has completed the application process and is a current customer, the TP will verify the validity of  this data element with the TSIN-registry.  The TP will also populate this data element for any Native Load customer with the TSIN-registered sink name..

	LoadDesc 
	
	START_TIME
	For each Network Customer which has completed the application process and is a current customer, the TP will populate this data element of the template.  The TP will also populate this data element for any Native Load customer. If a customer’s load has been established for more than 5 years prior to the implementation date START_TIME may be set equal to a date 5 years prior to the implemention date
. 

	LoadDesc 
	
	STOP_TIME
	For each Network Customer which has completed the application process and is a current customer, the TP will populate this data element of the template.  The TP will also populate this data element for any Native Load customer.   For Native Load customers and Network Customers with evergreen NITS agreements, this value may be set equal to null.

	LoadDesc 
	
	REQUEST_TYPE
	For each Network Customer which has completed the application process and is a current customer, the TP will populate this data element with a value “Application”.  The TP will also populate this data element for any Native Load customer with a value of “Application”)

	TBD (DNR)
	
	Master_Ref
	The TP will populate this data element for each Designated Network Resource that has not terminated
,  (applies to both Network Customers and Native Load Customers)

	TBD (DNR)
	
	DNR_Ref
	The TP will populate this data element for each Designated Network Resource that has not terminated,  (applies to both Network Customers and Native Load Customers)

	TBD (DNR)
	
	Customer Name
	The TP will populate this data element for each Designated Network Resource that has not terminated,  (applies to both Network Customers and Native Load Customers)

	TBD (DNR)
	
	Customer Code
	The TP will populate this data element for each Designated Network Resource that has not terminated,  (applies to both Network Customers and Native Load Customers)

	TBD (DNR)
	
	START_TIME
	The TP will populate this data element for each Designated Network Resource that has not terminated,  (applies to both Network Customers and Native Load Customers)

	TBD (DNR)
	
	STOP_TIME
	The TP will populate this data element for each Designated Network Resource that has not terminated,  (applies to both Network Customers and Native Load Customers)

	TBD (DNR)
	
	System Location
	The TP will populate this data element for each Designated Network Resource that has not terminated,  (applies to both Network Customers and Native Load Customers)

	TBD (DNR)
	
	DNR_Capacity_Requested
	The TP will populate this data element for each Designated Network Resource that has not terminated,  (applies to both Network Customers and Native Load Customers)

	TBD (DNR)
	
	REQUEST_TYPE
	The TP will populate this data element for each Designated Network Resource that has not terminated,  (applies to both Network Customers and Native Load Customers)

	TBD (DNR)
	
	Resource Name
	The TP will populate this data element for each Designated Network Resource that has not terminated,  (applies to both Network Customers and Native Load Customers)

	TBD (DNR)
	
	Generator Capacity
	The TP will populate this data element for each Designated Network Resource that has not terminated,  (applies to both Network Customers and Native Load Customers)

	TBD (DNR)
	
	Delivery point on the TP’s transmission system

	The TP will populate this data element for each Designated Network Resource that has not terminated,  (applies to both Network Customers and Native Load Customers)

	TBD (DNR)
	
	Geographic Location
	The TP will populate this data element for each Designated Network Resource that has not terminated,  (applies to both Network Customers and Native Load Customers)

	TBD (DNR)
	
	Electrical Location
	The TP will populate this data element for each Designated Network Resource that has not terminated,  (applies to both Network Customers and Native Load Customers)

	TBD (DNR)
	
	REQUEST_TYPE
	The TP will populate this data element for each Designated Network Resource that has not terminated,  (applies to both Network Customers and Native Load Customers)

	TBD (NITS NSR)
	
	CONTINUATION_FLAG
	Applies only to a TP that elects to use the NITS NSR template: The TP will populate this data element to account for associated Designated Network Resource(s) that have not terminated in accordance with Business Practices,  (applies to both Network Customers and Native Load Customers)

	TBD (NITS NSR)
	
	Master_Ref
	Applies only to a TP that elects to use the NITS NSR template: The TP will populate this data element to account for associated Designated Network Resource(s) that have not terminated in accordance with Business Practices,  (applies to both Network Customers and Native Load Customers)

	TBD (NITS NSR)
	
	Transmission_Capacity_Ref
	Applies only to a TP that elects to use the NITS NSR template: The TP will populate this data element to account for associated Designated Network Resource(s) that have not terminated in accordance with Business Practices,  (applies to both Network Customers and Native Load Customers)

	TBD (NITS NSR)
	
	Customer Name
	Applies only to a TP that elects to use the NITS NSR template: The TP will populate this data element to account for associated Designated Network Resource(s) that have not terminated in accordance with Business Practices,  (applies to both Network Customers and Native Load Customers)

	TBD (NITS NSR)
	
	Customer Code
	Applies only to a TP that elects to use the NITS NSR template: The TP will populate this data element to account for associated Designated Network Resource(s) that have not terminated in accordance with Business Practices,  (applies to both Network Customers and Native Load Customers)

	TBD (NITS NSR)
	
	PATH_NAME
	Applies only to a TP that elects to use the NITS NSR template: The TP will populate this data element to account for associated Designated Network Resource(s) that have not terminated in accordance with Business Practices,  (applies to both Network Customers and Native Load Customers)

	TBD (NITS NSR)
	
	POINT_OF_RECEIPT
	Applies only to a TP that elects to use the NITS NSR template: The TP will populate this data element to account for associated Designated Network Resource(s) that have not terminated in accordance with Business Practices,  (applies to both Network Customers and Native Load Customers)

	TBD (NITS NSR)
	
	POINT_OF_DELIVERY
	Applies only to a TP that elects to use the NITS NSR template: The TP will populate this data element to account for associated Designated Network Resource(s) that have not terminated in accordance with Business Practices,  (applies to both Network Customers and Native Load Customers)

	TBD (NITS NSR)
	
	SOURCE
	Applies only to a TP that elects to use the NITS NSR template: The TP will populate this data element to account for associated Designated Network Resource(s) that have not terminated in accordance with Business Practices,  (applies to both Network Customers and Native Load Customers)

	TBD (NITS NSR)
	
	SINK
	Applies only to a TP that elects to use the NITS NSR template: The TP will populate this data element to account for associated Designated Network Resource(s) that have not terminated in accordance with Business Practices,  (applies to both Network Customers and Native Load Customers)

	TBD (NITS NSR)
	
	CAPACITY_REQUESTED
	Applies only to a TP that elects to use the NITS NSR template: The TP will populate this data element to account for associated Designated Network Resource(s) that have not terminated in accordance with Business Practices,  (applies to both Network Customers and Native Load Customers)

	TBD (NITS NSR)
	
	SERVICE_INCREMENT
	Applies only to a TP that elects to use the NITS NSR template: The TP will populate this data element to account for associated Designated Network Resource(s) that have not terminated in accordance with Business Practices,  (applies to both Network Customers and Native Load Customers)

	TBD (NITS NSR)
	
	TS_CLASS
	Applies only to a TP that elects to use the NITS NSR template: The TP will populate this data element to account for associated Designated Network Resource(s) that have not terminated in accordance with Business Practices,  (applies to both Network Customers and Native Load Customers)

	TBD (NITS NSR)
	
	TS_TYPE
	Applies only to a TP that elects to use the NITS NSR template: The TP will populate this data element to account for associated Designated Network Resource(s) that have not terminated in accordance with Business Practices,  (applies to both Network Customers and Native Load Customers)

	TBD (NITS NSR)
	
	TS_PERIOD
	Applies only to a TP that elects to use the NITS NSR template: The TP will populate this data element to account for associated Designated Network Resource(s) that have not terminated in accordance with Business Practices,  (applies to both Network Customers and Native Load Customers)

	TBD (NITS NSR)
	
	TS_WINDOW
	Applies only to a TP that elects to use the NITS NSR template: The TP will populate this data element to account for associated Designated Network Resource(s) that have not terminated in accordance with Business Practices,  (applies to both Network Customers and Native Load Customers)

	TBD (NITS NSR)
	
	TS_SUBCLASS
	Applies only to a TP that elects to use the NITS NSR template: The TP will populate this data element to account for associated Designated Network Resource(s) that have not terminated in accordance with Business Practices,  (applies to both Network Customers and Native Load Customers)

	TBD (NITS NSR)
	
	START_TIME
	Applies only to a TP that elects to use the NITS NSR template: The TP will populate this data element to account for associated Designated Network Resource(s) that have not terminated in accordance with Business Practices,  (applies to both Network Customers and Native Load Customers)

	TBD (NITS NSR)
	
	STOP_TIME
	Applies only to a TP that elects to use the NITS NSR template: The TP will populate this data element to account for associated Designated Network Resource(s) that have not terminated in accordance with Business Practices,  (applies to both Network Customers and Native Load Customers)

	TBD (NITS NSR)
	
	SELLER_COMMENTS
	Applies only to a TP that elects to use the NITS NSR template: The TP will populate this data element to account for associated Designated Network Resource(s) that have not terminated in accordance with Business Practices,  (applies to both Network Customers and Native Load Customers)

	TBD (NITS NSR)
	
	CUSTOMER_COMMENTS
	Applies only to a TP that elects to use the NITS NSR template: The TP will populate this data element to account for associated Designated Network Resource(s) that have not terminated in accordance with Business Practices,  (applies to both Network Customers and Native Load Customers)

	TBD (NITS NSR)
	
	REQUEST_TYPE
	Applies only to a TP that elects to use the NITS NSR template: The TP will populate this data element to account for associated Designated Network Resource(s) that have not terminated in accordance with Business Practices,  (applies to both Network Customers and Native Load Customers)

	Also include conversion of Network Secondary reservations to the new template(s) and removal of existing reservations.
	
	
	

	Also remove all reservations/requests with TS_TYPE - Network
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	


�How is this dates selected?


�Same magic number  Q


�Should this only apply to Indefinite Terminations?


�The definitions of POR and POD covers this.





Draft (NITS on OASIS) 08-02-2010

