General Issues:

Southern: 
· Comments are in the body of the standard

OATI: 
OATI Comments in response to request for informal comments on the draft WEQ-001-1nn Business Practice Standards for “NITS on OASIS” – 07/09/2010

Disclaimer: The following comments do not represent the final, formal or complete comments of OATI in regards to the proposed Business Practice Standards, nor are these comments presented with a specific recommendation on how the Standards might be re-crafted to meet the intent of these comments.  The comments are meant to provide additional information for consideration by the Subcommittee as they move these Standards forward to a formal Recommendation.

· Comments are in the body of the standard

SPP: 
· Comments are in the body of the standard

Duke Energy:

General Issues:
In general, these business practice standards lack an overall presentation of the entire process and it is difficult to understand how the pieces work together to allow a network customer to perform all tasks necessary to obtain network service, designate and undesignate network resources, rollover network service, and perform all other OASIS-related tasks associated with being a network customer.  

A root problem is that there is no transition plan either discussed or identified.  Given that new network customers are rather rare, as most load serving entities are already network customers, the transition plan, i.e., populating the OASIS with information about existing DNRs is by far the most important task if NITS on OASIS is to function properly.  Reading these standards, an existing network customer would not even be able to tell if the standards apply because it is not clear from these standards that information about its existing DNRs would be placed into OASIS in the first place.  

It is difficult to review these standards without the accompanying standards for WEQ 002, 003 and 013 (see 001-101.1).  Specifically, it is not clear which data items are to be entered into templates on OASIS, posted on OASIS as attachments, or posted on OASIS as acknowledgement of the off-OASIS or non-public delivery of material to the Transmission Provider (TP).

There are numerous references in the standards to TP-posted business practices.  Without an idea of what might be in such practices it is difficult to put these references into perspective.

Significant Substantive Issues:
1. Various:  It appears that many of the proposed standards are implemented at the TP’s option and that the TP is also given the option to waive OATT requirements.  More than a half-dozen standard contain the following language: 

“(1) meets the minimum requirements specified in the Transmission Provider’s posted business practices; and (2) is consistent with the Transmission Provider’s tariff requirements.”  

Or

“(1) meets the minimum requirements specified in the Transmission Provider’s posted business practices; or (2) is consistent with the Transmission Provider’s tariff requirements.”  

· There seems to be an inconsistency in the use of “and” and “or” in this “validation.”  The difference between the word “and” and the word “or” is rather significant.  An “and” means the customer must meet two standards.  An “or” means that the customer may not have to meet all the OATT requirements.  This raises the issue as to whether the subcommittee is recommending that these standards permit a TP to waive the collection of data required by pro forma OATT by merely posting a business practice.

· Comments are in the body of the standard

· NCEMC: Comments are in the body of the standard

Midwest ISO:

· Comments are in the body of the standard

BC Hydro: 
· Comments are in the body of the standard

NYISO:

The New York Independent System Operator, Inc. (“NYISO”) appreciates the opportunity to offer comments in response to NAESB’s June 21, Request for Informal Comments on the Network Integration Transmission Service (“NITS”) on OASIS Draft WEQ-001 Standards.  The NYISO’s comments explain why the NITS standards should not apply to the NYISO.  The NYISO has no issues at this time with the content of the NITS standards.  

The NYISO employs a “financial” transmission model that differs in many ways from the “physical reservation” model contemplated by the Federal Energy Regulatory Commission (“FERC”).  Most notably, in the NYISO model, transmission customers do not make reservations for NITS or Point-to-Point Transmission Service (“PTP service”).  A number of the differences are set forth in the NYISO’s Commission-approved OATT.  The NYISO has also obtained, or is seeking, various waivers of prior WEQ standards which reflect the differences between the NYISO’s and the pro forma OATT’s transmission model.

 With respect to NITs, the NYISO’s Open-Access Transmission Tariff (“OTT”) includes Commission-approved tariff language that is significantly different from that in the pro forma OATT.  In addition, no NYISO customer has ever elected NITS in the nearly eleven year period since the NYISO commenced operations.  The NYISO believes that this is largely because the NYISO OATT’s “financial version” of NITS is less attractive than the NYISO’s “financial version” of PTP service, which appears to satisfy all customer needs under the NYISO model. 

For all of these reasons, the NYISO does not believe that it should be subject to the NITS standards or required to make the investments necessary to comply with them.  In some instances, the NYISO OATT simply does not include the requirements that the standards address.  For example, the NYISO tariff does not require that the customers be permitted to designate or undesignate Network Resources through OASIS because the NYISO obtained a FERC waiver from that aspect of Order No. 890.  The absence of the standard pro forma designation/undesignation language would appear to automatically exempt the NYISO from compliance with the entirety of WEQ -001-105.2 and related standards.  

 Similarly, the NYISO’s Secondary Network Service tariff provisions are markedly diffrent than their pro forma equivalents.  In general, under the NYISO OATT, secondary service, if it were ever requested, would be treated the same as non-firm PTP service.  There would be no service reservation the NYISO would accommodate all requests, so long as there was no congestion, and would permit customers to upgrade to firm service by agreeing to pay congestion charges.  The NYISO OATT likewise does not include the pro forma provision specifying that secondary service requests will have higher priority than non-firm PTP uses.  It therefore appears to the NYISO that proposed WEQ 001-105.10 should not apply to the NYISO.  

 Furthermore, the NYISO has already obtained exemptions from various other technical WEQ standards that the proposed NITS standards are built upon.  For example, proposed WEQ 001-102.1.1 provides that customers shall submit NITS applications in accordance with the requirements of WEQ-002, WEQ-003, and WEQ-013.  The NYISO, however is currently exempt from all of WEQ-003, is exempt from significant portions of WEQ-002 and WEQ-013, and will soon be seeking a FERC waiver with respect to the remainder of the latter two standards.  Exempting the NYISO from the proposed WEQ standards would therefore be consistent with the NYISO’s waivers from other standards.

The NYISO ultimately intends to seek a FERC waiver from all of the proposed NITS standards.  In the mean time, the NYISO requests that the NITS standards be revised to acknowledge that they were designed for transmission providers that employ physical reservation transmission models and are not intended to require that entities, such as the NYISO, to abandon or modify financial reservation models.  Alternatively, NAESB should include such an acknowledgement in its filing letter to FERC.  

EPSA:

EPSA appreciates the tremendous amount of work that the subcommittee has done to produce this voluminous standard.  It should go a long way to standardizing the use of the OASIS to deal with NITS service.

We have two overall comments as well as a few suggestions for clarification and some editorial comments.

· Comments are in the body of the standard

BPA: 
· Comments are in the body of the standard

GTC: 
· Comments are in the body of the standard

ENTERGY: 
· Comments are in the body of the standard

Additions to WEQ-000-1 (Acronym)

NSR – Network Scheduling Right
Additions to WEQ-000-2 (Glossary)
Application – Shall have the same meaning as defined in the FERC Pro Forma OATT.  

Network Customer –Shall have the same meaning as defined in the FERC Pro Forma OATT. 

Network Resource – Shall have the same meaning as defined in the FERC Pro Forma OATT.

Network Load – Shall have the same meaning as defined in the FERC Pro Forma OATT.

Network Scheduling Right (NSR) –Optional mechanism on OASIS to request transmission capacity or to post the scheduling rights associated with Network Integration Transmission Service. 

NCEMC:

Network Scheduling Right (NSR) –Optional mechanism on OASIS to request transmission capacity or to post the scheduling rights associated with Network Integration Transmission Service. 

WEQ-001 INDEX OF NITS REQUIREMENTS

001-101
NETWORK INTEGRATION TRANSMISSION SERVICE – GENERAL REQUIREMENTS

Southern:

001-101
NITS – GENERAL REQUIREMENTS

001-101.12
Designation and Re-designation of Network Resource Information Requirements:

Southern:

001-101.12
Designation and Re-designation of Network Resource Information Requirements

001-101.14
Designation Of Network Load Information Requirements

Duke Energy:

001-101.14
Designation of Network Load Information Requirements

001-101.15
Network Transmission Commitment Information Requirements

001-101.16
Generation Information Requirements

001-101.17
Transmission Customers’ System Information Requirements

001-101.18
Transmission Customer Description Information Requirements

001-101.19
Agent Information Requirements

Southern:

Duke Energy:

001-101.14
Designation of Network Load Information Requirements

001-101.15   Network Transmission Commitment Information Requirements

001-101.16
Generation Information Requirements

001-101.17
Transmission Customers’ System Information Requirements

001-101.18
Transmission Customer Description Information Requirements

001-101.19
Agent Information Requirements


 NCEMC:

001-101.14
Designation Of Network Load Information Requirements

001-101.15
Network Transmission Commitment Information Requirements

001-101.16
Generation Information Requirements

001-101.17
Transmission Customers’ System Information Requirements

001-101.18
Transmission Customer Description Information Requirements

001-101.19
Agent Information Requirements



001-102
NETWORK INTEGRATION TRANSMISSION SERVICE – APPLICATION

Southern:

001-102
NITS – APPLICATION

001-102.1 
Submission of NITS Application 

001-102.2
Initial Review of NITS Application

001-102.3
Invalid NITS Application

001-102.4
Deficient NITS Application

001-102.5
Completed NITS Application

001-103
NETWORK INTEGRATION TRANSMISSION SERVICE – EVALUATION OF NITS APPLICATION

Southern:

001-103
NITS – EVALUATION OF NITS APPLICATION

001-103.1
Review of  Completed NITS Application

001-103.2
Study of NITS Application

001-103.3
Declining a NITS Application

001-103.4
Refusal of Completed NITS Application

001-103.5
Counteroffer to NITS Application

001-103.6
Acceptance of NITS Application

001-103.7
Changes to NITS Application

001-103.8
Customer Actions

001-103.9
Final Disposition of NITS Application

001-104
NETWORK INTEGRATION TRANSMISSION SERVICE – ADMINISTRATIVE CHANGES

Southern:

001-104
NITS – ADMINISTRATIVE CHANGES

001-105
NETWORK INTEGRATION TRANSMISSION SERVICE – MODIFICATION TO SERVICE

Southern:

001-105
NITS – MODIFICATION TO SERVICE

001-105.1
General Requirements 
001-105.2
Designation of New Network Resource 
001-105.2.1
Submission of Request for Designation of New Network Resource

001-105.2.2
Initial Review of Request for Designation of New Network Resource

001-105.2.3
Invalid Request for Designation of New Network Resource

001-105.2.4
Deficient Request for Designation of New Network Resource
001-105.2.5
Completed Request for Designation of New Network Resource

001-105.2.6
Review of Completed Request for Designation of New Network Resource

001-105.2.7
Study of Request for Designation of New Network Resource

001-105.2.8
Decline of Request for Designation of New Network Resource

001-105.2.9
Refusal of Request for Designation of New Network Resource

001-105.2.10
Counteroffer to Request for Designation of New Network Resource

001-105.2.11
Acceptance of Request for Designation of New Network Resource

001-105.2.12
Customer Actions

001-105.2.13
Final Disposition of Request for Designation of New Network Resource

001-105.3     Temporary Termination of Network Resource

001-105.3.1
Request for Temporary Termination of Network Resource
001-105.3.2
Submission of Request for Temporary Termination of Network Resource 

001-105.3.3
Initial Review of Request for Temporary Termination of Network Resource 

001-105.3.4
Invalid Request for Temporary Termination of Network Resource 

001-105.3.5
Deficient Request for Temporary Termination of Network Resource 

001-105.3.6
Completed Request for Temporary Termination of Network Resource 

001-105.3.7
Acceptance of Request for Temporary Termination of Network Resource 

001-105.3.8
Customer Actions

001-105.3.9
Final Disposition of Request for Temporary Termination of Network Resource 

001-105.4
Indefinite Termination of Network Resource

001-105.4.1
Request for Indefinite Termination of Network Resource

001-105.4.2
Submission of Request for Indefinite Termination of Network Resource

001-105.4.3
Initial Review of Request for Indefinite Termination of Network Resource

001-105.4.4
Invalid Request for Indefinite Termination of Network Resource

001-105.4.5
Completed Request for Indefinite Termination Of Network Resource

Duke Energy:

001-105.4.5
Completed Request for Indefinite Termination of Network Resource

001-105.4.6
Acceptance of Request for Indefinite Termination of Network Resource

001-105.4.7
Customer Actions

001-105.4.8
Final Disposition of Request for Indefinite Termination of Network Resource

001-105.5
Designation of New Network Load
001-105.5.1
Request for Designation of New Network Load 

001-105.5.2
Submission of Request for Designation of New Network Load

001-105.5.3
Initial Review of Request for Designation of New Network Load

001-105.5.4
Invalid Request for Designation of New Network Load

001-105.5.5
Deficient Request for Designation of New Network Load
001-105.5.6
Completed Request for Designation of New Network Load

001-105.5.7
Review of Completed Request for Designation of New Network Load

001-105.5.8
Study of Request for Designation of New Network Load

001-105.5.9
Declining Request for Designation of New Network Load

001-105.5.10
Refusal of Request for Designation of New Network Load

001-105.5.11
Counteroffer to Request for Designation of New Network Load

001-105.5.12
Acceptance of Request for Designation of New Network Load

001-105.5.13
Customer Actions

001-105.5.14
Final Disposition of Request for Designation of New Network Load


001-105.6         Indefinite Termination of Network Load

001-105.6.1
Submission of Request for Indefinite Termination of Network Load

001-105.6.2
Initial Review of Request for Indefinite Termination of Network Load

001-105.6.3
Invalid Request for Indefinite Termination of Network Load

001-105.6.4
Completed Request for Indefinite Termination  of Network Load

Duke Energy:

001-105.6.4
Completed Request for Indefinite Termination of Network Load

Entergy:

001-105.6.4
Completed Request for Indefinite Termination of Network Load

001-105.6.5
Review of Request for Indefinite Termination  of Network Load

Duke Energy:

001-105.6.5
Review of Request for Indefinite Termination of Network Load

Entergy:

001-105.6.5
Review of Request for Indefinite Termination of Network Load

001-105.6.6
Declining Request for Indefinite Termination  of Network Load

Duke Energy:

001-105.6.6
Declining Request for Indefinite Termination of Network Load

Entergy:

001-105.6.6
Declining Request for Indefinite Termination of Network Load

001-105.6.7
Acceptance of Request for Indefinite Termination  of Network Load

Duke Energy:

001-105.6.7
Acceptance of Request for Indefinite Termination of Network Load

Entergy:

001-105.6.7
Acceptance of Request for Indefinite Termination of Network Load

001-105.6.8
Customer Actions

001-105.6.9
Final Disposition of Request for Indefinite Termination of Network Load

001-105.7
Network Scheduling Rights (NSR)

Southern:

001-105.7
NSR

001-105.7.4
Submission of Request for New Network Scheduling Right

Southern:

001-105.7.4
Submission of Request for New NSR

001-105.7.5
Initial Review of Request for New Network Scheduling Right

Southern:

001-105.7.5
Initial Review of Request for New NSR

001-105.7.6
Invalid Request for New Network Scheduling Right

Southern:

001-105.7.6
Invalid Request for New NSR

001-105.7.7
Completed Request for New Network Scheduling Right

Southern:

001-105.7.7
Completed Request for New NSR

001-105.7.8
Review of Completed Request for New Network Scheduling Right

Southern:

001-105.7.8
Review of Completed Request for New NSR

001-105.7.9
Study of Completed Request for New Network Scheduling Right
Southern:

001-105.7.9
Study of Completed Request for New NSR

001-105.7.10
Decline of Request for New Network Scheduling Right 

Southern:

001-105.7.10
Decline of Request for New NSR 

001-105.7.11
Refusal of Request for New Network Scheduling Right

Southern:

001-105.7.11
Refusal of Request for New NSR

001-105.7.12
Counteroffer to Request for New Network Scheduling Request

Southern:

001-105.7.12
Counteroffer to Request for New NSR

001-105.7.13
Acceptance of Request for New Network Scheduling Right 
Southern:

001-105.7.13
Acceptance of Request for New NSR 

001-105.7.14
Customer Actions

001-105.7.15
Final Disposition of Request for New Network Scheduling Right

Southern:

001-105.7.15
Final Disposition of Request for New NSR

001-105.8     Temporary Termination of Network Scheduling Right

Southern:

001-105.8     Temporary Termination of NSR

Duke Energy:

001-105.8     Temporary Termination of Network Scheduling Rights

Entergy:

001-105.8     Temporary Termination of Network Scheduling Right

001-105.8.4
Submission of Request for Temporary Termination of Network Scheduling Right 

Southern:

001-105.8.4
Submission of Request for Temporary Termination of NSR 

Duke Energy:

001-105.8.4
Submission of Request for Temporary Termination of Network Scheduling Rights 

001-105.8.5
Initial Review of Request for Temporary Termination of Network Scheduling Right
Southern:

001-105.8.5
Initial Review of Request for Temporary Termination of NSR
Duke Energy:

001-105.8.5
Initial Review of Request for Temporary Termination of Network Scheduling Rights

001-105.8.6
Invalid Request for Temporary Termination of Network Scheduling Right 

Southern:

001-105.8.6
Invalid Request for Temporary Termination of NSR 

Duke Energy:

001-105.8.6
Invalid Request for Temporary Termination of Network Scheduling Rights 

001-105.8.7

Completed Request for Temporary Termination of Network


Scheduling Right  

Southern:

001-105.8.7

Completed Request for Temporary Termination of NSR  

Duke Energy:

001-105.8.7

Completed Request for Temporary Termination of Network


Scheduling Rights  

001-105.8.8
Acceptance of Request for Temporary Termination of Network Scheduling Right 

Southern:

001-105.8.8
Acceptance of Request for Temporary Termination of NSR 

Duke Energy:

001-105.8.8
Acceptance of Request for Temporary Termination of Network Scheduling Rights 

001-105.8.9
Customer Actions

001-105.8.10
Final Disposition of Request for Temporary Termination of Network Scheduling Right  

Southern:

001-105.8.10
Final Disposition of Request for Temporary Termination of NSR  

Duke Energy:

001-105.8.10
Final Disposition of Request for Temporary Termination of Network Scheduling Rights  

001-105.9
Indefinite Termination of Network Scheduling Right

Southern:

001-105.9
Indefinite Termination of NSR

001-105.9.4
Submission of Request for Indefinite Termination of Network Scheduling Right 

Southern:

001-105.9.4
Submission of Request for Indefinite Termination of NSR 

Duke Energy:

001-105.9.4
Submission of Request for Indefinite Termination of Network Scheduling Rights 

001-105.9.5
Initial Review of Request for Indefinite Termination of Network Scheduling Right

Southern:

001-105.9.5
Initial Review of Request for Indefinite Termination of NSR

Duke Energy:

001-105.9.5
Initial Review of Request for Indefinite Termination of Network Scheduling Rights
001-105.9.6
Invalid Request for Indefinite Termination of Network Scheduling Right

Southern:

001-105.9.6
Invalid Request for Indefinite Termination of NSR

001-105.9.7
Completed Request for Indefinite Termination Of Network Scheduling Right

Southern:

001-105.9.7
Completed Request for Indefinite Termination Of NSR

Duke Energy:

001-105.9.7
Completed Request for Indefinite Termination of Network Scheduling Rights

001-105.9.8
Acceptance of Request for Indefinite Termination of Network Scheduling Right

Southern:

001-105.9.8
Acceptance of Request for Indefinite Termination of NSR

Duke Energy:

001-105.9.8
Acceptance of Request for Indefinite Termination of Network Scheduling Rights

001-105.9.9
Customer Actions

001-105.9.10
Final Disposition of Request for Indefinite Termination of Network Scheduling Right

Southern:

001-105.9.10
Final Disposition of Request for Indefinite Termination of NSR

Duke Energy:

001-105.9.10
Final Disposition of Request for Indefinite Termination of Network Scheduling Rights

001-105.10
Secondary Network Transmission Service 
001-105.10.1
Submission of Request for Secondary Network Transmission Service

001-105.10.2
Initial Review of Request for Secondary Network Transmission Service
Southern:

001-105.10.2
Initial Review of Request for Secondary Network Transmission Service

001-105.10.3
Invalid Request for Secondary Network Transmission Service

001-105.10.4

Completed Request for Secondary Network Transmission Service

001-105.10.5
Review of Completed Request for Secondary Network Transmission Service

001-105.10.6

Decline of Request for Secondary Network Transmission Service

001-105.10.7
Refusal of Request for Secondary Network Transmission Service 

001-105.10. 8
Counteroffer to Request for Secondary Network Transmission Service

001-105.10.9
Acceptance of Request for Secondary Network Transmission Service

001-105.10.10
Customer Actions

001-105.10.11
Final Disposition of Request for Secondary Network Transmission Service


001-105.11         Termination of Secondary Network Transmission Service

Southern:


001-105.11         Temporary/Indefinite Termination of Secondary Network Transmission Service

001-105.11.3
Request for Termination of Secondary Network Transmission Service

Southern:

001-105.11.3
Request for Temporary/Indefinite Termination of Secondary Network Transmission Service

001-105.11.4
Submission of Request for Termination of Secondary Network Transmission Service

Southern:

001-105.11.4
Submission of Request for Temporary/Indefinite Termination of Secondary Network Transmission Service

001-105.11.5
Initial Review of Request for Termination of Secondary Network Transmission Service

Southern:

001-105.11.5
Initial Review of Request for Temporary/Indefinite Termination of Secondary Network Transmission Service

001-105.11.6
Invalid Request for Termination of Secondary Network Transmission Service

Southern:

001-105.11.6
Invalid Request for Temporary/Indefinite Termination of Secondary Network Transmission Service

001-105.11.7
Completed Request for Termination of Secondary Network Transmission Service

Southern:

001-105.11.7
Completed Request for Temporary/Indefinite Termination of Secondary Network Transmission Service

001-105.11.8
Acceptance of Request for Termination of Secondary Network Transmission Service

Southern:

001-105.11.8
Acceptance of Request for Temporary/Indefinite Termination of Secondary Network Transmission Service

001-105.11.9
Customer Actions

001-105.11.10
Final Disposition of Request for Termination of Secondary Network Scheduling Right 
Southern:

001-105.11.10
Final Disposition of Request for Temporary/Indefinite Termination of Secondary Network Scheduling Right 

001-106
NETWORK INTEGRATION TRANSMISSION SERVICE – REQUEST FOR 


CONCOMITANT REQUESTS 
Southern:

001-106
NITS – REQUEST FOR 


CONCOMITANT REQUESTS 

001-106.6
Customer Actions

Southern:

001-106.7
Final Disposition of the Request For Concomitant Evaluation

001-107
NETWORK INTEGRATION TRANSMISSION SERVICE – ANNUAL UPDATES

Southern:

001-107
NITS – ANNUAL UPDATES

001-108
NETWORK INTEGRATION TRANSMISSION SERVICE – RENEWAL OF SERVICE (or ROLLOVER RIGHTS) 

Southern:

001-108
NITS – RENEWAL OF SERVICE (or ROLLOVER RIGHTS) 

001-101
NETWORK INTEGRATION TRANSMISSION SERVICE – GENERAL REQUIREMENTS

Southern:

001-101
NITS – GENERAL REQUIREMENTS

The following practices are defined in order to enhance consistency of the application process across OASIS nodes.

Midwest ISO:

The following practices are defined in order to enhance consistency of the application
 process across OASIS nodes.

Entergy:

The following practices are defined to enhance consistency of the process of requesting and processing NITS across OASIS nodes.

001-101.1
Arrangement of NITS, both the NITS Application and any subsequent modifications of service, shall be supported on OASIS as specified in the technical requirements of Business Practice Standards WEQ-002, WEQ-003 and WEQ-013. 

Entergy:

001-101.1
Oasis shall support the queuing and processing of all requests for NITS, including the NITS Application and any subsequent modifications of service, as specified in the technical requirements of Business Practice Standards WEQ-002, WEQ-003 and WEQ-013. 

001-101.2
OASIS shall support the capability for the Eligible Transmission Customer to submit the information required for the arrangement of NITS through multiple requests that together represent the service being requested.  This shall include multiple requests submitted as part of a NITS Application, as well as multiple requests submitted as a modification of service (including concomitant evaluations).

SPP:

001-101.2
OASIS shall support the capability for the Eligible Transmission Customer to submit the information required for the arrangement of NITS through multiple requests that together represent the service being requested.  This shall include multiple requests submitted as part of a single
 NITS Application, as well as multiple requests submitted as a modification of service (including concomitant evaluations).

Entergy:

001-101.2
OASIS shall support capability that allows the Eligible Transmission Customer to submit information required for the arrangement of NITS through multiple requests that together represent the service being requested.  This shall include multiple requests submitted as part of a NITS Application, as well as multiple requests submitted as modifications of service (including concomitant evaluations).

001-101.3
All requests for NITS and the current status of those requests shall be posted on OASIS using the defined values for STATUS as set forth in Business Practice Standards WEQ-013 for NITS.  

001-101.4
The Transmission Provider may opt to input information into OASIS on behalf of a Network Customer at the request of the Network Customer.  In such instances, the Transmission Provider must have a posted business practice that addresses the circumstances under which the Transmission Provider will input such information.
Southern:

 001-101.4
The Transmission Provider may opt to input information into OASIS on behalf of a Network Customer at the request of the Network Customer.  In such instances, the Transmission Provider must have a posted Business Practice that addresses the circumstances under which the Transmission Provider will input such information.

001- 101.5
The Transmission Provider shall provide the capability on OASIS for the Eligible Transmission Customer to enter, save, and revise information prior to the submittal of the NITS Application or modifications of service to the Transmission Provider (i.e., a pre-submittal workspace shall be available for the Eligible Transmission Customer to develop an Application over time). 

Duke Energy:

001- 101.5
The Transmission Provider shall provide the capability
 on OASIS for the Eligible Transmission Customer to enter, save, and revise information prior to the submittal of the NITS Application or modifications of service to the Transmission Provider (i.e., a pre-submittal workspace shall be available for the Eligible Transmission Customer to develop an Application over time). 

Midwest ISO:

001- 101.5
The Transmission Provider shall provide the capability on OASIS for the Eligible Transmission Customer to enter, save, and revise information prior to the submittal of the NITS Application or modifications of service to the Transmission Provider (i.e., a pre-submittal workspace shall be available for the Eligible Transmission Customer to develop an Application
 over time). 

Entergy:

001- 101.5
The Transmission Provider shall provide capability on OASIS to allow the Eligible Transmission Customer to enter, save, and revise information required for the arrangement of NITS prior to the submittal of the NITS Application or modifications of service to the Transmission Provider (i.e., a pre-submittal workspace shall be available for the Eligible Transmission Customer to develop an Application over time
). 

001-101.5.1
OASIS shall restrict access to and viewing of all information associated with this pre-submittal workspace to only the Eligible Transmission Customer and the Transmission Provider. 
Duke Energy:

001-101.5.1
OASIS shall restrict access to and viewing of all information associated with this pre-submittal workspace to only the Eligible Transmission Customer and the Transmission Provider
. 

BPA:

001-101.5.1
OASIS shall restrict access to and viewing of all information associated with this pre-submittal workspace to only the Eligible Transmission Customer, their Designation Agent 

and the Transmission Provider. 

001-101.5.2
OASIS shall have the functionality such that an Eligible Transmission Customer may replicate all or a portion of an existing NITS Application into a pre-submittal workspace for use in a subsequent NITS Application or modification of service.

Entergy:

001-101.5.2
OASIS shall support capability that allows an Eligible Transmission Customer to replicate all or a portion of an existing NITS Application into a pre-submittal workspace for use in a subsequent NITS Application or modification of service.

001-101.5.3
Audit and data retention requirements set forth in 18 CFR 37.6 and 37.7 are not applicable to the information in this pre-submittal workspace. 

Southern:

001-101.5.3
Audit and data retention requirements set forth in Title 18 CFR 37.6 and 37.7 are not applicable to the information in this pre-submittal workspace. 

Midwest ISO:

001-101.5.3
Audit and data retention requirements set forth in 18 CFR 37.6 and 37.7 are not applicable to the information in this pre-submittal workspace. 

001-101.5.3.1
The Transmission Provider may remove information in the pre-submittal workspace once the NITS Application has been submitted.
SPP:

001-101.5.3.1
The Transmission Provider may remove information in the pre-submittal workspace once the NITS Application has been changed to a status of COMPLETED
.

Duke Energy:

001-101.5.3.1
The Transmission Provider may remove information 
from the pre-submittal workspace once the NITS Application has been submitted.

Entergy:

001-101.5.3.1
OASIS shall support capability that transfers all information in the pre-submittal workspace to the NITS Application upon submission of the NITS Application.

001-101.5.3.2
If submission of the NITS Application has not occurred within 6 months of the time of the last update, the Transmission Provider may remove the NITS Application and all associated information from the pre-submittal workspace, unless the Eligible Transmission Customer has requested a longer retention period.

Midwest ISO:

001-101.5.3.2
If submission of the NITS Application has not occurred within 6 months of the time of the last update, the Transmission Provider may remove the NITS Application and all associated information from the pre-submittal workspace, unless the Eligible Transmission Customer has requested a longer retention period.

BPA:

001-101.5.3.2
If submission of the NITS Application has not occurred within 6 months of the time of the last update, the Transmission Provider may remove the NITS Application and all associated information from the pre-submittal workspace, unless the Eligible Transmission Customer has requested and the Transmission Provider has agreed to
 a longer retention period.

Entergy:

001-101.5.3.2
If information is entered into the pre-submittal workspace on OASIS by the Eligible Transmission Customer, but a NITS Application is not submitted within 6 months of the time of the last update to the pre-submittal workspace, the Transmission Provider may remove  all information from the pre-submittal workspace, unless the Eligible Transmission Customer has requested a longer retention period
.

001-101.5.3.3
The Eligible Transmission Customer may remove the information contained in the pre-submittal workspace at any time prior to submission.

Duke Energy:

001-101.5.3.3
The Eligible Transmission Customer may remove the information contained in the pre-submittal workspace at any time prior to submission
.

Entergy:

001-101.5.3.3
The Eligible Transmission Customer may remove the information contained in the pre-submittal workspace at any time prior to submission to the Transmission Provider.

001- 101.6
OASIS shall have upload functionality such that an Eligible Transmission Customer may, as an alternative to entering the information directly on OASIS, complete the Application off OASIS and upload the NITS Application to OASIS for additional modifications in the pre-submittal workspace or for submittal of the request(s) to the Transmission Provider. 

BPA:

001- 101.6
OASIS shall have upload functionality such that an Eligible Transmission Customer, or their Designated Agent, 
 may, as an alternative to entering the information directly on OASIS, complete the Application off OASIS and upload the NITS Application to OASIS for additional modifications in the pre-submittal workspace or for submittal of the request(s) to the Transmission Provider. 

Entergy:

001- 101.6
As an alternative to entering the information required for the arrangement of NITS directly on OASIS, OASIS shall support capability that allows  an Eligible Transmission Customer to complete the Application off OASIS and upload the NITS Application to OASIS for additional modifications in the pre-submittal workspace or for submittal to the Transmission Provider
. 

001-101.7
The Eligible Transmission Customer may set the status of NITS-related request(s) (whether uploaded or entered directly into OASIS) to PENDING to indicate that the Eligible Transmission Customer desires to save the information entered and continue working on the request(s) in the pre-submittal workspace.  Alternatively, the Eligible Transmission Customer may submit its NITS-related request(s) (whether uploaded or entered directly into OASIS) to indicate the Eligible Transmission Customer’s desire to formally submit the request(s).

Duke Energy:

001-101.7
The Eligible Transmission Customer may set the status of NITS-related request(s) (whether uploaded or entered directly into OASIS) to PENDING
 to indicate that the Eligible Transmission Customer desires to save the information entered and continue working on the request(s) in the pre-submittal workspace.  Alternatively, the Eligible Transmission Customer may submit its NITS-related request(s) (whether uploaded or entered directly into OASIS) to indicate the Eligible Transmission Customer’s desire to formally submit the request(s).

BPA:

001-101.7
The Eligible Transmission Customer may set the status of NITS-related request(s) (whether uploaded to or entered directly on OASIS) to PENDING to indicate that the Eligible Transmission Customer desires to save the information entered and continue working on the request(s) in the pre-submittal workspace.  Alternatively, the Eligible Transmission Customer may submit its NITS-related request(s) (whether uploaded to or entered directly on OASIS) to indicate the Eligible Transmission Customer’s desire to formally submit the request(s).

001- 101.8
Any required information, including deposits, which is not supplied electronically through OASIS shall be provided to the Transmission Provider by some other method, as specified in the Transmission Provider’s posted business practice. 

Southern:

001- 101.8
Any required information, including deposits, which is not supplied electronically through OASIS shall be provided to the Transmission Provider by some other method, as specified in the Transmission Provider’s posted Business Practice. 

Entergy:

001- 101.8
Any required information, including deposits, which is not supplied electronically through OASIS shall be provided to the Transmission Provider by some other method, as specified in the Transmission Provider’s posted business practice
. 

001-101.9
The Transmission Provider will evaluate all NITS related requests consistent with the Transmission Provider’s posted business practice(s) and tariff. 

Southern:

001-101.9
The Transmission Provider will evaluate all NITS related requests consistent with the Transmission Provider’s posted Business Practice(s) and tariff. 

Entergy: (add section)

001-101.10
Pre-confirmation has no effect on the priority of a NITS Application or modification of network service.
001-101.10
Consistent with the requirements in Business Practice Standard WEQ-013-x, Transmission 
Providers are required to post a reason for certain STATUS changes.

Southern:

001-101.10
Consistent with the requirements in Business Practice Standard WEQ-013-x, Transmission
Providers are required to post a reason for certain STATUS changes.

Entergy:

001-101.11
Consistent with the requirements in Business Practice Standard WEQ-013-x, Transmission 
Providers are required to post a reason for certain STATUS changes.

001-101.11
The Eligible Transmission Customer shall have the ability to remove any NITS Application or request for modification of service from further consideration by setting that request’s status to WITHDRAWN at any time prior to confirmation.

Duke Energy:

001-101.11
The Eligible Transmission Customer shall have the ability to remove a NITS Application or request for modification of service 
from further consideration by setting that request’s status to WITHDRAWN at any time prior to confirmation.

Entergy:

001-101.12
The Eligible Transmission Customer shall have the ability to remove any NITS Application or request for modification of service from further consideration by setting that request’s status to WITHDRAWN at any time prior to confirmation.

001-101.12
Designation and Re-designation of Network Resource Information Requirements:

Southern:

001-101.12
Designation and Re-designation of Network Resource Information Requirements

Duke Energy:

001-101.12
Designation and Re-designation of Network Resource Information Requirements

NCEMC:

001-101.12
Designation and Re-designation of Network Resource Information Requirements:

Entergy:

001-101.13
Network Resource Information and Identification Requirements:

001-101.12.1
For off-system Network Customer-owned generation, the Network Customer should use the TSIN-registered source name if available. If not available, the Network Customer should use a Transmission Provider-approved (the Transmission Provider providing this network service) name which could later be modified to reflect the TSIN-registered name (after the initial submission, but prior to confirmation of the DNR).
Duke Energy:

001-101.12.1
For off-system Network Customer-owned generation, the Network Customer should use the TSIN-registered source name if available
. If not available, the Network Customer should use a Transmission Provider-approved (the Transmission Provider providing this network service)
 name which could later be modified to reflect the TSIN-registered name (after the initial submission, but prior to confirmation of the DNR)
. 

BPA:

001-101.12.1
For off-system Network Customer-owned or leased (check OATT language) 
generation, the Network Customer shall use the TSIN-registered source name if available. If not available, the Network Customer shall use a Transmission Provider-approved (the Transmission Provider providing this network service) name which could later be modified to reflect the TSIN-registered name (after the initial submission, but prior to ACCEPTED or COUNTEROFFER 
 of the DNR).

Entergy:

001-101.13.1
For off-system Network Customer-owned generation, the Network Customer should use the TSIN-registered source name if available. If not available, the Network Customer should use a source name approved by the Transmission Provider providing this network service, whichsource name may be later modified to reflect the TSIN-registered name. Such modification should occur after the initial submission, but prior to confirmation of the request(s).

001-101.12.2
For off-system Network Customer-owned generation, the electrical location will be identified by the BAA where the generation is electrically located and the geographical location will be identified by the county and state. 

Duke Energy:

001-101.12.2
For off-system Network Customer-owned generation, the electrical location will be identified by the BAA
 where the generation is electrically located and the geographical location will be identified by the county and state. 

NCEMC:

001-101.12.2
For off-system Network Customer-owned generation, the electrical location will be identified by the BAA where the generation is electrically located and the geographical location will be identified by the county and state. 

Entergy:

001-101.13.2
For off-system Network Customer-owned generation, the electrical location will be identified by the BAA where the generation is electrically located and the geographical location will be identified by the county and state. 

001-101.12.3
For on-system Network Customer-owned generation, the Network Customer should use the TSIN-registered source name if available. If not available, the Network Customer should use a Transmission Provider-approved (the Transmission Provider providing this network service) name which could later be modified to reflect the TSIN-registered name (after the initial submission, but prior to confirmation of the DNR). 

Duke Energy:

001-101.12.3
For on-system Network Customer-owned generation, the Network Customer should use the TSIN-registered source name if available. If not available, the Network Customer should use a Transmission Provider-approved (the Transmission Provider providing this network service)
 name which could later be modified to reflect the TSIN-registered name (after the initial submission, but prior to confirmation of the DNR). 

BPA:

001-101.12.3
For on-system Network Customer-owned generation, the Network Customer shall use the TSIN-registered source name if available. If not available, the Network Customer shall use a Transmission Provider-approved (the Transmission Provider providing this network service) name which could later be modified to reflect the TSIN-registered name (after the initial submission, but prior to  ACCEPTED or COUNTEROFFER
 of the DNR). 

Entergy:

001-101.13.3
For on-system Network Customer-owned generation, the Network Customer should use the TSIN-registered source name if available. If not available, the Network Customer should use a source name approved by the Transmission Provider  providing this network service, which source name may be later modified to reflect the TSIN-registered name. Such modification should occur after the initial submission, but prior to confirmation of the request(s).  

001-101.12.4
For on-system Network Customer-owned generation, the electrical location will be identified by the BAA where the generation is electrically located and the geographical location will be identified by the county and state.
SPP:

001-101.12.4
For on-system Network Customer-owned generation, the electrical location will be identified by the BAA where the generation is electrically located and the geographical location will be identified by the county/parish and state.

Duke Energy:

001-101.12.4
For on-system Network Customer-owned generation, the electrical location will be identified by the BAA where the generation is electrically located and the geographical location will be identified by the county and state.

NCEMC:

001-101.12.4
For on-system Network Customer-owned generation, the electrical location will be identified by the BAA where the generation is electrically located and the geographical location will be identified by the county and state.

Entergy:

001-101.13.4
For on-system Network Customer-owned generation, the electrical location will be identified by the BAA where the generation is electrically located and the geographical location will be identified by the county and state.

001-101.12.5
For off-system purchased power, the electrical location is the point of receipt (on the Transmission Provider’s system referenced in the Customer’s Application or request for modification of NITS) and the geographical location is “not applicable”. 

Southern:

001-101.12.5
For off-system purchased power, the electrical location is the point of receipt on the Transmission Provider’s system (referenced in the Network Customer’s Application or request for modification of NITS) and the geographical location is “not applicable”.
BPA:

001-101.12.5
For off-system purchased power
, the electrical location is the point of receipt (on the Transmission Provider’s system referenced in the Customer’s Application or request for modification of NITS)  and the geographical location will be identified by the county and state, if available. 

Entergy:

001-101.13.5
For off-system purchased power, the electrical location is the point of receipt (on the Transmission Provider’s system referenced in the request(s) and the geographical location is “not applicable”. 

001-101.12.6
For on-system purchased power, the electrical location is the point of receipt (on the Transmission Provider’s system referenced in the Customer’s Application or request for modification of service) and the geographical location is “not applicable”. 

Southern:

001-101.12.6
For on-system purchased power, the electrical location is the point of receipt on the Transmission Provider’s system (referenced in the Network Customer’s Application or request for modification of service) and the geographical location is “not applicable”. 

BPA:

001-101.12.6
For on-system purchased power
, the electrical location is the point of receipt (on the Transmission Provider’s system referenced in the Customer’s Application or request for modification of service)  and the geographical location will be identified by the county and state, if available. 

Entergy:

001-101.13.6
For on-system purchased power, the electrical location is the point of receipt on the Transmission Provider’s system referenced in the request(s) and the geographical location is “not applicable”. 

001-101.13
Pre-confirmation has no effect on the priority of a NITS Application or modification of network service.

Duke Energy:

001-101.13
Pre-confirmation has no effect on the priority of a NITS Application or modification of network service
.

NCEMC:

001-101.13
Pre-confirmation has no effect on the priority of a NITS Application or modification of network service.

BPA:

001-101.13
Pre-confirmation has no effect on the reservation priority of a NITS Application or modification of network service.

Entergy:

001-102
NETWORK INTEGRATION TRANSMISSION SERVICE – APPLICATION

Southern:

001-102
NITS – APPLICATION

Duke Energy:

001-102
NETWORK INTEGRATION TRANSMISSION SERVICE – APPLICATION

001- 102.1 
Submission of NITS Application 

OATI:

001- 102.1 
Submission of NITS Application 

001- 102.1.1 
An Eligible Transmission Customer shall submit, or arrange for the submittal of, the NITS Application on the Transmission Provider’s OASIS in accordance with the technical requirements of WEQ Standards WEQ-002, WEQ-003 and WEQ-013. 

Southern:

001- 102.1.1 
An Eligible Transmission Customer shall submit, or arrange for the submittal of, the NITS Application on the Transmission Provider’s OASIS in accordance with the technical requirements of Business Practice Standards WEQ-002, WEQ-003 and WEQ-013. 

Duke Energy:

001- 102.1.1 
An Eligible Transmission Customer shall submit, or arrange for the submittal of, the NITS Application on the Transmission Provider’s OASIS in accordance with the technical requirements of WEQ Standards WEQ-002, WEQ-003 and WEQ-013
. 

BPA:

001- 102.1.1 
An Eligible Transmission Customer
 shall submit; the NITS Application on the Transmission Provider’s OASIS in accordance with the technical requirements of WEQ Standards WEQ-002, WEQ-003 and WEQ-013. 

001-102.1.1.1
OASIS shall assign a unique identifier in association with each NITS Application.  

Entergy:

001-102.1.1.1
OASIS shall assign a unique identifier to each NITS Application.  

001-102.1.1.2
An Eligible Transmission Customer may enter and update information on OASIS in association with a NITS Application prior to submission to the Transmission Provider by setting the status of the NITS Application to PENDING. 

Entergy:

001-102.1.1.2
An Eligible Transmission Customer may enter and update information to support a NITS Application on OASIS prior to submission to the Transmission Provider by setting the status of the NITS Application to PENDING. 

001-102.1.1.2.1
All related information (e.g., load forecasts, resource designations, etc.) associated with such a PENDING NITS Application (whether such information is submitted electronically or by other means) shall not be considered submitted until the NITS Application is submitted in its entirety. 
Duke Energy:

001-102.1.1.2.1
All related information (e.g., load forecasts, resource designations, etc.) associated with such a PENDING NITS Application (whether such information is submitted electronically or by other means) shall not be considered submitted until the NITS Application is submitted in its entirety. 

Entergy:

001-102.1.1.2.1
Information (e.g., load forecasts, resource designations, etc.) associated with a PENDING NITS Application (whether provided electronically or by other means) shall not be considered  in support of such NITS Application until the NITS Application is submitted to the Transmission Provider. 

001- 102.1.2
The Transmission Provider shall specify, through the posting of a business practice, the minimum information required for a NITS Application. 

Southern:

001- 102.1.2
The Transmission Provider shall specify, through the posting of a Business Practice, the minimum information required for a NITS Application. 

NCEMC:

001- 102.1.2
The Transmission Provider shall specify, through the posting of a business practice, the minimum information required for a NITS Application. 

EPSA:

001- 102.1.2
The Transmission Provider shall specify, through the posting of a business practice, the minimum information required for a NITS Application. 

Entergy:

001- 102.1.2
The Transmission Provider shall specify, through the posting of a business practice, the minimum information required for submission of a NITS Application. 

001- 102.1.3
The Eligible Transmission Customer shall indicate to the Transmission Provider that the NITS Application is ready for Transmission Provider review by submitting the the NITS Application on OASIS.  

Duke Energy:

001- 102.1.3
The Eligible Transmission Customer shall indicate to the Transmission Provider that the NITS Application is ready for Transmission Provider review by submitting the the NITS Application on OASIS.  

001-102.1.3.1
OASIS shall set the queue time for the NITS Application to the time that the Eligible Transmission Customer submits the NITS Application, which shall set the status of the NITS Application to QUEUED.

Southern:

001-102.1.3.1
OASIS shall set the queue time for the NITS Application to the time that the Eligible Transmission Customer submits the NITS Application, and set the status of the NITS Application to QUEUED.

Duke Energy:

001-102.1.3.1
OASIS shall set the queue time for the NITS Application
 to the time that the Eligible Transmission Customer submits the NITS Application, which shall set the status of the NITS Application to QUEUED.

Entergy:

001-102.1.3.1
OASIS shall set the queue time for the NITS Application to the time that the Eligible Transmission Customer submits the NITS Application, and shall set the status of the NITS Application to QUEUED.

001-102.1.3.2
The Transmission Provider shall acknowledge the queued NITS Application  by setting the status of the NITS Application to RECEIVED.

Duke Energy:

001-102.1.3.2
The Transmission Provider shall acknowledge the queued NITS Application  by setting the status of the NITS Application
 to RECEIVED.

001-102.2
Initial Review of NITS Application

001- 102.2.1
The Transmission Provider shall review the received NITS Application to determine if the NITS Application: (1) meets the minimum requirements specified in the Transmission Provider’s posted business practices; or (2) is complete consistent with the Transmission Provider’s tariff requirements.

Southern:

001- 102.2.1
The Transmission Provider shall review the received NITS Application to determine if the NITS Application: (1) meets the minimum requirements specified in the Transmission Provider’s posted Business Practices; or (2) is complete consistent with the Transmission Provider’s tariff requirements.

SPP:

001- 102.2.1
The Transmission Provider shall review the received NITS Application to determine if the NITS Application: (1) meets the minimum requirements specified in the Transmission Provider’s posted business practices; or (2) is complete consistent with the Transmission Provider’s tariff requirements or both
.

Duke Energy:

001- 102.2.1
The Transmission Provider shall review the received NITS Application to determine if the NITS Application: (1) meets the minimum requirements specified in the Transmission Provider’s posted business practices; or
 (2) is complete consistent with the Transmission Provider’s tariff requirements.

NCEMC:

001- 102.2.1
The Transmission Provider shall review the received NITS Application to determine if the NITS Application: (1) meets the minimum requirements specified in the Transmission Provider’s posted business practices; or (2) is complete consistent with the Transmission Provider’s tariff requirements.

001- 102.2.2
The Transmission Provider shall respond to a NITS Application by setting the status to one of the following:

I.
INVALID 

II.
DEFICIENT 

III.
COMPLETED 

EPSA:

001- 102.2.2
The Transmission Provider shall respond to a NITS Application by setting the status to one of the following:

I.
INVALID 

II.
DEFICIENT 

III.
COMPLETED 



001-102.3
Invalid NITS Application

001-102.3.1
If the required information, as specified in Business Practice Standard WEQ-001-102.2.1, is not provided, the Transmission Provider shall set the status of the NITS Application to INVALID and the NITS Application shall be deemed withdrawn.

Entergy:

001-102.3.1
If the required information, as specified in Business Practice Standard WEQ-001-102.2.1, has not been provided in the NITS Application, the Transmission Provider shall set the status of the NITS Application to INVALID and the NITS Application shall be deemed withdrawn.

001-102.3.2
In response to a state of INVALID, the Eligible Transmission Customer may submit a new or revised NITS Application, which will be assigned a new unique identifier and a new queue time.   

001-102.4
Deficient NITS Application

001-102.4.1
For each portion of information required in a NITS Application that has not been deemed INVALID, the Transmission Provider shall determine if such portion can be deemed sufficient.

Duke Energy:

001-102.4.1
For each portion
 of information required in a NITS Application that has not been deemed INVALID, the Transmission Provider shall determine if such portion can be deemed sufficient.

Entergy:

001-102.4.1
For a NITS Application that has not been deemed INVALID, the Transmission Provider shall determine if each portion of the required information submitted can be deemed sufficient.

001-102.4.1.1
If deemed sufficient, the Transmission Provider shall set the status of such portion to COMPLETED.  

001-102.4.1.2
If not deemed sufficient, the Transmission Provider shall set the status of such portion to DEFICIENT

Duke Energy:

001-102.4.1.2
If not deemed sufficient, the Transmission Provider shall set the status of such portion to DEFICIENT
.

001-102.4.2
If the Transmission Provider determines that any portion of the NITS Application is DEFICIENT, the Transmission Provider shall set the status of the overall NITS Application to DEFICIENT and begin informal communications with the Eligible Transmission Customer to resolve those deficiencies.
Southern:

001-102.4.2
If the Transmission Provider determines that any portion of the NITS Application is deficient, the Transmission Provider shall set the status of the overall NITS Application to DEFICIENT and begin informal communications with the Eligible Transmission Customer to resolve those deficiencies.

SPP:

001-102.4.2
If the Transmission Provider determines that any portion of the NITS Application is DEFICIENT, the Transmission Provider shall set the status of the overall NITS Application to DEFICIENT and begin 
communications with the Eligible Transmission Customer to resolve those deficiencies.

Duke Energy:

001-102.4.2
If the Transmission Provider determines that any portion of the NITS Application is DEFICIENT
, the Transmission Provider shall set the status of the overall NITS Application to DEFICIENT and begin informal communications with the Eligible Transmission Customer to resolve those deficiencies.

001-102.4.3
The Eligible Transmission Customer shall remedy the deficiency and shall set that portion’s status to REEVALUATE. Once all deficiencies have been remedied, the Eligible Transmission Customer shall set the status of the overall NITS Application to REEVALUATE.

Entergy:

001-102.4.3
The Eligible Transmission Customer shall remedy the deficiency and shall set that portion’s status to REEVALUATE. Once all deficiencies have been remedied, the Eligible Transmission Customer shall set the status of the overall NITS Application to REEVALUATE.

001-102.4.4
Upon notification that the Eligible Transmission Customer has provided the necessary information, either on or off OASIS as appropriate, to remedy an identified deficiency, the Transmission Provider shall review the provided information and take one of the following actions:

001-102.4.4.1
If the information provided for any portion identified as deficient fails to remedy that deficiency, the Transmission Provider shall set the status for that portion of the NITS Application to DEFICIENT.

001-102.4.4.2
If the information provided for any portion identified as deficient remedies that deficiency, the Transmission Provider shall set the status for that portion of the NITS Application to COMPLETED.

001-102.4.4.3
If the information provided for any portion of the NITS Application is deemed by the Transmission Provider to be a significant change to the overall NITS Application, the Transmission Provider shall set the status of the overall NITS Application to DECLINED.

Duke Energy:

001-102.4.4.3
If the information provided for any portion of the NITS Application is deemed by the Transmission Provider to be a significant change to the overall NITS Application, the Transmission Provider shall set the status of the overall NITS Application to DECLINED
.

NCEMC:

001-102.4.4.3
If the information provided for any portion of the NITS Application is deemed by the Transmission Provider to be a significant change to the overall NITS Application, the Transmission Provider shall set the status of the overall NITS Application to DECLINED.

001-102.4.5
If the Eligible Transmission Customer fails to respond to the Transmission Provider’s notice of a DEFICIENT NITS Application, or fails to remedy the deficiency to the Transmission Provider’s satisfaction in a timely manner, the Transmission Provider shall set the status of the overall NITS Application to DECLINED.

Southern:

001-102.4.5
If the Eligible Transmission Customer fails to respond to the Transmission Provider’s notice of a deficient NITS Application, or fails to remedy the deficiency to the Transmission Provider’s satisfaction in a timely manner, the Transmission Provider shall set the status of the overall NITS Application to DECLINED.

Duke Energy:

001-102.4.5
If the Eligible Transmission Customer fails to respond to the Transmission Provider’s notice of a DEFICIENT NITS Application, or fails to remedy the deficiency to the Transmission Provider’s satisfaction in a timely manner, the Transmission Provider shall set the status of the overall NITS Application to DECLINED
.

NCEMC:

001-102.4.5
If the Eligible Transmission Customer fails to respond to the Transmission Provider’s notice of a DEFICIENT NITS Application, or fails to remedy the deficiency to the Transmission Provider’s satisfaction in a timely manner, the Transmission Provider shall set the status of the overall NITS Application to DECLINED.

001-102.4.6
If the Eligible Transmission Customer fails to provide the necessary credit or deposit information in a timely manner, the Transmission Provider may respond by setting the status of the overall NITS Application to DECLINED.

001-102.4.7
In response to a state of DECLINED, the Eligible Transmission Customer may submit a new or revised NITS Application, which will be assigned a new unique identifier and a new queue time. 

Duke Energy:

001-102.4.7
In response to a state of DECLINED, the Eligible Transmission Customer may submit a new or revised NITS Application
, which will be assigned a new unique identifier and a new queue time. 

001-102.4.8
When the Transmission Provider determines that all deficiencies have been remedied and all portions of the NITS Application have been identified as COMPLETED, the Transmission Provider shall continuing processing the NITS Application in accordance with the Business Practice Standards for a Completed NITS Application.

Southern:

001-102.4.8
When the Transmission Provider determines that all deficiencies have been remedied and all portions of the NITS Application have been identified as COMPLETED, the Transmission Provider shall continue processing the NITS Application in accordance with the NITS Business Practice Standards for a Completed NITS Application.

001-102.5
Completed NITS Application

OATI:

001-102.5
Completed NITS Application

001-102.5.1
When the Transmission Provider has reviewed the entire NITS Application for completeness and all portions associated with the NITS Application have been deemed as COMPLETED, the Transmission Provider shall indicate that the NITS Application is deemed complete by setting the status of the overall NITS Application to COMPLETED.

Entergy:

001-102.5.1
When the Transmission Provider has reviewed the entire NITS Application for completeness and all portions associated with the NITS Application have been deemed COMPLETED, the Transmission Provider shall indicate that the NITS Application is deemed complete by setting the status of the overall NITS Application to COMPLETED.

001-102.5.2
OASIS shall set the queue time of all COMPLETED portions associated with the COMPLETED NITS Application to be equal to the queue time established on the initial submission of the NITS Application.

Duke Energy:

001-102.5.2
OASIS shall set the queue time 
of all COMPLETED portions associated with the COMPLETED NITS Application to be equal to the queue time established on the initial submission of the NITS Application.

001-102.5.3
The COMPLETED NITS Application and set of associated COMPLETED portions shall constitute the entire initial NITS Application to be considered and evaluated further for granting of service by the Transmission Provider.

Duke Energy:

001-102.5.3
The COMPLETED NITS Application and set of associated COMPLETED portions  shall constitute the entire initial NITS Application to be considered and evaluated further for granting of service by the Transmission Provider.

NOTE:  Should we Include Ancillary Service requests as a NITS subtemplate?
NCEMC:

NOTE:  Should we Include Ancillary Service requests as a NITS subtemplate?

Midwest ISO:

NOTE:  Should we Include Ancillary Service requests as a NITS subtemplate?

BC Hydro:

NOTE:  Should we Include Ancillary Service requests as a NITS subtemplate?

BPA:

NOTE:  Should we Include Ancillary Service requests as a NITS subtemplate?

Entergy:

NOTE:  Should we Include Ancillary Service requests as a NITS subtemplate?

Note:  Should we allow some wiggle room in giving the “reason for denial” there is a need to be sensitive to the insufficient credit or deposit denial information posted publicly on OASIS? Or should the NAESB standards be silent?
NCEMC:

Note:  Should we allow some wiggle room in giving the “reason for denial” there is a need to be sensitive to the insufficient credit or deposit denial information posted publicly on OASIS? Or should the NAESB standards be silent?

Midwest ISO:

Note:  Should we allow some wiggle room in giving the “reason for denial” there is a need to be sensitive to the insufficient credit or deposit denial information posted publicly on OASIS? Or should the NAESB standards be silent?

BC Hydro:

Note:  Should we allow some wiggle room in giving the “reason for denial” there is a need to be sensitive to the insufficient credit or deposit denial information posted publicly on OASIS? Or should the NAESB standards be silent?

BPA:

Note:  Should we allow some wiggle room in giving the “reason for denial” there is a need to be sensitive to the insufficient credit or deposit denial information posted publicly on OASIS? Or should the NAESB standards be silent?

Entergy:

Note:  Should we allow some wiggle room in giving the “reason for denial” there is a need to be sensitive to the insufficient credit or deposit denial information posted publicly on OASIS? Or should the NAESB standards be silent?

001-103
NETWORK INTEGRATION TRANSMISSION SERVICE – EVALUATION OF NITS APPLICATION

Southern:

001-103
NITS – EVALUATION OF NITS APPLICATION

001-103.1
Review of  Completed NITS Application

001- 103.1.1
Once the Transmission Provider has set the status of the NITS Application to COMPLETED, the Transmission Provider shall change the status of the overall NITS Application to one of the following to indicate its evaluation of the NITS Application:

I.
STUDY

II.
DECLINED

III.
REFUSED 

IV.
COUNTEROFFER

V.
ACCEPTED 

001-103.2
Study of NITS Application

001-103.2.1 
If the Transmission Provider determines that some level of study is required or is being performed, the Transmission Provider shall set the status of the overall NITS Application to STUDY.

001-103.2.2
The Transmission Provider may update the status of any portion of the NITS Application, as appropriate.

EPSA:

001-103.2.2
The Transmission Provider may update the status of any portion of the NITS Application, as appropriate.

001-103.2.3
From the status of STUDY, the Transmission Provider shall set the status of the overall NITS Application to DECLINED, REFUSED, COUNTEROFFER, or ACCEPTED.

001-103.3
Declining a NITS Application

001-103.3.1
If at any time in the evaluation of the NITS Application or conduct of any associated study(ies) the Transmission Provider determines that the Eligible Transmission Customer has failed to meet any of the terms and conditions for the provision of service (e.g., failure to provide a required deposit, execute a study agreement, etc.), the Transmission Provider shall set the status of the overall NITS Application to DECLINED.

Duke Energy:

001-103.3.1
If at any time in the evaluation of the NITS Application or conduct of any associated study(ies) the Transmission Provider determines that the Eligible Transmission Customer has failed to meet any of the terms and conditions for the provision of service (e.g., failure to provide a required deposit, execute a study agreement, etc.), the Transmission Provider shall set the status of the overall NITS Application to DECLINED
.

001-103.3.2
In response to a state of DECLINED, the Eligible Transmission Customer may submit a new or revised NITS Application, which will be assigned a new unique identifier and a new queue time. 

001-103.4
Refusal of Completed NITS ApplicationDuke Energy:

001-103.4
Refusal of Completed NITS Application

001-103.4.1
If at any time in the evaluation of the NITS Application or conduct of any associated study(ies) the Transmission Provider determines that the requested service cannot be provided due to insufficient transfer capability consistent with the Transmission Provider’s tariff requirements, the Transmission Provider may set the status of the overall NITS Application to REFUSED.

Duke Energy:

001-103.4.1
If at any time in the evaluation of the NITS Application or conduct of any associated study(ies) the Transmission Provider determines that the requested service cannot be provided due to insufficient transfer capability consistent with the Transmission Provider’s tariff requirements, the Transmission Provider may set the status of the overall NITS Application to REFUSED
.

NCEMC:

001-103.4.1
If at any time in the evaluation of the NITS Application or conduct of any associated study(ies) the Transmission Provider determines that the requested service cannot be provided due to insufficient transfer capability consistent with the Transmission Provider’s tariff requirements, the Transmission Provider may set the status of the overall NITS Application to REFUSED.

001-103.4.2
In response to a state of REFUSED, the Eligible Transmission Customer may submit a new or revised NITS Application, which will be assigned a new unique identifier and a new queue time.

NCEMC:

001-103.4.2
In response to a state of REFUSED, the Eligible Transmission Customer may submit a new or revised NITS Application, which will be assigned a new unique identifier and a new queue time.

001-103.5
Counteroffer to NITS Application

001-103.5.1
When, in the evaluation of the NITS Application or on completion of any applicable study(ies), and, where appropriate, through informal communication with the Eligible Transmission Customer, the Transmission Provider determines that the requested transmission service may only be granted in part or subject to modification to the NITS Application, the Transmission Provider shall set the status of the overall NITS Application to COUNTEROFFER.

Entergy:

001-103.5.1
When, in the evaluation of the NITS Application or on completion of any applicable study(ies), and, where appropriate, through informal communication with the Eligible Transmission Customer, the Transmission Provider determines that the requested transmission service may only be granted in part, the Transmission Provider shall set the status of the overall NITS Application to COUNTEROFFER.

001-103.5.2
Prior to setting the status of the overall NITS Application to COUNTEROFFER, the Transmission Provider shall update the status associated with each portion of the Completed Application as follows:

001-103.5.2.1
The status of any portion of the NITS Application that will not be granted shall be set to the appropriate final state.

001-103.5.2.2
The status of any portion of the NITS Application that is to be granted as specified in the NITS Application shall be set to ACCEPTED.

001-103.5.2.3
The status of any portion of the NITS Application that is to be granted but at a reduced capacity or subject to other modification from that specified in the NITS Application shall be set to COUNTEROFFER.

Entergy:

001-103.5.2.3
The status of any portion of the NITS Application that is to be granted but at a reduced capacityshall be set to COUNTEROFFER
.

001-103.6
Acceptance of NITS Application

001-103.6.1
When, in the evaluation of the NITS Application or on completion of any applicable study(ies), and, where appropriate, through informal communication with the Eligible Transmission Customer, the Transmission Provider determines that the requested transmission service may be granted, the Transmission Provider shall set the status of the overall NITS Application to ACCEPTED.

SPP:

001-103.6.1
When, in the evaluation of the NITS Application or on completion of any applicable study(ies), and, where appropriate, through 
communication with the Eligible Transmission Customer, the Transmission Provider determines that the requested transmission service may be granted, the Transmission Provider shall set the status of the overall NITS Application to ACCEPTED.

001-103.6.2
Prior to setting the status of the overall NITS Application to ACCEPTED, the Transmission Provider shall update the status associated with each portion of the NITS Application to ACCEPTED.

BPA:

001-103.6.2

Prior to setting the status of the overall NITS Application to ACCEPTED, the Transmission Provider shall update the status associated with each portion of the NITS Application to ACCEPTED.

001-103.7
Changes to NITS Application

001-103.7.1
Based on the evaluation results and informal communications between the Transmission Provider and the Eligible Transmission Customer, the Eligible Transmission Customer may submit a request to change the completed NITS Application while the overall NITS Application is under evaluation with a status of either COMPLETED or STUDY. These changes may include updates to existing portions, withdrawal of certain portions and/or submission of additional portions of information to be incorporated into the NITS Application.

SPP:

001-103.7.1
Based on the evaluation results and 
communications between the Transmission Provider and the Eligible Transmission Customer, the Eligible Transmission Customer may submit a request to change the completed NITS Application while the overall NITS Application is under evaluation with a status of either COMPLETED or STUDY. These changes may include updates to existing portions, withdrawal of certain portions and/or submission of additional portions of information to be incorporated into the NITS Application.

001-103.7.1.1
Any request submitted by the Eligible Transmission Customer associated with a NITS Application where the overall NITS Application status is ACCEPTED or COUNTEROFFER shall be deemed a request for modifications to service and held for later evaluation until the overall NITS Application is set to a final state.

Duke Energy:

001-103.7.1.1
Any request submitted by the Eligible Transmission Customer associated with a NITS Application where the overall NITS Application status is ACCEPTED or COUNTEROFFER shall be deemed a request for modifications to service and held for later evaluation until the overall NITS Application is set to a final state.

001-103.7.1.2
Any request submitted by the Eligible Transmission Customer associated with a NITS Application where the overall NITS Application status is CONFIRMED shall be treated as a request for modifications to service and processed in accordance with the applicable Business Practice Standards.

Southern:

001-103.7.1.2
Any request submitted by the Eligible Transmission Customer associated with a NITS Application where the overall NITS Application status is CONFIRMED shall be treated as a request for modifications to service and processed in accordance with the applicable Business Practice Standards WEQ-001-105.

SPP:

001-103.7.1.2
Any request submitted by the Eligible Transmission Customer associated with a NITS Application where the overall NITS Application status is CONFIRMED shall be treated as a request for modifications to service and processed in accordance with the applicable Business
 Practice Standards.

Duke Energy:

001-103.7.1.2
Any request submitted by the Eligible Transmission Customer associated with a NITS Application where the overall NITS Application status is CONFIRMED
 shall be treated as a request for modifications to service and processed in accordance with the applicable Business Practice Standards found in WEQ-001-105.

001-103.7.1.3
Any request submitted by the Eligible Transmission Customer associated with a NITS Application where the overall NITS Application status is in any final state shall be deemed INVALID.

Duke Energy:

001-103.7.1.3
Any request submitted by the Eligible Transmission Customer associated with a NITS Application where the overall NITS Application status is in any final state shall be deemed INVALID
.

001-103.7.2
The Transmission Provider shall evaluate all Eligible Transmission Customer requests to change the NITS Application and may take one of the following actions:

Duke Energy:

001-103.7.2
The Transmission Provider shall evaluate all Eligible Transmission Customer requests to change the NITS Application
 and may take one of the following actions:

EPSA:

001-103.7.2
The Transmission Provider shall evaluate all Eligible Transmission Customer requests to change the NITS Application and may take one of the following actions:

Entergy:

001-103.7.2
The Transmission Provider shall evaluate all requests by the Eligible Transmission Customer to change the NITS Application and may take one of the following actions with respect to such requests:

001-103.7.2.1
The Transmission Provider may deny the change in the service being requested when the change is outside the scope of the informal communications and mutual agreement between the Transmission Provider and Eligible Transmission Customer. The Transmission Provider will update the status associated with the requested change to an appropriate final state.

SPP:

001-103.7.2.1
The Transmission Provider may deny the change in the service being requested when the change is outside the scope of the 
communications and mutual agreement between the Transmission Provider and Eligible Transmission Customer. The Transmission Provider will update the status associated with the requested change to an appropriate final state.

Duke Energy:

001-103.7.2.1
The Transmission Provider may deny the change in the service being requested when the change is outside the scope of the informal communications and mutual agreement between the Transmission Provider and Eligible Transmission Customer. The Transmission Provider will update the status associated with the requested change to an appropriate final state
.

Entergy:

001-103.7.2.1
The Transmission Provider may deny the change in the service being requested when the change is outside the scope of the informal communications and mutual agreement between the Transmission Provider and Eligible Transmission Customer. The Transmission Provider will update the status associated with the requested change to an appropriate final state.

001-103.7.2.2
The Transmission Provider may hold the change until the status of the overall NITS Application is set to an appropriate final state and treat the request as a request for modifications to service.

Duke Energy:

001-103.7.2.2
The Transmission Provider may hold the change until the status of the overall NITS Application is set to an appropriate final state and treat the request as a request for modifications to service
.

Entergy:

001-103.7.2.2
The Transmission Provider may hold the change until the status of the overall NITS Application is set to an appropriate final state and treat the request as a request for modification to service.

001-103.7.2.3
The Transmission Provider may approve the change and incorporate the change into the NITS Application by setting its status to COMPLETED.  The final status of the change will be reflected when the Transmission Provider sets the overall NITS Application to an appropriate final state.

001-103.8
Customer Actions

001-103.8.1
After the status of the overall NITS Application has been set to ACCEPTED or COUNTEROFFER by the Transmission Provider, the Eligible Transmission Customer may confirm or withdraw the overall NITS Application by setting the status to CONFIRMED or WITHDRAWN. 

Duke Energy:

001-103.8.1
After the status of the overall NITS Application has been set to ACCEPTED or COUNTEROFFER by the Transmission Provider, the Eligible Transmission Customer may confirm or withdraw the overall NITS Application by setting the status to CONFIRMED or WITHDRAWN
. 

001-103.8.1.1
Confirming the overall NITS Application shall constitute the Network Customer’s agreement to receive service as specified in all portions of the NITS Application which are not already in a final state. Confirming the overall NITS Application shall set the status of all portions of the NITS Application not already in a final state to CONFIRMED.

Duke Energy:

001-103.8.1.1
Confirming the overall NITS Application shall constitute the Network Customer’s agreement to receive service as specified in all portions of the NITS Application which are not already in a final state. Confirming the overall NITS Application shall set the status of all portions of the NITS Application not already in a final state to CONFIRMED. The Network Customer is not permitted to CONFIRM only a portion of the NITS Application.

Entergy:

001-103.8.1.1
Confirming the overall NITS Application shall constitute the Network Customer’s agreement to receive service as specified in all portions of the NITS Application that are not already in a final state. Confirming the overall NITS Application shall set the status of all portions of the NITS Application not already in a final state to CONFIRMED
.

001-103.8.1.1.1
If the Eligible Transmission Customer requests the Transmission Provider to file an unexecuted service agreement at FERC, the Eligible Transmission Customer, at the time of setting the status of the overall NITS Application to CONFIRMED, shall also set the indicator to “Unexecuted” (language in blue to be determined by tech team).
Duke Energy:

001-103.8.1.1.1
If the Eligible Transmission Customer requests the Transmission Provider to file an unexecuted service agreement at FERC, the Eligible Transmission Customer, at the time of setting the status of the overall NITS Application to CONFIRMED, shall also set the indicator to “Unexecuted” (language in blue to be determined by tech team).

Entergy:

001-103.8.1.1.1
If the Eligible Transmission Customer requests the Transmission Provider to file an unexecuted service agreement at FERC, the Eligible Transmission Customer, at the time of setting the status of the overall NITS Application to CONFIRMED, shall also set the indicator to “Unexecuted” (language in green to be determined by tech team).
001-103.8.1.2
Withdrawal of the overall NITS Application shall constitute the Eligible Transmission Customer’s withdrawal of all portions of the NITS Application which are not already in a final state.  Withdrawal of the overall NITS Application shall set the status of the portions not already in a final state to WITHDRAWN.

Duke Energy:

001-103.8.1.2
Withdrawal of the overall NITS Application shall constitute the Eligible Transmission Customer’s withdrawal of all portions of the NITS Application which are not already in a final state
.  Withdrawal of the overall NITS Application shall set the status of the portions not already in a final state to WITHDRAWN.

BPA:

001-103.8.1.2
Withdrawal of the overall NITS Application shall constitute the Eligible Transmission Customer’s withdrawal  of all portions of the NITS Application  including those portions already in a final state.  Withdrawal of the overall NITS Application shall set the status of all portions of the NITS Application to WITHDRAWN
.

Entergy:

001-103.8.1.2
Withdrawal of the overall NITS Application shall constitute the Eligible Transmission Customer’s withdrawal of all portions of the NITS Application that are not already in a final state.  Withdrawal of the overall NITS Application shall set the status of the portions not already in a final state to WITHDRAWN.

001-103.8.1.3
The Eligible Transmission Customer may not act on the individual portions of the NITS Application.  Once the overall NITS Application is in a final state of CONFIRMED, any subsequent changes must be made through a request for modifications to service as provided for in the Business Practice Standards.
Southern:

  001-103.8.1.3
The Eligible Transmission Customer may not act on the individual portions of the NITS Application.  Once the overall NITS Application is in a final state of CONFIRMED, any subsequent changes must be made through a request for modifications to service as provided for in the Business Practice Standards WEQ-001-105.

Duke Energy:

001-103.8.1.3

The Eligible Transmission Customer may not act on the individual portions of the NITS Application.  Once the overall NITS Application is in a final state of CONFIRMED, any subsequent changes must be made through a request for modifications to service as provided for in the Business Practice Standards.

Entergy:

001-103.8.1.3
The Eligible Transmission Customer may not act on the individual portions of the NITS Application.  Once the overall NITS Application is in a final state of CONFIRMED, any subsequent changes must be made through a request for modification to service as provided for in the Business Practice Standards.

001-103.9
Final Disposition of NITS Application

001-103.9.1
Once CONFIRMED the overall NITS Application shall remain CONFIRMED throughout its term of service regardless of subsequent modifications unless otherwise mutually agreed; in which case the Transmission Provider shall set the status of the overall NITS Application to ANNULLED.

Duke Energy:

001-103.9.1
Once CONFIRMED the overall NITS Application shall remain CONFIRMED throughout its term of service regardless of subsequent modifications unless otherwise mutually agreed; in which case the Transmission Provider shall set the status of the overall NITS Application to ANNULLED
.

Entergy:

001-103.9.1
Once CONFIRMED the overall NITS Application shall remain CONFIRMED throughout its term of service regardless of subsequent modifications to service unless otherwise mutually agreed; in which case the Transmission Provider shall set the status of the overall NITS Application to ANNULLED.

001-103.9.1.1
If the Transmission Provider filed an unexecuted service agreement, upon receiving a Commission order, the Eligible Transmission Customer shall use the modifications to service as provided for in the Business Practice Standards to modify the overall NITS Application to reflect the Commission order.

Southern:

001-103.9.1.1
If the Transmission Provider filed an unexecuted service agreement, upon receiving a Commission order, the Eligible Transmission Customer shall use the modifications to service as provided for in the Business Practice Standards WEQ-001-105 to modify the overall NITS Application to reflect the Commission order.

Duke Energy:

001-103.9.1.1
If the Transmission Provider filed an unexecuted service agreement, upon receiving a Commission order, the Eligible Transmission Customer shall use the modifications to service as provided for in the Business Practice Standards
 to modify the overall NITS Application to reflect the Commission order.

001-103.9.1.1.2
If the Eligible Transmission Customer determines not to accept the terms and conditions order by the Commission, the Eligible Transmission Customer may direct the Transmission Provider to set the status of the overall NITS Application to ANNULLED.

Duke Energy:

001-103.9.1.1.2
If the Eligible Transmission Customer determines not to accept the terms and conditions order by the Commission, the Eligible Transmission Customer may direct the Transmission Provider to set the status of the overall NITS Application to ANNULLED
.

Entergy:

001-103.9.1.1.2
If the Eligible Transmission Customer determines not to accept the terms and conditions ordered by the Commission, the Eligible Transmission Customer may direct the Transmission Provider to set the status of the overall NITS Application to ANNULLED.

001-103.9.2
If, within the time limits specified in the Transmission Provider’s tariff for execution of a service agreement or for requesting filing of an unexecuted service agreement, the Eligible Transmission Customer fails to set the status to CONFIRMED or WITHDRAWN or does not request the Transmission Provider to file at FERC an unexecuted service agreement, the Transmission Provider may set the status of the overall NITS Application to RETRACTED. Retraction of the overall NITS Application shall set the status of the portions not already in a final state to RETRACTED.

Entergy:

001-103.9.2
If, within the time limits specified in the Transmission Provider’s tariff for execution of a service agreement or for requesting filing of an unexecuted service agreement, the Eligible Transmission Customer fails to set the status to CONFIRMED or WITHDRAWN or does not request the Transmission Provider to file at FERC an unexecuted service agreement, the Transmission Provider may set the status of the overall NITS Application to RETRACTED. Retraction of the overall NITS Application shall set the status of the portions not already in a final state to RETRACTED
.

001-103.9.2.1
In response to a state of RETRACTED, the Eligible Transmission Customer may submit a new or revised NITS Application, which will be assigned a new unique identifier and a new queue time. 

001-104 
NETWORK INTEGRATION TRANSMISSION SERVICE – ADMINISTRATIVE CHANGES

Southern:

001-104 
NITS – ADMINISTRATIVE CHANGES

OATI:

001-104 
NETWORK INTEGRATION TRANSMISSION SERVICE – ADMINISTRATIVE CHANGES

EPSA:

001-104 
NETWORK INTEGRATION TRANSMISSION SERVICE – ADMINISTRATIVE CHANGES

001-104.1
The Network Customer shall have the right to request administrative changes pursuant to the NITS Application. 
001-104.2
A request for administrative changes shall be submitted to the Transmission Provider on OASIS with explicit reference to the associated NITS Application.

Duke Energy:

001-104.2
A request for administrative changes 
shall be submitted to the Transmission Provider on OASIS with explicit reference to the associated NITS Application.
001-104.3
A request for administrative changes shall be submitted as pre-confirmed. 

NCEMC:

001-104.3
A request for administrative changes shall be submitted as pre-confirmed. 

001-104.4
The Transmission Provider will accept or deny the request within 7 days of receipt of the request. The Transmission Provider will provide a reason for denial of a request for administrative change.

SPP:

001-104.4
The Transmission Provider will accept or deny the request within 7
 days of receipt of the request. The Transmission Provider will provide a reason for denial of a request for administrative change.

Duke Energy:

001-104.4
The Transmission Provider will accept or deny the request within 7 days of receipt of the request. The Transmission Provider will provide a reason for denial of a request for administrative change.

001-104.5
The following types of requests will be considered administrative changes:

· Requests to change customer description information

· Requests to change agent information

· Generation Information
· Transmission Customer System Information
Duke Energy:

001-104.5
The following types of requests 
will be considered administrative changes:

· Requests to change customer description information

· Requests to change agent information

· Generation Information

· Transmission Customer System Information

BPA:

001-104.5
The following types of requests may be considered administrative changes:

· Requests to change non-material customer description information

· Requests to change non-material agent information

· Requests to change non-material generation information
· Requests to change non-material
· Transmission Customer system information
Entergy:

001-104.5
The following types of requests will be considered administrative changes:

· Requests to change customer description information

· Requests to change agent information

· Generation Information
· Transmission Customer System Information

001-105
NETWORK INTEGRATION TRANSMISSION SERVICE – MODIFICATION TO SERVICE

Southern:

001-105
NITS – MODIFICATION TO SERVICE

001-105.1
General Requirements 

OATI:

001-105.1
General Requirements 

001-105.1.1
The Network Customer shall have the right to request modification to service pursuant to the NITS Application. 

Entergy:

001-105.1.1
The Network Customer shall have the right to request modification to service, which shall modify the customer’s NITS granted pursuant to the NITS Application. 

Entergy:(added section)

001- 105.1.2 
The Network Customer shall submit, or arrange for the submittal of, the request for modification to service on the Transmission Provider’s OASIS in accordance with the technical requirements of WEQ Standards WEQ-002, WEQ-003 and WEQ-013. 

Entergy:(added section)

001-105.1.3
OASIS shall assign a unique identifier to each request for modification to service.  

001-105.1.2
A request for modification to service shall be submitted to the Transmission Provider on OASIS with explicit reference to the associated NITS Application.

Entergy:

001-105.1.4
A request for modification to service shall be submitted to the Transmission Provider on OASIS with explicit reference to the associated NITS Application.

001-105.1.3
A request for modification to service shall be queued and evaluated in the same manner as any other request, subject to the other requirements of the Business Practice Standards. 

Southern:

001-105.1.3
A request for modification to service shall be queued and evaluated in the same manner as any other request, subject to the other requirements of the NITS Business Practice Standards. 

Duke Energy:

001-105.1.3
A request for modification to service shall be queued
 and evaluated in the same manner as any other request, subject to the other requirements of the Business Practice Standards. 

Entergy:

001-105.1.5
A request for modification to service shall be queued and evaluated in the same manner as any other request, subject to the other requirements of the Business Practice Standards. 

001-105.1.4
The following timing requirements shall apply to all requests for secondary network service and modifications to service requests, including NSR, DNR, and DNL requests:

NCEMC:

001-105.1.4
The following timing requirements shall apply to all requests for secondary network service and modifications to service requests, including NSR, DNR, and DNL requests:

Entergy:

001-105.1.6
The following timing requirements shall apply to all requests for modifications to service requests, including NSR, DNR, and DNL requests, as well as all requests for secondary network service:

Table 105-a

NITS ReqUest Timing Requirements8

NCEMC:

Table 105-a

NITS ReqUest Timing Requirements8



	Class 
	Duration
Duke Energy:

Service Increment 

	Time Queued Prior to Start


	Provider Evaluation Time Limit1
	Customer Confirmation Time Limit2 After ACCEPTED 

or COUNTEROFFER3
	Provider Counter Time Limit after REBID4

	Secondary Network (Non-firm)
	Hourly

Hourly

Hourly

Daily

Weekly

Monthly
	< 1 hour

> 1 hour

Day ahead

N/A

N/A

N/A
	Best effort

30 minutes

30 minutes

30 minutes

4 hours

2 days5
	5 minutes

5 minutes

30 minutes

2 hours

24 hours

24 hours
	5 minutes

5 minutes

10 minutes

10 minutes

4 hours

4 hours

	DNR/DNL/NSR (firm)
	Daily

Daily

Weekly

Monthly

Yearly
	<24 hours

N/A

N/A

N/A

60 days7
	Best effort

30 days6
30 days6
30 days6
30 days6
	2 hours

24 hours

48 hours

4 days

15 days
	30 minutes

4 hours

4 hours

4 hours

4 hours

	Temporary Termination of Network Resource or Network Load
Entergy:

Temporary Termination of Network Resource, Network Load or Network Scheduling Right


	Hourly

Hourly

Hourly

Daily

Daily

Weekly

Monthly

Yearly
	< 1 hour

> 1hour

Day ahead

<24 hours

N/A

N/A

N/A

60 days7
	Best effort

30 min

30 min

Best effort

30 days6
30 days6
30 days6
30 days6
	5 minutes

5 minutes

30 minutes

2 hours

24 hours

48 hours

4 days

15 days
	5 minutes

5 minutes

10 min

30 min

4 hours

4 hours

4 hours

4 hours

	Indefinite Termination of Network Resource or Network Load

Entergy:

Indefinite Termination of Network Resource, Network Load or Network Scheduling Right


	N/A
	Best effort
	Best effort
	Best effort
	N/A



Notes for Table 105-A:

.
1
Consistent with regulations and filed tariffs, measurement starts at the time the request is QUEUED.

Duke Energy:

1
Consistent with regulations and filed tariffs, measurement starts at the time the request is QUEUED
.

2
Confirmation time limits are not to be interpreted to extend scheduling deadlines or to override pre-emption deadlines.

Duke Energy:

2
Confirmation time limits are not to be interpreted to extend scheduling deadlines 
or to override pre-emption deadlines.

3
Measurement starts at the time the request is first moved to either ACCEPTED or COUNTEROFFER.  The time limit does not reset on subsequent changes of state.

4
Measurement starts at the time the Network Customer changes the state to REBID.  The measurement resets each time the request is changed to REBID.

5
Days are defined as calendar days.

6

Transmission Providers shall make best efforts to respond within 72 hours or prior to the scheduling deadline, whichever is earlier.

7
Whenever feasible and on a nondiscriminatory basis, Transmission Providers should accommodate requests made with less than 60 days notice.

8 
Each of the single requests in a concomitant evaluation shall have the timing requirement specified above for that type of request or, if applicable for PTP, the timing requirement specified in Table 4.2, Request Timing Requirements. .

Duke Energy:

8 
Each of the single requests in a concomitant evaluation shall have the timing requirement specified above for that type of request and, when applicable for PTP, the timing requirement specified in Table 4.2, Request Timing Requirements
.

Entergy
:

001-105.1.5
The following requests are considered modifications of service:

· Request for designation of a new Network Resource

· Request for temporary termination of Network Resource with redesignation of same Network Resource
· Request for indefinite termination of Network Resource

· Request for designation of new Network Load

· Request for indefinite termination of Network Load

· Request for new Network Scheduling Rights
· Request for temporary termination of Network Scheduling Rights

· Request for indefinite termination of Network Scheduling Rights
· Request for Secondary Network Transmission Service 
· Request for termination of Secondary Network Transmission Service
Southern:

001-105.1.5
The following requests are considered modifications of service:
· Request for designation of a new Network Resource

· Request for temporary termination of Network Resource with redesignation of same Network Resource

· Request for indefinite termination of Network Resource

· Request for designation of new Network Load

· Request for indefinite termination of Network Load

· Request for new NSRs
· Request for temporary termination of NSRs

· Request for indefinite termination of NSRs
· Request for Secondary Network Transmission Service 
· Request for temporary/indefinite termination of Secondary Network Transmission Service

Entergy:

001-105.1.7
The following requests are considered modifications of service:

· Request for designation of a new Network Resource

· Request for temporary termination of Network Resource with redesignation of same Network Resource

· Request for indefinite termination of Network Resource

· Request for designation of new Network Load

· Request for indefinite termination of Network Load

· Request for new Network Scheduling Rights
· Request for temporary termination of Network Scheduling Rights

· Request for indefinite termination of Network Scheduling Rights
· Request for Secondary Network Transmission Service 
· Request for termination of Secondary Network Transmission Service

001-105.2
Designation of New Network Resource 

OATI:

001-105.2
Designation of New Network Resource 

001-105.2.1
Submission of Request for Designation of New Network Resource

001-105.2.1.1
The Network Customer may enter and update information on OASIS in association with a request for designation of new Network Resource prior to submission to the Transmission Provider by setting the status of the request to PENDING.

EPSA:

001-105.2.1.1
The Network Customer may enter and update information on OASIS in association with a request for designation of new Network Resource prior to submission to the Transmission Provider by setting the status of the request to PENDING.

Entergy:

001-105.2.1.1
The Network Customer may enter and update information on OASIS to support a request for designation of new Network Resource prior to submission to the Transmission Provider by setting the status of the request to PENDING.

001- 105.2.1.2
The Transmission Provider may specify, in its posted business practice, the minimum information required to designate a new Network Resource
Southern:

.001- 105.2.1.2
The Transmission Provider may specify, in its posted Business Practice, the minimum information required to designate a new Network Resource
Duke Energy:

001- 105.2.1.2

The Transmission Provider may specify, in its posted business practice, the minimum information required to designate a new Network Resource.  

NCEMC:

001- 105.2.1.2
The Transmission Provider may specify, in its posted business practice, the minimum information required to designate a new Network Resource

Entergy:

001- 105.2.1.2
The Transmission Provider may specify, in a posted business practice, the minimum information required to designate a new Network Resource

001- 105.2.1.3
The Network Customer shall indicate to the Transmission Provider that the request for designation of new Network Resource is ready for Transmission Provider review by submitting the request on OASIS.  

Duke Energy:

001- 105.2.1.3
The Network Customer shall indicate to the Transmission Provider that the request for designation of new Network Resource is ready for Transmission Provider review by submitting the request on OASIS
.  

001-105.2.1.3.1
OASIS shall set the queue time for the request for designation of new Network Resource to the time that the Network Customer submits the request, which shall set the status of the request to QUEUED.

Entergy:

001-105.2.1.3.1
OASIS shall set the queue time for the request for designation of new Network Resource to the time that the Network Customer submits the requestand shall set the status of the request to QUEUED.

001-105.2.1.3.2
The Transmission Provider shall acknowledge the queued request for designation of new Network Resource by setting the status of the request to RECEIVED.

001-105.2.2
Initial Review of Request for Designation of New Network Resource

001- 105.2.2.1
The Transmission Provider shall review the request for designation of New Network Resource to determine if the request: (1) meets the minimum requirements specified in the Transmission Provider’s posted business practices; or (2) is complete consistent with the Transmission Provider’s tariff requirements.

Southern:

001- 105.2.2.1
The Transmission Provider shall review the request for designation of New Network Resource to determine if the request: (1) meets the minimum requirements specified in the Transmission Provider’s posted Business Practices; or (2) is complete consistent with the Transmission Provider’s tariff requirements.

Duke Energy:

001- 105.2.2.1
The Transmission Provider shall review the request for designation of New Network Resource to determine if the request: (1) meets the minimum requirements specified in the Transmission Provider’s posted business practices; or
 (2) is complete consistent with the Transmission Provider’s tariff requirements.

001- 105.2.2.2
The Transmission Provider shall respond to the request by setting the status to one of the following:

I.
INVALID 

II.
DEFICIENT 

III.
COMPLETED 

001-105.2.3
Invalid Request for Designation of New Network Resource

001-105.2.3.1
If the required information, as specified in Business Practice Standard WEQ-001-105.2.2.1, is not provided, the Transmission Provider shall set the status of the request for designation of new Network Resource to INVALID and the request shall be deemed withdrawn.

BPA:

001-105.2.3.1
If the required information, as specified in Business Practice Standard WEQ-001-105.2.2.1, is not provided, the Transmission Provider shall set the status of the request for designation of new Network Resource to INVALID and the request shall be deemed withdrawn and will receive no further consideration.

Entergy:

001-105.2.3.1
If the required information, as specified in Business Practice Standard WEQ-001-105.2.2.1, has not been provided in the request for designation of new Network Resource, the Transmission Provider shall set the status of the request for designation of new Network Resource to INVALID and the request shall be deemed withdrawn.

001-105.2.3.2
In response to a state of INVALID, the Network Customer may submit a new or revised request for designation of new Network Resource, which will be assigned a new unique identifier and a new queue time.   

Duke Energy:

001-105.2.3.2
In response to a state of INVALID, the Network Customer may submit a new or revised request for designation of new Network Resource, which will be assigned a new unique identifier 
and a new queue time.   

001-105.2.4
Deficient Request for Designation of New Network Resource
001-105.2.4.1
For each request for designation of new Network Resource which is not INVALID, the Transmission Provider shall determine if the request can be deemed sufficient.
SPP:

  001-105.2.4.1
For each valid request for designation of new Network Resource
, the Transmission Provider shall determine if the request can be deemed sufficient.

Entergy:

001-105.2.4.1
For each request for designation of new Network Resource that is not INVALID, the Transmission Provider shall determine if the request can be deemed sufficient.

001-105.2.4.1.1
If deemed sufficient, the Transmission Provider shall set the status of such request to COMPLETED.  

001-105.2.4.1.2
If not deemed sufficient, the Transmission Provider shall set the status of such request to DEFICIENT. 

Duke Energy:

001-105.2.4.1.2
If not deemed sufficient
, the Transmission Provider shall set the status of such request to DEFICIENT. 

001-105.2.4.2
A request for designation of a new Network Resource shall be accompanied by an attestation in accordance with the Transmission Provider’s tariff. 

Duke Energy:

001-105.2.4.2
A request for designation of a new Network Resource shall be accompanied by an attestation in accordance with the Transmission Provider’s tariff. 

NCEMC:

001-105.2.4.2
A request for designation of a new Network Resource shall be accompanied by an attestation in accordance with the Transmission Provider’s tariff. 

EPSA:

001-105.2.4.2
A request for designation of a new Network Resource shall be accompanied by an attestation in accordance with the Transmission Provider’s tariff. 

GTC:

001-105.2.4.2
An attestation must be submitted prior to thenew Network Resource service request being confirmed and in accordance with the Transmission Provider’s tariff
. 

GTC: add section

001-105.2.4.2.1
A request for designation of a new preconfirmed Network Resource shall be accompanied by an attestation in accordance with the Transmission Provider’s tariff
.

001-105.2.4.3
If the Transmission Provider determines that the request for designation of new Network Resource is DEFICIENT, the Transmission Provider shall set the request to DEFICIENT and begin informal communications with the Network Customer to resolve the deficiencies.
Southern:

 001-105.2.4.3
If the Transmission Provider determines that the request for designation of new Network Resource is deficient, the Transmission Provider shall set the request to DEFICIENT and begin informal communications with the Network Customer to resolve the deficiencies.

Entergy:

001-105.2.4.3
If the Transmission Provider determines that the request for designation of new Network Resource is DEFICIENT (which deficiency may include the failure to provide the attestation required in WEQ 001-105.2.4.3), the Transmission Provider shall set the request to DEFICIENT and begin informal communications with the Network Customer to resolve the deficiencies.

001-105.2.4.4
The Network Customer shall remedy the deficiency and shall set the status of the request for designation of new Network Resource to REEVALUATE.

001-105.2.4.5
Upon notification that the Network Customer has provided the necessary information, either on or off OASIS as appropriate, to remedy an identified deficiency, the Transmission Provider shall review the provided information and take one of the following actions:

001-105.2.4.5.1
If the information provided fails to remedy the deficiency, the Transmission Provider shall set the status of the request for designation of new Network Resource to DEFICIENT.

001-105.2.4.5.2
If the information provided remedies the deficiency, the Transmission Provider shall set the status of the request for designation of new Network Resource to COMPLETED.

001-105.2.4.5.3
If the information provided is deemed by the Transmission Provider to be a significant change to the request for designation of new Network Resource, the Transmission Provider shall set the status of the request to DECLINED.

NCEMC:

001-105.2.4.5.3
If the information provided is deemed by the Transmission Provider to be a significant change to the request for designation of new Network Resource, the Transmission Provider shall set the status of the request to DECLINED.

001-105.2.4.6
If the Network Customer fails to respond to the Transmission Provider’s notice of a DEFICIENT request for designation of new Network Resource, or fails to remedy the deficiency to the Transmission Provider’s satisfaction in a timely manner, the Transmission Provider shall set the status of the request to DECLINED.

Southern:

001-105.2.4.6
If the Network Customer fails to respond to the Transmission Provider’s notice of a deficient request for designation of new Network Resource, or fails to remedy the deficiency to the Transmission Provider’s satisfaction in a timely manner, the Transmission Provider shall set the status of the request to DECLINED.

Duke Energy:

001-105.2.4.6
If the Network Customer fails to respond to the Transmission Provider’s notice of a DEFICIENT request for designation of new Network Resource, or fails to remedy the deficiency to the Transmission Provider’s satisfaction in a timely manner
, the Transmission Provider shall set the status of the request to DECLINED.

001-105.2.4.7
In response to a state of DECLINED, the Network Customer may submit a new or revised request for designation of new Network Resource, which will be assigned a new unique identifier and a new queue time.  

001-105.2.5
Completed Request for Designation of New Network Resource

001-105.2.5.1
Once the Transmission Provider has set the status of the request for designation of new Network Resource to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these Business Practice Standards.

Southern:

001-105.2.5.1
Once the Transmission Provider has set the status of the request for designation of new Network Resource to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these NITS Business Practice Standards.

Duke Energy:

001-105.2.5.1
Once the Transmission Provider has set the status of the request for designation of new Network Resource to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these 
Business Practice Standards.

001-105.2.6
Review of Completed Request for Designation of New Network Resource

001- 105.2.6.1
Once the Transmission Provider has set the status of the request for designation of new Network Resource to COMPLETED, the Transmission Provider shall respond to the request, consistent with filed tariffs, within the Provider Response Time Limit defined in Table 105-A NITS Request Timing Requirements. The time limit is measured from the time the request is QUEUED.  To indicate its evaluation of the request, a Transmission Provider may respond by setting the state of the request to one of the following:
I.
STUDY

II.
DECLINED

III.
REFUSED 

IV.
COUNTEROFFER

V.
ACCEPTED 

Duke Energy:

001- 105.2.6.1
Once the Transmission Provider has set the status of the request for designation of new Network Resource to COMPLETED, the Transmission Provider shall respond to the request, consistent with filed tariffs, within the Provider Response Time Limit defined in Table 105-A NITS Request Timing Requirements. The time limit is measured from the time the request 
is QUEUED.  To indicate its evaluation of the request, a Transmission Provider may respond by setting the state of the request to one of the following:
I.
STUDY

II.
DECLINED

III.
REFUSED 

IV.
COUNTEROFFER



V.     ACCEPTED
Entergy:

001- 105.2.6.1
Once the Transmission Provider has set the status of the request for designation of new Network Resource to COMPLETED, the Transmission Provider shall respond to the request, consistent with filed tariffs, within the Provider Response Time Limit defined in Table 105-A NITS Request Timing Requirements. The time limit is measured from the time the request is QUEUED.  To indicate its evaluation of the request, a Transmission Provider may respond by setting the state of the request to one of the following:
I.
STUDY

II.
DECLINED

III.
REFUSED 

IV.
COUNTEROFFER

V.
ACCEPTED 

001-105.2.7
Study of Request for Designation of New Network Resource

Duke Energy:

001-105.2.7
Study of Request for Designation of New Network Resource

001-105.2.7.1 
If the Transmission Provider determines that some level of study is required or is being performed, the Transmission Provider shall set the status of the request for designation of new Network Resource to STUDY.

001-105.2.7.2
From the status of STUDY, the Transmission Provider shall set the status of the request for designation of new Network Resource to DECLINED, REFUSED, COUNTEROFFER, or ACCEPTED.

001-105.2.8
Decline of Request for Designation of New Network Resource

001-105.2.8.1
If, at any time in the evaluation of the request for designation of new Network Resource or conduct of any associated study(ies), the Transmission Provider determines that the Network Customer has failed to meet any of the terms and conditions for the designation of a new Network Resource (e.g., failure to execute a study agreement, etc.), the Transmission Provider shall set the status of the request to DECLINED.

001-105.2.8.2
In response to a state of DECLINED, the Eligible Transmission Customer may submit a new or revised request for designation of new Network Resource, which will be assigned a new unique identifier and a new queue time.   

001-105.2.9
Refusal of Request for Designation of New Network Resource

001-105.2.9.1
If, at any time in the evaluation of the request for designation of new Network Resource or conduct of any associated study(ies), the Transmission Provider determines that the requested service cannot be provided due to lack of availability of transfer capability consistent with the Transmission Provider’s tariff requirements, the Transmission Provider may set the status of the request to REFUSED.

Duke Energy:

001-105.2.9.1
If, at any time in the evaluation of the request for designation of new Network Resource or conduct of any associated study(ies), the Transmission Provider determines that the requested service cannot be provided due to lack of availability of transfer capability
 consistent with the Transmission Provider’s tariff requirements, the Transmission Provider may set the status of the request to REFUSED.

001-105.2.9.2
In response to a state of REFUSED, the Network Customer may submit a new or revised request for designation of new Network Resource, which will be assigned a new unique identifier and a new queue time.  

001-105.2.10
Counteroffer to Request for Designation of New Network Resource

001-105.2.10.1
When, in the evaluation of a request for designation of new Network Resource or on completion of any applicable study(ies), the Transmission Provider determines that the request may be granted in part, the Transmission Provider may make a partial offer.  

Duke Energy:

001-105.2.10.1
When, in the evaluation of a request for designation of new Network Resource or on completion of any applicable study(ies), the Transmission Provider determines that the request may be granted in part, the Transmission Provider may make a partial offer
.  

001-105.2.10.1.1
If the Transmission Provider makes a partial offer, the Transmission Provider shall notify the Network Customer on OASIS by setting the amount of Network Resource capacity granted over time in the response to the request for designation of new Network Resource and setting the status of such request to COUNTEROFFER.

001-105.2.11
Acceptance of Request for Designation of New Network Resource

001-105.2.11.1
When, in the evaluation of the request for designation of new Network Resource or on completion of any applicable study(ies), the Transmission Provider determines that the request can be approved, the Transmission Provider shall set the status of the request to ACCEPTED.

001-105.2.12
Customer Actions

001-105.2.12.1
From ACCEPTED or COUNTEROFFER, the Network Customer may change the status of the request for designation of new Network Resource to CONFIRMED, REBID or WITHDRAWN. The Network Customer has the amount of time designated as Customer Confirmation Time Limit in Table 105-A NITS Request Timing Requirements to change the state of the request. The Customer Confirmation Time Limit is measured from the first time the request is moved to ACCEPTED or COUNTEROFFER, and is not reset with subsequent iterations of negotiation.
Entergy:

001-105.2.12.1
From ACCEPTED or COUNTEROFFER, the Network Customer may change the status of the request for designation of new Network Resource to CONFIRMED, REBID or WITHDRAWN. The Network Customer has the amount of time designated as Customer Confirmation Time Limit in Table 105-A NITS Request Timing Requirements to change the state of the request. The Customer Confirmation Time Limit is measured from the first time the request is moved to ACCEPTED or COUNTEROFFER, and is not reset with subsequent iterations of negotiation.

001-105.2.12.1.1
The Network Customer shall use the request status of REBID to initiate or continue the negotiation of designated Network Resource capacity.

Entergy:

001-105.2.12.1.1
The Network Customer shall use the request status of REBID to initiate negotiation of designated Network Resource capacity or engage in continued negotiation of the designated Network Resource capacity with the Transmission Provider.

01-105.2.12.1.2
Should the Network Customer elect to initiate or continue the negotiation of designated Network Resource capacity by moving the request to REBID, the Transmission Provider shall respond by taking the request to a DECLINED, SUPERSEDED, ACCEPTED, or COUNTEROFFER state within the Provider Counter Time Limit, specified in Table 105-A NITS Request Timing Requirements. The Transmission Provider response time is measured from the most recent REBID time.

SPP:

01-105.2.12.1.2
Should the Network Customer elect to initiate or continue the negotiation of designated Network Resource capacity by moving the request to REBID, the Transmission Provider shall respond by taking the request to a DECLINED, SUPERSEDED, ACCEPTED, or COUNTEROFFER state within the Provider Counter Time Limit as specified in Table 105-A NITS Request Timing Requirements. The Transmission Provider response time is measured from the most recent REBID time.

Entergy:

01-105.2.12.1.2
Should the Network Customer elect to initiate or continue the negotiation of designated Network Resource capacity by moving the request to REBID, the Transmission Provider shall respond by taking the request to a DECLINED, SUPERSEDED, ACCEPTED, or COUNTEROFFER state within the Provider Counter Time Limit, specified in Table 105-A NITS Request Timing Requirements. The Transmission Provider response time is measured from the most recent REBID time.

001-105.2.12.1.3
The Network Customer may negotiate for a lower amount of designated Network Resource capacity by updating the capacity requested over time and changing the request status from ACCEPTED or COUNTEROFFER to REBID. For example, the Network Customer may choose to coordinate the level of transmission service purchased across multiple Transmission Providers. Negotiation for a higher amount of designated Network Resource capacity shall not be allowed. A Network Customer seeking additional designated Network Resource capacity must submit a new request, which will be assigned a new unique identifier and a new queue time.

SPP:

001-105.2.12.1.3
The Network Customer may negotiate for a lower amount of designated Network Resource capacity by updating the capacity requested and
 changing the request status from ACCEPTED or COUNTEROFFER to REBID. For example, the Network Customer may choose to coordinate the level of transmission service purchased across multiple Transmission Providers. Negotiation for a higher amount of designated Network Resource capacity shall not be allowed. A Network Customer seeking additional designated Network Resource capacity must submit a new request, which will be assigned a new unique identifier and a new queue time.

Duke Energy:

001-105.2.12.1.3
The Network Customer may negotiate for a lower amount of designated Network Resource capacity by updating the capacity requested over time and changing the request status from ACCEPTED or COUNTEROFFER to REBID. For example, the Network Customer may choose to coordinate the level of transmission service purchased across multiple Transmission Providers. Negotiation for a higher amount of designated Network Resource capacity shall not be allowed. 
A Network Customer seeking additional designated Network Resource capacity must submit a new request, which will be assigned a new unique identifier and a new queue time.

Entergy:

001-105.2.12.1.3
The Network Customer may negotiate for a lower amount of designated Network Resource capacity by updating the capacity requested over time and changing the request status from ACCEPTED or COUNTEROFFER to REBID. For example, the Network Customer may choose to coordinate the level of transmission service purchased across multiple Transmission Providers. Negotiation for a higher amount of designated Network Resource capacity shall not be allowed. A Network Customer seeking additional designated Network Resource capacity must submit a new request, which will be assigned a new unique identifier and a new queue time.

001-105.2.12.1.4
Once the status of the request for designation of a new Network Resource has been set to ACCEPTED or COUNTEROFFER, the Network Customer may change the status to CONFIRMED after positively acknowledging agreement by setting the requested designated Network Resource capacity equal to the Transmission Provider’s granted designated Network Resource capacity.  Alternatively, the Network Customer may withdraw the request or the Network Customer may continue negotiations consistent with Business Practice Standard WEQ-001-105.2.12.1.2.
001-105.2.13
Final Disposition of Request for Designation of New Network Resource

001-105.2.13.1
Once CONFIRMED, the request for designation of new Network Resource will be incorporated into the associated CONFIRMED NITS Application.

001-105.2.13.2
If, within the time limits specified in Table 105-A NITS Request Timing Requirements, the Network Customer fails to set the status of the ACCEPTED request for designation of new Network Resource to CONFIRMED or WITHDRAWN, the Transmission Provider may set the status of the request to RETRACTED.
SPP:

 001-105.2.13.2
If
, the Network Customer fails to set the status of the ACCEPTED request for designation of new Network Resource to CONFIRMED or WITHDRAWN, within the time limits specified in Table 105-A NITS Request Timing Requirements, the Transmission Provider may set the status of the request to RETRACTED.

001-105.2.13.2.1
In response to a state of RETRACTED, the Network Customer may submit a new or revised request for designation of new Network Resource, which will be assigned a new unique identifier and a new queue time.

001-105.2.13.3
Should the Network Customer elect to initiate or continue the negotiation of Network Resource capacity by changing the status of the request for designation of new Network Resource to REBID, the Transmission Provider shall respond by changing the status of the request to DECLINED, ACCEPTED, or COUNTEROFFER within the Provider Counter Time Limit, specified in Table 105-A NITS Request Timing Requirements. The Transmission Provider response time is measured from the most recent REBID time
.

SPP:

001-105.2.13.3
Should the Network Customer elect to initiate or continue the negotiation of Network Resource capacity by changing the status of the request for designation of new Network Resource to REBID, the Transmission Provider shall respond by changing the status of the request to DECLINED, ACCEPTED, or COUNTEROFFER within the Provider Counter Time Limit as specified in Table 105-A NITS Request Timing Requirements. The Transmission Provider response time is measured from the most recent REBID time.

Duke Energy:

001-105.2.13.3
Should the Network Customer elect to initiate or continue the negotiation of Network Resource capacity by changing the status of the request for designation of new Network Resource to REBID, the Transmission Provider shall respond by changing the status of the request to DECLINED, ACCEPTED, or COUNTEROFFER within the Provider Counter Time Limit, specified in Table 105-A NITS Request Timing Requirements. The Transmission Provider response time is measured from the most recent REBID time.

NCEMC:

001-105.2.13.3
Should the Network Customer elect to initiate or continue the negotiation of Network Resource capacity by changing the status of the request for designation of new Network Resource to REBID, the Transmission Provider shall respond by changing the status of the request to DECLINED, ACCEPTED, or COUNTEROFFER within the Provider Counter Time Limit, specified in Table 105-A NITS Request Timing Requirements. The Transmission Provider response time is measured from the most recent REBID time.

Entergy:

001-105.2.13.3.1
The Transmission Provider shall positively acknowledge agreement to the designation of new Network Resource capacity over time by setting the granted Network Resource capacity equal to the Network Customer’s requested Network Resource capacity prior to acceptance of the request.
SPP:

001-105.2.13.3.1
The Transmission Provider shall positively acknowledge agreement to the designation of new Network Resource capacity by setting the granted Network Resource capacity equal to the Network Customer’s requested Network Resource capacity prior to acceptance of the request.

Duke Energy:

001-105.2.13.3.1
The Transmission Provider shall positively acknowledge agreement to the designation of new Network Resource capacity over time by setting the granted Network Resource capacity equal to the Network Customer’s requested Network Resource capacity prior to acceptance of the request.   

Entergy:

001-105.3
Temporary Termination
 of Network Resource

OATI:

001-105.3
Temporary Termination
 of Network Resource

001-105.3.1
Request for Temporary Termination of Network Resource
001-105.3.1.1
The Transmission Provider shall not refuse a valid and complete request for temporary termination of Network Resource.

Duke Energy:

001-105.3.1.1
The Transmission Provider shall not refuse a valid and complete request for temporary termination
 of Network Resource.

NCEMC:

001-105.3.1.1
The Transmission Provider shall not refuse a valid and complete request for temporary termination of Network Resource.

001-105.3.1.2
A request for temporary termination of Network Resource must include a redesignation of the same resource.

Duke Energy:

001-105.3.1.2
A request for temporary termination of Network Resource must include a redesignation of the same resource.

NCEMC:

001-105.3.1.2
A request for temporary termination of Network Resource must include a redesignation of the same resource.

001-105.3.2
Submission of Request for Temporary Termination of Network Resource 

001-105.3.2.1
The Network Customer may enter and update information on OASIS in association with a request for temporary termination of Network Resource prior to submission to the Transmission Provider by setting the status of the request to PENDING.

Entergy:

001-105.3.2.1
The Network Customer may enter and update information on OASIS to support with a request for temporary termination of Network Resource prior to submission to the Transmission Provider by setting the status of the request to PENDING.

001- 105.3.2.2
The Network Customer shall indicate to the Transmission Provider that the request for temporary termination of Network Resource is ready for Transmission Provider review by submitting the request on OASIS.  

001-105.3.2.3
OASIS shall set the queue time for the request for temporary termination of Network Resource to the time that the Network Customer submits the request, which shall set the request to QUEUED.

Duke Energy:

001-105.3.2.3
OASIS shall set the queue time for the request for temporary termination of Network Resource to the time that the Network Customer submits the request, which 
shall set the request to QUEUED.

Entergy:

001-105.3.2.3
OASIS shall set the queue time for the request for temporary termination of Network Resource to the time that the Network Customer submits the request and shall set the request to QUEUED.

001-105.3.2.4
The Transmission Provider shall acknowledge the queued request for temporary termination of a Network Resource by setting the status of the request to RECEIVED.

001-105.3.3
Initial Review of Request for Temporary Termination of Network Resource 

001- 105.3.3.1
The Transmission Provider shall review the request for temporary termination of Network Resource to determine if the request: (1) meets the minimum requirements specified in the Transmission Provider’s posted business practices; and (2) is complete consistent with the Transmission Provider’s tariff requirements.

Southern:

001- 105.3.3.1
The Transmission Provider shall review the request for temporary termination of Network Resource to determine if the request: (1) meets the minimum requirements specified in the Transmission Provider’s posted Business Practices; and (2) is complete consistent with the Transmission Provider’s tariff requirements.

NCEMC:

001- 105.3.3.1
The Transmission Provider shall review the request for temporary termination of Network Resource to determine if the request: (1) meets the minimum requirements specified in the Transmission Provider’s posted business practices; and (2) is complete consistent with the Transmission Provider’s tariff requirements.

001- 105.3.3.2
The Transmission Provider shall respond to a request by setting the status to one of the following:

I.
INVALID 

II.
DEFICIENT 

III.
COMPLETED 

001-105.3.4
Invalid Request for Temporary Termination of Network Resource 

001-105.3.4.1
If the information necessary to process the request for temporary termination of a Network Resource is not provided, the Transmission Provider shall set the state of the request for temporary termination of Network Resource to INVALID and the request shall be deemed withdrawn.

Entergy:

001-105.3.4.1
If the information necessary to process the request for temporary termination of a Network Resource has not been provided, the Transmission Provider shall set the state of the request for temporary termination of Network Resource to INVALID and the request shall be deemed withdrawn.

001-105.3.4.2
In response to a state of INVALID, the Network Customer may submit a new or revised request for temporary termination of Network Resource, which will be assigned a new unique identifier and a new queue time. 

Duke Energy:

001-105.3.4.2
In response to a state of INVALID, the Network Customer may submit a new or revised request for temporary termination of Network Resource
, which will be assigned a new unique identifier and a new queue time. 

001-105.3.5
Deficient Request for Temporary Termination of Network Resource 

001-105.3.5.1
For each request for temporary termination of Network Resource which is not INVALID, the Transmission Provider shall determine if the request can be deemed sufficient. 
Entergy:

001-105.3.5.1
For each request for temporary termination of Network Resource that is not INVALID, the Transmission Provider shall determine if the request can be deemed sufficient. 

001-105.3.5.1.1
If deemed sufficient, the Transmission Provider shall set the status of such request to COMPLETED.  

001-105.3.5.1.2
If not deemed sufficient, the Transmission Provider shall set the status of such request to DEFICIENT

Duke Energy:

001-105.3.5.1.2
If not deemed sufficient
, the Transmission Provider shall set the status of such request to DEFICIENT
001-105.3.5.2
A request for redesignation of a Network Resource shall meet the following requirements:

001-105.3.5.2.1
The request shall be accompanied by an attestation in accordance with the Transmission Provider’s tariff. 

Duke Energy:

001-105.3.5.2.1
The request shall be accompanied by an attestation in accordance with the Transmission Provider’s tariff. 


001-105.3.5.2.2
The STOP_TIME for the temporary termination of Network Resource is the effective date and time that the same Network Resource is redesignated.

Duke Energy:

001-105.3.5.2.2
The STOP_TIME 
for the temporary termination of Network Resource is
 the effective date and time that the same Network Resource is redesignated.

001-105.3.5.2.3
The request shall include all data necessary consistent with the requirements for designation of a new Network Resource.  Previously submitted data may be incorporated by reference with the exception of the attestation required in Business Practice Standard WEQ 001-105.3.5.2.

001-105.3.5.3
If the Transmission Provider determines that the request for temporary termination of Network Resource is DEFICIENT, the Transmission Provider shall set the request to DEFICIENT and begin informal communications with the Eligible Transmission Customer to resolve the deficiencies.
Southern:

001-105.3.5.3
If the Transmission Provider determines that the request for temporary termination of Network Resource is deficient, the Transmission Provider shall set the request to DEFICIENT and begin informal communications with the Eligible Transmission Customer to resolve the deficiencies.

SPP:

001-105.3.5.3
If the Transmission Provider determines that the request for temporary termination of Network Resource is DEFICIENT, the Transmission Provider shall set the request to DEFICIENT and begin 
communications with the Eligible Transmission Customer to resolve the deficiencies.

001-105.3.5.4
The Network Customer shall remedy the deficiency and shall set the status of the request for temporary termination of Network Resource to REEVALUATE.

001-105.3.5.5
Upon notification that the Network Customer has provided the necessary information, either on or off OASIS as appropriate, to remedy an identified deficiency, the Transmission Provider shall review the provided information and take one of the following actions:

001-105.3.5.5.1
If the information provided fails to remedy the deficiency, the Transmission Provider shall set the status of the request for temporary termination of Network Resource to DEFICIENT.

001-105.3.5.5.2
If the information provided remedies the deficiency, the Transmission Provider shall set the status of the request for temporary termination of Network Resource to COMPLETED.

001-105.3.5.5.3
If the information provided is deemed by the Transmission Provider to be a significant change to the request for temporary termination of Network Resource, the Transmission Provider shall set the status of the request to DECLINED.

NCEMC:

001-105.3.5.5.3
If the information provided is deemed by the Transmission Provider to be a significant change to the request for temporary termination of Network Resource, the Transmission Provider shall set the status of the request to DECLINED.

001-105.3.5.6
If the Network Customer fails to respond to the Transmission Provider’s notice of a DEFICIENT request for temporary termination of Network Resource, or fails to remedy the deficiency to the Transmission Provider’s satisfaction in a timely manner, the Transmission Provider shall set the status of the request to DECLINED.

Southern:

001-105.3.5.6
If the Network Customer fails to respond to the Transmission Provider’s notice of a deficient request for temporary termination of Network Resource, or fails to remedy the deficiency to the Transmission Provider’s satisfaction in a timely manner, the Transmission Provider shall set the status of the request to DECLINED.

001-105.3.5.7
In response to a state of DECLINED, the Network Customer may submit a new or revised request for temporary termination of Network Resource, which will be assigned a new unique identifier and a new queue time.  

001-105.3.6
Completed Request for Temporary Termination of Network Resource 

001-105.3.6.1
Once the Transmission Provider has set the status of the request for temporary termination of Network Resource to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these Business Practice Standards.

Southern:

001-105.3.6.1
Once the Transmission Provider has set the status of the request for temporary termination of Network Resource to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these NITS Business Practice Standards.

001-105.3.7
Acceptance of Request for Temporary Termination of Network Resource 

001-105.3.7.1
Once the status of the request for temporary termination of Network Resource is set to COMPLETED, the status of the request shall then be changed to ACCEPTED.

Entergy:

001-105.3.7.1
Once the status of the request for temporary termination of Network Resource is set to COMPLETED, OASIS shall set the status of the request to ACCEPTED.

001-105.3.8
Customer Actions

001-105. 3.8.1
After the status of the request for temporary termination of Network Resource has been set to ACCEPTED by the Transmission Provider, the Network Customer may confirm or withdraw the request by setting the status to CONFIRMED or WITHDRAWN.
Entergy:

001-105. 3.8.1
After the status of the request for temporary termination of Network Resource has been set to ACCEPTED by the Transmission Provider, the Network Customer may confirm or withdraw the request by setting the status to CONFIRMED or WITHDRAWN.  The Network Customer has the amount of time designated as Customer Confirmation Time Limit in Table 105-A NITS Request Timing Requirements to change the state of the request.  The Customer Confirmation Time Limit is measured from the first time the request is moved to ACCEPTED..

001-105.3.9
Final Disposition of Request for Temporary Termination of Network Resource 

001-105.3.9.1
Once CONFIRMED, the request for temporary termination of Network Resource will be incorporated into the associated CONFIRMED NITS Application.

NCEMC:

001-105.3.9.1
Once CONFIRMED, the request for temporary termination of Network Resource will be incorporated into the associated CONFIRMED NITS Application.

001-105.3.9.2
If, within the time limits specified in Table 4-A, the Network Customer fails to set the status of the ACCEPTED request for temporary termination of Network Resource to CONFIRMED or WITHDRAWN, the Transmission Provider may set the status of the request to RETRACTED. 

001-105.3.9.2.1
In response to a state of RETRACTED, the Network Customer may submit a new or revised request for temporary termination of Network Resource, which will be assigned a new unique identifier and a new queue time.   

001-105.4
Indefinite Termination of Network Resource

OATI:

001-105.4
Indefinite Termination of Network Resource

Duke Energy:

001-105.4
Indefinite Termination 
of Network Resource

001-105.4.1
Request for Indefinite Termination of Network Resource
001-105.4.1.1
The Transmission Provider shall not refuse a valid and complete request for indefinite termination of Network Resource.
001-105.4.2
Submission of Request for Indefinite Termination of Network Resource

001-105.4.2.1
The Network Customer may enter and update information on OASIS in association with a request for indefinite termination of Network Resource prior to submission to the Transmission Provider by setting the status of the request to PENDING. 

Entergy:

001-105.4.2.1
The Network Customer may enter and update information on OASIS to support with a request for indefinite termination of Network Resource prior to submission to the Transmission Provider by setting the status of the request to PENDING. 

001- 105.4.2.2
The Network Customer shall indicate to the Transmission Provider that the request for indefinite termination of Network Resource is ready for Transmission Provider review by submitting the request on OASIS.  

001-105.4.2.2.1
OASIS shall set the queue time for the request for indefinite termination of Network Resource to the time that the Network Customer submits the request, which shall set the request to QUEUED.

Entergy:

001-105.4.2.2.1
OASIS shall set the queue time for the request for indefinite termination of Network Resource to the time that the Network Customer submits the request and shall set the request to QUEUED.

001-105.4.2.2.2
The Transmission Provider shall acknowledge the queued request for indefinite termination of Network Resource by setting the status of the request to RECEIVED.

001-105.4.3
Initial Review of Request for Indefinite Termination of Network Resource

001- 105.4.3.1
The Transmission Provider shall review the request for indefinite termination of Network Resource to determine if the request is consistent with the Transmission Provider’s tariff requirements.

001- 105.4.3.2
The Transmission Provider shall respond to a request for indefinite termination of Network Resource by setting the status to one of the following:

I.
INVALID 

II.
COMPLETED

001-105.4.4
Invalid Request for Indefinite Termination of Network Resource

001-105.4.4.1
If the information necessary to process the request for indefinite termination of Network Resource is not provided, the Transmission Provider shall set the state of the request to INVALID and the request shall be deemed withdrawn.

NCEMC:

001-105.4.4.1
If the information necessary to process the request for indefinite termination of Network Resource is not provided, the Transmission Provider shall set the state of the request to INVALID and the request shall be deemed withdrawn.

Entergy:

001-105.4.4.1
If the information necessary to process the request for indefinite termination of Network Resource has not been provided, the Transmission Provider shall set the state of the request to INVALID and the request shall be deemed withdrawn.

001-105.4.4.2
In response to a state of INVALID, the Network Customer may submit a new or revised request for indefinite termination of Network Resource, which will be assigned a new unique identifier and a new queue time.   

001-105.4.5
Completed Request for Temporary Termination of Network Resource
Southern:

001-105.4.5
Completed Request for Indefinite Termination of Network Resource

Duke Energy:

001-105.4.5
Completed Request for Temporary
 Termination of Network Resource 

BC Hydro:

001-105.4.5
Completed Request for Indefinite Termination of Network Resource

Entergy:

001-105.4.5
Completed Request for Indefinite Termination of Network Resource

001-105.4.5.1
Once the Transmission Provider has set the status of the request for temporary termination of Network Resource to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these Business Practice Standards.

Southern:

001-105.4.5.1
Once the Transmission Provider has set the status of the request for indefinite termination of Network Resource to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these NITS Business Practice Standards.

Duke Energy:

001-105.4.5.1
Once the Transmission Provider has set the status of the request for temporary termination of Network Resource to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these Business Practice Standards.

BC Hydro:

001-105.4.5.1
Once the Transmission Provider has set the status of the request for indefinite termination of Network Resource to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these Business Practice Standards.

Entergy:

001-105.4.5.1
Once the Transmission Provider has set the status of the request for indefinite termination of Network Resource to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these Business Practice Standards.

001-105.4.6
Acceptance of Request for Indefinite Termination of Network Resource

001-105.4.6.1
Once the request for indefinite termination of Network Resource is deemed to be valid, the Transmission Provider shall set the status of the request to ACCEPTED.

001-105.4.7
Customer Actions

001-105.4.7.1
After the status of the request for indefinite termination of Network Resource has been set to ACCEPTED by the Transmission Provider, the Network Customer may confirm or withdraw the request by setting the status to CONFIRMED or WITHDRAWN. 

001-105.4.8
Final Disposition of Request for Indefinite Termination of Network Resource

001-105.4.8.1
Once CONFIRMED, the request for indefinite termination of Network Resource will be incorporated into the associated CONFIRMED NITS Application.

001-105.4.8.2
If, within the time limits specified in Table 105-A NITS Request Timing Requirements the Network Customer fails to set the status of the ACCEPTED request for indefinite termination of Network Resource to CONFIRMED or WITHDRAWN, the Transmission Provider may set the status of the request to RETRACTED. 

Entergy:

001-105.4.8.2
If the Network Customer fails to set the status of the ACCEPTED request for indefinite termination of Network Resource to CONFIRMED or WITHDRAWN, the Transmission Provider may set the status of the request to RETRACTED
. 

001-105.4.8.2.1
In response to a state of RETRACTED, the Network Customer may submit a new or revised request for indefinite termination of Network Resource, which will be assigned a new unique identifier and a new queue time.   

001-105.5
Designation of New Network Load

OATI:

001-105.5
Designation of New Network Load

Duke Energy:

001-105.5
Designation of New
 Network Load
001-105.5.1
Request for Designation of New Network Load

Duke Energy:

001-105.5.1
Request for Designation of New Network Load

001-105.5.1.1
A request for designation of new Network Load may be a request for either (1) a load addition to a Network Customer’s existing Network Load(s) exclusive of projected load growth or (2) a load addition that is separate from a Network Customer’s existing Network Load(s).

NCEMC:

001-105.5.1.1
A request for designation of new Network Load may be a request for either (1) a load addition to a Network Customer’s existing Network Load(s) exclusive of projected load growth or (2) a load addition that is separate from a Network Customer’s existing Network Load(s).

001-105.5.1.1.1
If the Network Customer’s preference is for a load addition to the Network Customer’s existing Network Load(s) exclusive of projected load growth, then the Network Customer shall set the “Addition to Existing Load” flag to “Y” and reference the unique identifier assigned to the existing designated Network Load. (Technical team to determine whether to use a flag or a data element such as “type of load addition”. The set flag language might belong in WEQ-002 and/or WEQ-013)
Duke Energy:

001-105.5.1.1.1
If the Network Customer’s preference is for a load addition to the Network Customer’s existing Network Load(s) exclusive of projected load growth
, then the Network Customer shall set the “Addition to Existing Load” flag to “Y” and reference the unique identifier assigned to the existing designated Network Load. (Technical team to determine whether to use a flag or a data element such as “type of load addition”. The set flag language might belong in WEQ-002 and/or WEQ-013)
001-105.5.1..12
If the Network Customer’s preference is for a load addition that is separate from a Network Customer’s existing Network Load(s), then the Network Customer shall set the “Addition to Existing Load” flag to “N”.

Southern:

001-105.5.1.1.2
If the Network Customer’s preference is for a load addition that is separate from a Network Customer’s existing Network Load(s), then the Network Customer shall set the “Addition to Existing Load” flag to “N”.

Duke Energy:

001-105.5.1..12
If the Network Customer’s preference is for a load addition that is separate from a Network Customer’s existing Network Load(s)
, then the Network Customer shall set the “Addition to Existing Load” flag to “N”.

Entergy:

001-105.5.1.12
If the Network Customer’s preference is for a load addition that is separate from a Network Customer’s existing Network Load(s), then the Network Customer shall set the “Addition to Existing Load” flag to “N”.

001-105.5.1.1.3
If the Network Customer has no preference, then the Network Customer shall set the “Addition to Existing Load” flag to null.

Duke Energy:

001-105.5.1.1.3
If the Network Customer has no preference
, then the Network Customer shall set the “Addition to Existing Load” flag to null.

Entergy:

001-105.5.1.1.3
If the Network Customer has no preference, then the Network Customer shall leave the value of the “Addition to Existing Load” flag as null.

001-105.5.1.1.4
The Transmission Provider shall accommodate the Network Customer’s preference if the ability to do so is indicated by the results of the evaluation of the request.

Duke Energy:

001-105.5.1.1.4
The Transmission Provider shall accommodate the Network Customer’s preference
 if the ability to do so is indicated by the results of the evaluation of the request.

001-105.5.2
Submission of Request for Designation of New Network Load

001-105.5.2.1
The Network Customer may enter and update information on OASIS in association with a request for designation of new Network Load prior to submission to the Transmission Provider by setting the status of the request to PENDING.

Entergy:

001-105.5.2.1
The Network Customer may enter and update information on OASIS to support a request for designation of new Network Load prior to submission to 
the Transmission Provider by setting the status of the request to PENDING.

001- 105.5.2.2
The Transmission Provider may specify, in its posted business practice, the minimum information required to designate a new Network Load. 

Southern:

001- 105.5.2.2
The Transmission Provider may specify, in its posted Business Practice, the minimum information required to designate a new Network Load. 

NCEMC:

001- 105.5.2.2
The Transmission Provider may specify, in its posted business practice, the minimum information required to designate a new Network Load. 

Entergy:

001- 105.5.2.2
The Transmission Provider may specify, in a posted business practice, the minimum information required to designate a new Network Load
. 

001- 105.5.2.3
The Network Customer shall indicate to the Transmission Provider that the request for designation of new Network Load is ready for Transmission Provider review by submitting the request on OASIS.  

001-105.5.2.3.1
OASIS shall set the queue time for the request for designation of new Network Load to the time that the Network Customer submits the request, which shall set the request to QUEUED.

Entergy:

001-105.5.2.3.1
OASIS shall set the queue time for the request for designation of new Network Load to the time that the Network Customer submits the request and shall set the request to QUEUED.

001-105.5.2.3.2
The Transmission Provider shall acknowledge the queued request for designation of new Network Load by setting the status of the request to RECEIVED.

001-105.5.3
Initial Review of Request for Designation of New Network Load

001-105.5.3.1
The Transmission Provider shall review the request for designation of new Network Load to determine if the request: (1) meets the minimum requirements specified in the Transmission Provider’s posted business practices; and (2) is complete consistent with the Transmission Provider’s tariff requirements.

Southern:

001-105.5.3.1
The Transmission Provider shall review the request for designation of new Network Load to determine if the request: (1) meets the minimum requirements specified in the Transmission Provider’s posted Business Practices;
 and (2) is complete consistent with the Transmission Provider’s tariff requirements.

001-105.5.3.2
The Transmission Provider shall respond to a request for designation of new Network Load by setting the status to one of the following:

I.
INVALID 

II.
DEFICIENT 

III.
COMPLETED 

001-105.5.4
Invalid Request for Designation of New Network Load

001-105.5.4.1
If the required information, as specified in Business Practice Standard WEQ-001-105.5.2.1, is not provided, the Transmission Provider shall set the state of the request for designation of new Network Load to INVALID and the request shall be deemed withdrawn.

Entergy:

001-105.5.4.1
If the required information, as specified in Business Practice Standard WEQ-001-105.5.2.1, has not been provided in the request for designation of new Network Load, the Transmission Provider shall set the state of the request for designation of new Network Load to INVALID and the request shall be deemed withdrawn.

001-105.5.4.2
In response to a state of INVALID, the Network Customer may submit a new or revised request for designation of new Network Load, which will be assigned a new unique identifier and a new queue time.   

001-105.5.5
Deficient Request for Designation of New Network Load
001-105.5.5.1
For each request for designation of new Network Load which is not INVALID, the Transmission Provider shall determine if the request can be deemed sufficient. 

Entergy:

001-105.5.5.1
For each request for designation of new Network Load that is not INVALID, the Transmission Provider shall determine if the request can be deemed sufficient. 

001-105.5.5.1.1
If deemed sufficient, the Transmission Provider shall set the status of such request to COMPLETED.  
001-105.5.5.1.2
If not deemed sufficient, the Transmission Provider shall set the status of such request to DEFICIENT. 

001-105.5.5.2
If the Transmission Provider determines that the request for designation of new Network Load is DEFICIENT, the Transmission Provider shall set the request to DEFICIENT and begin informal communications with the Network Customer to resolve the deficiencies.
Southern:

001-105.5.5.2
If the Transmission Provider determines that the request for designation of new Network Load is deficient, the Transmission Provider shall set the request to DEFICIENT and begin informal communications with the Network Customer to resolve the deficiencies.

001-105.5.5.3
The Network Customer shall remedy the deficiency and shall set the status of the request for designation of new Network Load to REEVALUATE.

001-105.5.5.4
Upon notification that the Network Customer has provided the necessary information, either on or off OASIS as appropriate, to remedy an identified deficiency, the Transmission Provider shall review the provided information and take one of the following actions:

001-105.5.5.4.1
If the information provided fails to remedy the deficiency, the Transmission Provider shall set the status of the request for designation of new Network Load to DEFICIENT.

001-105.5.5.4.2
If the information provided remedies the deficiency, the Transmission Provider shall set the status of the request for designation of new Network Load to COMPLETED.

001-105.5.5.4.3
If the information provided is deemed by the Transmission Provider to be a significant change to the request for designation of new Network Load, the Transmission Provider shall set the status of the request to DECLINED.

NCEMC:

001-105.5.5.4.3
If the information provided is deemed by the Transmission Provider to be a significant change to the request for designation of new Network Load, the Transmission Provider shall set the status of the request to DECLINED.

BPA:

001-105.5.5.4.3
If the information provided is deemed by the Transmission Provider to be a significant
 change to the request for designation of new Network Load, the Transmission Provider shall set the status of the request to DECLINED.

001-105.5.5.5
If the Network Customer fails to respond to the Transmission Provider’s notice of a DEFICIENT request for designation of new Network Load, or fails to remedy the deficiency to the Transmission Provider’s satisfaction in a timely manner, the Transmission Provider shall set the status of the request to DECLINED.

Southern:

001-105.5.5.5
If the Network Customer fails to respond to the Transmission Provider’s notice of a deficient
 request for designation of new Network Load, or fails to remedy the deficiency to the Transmission Provider’s satisfaction in a timely manner, the Transmission Provider shall set the status of the request to DECLINED.

001-105.5.5.6
In response to a state of DECLINED, the Network Customer may submit a new or revised request for designation of new Network Load, which will be assigned a new unique identifier and a new queue time.  

001-105.5.6
Completed Request for Designation of New Network Load

001-105.5.6.1
Once the Transmission Provider has set the status of the request for designation of new Network Load to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these Business Practice Standards.

Southern:

001-105.5.6.1
Once the Transmission Provider has set the status of the request for designation of new Network Load to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these NITS Business Practice Standards.

001-105.5.7
Review of Completed Request for Designation of New Network Load

Duke Energy:

001-105.5.7
Review of Completed Request for Designation of New Network Load

001- 105.5.7.1
Once the Transmission Provider has set the status of the request for designation of new Network Load to COMPLETED, the Transmission Provider shall respond to the request, consistent with filed tariffs, within the Provider Response Time Limit defined in Table 105-A NITS Request Timing Requirements. The time limit is measured from the time the request is QUEUED.  To indicate its evaluation of the request, a Transmission Provider may respond by setting the state of the request to one of the following:
I.    STUDY

II.   DECLINED

III.  REFUSED 

IV.  COUNTEROFFER

V.  ACCEPTED 

Entergy:

001- 105.5.7.1
Once the Transmission Provider has set the status of the request for designation of new Network Load to COMPLETED, the Transmission Provider shall respond to the request, consistent with filed tariffs, within the Provider Response Time Limit defined in Table 105-A NITS Request Timing Requirements. The time limit is measured from the time the request is QUEUED.  To indicate its evaluation of the request, a Transmission Provider may respond by setting the state of the request to one of the following:

I.    STUDY

II.   DECLINED

III.  REFUSED 

IV.  COUNTEROFFER

V.  ACCEPTED 

001-105.5.8
Study of Request for Designation of New Network Load

001-105.5.8.1 
If the Transmission Provider determines that some level of study is required or is being performed, the Transmission Provider shall set the status of the request for designation of new Network Load to STUDY.

001-105.5.8.2
The Transmission Provider shall, through its evaluation, determine whether the request for designation of new Network Load is either (1) a load addition to a Network Customer’s existing Network Load(s) exclusive of projected load growth or (2) a load addition that is separate from a Network Customer’s existing Network Load(s) and shall set the “Addition to Existing Load” flag to either “Y” or “N”, as appropriate.

Duke Energy:

001-105.5.8.2
The Transmission Provider shall, through its evaluation, determine whether the request for designation of new Network Load is either (1) a load addition to a Network Customer’s existing Network Load(s) exclusive of projected load growth or (2) a load addition that is separate from a Network Customer’s existing Network Load(s) and shall set the “Addition to Existing Load” flag to either “Y” or “N”, as appropriate.


001-105.5.8.2.1
If the Transmission Provider sets the “Addition to Existing Load” flag to “Y”, then the Transmission Provider shall ensure that the unique identifier assigned to the existing designated Network Load is referenced in the request (this may be “response to request” – technical team to determine and may also look at using counteroffer-type functionality).
Duke Energy:

001-105.5.8.2.1
If the Transmission Provider sets the “Addition to Existing Load” flag to “Y”, then the Transmission Provider shall ensure that the unique identifier 
assigned to the existing designated Network Load is referenced in the request (this may be “response to request” – technical team to determine and may also look at using counteroffer-type functionality).

001-105.5.8.3
From the status of STUDY, the Transmission Provider shall set the status of the request for designation of new Network Load to DECLINED, REFUSED, COUNTEROFFER or ACCEPTED.

001-105.5.9
Declining Request for Designation of New Network Load

001-105.5.9.1
If, at any time in the evaluation of the request for designation of new Network Load or conduct of any associated study(ies), the Transmission Provider determines that the Network Customer has failed to meet any of the terms and conditions for the designation of a new Network Load (e.g., failure to execute a study agreement, etc.), the Transmission Provider shall set the status of the request to DECLINED.

001-105.5.9.2
In response to a state of DECLINED, the Network Customer may submit a new or revised request for designation of new Network Load, which will be assigned a new unique identifier and a new queue time.   

001-105.5.10
Refusal of Request for Designation of New Network Load

001-105.5.10.1
If, at any time in the evaluation of the request for designation of new Network Load or conduct of any associated study(ies), the Transmission Provider determines that the requested service cannot be provided due to lack of availability of transfer capability consistent with the Transmission Provider’s tariff requirements, the Transmission Provider may set the status of the request to REFUSED.

Duke Energy:

001-105.5.10.1
If, at any time in the evaluation of the request for designation of new Network Load or conduct of any associated study(ies), the Transmission Provider determines that the requested service cannot be provided due to lack of availability of transfer capability
 consistent with the Transmission Provider’s tariff requirements, the Transmission Provider may set the status of the request to REFUSED.

NCEMC:

001-105.5.10.1
If, at any time in the evaluation of the request for designation of new Network Load or conduct of any associated study(ies), the Transmission Provider determines that the requested service cannot be provided due to lack of availability of transfer capability consistent with the Transmission Provider’s tariff requirements, the Transmission Provider may set the status of the request to REFUSED.

001-105.5.10.2
In response to a state of REFUSED, the Network Customer may submit a new or revised request for designation of new Network Load, which will be assigned a new unique identifier and a new queue time.  

001-105.5.11
Counteroffer to Request for Designation of New Network Load

Duke Energy:

001-105.5.11
Counteroffer to Request for Designation of New Network Load

001-105.5.11.1
When, in the evaluation of a request for designation of new Network Load or on completion of any applicable study(ies), the Transmission Provider determines that the request may be granted in part, the Transmission Provider may make a partial offer.  

001-105.5.11.1.1
If the Transmission Provider makes a partial offer, the Transmission Provider shall notify the Network Customer on OASIS by setting the amount of Network Load capacity granted over time in the response to the request for designation of new Network Load and setting the status of such request to COUNTEROFFER.

001-105.5.12
Acceptance of Request for Designation of New Network Load

001-105.5.12.1
When, in the evaluation of the request for designation of new Network Load or on completion of any applicable study(ies), the Transmission Provider determines that the request can be approved, the Transmission Provider shall set the status of the request to ACCEPTED.

001-105.5.13
Customer Actions

001-105.5.13.1
From ACCEPTED or COUNTEROFFER, the Network Customer may change the status of the request for designation for new Network Load to CONFIRMED, REBID or WITHDRAWN. The Network Customer has the amount of time designated as Customer Confirmation Time Limit in Table 105-A NITS Request Timing Requirements to change the state of the request. The Customer Confirmation Time Limit is measured from the first time the request is moved to ACCEPTED or COUNTEROFFER, and is not reset with subsequent iterations of negotiation. 

Entergy:

001-105.5.13.1
From ACCEPTED or COUNTEROFFER, the Network Customer may change the status of the request for designation for new Network Load to CONFIRMED, REBID or WITHDRAWN. The Network Customer has the amount of time designated as Customer Confirmation Time Limit in Table 105-A NITS Request Timing Requirements to change the state of the request. The Customer Confirmation Time Limit is measured from the first time the request is moved to ACCEPTED or COUNTEROFFER, and is not reset with subsequent iterations of negotiation. 

001-105.5.13.1.1
The Network Customer shall use the request status of REBID to initiate or continue the negotiation of designated Network Load capacity.

Entergy:

001-105.5.13.1.1
The Network Customer shall use the request status of REBID to initiate negotiation of designated Network Load capacity or engage in continued negotiation of the designated Network Load capacity with the Transmission Provider.

001-105.5.13.1.2
Should the Network Customer elect to initiate or continue the negotiation of designated Network Load capacity by moving the request to REBID, the Transmission Provider shall respond by taking the request to a DECLINED, SUPERSEDED, ACCEPTED, or COUNTEROFFER state within the Provider Counter Time Limit, specified in Table 105-A NITS Request Timing Requirements. The Transmission Provider response time is measured from the most recent REBID time.

001-105.5.14
Final Disposition of Request for Designation of New Network Load

001-105.5.14.1
Once CONFIRMED, the request for designation of new Network Load will be incorporated into the associated CONFIRMED NITS Application.

001-105.5.14.2
If, within the time limits specified in Table 105-A NITS Request Timing Requirements, the Network Customer fails to set the status of the ACCEPTED request for designation of new Network Load to CONFIRMED or WITHDRAWN, the Transmission Provider may set the status of the request to RETRACTED. 

001-105.5.14.2.1
In response to a state of RETRACTED, the Network Customer may submit a new or revised request for designation of new Network Load, which will be assigned a new unique identifier and a new queue time.
Entergy:

001-105.5.14.2.1
In response to a state of RETRACTED, the Network Customer may submit a new or revised request for designation of new Network Load, which will be assigned a new unique identifier and a new queue time
.

001-105.5.14.3
Should the Network Customer elect to initiate or continue the negotiation of Network Load capacity by changing the status of the request for designation of new Network Load to REBID, the Transmission Provider shall respond by changing the status of the request to DECLINED, ACCEPTED, or COUNTEROFFER within the Provider Counter Time Limit, specified in Table 105-A NITS Request Timing Requirements. The Transmission Provider response time is measured from the most recent REBID time.

Southern:

Add space between paragraph

Duke Energy:

001-105.5.14.3
Should the Network Customer elect to initiate or continue the negotiation of Network Load capacity by changing the status of the request for designation of new Network Load to REBID, the Transmission Provider shall respond by changing the status of the request to DECLINED, ACCEPTED, or COUNTEROFFER within the Provider Counter Time Limit, specified in Table 105-A NITS Request Timing Requirements. The Transmission Provider response time is measured from the most recent REBID time
.

Entergy:



001-105.5.14.3.1
The Transmission Provider shall positively acknowledge agreement to the designation of new Network Load capacity over time by setting the granted Network Load capacity equal to the Network Customer’s requested Network Load capacity prior to acceptance of the request.   

Duke Energy:

001-105.5.14.3.1
The Transmission Provider shall positively acknowledge agreement to the designation of new Network Load capacity over time by setting the granted Network Load capacity equal to the Network Customer’s requested Network Load capacity prior to acceptance of the request.   

Entergy:



001-105.6
Indefinite Termination of Network Load

OATI:

001-105.6
Indefinite Termination of Network Load



001-105.6.1
Submission of Request for Indefinite Termination of Network Load

001-105.6.1.1
The Network Customer may enter and update information on OASIS in association with a request for indefinite termination of Network Load prior to submission to the Transmission Provider by setting the status of the request to PENDING. 

Duke Energy:

001-105.6.1.1
The Network Customer may enter and update information on OASIS in association with a request for indefinite termination of Network Load 
prior to submission to the Transmission Provider by setting the status of the request to PENDING. 

Entergy:

001-105.6.1.1
The Network Customer may enter and update information on OASIS to support with a request for indefinite termination of Network Load prior to submission to the Transmission Provider by setting the status of the request to PENDING. 

001- 105.6.1.2
The Network Customer shall indicate to the Transmission Provider that the request for indefinite termination of Network Load is ready for Transmission Provider review by submitting the request on OASIS.  

001-105.6.1.2.1
OASIS shall set the queue time for the request for indefinite termination of Network Load to the time that the Network Customer submits the request, which shall set the request to QUEUED.

Entergy:

001-105.6.1.2.1
OASIS shall set the queue time for the request for indefinite termination of Network Load to the time that the Network Customer submits the request and shall set the request to QUEUED.

001-105.6.1.2.2
The Transmission Provider shall acknowledge the queued request for indefinite termination of Network Load by setting the status of the request to RECEIVED.

001-105.6.2
Initial Review of Request for Indefinite Termination of Network Load

001- 105.6.2.1
The Transmission Provider shall review the request for indefinite termination of Network Load to determine if the request is consistent with the Transmission Provider’s tariff requirements.

001- 105.6.2.2
The Transmission Provider shall respond to a request for indefinite termination of Network Load by setting the status to one of the following:

I.
INVALID 

II.
COMPLETED

001-105.6.3
Invalid Request for Indefinite Termination of Network Load

001-105.6.3.1
If the information necessary to process the request for indefinite termination of Network Load is not provided, the Transmission Provider shall set the state of the request to INVALID and the request shall be deemed withdrawn.

NCEMC:

001-105.6.3.1
If the information necessary to process the request for indefinite termination of Network Load is not provided, the Transmission Provider shall set the state of the request to INVALID and the request shall be deemed withdrawn.

Entergy:

001-105.6.3.1
If the information necessary to process the request for indefinite termination of Network Load has not been provided, the Transmission Provider shall set the state of the request to INVALID and the request shall be deemed withdrawn.

001-105.6.3.2
In response to a state of INVALID, the Network Customer may submit a new or revised request for indefinite termination of Network Load, which will be assigned a new unique identifier and a new queue time.  

001-105.6.4
Completed Request for Indefinite Termination of Network Load 

001-105.6.4.1
Once the Transmission Provider has set the status of the request for indefinite termination of Network Load to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these Business Practice Standards.

Southern:

001-105.6.4.1
Once the Transmission Provider has set the status of the request for indefinite termination of Network Load to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these NITS Business Practice Standards.

001-105.6.5
Review of Completed Request for Indefinite Termination of Network Load

001- 105.6.5.1
Once the Transmission Provider has set the status of the request for indefinite termination of Network Load to COMPLETED, the Transmission Provider shall respond to the request, consistent with filed tariffs, within the Provider Response Time Limit defined in Table 105-A Network Timing Requirements. The time limit is measured from the time the request is QUEUED.  To indicate its evaluation of the request, a Transmission Provider may respond by setting the state of the request to one of the following:
I.   DECLINED

II.  ACCEPTED 

BPA:

001- 105.6.5.1
Once the Transmission Provider has set the status of the request for indefinite termination of Network Load to COMPLETED, the Transmission Provider shall respond to the request, consistent with filed tariffs and posted business practices
, within the Provider Response Time Limit defined in Table 105-A Network Timing Requirements. The time limit is measured from the time the request is QUEUED.  To indicate its evaluation of the request, a Transmission Provider may respond by setting the state of the request to one of the following:
I.   DECLINED

II.  ACCEPTED 

Entergy:

001- 105.6.5.1
Once the Transmission Provider has set the status of the request for indefinite termination of Network Load to COMPLETED, the Transmission Provider shall respond to the request, consistent with filed tariffs, within the Provider Response Time Limit defined in Table 105-A Network Timing Requirements. The time limit is measured from the time the request is QUEUED.  To indicate its evaluation of the request, a Transmission Provider may respond by setting the state of the request to one of the following:

I.   DECLINED

II.  ACCEPTED 

001-105.6.6
Declining Request for Indefinite Termination of Network Load

001-105.6.6.1
If, at any time in the evaluation of the request for indefinite termination of Network Load, the Transmission Provider determines that the Network Customer has failed to meet any of the terms and conditions for the indefinite termination of Network Load, the Transmission Provider shall set the status of the request to DECLINED.

001-105.6.6.2
In response to a state of DECLINED, the Network Customer may submit a new or revised request for indefinite termination of Network Load, which will be assigned a new unique identifier and a new queue time.   

001-105.6.7
Acceptance of Request for Indefinite Termination of Network Load

001-105.6.7.1
When, in the evaluation of the request for indefinite termination of Network Load, the Transmission Provider determines that the request can be approved, the Transmission Provider shall set the status of the request to ACCEPTED.

001-105.6.8
Customer Actions

001-105.6.8.1
After the status of the request for indefinite termination of Network Load has been set to ACCEPTED by the Transmission Provider, the Network Customer may confirm or withdraw the request by setting the status to CONFIRMED or WITHDRAWN.

001-105.6.9
Final Disposition of Request for Indefinite Termination of Network Load

001-105.6.9.1
Once CONFIRMED, the request for indefinite termination of Network Load will be incorporated into the associated CONFIRMED NITS Application.

001-105.6.9.2
If, within the time limits specified in Table 105-A NITS Request Timing Requirements, the Network Customer fails to set the status of the ACCEPTED request for indefinite termination of Network Resource to CONFIRMED or WITHDRAWN, the Transmission Provider may set the status of the request to RETRACTED.

Entergy:

001-105.6.9.2
If the Network Customer fails to set the status of the ACCEPTED request for indefinite termination of Network Resource to CONFIRMED or WITHDRAWN, the Transmission Provider may set the status of the request to RETRACTED
.

001-105.6.9.2.1
In response to a state of RETRACTED, the Network Customer may submit a new or revised request indefinite termination of Network Load, which will be assigned a new unique identifier and a new queue time.   

001-105.7
Network Scheduling Rights

Southern:

001-105.7
NSR

OATI:

001-105.7
Network Scheduling Rights

Duke Energy:

001-105.7
Network Scheduling Right


001-105.7
Network Scheduling Rights
001-105.7.1
Use of a request for new Network Scheduling Right is optional at the discretion of the Transmission Provider. Such request will be processed in accordance with the following:

Southern:

001-105.7.1
Use of a request for new NSR is optional at the discretion of the Transmission Provider. Such request will be processed in accordance with the following:

NCEMC:

001-105.7.1
Use of a request for new Network Scheduling Right is optional at the discretion of the Transmission Provider. Such request will be processed in accordance with the following:

000-105.7.1.1
The Transmission Provider may complete a request for new Network Scheduling Right and may set the status of the request to ACCEPTED.
Southern:

000-105.7.1.1
The Transmission Provider may complete a request for new NSR and may set the status of the request to ACCEPTED.

Entergy:

000-105.7.1.1
The Transmission Provider may complete a request for new Network Scheduling Right on behalf of the Network Customer and may set the status of the request to ACCEPTED.

001-105.7.1.2
Alternatively, the Network Customer may submit a request for new Network Scheduling Right.

Southern:

001-105.7.1.2
Alternatively, the Network Customer may submit a request for new NSR.

001-105.7.2
The Transmission Provider may specify, in its posted business practice, if the request for new Network Scheduling Right shall be used and the process the Transmission Provider requires based on the Business Practice Standard WEQ-001-105.7.1 for completion of a request for temporary termination of Network Scheduling Right.

Southern:

001-105.7.2
The Transmission Provider may specify, in its posted Business Practice, if the request for new NSR shall be used and the process the Transmission Provider requires based on the Business Practice Standard WEQ-001-105.7.1 for completion of a request for temporary termination of NSR.

Duke Energy:

001-105.7.2
The Transmission Provider may
 
specify, in its posted business practice, if the request for new Network Scheduling Right shall be used and the process the Transmission Provider requires based on the Business Practice Standard WEQ-001-105.7.1 for completion of a request for temporary termination of Network Scheduling Right
.

NCEMC:

001-105.7.2
The Transmission Provider may specify, in its posted business practice, if the request for new Network Scheduling Right shall be used and the process the Transmission Provider requires based on the Business Practice Standard WEQ-001-105.7.1 for completion of a request for temporary termination of Network Scheduling Right.


Entergy:

001-105.7.2
The Transmission Provider may specify, in a posted business practice, if the request for new Network Scheduling Right shall be used and the process the Transmission Provider requires based on the Business Practice Standard WEQ-001-105.7.1 for completion of a request for temporary termination of Network Scheduling Right
.

001-105.7.3
A request for new Network Scheduling Right shall only be granted when associated with a Designated Network Resource(s). The capacity granted for request for new Network Scheduling Right may be different from the capacity granted for Designated Network Resource(s).

001-105.7.3.1
place holder for discussion for coordination of queueing and evaluation of DNR/NSR requests. See OATI in section 105.2

Southern:

001-105.7.3
A request for new NSR shall only be granted when associated with a DNR(s). The capacity granted for request for new NSR may be different from the capacity granted for DNR(s).

Duke Energy:

001-105.7.3
A request for new Network Scheduling Right shall only be granted when associated with a Designated Network Resource(s). 
The capacity granted for request for new Network Scheduling Right may be different 
from the capacity granted for Designated Network Resource(s).

001-105.7.4
Submission of Request for New Network Scheduling Right

Southern:

001-105.7.4
Submission of Request for New NSR 

001-105.7.4.1
The Network Customer may enter and update information on OASIS in association with a request for new Network Scheduling Right prior to submission to the Transmission Provider by setting the status of the request to PENDING.

Southern:

001-105.7.4.1
The Network Customer may enter and update information on OASIS in association with a request for new NSR prior to submission to the Transmission Provider by setting the status of the request to PENDING.

Entergy:

001-105.7.4.1
The Network Customer may enter and update information on OASIS to support 
with a request for new Network Scheduling Right prior to submission to the Transmission Provider by setting the status of the request to PENDING.

001- 105.7.4.2
The Transmission Provider may specify, in its posted business practice, the minimum information required for new Network Scheduling Right.
Southern:

001- 105.7.4.2
The Transmission Provider may specify, in its posted Business Practice, the minimum information required for new NSR.  

Duke Energy:

001- 105.7.4.2
The Transmission Provider may
 specify, in its posted business practice, the minimum information required for new Network Scheduling Right.  

Entergy:

001- 105.7.4.2
The Transmission Provider may specify, in a posted business practice, the minimum information required for new Network Scheduling Right
.

001- 105.7.4.3
The Network Customer shall indicate to the Transmission Provider that the request for new Network Scheduling Right is ready for Transmission Provider review by submitting the request on OASIS.
Southern:

001- 105.7.4.3
The Network Customer shall indicate to the Transmission Provider that the request for new NSR is ready for Transmission Provider review by submitting the request on OASIS.

001-105.7.4.3.1
OASIS shall set the queue time for the request for new Network Scheduling Right to the time that the Network Customer submits the request, which shall set the status of the request to QUEUED.

Southern:

001-105.7.4.3.1
OASIS shall set the queue time for the request for new NSR to the time that the Network Customer submits the request, which shall set the status of the request to QUEUED.

Entergy:

001-105.7.4.3.1
OASIS shall set the queue time for the request for new Network Scheduling Right to the time that the Network Customer submits the request and shall set the status of the request to QUEUED.

001-105.7.4.3.2
The Transmission Provider shall acknowledge the queued request for new Network Scheduling Right by setting the status of the request to RECEIVED.

Southern:

001-105.7.4.3.2
The Transmission Provider shall acknowledge the queued request for new NSR 
by setting the status of the request to RECEIVED.

001-105.7.5
Initial Review of Request for New Network Scheduling Right

Southern:

001-105.7.5
Initial Review of Request for New NSR

001- 105.7.5.1
The Transmission Provider shall review the request for New Network Scheduling Right to determine if the request: (1) meets the minimum requirements specified in the Transmission Provider’s posted business practices; or (2) is consistent with the Transmission Provider’s tariff requirements.

Southern:

001- 105.7.5.1
The Transmission Provider shall review the request for New NSR to determine if the request: (1) meets the minimum requirements specified in the Transmission Provider’s posted Business Practices; or (2) is consistent with the Transmission Provider’s tariff requirements.

Duke Energy:

001- 105.7.5.1
The Transmission Provider shall review the request for New Network Scheduling Right to determine if the request: (1) meets the minimum requirements specified in the Transmission Provider’s posted business practices; or (2) is consistent with the Transmission Provider’s tariff requirements.


001- 105.7.5.2
The Transmission Provider shall respond to a request by setting the status to one of the following:

I.
INVALID 

II.
COMPLETED

001-105.7.6
Invalid Request for New Network Scheduling Right

Southern:

001-105.7.6
Invalid Request for New NSR

001-105.7.6.1
If the required information, as specified in Business Practice Standard WEQ-001-105.7.4.1, is not provided, the Transmission Provider shall set the state of the request for new Network Scheduling Right to INVALID and the request shall be deemed withdrawn.

Southern:

001-105.7.6.1
If the required information, as specified in Business Practice Standard WEQ-001-105.7.4.1, is not provided, the Transmission Provider shall set the state of the request for new NSR 
to INVALID and the request shall be deemed withdrawn.

Entergy:

001-105.7.6.1
If the required information, as specified in Business Practice Standard WEQ-001-105.7.4.1, has not been provided in the request for new Network Scheduling Right, the Transmission Provider shall set the state of the request for new Network Scheduling Right to INVALID and the request shall be deemed withdrawn.

001-105.7.6.2
In response to a state of INVALID, the Network Customer may submit a new or revised request for new Network Scheduling Right, which will be assigned a new unique identifier and a new queue time. 
Southern:

001-105.7.6.2
In response to a state of INVALID, the Network Customer may submit a new or revised request for new NSR, which will be assigned a new unique identifier and a new queue time. 

001-105.7.7
Completed Request for New Network Scheduling Right

Southern:

001-105.7.7
Completed Request for New NSR

Duke Energy:

001-105.7.7
Completed Request for New Network Scheduling Right

001-105.7.7.1
Once the Transmission Provider has set the status of the request for a new Network Scheduling Right to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these Business Practice Standards.

Southern:

001-105.7.7.1
Once the Transmission Provider has set the status of the request for a new NSR to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these Business Practice Standards.

001-105.7.8
Review of Completed Request for New Network Scheduling Right

Southern:

001-105.7.8
Review of Completed Request for New NSR

001- 105.7.8.1
Once the Transmission Provider has set the status of the request for new Network Scheduling Right to COMPLETED, the Transmission Provider shall respond to the request, consistent with filed tariffs, within the Provider Response Time Limit defined in Table 105-A NITS Request Timing Requirements. The time limit is measured from the time the request is QUEUED.  To indicate its evaluation of the request, a Transmission Provider may respond by setting the state of the request to one of the following:
I.
STUDY

II.
DECLINED

III.
REFUSED 

IV.
COUNTEROFFER

V.
ACCEPTED 

Southern:

001- 105.7.8.1
Once the Transmission Provider has set the status of the request for new NSR 
to COMPLETED, the Transmission Provider shall respond to the request, consistent with filed tariffs, within the Provider Response Time Limit defined in Table 105-A NITS Request Timing Requirements. The time limit is measured from the time the request is QUEUED.  To indicate its evaluation of the request, a Transmission Provider may respond by setting the state of the request to one of the following:
I.
STUDY

II.
DECLINED

III.
REFUSED 

IV.
COUNTEROFFER

V.
ACCEPTED 

001-105.7.9
Study of Request for New Network Scheduling Right

Southern:

001-105.7.9
Study of Request for New NSR

Duke Energy:

001-105.7.9
Study of Request for New Network Scheduling Right

001-105.7.9.1 
If the Transmission Provider determines that some level of study is required or is being performed, the Transmission Provider shall set the status of the request for new Network Scheduling Right to STUDY.

Southern:

001-105.7.9.1 
If the Transmission Provider determines that some level of study is required or is being performed, the Transmission Provider shall set the status of the request for new NSR to STUDY.

001-105.7.9.2
From the status of STUDY, the Transmission Provider shall set the status of the request for new Network Scheduling Right to DECLINED, REFUSED, COUNTEROFFER, or ACCEPTED.

Southern:

001-105.7.9.2
From the status of STUDY, the Transmission Provider shall set the status of the request for new NSR to DECLINED, REFUSED, COUNTEROFFER, or ACCEPTED.

001-105.7.10
Decline of Request for New Network Scheduling Right

Southern:

001-105.7.10
Decline of Request for New NSR

001-105.7.10.1
If, at any time in the evaluation of the request for a new Network Scheduling Right or conduct of any associated study(ies), the Transmission Provider determines that the Network Customer has failed to meet any of the terms and conditions for new Network Scheduling Right (e.g., failure to execute a study agreement, etc.), the Transmission Provider shall set the status of the request to DECLINED.

Southern:

001-105.7.10.1
If, at any time in the evaluation of the request for a new nsr or conduct of any associated study(ies), the Transmission Provider determines that the Network Customer has failed to meet any of the terms and conditions for new NSR (e.g., failure to execute a study agreement, etc.), the Transmission Provider shall set the status of the request to DECLINED.

001-105.7.10.2
In response to a state of DECLINED, the Network Customer may submit a new or revised request for new Network Scheduling Right, which will be assigned a new unique identifier and a new queue time.

Southern:

001-105.7.10.2
In response to a state of DECLINED, the Network Customer may submit a new or revised request for new NSR, which will be assigned a new unique identifier and a new queue time.

001-105.7.11
Refusal of Request for New Network Scheduling Right

Southern:

001-105.7.11
Refusal of Request for New NSR

001-105.7.11.1
If, at any time in the evaluation of the request for new Network Scheduling Right or conduct of any associated study(ies), the Transmission Provider determines that the requested service cannot be provided due to lack of availability of transfer capability consistent with the Transmission Provider’s tariff requirements, the Transmission Provider may set the status of the request to REFUSED.

Southern:

001-105.7.11.1
If, at any time in the evaluation of the request for new NSR or conduct of any associated study(ies), the Transmission Provider determines that the requested service cannot be provided due to lack of availability of transfer capability consistent with the Transmission Provider’s tariff requirements, the Transmission Provider may set the status of the request to REFUSED.

001-105.7.11.2
In response to a state of REFUSED, the Network Customer may submit a new or revised request for new Network Scheduling Right, which will be assigned a new unique identifier and a new queue time.

Southern:

001-105.7.11.2
In response to a state of REFUSED, the Network Customer may submit a new or revised request for new NSR, which will be assigned a new unique identifier and a new queue time.

001-105.7.12
Counteroffer to Request for New Network Scheduling Right

Southern:

001-105.7.12
Counteroffer to Request for New NSR

001-105.7.12.1
When, in the evaluation of a request for new Network Scheduling Right or on completion of any applicable study(ies), the Transmission Provider determines that the request may be granted in part, the Transmission Provider may make a partial offer.
Southern:

001-105.7.12.1
When, in the evaluation of a request for new NSR or on completion of any applicable study(ies), the Transmission Provider determines that the request may be granted in part, the Transmission Provider may make a partial offer.

001-105.7.12.1.1
If the Transmission Provider makes a partial offer, the Transmission Provider shall notify the Network Customer on OASIS by setting the amount of Network Scheduling Right capacity granted over time in the response to the request for new Network Scheduling Right and setting the status of such request to COUNTEROFFER.

Southern:

001-105.7.12.1.1
If the Transmission Provider makes a partial offer, the Transmission Provider shall notify the Network Customer on OASIS by setting the amount of NSR capacity granted over time in the response to the request for new NSR and setting the status of such request to COUNTEROFFER.

001-105.7.13
Acceptance of Request for New Network Scheduling Right

Southern:

001-105.7.13
Acceptance of Request for New NSR

001-105.7.13.1
When, in the evaluation of the request for new Network Scheduling Right or on completion of any applicable study(ies), the Transmission Provider determines that the request can be approved, the Transmission Provider shall set the status of the request to ACCEPTED.

Southern:

001-105.7.13.1
When, in the evaluation of the request for new NSR or on completion of any applicable study(ies), the Transmission Provider determines that the request can be approved, the Transmission Provider shall set the status of the request to ACCEPTED.

001-105.7.14
Customer Actions

001-105.7.14.1
From ACCEPTED or COUNTEROFFER, the Network Customer may change the status of the request for new Network scheduling Right to CONFIRMED, REBID or WITHDRAWN. The Network Customer has the amount of time designated as Customer Confirmation Time Limit in Table 105-A NITS Request Timing Requirements to change the state of the request. The Customer Confirmation Time Limit is measured from the first time the request is moved to ACCEPTED or COUNTEROFFER, and is not reset with subsequent iterations of negotiation.
Southern:

001-105.7.14.1
From ACCEPTED or COUNTEROFFER, the Network Customer may change the status of the request for new NSR to CONFIRMED, REBID or WITHDRAWN. The Network Customer has the amount of time designated as Customer Confirmation Time Limit in Table 105-A NITS Request Timing Requirements to change the state of the request. The Customer Confirmation Time Limit is measured from the first time the request is moved to ACCEPTED or COUNTEROFFER, and is not reset with subsequent iterations of negotiation.

001-105.7.14.1.1
The Network Customer shall use the request status of REBID to initiate or continue the negotiation of Network Scheduling Right capacity.

Southern:

001-105.7.14.1.1
The Network Customer shall use the request status of REBID to initiate or continue the negotiation of NSR capacity
.

Entergy:

001-105.7.14.1.1
The Network Customer shall use the request status of REBID to initiate negotiation of Network Scheduling Right capacity or engage in continued negotiation of the Network Scheduling Right capacity with the Transmission Provider.

001-105.7.14.1.2
Should the Network Customer elect to initiate or continue the negotiation of Network Scheduling Right capacity by moving the request to REBID, the Transmission Provider shall respond by taking the request to a DECLINED, SUPERSEDED, ACCEPTED, or COUNTEROFFER state within the Provider Counter Time Limit, specified in Table 105-A NITS Request Timing Requirements. The Transmission Provider response time is measured from the most recent REBID time.

Southern:

001-105.7.14.1.2
Should the Network Customer elect to initiate or continue the negotiation of NSR capacity by moving the request to REBID, the Transmission Provider shall respond by taking the request to a DECLINED, SUPERSEDED, ACCEPTED, or COUNTEROFFER state within the Provider Counter Time Limit, specified in Table 105-A NITS Request Timing Requirements. The Transmission Provider response time is measured from the most recent REBID time.

001-105.7.14.1.3
The Network Customer may negotiate for a lower amount of Network Scheduling Right capacity by updating the capacity requested over time and changing the request status from ACCEPTED or COUNTEROFFER to REBID. For example, the Network Customer may choose to coordinate the level of transmission service purchased across multiple Transmission Providers. Negotiation for a higher amount of Network Scheduling Right capacity shall not be allowed. A Network Customer seeking additional Network Scheduling Right capacity must submit a new request, which will be assigned a new unique identifier and a new queue time.

Southern:

001-105.7.14.1.3
The Network Customer may negotiate for a lower amount of NSR capacity by updating the capacity requested over time and changing the request status from ACCEPTED or COUNTEROFFER to REBID. For example, the Network Customer may choose to coordinate the level of transmission service purchased across multiple Transmission Providers. Negotiation for a higher amount of NSR capacity shall not be allowed. A Network Customer seeking additional NSR capacity must submit a new request, which will be assigned a new unique identifier and a new queue time.

001-105.7.14.1.4
Once the status of the request for new Network Scheduling Right has been set to ACCEPTED or COUNTEROFFER, the Network Customer may change the status to CONFIRMED after positively acknowledging agreement by setting the requested Network Scheduling Right capacity equal to the Transmission Provider’s granted Network Scheduling Right capacity.  Alternatively, the Network Customer may withdraw the request or the Network Customer may continue negotiations consistent with Business Practice Standard WEQ-001-105-.7.14.1.2.

Southern:

001-105.7.14.1.4
Once the status of the request for new NSR has been set to ACCEPTED or COUNTEROFFER, the Network Customer may change the status to CONFIRMED after positively acknowledging agreement by setting the requested NSR capacity equal to the Transmission Provider’s granted NSR capacity.  Alternatively, the Network Customer may withdraw the request or the Network Customer may continue negotiations consistent with Business Practice Standard WEQ-001-105-.7.14.1.2.

001-105.7.15
Final Disposition of Request for New Network Scheduling Right

Southern:

001-105.7.15
Final Disposition of Request for New NSR

001-105.7.15.1
Once CONFIRMED, the request for new Network Scheduling Right will be incorporated into the associated CONFIRMED NITS Application.

Southern:

001-105.7.15.1
Once CONFIRMED, the request for new NSR will be incorporated into the associated CONFIRMED NITS Application.

001-105.7.15.2
If, within the time limits specified in Table 105-A NITS Request Timing Requirements, the Network Customer fails to set the status of the ACCEPTED request for new Network Scheduling Right to CONFIRMED or WITHDRAWN, the Transmission Provider may set the status of the request to RETRACTED. 

Southern:

001-105.7.15.2
If, within the time limits specified in Table 105-A NITS Request Timing Requirements, the Network Customer fails to set the status of the ACCEPTED request for new NSR to CONFIRMED or WITHDRAWN, the Transmission Provider may set the status of the request to RETRACTED.

001-105.7.15.2.1
In response to a state of RETRACTED, the Network Customer may submit new or revised request for a new Network Scheduling Right, which will be assigned a new unique identifier and a new queue time.

Southern:

001-105.7.15.2.1
In response to a state of RETRACTED, the Network Customer may submit new or revised request for a new NSR, which will be assigned a new unique identifier and a new queue time.

Entergy:

001-105.7.15.2.1
In response to a state of RETRACTED, the Network Customer may submit new or revised request for a new Network Scheduling Right, which will be assigned a new unique identifier and a new queue time
.

001-105.7.15.3
Should the Network Customer elect to initiate or continue the negotiation of Network Scheduling Right capacity by changing the status of the request for new Network Scheduling Right to REBID, the Transmission Provider shall respond by changing the status of the request to DECLINED, ACCEPTED, or COUNTEROFFER within the Provider Counter Time Limit, specified in Table 105-A NITS Request Timing Requirements. The Transmission Provider response time is measured from the most recent REBID time.

Southern:

001-105.7.15.3
Should the Network Customer elect to initiate or continue the negotiation of NSR capacity by changing the status of the request for new NSR to REBID, the Transmission Provider shall respond by changing the status of the request to DECLINED, ACCEPTED, or COUNTEROFFER within the Provider Counter Time Limit, specified in Table 105-A NITS Request Timing Requirements. The Transmission Provider response time is measured from the most recent REBID time.

Duke Energy:

001-105.7.15.3
Should the Network Customer elect to initiate or continue the negotiation of Network Scheduling Right capacity by changing the status of the request for new Network Scheduling Right to REBID, the Transmission Provider shall respond by changing the status of the request to DECLINED, ACCEPTED, or COUNTEROFFER within the Provider Counter Time Limit, specified in Table 105-A NITS Request Timing Requirements. The Transmission Provider response time is measured from the most recent REBID time
.

Entergy:

001-105.7.15.3.1
The Transmission Provider shall positively acknowledge agreement to new Network Scheduling Right capacity over time by setting the granted Network Scheduling Right capacity equal to the Network Customer’s requested Network Scheduling Right capacity prior to acceptance of the request.

Southern:

001-105.7.15.3.1
The Transmission Provider shall positively acknowledge agreement to new NSR capacity over time by setting the granted NSR capacity equal to the Network Customer’s requested  NSR capacity prior to acceptance of the request.

Duke Energy:

001-105.7.15.3.1
The Transmission Provider shall positively acknowledge agreement to new Network Scheduling Right capacity over time by setting the granted Network Scheduling Right capacity equal to the Network Customer’s requested Network Scheduling Right capacity prior to acceptance of the request.

Entergy:

001-105.8
Temporary Termination of Network Scheduling Right

Southern:

001-105.8
Temporary Termination of NSR

OATI:

001-105.8
Temporary Termination of Network Scheduling Right

001-105.8.1
Use of a request for temporary termination of Network Scheduling Right is optional at the discretion of the Transmission Provider. Such request will be processed in accordance with the following:

Southern:

001-105.8.1
Use of a request for temporary termination of NSR is optional at the discretion of the Transmission Provider. Such request will be processed in accordance with the following:

Duke Energy:

001-105.8.1
Use of a request for temporary termination of Network Scheduling Right is optional at the discretion of the Transmission Provider. Such request will be processed in accordance with the following:

001-105.8.1.1
The Transmission Provider may complete a request for temporary termination of Network Scheduling Right and may set the status of the request to ACCEPTED. 

Southern:

001-105.8.1.1
The Transmission Provider may complete a request for temporary termination of NSR and may set the status of the request to ACCEPTED. 

Entergy:

001-105.8.1.1
The Transmission Provider may complete a request for temporary termination of Network Scheduling Right on behalf of the Network Customer and may set the status of the request to ACCEPTED. 

001-105.8.1.2
Alternatively, the Network Customer may submit a request for temporary termination of Network Scheduling Right.

Southern:

001-105.8.1.2
Alternatively, the Network Customer may submit a request for temporary termination of NSR.

001-105.8.2
The Transmission Provider may specify, in its posted business practice, if the request for temporary termination of Network Scheduling Right shall be used and the process the Transmission Provider requires based on the Business Practice Standard WEQ-001-105.8.1 for completion of a request for temporary termination of Network Scheduling Right.

Southern:

001-105.8.2
The Transmission Provider may specify, in its posted Business Practice, if the request for temporary termination of NSR shall be used and the process the Transmission Provider requires based on the Business Practice Standard WEQ-001-105.8.1 for completion of a request for temporary termination of NSR.

Entergy:

001-105.8.2
The Transmission Provider may specify, in a posted business practice, if the request for temporary termination of Network Scheduling Right shall be used and the process the Transmission Provider requires based on the Business Practice Standard WEQ-001-105.8.1 for completion of a request for temporary termination of Network Scheduling Right
.

001-105.8.3
A request for temporary termination of Network Scheduling Right shall only be granted when associated with a request for temporary termination of Designated Network Resource. The capacity granted for request for temporary termination of Network Scheduling Right may be different from the capacity granted for a request for temporary termination of a Designated Network Resource(s).

Southern:

001-105.8.3
A request for temporary termination of NSR shall only be granted when associated with a request for temporary termination of DNR. The capacity granted for request for temporary termination of NSR may be different from the capacity granted for a request for temporary termination of a DNR(s).

Duke Energy:

001-105.8.3
A request for temporary termination of Network Scheduling Right shall only be granted when associated with a request for temporary termination of Designated Network Resource. 
The capacity granted for request for temporary termination of Network Scheduling Right may be different from the capacity granted for a request for temporary termination of a Designated Network Resource(s).

001-105.8.4
Submission of Request for Temporary Termination of Network Scheduling Right 

Southern:

001-105.8.4
Submission of Request for Temporary Termination of NSR 

001-105.8.4.1
The Network Customer may enter and update information on OASIS in association with a request for temporary termination of Network Scheduling Right prior to submission to the Transmission Provider by setting the status of the request to PENDING.

Southern:

001-105.8.4.1
The Network Customer may enter and update information on OASIS in association with a request for temporary termination of NSR prior to submission to the Transmission Provider by setting the status of the request to PENDING.

Entergy:

001-105.8.4.1
The Network Customer may enter and update information on OASIS to support with a request for temporary termination of Network Scheduling Right prior to submission to the Transmission Provider by setting the status of the request to PENDING.

001-105.8.4.2
The Network Customer shall indicate to the Transmission Provider that the request for temporary termination of Network Scheduling Right is ready for Transmission Provider review by submitting the request on OASIS. 
Southern:

001-105.8.4.2
The Network Customer shall indicate to the Transmission Provider that the request for temporary termination of NSR is ready for Transmission Provider review by submitting the request on OASIS. 

001-105.8.4.3
OASIS shall set the queue time for the request for temporary termination of Network Scheduling Right to the time that the Network Customer submits the request, which shall set the request to QUEUED.

Southern:

001-105.8.4.3
OASIS shall set the queue time for the request for temporary termination of NSR to the time that the Network Customer submits the request, which shall set the request to QUEUED.

Entergy:

001-105.8.4.3
OASIS shall set the queue time for the request for temporary termination of Network Scheduling Right to the time that the Network Customer submits the request and shall set the request to QUEUED.

001-105.8.4.4
The Transmission Provider shall acknowledge the queued request for temporary termination of Network Scheduling Right by setting the status of the request to RECEIVED.

Southern:

001-105.8.4.4
The Transmission Provider shall acknowledge the queued request for temporary termination of NSR by setting the status of the request to RECEIVED.

001-105.8.5
Initial Review of Request for Temporary Termination of Network Scheduling Right 

Southern:

001-105.8.5
Initial Review of Request for Temporary Termination of NSR 

001- 105.8.5.1
The Transmission Provider shall review the request for temporary termination of Network Scheduling Right to determine if the request: (1) meets the minimum requirements specified in the Transmission Provider’s posted business practices; and (2) is consistent with the Transmission Provider’s tariff requirements.

Southern:

001- 105.8.5.1
The Transmission Provider shall review the request for temporary termination of NSR to determine if the request: (1) meets the minimum requirements specified in the Transmission Provider’s posted Business Practices; and (2) is consistent with the Transmission Provider’s tariff requirements.

Duke Energy:

001- 105.8.5.1
The Transmission Provider shall review the request for temporary termination of Network Scheduling Right to determine if the request: (1) meets the minimum requirements specified in the Transmission Provider’s posted business practices; and (2) is consistent with the Transmission Provider’s tariff requirements.

001- 105.8.5.2
The Transmission Provider shall respond to a request by setting the status to one of the following:

I.
INVALID 

II.
COMPLETED

001-105.8.6
Invalid Request for Temporary Termination of Network Scheduling Right 

Southern:

001-105.8.6
Invalid Request for Temporary Termination of NSR 

001-105.8.6.1
If the information necessary to process the request for temporary termination of Network Scheduling Right is not provided, the Transmission Provider shall set the state of the request for temporary termination of a Network Scheduling Right to INVALID and the request shall be deemed withdrawn.

Southern:

001-105.8.6.1
If the information necessary to process the request for temporary termination of NSR is not provided, the Transmission Provider shall set the state of the request for temporary termination of a NSR to INVALID and the request shall be deemed withdrawn.

Duke Energy:

001-105.8.6.1
If the information necessary to process the request for temporary termination of Network Scheduling Right is not provided, the Transmission Provider shall set the state of the request for temporary termination of a Network Scheduling Right to INVALID 
and the request shall be deemed withdrawn.

Entergy:

001-105.8.6.1
If the information necessary to process the request for temporary termination of Network Scheduling Right has been provided, the Transmission Provider shall set the state of the request for temporary termination of a Network Scheduling Right to INVALID and the request shall be deemed withdrawn.

001-105.8.6.2
In response to a state of INVALID, the Network Customer may submit a new or revised request for temporary termination of Network Scheduling Right, which will be assigned a new unique identifier and a new queue time.

Southern:

001-105.8.6.2
In response to a state of INVALID, the Network Customer may submit a new or revised request for temporary termination of NSR, which will be assigned a new unique identifier and a new queue time.

001-105.8.7
Completed Request for Temporary Termination of Network Scheduling Right 

Southern:

001-105.8.7
Completed Request for Temporary Termination of NSR 

001-105.8.7.1
Once the Transmission Provider has set the status of the request for temporary termination of Network Scheduling Right to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these Business Practice Standards.

Southern:

001-105.8.7.1
Once the Transmission Provider has set the status of the request for temporary termination of NSR to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these NITS Business Practice Standards.

001-105.8.8
Acceptance of Request for Temporary Termination of Network Scheduling Right 

Southern:

001-105.8.8
Acceptance of Request for Temporary Termination of NSR 

001-105.8.7.1
Once the status of the request for temporary termination of Network Scheduling Right is set to COMPLETED, the status of the request shall then be changed to ACCEPTED.

Southern:

001-105.8.8.1
Once the status of the request for temporary termination of NSR is set to COMPLETED, the status of the request shall then be changed to ACCEPTED.

Entergy:

001-105.8.7.1
Once the status of the request for temporary termination of Network Scheduling Right is set to COMPLETED, OASIS shall set the status of the request to ACCEPTED.

001-105.8.9
Customer Actions

001-105.8.8.1
After the status of the request for temporary termination of Network Scheduling Right has been set to ACCEPTED by the Transmission Provider, the Network Customer may confirm or withdraw the request by setting the status to CONFIRMED or WITHDRAWN. 

Southern:

001-105.8.9.1
After the status of the request for temporary termination of NSR has been set to ACCEPTED by the Transmission Provider, the Network Customer may confirm or withdraw the request by setting the status to CONFIRMED or WITHDRAWN. 

001-105.8.10
Final Disposition of Request for Temporary Termination of Network Scheduling Right 

Southern:

001-105.8.10
Final Disposition of Request for Temporary Termination of NSR 

001-105.8.9.1
Once CONFIRMED, the request for temporary termination of Network Scheduling Right will be incorporated into the associated CONFIRMED NITS Application.

Southern:

001-105.8.10.1
Once CONFIRMED, the request for temporary termination of NSR will be incorporated into the associated CONFIRMED NITS Application.

001-105.8.9.2
A CONFIRMED request for temporary termination of Network Scheduling Right shall only be effective for the duration of the granted request. 
Southern:

001-105.8.10.2
A CONFIRMED request for temporary termination of NSR shall only be effective for the duration of the granted request. 

001-105.8.9.3
If, within the time limits specified in Table 105-A NITS Request Timing Requirements, the Network Customer fails to set the status of the ACCEPTED request for temporary termination of Network Scheduling Right to CONFIRMED or WITHDRAWN, the Transmission Provider may set the status of the request to RETRACTED. 

Southern:

001-105.8.10.3
If, within the time limits specified in Table 105-A NITS Request Timing Requirements, the Network Customer fails to set the status of the ACCEPTED request for temporary termination of NSR to CONFIRMED or WITHDRAWN, the Transmission Provider may set the status of the request to RETRACTED. 

001-105.8.9.4.1
In response to a state of RETRACTED, the Network Customer may submit a new or revised request for temporary termination of Network Scheduling Right, which will be assigned a new unique identifier and a new queue time.

Southern:

001-105.8.10.4
In response to a state of RETRACTED, the Network Customer may submit a new or revised request for temporary termination of NSR, which will be assigned a new unique identifier and a new queue time.

001-105.9
Indefinite Termination of Network Scheduling Right

Southern:

001-105.9
Indefinite Termination of NSR

OATI:

001-105.9
Indefinite Termination of Network Scheduling Right

001-105.9.1
Use of a request for indefinite termination of Network Scheduling Right is optional at the discretion of the Transmission Provider. Such request will be processed in accordance with the following:

Southern:

001-105.9.1
Use of a request for indefinite termination of NSR is optional at the discretion of the Transmission Provider. Such request will be processed in accordance with the following:

Duke Energy:

001-105.9.1
Use of a request for indefinite termination of Network Scheduling Right is optional at the discretion of the Transmission Provider. Such request will be processed in accordance with the following:

001-105.9.1.1
The Transmission Provider may complete a request for indefinite termination of Network Scheduling Right and may set the status of the request to ACCEPTED. 

Southern:

001-105.9.1.1
The Transmission Provider may complete a request for indefinite termination of NSR and may set the status of the request to ACCEPTED. 

Entergy:

001-105.9.1.1
The Transmission Provider may complete a request for indefinite termination of Network Scheduling Right on behalf of the Network Customer and may set the status of the request to ACCEPTED. 

001-105.9.1.2
Alternatively, the Network Customer may submit a request for indefinite termination of Network Scheduling Right.

Southern:

001-105.9.1.2
Alternatively, the Network Customer may submit a request for indefinite termination of NSR.

001-105.9.2
The Transmission Provider may specify, in its posted business practice, if the request for indefinite termination of Network Scheduling Right shall be used and the process the Transmission Provider requires based on the Business Practice Standard WEQ-001-105.9.1 for completion of a request for temporary termination of Network Scheduling Right.

Southern:

001-105.9.2
The Transmission Provider may specify, in its posted Business Practice, if the request for indefinite termination of NSR shall be used and the process the Transmission Provider requires based on the Business Practice Standard WEQ-001-105.9.1 for completion of a request for temporary termination of NSR.

Duke Energy:

001-105.9.2
The Transmission Provider may specify, in its posted business practice, if the request for indefinite termination of Network Scheduling Right shall be used and the process the Transmission Provider requires based on the Business Practice Standard WEQ-001-105.9.1 for completion of a request for indefinite termination of Network Scheduling Right.

Entergy:

001-105.9.2
The Transmission Provider may specify, in a posted business practice, if the request for indefinite termination of Network Scheduling Right shall be used and the process the Transmission Provider requires based on the Business Practice Standard WEQ-001-105.9.1 for completion of a request for temporary termination of Network Scheduling Right
.

001-105.9.3
A request for indefinite termination of Network Scheduling Right shall only be granted when associated with a request for indefinite termination of Designated Network Resource. The capacity granted for request for indefinite termination of Network Scheduling Right may be different from the capacity granted for a request for indefinite termination of a Designated Network Resource(s).

Southern:

001-105.9.3
A request for indefinite termination of NSR shall only be granted when associated with a request for indefinite termination of DNR. The capacity granted for request for indefinite termination of NSR may be different from the capacity granted for a request for indefinite termination of a DNR(s).

001-105.9.4
Submission of Request for Indefinite Termination of Network Scheduling Right

Southern:

001-105.9.4
Submission of Request for Indefinite Termination of NSR

001-105.9.4.1
The Network Customer may enter and update information on OASIS in association with a request for indefinite termination of Network Scheduling Right prior to submission to the Transmission Provider by setting the status of the request to PENDING. 

Southern:

001-105.9.4.1
The Network Customer may enter and update information on OASIS in association with a request for indefinite termination of NSR prior to submission to the Transmission Provider by setting the status of the request to PENDING. 

Entergy:

001-105.9.4.1
The Network Customer may enter and update information on OASIS to support with a request for indefinite termination of Network Scheduling Right prior to submission to the Transmission Provider by setting the status of the request to PENDING. 

001-105.9.4.2
The Network Customer shall indicate to the Transmission Provider that the request for indefinite termination of Network Scheduling Right is ready for Transmission Provider review by submitting the request on OASIS.  

Southern:

001-105.9.4.2
The Network Customer shall indicate to the Transmission Provider that the request for indefinite termination of NSR is ready for Transmission Provider review by submitting the request on OASIS.  

001-105.9.4.2.1
OASIS shall set the queue time for the request for indefinite termination of Network Scheduling Right to the time that the Network Customer submits the request, which shall set the request to QUEUED.

Southern:

001-105.9.4.2.1
OASIS shall set the queue time for the request for indefinite termination of NSR to the time that the Network Customer submits the request, which shall set the request to QUEUED.

Entergy:

001-105.9.4.2.1
OASIS shall set the queue time for the request for indefinite termination of Network Scheduling Right to the time that the Network Customer submits the request and shall set the request to QUEUED.

001-105.9.4.2.2
The Transmission Provider shall acknowledge the queued request for indefinite termination of Network Scheduling Right by setting the status of the request to RECEIVED.

Southern:

001-105.9.4.2.2
The Transmission Provider shall acknowledge the queued request for indefinite termination of NSR by setting the status of the request to RECEIVED.

001-105.9.5
Initial Review of Request for Indefinite Termination of a Network Scheduling Right

Southern:

001-105.9.5
Initial Review of Request for Indefinite Termination of a NSR

001-105.9.5.1
The Transmission Provider shall review the request for indefinite termination of Network Scheduling Right to determine if the request is consistent with the Transmission Provider’s tariff requirements.

Southern:

001-105.9.5.1
The Transmission Provider shall review the request for indefinite termination of NSR to determine if the request is consistent with the Transmission Provider’s tariff requirements.

001-105.9.5.2
The Transmission Provider shall respond to a request for indefinite termination of Network Scheduling Right by setting the status to one of the following:

I.
INVALID 

II.
COMPLETED

Southern:

001-105.9.5.2
The Transmission Provider shall respond to a request for indefinite termination of NSR by setting the status to one of the following:

I.
INVALID 

II.
COMPLETED

001-105.9.6
Invalid Request for Indefinite Termination of Network Scheduling Right

Southern:

001-105.9.6
Invalid Request for Indefinite Termination of NSR

001-105.9.6.1
If the information necessary to process the request for indefinite termination of a Network Scheduling Right is not provided, the Transmission Provider shall set the state of the request to INVALID and the request shall be deemed withdrawn.

Southern:

001-105.9.6.1
If the information necessary to process the request for indefinite termination of a NSR is not provided, the Transmission Provider shall set the state of the request to INVALID and the request shall be deemed withdrawn.

Entergy:

001-105.9.6.1
If the information necessary to process the request for indefinite termination of a Network Scheduling Right has not been provided, the Transmission Provider shall set the state of the request to INVALID and the request shall be deemed withdrawn.

001-105.9.6.2
In response to a state of INVALID, the Network Customer may submit a new or revised request for indefinite termination of a Network Scheduling Right, which will be assigned a new unique identifier and a new queue time. 
Southern:

001-105.9.6.2
In response to a state of INVALID, the Network Customer may submit a new or revised request for indefinite termination of a NSR, which will be assigned a new unique identifier and a new queue time. 

001-105.9.7
Completed Request for Indefinite Termination of Network Resource 

Southern:

001-105.9.7
Completed Request for Indefinite Termination of NSR 

001-105.9.7.1
Once the Transmission Provider has set the status of the request for indefinite termination of Network Resource to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these Business Practice Standards.

Southern:

001-105.9.7.1
Once the Transmission Provider has set the status of the request for indefinite termination of NSR to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these NITS Business Practice Standards.

001-105.9.8
Acceptance of Request for Indefinite Termination of Network Scheduling Right

Southern:

001-105.9.8
Acceptance of Request for Indefinite Termination of NSR

001-105.9.8.1
Once the request for indefinite termination of Network Scheduling Right is deemed to be valid, the Transmission Provider shall set the status of the request to ACCEPTED.

Southern:

001-105.9.8.1
Once the request for indefinite termination of NSR is deemed to be valid, the Transmission Provider shall set the status of the request to ACCEPTED.

001-105.9.9
Customer Actions

001-105.9.9.1
After the status of the request for indefinite termination of Network Scheduling Right has been set to ACCEPTED by the Transmission Provider, the Network Customer may confirm or withdraw the request by setting the status to CONFIRMED or WITHDRAWN. 

Southern:

001-105.9.9.1
After the status of the request for indefinite termination of NSR has been set to ACCEPTED by the Transmission Provider, the Network Customer may confirm or withdraw the request by setting the status to CONFIRMED or WITHDRAWN. 

001-105.9.10
Final Disposition of Request for Indefinite Termination of Network Scheduling Right

Southern:

001-105.9.10
Final Disposition of Request for Indefinite Termination of NSR

001-105.9.10.1
Once CONFIRMED, the request for indefinite termination of Network Scheduling Right will be incorporated into the associated CONFIRMED NITS Application.

Southern:

001-105.9.10.1
Once CONFIRMED, the request for indefinite termination of NSR will be incorporated into the associated CONFIRMED NITS Application.

001-105.9.10.2
If, within the time limits specified in Table 105-A NITS Request Timing Requirements the Network Customer fails to set the status of the ACCEPTED request for indefinite termination of Network Scheduling Right to CONFIRMED or WITHDRAWN, the Transmission Provider may set the status of the request to RETRACTED. 

Southern:

001-105.9.10.2
If, within the time limits specified in Table 105-A NITS Request Timing Requirements the Network Customer fails to set the status of the ACCEPTED request for indefinite termination of NSR to CONFIRMED or WITHDRAWN, the Transmission Provider may set the status of the request to RETRACTED. 

Entergy:

001-105.9.10.2
If the Network Customer fails to set the status of the ACCEPTED request for indefinite termination of Network Scheduling Right to CONFIRMED or WITHDRAWN, the Transmission Provider may set the status of the request to RETRACTED
. 

001-105.9.10.2.1
In response to a state of RETRACTED, the Network Customer may submit a new or revised request for indefinite termination of Network Scheduling Right, which will be assigned a new unique identifier and a new queue time.

Southern:

001-105.9.10.2.1
In response to a state of RETRACTED, the Network Customer may submit a new or revised request for indefinite termination of NSR, which will be assigned a new unique identifier and a new queue time.

10/12/2010 ALL COMMENTS IN THE HIGHLIGHTED SECTION ABOVE HAVE BEEN RESOLVED BY DELETING THIS SECTION
001-105.10
Secondary Network Transmission Service 

OATI:

001-105.10
Secondary Network Transmission Service 

001-105.10.1
Submission of Request for Secondary Network Transmission Service 

001-105.10.1.1
The Network Customer may enter and update information on OASIS in association with a request for Secondary Network Transmission Serviceprior to submission to the Transmission Provider by setting the status of the request to PENDING.

Duke Energy:

001-105.10.1.1
The Network Customer may enter and update information on OASIS in association with a request for Secondary Network Transmission Service prior to submission to the Transmission Provider by setting the status of the request to PENDING.

Entergy:

001-105.10.1.1
The Network Customer may enter and update information on OASIS to support with a request for Secondary Network Transmission Serviceprior to submission to the Transmission Provider by setting the status of the request to PENDING.

001- 105.10.1.2
The Transmission Provider may specify, in its posted business practice, the minimum information required for Secondary Network Transmission Service.
Southern:

001- 105.10.1.2
The Transmission Provider may specify, in its posted Business Practice, the minimum information required for Secondary Network Transmission Service.

NCEMC:

001- 105.10.1.2
The Transmission Provider may specify, in its posted business practice, the minimum information required for Secondary Network Transmission Service.

BPA:

001- 105.10.1.2
The Transmission Provider may specify, in its posted business practice
, the minimum information required for Secondary Network Transmission Service.
Entergy:

001- 105.10.1.2
The Transmission Provider may specify, in a posted business practice, the minimum information required for Secondary Network Transmission Service
.

001- 105.10.1.3
The Network Customer shall indicate to the Transmission Provider that the request for Secondary Network Transmission Service is ready for Transmission Provider review by submitting the request on OASIS.  

001-105.10.1.3.1
OASIS shall set the queue time for the request for Secondary Network Transmission Service to the time that the Network Customer submits the request, which shall set the status of the request to QUEUED.

Entergy:

001-105.10.1.3.1
OASIS shall set the queue time for the request for Secondary Network Transmission Service to the time that the Network Customer submits the request and shall set the status of the request to QUEUED.

001-105.10.1.3.2
The Transmission Provider shall acknowledge the queued request for Secondary Network Transmission Service by setting the status of the request to RECEIVED.

001-105.10.2
Initial Review of Request for Secondary Network Transmission Service
001- 105.10.2.1
The Transmission Provider shall review the request for Secondary Network Transmission Service to determine if the request: (1) meets the minimum requirements specified in the Transmission Provider’s posted business practices; or (2) is consistent with the Transmission Provider’s tariff requirements.

Southern:

001- 105.10.2.1
The Transmission Provider shall review the request for Secondary Network Transmission Service to determine if the request: (1) meets the minimum requirements specified in the Transmission Provider’s posted Business Practices; or (2) is consistent with the Transmission Provider’s tariff requirements.

BPA:

001- 105.10.2.1
The Transmission Provider shall review the request for Secondary Network Transmission Service to determine if the request: (1) meets the minimum requirements specified in the Transmission Provider’s posted business practices; or (2) is consistent with the Transmission Provider’s tariff requirements.

001- 105.10.2.2
The Transmission Provider shall respond to a request by setting the status to one of the following:

I.
INVALID 

II.
COMPLETED

001-105.10.3
Invalid Request for Secondary Network Transmission Service
001-105.10.3.1
If the required information, as specified in Business Practice Standard WEQ-001-105.10.2.1, is not provided, the Transmission Provider shall set the state of the request for Secondary Network Transmission Service to INVALID and the request shall be deemed withdrawn.

Entergy:

001-105.10.3.1
If the required information, as specified in Business Practice Standard WEQ-001-105.10.2.1, has not been provided in the request for Secondary Network Transmission Service, the Transmission Provider shall set the state of the request for Secondary Network Transmission Service to INVALID and the request shall be deemed withdrawn.

001-105.10.3.2
In response to a state of INVALID, the Network Customer may submit a new or revised request for Secondary Network Transmission Service, which will be assigned a new unique identifier and a new queue time.   

001-105.10.4
Completed Request for Secondary Network Transmission Service
001-105.10.4.1
Once the Transmission Provider has set the status of the request for Secondary Network Transmission Service to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these Business Practice Standards.

Southern:

001-105.10.4.1
Once the Transmission Provider has set the status of the request for Secondary Network Transmission Service to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these NITS Business Practice Standards.

001-105.10.5
Review of Completed Request for Secondary Network Transmission Service 

001- 105.10.5.1
Once the Transmission Provider has set the status of the request for Secondary Network Transmission Service to COMPLETED, the Transmission Provider shall respond to the request, consistent with filed tariffs, within the Provider Response Time Limit defined in Table 105-A NITS Request  Timing Requirements. The time limit is measured from the time the request is QUEUED.  To indicate its evaluation of the request, a Transmission Provider may respond by setting the state of the request to one of the following:
I.
DECLINED

II.
REFUSED 

III.
COUNTEROFFER

IV.
ACCEPTED 

Entergy:

001- 105.10.5.1
Once the Transmission Provider has set the status of the request for Secondary Network Transmission Service to COMPLETED, the Transmission Provider shall respond to the request, consistent with filed tariffs, within the Provider Response Time Limit defined in Table 105-A NITS Request Timing Requirements. The time limit is measured from the time the request is QUEUED.  To indicate its evaluation of the request, a Transmission Provider may respond by setting the state of the request to one of the following:
I.
DECLINED

II.
REFUSED 

III.
COUNTEROFFER

IV.
ACCEPTED 

001-105.10.6
Decline of Request for Secondary Network Transmission Service
001-105.10.6.1
If, at any time in the evaluation of the request for Secondary Network Transmission Service, the Transmission Provider determines that the Network Customer has failed to meet any of the terms and conditions for Secondary Network Transmission Service, the Transmission Provider shall set the status of the request to DECLINED.

001-105.10.6.2
In response to a state of DECLINED, the Network Customer may submit a new or revised request for Secondary Network Transmission Service, which will be assigned a new unique identifier and a new queue time.

001-105.10.7
Refusal of Request for Secondary Network Transmission Service
001-105.10.7.1
If, at any time in the evaluation of the request for Secondary Network Transmission Service, the Transmission Provider determines that the requested service cannot be provided due to lack of availability of transfer capability consistent with the Transmission Provider’s tariff requirements, the Transmission Provider may set the status of the request to REFUSED.

001-105.10.7.2
In response to a state of REFUSED, the Network Customer may submit a new or revised request for Secondary Network Transmission Service, which will be assigned a new unique identifier and a new queue time.

001-105.10.8
Counteroffer to Request for Secondary Network Transmission Service
001-105.10.8.1
When, in the evaluation of a request for Secondary Network Transmission Service, the Transmission Provider determines that the request may be granted in part, the Transmission Provider may make a partial offer.  

001-105.10.8.1.1
If the Transmission Provider makes a partial offer, the Transmission Provider shall notify the Network Customer on OASIS by setting the amount of Secondary Network Transmission Service capacity granted over time in the response to the request for Secondary Network Transmission Service and setting the status of such request to COUNTEROFFER.

001-105.10.9
Acceptance of Request for Secondary Network Transmission Service
001-105.10.9.1
When, in the evaluation of the request for Secondary Network Transmission Service, the Transmission Provider determines that the request can be approved, the Transmission Provider shall set the status of the request to ACCEPTED.

001-105.10.10
Customer Actions

001-105.10.10.1
From ACCEPTED or COUNTEROFFER, the Network Customer may change the status of the request for Secondary Network Transmission Service to CONFIRMED, REBID or WITHDRAWN. The Network Customer has the amount of time designated as Customer Confirmation Time Limit in Table 105-A NITS Request Timing Requirements to change the state of the request. The Customer Confirmation Time Limit is measured from the first time the request is moved to ACCEPTED or COUNTEROFFER, and is not reset with subsequent iterations of negotiation. 

001-105.10.10.1.1
The Network Customer shall use the request status of REBID to initiate or continue the negotiation of Secondary Network Transmission Service capacity.

Entergy:

001-105.10.10.1.1
The Network Customer shall use the request status of REBID to initiate negotiation of Secondary Network Transmission Service capacity or engage in continued negotiation of the Secondary Network Transmission Service capacity with the Transmission Provider.

001-105.10.10.1.2
Should the Network Customer elect to initiate or continue the negotiation of Secondary Network Transmission Service capacity by moving the request to REBID, the Transmission Provider shall respond by taking the request to a DECLINED, SUPERSEDED, ACCEPTED, or COUNTEROFFER state within the Provider Counter Time Limit, specified in Table 105-A NITS Request Timing Requirements. The Transmission Provider response time is measured from the most recent REBID time.

001-105.10.10.1.3
The Network Customer may negotiate for a lower amount of Secondary Network Transmission Service capacity by updating the capacity requested over time and changing the request status from ACCEPTED or COUNTEROFFER to REBID. Negotiation for a higher amount of Secondary Network Transmission Service capacity shall not be allowed. A Network Customer seeking additional Secondary Network Transmission Service capacity must submit a new request, which will be assigned a new unique identifier and a new queue time.

001-105.10.10.1.4
Once the status of the request for Secondary Network Transmission Service has been set to ACCEPTED or COUNTEROFFER, the Network Customer may change the status to CONFIRMED after positively acknowledging agreement by setting the requested Secondary Network Transmission Service capacity equal to the Transmission Provider’s granted Secondary Network Transmission Service capacity.  Alternatively, the Network Customer may withdraw the request or the Network Customer may continue negotiations consistent with Business Practice Standard WEQ-001-105-.10.10.1.2.

001-105.10.11
Final Disposition of Request for Secondary Network Transmission Service
001-105.10.11.1
Once CONFIRMED, the request for Secondary Network Transmission Service will be incorporated into the associated CONFIRMED NITS Application.

NCEMC:

001-105.10.11.1
Once CONFIRMED, the request for Secondary Network Transmission Service will be incorporated into the associated CONFIRMED NITS Application.


BPA:

001-105.11.1
Use of a request for termination of Secondary Network Transmission Service, either for temporary termination or indefinite termination, is optional 
at the discretion of the Transmission Provider.

001-105.10.11.2
If, within the time limits specified in Table 105-A NITS Request Timing Requirements, the Network Customer fails to set the status of the ACCEPTED request for Secondary Network Transmission Service to CONFIRMED or WITHDRAWN, the Transmission Provider may set the status of the request to RETRACTED. 

001-105.10.11.2.1
In response to a state of RETRACTED, the Network Customer may submit a new or revised request for Secondary Network Transmission Service, which will be assigned a new unique identifier and a new queue time.

Entergy:

001-105.10.11.2.1
In response to a state of RETRACTED, the Network Customer may submit a new or revised request for Secondary Network Transmission Service, which will be assigned a new unique identifier and a new queue time
.

001-105.10.11.3
Should the Network Customer elect to initiate or continue the negotiation of Secondary Network Transmission Service capacity by changing the status of the request for Secondary Network Transmission Service to REBID, the Transmission Provider shall respond by changing the status of the request to DECLINED, ACCEPTED, or COUNTEROFFER within the Provider Counter Time Limit, specified in Table 105-A NITS Request Timing Requirements. The Transmission Provider response time is measured from the most recent REBID time.

Entergy:

001-105.10.11.3.1
The Transmission Provider shall positively acknowledge agreement to Secondary Network Transmission Service capacity over time by setting the granted Secondary Network Transmission Service capacity equal to the Network Customer’s requested Secondary Network Transmission Service capacity prior to acceptance of the request.

NCEMC:

001-105.10.11.3.1
The Transmission Provider shall positively acknowledge agreement to Secondary Network Transmission Service capacity over time by setting the granted Secondary Network Transmission Service capacity equal to the Network Customer’s requested Secondary Network Transmission Service capacity prior to acceptance of the request.

Entergy:



001-105.11
Termination of Secondary Network Transmission Service
Southern:

001-105.11
Temporary/Indefinite Termination of Secondary Network Transmission Service

OATI:

001-105.11
Termination of Secondary Network Transmission Service

001-105.11.1
Use of a request for termination of Secondary Network Transmission Service, either for temporary termination or indefinite termination, is optional at the discretion of the Transmission Provider.

Duke Energy:

001-105.11.1
Use of a request for termination of Secondary Network Transmission Service, either for temporary termination or indefinite termination, is optional at the discretion of the Transmission Provider.

001-105.11.2
If a request for termination of Secondary Network Transmission Service is used, the Transmission Provider shall specify such use in its posted business practice.

Southern:

001-105.11.2
If a request for temporary/indefinite termination of Secondary Network Transmission Service is used, the Transmission Provider shall specify such use in its posted Business Practice.

001-105.11.3
Request for Termination of Secondary Network Transmission Service
Southern:

001-105.11.3
Request for Temporary/Indefinite Termination of Secondary Network Transmission Service

001-105.11.3.1
The Transmission Provider shall not refuse a valid and complete request for termination of Secondary Network Transmission Service.

Southern:

001-105.11.3.1
The Transmission Provider shall not refuse a valid and complete request for temporary/indefinite termination of Secondary Network Transmission Service.

001-105.11.4
Submission of Request for Termination of Secondary Network Transmission Service 

Southern:

001-105.11.4
Submission of Request for Temporary/Indefinite Termination of Secondary Network Transmission Service 

001-105.11.4.1
The Network Customer may enter and update information on OASIS in association with a request for termination of Secondary Network Transmission Service prior to submission to the Transmission Provider by setting the status of the request to PENDING.

Southern:

001-105.11.4.1
The Network Customer may enter and update information on OASIS in association with a request for temporary/indefinite termination of Secondary Network Transmission Service prior to submission to the Transmission Provider by setting the status of the request to PENDING.

Entergy:

001-105.11.4.1
The Network Customer may enter and update information on OASIS to support with a request for termination of Secondary Network Transmission Service prior to submission to the Transmission Provider by setting the status of the request to PENDING.

001-105.11.4.2
The Network Customer shall indicate to the Transmission Provider that the request for termination of Secondary Network Transmission Service is ready for Transmission Provider review by submitting the request on OASIS.  

Southern:

001-105.11.4.2
The Network Customer shall indicate to the Transmission Provider that the request for temporary/indefinite termination of Secondary Network Transmission Service is ready for Transmission Provider review by submitting the request on OASIS.  

001-105.11.4.3
OASIS shall set the queue time for the request for termination of Secondary Network Transmission Service to the time that the Network Customer submits the request, which shall set the request to QUEUED.

Southern:

001-105.11.4.3
OASIS shall set the queue time for the request for temporary/indefinite termination of Secondary Network Transmission Service to the time that the Network Customer submits the request, which shall set the request to QUEUED.

Entergy:

001-105.11.4.3
OASIS shall set the queue time for the request for termination of Secondary Network Transmission Service to the time that the Network Customer submits the request and shall set the request to QUEUED.

001-105.11.4.4
The Transmission Provider shall acknowledge the queued request for termination of Secondary Network Transmission Serviceby setting the status of the request to RECEIVED.

Southern:

001-105.11.4.4
The Transmission Provider shall acknowledge the queued request for temporary/indefinite termination of Secondary Network Transmission Serviceby setting the status of the request to RECEIVED.

BPA:

001-105.11.4.4
The Transmission Provider shall acknowledge the queued request for termination of Secondary Network Transmission Service by setting the status of the request to RECEIVED.

001-105.11.5
Initial Review of Request for Termination of Secondary Network Transmission Service 

Southern:

001-105.11.5
Initial Review of Request for Temporary/Indefinite Termination of Secondary Network Transmission Service 

001- 105.11.5.1
The Transmission Provider shall review the request for termination of Secondary Network Transmission Service to determine if the request: (1) meets the minimum requirements specified in the Transmission Provider’s posted business practices; and (2) is consistent with the Transmission Provider’s tariff requirements.

Southern:

001- 105.11.5.1
The Transmission Provider shall review the request for temporary/indefinite termination of Secondary Network Transmission Service to determine if the request: (1) meets the minimum requirements specified in the Transmission Provider’s posted Business Practices; and (2) is consistent with the Transmission Provider’s tariff requirements.

001- 105.11.5.2
The Transmission Provider shall respond to a request by setting the status to one of the following:

I.
INVALID 

II.
COMPLETED

001-105.11.6
Invalid Request for Termination of Secondary Network Transmission Service 

Southern:

001-105.11.6
Invalid Request for Temporary/Indefinite Termination of Secondary Network Transmission Service 

001-105.11.6.1
If the information necessary to process the request for termination of Secondary Network Transmission Service is not provided, the Transmission Provider shall set the state of the request for termination of Secondary Network Transmission Service to INVALID and the request shall be deemed withdrawn.

Southern:

001-105.11.6.1
If the information necessary to process the request for temporary/indefinite termination of Secondary Network Transmission Service is not provided, the Transmission Provider shall set the state of the request for temporary/indefinite termination of Secondary Network Transmission Service to INVALID and the request shall be deemed withdrawn.

Entergy:

001-105.11.6.1
If the information necessary to process the request for termination of Secondary Network Transmission Service has not been provided, the Transmission Provider shall set the state of the request for termination of Secondary Network Transmission Service to INVALID and the request shall be deemed withdrawn.

001-105.11.6.2
In response to a state of INVALID, the Network Customer may submit a new or revised request for termination of Secondary Network Transmission Service, which will be assigned a new unique identifier and a new queue time.

Southern:

001-105.11.6.2
In response to a state of INVALID, the Network Customer may submit a new or revised request for temporary/indefinite termination of Secondary Network Transmission Service, which will be assigned a new unique identifier and a new queue time.

001-105.11.7
Completed Request for Termination of Secondary Network Transmission Service 

Southern:

001-105.11.7
Completed Request for Temporary/Indefinite Termination of Secondary Network Transmission Service 

001-105.11.7.1
Once the Transmission Provider has set the status of the request for termination of Secondary Network Transmission Service to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these Business Practice Standards.

Southern:

001-105.11.7.1
Once the Transmission Provider has set the status of the request for temporary/indefinite termination of Secondary Network Transmission Service to COMPLETED, the Transmission Provider shall continue processing the request in accordance with these NITS Business Practice Standards.

001-105.11.8
Acceptance of Request for Termination of Secondary Network Transmission Service
Southern:

001-105.11.8
Acceptance of Request for Temporary/Indefinite Termination of Secondary Network Transmission Service

001-105.11.8.1
Once the status of the request for termination of Secondary Network Transmission Service is set to COMPLETED, the status of the request shall then be changed to ACCEPTED.

Southern:

001-105.11.8.1
Once the status of the request for temporary/indefinite termination of Secondary Network Transmission Service is set to COMPLETED, the status of the request shall then be changed to ACCEPTED.

Entergy:

001-105.11.8.1
Once the status of the request for termination of Secondary Network Transmission Service is set to COMPLETED, OASIS shall set the status of the request  to ACCEPTED.

001-105.11.9
Customer Actions

001-105.11.9.1
After the status of the request for termination of Secondary Network Transmission Service has been set to ACCEPTED by the Transmission Provider, the Network Customer may confirm or withdraw the request by setting the status to CONFIRMED or WITHDRAWN. 

Southern:

001-105.11.9.1
After the status of the request for temporary/indefinite termination of Secondary Network Transmission Service has been set to ACCEPTED by the Transmission Provider, the Network Customer may confirm or withdraw the request by setting the status to CONFIRMED or WITHDRAWN. 

Duke Energy:

001-105.11.9.1
After the status of the request for termination of Secondary Network Transmission Service has been set to ACCEPTED by the Transmission Provider, the Network Customer may confirm or withdraw the request by setting the status to CONFIRMED or WITHDRAWN
. 

001-105.11.10
Final Disposition of Request for Termination of Secondary Network Transmission Service 

Southern:

001-105.11.10
Final Disposition of Request for Temporary/Indefinite Termination of Secondary Network Transmission Service 

001-105.11.10.1
Once CONFIRMED, the request for termination of Secondary Network Transmission Service will be incorporated into the associated CONFIRMED NITS Application.

Southern:

001-105.11.10.1
Once CONFIRMED, the request for temporary/indefinite termination of Secondary Network Transmission Service will be incorporated into the associated CONFIRMED NITS Application
.

NCEMC:

001-105.11.10.1
Once CONFIRMED, the request for termination of Secondary Network Transmission Service will be incorporated into the associated CONFIRMED NITS Application.

001-105.11.10.2
If, within the time limits specified in Table 105-A NITS Request Timing Requirements, the Network Customer fails to set the status of the ACCEPTED request for termination of Secondary Network Transmission Service to CONFIRMED or WITHDRAWN, the Transmission Provider may set the status of the request to RETRACTED. 

Southern:

001-105.11.10.2
If, within the time limits specified in Table 105-A NITS Request Timing Requirements, the Network Customer fails to set the status of the ACCEPTED request for temporary/indefinite termination of Secondary Network Transmission Service to CONFIRMED or WITHDRAWN, the Transmission Provider may set the status of the request to RETRACTED. 

001-105.11.10.2.1
In response to a state of RETRACTED, the Network Customer may submit a new or revised request for termination of Secondary Network Transmission Service, which will be assigned a new unique identifier and a new queue time.

Southern:

001-105.11.10.2.1
In response to a state of RETRACTED, the Network Customer may submit a new or revised request for temporary/indefinite termination of Secondary Network Transmission Service, which will be assigned a new unique identifier and a new queue time.

Entergy:

001-105.11.10.2
If, within the time limits specified in Table 105-A NITS Request Timing Requirements, the Network Customer fails to set the status of the ACCEPTED request for termination of Secondary Network Transmission Service to CONFIRMED or WITHDRAWN, the Transmission Provider may set the status of the request to RETRACTED
.
 

HIGHLIGHTED SECTION ON SECONDARY NETWORK TO BE REWRITTEN BY BARBARA REHMAN
Note:  Need implementation plan 
NCEMC:

Note:  Need implementation plan 

001-106
NETWORK INTEGRATION TRANSMISSION SERVICE – CONCOMITANT REQUESTS 

Southern:

001-106
NETWORK INTEGRATION TRANSMISSION SERVICE – REQUEST FOR CONCOMITANT EVALUATION

Southern:

001-106
NITS – REQUEST FOR CONCOMITANT REQUESTS

OATI:

001-106
NETWORK INTEGRATION TRANSMISSION SERVICE – REQUEST FOR CONCOMITANT EVALUATION

· Suggest that the standard cite examples or list ALL the legal concomitant transactions, such as:
· Termination (Temporary or Indefinite) of existing DNR with concomitant request for a new Designated Network Resource, i.e., redesignation of Network Resource.
· Termination (Temporary or Indefinite) of existing DNR with concomitant request for a new Point-to-Point Transmission Service.
· (others?)
10/12/2010:  Subcommittee determined not to include a list of all circumstances that should be evaluated concomitantly.
# 1 Subcommittee determined to define “Concomitant Evaluation” in the Glossary
Concepts from OATT Section 30.3 V

· One transmission service request related to one request for temporary termination of a Network Resource

· The two requests must be approved or denied as a single request

· Such concomitant evaluations must take into account the termination of those Network Resources as well as all competing requests of higher priority

#2 Concomitant Evaluation is the evaluation of a request for new PTP or Network transmission service submitted in conjunction with a request for undesignation of a Network Resource.
The Standard (WEQ-001-106) will include the following concept:  Such concomitant evaluations must take into account the termination of those Network Resources as well as all competing requests of higher priority

10/12/2010:  The subcommittee decided to use Option #2 above for the definition of Concomitant Evaluation.
· Disagree with the one-to-one nature of the requests that may qualify.  Would seem that the ability to terminate a DNR and request multiple PTP reservations to be a viable option useful to customers.

· Rather than citing use of a ‘concomitant’ flag suggest that the standard simply require that the requests to be considered for concomitant evaluation be clearly identified; transaction model suggested for NITS template was envisioned to simply use a ‘transaction’ envelop for the concomitant requests rather than a flag on the individual requests.

· Suggest 001-106.4 be broadened to simply say that the individual requests included for concomitant evaluation shall each be bound by the processing requirements associated with the type of request submitted per standards called out in 001-105.  This would then include both the TPs obligations and process and TCs obligations and process.

· Rather than forcing concomitant requests to all go through a negotiation process, suggest it would be simpler to require that once all requests in concomitant eval are confirmed (by whatever means appropriate for that request), all concomitant requests are deemed complete and confirme; if any one of the concomitant requests is not taken to confirmed state, all the other concomitant requests are considered null and void.
001-106.1
The Network Customer shall only be allowed to request a concomitant evaluation of a request for temporary or indefinite termination of a single Network Resource and a single request for NITS or PTP transmission service on a single transmission provider system.

Duke Energy:

001-106.1
The Network Customer shall only be allowed to request a concomitant evaluation of a request for temporary or indefinite termination of a single Network Resource and a single request for NITS or PTP transmission service on a single transmission provider system
.

Entergy:

001-106.1
The Network Customer shall only be allowed to request a concomitant evaluation of a single request for temporary or indefinite termination of a single Network Resource and a single request for NITS or PTP transmission service on a single transmission provider system.

001-106.1.1
To be eligible for concomitant evaluation, each of the two single requests must be submitted together with the concomitant evaluation flag set. 

NCEMC:

001-106.1.1
To be eligible for concomitant evaluation, each of the two single requests must be submitted together with the concomitant evaluation flag set. 

Entergy:

001-106.1.1
To be eligible for concomitant evaluation, the two single requests must be submitted together with the concomitant evaluation flag set. 

001-106.1.2
The Transmission Provider shall evaluate the two single requests concomitantly.

Entergy:

001-106.1.2
The Transmission Provider shall then evaluate the two single requests concomitantly within the Provider Evaluation Time Limit specified in Table 105-A NITS Request Timing Requirements that is applicable to the single request for temporary or indefinite termination of a single Network Resource.

001-106.1.3
The Transmission Provider shall assign the same queue time to each of the two single requests.

BPA:

001-106.1.3
The Transmission Provider shall assign the same queue time to each of the two single requests, whichever queue time is later..

001-106.2
A single request that does not have the concomitant evaluation flag set will be considered a separate request and will be processed according to the appropriate Business Practice Standards for such request.

001-106.3
Denial of one of the single requests shall result in denial of both single requests. 

Duke Energy:

001-106.3
Denial
 of one of the single requests shall result in denial of both single requests. 

001- 106.4
The Transmission Provider shall respond to a request for concomitant evaluations by setting the appropriate status according to the Business Practice Standards for each of the single requests.

001-106.5
If the Transmission Provider counteroffers the single request paired with the request for temporary or indefinite termination of Network Resource, the status of both requests shall be set to COUNTEROFFER.

Entergy:

001-106.5
If the Transmission Provider counteroffers the single request for NITS or PTP transmission service that is paired with the request for temporary or indefinite termination of Network Resource, the status of both requests shall be set to COUNTEROFFER.

Entergy (add section)

001-106.5.1
The Network Customer shall use the request status of REBID to initiate 



negotiation of the two single requests or engage in continued negotiation of the two single requests with the Transmission Provider.

001-106.5.1
Should the Network Customer elect to initiate or continue the negotiation of the two single requests by changing the status of the two requests to REBID, the Transmission Provider shall respond by changing the status of the request to DECLINED, ACCEPTED, or COUNTEROFFER within the Provider Counter Time Limit, specified in the applicable timing table (i.e., Table 105-A NITS Request Timing Requirements or for PTP, Table 4.2 Request Timing Requirements). The Transmission Provider response time is measured from the most recent REBID time.

Entergy:

001-106.5.1.1
Should the Network Customer elect to initiate or continue the negotiation of the two single requests by changing the status of the two requests to REBID, the Transmission Provider shall respond by changing the status of the request to DECLINED, ACCEPTED, or COUNTEROFFER within the Provider Counter Time Limit, specified in the applicable timing table (i.e., Table 105-A NITS Request Timing Requirements or for PTP, Table 4.2 Request Timing Requirements). The Transmission Provider response time is measured from the most recent REBID time.

001-106.6
Customer Actions

001-106.6.1
After the status of the two single requests has been set to ACCEPTED by the Transmission Provider, the Network Customer may confirm or withdraw the requests by setting the status of each single request to CONFIRMED or WITHDRAWN. 

001-106.6.2
Final Disposition of the Request For Concomitant Evaluation

Southern:

001-106.7
Final Disposition of the Request For Concomitant Evaluation

001-106.6.2.1
Once CONFIRMED, the two single requests will be incorporated into the associated CONFIRMED NITS Application.

Southern:

001-106.7.1
Once CONFIRMED, the two single requests will be incorporated into the associated CONFIRMED NITS Application.

001-106.6.3
If, within the time limits specified in Table 105-A NITS Request Timing Requirements the Network Customer fails to set the status of the two single ACCEPTED requests to CONFIRMED or WITHDRAWN, the Transmission Provider may set the status of the two single requests to RETRACTED. 

Southern:

001-106.7.2
If, within the time limits specified in Table 105-A NITS Request Timing Requirements the Network Customer fails to set the status of the two single ACCEPTED requests to CONFIRMED or WITHDRAWN, the Transmission Provider may set the status of the two single requests to RETRACTED. 

Entergy:

001-106.6.3
If, within the time limits specified in Table 105-A NITS Request Timing Requirements that are applicable to the single request for temporary or indefinite termination of a single Network Resource the Network Customer fails to set the status of the two single ACCEPTED requests to CONFIRMED or WITHDRAWN, the Transmission Provider may set the status of the two single requests to RETRACTED. 

001-106.6.3.1
In response to a state of RETRACTED, the Network Customer may submit a new or revised request for concomitant evaluation consisting of two single requests, which will both be assigned a new unique identifier and a new queue time.   

Southern:

001-106.7.2.1
In response to a state of RETRACTED, the Network Customer may submit a new or revised request for concomitant evaluation consisting of two single requests, which will both be assigned a new unique identifier and a new queue time.   

001-106.7
If one of the single requests is DISPLACED, the treatment of the other single request shall be as specified in the Transmission Provider’s posted business practice.
Southern:

001-106.7.3
If one of the single requests is DISPLACED, the treatment of the other single request shall be as specified in the Transmission Provider’s posted business practice.
Duke Energy:

001-106.7
If one of the single requests is DISPLACED, the treatment of the other single request shall be as specified in the Transmission Provider’s posted business practice.

001-107
NETWORK INTEGRATION TRANSMISSION SERVICE – ANNUAL UPDATES

Southern:

001-107
NITS – ANNUAL UPDATES

001-107.1
Consistent with the Transmission Provider’s tariff requirements and, if applicable, the network operating agreement or Transmission Provider’s posted business practice, the Transmission Customer shall provide annual updates of Network Load and Network Resource forecasts, as specified in Business Practice Standards WEQ-002..

Southern:

001-107.1
Consistent with the Transmission Provider’s tariff requirements and, if applicable, the network operating agreement or Transmission Provider’s posted Business Practice, the Network Customer shall provide annual updates of Network Load and Network Resource forecasts, as specified in Business Practice Standards WEQ-002..

Duke Energy:

001-107.1
Consistent with the Transmission Provider’s tariff requirements and, if applicable, the network operating agreement or Transmission Provider’s posted business practice, the Transmission Customer shall provide annual updates of Network Load and Network Resource forecasts, as specified in Business Practice Standards WEQ-002..

Duke Energy:

001-107.1
Consistent with the Transmission Provider’s tariff requirements and, if applicable, the network operating agreement or Transmission Provider’s posted business practice, the Transmission Customer shall provide annual updates of Network Load and Network Resource forecasts, as specified in Business Practice Standards WEQ-002..

NCEMC:

001-107.1
Consistent with the Transmission Provider’s tariff requirements and, if applicable, the network operating agreement or Transmission Provider’s posted business practice, the Transmission Customer shall provide annual updates of Network Load and Network Resource forecasts, as specified in Business Practice Standards WEQ-002..

BPA:

001-107.1
Consistent with the Transmission Provider’s tariff requirements and, if applicable, the network operating agreement or Transmission Provider’s posted business practice, the Transmission Customer shall provide annual updates of Network Load and Network Resource forecasts, as specified in Business Practice Standards WEQ-002.

001-108
NETWORK INTEGRATION TRANSMISSION SERVICE – RENEWAL OF SERVICE (or ROLLOVER RIGHTS) 

Southern:

001-108
NITS – RENEWAL OF SERVICE (or ROLLOVER RIGHTS) 

OATI:

001-108
NETWORK INTEGRATION TRANSMISSION SERVICE – RENEWAL OF SERVICE (or ROLLOVER RIGHTS) 

Duke Energy:

001-108
NETWORK INTEGRATION TRANSMISSION SERVICE – RENEWAL OF SERVICE (or ROLLOVER RIGHTS) 

Duke Energy:

001-108
NETWORK INTEGRATION TRANSMISSION SERVICE – RENEWAL OF SERVICE (or ROLLOVER RIGHTS) 

001-108.1
Consistent with the Transmission Provider’s tariff requirements and, if applicable, the Transmission Provider’s posted business practice, renewal of service ( rollover rights) may be provided, as specified in Business Practice Standards WEQ-002.

Southern:

001-108.1
Consistent with the Transmission Provider’s tariff requirements and, if applicable, the Transmission Provider’s posted Business Practice, renewal of service ( rollover rights) may be provided, as specified in Business Practice Standards WEQ-002.

NCEMC:

001-108.1
Consistent with the Transmission Provider’s tariff requirements and, if applicable, the Transmission Provider’s posted business practice, renewal of service ( rollover rights) may be provided, as specified in Business Practice Standards WEQ-002.

State Table for NITS Application 

and Request for Modification to Service
(* indicates final state)

	State
	TP Response Options

Southern:

Transmission Provider Response Options
	TC Response Options

Southern:

Transmission/Network Customer Response Options



	PENDING
	
	

	QUEUED
	RECEIVED
	WITHDRAWN*

	RECEIVED
	INVALID*

DEFICIENT

COMPLETED

STUDY

COUNTEROFFER

ACCEPTED

DECLINED
	WITHDRAWN*

	DEFICIENT
	INVALID*

DECLINED*

COMPLETED
	REEVALUATE

WITHDRAWN*

	REEVALUATE
	INVALID*

DEFICIENT

COMPLETED

STUDY

COUNTEROFFER

ACCEPTED

DECLINED*
	WITHDRAWN*

REEVALUATE

	COMPLETED
	REFUSED

DECLINED

STUDY

COUNTEROFFER

ACCEPTED
	WITHDRAWN*

	STUDY
	COUNTEROFFER

ACCEPTED

REFUSED

DECLINED*
	WITHDRAWN*

	COUNTEROFFER
	RETRACTED*
	WITHDRAWN*

CONFIRMED*

REBID

	REBID
	COUNTEROFFER

SUPERSEDED
Entergy:

DISPLACED
ACCEPTED

DECLINED
	

	ACCEPTED
	RETRACTED*

Entergy:

SUPERSEDED

DISPLACED
	WITHDRAWN*

CONFIRMED*

REBID

	CONFIRMED*
	ANNULLED*

(may be used if customer fails to complete the Application process such as failing to participate in the service agreement phase of the Application process)

Southern:

ANNULLED*

(may be used if Transmission/Network Customer fails to complete the NITS Application process such as failing to participate in the service agreement phase of the NITS Application process)


	


Note: The following changes in state definitions will be applicable to WEQ-003 and WEQ-013. May also need to change the title of the State Diagram in WEQ-013 to reflect that it is applicable to Point-to-Point only.
Entergy:

Note: The following changes in state definitions will be applicable to WEQ-003 and WEQ-013. May also need to change the title of the State Diagram in WEQ-013 to reflect that it is applicable to Point-to-Point only
.
QUEUED – initial status assigned by TSIP on receipt of "customer services purchase request".

INVALID – assigned by TSIP or Transmission Provider indicating an invalid field in the request, such as improper POR, POD, source, sink, etc. or that the request does not meet minimum information or data requirements. (Final state). 

RECEIVED – assigned by Seller to acknowledge QUEUED requests and indicate the service request is being evaluated, including for completing the required ancillary services.

STUDY – assigned by Seller to indicate some level of study is required or being performed to evaluate service request.

REFUSED – assigned by Seller to indicate service request has been denied due to lack of availability of transfer capability. (Final state).

COUNTEROFFER – assigned by Seller to indicate that a new OFFER_PRICE and/or CAPACITY_GRANTED over time is being proposed in the negotiation of requested service (e.g., offering of partial service or negotiation of price).

REBID – assigned by Transmission Customer to indicate that a new BID_PRICE and/or CAPACITY_REQUESTED over time is being proposed. 

SUPERSEDED – assigned by Seller when a request which has not yet been confirmed is preempted by another reservation request. (Final state).

ACCEPTED – assigned by Seller to indicate the service request at the designated BID_PRICE and CAPACITY_REQUESTED has been approved/accepted. Depending upon the type of ancillary services required, the Seller may or may not require all ancillary service reservations to be completed before accepting a request.

DECLINED – assigned by the Seller to indicate that the terms and conditions, such as the BID_PRICE, are unacceptable and that negotiations are terminated or that contractual terms have not been met or the Transmission Customer has failed to respond to a deficiency within an established timeframe.  (Final state).

RETRACTED - assigned by Seller when the Transmission Customer fails to confirm or withdraw the request within the required time period. (Final state).

WITHDRAWN – assigned by the Transmission Customer during request evaluation to withdraw the request from any further action. (Final state).

CONFIRMED – assigned by the Transmission Customer in response to the Seller posting "ACCEPTED" status, to confirm service. (Final state, unless overridden by DISPLACED or ANNULLED state).

DISPLACED – assigned by Seller when a "CONFIRMED" reservation from a Transmission Customer is displaced by a higher priority reservation, and the Transmission Customer is not offered or has not exercised right of first refusal (i.e. refused to match terms of new request). (Final state).

BPA:

DISPLACED – assigned by Seller when a "CONFIRMED" reservation from a Transmission Customer is displaced by a higher priority reservation, and the Transmission Customer is not offered or has not exercised right of first refusal (i.e. refused to match terms of new request). (Final state).

ANNULLED – assigned by the Seller when, by mutual agreement with the Transmission Customer, a confirmed reservation is to be voided or assigned unilaterally by the Seller when a confirmed reservation is to be voided. (Final state).

BPA:

ANNULLED – assigned by the Seller when, by mutual agreement with the Transmission Customer, a confirmed reservation is to be voided or assigned unilaterally by the Seller when a confirmed reservation is to be voided. (Final state).

DEFICIENT – assigned by Transmission Provider indicating a correctable deficiency in the NITS  Application or request for modification to service, such as missing, incomplete, or inconsistent information. 

REEVALUATE – assigned by the Eligible Transmission Customer or Network Customer during the NITS Application or request for modification to service process indicating that deficiencies have been corrected and submitting the revised NITS Application or request for modification to service to the Transmission Provider for review.
NCEMC:

REEVALUATE
 – assigned by the Eligible Transmission Customer or Network Customer during the NITS Application or request for modification to service process indicating that deficiencies have been corrected, and the Customer is submitting the revised NITS Application or request for modification to service to the Transmission Provider for review.
COMPLETED – assigned by Transmission Provider to acknowledge receipt of a completed NITS Application or request for modification to service and to indicate the request is being evaluated, including for completing the required ancillary services.
PENDING – assigned by OASIS prior to submission by the Network Customer, to be used while developing a NITS Application or a request for modification to service.

APPENDIX XXX

	  STATE
	PENDING
	QUEUED
	RECEIVED
	DEFICIENT
	REEVALUATE
	COMPLETED
	STUDY
	COUNTER

-OFFER
	REBID
	ACCEPTED
	CONFIRMED

	Initial Application

Southern: Initial NITS Application
	Y
	Y
	Y
	Y
	Y
	Y
	Y
	Y
	N
	Y
	Y

	Designate Network Resource

Southern:  DNR
	Y
	Y
	Y
	Y
	Y
	Y
	Y
	Y
	Y
	Y
	Y

	Temporary Termination  of Network Resource (with redesignation of the same Network Resource)
	Y
	Y
	Y
	Y
	Y
	Y
	N
	N
	N
	Y
	Y

	Designate Network Load
	Y
	Y
	Y
	Y
	Y
	Y
	Y
	Y
	N

Entergy:

Y

Per 

105.5.13.11
	Y
	Y

	Indefinite Termination of Network Resource
	Y
	Y
	Y
	N
	N
	Y
	N
	N
	N
	Y
	Y

	Indefinite Termination Of Network Load
	Y
	Y
	Y
	N
	N


	Y
	N
	N
	N
	Y
	Y

	Request Network Scheduling Rights

Southern: Request NSR
	Y
	Y
	Y
	N
	N
	Y
	Y
	Y
	Y
	Y
	Y

	Temporary Termination Network Scheduling Rights

Southern: Temporary Termination NSR
	Y
	Y
	Y
	N
	N
	Y
	N
	N
	N
	Y
	Y

	Indefinite Termination Network  Scheduling Rights

Southern: Temporary Termination NSR
	Y
	Y
	Y
	N
	N
	Y
	N
	N
	N
	Y
	Y

	Request Secondary Network Resource
	Y
	Y
	Y
	N
	N
	Y
	N
	Y
	Y
	Y
	Y

	Terminate Secondary Network Resource

Southern: Temporary/Indefinite Termination Secondary Network Resource
	Y
	Y
	Y
	N
	N
	Y
	N
	N
	N
	Y
	Y


� Termination of a Network Resource is also known as “Undesignation” of a Network Resource.


� Termination of a Network Resource is also known as “Undesignation” of a Network Resource.





�As defined and based on our understanding of the concept of Network Scheduling Rights, NCEMC is opposed to such an optional mechanism that may reduce consistency and uniformity among Transmission Providers of which these proposed NITS standards are intended to promote. We would suggest including an explanation of why or when a DNR cannot serve the same purpose as a NSR with some TPs, while it may not for others.  We suggest responses to the following scenario to illustrate the difference between DNRs and NSRs: When the NSR is less than the DNR for a couple of Network Resources of a Network Customer, what happens when one Network Resource experiences a forced outage thus triggering the scenario when a Network Customer may need to use more of the second DNR?  If the Network Customer doesn’t have adequate NSRs to use the second DNR, then will they need to submit another NSR for the additional needed capacity/scheduling right during this emergency period in which the DNR should have provided anyway for the Network Customer?


�These portions of the standards are missing, but are included in the Index.


�Where are standards for above sections ?  These are missing as someone has already point out. We realize that the technical requirements may be provided in WEQ-002, 003 or 013 per email on 7/1, but this may have clarified some issues that are commented on later below.


�Midwest ISO (MK): Should this be capitalized or not capitalized? I see in other areas it is capitalized; even when the word is alone. For example, below in 001-101.5 (in the last line).


�Is clarification needed to show that multiple requests may be submitted for one NITS app?


�Is this capability in an ATC item?  Perhaps “functionality” would be a better word.


�Should the word application be capitalized or not capitalized? I see in other areas it is capitalized; even when the word is alone. For example, below in WEQ-001-101.5:


001- 101.5	The Transmission Provider shall provide the capability on OASIS for the Eligible Transmission Customer to enter, save, and revise information prior to the submittal of the NITS Application or modifications of service to the Transmission Provider (i.e., a pre-submittal workspace shall be available for the Eligible Transmission Customer to develop an Application over time). 


�Suggest adding a definition to the glossary that defines the pre-submittal workspace as the capability described in 101.5 above to ensure that users of the standard understand and connect the above capability to the term “pre-submittal workspace” as used below in 101.6.


�Shouldn’t the customer’s agent be permitted to view and edit this info?


�Also see 102.1.1


�Include where appropriate in standard


�Midwest ISO (MK): What is the suggestion in WEQ-001-101.5.3? If it is not mentioned (if the subcommitte delete’s 001-101.5.3) wouldn’t that suffice? Why is this mentioned here? Where it is not mentioned, does that mean 18 CFR 37.6 and 37.7 do apply?


�Do you want to remove any of the information prior to the app being completed so that the customer can modify it? What if it is Deficient and the TP removes all information, then the customer has to start over with no information.


�from


�What is the suggestion in WEQ-001-101.5.3? If it is not mentioned (if the subcommitte delete’s 001-101.5.3) wouldn’t that suffice? Why is this mentioned here? Where it is not mentioned, does that mean 18 CFR 37.6 and 37.7 do apply?





001-101.5.3	Audit and data retention requirements set forth in 18 CFR 37.6 and 37.7 are not applicable to the information in this pre-submittal workspace. 





Doesn’t WEQ-001-101.5.3.2 suffice?


001-101.5.3.2	If submission of the NITS Application has not occurred within 6 months of the time of the last update, the Transmission Provider may remove the NITS Application and all associated information from the pre-submittal workspace, unless the Eligible Transmission Customer has requested a longer retention period.


�This is to avoid indefinite retention of data in the workspace


�This provision appeared to equate the entering of information into the pre-submittal workspace (no matter how incomplete) with the actual submitted NITS application.  The revisions are an attempt to clarify this distinction.





�Strike this phrase.  The TC should be able to remove info from the pre-submittal workspace after submission too.


�Is there a better way to include Designated Agent, where appropriate?  There are serveral places where it could be included.


�Do we need to provide a cross-reference to or otherwise reference the CSV and other upload capabilities provided for in the standards (e.g., WEQ 002-3.3(e) or the new technical requirement for NITS)?  Entergy submits that customers may not immediately correlate the upload functionality and – if we envision the same or similar upload methods 0 it may be helpful to users to indicate exactly what they will need to do to upload.


�While not an OASIS term, OATI’s OASIS permits a query of all pending requests.  This query return all requests which are not yet in a final state.  I suggest another status be designated to minimize confusion upon implementation.  Perhaps PREPARE, or something similar could be the new state title.


�Entergy suggests that this provision could be construed to REQUIRE transmission providers to post a business practice governing non-OASIS aspects of requesting NITS.  Is this the intent?  If not, revision is suggested for clarity.  Additionally, if this is intended to be included in the required business practice on the NITS application (001-102.1.2) then it is suggested that a reference to that provision be included in this provision.


�Does this include a concomitant request which has a pre-confirmed PTP component?


�Organize a bit better.  12.1 off-system, 12.2 off-system, 12.3 on-system, 12.4 on-system, 12.5 off-system, 12.6 on-system


�What does Designation and Re-designation have to do with Network Resource Information Requirements provided?  This is not apparent.


�Where is this name to be  used?


�How will the TP know which name is being used?  Is there a flag to prevent the confirmation until the TSIN-registered name is in place?  Also 001-101.12.3.


�Will we require the Cstomer to register and use source names in TSIN?


�Apply to entire standard where appropriate.


�The TP wants certainity of which DNR they are evaluating since a name change could include a different location. 


�IS BAA defined? YES.


�What is BAA, Balancing Authority? If so, it should be just BA.  I would suggest spelling it out completely for first time.


�Isn’t this redundant for on-system generation?


�Will we require the Cstomer to register and use source names in TSIN?


�The TP wants certainity of which DNR they are evaluating since a name change could include a different location. 


�Same requirement as 001-12.2.  Why differentiate between on-system and off-system?


�What is BAA, Balancing Authority? If so, it should be just BA.  I would suggest spelling it out completely for first time.


�Should this mirror language in  001-101-12.2?  In some instances purchased output from specific generators can be identified by county and state.


�Should this mirror language in  001-101-12.2?  In some instances purchased output from specific generators can be identified by county and state.


�What is meant by “Prec-confirmation has no effect on modification of network service”?


�Given this is about Network Resource Requirements, does this mean that Pre-confirmation is NOT a requirement for DNRs?  Most all TPs that we do business with TODAY do have this as a requirement.


�My assumption is we will want to integrate all existing into the new process but not require resubmission of all infomration elextronically. We should discuss how we will manage entering pertinent infomration from the existing NITS Applications and DNR’s (TSR’s, NSR’s)


�It is unfortunate that the Subcommittee could not reach any consensus on what can constitute a minimum set of required information necessary to garner a queue position relative to other requests for service.  This exposes Customers to a wide-range of potential TP specific BPs which might range from simply supplying an intent for service over a designated timeframe to a named load to requirements for full submission of all required information regarding DNRs, DNLs, Load and Resource forecasts and all supporting information at the time of the initial request.  It is recommended that the Subcommittee revisit the issue to establish such a minimum set of information to prevent such wide disparity in potential requirements between even adjacent TPs under the same regional planning regimes.  The following is proposed as a potential list for such a requirement:


Identification of the NITS Customer with attestation as to eligibility to be a NITS Customer


Initial term of service


Declaration of the nature of the load to be served under the NITS agreement in sufficient detail (this is probably a bit too subjective but something to the effect of identifying the load and its magnitude)


�Where are these?  


�See 101.5.1.for including Designated Agent  or use deleted text below, consistently.


�Reference the NAESB status for a submitted Application.  (e.g., NITS application with PENDING status)


�According to this standard, one TP may require everything in the Pro Forma OATT section 29.2(i) through 29.2(ix) in order to obtain a queue time for NITS Application while another might only require parts (i) and (ii) of 29.2 to get such a queue time. To avoid this inconsistency in implementation of this standard, NCEMC would suggest that the NAESB OS at least consider proposing a subset of requirements for ALL TPs to take into consideration in order to establish a queue time for the Application.


�The first substantive comment deals with the establishment of queue priority for NITS applications.  Section 001-102.1.2 states that “The Transmission Provider shall specify, through the posting of a business practice, the minimum information required for a NITS Application.”  





We believe that it is inappropriate that the amount of information required to file an application and thereby establish queue position be a matter for individual TP’s to decide.  A complete NITS application is a bulky document with many parts.  It seems inappropriate therefore that the standard allows for some TPs to require the entire completed application before a customer can establish a queue position while others may require only a name and address to identify the applicant.  We recognize a concern with the possibility that a customer could file a severely deficient application and block the queue.  However, we believe this risk is mitigated by the requirement that a customer rectify deficiencies in an application “in a timely manner” at the risk of losing their queue position.  Therefore a customer that is still missing much of the information required to complete a full application would be unlikely to meet the timeliness requirement and is therefore unlikely to retain their queue position anyway.  Any concern in this regard is therefore likely to be self-correcting.  We believe therefore that an initial filing should require a very minimal amount of information.  Most importantly however, the specification of exactly what information is required should be part of this standard.  


�Reference the NAESB status for a submitted Application. Obviously, the status must be changed from PENDING by the customer because the status of PENDING indicates that it is still be worked on prior to submission…see 001-101.7.


�Are all of the components given this same queue time and STATUS?  YES.


�Are all of the components given this same status?  YES


�Business practices and OATT requirements could apply if the BP is more detailed or different.


�Is this a TP option…#1 or #2, not both?  What is the purpose of requiring TP to establish a minimum requirement in 001-102.1.2 but ignor it in this practice?


�According to this standard, one TP may require everything in the Pro Forma OATT section 29.2(i) through 29.2(ix) in order to obtain a queue time for NITS Application while another might only require parts (i) and (ii) of 29.2 to get such a queue time. To avoid this inconsistency in implementation of this standard, NCEMC would suggest that the NAESB OS at least consider proposing a subset of requirements for ALL TPs to take into consideration in order to establish a queue time for the Application.


�Our second substantive comment relates to Section 102.2.2.  The early processing of the application provides that the TP will make an assessment of the information received to determine if it is INVALID in which case queue position is lost or merely DEFICIENT in which case there is an opportunity to rectify the deficiencies without the loss of the queue position.  Yet again, in Section 102.4.4.3, if a Customer’s attempt to rectify a deficiency is “deemed by the TP to be a significant change to the overall NITS Application” the application can be DECLINED and queue position forfeited.    





In many circumstances, loss of queue position can represent a substantial penalty to a Customer.  Therefore, in the two circumstances cited above which also re-occur elsewhere in other sections of the standard, there needs to be a specific “bright line” test included in the standard such that actions that can lead to loss of queue position are well known and consistently applied, not just by individual TPs but across Transmission Provider service territories.   








OASIS Subcommittee Response, 08/24/2010:





In response), FERC does not require standardization of validity checks.  Currently, the bright line test for invalid is specified in 102.3.2 and also the bright line test for determining “invalid” is posted in the TP’s business practice and tariff.  For deficient, the bright line test is specified in 102.4.6 - 7.








�What is meant by “portion”?  Is this the same “portion” as mentioned in 001-101.5.2?  Is it one of the “multiple requests” discussed in 001-101.2?  This term is use primarily in the “Application” business practices (additional uses are found elsewhere in 001-102.4, 001-103.2, 001-103.5, 001-103.6, 001-103.8 and 103.9).  While the subcommittee may understand this concept, it is not clear to others…particularly in light of the word’s use in 001-101.5.2.  Apparently, based on subsequent uses of the term, each portion has a status field. Under some circumstances these status values, is aligned with the application’s status but, at other times they are handled independent of the Application’s status field. Should these “portions” with status fields also be given “unique identifiers” as referenced in 001-102.1.1.1 for the application.  Will they have the same unique identifier as the application or will they not have unique identifiers.  The current business practices make no provision for assignment of unique identifiers to the “portions” nor is there any assignment of unique identifiers in 001-105.


�On first blush, this doesn’t make sense. If any part is invalid then shouldn’t the  whole application be found INVALID?  If the application is found to be INVALID, why proceed to evaluate the other “portions”?


�Should the status of all of the portions be set to DEFICIENT as well?  The atatus of the portions should to be addressed…either they are changed or they are not.  Will it be permissible to have a DEFICIENT application but have COMPLETED components.





08/30/10 – ok.


�This could be formal communications if the TP states so in the business practices such as an OASIS posting with a timing requirement.


�Should the status of all of the portions be set to DEFICIENT as well?  The atatus of the portions needs to be addressed…either they are changed or they are not.


�Should the status of all of the portions be set to DECLINED as well?  The status of the portions needs to be addressed…either they are changed or they are not.





08/30/10 – Each portion will have its own status.


�What constitutes “significant change”? In Our Business Practice we should give an example of what may be considered  significant. 001-105.1.5 is a list of what is considered modifications of service, maybe we include that list in our Business Practice. I'm not sure how we want to do this.  Perhaps that the list of modifications in 105.1.5 should almost come before this section. 





08/30/10 – This will be decided by the TP. 


�If an Eligible Transmission Customer is responding to what it believes to be sufficient response to a deficiency in its interpretation, what would constitute “a significant change”?  Is this according to the discretion of each TP?  If so, what is the criteria?  If there are none (I don’t believe there are any for NITS Applications), then the NAESB OS should propose such criteria for use by TP. �


08/30/10 – This will be decided by the TP.


�Also, there is no mention of a change in status by the TP to DECLINED in 001-102.2.2.  Shouldn’t DECLINED be listed in 102.2.2 given it is provided here?


�Should the status of any or all of the portions be set to DECLINED as well?  The atatus of the portions needs to be addressed…either they are changed or they are not.  





�Same question as above for DECLINED.


�May the COMPLETED portions (referenced by “unique identifier” of the DECLINED application) be directly incorporated into the new or revised NITS application OR will the customer have to create new portions which may be identical to the COMPLETED portions of the DECLINED application?


�In the same vein as the above comment to a clear minimum standard for the initial application, it is unfortunate that a clear minimum set of required information could not be established to constitute a completed application.  The following is proposed as a potential list for such a requirement:


Load forecast information for a minimum of 10-years


Resource forecast information for a minimum of 10-years


DNLs for a minimum of the first 1 (n?) year(s) of service


DNRs in sufficient amounts to meet all DNLs for a minimum of the first 1 (n?) year(s) of service


All supporting information required for above (also a bit subjective, but TP must have ability to require information in sufficient detail to move application forward)





08/30/10 – addressed previously.


�The portions do not have a queue time until the overall NITS application is COMPLETED?  They should be given the queue time at the time of submission.  Otherwise, other customers will not know at the time of their submission (e.g., PTP request) about portions of a NITS application  which will be evaluated prior to their request.  It will look like someone jumped the queue.


�Not sure what might be criteria for a such a subtemplate. Please explain at Carmel meeting.  It seems that 001-5 sufficiently addresses Ancillary Services and given that, the TP should simply indicate in its NITS BP what it MANDATORY versus REQUIRED versus OPTIONAL for the NITS Application.  If this is the criteria requiring a NITS subtemplate, then our answer would be YES.


�Since Midwest ISO provides an Ancillary Service Market, a NITS subtemplate would not apply.


�Not required for BC Hydro.


�BPA can support the inclusion of an Ancillary Services subtemplate for required Ancillary Services.  If the Customer elects to self supply, they have to follow the Transmission  Provider business practice for eligibility and other requriements.  For all Ancillary Services, if the TP has a business practice the Customer should refer to that business practice to determine eligibility.


�It is Entergy’s recommendation that ancillary service linkage and requests similar to those for PTP should be incorporated into the standard.





�NCEMC’s preference would be that NAESB standards be silent on this matter.  


�Midwest ISO believes insufficient credit or deposit would fall under an “incomplete application” as reason for denial.


�Prefer NAESB standards be silent on this.


�Yes, BPA agrees that the standard should be silent on reason for denial.


�It is suggested that the standards be silent on the reasons for denial.  There are currently no required reasons in the PTP standards just that a reason must be provided.  Each TP should be allowed to formulate their own reasons.


�Section 103.2.2 indicates that TPs “may” update the portions of applications “as appropriate”.  However Section 103.5.2 and elsewhere indicate that TPs “shall” update the status of portions of an application.  These appear to be contradictory.  Is there a requirement for the status of portions of the application to be routinely updated?





08/30/2010 – deleted provision.


�Should the status of all of the portions be set to DECLINED as well?  The status of the portions needs to be addressed…either they are changed or they are not.


�There should never be a refusal of a NITS application that is complete “due to insufficient transmission capacity” and thus Duke is unclear why this status exists.  There are circumstances where an individual DNR request could be refused, i.e., where the service is requested to start (and/or conclude) before needed upgrades could be built.  Duke has never heard of anyone asking to become a network customer for only a year or two, such that there would not be time to build any needed upgrades (and given that 99% of new customers are just pre-Order 888 grandfathered customers switching to OATT service or existing customers breaking away from a joint agency, upgrades are rarely required for new network customers.)  Presumably, if a network load asked for service from the “wrong” TP (i.e., a load asked for network service from a TP in another Interconnection), it could be rejected; presumably such a grossly erroneous application would never even get to the completed stage.  





08/30/10 - Although limited, there may be circumstances where a TP needs to refuse an application.


�Should the TP be permitted to terminate a NITS Application because there  is not sufficient transfer capability for one of the DNRs?  That seems to be an extreme position, particularly when there isn’t a required threshold for DNR capacity.  (e.g., customer has 2000 MW load, and requests six 500 MW of DNRs….TP cannot accommodate one of the DNRs….so the whole package is tanked!)


�NCEMC questions whether the TP should completely refuse the NITS Application in its entirety due to “lack of ATC”, or due to posted ATC that has not been updated for quite some time (more than 6-12 months) and is based on outdated models. It becomes apparent from talking with the TP that it is based on an outdated model which when updated will indicate that there is sufficient ATC. The NITS Customer has to serve its load.  There should be an alternative approach rather than to resubmit the NITS Application for a later date only to be refused again due to “lack of ATC”.  There should be a mutual responsibility to enable the NITS customer wanting NITS service to serve its NITS load.  Is PTP its only option when serving load inside a BA when there is NO ATC?


�NCEMC questions whether the TP should completely refuse the NITS Application in its entirety due to “lack of ATC”, or due to posted ATC that has not been updated for quite some time (more than 6-12 months) and is based on outdated models. It becomes apparent from talking with the TP that it is based on an outdated model which when updated will indicate that there is sufficient ATC. The NITS Customer has to serve its load.  There should be an alternative approach rather than to resubmit the NITS Application for a later date only to be refused again due to “lack of ATC”.  There should be a mutual responsibility to enable the NITS customer wanting NITS service to serve its NITS load.  Is PTP its only option when serving load inside a BA when there is NO ATC?


�COUNTEROFFER is used to counter price or capacity granted only.  Unless the definition is to be modified, the proposed revisions are suggested.  Entergy also notes (for discussion purposes only) that the standards require counteroffer here, but allow it (subject to TP discretion) elsewhere.  


�This could be formal communications if the TP states so in the business practices such as an OASIS posting with a timing requirement.


�Will there be a 2-step pre-confirmation process similar to PTP pre-confimation process (WEQ-2) if the customer pre-confirms?


�This could be formal communications if the TP states so in the business practices such as an OASIS posting with a timing requirement.


�This isn’t worded very clearly. What queue time will be given to the “deemed modification”?  Is it the same queue time of the application’s queue time or something different?


�Whose Business Practice Standards? NAESB, TP, NERC Should this be stated?


�103.7.1 addresses treatment of requests while in either COMPLETED or STUDY status.  As a sub-practice, why are requests that are submitted when the status is CONFIRMED included here?


�CONFIRMED is a final state.  ? This conflicts  with 001-103.7.1.2.  Also, 103.7.1.1 addresses treatment of requests while in either COMPLETED or STUDY status, not NITS applications in a final state.


�Where is this applicable…it is not a subset of 103.7.1, so it isn’t bounded by requests that are submitted after the NITS application is deemed COMPLETED but before being CONFIRMED?  Does this apply to subsequent  modifications after confirmation?  Does it apply to changes prior to the NITS application being deemed COMPLETED?


�Section 103.7.2 deals with requested updates to NITS Applications.  Sections 103.7.2.1, .2 and .3 appear to provide the full range of options from DENY to ACCEPT to hold and process upon completion of the processing of the application, presumably with queue time established by the time that the update is requested.  There does not appear to be any established criteria for determining the appropriate response.


�This could be formal communications if the TP states so in the business practices such as an OASIS posting with a timing requirement.


�What is the approprite final state?


�Don’t understand how this is done.  Will the TC be permitted to modify fields in a portion?  If so, does the TP reject the change and continue to evaluate?  Then, after the final state is set to CONFIRMED, will the TP have the responsibility to begin evaluation on the requested change…will the TP create the request based on the fields which were rejected?  What will the queue time be for this change in the portion?  This should not be an option.


�No REBID.  Would REBID be needed if another round of studies were needed or would that be dealt with in the informal communications?  It isn’t clear what is expected.


�This provision may need additional clarification as it may confuse some TCs relative to when they would have some portions unevaluated, but the overall status as ACCEPTED or COUNTEROFFER.  If the instance is limited to that described in 001.103.7 then such should be indicated for clarity.


�What language in blue?


�And no portion can be in a CONNFIRMED state until the NITS application is in a CONFIRMED state.  This is communicated in 001-103.8.1.3 but needs to be moved so that it is presented prior to this business practice.





08/30/2010 – Final state refers to denials and INVALID and not CONFIRMED.


�The intent here is to use language similar to  DENIED language in 001-106.3.


�This should be moved to 103.8.1.1 to address the REBID question before it is asked.


�Would this be a similar Business Practice as 001-103.7.12. Request for modification  examples are found in 105.1.5. Reference should be made defining a request for modifications.


�Or permit the end date to be modified if the NITS Application was termiated after service was initiated.  In such an instance, the status of the portions need to be modified as well.  OR some other mechanism for modification which is enacted for all portions.


�State where in the BP this can be found.


�Or the end date is modified if the NITS Application was termiated after service was initiated.  In such an instance, the status of the portions need to be modified as well.


�For discussion purposes only, should the setting of the overall application to RETRACTED set all portions to RETRACTED?


�Seriously question whether changes in the nature of Generation backing a Resource (which is only reason generation information is supplied) is considered an administrative change.  If the Generation is being decommissioned, would think that constitutes a fairly significant modification of any DNR that may be backed by that Generation.





08/31/10 – The example given would require a modification to service and would not be an administrative change.


�In Section 104 dealing with Administrative Changes to service, the flow of the document would seem to be smoother if the information in Section 104.5, providing a definition of the administrative changes, was placed as Section 104.1 or 104.2.





08/31/10 – The information referred to in 104.5 was revised.


�Is there a need to permit the customer to identify when the change becomes effective?  What are “administrative changes”…are they ever material changes?


�NCEMC doesn’t understand the basis for administrative changes having to be submitted “pre-confirmed” given they are not competing for a queue position. This should NOT be a requirement in our opinion.   





08/31/10 – This is to streamline the process for the TC.  We agree that there is no competition for queue position on a request for administrative change.  The timing of the effective date for the change is being addressed in a separate action documented in the NITS Master.


�Calendar or Business days?


�Is that 7 calendar days or business days? Is 7days enough time to do all the contract and credit checks?


�Are these the only things that can be considered administrative changes?  If so, then it might be good to start the sentence with the word “Only”.


�Is change of DUNs number or change in TSIN acronym an administrative change?


�How?  What are agents?  Agents have not been discussed elsewhere…only in BP 001-101.19, which is missing.


Could be interpeted to have a b�ig scope.  Does it include switching generation resources.  Is this addressed anywhere?


�What is this?


�It is suggested that these portions are not covered explicitly within other sections of the standard and should be covered in greater detail especially with respect to declaration of agents and agent changes. 


�See comment on 001-103.7.2.2.





08/31/10 – Addressed in revisions to 001-103.7.2.2.


�Given it is allowed to do Temporary Termination of a Network Resource for less than Daily (Hourly) for a future period , what if something occurs before the actual start of the Temporary Termination and the Network Customer needs to re-designate that Network Resource for that period less than Daily?  Given DNRs are not allowed for less than Daily intervals, is the Network Customer then only allowed to re-designate that same DNR, or allowed to annul the Temporary Termination for less than Daily?


�Given it is allowed to do Temporary Termination of a Network Resource for less than Daily (Hourly) for a future period , what if something occurs before the actual start of the Temporary Termination and the Network Customer needs to re-designate that Network Resource for that period less than Daily?  Given DNRs are not allowed for less than Daily intervals, is the Network Customer then only allowed to re-designate that same DNR, or allowed to annul the Temporary Termination for less than Daily?


�It appears that all classes must be offered in specific Service Increments.  It would make sense for the PTP definitions for service increments to be used for these as well.


�In instances where time is needed to resolve a DEFICIENT request, there may not be enough time allocated to the TP to perform an evaluation.


�Where are the scheduling deadlines for network service.  Duke could not find them in the Pro Forma OATT or these business practices.


�This footnote suggests that each piece of a concomitant requests is addressed individually.  It doesn’t make sense to accept one component of a concomitant request before knowing if the other component is acceptable.  


�it appears in the standards that the table is intended to apply to the temporary and indefnite termination of Network Scheduling Rights, but such was not indicated in the table.  Entergy suggests the proposed revisions to address this discrepancy.


�Agree that the designation of a new network resource is basically a mirror of the initial application process in that it may be deemed deficient, etc.


There should be some obligation established in this section that IF the customer requires additional transmission service in excess of their currently held ‘NSRs’ that a request for such additional transmission service must be submitted ‘concomittant’ with the request for the new DNR.  It seems inappropriate that the Customer should be allowed to request additional transmission service outside of a request for an additional DNR.  In Subcommittee discussions, there did appear to be practices that a new DNR could be established that leveraged existing transmission capacity.  But is there really a valid case for allowing the acceptance of a DNR with no assessment of its deliverability to the NITS load?  It would seem that granting of DNR status should come with some commitment and assurance of transmission for delivery – maybe pre-existing but at least some form of transmission capacity commitment and payment.


105.2.13.3 and 105.2.13.3.1 seem out of place in this section.  At this point the standard is discussing Final Disposition yet these deal with negotiation and capacity set by TP at time of acceptance.  105.2.13.3 is very similar to 105.2.12.1.2 and not sure of the reason for duplication.





9/8/10 response to bullet two:  Each TP using NSR will determine whether a one to one relationship with DNR is required.  Will have discussion in 001-105.7.


In response to the third bullet we will have the discussion when we get to the correct section


�Section 001-105.2.1.1 dealing with designation of new DNRs is analogous to 101.5 regarding the provision of a “workspace” on the OASIS to create an application.  However, there is no section analogous to 101.6 allowing for the upload functionality.  A similar comment could be made elsewhere in the standard.





09/08/10 response: in the definition of Pre-submittal Workspace we allowed the use for modification of service.


�This is an optional posting of a busines practice.  If the TP does not post a business practice, what is required?  May a TP post that there is no minimum requirement and, thereby forego the posting of anything of this nature on OASIS?  Does the providers business practice superscede the tariff?


�NCEMC suggests that the “may” should be replaced with “shall” to promote consistency and uniformity. As it stands, the minimum information for one TP might be everything within OATT section 29.2(v), 29.2(viii) and 29.2(ix), while another TP may require only a subset of these items at the time of the request submittal for a new DNR. A Network Customer MUST know what is the minimum information for queuing a DNR to serve load and often times all of the items in the OATT sections above may not be available until later.


�Entergy suggests that this provision could be construed to REQUIRE transmission providers to post a business practice.  If not, revision is suggested for clarity.  


�Does submitting the request involve a change in status.  Customer set the status in 105.2.1.1 so it would make sense that a status change is needed here.





090810 response yes the status will change from PRESUBMITTED to QUEUED as per 105-2.1.3.1


�Either one?


�Same as 105.2.1.2


�Shouldn’t 001-105.2.1.3.1 be modified to also assign the unique identifier?  Similar coment for DNLs & NSRs.


�Seems a little confusing. If it is not Invalid then isn’t it valid?


�Is there something wrong with saying “deemed insufficient”  the play on words is confusing.


�What does this mean?  FERC made it pretty clear in Order 890 that an attestation was required…this looks like it is optional.  NAESB should build this into a standard template rather than letting each TP handle it differently.  It would be easier for customers if it were consistent.


�Per Order 890B, P182, the FERC indicated that the attestation can be submitted at the time a resource designation is CONFIRMED, rather than REQUESTED. NCEMC suggests that above be modified to indicate that the request may include the attestation when requested but must not required it until just before confirmation.  In addition, we would like to clarify that an Attestation comment within the “comment” field of the DNR is not needed in addition to the signed Attestation document that has to be submitted separately.  Some TPs require that TODAY.


�Section 001-105.2.4.2 states that “a request for designation of a new Network Resource shall be accompanied by an attestation in accordance with the Transmission Provider’s tariff”.  Is the attestation not specified in the FERC Order?  If so, what is being specified in the TP’s tariff?  


�What does this mean?  FERC made it pretty clear in Order 890 that an attestation was required…this looks like it is optional.  NAESB should build this into a standard template rather than letting each TP handle it differently.  It would be easier for customers if it were consistent.


Commission Order 890.  Paragraph 1531. In response to Entergy’s request, we agree that attestations will not be required to be submitted until the service request is confirmed.  However, if the request is preconfirmed we agree that the attestation must be provided at the time the request is submitted.





�What does this mean?  FERC made it pretty clear in Order 890 that an attestation was required…this looks like it is optional.  NAESB should build this into a standard template rather than letting each TP handle it differently.  It would be easier for customers if it were consistent.


�For a DNR, what would be considered a “significant change” versus a “reasonable change” that might be allowed?  ��Also, why isn’t DECLINED included in 001-105.2.2.2 above?


�What is a timely manner?


�“according  to the following” might read better.


�“the time and date the request” 


�What do you study for a new DNR/what determins a counteroffer of a DNR?  The transmission request would determin this wouldn’t it?   Are studies necessary if the TP opts to utilize NSRs?  This is somewhat confusing if the optional template is used.


�So,  a  DNR is always a  request for transfer capability!


�Is the study process different than for PTP?  Doesn’t the tariff require offers of partial service even before performing studies if partial service is known to be available?  How is the scenario handled if partial service is accepted and a study agreement is executed?  As written, this section  appears that when a customer refuses partial service it terminates any studies that may be needed to ascertain if full capacity can be supported with upgrades?


�Is this needed?


�Is this higher than the counteroffer or higher than the original request? Wouldn’t the customer who was offered partial service and who wishes to have studies done to find out what upgrades are needed to accommodate the full requested capacity counteroffer at full requested capacity?  How can a customer lock in the counteroffered capacity and also have the TP proceed with the studies?


�Seems to flow a little better this way.


�OATI 105.2.13.3 and 105.2.13.3.1 seem out of place in this section.  At this point the standard is discussing Final Disposition yet these deal with negotiation and capacity set by TP at time of acceptance.  105.2.13.3 is very similar to 105.2.12.1.2 and not sure of the reason for duplication.





09/09/10 response:  The sections have been moved to the top of section 13.  Due to the titling of section 12 as customer actions, the subcommittee preferred placement in section 13.





�Delete these …they were covered in 105.2.12


�Given the request for a NEW DNR is at the CONFIRMED state at this point, this standard as well as 001-105.2.13.3.1 seems to be redundant given the standards 001-105.2.12.1.2 and 001-105.2.12.1.3.  Please explain the differences.


�Delete these …they were covered in 105.2.12


�Disagree with the complexity that is proposed for this action.





09/09/10 – subcommittee consensus is to keep the process consistent wrt invalid, deficient, completed.





Suggest that temporary termination MUST be accompanied by an attestation as to the eligibility of the DNR to be a DNR under the exact same terms and conditions as originally approved as a DNR to qualify as a VALID temporary termination.





09/09/10 – an attestation is required at 001-105.3.2.5.2.1





If the Customer cannot attest that the nature of the resource is identical in nature to that as currently designated, the temporary undesignation is considered INVALID and the Customer must submit an Indefinite Undesignation and a request to Designate a New Network Resource and go through that process.  


09/09/10 - an attestation is required at 001-105.3.2.5.2.1.


The Temporary Termination should be a pure administrative process that releases firm NITS capacity for the period of undesignation only and it is reinstated without further action by Customer or Provider.  If for some reason the nature of the resource changes over the term of the undesignation – like new PPA contract executed with different Ts&Cs it should constitute a material change to the nature of the DNR and be evaluated as a NEW DNR.  Assumption of these temporary terminations is they are relatively short-term in nature in support of 3rd party or off-system sales but the underlying fundamental nature of the resource is unchanged at the end of the temporary termination. 


09/09/10 - The redesignation and attestion requirements should address this concern.


Per above, there should only be an INVALID determination if Customer fails to supply the attestation for redesignation, and an acceptance/confirmation process – no deficiencies, completed application etc.  It is hard to envision a reason for TP denial of Temporary Termination that includes appropriate re-attestation.


09/09/10 - The redesignation and attestion requirements should address this concern.


Under Final Disposition of the Temporary Termination, however, should have some obligation placed on the TP that IF such temporary termination results in a situation that the aggregate transmission capacity held by the Customer (in NSRs) exceeds that required to reliably delivery the remaining aggregate DNRs to DNLs, the TP shall release such capacity to ATC for sale/purchase by any other eligible customer.


09/09/10 - Release of capacity is addressed in the postback methodology (WEQ 001-18) and the ATC IDs as a part of MOD-28, 29, 30 compliance.


In summary, believe the process should be very minimal: a request for temporary termination, determination that it is valid, and finally accepted/confirmed termination.


�Would prefer that this  be called temporary undesignation and change the footnote. Could the subcommittee provide a defintion of temporary termination or temporary undesignation…isn’t it undesignation with subsequent redesignation?


�Not sure what is meant by “valid and complete”.  Is that everything as required in OATT 30.3(i) through 30.3(v)?


�The end date of the request to temporarily terminate would automatically indiate the redesignation. If at some point during the temporary undesignation  period the customer wished tor redesigante, then they would create a Designation of  New Network Resource and the process begins again. 


�Please consider adding “, or a portion of the same resource” behind “the same resource” to clarify that it must be the same resource but not the same amount as original DNR per 30.3 of the OATT.


�Does OASIS set the status to QUEUED or does the Customer?  


�This standard seems to imply that there might be a posted BP indicating minimum information to submit a request for Temporary Termination of a Network Resource. Should there be a standard indicating that a TP may post one similar to 001-105.2.1.2?


�Why the option to revise? INVALID is a Final State, is it not?


�“deemed insufficient”  or “deemed deficient”


�The attestation should be part of the minimum requirements. The end date of the request to temporarily terminate would automatically indiate the redesignation. If at some point during the temporary undesignation  period the customer wished tor redesigante, then they would create a Designation of  New Network Resource and the process begins again to the end date of the temprary termination


�Free form?  Everyone to do it differently?


�Is this a template data element?  Is it appropriate to have such an item in WEQ 001?


�Use “shall also be” instead of “is” and also…is STOP_TIME have both date and time elements?   Should this be STOP_DATE and STOP_TIME?





09/09/10 – STOP_TIME data element already includes date and time.


�This step makes sense but it does not flow with 105.3.5.2.2 or 105.3.1.2





09/09/10 – modified flow and included references.


�This could be formal communications if the TP states so in the business practices such as an OASIS posting with a timing requirement.


�What is the criteria for a “significant change” for a request for temporary termination of a Network Resource?





09/09/10 – already discussed in 102.4.4.3, resulted in maintenance of “significant change”.��Shouldn’t DECLINED be included in the status within 105.3.3.2?





09/09/10 – subcommittee chose not to include subsequent status changes after deficient.  They are included in the status table.


�Why does a temporary termination of a Network Resource trigger the need to update the Confirmed NITS Application?  Please explain why such a step is needed.  If it is needed, can’t be included in a next periodic update of the NITS Agreement/Application rather than such a minor event as this?


�Believe this process should be very minimal: a request for indefinite termination, determination that it is valid, and finally accepted/confirmed termination.  Do not believe all the states of COMPLETED, etc. need to be included.


Under Final Disposition of the Indefinite Termination, however, should have some obligation placed on the TP that IF such indefinite termination results in a situation that the aggregate transmission capacity held by the Customer (in NSRs) exceeds that required to reliably deliver the remaining aggregate DNRs to DNLs, the TP shall release such capacity to ATC for sale/purchase by any other eligible customer.


�The standards do not explain the difference between temporary termination and indefinite termination.  These are not defined terms, so it would seem that the reader could go to any dictionary to divine the difference.  More information is needed.


�If there is anything required other than the date and time that the termination is to become effective, and the identification and capacity of the resource(s) or portions thereof to be indefinitely terminated, then there needs to be a standard indicating that. If not, then I would suggest referencing the OATT section 30.3 within for clarification.


�Is this the same as Indefinite?


�There are no time limits in the table for confirmation of indefinite termination.  Entergy suggests that some time limits for retraction are necessary.  If none can be agreed upon, the above revision is suggested.


�Agree that the designation of a new network load is basically a mirror of the initial application process in that it may be deemed deficient, etc.


105.5.14.3 and 105.5.14.3.1 seem out of place in this section.  At this point the standard is discussing Final Disposition yet these deal with negotiation and capacity set by TP at time of acceptance.  105.5.14.3 is very similar to 105.5.13.1.2 and not sure of the reason for duplication.





9/29/10:  Please see rewritten text for this section.


�Isn’t this is for both new load and addition to existing load?  Why include “new” in the title?





9/29/2010:  Resolved 


�The NAESB definition of New Network Load is as follows:  “A request for designation of new Network Load may be a request for either (1) a load addition to a Network Customer’s existing Network Load(s) exclusive of projected load growth or (2) a load addition that is separate from a Network Customer’s existing Network Load(s).”  


In Section 29.2, the OATT states that applicants must provide:  “A description of the Network Load at each delivery point.  This description should separately identify and provide the Eligible Customer’s best estimate of the total loads to be served at each transmission voltage level, and the loads to be served from each Transmission Provider substation at the same transmission voltage level.  The description should include a ten (10) year forecast of summer and winter load and resource requirements beginning with the first year after the service is scheduled to commence.”  Also, in Section 31.2, the OATT to assumes that a New Network Load is a load that needs to be interconnected, which likewise implies that the concept of New Network Load really means a new delivery point.  Duke believes that the standards associated with New Network Load add onerous processes with regard to a process that should consist of nothing more than adding a new delivery point to the list of existing delivery points for a network customer.


The flaws in the NAESB approach are multiple, but are readily fixed.


Duke is not certain what (2) even means, unless it means a new delivery point is being requested; if so, it should simply say that.  If (2) means something different from a new delivery point, it must be clarified


Any designation of a New Network Load, that is, a new delivery point, should be documented via the annual load forecast.  The distribution and transmission engineers who are responsible for reviewing operations, reliability, engaging in long-term planning and thus ultimately make the initial decisions as to when and where delivery points are needed, do not use OASIS and do not have OASIS training.  That process involves multiple meetings, communications, site visits, etc. that should not and cannot “occur” on OASIS.  Once the relevant decision is made by the relevant experts to add a delivery point, the next annual forecast will include a designation of the New Network Load, i.e., the new delivery point.  That annual forecast in effect serves to modify the Application, as Section 31.2 requires, and designates the New Network Load.  


Presumably, 001-101.14 will require that a list of delivery points be provided with an Application, as that is required by OATT Section 29.2.  (For existing network customers, that field will have to populated in the transition period.)  While the list of delivery points provided under Section 29.2 will be coupled with a non-public 10-year load forecast for such points, as discussed below, 001-101.14 should require that all delivery points be extracted from the non-public information and posted publicly (whether populated by the customer or the TP).  Due to CEEI concerns, such points may merely be listed as Delivery Point 1, Delivery Point 2, etc., they will not necessarily be geographic locations.  


Duke does not believe that subsection (1) of the definition of New Network Load reflects the OATT concept of New Network Load.  Duke likewise cannot discern a situation where (1) would arise.  Given the annual frequency of load growth forecasts and the ability to update more often, there should not be unforecast load growth.  I.e., an economic boom would never be so sudden that significant new load, i.e., load that would require upgrades, would appear in mere months.  Other forms of load “growth” (really load transfers) also are readily forecast.  Annexations typically take years to complete and would always make their way into load forecasts or the acquiring entities.  Municipalization likewise takes years to complete and would be planned for in advance.  When a joint agency adds members, again, it normally does so well in advance, as the joint agency must acquire supplies to serve its new load.  Duke has heard that in the West, some Network Customers can actually switch Transmission Providers, but again, such a change would likely take significant time to implement.  Thus, the concept reflected in (1) is unnecessary.  A policy where any unforecasted load growth would trigger the possibility of a study and a requirement to designate the new load on OASIS could lead to over-inflated load growth forecasts.  


In sum, NAESB seems to be adding a standard that will create substantial confusion, as it is incomprehensible when a designation is required.  Duke objects to a requirement that any request for a new delivery point must be made on OASIS, as the persons making such requests are not conversant in OASIS.  Rather, once a determination to add a point has been made, the new delivery point should be “designated” by including it in the next relevant load forecast.  Duke interprets 001-101.4 to permit the TP to then extract the delivery point information from the annual forecast and update the delivery point list annually.


9/29/10:  Resolved  


�Usually the Network Customer has advanced notice of a New Network Load such that it can be accounted for in the next update to the required Load Forecast. Why can’t this New Network Load be addressed through the Load Forecast update rather than this process?  Also, if a Network Customer submits a New Network Load in a given year, will it continue to be a New Network Load OR would it be incorporated in a future year(s) projected load forecast? We may want to address these items in the requirements above given this is a new concept for Network Customers.


9/29/10:  resolved


�Is this tariff language?  What does it mean? 





9/29/10:  paragraph was deleted.


�Is this tariff lanugage?





9/29/10:  paragraph was deleted.


�What might this be?





9/29/10:  paragraph was deleted.


�What does this look like in real life?  This makes the accomodation optional on the part of the TP…what are  the ramifications?





9/29/10:  paragraph was deleted.


�9/29/10:  paragraph was deleted.


�NCEMC suggests that the “may” should be replaced with “shall” to promote consistency and uniformity.  A Network Customer MUST know what is the minimum information required for queuing a DNL to serve this New Network Load.





9/29/10:  paragraph was deleted.


�Entergy suggests that this provision could be construed to REQUIRE transmission providers to post a business practice.  If not, revision is suggested for clarity.  





9/29/10:  paragraph was deleted.


�9/29/10:  paragraph was deleted.


�9/29/10:  paragraph was deleted.


�9/29/10:  paragraph was deleted.


�9/29/10:  paragraph was deleted.


�9/29/10:  paragraph was deleted.


�What is the criteria for a “significant change” for a request for temporary designation of new Network Load?��Shouldn’t DECLINED be included in the status within 105.5.3.2?





9/29/10:  paragraph was deleted.


�To be consistent with the Tariff, should this not be “material” change?  Is that what NAESB was referring to?  If so, make sure the document is consistent with Tariff language.  





9/29/10:  paragraph was deleted.


�9/29/10:  paragraph was deleted.


�9/29/10:  paragraph was deleted.


�The concept of Network Scheduling Right (NSR) may provide some value to deal with unique situations in the Western Interconnection, but it is not well integrated into the standards.  Where the NSR concept is used, it must be clear DNRs provide no associated scheduling rights.  That is, the standards should lay out that the designation of a DNR either comes with associated scheduling rights and the NSR concept is not used or that a DNR comes with no scheduling rights and NSRs must be obtained in addition to DNR status.  A situation where some DNRs have scheduling rights, but others do not, appears facially untenable – how would a customer know which DNRs have rights and which do not?  In effect, TPs that view DNRs and scheduling rights as a one and the same use one process and other TPs use the other process.  The standard must be clear that depending on the option of the TP, to use NSRs or not use them, there is an additional and crucial step -- every request for a new DNR needs to also be made with a request for an NSR or a request for association with an existing NSR.  Also, if NSRs are used, an Application for NITS service should have associated with it a process for seeking the associated NSRs.  Currently, the NSR is only mentioned in the modification of service.  





9/29/10:  Moved to parking lot


�9/29/10:  paragraph was deleted.


�Does this override the customer’s selection in 105.5.1?


�9/29/10:  paragraph was deleted.


�What is this?


�9/29/10:  paragraph was deleted.


�So…a request for new network load is a request for ATC!  If it cannot be accomodated, the TP is under obligation to make system changes to accommodate the load.


�9/29/10:  paragraph was deleted.


�In paragraph 219 of Order 890B, the FERC reminded /clarified the obligation of the TP to adequately plan for the needs of its network customers on a basis comparable to the delivery of its own generating and purchased resources to its native load customers as well as the obligation of network customers to provide information about its forecasted network loads and network resources. So while above might happen on a case by case basis, the TP is obligated to plan its system to meet the Network Customers existing and future loads with sufficient advanced notice.  Also, the request for designation of new network load should not be refused as a result of posted ATC that has not been updated for quite some time (more than 6-12 months) and is based on outdated models.





9/29/10:  paragraph was deleted.


�Isn’t the issue more one of when the request can be accomodated rather that the magnitude of load cannot be accomodated?  i.e.,  when can the upgrades be completed?





9/29/10:  paragraph was deleted.


�9/29/10:  paragraph was deleted.


�


�This appears to be covered above in the concept that there could be iterative negotiations.


�9/29/10:  paragraph was deleted.


�9/29/10:  paragraph was deleted.


�Strike this…same as 105.5.13.1.2


�9/29/10:  paragraph was deleted.


�9/29/10:  paragraph was deleted.


�This section should be moved ahead of 105.5.14.1.  It doen’t have a logical flow to address response to a REBID after discussion of “Once CONFIRMED…”





9/29/10:  paragraph was deleted.


�9/29/10:  paragraph was deleted.


�Believe this process should be very minimal: a request for indefinite termination, determination that it is valid, and finally accepted/confirmed termination.  Do not believe all the states of COMPLETED, etc. need to be included.


Under Final Disposition of the Indefinite Termination, however, should have some obligation placed on the TP that IF such indefinite termination results in a situation that the aggregate transmission capacity held by the Customer (in NSRs) exceeds that required to reliably deliver the remaining aggregate DNRs to DNLs, the TP shall release such capacity to ATC for sale/purchase by any other eligible customer.  9/29/10:  Discussion held with no participants concurring with this position.


9/29/10:  paragraph was deleted. 


�9/29/10:  paragraph was deleted.


�What is meant by “indefinite”?  Is it tariff language?  Since this is the only kind of termination of network load, why “termination” it need an adjective?


�9/29/10:  paragraph was deleted.


�9/29/10:  paragraph was deleted.


�If there is anything required other than what is required in section 31 of the OATT, then there needs to be a standard indicating that the TP may post such requirements in the BPs. If not, then I would suggest referencing the OATT section 31 within the standard for clarification. 





9/29/10:  paragraph was deleted.


�9/29/10:  paragraph was deleted.


�9/29/10:  paragraph was deleted.





���Make consistent throughout document.  Generally, the Transmission Provider’s BPs will hold this level of detailed information as to when a customer shall respond and what states the request will be placed in.  





9/29/10:  paragraph was deleted.


�9/29/10:  paragraph was deleted.





�There are no time limits in the table for confirmation of indefinite termination.  Entergy suggests that some time limits for retraction are necessary.  If none can be agreed upon, the above revision is suggested.


�9/29/10:  paragraph was deleted.





�Question whether modification to what the Subcommittee refers to as NSRs is subject to modification with respect to ‘Termination’.  Recognizing that there are differences of opinions between TPs on whether designation of a network resource implies a  request for an ‘NSR’ or whether such a request for transmission to back delivery of a DNR is separate and distinct from the ability to make such a designation, it would seem that for the purposes of undesignation the release of ATC committed to back a DNR really needs to be in the hands of the TP.  To do otherwise might allow a NITS customer to hold transmission rights for a resource that has been undesignated.  There may well be arguments that one could add a DNR but not request additional ‘NSRs’ to support that designation, but the ability to undesignated a DNR without releasing capacity committed in support of that resource would seem to constitute a mechanism for ‘hoarding’ transmission rights from other uses.





08/31/10 – comment moved to this section (NSR) by subcommittee from 105.1.1 without resolution.





9/29/10:  Moved to Parking Lot - may be addressed in WEQ 013.


�9/29/10:  The subcommittee agrees that the WEQ 001 Standards will not require linkage between NSR/DNR/DNL, but this does not address what will be included in WEQ 013


�Disagree with the premise that such a request for additional rights or even release of rights (but at least release advantages others) can be made outside of a request for a new DNR or new DNL.  It is not clear from the standards whether this transaction can be requested on its own.


If this section is retained, would suggest that it be clearly indicated that the request for additional NSRs must ONLY be made concomitant with a new DNR or DNL.


The subcommittee should determine the rationale for allowing a customer to make explicit requests for NSRs with clear understanding of why these might be required to be requested explicitly rather than implicitly or in lock step with a new DNR/DNL.  For many TPs (maybe a majority?) one would think the information supplied in the request for DNR (e.g., capacity, POR at point of import for off-system resources, etc.) along with the existing approved DNLs provide all the information one would need to assess and grant transmission capacity rights to the NITS Customer and adjust ATC.


105.7.15.3 and 105.7.15.3.1 seem out of place in this section.  At this point the standard is discussing Final Disposition yet these deal with negotiation and capacity set by TP at time of acceptance.  105.7.15.3 is very similar to 105.7.14.1.2 and not sure of the reason for duplication.





9/29/10:  Comment resolved





�I don’t see any information about how a TC assigns a DNR to an existing NSR.





9/29/10:  comment resolved


�The concept of Network Scheduling Right (NSR) may provide some value to deal with unique situations in the Western Interconnection, but it is not well integrated into the standards.  Where the NSR concept is used, it must be clear DNRs provide no associated scheduling rights.  That is, the standards should lay out that the designation of a DNR either comes with associated scheduling rights and the NSR concept is not used or that a DNR comes with no scheduling rights and NSRs must be obtained in addition to DNR status.  A situation where some DNRs have scheduling rights, but others do not, appears facially untenable – how would a customer know which DNRs have rights and which do not?  In effect, TPs that view DNRs and scheduling rights as a one and the same use one process and other TPs use the other process.  The standard must be clear that depending on the option of the TP, to use NSRs or not use them, there is an additional and crucial step -- every request for a new DNR needs to also be made with a request for an NSR or a request for association with an existing NSR.  Also, if NSRs are used, an Application for NITS service should have associated with it a process for seeking the associated NSRs.  Currently, the NSR is only mentioned in the modification of service.  





9/29/10:  Moved to Parking Lot - may be addressed in WEQ -013


�9/29/10


Resolved


�As defined and based on our understanding of the concept of Network Scheduling Rights, NCEMC is opposed to such an optional mechanism that may reduce consistency and uniformity among Transmission Providers of which these proposed NITS standards are intended to promote. We would suggest including an explanation of why or when a DNR cannot serve the same purpose as a NSR with some TPs, while it may not for others. 


9/29/10:  Resolved 


We suggest responses to the following scenario to illustrate the difference between DNRs and NSRs: When the NSR is less than the DNR for a couple of Network Resources of a Network Customer, what happens when one Network Resource experiences a forced outage thus triggering the scenario when a Network Customer may need to use more of the second DNR. If the Network Customer doesn’t have adequate NSRs to use the second DNR, then will they need to submit another NSR for the additional needed capacity/scheduling right during this emergency period in which the DNR should have provided anyway for the Network Customer?


9/29/10:  Copied to parking lot, potentially addressed by WEQ 013


�9/29/10 - resolved


�9/29/10:  This section has undergone language and numbering changes, but the underlying intent of this section is to allow the TP to post NSRs by creating an OASIS template record with a status of ACCEPTED. 


Resolved.


�9/29/10


resolved


�9/29/10


Section removed from standard


�Would this be required for any TP that is using NSR?


�9/29/10


Resolved


�This sentence does not make sense….how did the discussion of termination of NSR get into this sentence?  Is this some kind of concomitant request?


�r9/29/10


 section removed


�This standard is confusing in the manner in which it is written, or as it is provided in this introductory section on NSRs. Not sure what the concept is here. Should this be included in 001-105.8.4 instead of here?


�9/29/10


Section removed from standard


�Entergy suggests that this provision could be construed to REQUIRE transmission providers to post a business practice.  If not, revision is suggested for clarity.  


�9/29/10


resoved – addresesed in  another section





�9/29/10


To be addressed in WEQ 002 and WEQ 013.





�9/29/10 – addressed.


�Apparently NSRs will not be used in conjunction with non-designated resources.  How will non-designated resources’ scheduling rights be identified?





If a TP requires a NSR request in conjunction with a DNR but the TC does not submit the NSR, may the DNR be denied?





9/29/10:  1st paragraph  in this comment resolved, 2nd paragraph moved to parking lot.


�May it exceed the DNR capacity?





9/29/10:  Moved to parking lot


�9/29/10:  resolved


�9/29/10:  resolved


�9/29/10:  resolved


�9/29/10:  resolved


�How does this differ from 105.7.2?





9/29/10:  resolved


�Entergy suggests that this provision could be construed to REQUIRE transmission providers to post a business practice.  If not, revision is suggested for clarity.  


�9/29/10:  resolved


�9/29/10:  resolved


�9/29/10:  resolved


�9/29/10:  resolved


�9/29/10:  resolved


�9/29/10:  resolved


�9/29/10:  resolved


�I don’t think you’ll find any tariff requriements for this so I don’t think this phrase is needed.


�9/29/10:  resolved


�9/29/10:  resolved


�9/29/10:  resolved


�9/29/10:  resolved


�9/29/10:  resolved


�9/29/10:  resolved


�This processing should be done in parallel (concomitantly) with the DNR request that triggered it. 





9/29/10:  resolved


�9/29/10:  resolved


�9/29/10:  resolved


�9/29/10:  resolved


�9/29/10:  resolved


�How is this different from the study that is done for a DNR?  I don’t believe the pro forma tariff makes provisions for this study apart from a new DNR request.





10/12/2010 paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�This appears to be covered above in the concept that there could be iterative negotiations.





10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�Already covered in 105.7.14.1.3





10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�This seems out of place…it should be in the section(s) associated with TP accepatance.





10/12/2010:  paragraph removed; resolved


�10/12/2010:  Note:  Entergy’s comment was to delete this paragraph; it has been deleted.


�10/12/2010:  rewritten and resolved


�As cited above, seriously question that the Customer requires the ability to perform these actions.  The TP should manage the transmission rights to reliably deliver DNRs to DNLs and unclear why the Customer should be placed in a position to know what or how much capacity to release.  


Suggestion was made above that the release of NSRs is an obligation of the TP when assessing temporary or indefinite termination of a DNR (or DNL).  But, not sure why the TP would want to or be required to complete a request for termination of NSR as suggested in 105.8.1.1.


Subcommittee should determine if it is legitimate for a Customer to terminate a DNR and NOT release capacity from an NSR.  Or if the Customer may legitimately request release of only a portion of NSR that backs delivery of a terminated DNR.


If these concepts are retained, these are very simple transactions and do not require explicit use of COMPLETED, etc.; request is submitted, determined if valid and accepted/confirmed.





10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�Entergy suggests that this provision could be construed to REQUIRE transmission providers to post a business practice.  If not, revision is suggested for clarity.





  10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�It would seem that for TPs that use NSR, the termination of a DNRwould normally require submission of a request to terminate the NSR.  This practice makes it optional for the TC to ingor the NSR ramifications.





10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�10/12/2010:  paragraph removed; resolved


�Not needed.


�If invalid, does it have any bearing on the request for temporary termination of a DNR?


�As cited above, seriously question that the Customer requires the ability to perform these actions.  The TP should manage the transmission rights to reliably deliver DNRs to DNLs and unclear why the Customer should be placed in a position to know what or how much capacity to release.  


Suggestion was made above that the release of NSRs is an obligation of the TP when assessing temporary or indefinite termination of a DNR (or DNL).  But, not sure why the TP would want to or be required to complete a request for termination of NSR as suggested in 105.8.1.1.


Subcommittee should determine if it is legitimate for a Customer to terminate a DNR and NOT release capacity from an NSR.  Or if the Customer may legitimately request release of only a portion of NSR that backs delivery of a terminated DNR.


If these concepts are retained, these are very simple transactions and do not require explicit use of COMPLETED, etc.; request is submitted, determined if valid and accepted/confirmed.


�Entergy suggests that this provision could be construed to REQUIRE transmission providers to post a business practice.  If not, revision is suggested for clarity.  


�There are no time limits in the table for confirmation of indefinite termination.  Entergy suggests that some time limits for retraction are necessary.  If none can be agreed upon, the above revision is suggested.


�10/12/2010:  All comments in the highlighted section have been addressed by deleting the section


�Suggest that these BPs state that arrangement of secondary service follows the OASIS PTP transaction data model, but may be submitted through either the standard PTP OASIS template specifications or the NITS template specifications.


Disagree with explicit requirement (shall...set) for use of RECEIVED, COMPLETED; this forces a status state transition that is supported but not required today.


105.10.11.3 and 105.10.11.3.1 seem out of place in this section.  At this point the standard is discussing Final Disposition yet these deal with negotiation and capacity set by TP at time of acceptance.  105.10.11.3 is very similar to 105.10.10.1.2 and not sure of the reason for duplication.





10/12/2010:  Comments will be addressed in re-write


�10/12/2010:  Comments will be addressed in re-write





�10/12/2010:  Comments will be addressed in re-write


�10/12/2010:  Comments previously addressed.


�NCEMC suggests that the “may” should be replaced with “shall” to promote consistency and uniformity.  The Network Customer must know the minimum requirements for submitting a request for secondary network transmission service. 





10/12/2010:  Comments previously addressed


� Suggest a statement at the beginning of the BP something to the effect that the TPs approved tariff, posted business practices, executed contracts supersede these NAESB BPs.





10/12/2010:  Comments previously addressed


�Entergy suggests that this provision could be construed to REQUIRE transmission providers to post a business practice.  If not, revision is suggested for clarity. 





10/12/2010:  Comments previously addressed 


�10/12/2010:  Comments will be addressed in re-write


�10/12/2010:  Comments previously addressed 


�10/12/2010:  Comments previously addressed 





�10/12/2010:  Comments will be addressed in re-write


�10/12/2010:  Comments will be addressed in re-write


�10/12/2010:  Comments will be addressed in re-write


�10/12/2010:  Comments will be addressed in re-write


�Why do requests for Secondary Network Transmission Service trigger a need to modify the CONFIRMED NITS Application given that this service might be used very frequently, but generally speaking are temporary in nature?  In addition, section 28.4 of the Pro Forma OATT states that “Secondary service shall not require the filing of an Application for Network Integration Transmission Service under the Tariff.” Thus, this standard is not needed and should be eliminated in our opinion.


�10/12/2010:  Resolved


�BPA supports making this optional here.


�10/12/2010:  To be resolved in re-write


�This appears to be covered above in the concept that there could be iterative negotiations.





10/12/2010:  To be resolved in re-write


� 10/12/2010:   Entergy recommended deletion of this section.. To be resolved in re-write


�What is criteria for a “valid and complete request” for termination of Secondary Network Transmission Service?





10/12/2010:  resolved


�10/12/2010:   Entergy recommended deletion of this section.. To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�Believe this process should be very minimal: a request for termination, determination that it is valid, and finally accepted/confirmed termination.  Do not believe all the states of COMPLETED, etc. need to be included.


Should consideration be given for this and the other termination of service modifications to be required to be pre-confirmed?





10/12/2010:  Matter discussed and resolved that we will not require pre-confirmed requests as a NAESB BP.


�Why is it optional?  It should be mandatory, otherwise, a customer will be forced to retain reserved secorndary network service, which may be blocking another desired use of the transmission capacity.  It isn’t optional for release of PTP NF Secondary.  Could lead to queue hording if not changed.





10/12/2010:  discussed, optional to TP, based on TP’s practices of releasing unscheduled NF into the market.


�10/12/2010:    Discucussed, To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�These requests should be submitted pre-confimred, as is done with release of PTP NF Secondary.





10/12/2010:  Matter discussed and resolved that we will not require pre-confirmed requests as a NAESB BP.. 


�10/12/2010:    To be resolved in re-write


�10/12/2010:  Matter discussed and resolved that we will not require pre-confirmed requests as a NAESB BP.





�For the same reasons given for 001-105.10.11.1, it is our opinion that this standard is not needed and should be eliminated.





10/12/2010:  Matter discussed and resolved that we will not require pre-confirmed requests as a NAESB BP.


�10/12/2010:    To be resolved in re-write


�10/12/2010:    To be resolved in re-write


�Termination of Secondary Network Transmission Service does not appear to be covered in the referenced table unless it is to be considered in the 1st row.  If that is the case, the table should be clarified that the Secondary Network Transmission Service row applies to the request for new service and the termination of service.





10/12/2010:  Resolved by adding to Table 105-A


�


�Not certain if an implementation plan is needed for the entire section 001-105 or only for Secondary Network Service.  NCEMC recommends an implementation plan that includes field testing of the NITS standards but would suggest that the period for such transition be kept to a minimum, such as 6 months.


�Suggest that the standard cite examples or list ALL the legal concomitant transactions, such as:


Termination (Temporary or Indefinite) of existing DNR with concomitant request for a new Designated Network Resource, i.e., redesignation of Network Resource.


Termination (Temporary or Indefinite) of existing DNR with concomitant request for a new Point-to-Point Transmission Service.


(others?)


Disagree with the one-to-one nature of the requests that may qualify.  Would seem that the ability to terminate a DNR and request multiple PTP reservations to be a viable option useful to customers.


Rather than citing use of a ‘concomitant’ flag suggest that the standard simply require that the requests to be considered for concomitant evaluation be clearly identified; transaction model suggested for NITS template was envisioned to simply use a ‘transaction’ envelop for the concomitant requests rather than a flag on the individual requests.


Suggest 001-106.4 be broadened to simply say that the individual requests included for concomitant evaluation shall each be bound by the processing requirements associated with the type of request submitted per standards called out in 001-105.  This would then include both the TPs obligations and process and TCs obligations and process.


Rather than forcing concomitant requests to all go through a negotiation process, suggest it would be simpler to require that once all requests in concomitant eval are confirmed (by whatever means appropriate for that request), all concomitant requests are deemed complete and confirme; if any one of the concomitant requests is not taken to confirmed state, all the other concomitant requests are considered null and void.


�10/12/2010:  Subcommittee discussed and will retain one to one concopt, consistent with WEQ-001-106.1


�10/12/2010:  Deferred by Subcommittee to the Technical Team


�


�There also needs to be a recognition that there may also be a request for termination of a NSR.


�Should the “concomitant evaluation flag set” be a Tech Team item and thus should be highlighted GREEN?


�Does denial of the NSR result in denial of these two requests?


�Is this a requirement for every TP to speciy something in a BP?  The standard should define the default rather than requiring all TPs to post a practice.


�Will WEQ-002 require posting this forecast info to a template?  If so, it will be an additional burden on customers.


�To avoid customer concern, this standard should indicate that the referenced (yet-to-be-developed) WEQ-002 standard will not require public disclosure of the annual forecast of load and resources on OASIS.


�Since the current posted NAESB WEQ Business Practice Standards Booklet (version 002.1 with approved on December 14, 2009) currently contains nothing in WEQ-002 regarding this annual update of Network Loads or Network Resource forecasts, it is difficult to provide any meaningful comments.  Is the annual update to be done over the OASIS or is optional to provide the annual update on or off OASIS?


�This description is inadequate.  This section should clearly delineate the difference in process for renewal of the contract based on the 5yr/1yr requirements for rollover, vs. any ability of the customer to extend the term of a valid DNR and how that is accomplished.


Notice of intent to renew the NITS agreement should be tendered within the required renewal deadline.  Should or must it also contain any or all information on the extension of any existing DNRs?  


An extension of DNR could be new transaction required for 001-105 to allow the Customer to request move of end time of existing DNR for term and maybe capacity specified.  This is similar to request to renew the PTP reservations with rollover.


One would think that an extension of existing DNR should NOT be considered a ‘new DNR’ subject to all the study implications, etc., of a true new DNR, and be more like renewal of PTP but subject to some conditions.  Should this be that the DNR is min 5 years?  Or should it be that the DNR has been identified and included in the Resource Forecast?


If you consider DNRs themselves to have some type of ‘evergreen’ rights, is it (should it be) based on the same 5yr/1yr cycle as PTP?  FERC was clear on the NITS agreement/contract but not on the DNRs that comprise that agreement.


If you do have some minimum term or such to id DNRs that have some form of renewal rights, may need to allow the term of DNRs to go beyond the terms of the existing NITS agreement.  Should the NITS agreement not be renewed, the DNR beyond term of that agreement is released.  For example at year three of existing NITS contract, have a new DNR under a 10-year PPA.  Should consider that it is legitimate to request that DNR for the full 10 years, study its deliverability for the full 10 years, incorporate that into transmission plan, etc.  But if at year 4 the customer elects to not re-up their NITS service, all things surrounding the DNR are terminated early at the end of the NITS agreement.  If they do renew, the DNR stands.


Note that from a transaction point of view, do not view the renewal or extension of NITS application(agreement) or any DNR/DNL as anything more than a REQUEST to move the end/stop time of the associated contract, and if accepted/approved, is simply reflected by resetting the end time of that contract – not a whole new contract or whatever, as is done in PTP TSRs.


�This is inadequate treatment for rollover rights.


�The failure to develop standards on rollover rights remains troubling to Duke as it may lead to very differing business practices and litigation is likely to result.  NAESB should obtain clarity from FERC as to how rollover rights should work and develop accompanying standards.  


�NCEMC is puzzled as to WHY this is an optional standard rather than and required one.  We are of the opinion that the “may” should be replaced by “shall”.


�the statuses of SUPERSEDED and DISPLACED should be applicable to secondary network or the PTP portions of concomitant requests only.  A footnote to clarify same is suggested.


�These two defined terms do not appear to be used/addressed in the standards. They may be useful if a TP competes NT service, loses the competition and needs a means to "override" a CONFIRMED NT award in such scenario.  


�These two defined terms do not appear to be used/addressed in the standards. They may be useful if a TP competes NT service, loses the competition and needs a means to "override" a CONFIRMED NT award in such scenario.  


�For the term REEVALUATE, please add a “,” before “and”, and also the text “the Customer is” between “and” and “submitting” such that the definition reads as follows:�“assigned by the Eligible Transmission Customer or Network Customer during the NITS Application or request for modification to service process indicating that deficiencies have been corrected, and the Customer is submitting the revised NITS Application or request for modification to service to the Transmission Provider for review.






Page 130 of 141
9-10-2010

9/29/10 Revised with subcommittee responses


