[image: image3.png]



[image: image3.png]
RECOMMENDATION TO NAESB EXECUTIVE COMMITTEE

                                       For Quadrant: 
Retail Electric Quadrant

                                       Requesters:

Smart Grid PAP 10 Subcommittee
                                       Request No.: 

REQ 2010 AP Item 9(d)

                                       Request Title:
Business Practices and Information Models to Support Priority Action Plan 10 – Standardized Energy Usage Information

1.  RECOMMENDED ACTION:
EFFECT OF EC VOTE TO ACCEPT RECOMMENDED ACTION:

	X
	Accept as requested
	X
	Change to Existing Practice

	
	Accept as modified below
	
	Status Quo

	
	Decline
	
	


2.  TYPE OF DEVELOPMENT/MAINTENANCE

	Per Request:
	Per Recommendation:

	X
	Initiation
	X
	Initiation

	
	Modification
	
	Modification

	
	Interpretation
	
	Interpretation

	
	Withdrawal
	
	Withdrawal

	
	
	
	

	X
	Principle
	X
	Principle

	X
	Definition
	X
	Definition

	X
	Business Practice Standard
	X
	Business Practice Standard

	X
	Document
	X
	Document

	X
	Data Element
	X
	Data Element

	X
	Code Value
	X
	Code Value

	
	X12 Implementation Guide
	
	X12 Implementation Guide

	X
	Business Process Documentation
	X
	Business Process Documentation


3.  RECOMMENDATION

SUMMARY:


The Joint Retail Electric (REQ) and Wholesale Electric (WEQ) Quadrants’ PAP 10 Smart Grid Subcommittee submit this Recommendation for 2010 Retail Annual Plan Item No. 9d and 2010 WEQ Annual Plan Item No. 6d – Business Practices and Information Models to support Priority Action Plan 10, “Standardized Energy Usage Information,” based on the Tiger Team Report issued on June 7, 2010 by the NIST SGIP PAP10 Committee.  

In initiating this standards development, NAESB agreed to by year-end 2010 develop an energy use information model standard defining a common data format that may be used when information is communicated between utilities, third parties and energy use customers, via customer devices and/or third party energy services providers.  The energy usage information model standard will enable the exchange of detailed energy information in a consistent format for use by customers, utilities, service providers, consumer devices, and energy applications.  Doing so will let Customers track their power use and help them manage energy consumption and cost.  Without a standardized format for representing energy use data, a variety of approaches could emerge, leading to incompatibilities among energy management products and services.  It is planned that the American Society of Heating, Refrigerating and AC Engineers (ASHRAE) will extend the NAESB standard to create a facilities information model providing additional energy use data elements for facility energy management including buildings.   
RECOMMENDED STANDARDS:

REQ.18
CUSTOMER ENERGY USAGE INFORMATION COMMUNICATION

EXECUTIVE SUMMARY

This document establishes  the Model Business Practices for Retail Customer energy usage information communication.  Specifically, these Model Business Practices establish  an information model for energy usage information.  These Model Business Practices do not limit the form or function of the information model and is exemplary, but not exclusive, of the information   that may be communicated in a consistent format among a variety of Entities, including, but not limited to, Distribution Companies, energy service providers, meter-reading entities, and Retail Customers.  Such communication may occur via multiple systems and devices.  Establishment of this energy usage information model will standardize a common data format which may be used when information is communicated between Distribution Companies, third parties and energy use Retail Customers which may aid Retail Customers in tracking and managing their energy use. 
These Model Business Practices do not require that wholesale electricity markets administered by system operators adopt this energy usage information model since system operators generally do not maintain or have access to the system of record for individual Retail Customer energy usage information and load data or individual Retail Customer forecasted usage and load data.  These Model Business Practices are not intended to replace applicable Governing Documents, and, in the event of a conflict, the latter documents shall have precedence over these Model Business Practices. Without limiting the foregoing, these Model Business Practices are only applicable to the extent the information covered by this energy usage information model is collected, managed or communicated pursuant to the applicable Governing Documents.  Retail Customer energy usage information communication encompasses a variety of interactions between Distribution Companies, Retail Customers and energy services providers.  In a business environment where best practices are voluntary, these Model Business Practices may be applied within the context of regulatory or other market requirements and agreements. 

INTRODUCTION

The North American Energy Standards Board (NAESB) is a voluntary, non-profit organization comprised of members from all aspects of the natural gas and electric industries.  Within NAESB, the Retail Electric Quadrant (REQ) and the Retail Gas Quadrant (RGQ) focus on issues impacting the retail sale of energy to end-use Retail Customers.  REQ / RGQ Model Business Practices are intended to provide guidance to Market Participants. The focus of these Model Business Practices is the representation of Retail Customer energy usage information.  The scope of the energy usage information model is not intended to characterize the data information model for communication of  billing or related information. 
These Model Business Practices are voluntary and do not address policy issues that are the subject of state legislation or regulatory decisions.  These Model Business Practices have been adopted with the realization that as the industry evolves, additional and amended Model Business Practices may be necessary.  Any industry participant seeking additional or amended Model Business Practices (including principles, definitions, data elements, process descriptions, and technical implementation instructions) should submit a request to the NAESB office, detailing the change, so that the appropriate process may take place to amend the Model Business Practice.
BUSINESS PROCESSES AND PRACTICES

Overview 

This Model Business Practice establishes  the Model Business Practices for Retail Customer energy usage information communication.  Specifically, these Model Business Practices establish an information model for energy usage information.  The Model Business Practice does not limit the form or function of the information model and is exemplary, but not exclusive, of the information that may be communicated in a consistent format among a variety of Entities, including, but not limited to, Distribution Companies, energy service providers, meter-reading entities, and Retail Customers.  Such communication may occur via multiple systems and devices.  Establishment of this energy usage information model will standardize a common data format which may be used when information is communicated between Distribution Companies, third parties and energy use customers which may aid Retail Customers in tracking and managing their energy use. 
The business processes and practices described below are not presently applicable to wholesale markets because wholesale markets do not generally communicate directly with Retail Customers and are not the system of record for individual Retail Customer energy usage information or individual Retail Customer load forecast. The energy usage information model and these model business processes and practices are not required of system operators.  As the energy usage information model and these business processes and practices evolve, system operators may determine that use of the energy usage information model in these Model Business Practices can be applied to other information.  However, such use is not intended to replace or supplant applicable Governing Documents. Without limiting the foregoing, these Model Business Practices are only applicable to the extent the information covered by this energy usage information model is collected, managed or communicated pursuant to the applicable  Governing Documents. 
REQ.18.1
Principles

REQ.18.1.1
Overall Principles

REQ.18.1.1.1 These Model Business Practices provide an energy usage information model, which defines a collection of structured energy usage information classes and attributes that may be used to enable Retail Customer management of their energy usage and costs.
REQ.18.1.1.2 The energy usage information model is specified in UML, as a syntax neutral notation, so that it may be used within exchange protocols using a variety of specific representation syntax and exchange mechanisms, specified outside these Model Business Practices. 
REQ.18.1.1.3 The recommended use of the energy usage information model is in implementation specifications exposing Retail Customer energy usage information. Specifications that conform to the energy usage information model shall contain equivalent required and optional classes and attributes, thus resulting in straightforward (preferably lossless) transformations between conformant specifications.
REQ.18.1.1.4 Neither the energy usage information model, nor these Model Business Practices establish or govern ownership or any other rights in any information or data; such ownership and other rights are subject to and governed by the Governing Documents and/or Applicable Regulatory Authority.

REQ.18.1.1.5 Neither the energy usage information model nor these Model Business Practices create any requirement to collect, manage or communicate any information.
REQ.18.1.1.6 While these Model Business Practices define an information model which may be used when energy usage information is communicated, the Governing Documents and/or Applicable Regulatory Authority determine the ownership of the data, the access to the data, what systems and hardware are necessary to provide this information, and how it is paid for. There are no assumed or implied regulations in these Model Business Practices.
REQ.18.1.1.7 Appendix A represents a series of capabilities for the expressiveness of these Model Business Practices and are specifically not intended to be requirements for the use of or on users of these Model Business Practices.  
REQ.18.2 Energy Usage Abbreviations, Acronyms and Definition of Terms

REQ.18.2.1 Business Definitions

RXQ.00.xx

Applicable Regulatory Authority
The state regulatory agency or other governing body that provides oversight, policy guidance, and direction to any parties involved in the process of providing energy to Retail Customers through regulations and orders.

RXQ.00.xx

Retail Customer
Any Entity that takes gas and/or electric service for its own consumption.
RXQ.00.xx

Distribution Company
 A regulated Entity which provides distribution services and may provide energy and/or transmission/transportation services in a given area.
RXQ.00.xx

Entity
A person or organization with sufficient legal standing to enter into a contract or arrangement with another such person or organization (as such legal standing may be determined by those parties) for the purpose of conducting and/or coordinating energy transactions.

RXQ.00.xx

Governing Documents
Documents that govern the interactions among parties, including but not limited to: applicable law, regulatory documents (e.g. tariffs, rules, regulations), contractual agreements, and Distribution Company Operational Manuals and other relevant models and procedures.

RXQ.00.xx

Market Participant
A Distribution Company, Supplier, Registration Agent, settlement agent, meter reading Entity or other party engaged in the process of providing competitive retail energy or another specified program to Retail Customers.
RXQ.00.xx

Model Business Practices
Electric and gas industry processes and procedures developed by interested parties representing the NAESB Retail Gas and Electric Quadrants’ segments and ratified by the NAESB Retail Gas and Electric Quadrants’ members
RXQ.00.xx

Supplier

Entities engaged in the competitive sale of energy to end-users.

REQ.18.2.2 Technical Definitions

This section contains technical terms and abbreviations used in this recommendation.  Additional Technical Definitions are located in REQ.18.4.1.
REQ.18.2.3
Abbreviations and Acronyms
	Abbreviation / Acronym 
	Meaning

	UML
	Unified Modeling Language

	EMS
	Energy Management System

	ESI
	Energy Services Interface

	PEV
	Plug-in Electric Vehicle

	EISA
	Energy Independence and Security Act

	DOE
	Department of Energy

	HAN
	Home Area Network

	XMI
	XML Media Exchange

	XML
	Extensible Markup Language

	HTML
	HyperText Markup Language

	XSD
	XML Schema Definition Language

	CIM
	Common Information Model

	IEC
	Internationl Electrotechnical Commission


REQ.18.3 Energy Usage Information Model Business Practices

REQ.18.3.1 Introduction

The focus of these Model Business Practices is the representation of energy usage information, which includes historic, present, and future projected usage and load together with the time period(s) for that information.

These Model Business Practices drew upon actors and use cases defined by the following groups:

· Energy Information Standards Alliance (the EIS Alliance) [EIS]

· NAESB Survey and Consolidation of PAP10 Use Cases [NAESB PAP10]

· UCAIug OpenADE [ADE]

· ZigBee/Home Plug Smart Energy Profile 2.0 Market Requirements [SEP MRD]

The relevant use cases are summarized as follows:

REQ.18.3.1.1 The energy service provider and/or Distribution Company communicates historic and present energy usage information and load information to the Retail Customer or facility.

REQ.18.3.1.2 The Retail Customer or facility communicates future projected usage and load information to the energy service provider, Distribution Company, or grid operations.

REQ.18.3.1.3 The energy service provider and/or Distribution Company communicates their projection of usage and load to the Retail Customer or facility.

REQ.18.3.1.4 Devices within a facility communicate their present and future projected usage and load to controllers or facility EMS for aggregation and to be a component of facility aggregated future projected usage and load.

REQ.18.3.1.5 Devices, business processes, EMS, ESI, and other functional units within the facility communicate usage and load information among themselves.

REQ.18.3.1.6 These Model Business Practices are limited to the seed specification which shall be usable by others to build standards, or additional Model Business Practices, and/or specification for exchange of energy usage information and load information appropriate to their needs without overly constraining those uses or including information that is not required in all implementations of specifications for exchanging load and usage.

REQ.18.3.2 Energy Usage Information Model Format and Use
The energy usage information model is developed using a UML modeling tool.  The model classes, attributes, types and descriptions are included in REQ 18.4.  The model is made available as XMI, which is the standard XML import/export format for UML.  The model is exported as HTML and made available as a downloadable archive viewable with a web browser.
REQ.18.3.3 Energy Usage Information Model Technical Considerations

REQ.18.3.3.1 The energy usage information model may be used as the basis for smart grid interfaces between parties exchanging customer energy usage information between energy services providers, consumers, and others. 

REQ.18.3.3.2 The energy usage information model permits schemas to be generated from it using XSD and other format specification languages. Profiles may be constructed from the energy usage information model for this purpose.
REQ.18.3.3.3 Implementations may include all or a subset of the elements defined in the model, possibly using a profile of the energy usage information model. 

REQ.18.3.3.4 The informative example XSD should conform to Naming and Design Rules best practices as described by IEC 62361-100, Naming and Design Rules for CIM Profiles to XML Schema Mapping.

REQ.18.3.3.5 The model facilitates the use of multiple information exchange standards. The specifics are left to implementing specifications to define. 

REQ.18.3.3.6 Though there may be elements useful for the transfer of security-related information elements in the energy usage information model, the specific details related to how to protect sensitive information, and how to authorize specific roles or identities to have access are not defined in these Model Business Practices. 

REQ.18.3.4 Conformance
REQ.18.3.4.1 A specification that claims conformance and refines or extends these Model Business Practices shall produce information for exchange that can be transformed algorithmically (that is based on the Model Business Practices alone) into a form that can be validated through the method described in REQ.18.3.4.2.

This requirement means that various formats for representation and exchange and various subsets and potentially supersets of information content are envisioned based on these Model Business Practices. The use of the informative example schema is not to impose its direct use in message validation. Rather, its use is intended to facilitate verification of conformance to the energy usage information model with respect to message content and semantics without imposing constraints on specific message payload schemas and data element representations. Some representations are anticipated to be entirely binary in nature. Others will trade off strings for integer representations of information contents.  Regardless of the means, the information should be convertible to be testable as described herein.

REQ.18.3.4.2 A specification that claims conformance to these Model Business Practices shall describe and define an automatable transformation between that specification’s model to and from this UML model, including indicating attributes used and not used, as well as the version of the model stated in NAESB_EUI_Version.

REQ.18.3.4.3 A specification that claims conformance to these Model Business Practices shall map corresponding energy usage information model components to and from at least the following required core model attributes as exchanged between data provider and data consumer in their defined messages (Refer to REQ.18.4.1 for definitions of capitalized terms):

· One or more measurement or summary containers: IntervalReading, Reading, ElectricPowerQualitySummary, UsageSummary
· At least two of the following attributes, for each IntervalReading: start, end, duration (from the DateTimeInterval)
· The attribute “value” (the value of the measurement, from IntervalReading or Reading) 

· ReadingType – name, defaultQuality, direction, kind, multiplier, name, unit

· Association to ReadingType for each measurement (IntervalReading or Reading) 
· Measurement source / location – UsagePoint.name and association to measurements or summary
REQ.18.4 Energy Usage Information Model

The energy usage information model herein is organized consistent with several related models, including the IEC TC57 Common Information Model [IEC 61968 Part 9], ZigBee Smart Energy Profile 2.0 [SEP2.0], that are defined by the Energy Information Standards Alliance [EIS Alliance] and Open Automated Data Exchange [OpenADE]. The energy usage information model, where possible, uses classes, information elements and attribute names drawn from the CIM and the cited references.

The starting point for the energy usage information model is the UsagePoint. UsagePoints identify key references for the information set optionally including identification of the Retail Customer, the location, and the physical asset. UsagePoint is associated in turn with zero or more MeterReadings. A MeterReading composes information about a particular measurement such as kWh or kW. A MeterReading has a ReadingType which describes the nature of the measurement including its units of measure, and zero or more IntervalReadings or Readings and associated quality information. UsagePoint may also be associated with summary information on load and usage, and optionally, power quality.  For applications requiring third party access to this information, additional classes are identified to facilitate associating Retail Customer and Retail Customer agreement information with the measurements available at a UsagePoint.

To find the usage or load in a particular interval, identify the appropriate UsagePoint, select the MeterReading of interest (measurement) and then select the IntervalReading or Reading associated with the given interval.

The energy usage information model includes many optional components. The complete set of information expressable using the energy usage information model satisfies a wide range of applicability requirements identified by the industry. Users of these Model Business Practices may optionally take advantage of these extended definitions based on need without requiring them. Applications built on the energy usage information model may elect which optional components to present. However, clients of this information can be expected to recognize all components provided in the application.

REQ.18.3.4.3 idenitifies the set of core model elements that shall be supported by specifications claiming conformance to these Model Business Practices. The following class diagram illustrates a view
 of this core of the energy usage information model:

[image: image1.emf]class Minimal View

UsagePoint

+ name:  String [0..1]

+ description:  String [0..1]

Reading

+ cost:  Float [0..1]

+ timeStamp:  AbsoluteDateTime [0..1]

+ value:  Float [0..1]

ReadingQuality

+ quality:  QualityOfReading [0..1]

ReadingType

+ defaultQuality:  QualityOfReading [0..1]

+ direction:  ReadingDirection [0..1]

+ intervalLength:  Duration [0..1]

+ kind:  ReadingKind [0..1]

+ multiplier:  UnitMultiplier [0..1]

+ name:  String [0..1]

+ unit:  UnitSymbol [0..1]

IntervalReading

+ cost:  Float [0..1]

+ interval:  DateTimeInterval [0..1]

+ value:  Float [0..1]

MeterReading

+ name:  String [0..1]

+ valuesInterval:  DateTimeInterval [0..1]

+UsagePoint 1

+MeterReadings 0..*

+Readings

0..*

+MeterReadings

0..*

+ReadingQualities

0..*

+IntervalReading

1

+MeterReading

0..1

+IntervalReadings 0..*

+MeterReadings

0..*

+ReadingType

0..1

+ReadingQualities

0..*

+Reading

0..1


Figure 1: Energy Usage Information Model – Minimal View
The full energy usage information model, illustrated in Figure 2,  forms the basis of the required Model Business Practices.  Note, some minor classes detailed in REQ.18.4.1 are omitted from the diagram to aid in readability (e.g. DateTimeInterval).
[image: image2.emf]class NAESB PAP10 EUI Diagram

ServiceSupplier

+ kind:  SupplierKind [0..1]

+ name:  String [0..1]

Customer

+ name:  String [0..1]

CustomerAgreement

+ name:  String [0..1]

CustomerAuthorisation

+ name:  String [0..1]

+ validityInterval:  DateTimeInterval [0..1]

EndDeviceAsset

+ name:  String [0..1]

EnergyUsageInformation

IntervalBlock

IntervalReading

+ cost:  Float [0..1]

+ interval:  DateTimeInterval [0..1]

+ value:  Float [0..1]

MeterReading

+ name:  String [0..1]

+ valuesInterval:  DateTimeInterval [0..1]

PositionPoint

+ xPosition:  String [0..1]

+ yPosition:  String [0..1]

+ zPosition:  String [0..1]

Reading

+ cost:  Float [0..1]

+ timeStamp:  AbsoluteDateTime [0..1]

+ value:  Float [0..1]

ReadingQuality

+ quality:  QualityOfReading [0..1]

ReadingType

+ defaultQuality:  QualityOfReading [0..1]

+ direction:  ReadingDirection [0..1]

+ intervalLength:  Duration [0..1]

+ kind:  ReadingKind [0..1]

+ multiplier:  UnitMultiplier [0..1]

+ name:  String [0..1]

+ unit:  UnitSymbol [0..1]

ServiceCategory

+ kind:  ServiceKind [0..1]

ServiceDeliveryPoint

+ name:  String [0..1]

UsagePoint

+ name:  String [0..1]

+ description:  String [0..1]

UsageSummary

+ billingPeriod:  DateTimeInterval [0..1]

+ billLastPeriod:  Float [0..1]

+ billToDate:  Float [0..1]

+ costAdditionalLastPeriod:  Float [0..1]

+ currency:  String [0..1]

+ currentBillingPeriodOverAllConsumption:  SummaryMeasurement [0..1]

+ currentDayLastYearNetConsumption:  SummaryMeasurement [0..1]

+ currentDayNetConsumption:  SummaryMeasurement [0..1]

+ currentDayOverallConsumption:  SummaryMeasurement [0..1]

+ peakDemand:  SummaryMeasurement [0..1]

+ previousDayLastYearOverallConsumption:  SummaryMeasurement [0..1]

+ previousDayNetConsumption:  SummaryMeasurement [0..1]

+ previousDayOverallConsumption:  SummaryMeasurement [0..1]

+ qualityOfReading:  SummaryQuality [0..1]

+ ratchetDemand:  SummaryMeasurement [0..1]

+ ratchetDemandPeriod:  DateTimeInterval [0..1]

ElectricPowerQualitySummary

+ flickerPlt:  Float [0..1]

+ flickerPst:  Integer [0..1]

+ harmonicVoltage:  Float [0..1]

+ longInterruptions:  Integer [0..1]

+ mainsVoltage:  Float [0..1]

+ measurementProtocol:  String [0..1]

+ powerFrequency:  Float [0..1]

+ rapidVoltageChanges:  Integer [0..1]

+ shortInterruptions:  Integer [0..1]

+ summaryInterval:  DateTimeInterval

+ supplyVoltageDips:  Integer [0..1]

+ supplyVoltageImbalance:  Integer [0..1]

+ supplyVoltageVariations:  Integer [0..1]

+ tempOvervoltage:  Integer [0..1]

TariffProfile

+ name:  String [0..1]

BaseCIM_CombinedVersion

+ date:  AbsoluteDate = 2010-10-01 {readOnly}

+ version:  String = iec61970CIM15v0... {readOnly}

NAESB_EUI_Version

+ date:  AbsoluteDate = 2010-10-22 {readOnly}

+ version:  String = naesb_eui_v01.00 {readOnly}

+Readings 0..*

+MeterReadings

0..*

+UsagePoint

1

+ElectricPowerQualitySummaries

0..*

+EnergyUsageInformation

0..*

+ServiceSuppliers

0..*

+ServiceSuppliers 1..*

+Customers

0..*

+Customer1

+CustomerAgreements

0..*

+CustomerAuthorisations

0..*

+CustomerAgreements

0..*

+UsagePoint

0..1

+EndDeviceAssets

0..*

+MeterReading

0..1

+IntervalReadings 0..*

+ReadingQualities 0..*

+IntervalReading

1

+IntervalBlocks 0..*

+IntervalReadings 0..*

+UsagePoint

0..1

+UsageSummaries 0..*

+IntervalBlocks

0..*

+MeterReading

0..1

+Customer 0..1

+UsagePoints

0..*

+UsagePoint

0..1

+PositionPoint

0..1

+ReadingQualities0..*

+Reading 0..1

+ServiceDeliveryPoints

0..*

+TariffProfile

0..1

+IntervalBlocks

0..*

+ReadingType 1

+ServiceSupplier

0..1

+ReadingTypes 0..*

+Readings0..*

+ReadingType

1

+MeterReadings

0..*

+ReadingType

0..1

+ServiceDeliveryPoints

0..*

+ServiceCategory0..1

+UsagePoints

0..*

+ServiceDeliveryPoints

0..*

+EnergyUsageInformation

0..*

+UsagePoints

0..*

+UsagePoint 1

+MeterReadings

0..*


Figure 2: Full Energy Usage Information Model
REQ.18.4.1 Energy Usage Information Model Details
The following sections contain the classes and attributes defined in the energy usage information model, along with their descriptions. Elements tagged with <<enumeration>> define the valid values for an enumerated data type and so do not have their own data type, and should be self explanatory, not requiring a definition. 
Terms in this section may be based upon IEC 61968 classes and their descriptions.  Some of the descriptions refer to other components of the IEC model (recognized as camel case terms) and these are not part of the energy usage information model or needed by the components in the energy usage information model.  To preserve accurate presentation of unaltered IEC classes, these terms have been retained.
REQ.18.4.1.1 AbsoluteDateTime  «Datatype»
Date and time as specified in ISO 8601. 
REQ.18.4.1.2

BaseCIM_CombinedVersion  MERGEFIELD Element.Stereotype
The combined version denotes the versions of the subpackages that have been combined into the total CIM model. This is a convenience instead of having to look at each subpackage.
	Name
	Type
	Description

	date
	AbsoluteDate
	Form is YYYY-MM-DD for example for January 5, 2009 it is 2009-01-05.




	version
	String
	MERGEFIELD Att.NotesForm is IEC61970CIMXXvYY_IEC61968CIMXXvYY_combined where XX is the major CIM package version and the YY is the minor version, and different packages could have different major and minor versions. For example IEC61970CIM13v18_IEC61968CIM10v16_combined.  Additional packages might be added in the future.


REQ.18.4.1.3

Boolean  «Primitive»
A type with the value space "true" and "false". 
REQ.18.4.1.4

Customer  MERGEFIELD Element.Stereotype
Organization receiving services from ServiceSupplier.
	Name
	Type
	Description

	name
	String
	The name is any free human readable and possibly non unique text naming the object.


REQ.18.4.1.5

CustomerAgreement  MERGEFIELD Element.Stereotype
Agreement between the customer and the Service Supplier to pay for service at a specific service location. It provides for the recording of certain billing information about the type of service provided at the service location and is used during charge creation to determine the type of service.
	Name
	Type
	Description

	name
	String
	The name is any free human readable and possibly non unique text naming the object.


REQ.18.4.1.6

CustomerAuthorisation  MERGEFIELD Element.Stereotype
Holds an authorisation for access to specific user-private data granted to a third party service provider. 
	Name
	Type
	Description

	name
	String
	The name is any free human readable and possibly non unique text naming the object.

	validityInterval
	DateTimeInterval
	Date and time interval this agreement is valid (from going into effect to termination).


REQ.18.4.1.7

DateTimeInterval  «Compound»
Interval of date and time.
	Name
	Type
	Description

	start
	AbsoluteDateTime
	Date and time that this interval started.

	end
	AbsoluteDateTime
	Date and time that this interval ended.

	duration
	Duration
	The duration of this interval, in seconds


REQ.18.4.1.8

Duration  «CIMDatatype»
An interval of time, specified in ISO 8601 compatible format.  
REQ.18.4.1.9

ElectricPowerQualitySummary  MERGEFIELD Element.Stereotype
MERGEFIELD Element.NotesA summary of power quality events. This information represents a summary of power quality information typically required by customer facility energy management systems. It is not intended to satisfy the detailed requirements of power quality monitoring. All values are as defined by measurementProtocol during the period. The standards typically also give ranges of allowed values; the information attributes are the raw measurements, not the "yes/no" determination by the various standards. See referenced standards for definition, measurement protocol  and period.

	Name
	Type
	Description

	flickerPlt
	Float
	A measurement of long term Rapid Voltage Change 

	flickerPst
	Integer
	A count of Rapid Voltage Change events during the summary interval period

	harmonicVoltage
	Float
	A measurement of the Harmonic Voltage during the period. For DC, distortion is with respect to a signal of zero Hz.

	longInterruptions
	Integer
	A count of Long Interruption events (as defined by measurementProtocol) during the summary interval period.

	mainsVoltage
	Float
	A measurement of the Mains [Signaling] Voltage during the summary interval period.

	measurementProtocol
	String
	A reference to the source used as the measurement protocol definition. e.g. "IEEE1519-2009", "EN50160"

	powerFrequency
	Float
	A measurement of the power frequency during the summary interval period.

	rapidVoltageChanges
	Integer
	A count of Rapid Voltage Change events during the summary interval period

	shortInterruptions
	Integer
	A count of Short Interruption events during the summary interval period 

	summaryInterval
	DateTimeInterval
	Interval of summary period

	supplyVoltageDips
	Integer
	A count of Supply Voltage Dip events during the summary interval period 

	supplyVoltageImbalance
	Integer
	A count of Supply Voltage Imbalance events during the summary interval period 

	supplyVoltageVariations
	Integer
	A count of Supply Voltage Variations during the summary interval period 

	tempOvervoltage
	Integer
	A count of Temporary Overvoltage events (as defined by measurementProtocol) during the summary interval period 


REQ.18.4.1.10

EndDeviceAsset  MERGEFIELD Element.Stereotype
MERGEFIELD Element.NotesAssetContainer that performs one or more end device functions. One type of EndDeviceAsset is a MeterAsset which can perform metering, load management, connect/disconnect, accounting functions, etc. Some EndDeviceAssets, such as ones monitoring and controlling air conditioner, refrigerator, pool pumps may be connected to a MeterAsset. All EndDeviceAssets may have communication capability defined by the associated ComFunction(s). An EndDeviceAsset may be owned by a consumer, a service provider, utility or otherwise.

There may be a related end device function that identifies a sensor or control point within a metering application or communications systems (e.g., water, gas, electricity).

Some devices may use an optical port that conforms to the ANSI C12.18 standard for communications.

	Name
	Type
	Description

	name
	String
	The name is any free human readable and possibly non unique text naming the object.


REQ.18.4.1.11

EnergyUsageInformation  MERGEFIELD Element.Stereotype
MERGEFIELD Element.NotesA collection of customer energy usage information. This class is a container, and has no attributes.   

REQ.18.4.1.12

Float  «Primitive»
A floating point number. The range is unspecified and not limited. 
REQ.18.4.1.13

IdentifiedObject  MERGEFIELD Element.Stereotype
This is a root class to provide common identification for all classes needing identification and naming attributes 
REQ.18.4.1.14

Integer  «Primitive»
An integer number. The range is unspecified and not limited. 
REQ.18.4.1.15

IntervalBlock  MERGEFIELD Element.Stereotype
Time sequence of Readings of the same ReadingType. 
REQ.18.4.1.16

IntervalReading  MERGEFIELD Element.Stereotype
Data captured over a specific interval of time. If not specified, the duration is the intervalLength of the associated ReadingType, where the full definition of the units of measure is located.
	Name
	Type
	Description

	cost
	Float
	The cost associated with this reading for this interval.

	interval
	DateTimeInterval
	The time interval associated with the reading. 

	value
	Float
	Value of this interval reading.


REQ.18.4.1.17

MeterReading  MERGEFIELD Element.Stereotype
Set of values obtained from the meter.
	Name
	Type
	Description

	name
	String
	The name is any free human readable and possibly non unique text naming the object.

	valuesInterval
	DateTimeInterval
	Date and time interval of the data items contained within this meter reading.


REQ.18.4.1.18

NAESB_EUI_Version  MERGEFIELD Element.Stereotype
This class contains the version of the NAESB model.
	Name
	Type
	Description

	date
	AbsoluteDate
	Form is YYYY-MM-DD for example for January 5, 2009 it is 2009-01-05.

	version
	String
	Form is naesb_eui_vXX.YY where XX is the major version and YY is the minor version.


REQ.18.4.1.19

Name  MERGEFIELD Element.Stereotype
MERGEFIELD Element.NotesThe Name class provides the means to define any number of human readable  names for an object. 

	Name
	Type
	Description

	name
	String
	Any free text that name the object.


REQ.18.4.1.20

NameType  MERGEFIELD Element.Stereotype
MERGEFIELD Element.NotesType of name. Possible values for attribute 'name' are implementation dependent but standard profiles may specify types. An enterprise may have multiple information technology systems each having its own local name for the same object, e.g. a planning system may have different names from an energy management system. An object may also have different names within the same IT system, e.g. localName and aliasName as defined in CIM version 14. Their definitions from CIM14 are

localName: A free text name local to a node in a naming hierarchy similar to a file directory structure. A power system related naming hierarchy may be: Substation, VoltageLevel, Equipment etc. Children of the same parent in such a hierarchy have names that typically are unique among them. The localName is a human readable name of the object. It is only used with objects organized in a naming hierarchy.
aliasName: A free text alternate name typically used in tabular reports where the column width is limited.

	Name
	Type
	Description

	name
	String
	Name of the name type.


REQ.18.4.1.21

NameTypeAuthority  MERGEFIELD Element.Stereotype
Authority responsible for creation and management of names of a given type; typically an organization or an enterprise system.
	Name
	Type
	Description

	name
	String
	Name of the name type authority.


REQ.18.4.1.22

PositionPoint  MERGEFIELD Element.Stereotype
Set of spatial coordinates that determine a point, defined in coordinate system "WGS 84" if not specified otherwise. Use a single position point instance to desribe a point-oriented location. Use a sequence of position points to describe a line-oriented object (physical location of non-point oriented objects like cables or lines), or area of an object (like a substation or a geographical zone - in this case, have first and last position point with the same values).
	Name
	Type
	Description

	xPosition
	String
	X axis position.

	yPosition
	String
	Y axis position.

	zPosition
	String
	(if applicable) Z axis position.


REQ.18.4.1.23

QualityOfReading  «enumeration»
List of codes indicating the quality of the reading
	Name
	Type
	Description

	estimated
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	forecast
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	mixed
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	raw
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	validated
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	normalizedForWeather
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	other
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes


REQ.18.4.1.24

Reading  MERGEFIELD Element.Stereotype
Specific value measured by a meter or other asset. Each Reading is associated with a specific ReadingType.
	Name
	Type
	Description

	cost
	Float
	Cost in a currency

	timeStamp
	AbsoluteDateTime
	The time when the value was last updated

	value
	Float
	Value of this reading.


REQ.18.4.1.25

ReadingDirection  «enumeration»
Direction of reading.

	Name
	Type
	Description

	forward
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	reverse
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	net
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	total
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes


REQ.18.4.1.26

ReadingKind  «enumeration»
Kind of reading.
	Name
	Type
	Description

	energy
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	power
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	demand
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	voltage
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	current
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	voltageAngle
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	currentAngle
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	phaseAngle
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	powerFactor
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	pressure
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	volume
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	date
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	time
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	frequency
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	other
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	carbon
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	carbonDioxide
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	currentAverage
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	currentRMS
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	currentTHD
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	distortionPower
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	HCH
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	methane
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	NOx
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	perfluorocarbons
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	phasorPower
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	quantityPowerQ45
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	quantityPowerQ60
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	SO2
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	sulfurHexafluoride
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	voltageAverage
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	voltageRMS
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	voltageTHD
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	relativeHumidity
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	volumetricFlow
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes


REQ.18.4.1.27

ReadingQuality  MERGEFIELD Element.Stereotype
Quality of a specific reading value or interval reading value. Note that more than one quality may be applicable to a given reading. Typically not used unless problems or unusual conditions occur (i.e., quality for each reading is assumed to be 'good' unless stated otherwise in associated reading quality).
	Name
	Type
	Description

	quality
	QualityOfReading
	Quality, to be specified if different than ReadingType.defaultQuality.


REQ.18.4.1.28

ReadingType  MERGEFIELD Element.Stereotype
Type of data conveyed by a specific Reading.
	Name
	Type
	Description

	defaultQuality
	QualityOfReading
	Characteristics of a data value conveyed by a specific Reading, which allow an application to understand how a specific Reading is to be interpreted.

	direction
	ReadingDirection
	Specifies the direction of flow of the measurement.

	intervalLength
	Duration
	(if incremental reading value) Length of increment interval.

	kind
	ReadingKind
	Kind of reading.

	multiplier
	UnitMultiplier
	Multiplier for 'unit'.

	name
	String
	The name is any free human readable and possibly non unique text naming the object.

	unit
	UnitSymbol
	Unit for the reading value.


REQ.18.4.1.29

Seconds  «Datatype»
Time, in seconds
	Name
	Type
	Description

	value
	Float
	Time, in seconds

	unit
	UnitSymbol
	MERGEFIELD Att.Notes

	multiplier
	UnitMultiplier
	MERGEFIELD Att.Notes


REQ.18.4.1.30

ServiceCategory  MERGEFIELD Element.Stereotype
Category of service provided to the customer.
	Name
	Type
	Description

	kind
	ServiceKind
	Kind of service.


REQ.18.4.1.31

ServiceDeliveryPoint  MERGEFIELD Element.Stereotype
Logical point on the network where the ownership of the service changes hands. It is one of potentially many service points within a service location, delivering service in accordance with a customer agreement. Used at the place where a meter may be installed.
	Name
	Type
	Description

	name
	String
	The name is any free human readable and possibly non unique text naming the object.


REQ.18.4.1.32

ServiceKind  «enumeration»
Kind of service.
	Name
	Type
	Description

	electricity
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	gas
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	water
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	time
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	heat
	MERGEFIELD Att.Datatype
	Includes hot water and steam MERGEFIELD Att.Notes

	refuse
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	sewerage
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	rates
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	tvLicence
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	internet
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	other
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	cold
	MERGEFIELD Att.Datatype
	Includes chilled water and ice


REQ.18.4.1.33

ServiceSupplier  MERGEFIELD Element.Stereotype
Organization that provides services to customers.
	Name
	Type
	Description

	kind
	SupplierKind
	Kind of supplier.

	name
	String
	The name is any free human readable and possibly non unique text naming the object.


REQ.18.4.1.34

String  «Primitive»
A string consisting of a sequence of 8 bit characters. The character encoding is UTF-8. The string length is unspecified and unlimited. 
REQ.18.4.1.35

SummaryMeasurement  MERGEFIELD Element.Stereotype
An aggregated summary measurement reading.
	Name
	Type
	Description

	multiplier
	UnitMultiplier
	The multiplier part of the unit of measure, e.g. "kilo" (k)

	timeStamp
	AbsoluteDateTime
	The date and time (if needed) of the summary measurement.

	unit
	UnitSymbol
	The units of the reading, e.g. "Wh"

	value
	Float
	The value of the summary measurement.


REQ.18.4.1.36

SummaryQuality  «enumeration»
List of codes indicating the quality of the summary.
	Name
	Type
	Description

	estimated
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	forecast
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	mixed
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	validated
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	raw
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	normalizedForWeather
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	other
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes


REQ.18.4.1.37

SupplierKind  «enumeration»
Kind of supplier.
	Name
	Type
	Description

	utility
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	retailer
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	other
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	district
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	intermediary
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	local
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes

	microgrid
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes


REQ.18.4.1.38

TariffProfile  MERGEFIELD Element.Stereotype
A schedule of charges; structure associated with Tariff that allows the definition of complex tariff structures such as step and time of use.
	Name
	Type
	Description

	name
	String
	The name is any free human readable and possibly non unique text naming the object.


REQ.18.4.1.39

UnitMultiplier  «enumeration»
The unit multipliers defined for the CIM
	Name
	Type
	Description

	p
	MERGEFIELD Att.Datatype
	Pico 10**-12

	n
	MERGEFIELD Att.Datatype
	Nano 10**-9

	micro
	MERGEFIELD Att.Datatype
	Micro 10**-6

	m
	MERGEFIELD Att.Datatype
	Milli 10**-3

	c
	MERGEFIELD Att.Datatype
	Centi 10**-2

	d
	MERGEFIELD Att.Datatype
	Deci 10**-1

	k
	MERGEFIELD Att.Datatype
	Kilo 10**3

	M
	MERGEFIELD Att.Datatype
	Mega 10**6

	G
	MERGEFIELD Att.Datatype
	Giga 10**9

	T
	MERGEFIELD Att.Datatype
	Tera 10**12

	none
	MERGEFIELD Att.Datatype
	MERGEFIELD Att.Notes


REQ.18.4.1.40

UnitSymbol  «enumeration»
The units defined for usage in the CIM
	Name
	Type
	Description

	VA
	MERGEFIELD Att.Datatype
	Apparent power in volt ampere

	W
	MERGEFIELD Att.Datatype
	Active power in watt

	VAr
	MERGEFIELD Att.Datatype
	Reactive power in volt ampere reactive

	VAh
	MERGEFIELD Att.Datatype
	Apparent energy in volt ampere hours

	Wh
	MERGEFIELD Att.Datatype
	Real energy in Watt hours

	VArh
	MERGEFIELD Att.Datatype
	Reactive energy in volt ampere reactive hours

	V
	MERGEFIELD Att.Datatype
	Voltage in volt

	ohm
	MERGEFIELD Att.Datatype
	Resistance in ohm

	A
	MERGEFIELD Att.Datatype
	Current in ampere

	F
	MERGEFIELD Att.Datatype
	Capacitance in farad

	H
	MERGEFIELD Att.Datatype
	Inductance in henry

	C
	MERGEFIELD Att.Datatype
	Relative temperature in degrees Celsius

	degC
	MERGEFIELD Att.Datatype
	Relative temperature in degrees Celsius. In the SI unit system the symbol is ºC. Electric charge is measured in coulomb that has the unit symbol C. To destinguish degree Celsius form coulomb the symbol used in the UML is degC. Reason for not using ºC is the special character º is difficult to manage in software.

	s
	MERGEFIELD Att.Datatype
	Time in seconds

	min
	MERGEFIELD Att.Datatype
	Time in minutes

	h
	MERGEFIELD Att.Datatype
	Time in hours

	deg
	MERGEFIELD Att.Datatype
	Plane angle in degrees

	rad
	MERGEFIELD Att.Datatype
	Plane angle in radians

	J
	MERGEFIELD Att.Datatype
	Energy in joule

	N
	MERGEFIELD Att.Datatype
	Force in newton

	S
	MERGEFIELD Att.Datatype
	Conductance in siemens

	none
	MERGEFIELD Att.Datatype
	Dimension less quantity, e.g. count, per unit, etc.

	Hz
	MERGEFIELD Att.Datatype
	Frequency in hertz

	g
	MERGEFIELD Att.Datatype
	Mass in gram

	Pa
	MERGEFIELD Att.Datatype
	Pressure in pascal (n/m2)

	m
	MERGEFIELD Att.Datatype
	Length in meter

	m2
	MERGEFIELD Att.Datatype
	Area in square meters

	m3
	MERGEFIELD Att.Datatype
	Volume in cubic meters

	thm
	MERGEFIELD Att.Datatype
	Energy, in therms

	m3/h
	MERGEFIELD Att.Datatype
	Volumetric flow, in cubic meters per hour

	ft3/h
	MERGEFIELD Att.Datatype
	Volumetric flow, in cubic feet per hour


REQ.18.4.1.41

UsagePoint  MERGEFIELD Element.Stereotype
Logical point on a network at which consumption or production is either physically measured (e.g. metered) or estimated (e.g. unmetered street lights).
	Name
	Type
	Description

	name
	String
	The name is any free human readable and possibly non unique text naming the object.

	Description
	String
	A human readable description of the object.


REQ.18.4.1.42

UsageSummary  MERGEFIELD Element.Stereotype
Summary of usage for a billing period
	Name
	Type
	Description

	billingPeriod
	DateTimeInterval
	The billing period to which the included measurements apply

	billLastPeriod
	Float
	The amount of the bill for the previous period

	billToDate
	Float
	The bill amount related to the billing period as of the date received

	costAdditionalLastPeriod
	Float
	Additional charges from the last billing period

	currency
	String
	The ISO 4217 code indicating the currency applicable to the bill amounts in the summary. See list at http://www.unece.org/cefact/recommendations/rec09/rec09_ecetrd203.pdf

	currentBillingPeriodOverAllConsumption
	SummaryMeasurement
	The total consumption for the billing period




	currentDayLastYearNetConsumption
	SummaryMeasurement
	The amount of energy consumed one year ago 




	currentDayNetConsumption
	SummaryMeasurement
	Net consumption for the current day (delivered - received)

	currentDayOverallConsumption
	SummaryMeasurement
	Overall energy consumption for the current day

	peakDemand
	SummaryMeasurement
	Peak demand recorded for the current period




	previousDayLastYearOverallConsumption
	SummaryMeasurement
	The amount of energy consumed on the previous day one year ago 




	previousDayNetConsumption
	SummaryMeasurement
	Net consumption for the previous day




	previousDayOverallConsumption
	SummaryMeasurement
	The total consumption for the previous day




	qualityOfReading
	QualityOfReading
	Indication of the quality of the summary readings

	ratchetDemand
	SummaryMeasurement
	The current ratchet demand value for the ratchet demand period

	ratchetDemandPeriod
	DateTimeInterval
	The period over which the ratchet demand applies


4.  SUPPORTING DOCUMENTATION
a. Description of Request:

b. Description of Recommendation:

c. Business Purpose:

d. Commentary/Rationale of Subcommittee(s)/Task Force(s):

NAESB Process for Standards Development - http://www.naesb.org//misc/naesb_process_for_standards_dev.doc 

� See e.g. EIS Alliance Use Cases v2 including UC-9, UC-11, UC-12, UC-14, UC-15, ZBHP_SE_MRD 3.1 "HAN Device Information Retrieval Request", and OpenADE use case “Publication”.


� See e.g. EIS Alliance Use Cases v2 including UC-3, UC-11, UC-14, UC-15, and OpenADE use case “Publication” 


� Needs refs. This is an addition to EIS and OpenADE.


� See e.g. EIS Alliance Use Cases v2 including UC-1, UC-2, UC-3, UC-8, UC-9.


� See e.g. EIS Alliance Use Cases v2 including UC-1, UC-2, UC-3


� This is but one of several views that might result from choices permitted in the context of the conformance paragraph.





3
REQ PAP 10 Business Practices and Information Model Recommendation
Revised and Adopted  by the REQ EC October 22, 2010

Page 1 of 27

[image: image4.wmf] 

[image: image5.wmf] 

_1075620371.doc
[image: image1.png]|

Lo







