Nomination, Scheduled Quantity – add Associated Receipt Contract and Associated Delivery Contract

Definition of “Associated Contract” (MA):

Associated contract that provides the rights or information needed to process a transaction with respect to a service requester’s contract.

Definition of “Downstream Contract Identifier” (BC):

This field identifies the contract of the party who is receiving the quantities from the service requester.

Condition:

For Nomination, Scheduled Quantities, Request for Confirmation and Confirmation Response – this field is used by transportation service providers who confirm by contract at custody transfer locations (e.g. pipeline to pipeline interconnects, pooling points, logical points.)

Definition of “Upstream Contract Identifier” (BC):

This field identifies the contract of the party who is supplying the quantities to the service requester.

Condition:

For Nomination -This field is used by transportation service providers who confirm by contract at custody transfer locations (e.g. pipeline to pipeline interconnects, pooling points, logical points.)

In certain circumstances, there may be the need for “Associated Receipt Contract” or “Associated Delivery Contract”.  For instance, in the case of a nomination for a mainline receipt, the Service Requester needs to identify the associated gathering contract on the TSP.  This information is not used for confirmation purposes (therefore does not fit the definition and condition of Up K) but instead provides information needed to process the transaction with respect to the location (not service requester contract and therefore does not meet the definition of Assoc K).

In addition, on one transaction, there may be an associated receipt contract and an associated delivery contract.  If the definition of Assoc K was to change to allow for “or at a location”, we would still need to be able to use Assoc K twice – once associated with the delivery and once associated with the receipt in order to process the transaction.

For those data elements that can be used more than once, a TSP should be able to display after the data element abbreviation an appropriate sub description/abbreviation on their EBB e.g.  Assoc K (Rec)

Nomination, Scheduled Quantity – Add Receipt Transaction Type and Delivery Transaction Type

Add data elements to accommodate discrete transaction types at a receipt location and at a delivery location.

If no new data elements add the following new codes to accommodate discrete transaction types associated with only receipt and only deliveries.  Similar to Reduction Reason, must be able to accommodate the sending of multiple TT.

Pool Receipt

Pool Delivery

Pool to Pool Receipt

Pool to Pool Delivery

Current Business Receipt

Current Business Delivery

Out of Cycle Receipt

Out of Cycle Delivery

Segmented Receipt

Segment Delivery

For those data elements that can be used more than once, a TSP should be able to display after the data element abbreviation an appropriate sub description/abbreviation on their EBB as well e.g.  TT (Rec) or TT (1)

Nomination, Scheduled Quantity – Add new data elements for Associated Receipt Location and Associated Delivery Location.

On Williston Basin’s system, when gas is scheduled on a secondary firm basis, the nomination line item must indicate which primary contract receipt/delivery points the secondary transaction is using MDRQ/MDDQ from in order to not exceed an individual primary contract path entitlement.  For example:

Contract FT-X  	Total Contract MDDQ = 10,000

Contract paths:  	Path 1 - Location A to Location B – 7000 MDDQ

Path 2 - Location C to Location D – 3000 MDDQ

Nomination

Cap Type	Assoc Rec	Receipt	Assoc De	Delivery	Qty

Primary 			Loc A				Loc B		5000

Secondary	Loc A		Loc F		Loc B		Loc G		1000

Primary			Loc C				Loc D		2000

Capacity Check

Path 1		Location A to Location B	7000

Less Primary			(5000)

Less Secondary		(1000)

Remaining Primary		1000

Path 2		Location C to Location D	3000

Less Primary			(2000)

Remaining Primary		1000

The capacity check will inform the shipper and TSP the amount of primary firm remaining on a contract path to be scheduled or capacity released in a subsequent cycle.

Scheduled Quantity – Add Receipt and Delivery Reduction Reason

Add data elements to accommodate discrete reduction reasons at a receipt location and at a delivery location.  Request is similar to what was requested under R99041.  R99041 ultimately instructed IR to send multiple reduction reasons.

If no new data elements add the following new codes to accommodate discrete reduction reasons associated with only receipt and only deliveries.

Quantity Reduced due to Bumping at Delivery Location

Quantity Reduced due to Bumping at Receipt Location

Storage Capacity Constraint at Injection Location

Storage Capacity Constraint at Withdrawal Location

Pipeline Maintenance – Emergency at Delivery Location

Pipeline Maintenance – Emergency at Receipt Location

Confirmation Response not Received at Delivery Location

Confirmation Response not Received at Receipt Location

Reduction due to Balancing at Receipt Location

Reduction due to Balancing at Delivery Location

For those data elements that can be used more than once, a TSP should be able to display after the data element abbreviation an appropriate sub description/abbreviation on their EBB as well e.g.  RR (Rec) without violating abbreviation standards.