Additions & Revisions to Existing Business Practice Standard WEQ-002

(OASIS S&CP)

Revisions to Existing Business Practice Standard WEQ-002

(OASIS S&CP)

Applicability

1) Eligible Transmission Customer including Affiliate, Assignee, Financially Obligated Transmission Customer (FOTC), Native Load Customer, Reseller, Seller, Transmission Customer, Network Customer, and 2) Transmission Provider including ISO, RTO

002-2.4	INTERNET TOOL REQUIREMENTS

Support for the following specific Internet tools is required, both for use over the public Internet as well as for any private connections between users and OASIS Nodes:

a. Browser Support

OASIS Nodes shall iensure that users running browsers commercially available and in common use shall have a fully functional user interface based on the Interface Requirements defined in Business Practice Standards WEQ-001-4WEQ-002-4 and WEQ-002-101.

3/13/12 accepted

Browser Characteristics (includes defined Business Practice Standards WEQ current versions):

Features and extensions (e.g. plug-ins), as supported by the latest generally available versions of both Netscape® and Internet Explorer® within 9 months of such generally available version

becoming commercially available. the top 3 commercial browsers, based on market share, are acceptable for use once adopted and incorporated in the generally available browsers of all top 3 commercial browsers.

3/13/12 accepted

b. HTMLBrowser-based Forms and Plugins

HTMLBrowser-based Forms shall be provided by the TSIPs to allow Transmission Customers to enter information to the OASIS Node. Generally available browser plugins may be used to provide browser-based forms.

3/13/12 accepted

002-3.3	ACCESS TO INFORMATION

c. Downloading Capability

Users shall be able to download from an OASIS Node the Transmission Service Information in electronic format as a file. The information required to be made available for downloading and rules for formatting of this downloaded data are described in Business Practice Standards WEQ-002-4.2 and WEQ-002-101.

e. Uploading Capability

Transmission Customers shall be able to programmatically upload to OASIS Nodes the same information as provided for in the filled-out on-line data entry forms. TSIPs shall ensure that programmatically uploaded forms are information is handled identically to forms filled out on-line. TSIPs shall provide forms templates that support the HTTP input of CSV records. This capability shall permit a Transmission Customer to upload CSV records using standard WWW browsers or additional client software to specify the location of the CSV records stored on the Transmission Customer's hard disk. The required templates and rules for formatting of this uploaded data are described in Business Practice Standards WEQ-002-4 and WEQ-002-101.

3/13/12 accepted

002-3.4	TRANSMISSION PROVIDER UPDATING REQUIREMENTS

c. OASIS Node Space for Reseller

To permit users to readily find Transmission Service Information for the transmission systems that they are interested in, TSIPs shall provide database space on their OASIS Node for all Resellers who have purchased, and who request to resell, transmission access rights for the power systems of the Transmission Providers supported by that OASIS Node. The right to resell transmission access right is not applicable to NITS.

3/13/12 accepted

d. Reseller Posting to Transmission Provider OASIS Node

The Resellers shall post the relevant Transmission Service Information on the OASIS Node associated with each Transmission Provider from whom the transmission access rights were originally purchased. The right to resell transmission access right is not applicable to NITS.

e. Reseller Posting Capabilities

The TSIPs shall ensure that the Resellers shall be able to post their Transmission Service Information to the appropriate OASIS Nodes using the same tools and capabilities as the Transmission Customers, meet the same performance criteria as the Transmission Providers, and allow users to view these postings on the same display page, using the same tables, as similar capacity being sold by the Transmission Providers. The right to resell transmission access right is not applicable to NITS.

h. Transaction Tracking by an Assignment Reference Number

All requests for purchase of transmission or ancillary services will be marked by a unique accounting number, called an Assignment Reference Number.

002-3.6	USER INTERACTION WITH AN OASIS NODE

There are three basic types of user interactions which must be supported by the OASIS Node. These interactions are defined in Business Practice Standard WEQ-002-4.3.

a. Query/Response

The simplest level of interactions is the query of posted information and the corresponding response. The user may determine the scope of the information queried by specifying values, through an HTMLbrowser-based form, a OASIS URL string, or an uploaded file, using Query Variables and their associated input values as defined with each OASIS Template in Business Practice Standard WEQ-002-4.3. The response will be either a HTMLbrowser-based display or a record oriented file, depending on the output format that the user requests. The TSIP may establish procedures to restrict the size of the response, if an overly broad query could result in a response that degrades the overall performance of the OASIS Node for their users.

3/13/12 accepted

b. Purchase Request

The second type of Transmission Customer interaction is the submittal of a request to purchase a service. The Transmission Customer completes an input form, a OASIS URL string or uploads a file and submits it to the OASIS Node. The uploaded file can either be a series of Query Variables or a record oriented file, as applicable and supported for the particular service(s) being requested. The Seller of the service, possibly off-line from the OASIS Node, processes the request and the status is updated accordingly. If the Seller approves the purchase request, then the Transmission Customer must again confirmed it. Once the Transmission Customer confirms an approved purchase, a reservation for those services is considered to exist, unless later the reservation is reassigned, displaced, or annulled or, if related to the arrangement of NITS, is later terminated or released.

3/13/12 accepted

c. Upload and Modify Postings

Transmission Customers who wish to resell their rights may upload a form, create the appropriate OASIS URL or upload a file to post services for sale. A similar process applies to eligible third party Sellers of ancillary services. The products are posted by the TSIP. The Seller may monitor the status of the services by requesting status information. Similarly the Seller may modify its posted Transmission Services by submitting a service modification request through a form, a OASIS URL query, or by uploading a file. The right to resell transmission access rights is not applicable to NITS.

3/13/12 accepted

002-4	GENERAL OASIS AND POINT-TO-POINT INTERFACE REQUIREMENTS

The following technical requirements apply to the posting and availability of information on OASIS related to general OASIS information and the arrangement of PTP Transmission and related Ancillary Services. The technical requirements for the arrangement of NITS are specified in Business Practice Standard WEQ-002-101.

3/13/12 accepted not accepted ptp change

002-4.2.1.3	Script Names

CGI scripts shall be located in the directory "data" as follows (case sensitive): http://(OASIS Node name)/OASIS/ (PRIMARY_PROVIDER_CODE) /data/(CGI script name)?(Query Variables)

Where:

(CGI script name) is the OASIS Template name in lower case (see section Business Practice Standard WEQ-002-4.3). Other CGI scripts may be defined as required to implement the HTML browser-based interface to the documented OASIS Templates.

(Query Variables) is a list of Query Variables with their settings formatted as defined by the HTTP protocol (i.e., OASIS URL encoded separated by ampersands).

Example:

To request the hourly scheduledetail OASIS Template at Transmission Provider WXYZ Co.

http://www.wxyz.com/OASIS/WXYZ/data/schedule ?templ=scheduledetail& ver=1.5& fmt=data &stime=19960412040000PD &sptime=19960412100000PD& pprov=wxyz

002-4.2.3	OASIS Template Constructs

002-4.2.3.1	OASIS Template Construction

Business Practice Standard WEQ-002-4.3 lists the set of OASIS Templates that shall be supported by all OASIS Nodes for accessing or posting of general OASIS and PTP information. These OASIS Templates are intended to be used precisely as shown for the transfer of data to/from OASIS Nodes, and identify, by Data Elements names, the information to be transferred. The construction of the OASIS Templates shall follow the rules described below:

3/13/12 accepted

b. Transfer Protocol

OASIS Templates are transferred using the HTTP protocol. OASIS Templates shall support both the "GET" and "POST" methods for transferring "query string" name/value pairs, as well as the OASIS specific CSV format for posting and retrieval of information from OASIS Nodes. HTML Browser-based screens and forms shall be implemented for each OASIS Template.

002-4.2.3.3	OASIS Template HTMLBrowser-based Screens

Though the exact form and content of the HTML browser-based screens and forms associated with the OASIS Templates are not dictated by this document, the following guidelines shall be adhered to for all HTMLbrowser-based screens and forms implemented on an OASIS Node:

3/13/12 accepted

a. Data Element Headings

Data displayed in any HTMLbrowser-based screen/form shall be labeled such that the associated data value(s) is(are) easily and readily identifiable as being associated with a particular OASIS Template Data Element. HTML "Hot-Links"Hyperlinks or other pointer mechanisms may be provided for Data Element headings in OASIS Templates which permit the user to access documentation describing the meaning, type, and format of the associated data.

3/13/12 accepted

b. Display Limitations

HTML Browser-based screens and forms shall be implemented in such a way to allow the display of all data specified for each OASIS Template. This may take the form of "wrapping" of lines of information on the screen, the use of horizontal and/or vertical scrolling, or the use of "Hot-Links" Hyperlinks or other pointer mechanisms. There is not necessarily a one-to-one relationship between HTMLbrowser-based screens implemented on OASIS Nodes, and their associated OASIS Template. However, all OASIS Template Data Elements shall be viewable through one or more HTML browser-based screens.

3/13/12 accepted

c. OASIS Template Navigation

HTML "Hot-Links" Hyperlinks or other pointer mechanisms may be provided to assist the navigation between screens/forms associated with related OASIS Templates.

002-4.2.4.1	Query Requirements

Query information is transferred to an OASIS Node using the HTTP protocol as a string of Query Variables in the form of name/value pairs. Query Variable name/value pairs are specified as a collection of encoded strings (e.g., blank characters replaced by plus (+) character, etc.) in the form of name=value, with each name/value pair separated by ampersands (&) (see section Business Practice Standard WEQ-002-4.2.6). OASIS Nodes shall support the following methods for users to input query information:

a. HTMLBrowser-Based Forms

HTMLBrowser-based FORM input and/or hypertext links shall be provided to allow users to specify OASIS Template Query Variables. This will be the easiest way to obtain information and should be the choice of most casual users and for simple requests. The exact nature and form of these HTML browser-based screens are not specified, and may differ between OASIS Nodes.

3/13/12 accepted Based needs to be lower case

002-4.2.4.2 	Response Requirements

In response to a validly formatted query for each query/response OASIS Template, the OASIS Node shall return the requested information in one of two forms based on the user specified OUTPUT_FORMAT Query Variable:

a. HTMLBrowser-Based

If the user requests the response to have the format of "HTML" (OUTPUT_FORMAT=HTML) then the response from the OASIS Node shall be a web page using the HTMLbrowser-based format. This shall be the default for all query/response OASIS Templates.

3/13/12 accepted Based needs to be lower case

002-4.2.5.1	Input Requirements

a. HTMLBrowser-Based Forms

HTMLBrowser-based FORM input shall be provided to allow users to specify the necessary input data associated with each input/response OASIS Template. This may be in the form of fill in blanks, buttons, pull-down selections, etc., and may use either the GET or POST methods. The exact nature and form of these HTML browser-based screens are not specified, and may differ between OASIS Nodes.

3/13/12 accepted Based needs to be lower case

002-4.2.8.2	Transmission Customer Company Information

OASIS Templates require that certain Transmission Customer company registration information be maintained. As an extension of the Transmission Customer company registration information of the host, domain and port identifiers for dynamic notification of changes in the Transmission Customer's purchase requests, a field should be added to the Transmission Customers company's registration information that would define/identify how notification would be delivered to that Transmission Customer company should a transmission or ancillary purchase request be directed to that Transmission Customer company as a Seller of a transmission or ancillary service. The pertinent information would be either a full HTTP protocol OASIS URL defining the protocol, host name, port, path, resource, etc. information or a "mailto:" OASIS URL with the appropriate mailbox address string. On receipt of any purchase request directed to that Transmission Customer company as SELLER via either the transrequest or ancrequest OASIS Templates, or on submission of any change in request information submitted to that Transmission Customer company as SELLER via either the transcust or anccust OASIS Templates, a notification message formatted as documented for the delivery of notification to the Transmission Customer, shall be formatted and directed to the Seller. At a minimum, OASIS Nodes shall maintain the following information for each Transmission Customer company:

a. Transmission Customer Company Code

4 character code for Transmission providers; 6 character code for Eligible Transmission Customers in accordance with Version 1.8.1 Electronic Tagging Functional Specifications requirements shall be maintained for each Transmission Customer company.

b. Default Contact

Unless specified for each individual user Affiliated with the Transmission Customer company, default contact information consisting of a phone number, fax number, and e-mail address shall be maintained for each Transmission Customer company.

c. Transmission Provider Affiliation

Each Eligible Transmission Customer shall be obligated to identify to the TSIP any affiliation with a Transmission Provider whose "OASIS Home Page" is on that OASIS Node.

d. Notification OASIS URL

For Transmission Customer companies using the OASIS URL notification mechanism for delivery of messages on each change of ancillary/transmission reservation STATUS, each Transmission Customer company shall provide the IP host name and port number to be used in delivering notification messages. OASIS Nodes shall have the right to refuse support for notification to any IP ports other than port 80.

002-4.2.10	Transaction Process

OASIS shall implement OASIS Templates that allow Transmission Customers and Sellers to enter, modify and consummate arrangements for PTP transmission and ancillary services. In addition to the OASIS Template interface defined by these Business Practice Standards WEQ-002-4.2.10.1 through WEQ-002-4.2.10.4, OASIS shall also provide a browser-based user interface that implements the same basic functionality provided by the OASIS Template interface through one or more displays or forms.

3/13/12 accepted

The OASIS PTP transaction process is controlled through the transaction REQUEST_TYPE, identifying the type of transaction being conducted, and STATUS, indicating the request’s current state in the transaction process. The Business Practice Standard WEQ-013-2 describes in detail the transaction process that must be implemented on OASIS for PTP and related ancillary service arrangements. The OASIS Implementation Guide also provides specific requirements and recommendations related to the handling of particular requests from both a technical and business process perspective.

3/13/12 accepted

002-4.2.10.1	Request Types

Transmission Customers must submit requests for PTP transmission and ancillary services to OASIS using one of the valid enumerated values in the REQUEST_TYPE Data Element. The valid values for REQUEST_TYPE are defined in the Business Practice Standard WEQ-003. Each REQUEST_TYPE is also defined and its use in the OASIS transaction process in the Business Practice Standard WEQ-013-2.1. The Implementation Guide also describes request type specific requirements.

3/13/12 accepted

002-4.2.10.3	Dynamic Notification

Transmission Customers may specify the delivery of dynamic notification messages on each change in STATUS or any other Data Element(s) associated with an ancillary, or PTP Transmission Service reservation. OASIS Nodes shall support the delivery of dynamic notification messages through either the HTTP protocol or by electronic mail. The selection of which mechanism is used and the contents of the messages delivered to the client program or e-mail address is defined by the content of the STATUS_NOTIFICATION Data Element as described in the next subsections. Regardless of whether this dynamic notification method is used or not, it shall still remain the user's responsibility to get the desired information, possibly through the use of a periodic "integrity request". OASIS Nodes shall not be obligated or liable to guarantee delivery/receipt of messages via the STATUS_NOTIFICATION mechanism other than on a "best effort" basis. As an extension of the Transmission Customer company registration information of the host, domain and port identifiers for dynamic notification of changes in the Transmission Customer's purchase requests, a field should be added to the Transmission Customer company's registration information that would define/identify how notification would be delivered to that Transmission Customer company should a PTP transmission or ancillary purchase request be directed to that Transmission Customer company as a Seller of a transmission or ancillary service. The pertinent information would be either a full HTTP protocol OASIS URL defining the protocol, host name, port, path, resource, etc. information or a "mailto:" OASIS URL with the appropriate mailbox address string. On receipt of any purchase request directed to that Transmission Customer Company as SELLER via either the transrequest or ancrequest OASIS Templates, or on submission of any change in request STATUS (or any other Data Elements associated with the request) to that Transmission Customer company as SELLER via either the transcust or anccust OASIS Templates, a notification message formatted as documented for the delivery of notification to the Transmission Customer, shall be formatted and directed to the Seller. This extension of dynamic notification is required only where the Transmission Provider has programmed its computer system for its own notification.

002-4.2.10.4	Use of Comments

PTP Transmission and ancillary service reservation OASIS templates support the following text Data Elements to be used to communicate information between parties (i.e., Transmission Provider, Reseller, and Transmission Customer) to a transaction:

- PRIMARY_PROVIDER_COMMENTS - for information to be communicated by the Transmission Provider to all other parties

- SELLER_COMMENTS - for information to be communicated by the Seller (either Transmission Provider or Reseller) to the Transmission Customer

- CUSTOMER_COMMENTS - for information to be communicated by the Transmission Customer to the Seller

- STATUS_COMMENTS - for information to be communicated by any party to all other parties

Use of these comments fields is at the discretion of the parties to the transaction with the exception that Sellers of services must indicate via SELLER_COMMENTS the reason for denial of any request for service (STATUS values of INVALID, REFUSED, or DENIED). Transactions which are subject to displacement, either before or after confirmation (STATUS values of SUPERSEDED or DISPLACED), shall also include a reference to the competing reservation request that initiated the displacement in the SELLER_COMMENTS.

002-4.2.12	Linking of Ancillary Services to Point-To-Point Transmission Services

3/13/12 not accepted

The requirements related to ancillary services are shown in transoffering (and updated using transupdate) using the ANC_SVC_REQ Data Element. The value must conform to the following syntax. The concept is to enter zero or more unique ancillary service requirements.

[ [y:x] 0 [;y:x] n ]

The letter y represents the code for the ancillary service per the OASIS Data Dictionary allowed values for AS_TYPE.

The letter x represents the code for requirements as follows:

- M - Mandatory, which implies that the Transmission Provider must provide the ancillary service

- R - Required, which implies that the ancillary service is required, but not necessarily from the Transmission Provider

- O - Optional, which implies that the ancillary service is not necessarily required, but could be provided

- U - Unknown, which implies that the requirements for the ancillary service are not known at this time

The letter n represents the maximum occurrences which shall not exceed the number of allowed values for AS_TYPE.

Ancillary services may be requested by a user from the Seller at the same time as Transmission Services are requested via the transrequest OASIS Template Data Element ANC_SVC_LINK. The value must conform to the following syntax. The concept is to enter zero or more unique ancillary service request as indicated by the ANC_SVC_REQ value as being Mandatory or Required.

[ [y:(AA[:xxx[:yyy[:nnn]]])] 0 [;y:(AA[:xxx[:yyy[:nnn]]])] n ]

The letter y represents the code for the ancillary service per the OASIS Data Dictionary allowed values for AS_TYPE.

The AA is the appropriate PRIMARY_PROVIDER_CODE, SELLER_CODE, or CUSTOMER_CODE, and represents the Transmission Customer company providing the ancillary services. "AA" may be unspecified for "xxx" type identical to "FT", in which case the ":" character must be present and precede the "FT" type. If multiple "AA" terms are necessary, then each "AA" grouping will be enclosed within parenthesis, with the overall group subordinate to the AS_TYPE specified within parenthesis.

The xxx represents either:

- "FT" to indicate that the Transmission Customer will determine ancillary services at a future time, or

- "SP" to indicate that the Transmission Customer will self-provide the ancillary services, or

- "RQ" to indicate that the Transmission Customer is asking the OASIS Node to initiate the process for making an ancillary services reservation with the indicated Transmission Provider or Reseller on behalf of the Transmission Customer. The Transmission Customer must then continue the reservation process with the Transmission Provider or Reseller. If the Transmission Services request is for pre-confirmed service, then the ancillary services shall also be pre-confirmed, or

- "AR" to indicate an Assignment Reference Number sequence follows.

The terms "yyy" and "nnn" are subordinate to the xxx type of "AR". The yyy represents the ancillary services reservation number (ASSIGMNENT_REF) and nnn represents the capacity of the reserved ancillary services. Square brackets are used to indicate optional elements and are not used in the actual linkage itself. Specifically, the :yyy is applicable to only the "AR" term and the :nnn may optionally be left off if the capacity of ancillary services is the same as for the Transmission Services, and optionally multiple ancillary reservations may be indicated by additional (xxx[:yyy[:nnn]]) enclosed within parenthesis.

If no capacity amount is indicated, the required capacity is assumed to come from the ancillary reservations in the order indicated in the codes, on an "as-needed" basis.

Examples for the handling of ancillary service linkage to Transmission Service requests/reservations are presented in Business Practice Standard WEQ-013-4.2.

002-4.3	GENERAL OASIS AND POINT-TO-POINT TEMPLATE DESCRIPTIONS

Unless otherwise specifically indicated, Business Practice Standard WEQ-002-4.3, including all subsections, shall only apply to PTP transmission and associated ancillary services. NITS OASIS Template requirements are specified in Business Practice Standard WEQ-002-101.

The following OASIS Templates define the Data Elements in fixed number and sequence which must be provided for all data transfers to and from the OASIS Nodes. The definitions of the Data Elements are listed in Business Practice Standard WEQ-003. TSIPs must provide a more detailed supplemental definition of the list of Sellers, Posted Paths, POR, POD, capacity types, ancillary service types and OASIS Templates online, clarifying how the terms are being used (see Business Practice Standard WEQ-00204.3.5.1, list ). If POR and POD are not used, then Posted Path must include directionality. Many of the OASIS Templates represent query/response interactions between the User and the OASIS Node. These interactions are indicated by the "query" and "response" section respectively of each OASIS Template. Some, as noted in their descriptions, are input information, sent from the user to the OASIS Node. The response is generally a mirror of the input, although in some OASIS Templates, the TSIP must add some information.

3/13/12 accepted spaces not accepted ptp

002-4.3.1	OASIS Template Summary

The following table provides a summary of the process areas, and OASIS Templates to be used by users to query information that will be downloaded or to upload information to the Transmission Provider’s OASIS. These processes define the functions that must be supported by an OASIS Node related to the General OASIS, PTP transmission and related ancillary service information.

| Process Area | Process Name |  | OASIS Template(s) |
| --- | --- | --- | --- |
| 4.3.2 Query/Response of Posted Services Being Offered | Transmission Capacity Offerings Available for Purchase (query/response) |  | transoffering |
|  | Ancillary Services Available for Purchase (query/response) |  | ancoffering |
| 4.3.3 Query/Response of Services Information | Transmission Services (query/response) |  | transserv |
|  | Ancillary Services (query/response) |  | ancserv |
| 4.3.4 Query/Response of Schedule details and Curtailments, Security Events, Reductions, and System Data | Transmission Schedule (query/response) |  | scheduledetail/1/ |
|  | Security Event (query/response) |  | security/1/ |
|  | Transmission Reservation Reduction (query/response) |  | reduction/1/ |
|  | System Data (query/response) |  | systemdata/1/ |
| 4.3.5 Query/Response of Lists of Information | List (query/response) |  | list/1/ |
| 4.3.6 Purchase Transmission Services | Transmission Customer Capacity Purchase Request (input/response) |  | transrequest |
|  | Status of Transmission Customer Purchase Request/Reservation (query/response) |  | transstatus |
|  | Renewal Provisions (query/response) |  | rollover |
|  | CCO Provisions (query/response) |  | cco |
|  | Coordinated Group Status (query/response) |  | cgstatus |
|  | Seller Approval of Purchase (input/response) |  | transsell |
|  | Transmission Customer Confirmation of Purchase (input/response) |  | transcust |
|  | Coordinated Group Update (input/response) |  | cgupdate |
|  | Seller to Reassign Service Rights to Another Transmission Customer (input/response) |  | transassign |
| 4.3.7 Seller Posting of Transmission Service | Seller Capacity Posting (Input) |  | transpost |
|  | Seller Capacity Modify (input/response) |  | transupdate |
| 4.3.8 Purchase of Ancillary Services | Transmission Customer Requests to Purchase Ancillary Services (input/response) |  | ancrequest |
|  | Ancillary Services Status (query/response) |  | ancstatus |
|  | Seller Approves Ancillary Service (input/response) |  | ancsell |
|  | Transmission Customer a Ancillary Service (input/response) |  | anccust |
|  | Seller to Reassign Service Rights to Another Transmission Customer (input/response) |  | ancassign |
| 4.3.9 Seller Posting of Ancillary Services | Seller Ancillary Services Posting (input/response) |  | ancpost |
|  | Seller Modify Ancillary Services Posting (input/response) |  | ancupdate |
| 4.3.10 Informal Messages | Transmission Provider/Transmission Customer Want Ads and Informal Message Posting Request (input/response) |  | messagepost/1/ |
|  | Message (query/response) |  | message/1/ |
|  | Transmission Provider/Reseller Message Delete Request (input/response) |  | messagedelete/1/ |
|  | Personnel Transfers (query/response) |  | personnel/1/ (optional template to be implemented at Transmission Provider’s discretion) |
|  | Discretion (query/response) |  | discretion/1/ (optional template to be implemented at Transmission Provider’s discretion) |
|  | Standards of Conduct (query/response) |  | stdconduct/1/ (optional template to be implemented at Transmission Provider’s discretion) |
| 4.3.11 OASIS Audit Log | OASIS Audit Log (query/response) |  | (various) /1/ |

/1/ These templates provide access to general OASIS information postings applicable to both PTP and NITS transactions on OASIS.

3/13/12 accepted

002-4.3.6.2 	Status of Transmission Customer Purchase Request/Reservation (transstatus)

The Status of Transmission Customer Purchase Request/Reservation (transstatus) is provided upon the request of any Transmission Customer or Transmission Provider to indicate the current status of one or more reservation records. Users may also view any transaction's status. However, the SOURCE and SINK may be masked for user requests until Transmission Providers must make source and sink information available at the time the request status posting is updated to show that a transmission request is confirmed.

Continuation records may be returned in association with a transmission reservation to convey information regarding: 1) sale or assignment of transmission rights on the secondary market (reassignments), 2) profiled requests, or 3) service over multiple Posted Paths. Each continuation record associated with a transmission reservation shall be identified by the CONTINUATION_FLAG Data Element set to 'Y' and include the ASSIGNMENT_REF Data Element.

When a transmission reservation request acquires its rights to Transmission Service as the result of a sale or assignment of transmission rights on the secondary market, the identity of the original reservation, capacity, and time interval over which rights are assigned to the new reservation are defined by the Data Elements REASSIGNED_REF, REASSIGNED_CAPACITY, REASSIGNED_START_TIME, and REASSIGNED_STOP_TIME. These Data Elements will be returned in continuation records when more than one set of reassignment information is associated with a reservation.

If the transmission reservation has an associated profile, either as a result of the Transmission Customer’s submission of CAPACITY_REQUESTED and/or BID_PRICE varying over time (support for Transmission Customer reservation profiles is at the discretion of the Transmission Provider or the Seller’s offering of Partial Service or negotiation of price, the set of Data Elements for START_TIME, STOP_TIME, CAPACITY_REQUESTED, BID_PRICE, CAPACITY_GRANTED, OFFER_PRICE and CEILING_PRICE will be returned in continuation records for each segment of the request/reservation profile.

If the Transmission Provider supports reservations submitted on multiple Posted Paths, continuation records specifying PATH_NAME, POINT_OF_RECEIPT, and POINT_OF_DELIVERY associated with the reservation would be returned in continuation records.

The AFFILIATE_FLAG will be set by the TSIP to indicate whether or not the Transmission Customer is an Affiliate of the Transmission Provider. The NEGOTIATED_PRICE_FLAG will be set by the TSIP to indicate whether the OFFER_PRICE is higher, lower, or the same as the BID_PRICE. Any time that a confirmed transmission reservation's rights to schedule up to the amount of CAPACITY_GRANTED is reduced, either due to secondary market sales, partial displacements, Transmission Provider initiated "recalls" of capacity, etc., the IMPACTED Data Element shall be incremented. Specific information regarding the MW level and reason for reduction in reserved capacity is viewable using the reduction OASIS Template.

The PRIMARY_PROVIDER_APPROVAL Data Element is set by the Transmission Provider to indicate their approval of PART_TRANSFER and FULL_TRANSFER requests. In all other types of transmission requests, this element will be null.

The PRIMARY_PROVIDER_PROVISIONS Data Element is set to ‘Y’ by the Transmission Provider when certain provisions are imposed on the reservation that may be viewed by issuing a query for the rollover and cco OASIS Templates, otherwise null shall be used.

Specification of a value of PROPOSED or ATTESTED in the CG_STATUS field indicates that the Transmission Customer is associating the requested service with a Coordinated Group for coordination of service across multiple Transmission Providers, i.e., a Coordinated Request. When initially submitted with a CG_STATUS of PROPOSED, the CG_DEADLINE data element shall be set to a value 24 hours after the request’s TIME_QUEUED.

3/13/12 accepted

Requests for additional PTP Transmission Service that are to be evaluated concomitantly with the termination of a DNR are designated with the CONCOMITANT_EVAL_FLAG data element set to ‘Y’. All other requests have this data element set to ‘N’.

3/13/12 accepted

Template: transstatus

- Query

SELLER_CODE*

SELLER_DUNS*

CUSTOMER_CODE*

CUSTOMER_DUNS*

PATH_NAME*

POINT_OF_RECEIPT*

POINT_OF_DELIVERY*

SERVICE_INCREMENT*

TS_CLASS*

TS_TYPE*

TS_PERIOD*

TS_WINDOW*

TS_SUBCLASS*

STATUS*

CG_STATUS*

START_TIME (Beginning time of service)

STOP_TIME

START_TIME_QUEUED (Beginning time queue)

STOP_TIME_QUEUED

NEGOTIATED_PRICE_FLAG

ASSIGNMENT_REF

REASSIGNED_REF

RELATED_REF

SALE_REF

REQUEST_REF

DEAL_REF

COMPETING_REQUEST_FLAG

TIME_OF_LAST_UPDATE

- Response

CONTINUATION_FLAG

ASSIGNMENT_REF

SELLER_CODE

SELLER_DUNS

CUSTOMER_CODE

CUSTOMER_DUNS

AFFILIATE_FLAG (Set by TSIP)

PATH_NAME

POINT_OF_RECEIPT

POINT_OF_DELIVERY

SOURCE

SINK

CAPACITY_REQUESTED

CAPACITY_GRANTED

SERVICE_INCREMENT

TS_CLASS

TS_TYPE

TS_PERIOD

TS_WINDOW

TS_SUBCLASS

NERC_CURTAILMENT_PRIORITY

OTHER_CURTAILMENT_PRIORITY

START_TIME

STOP_TIME

CEILING_PRICE

OFFER_PRICE

BID_PRICE

PRICE_UNITS

PRECONFIRMED

ANC_SVC_LINK

ANC_SVC_REQ

POSTING_REF

SALE_REF

REQUEST_REF

DEAL_REF

IMPACTED (Greater than 0, if another reservation impacts this

reservation)

COMPETING_REQUEST_FLAG

REQUEST_TYPE

RELATED_REF

NEGOTIATED_PRICE_FLAG ("L" if Seller accepted Price is lower than

OFFER_PRICE in transoffering Template; "H" if higher; otherwise

blank)

STATUS

STATUS_NOTIFICATION

STATUS_COMMENTS

TIME_QUEUED

RESPONSE_TIME_LIMIT

TIME_OF_LAST_UPDATE

PRIMARY_PROVIDER_COMMENTS

SELLER_REF

SELLER_COMMENTS

CUSTOMER_COMMENTS

SELLER_NAME

SELLER_PHONE

SELLER_FAX

SELLER_EMAIL

CUSTOMER_NAME

CUSTOMER_PHONE

CUSTOMER_FAX

CUSTOMER_EMAIL

REASSIGNED_REF

REASSIGNED_CAPACITY (Capacity from each previous transaction)

REASSIGNED_START_TIME

REASSIGNED_STOP_TIME

PRIMARY_PROVIDER_APPROVAL

PRIMARY_PROVIDER_PROVISIONS

ROLLOVER_WAVED

3/13/12 accepted

CG_STATUS

CG_DEADLINE

CONCOMITANT_EVAL_FLAG

002-4.3.11	OASIS Audit Log

The OASIS audit log report facility shall be implemented by the definition of the following OASIS Templates:

transofferingaudit		 - audit counterpart to transoffering

ancofferingaudit		 - audit counterpart to ancoffering

scheduledetailaudit	 - audit counterpart to scheduledetail

securityaudit		 - audit counterpart to security

systemdataaudit		 - audit counterpart to systemdata

transstatusaudit		 - audit counterpart to transstatus

cgstatusaudit		 - audit counterpart to cgstatus

cgupdateaudit		 - audit counterpart to cgupdate

ancstatusaudit		 - audit counterpart to ancstatus

personnelaudit		 - audit counterpart to personnel

discretionaudit		 - audit counterpart to discretion

stdconductaudit		 - audit counterpart to stdconduct

- 002-5.10	IMPLEMENTATION PLAN

- All OASIS Template interactions with OASIS under the latest set of ratified NAESB Business Practice Standards as set forth in the Business Practice Standard WEQ-002 are to use the following version identifier:

- VERSION=2.0

- Until adopted as regulation, OASIS Nodes shall be required to respond to OASIS Template queries and uploads in the VERSION=1.6 format for backward compatibility.

- OASIS implementations electing to support the NAESB Business Practice Standards prior to adoption as regulation must respond correctly to OASIS Template queries and uploads in both the VERSION=1.6 format and VERSION=2.0 format.

- Once adopted as regulation, OASIS Nodes shall be required to support the VERSION 2.0 OASIS Template format within two year of such adoption, and shall maintain support for VERSION 1.6 queries and uploads for the full two year implementation period.

- 3/13/12 not accepted subcommittee decided on 18 months

- Transmission Providers may maintain support for VERSION 1.6 queries and uploads as required to meet regulatory data retention requirements or may elect to convert historical data to be compliant with the VERSION 2.0 OASIS Template for data retention purposes.

002-5.10.1	OASIS DATA ELEMENT CONVERSION AND CROSS REFERENCE

3/13/12 accepted

The Transmission Provider shall have the responsibility for documenting all existing NITS Customer’s arrangements on OASIS using the new 2.0 template structures of the S&CP such that the following information maybe queried for all executed NITS agreements. All pending requests that are in progress may continue to be evaluated off OASIS but must be documented in a similar fashion after they have been executed. The NITS OASIS Data Templates that will be populated are as follows:

3/13/12 accepted

NITSService

NITSCustomer

NITSLoad

NITSGeneration (as applicable)

NITSResource

NITSDNRList

NITSSchedulingRights (optional)

Any existing NITS arrangements that have been confirmed and are active beyond the S&CP VERSION 2.0 conversion date using the PTP Transmission Service reservation templates must be converted to the above NITS OASIS Template structures and the reservations will no longer be active in the PTP application.

Any pending NITS requests that have been submitted prior to the S&CP VERSION 2.0 conversion date using the PTP Transmission Service reservation OASIS Templates may be completed using these OASIS Templates, but must be converted to the above NITS OASIS Template structures upon confirmation.

No NITS request shall be permitted using the PTP Transmission Service reservation templates after the S&CP VERSION 2.0 conversion date.

The following table documents the mapping of Data Element changes from OASIS S&CP VERSION 1.6 to VERSION 2.0.

| Template | Deleted Data Element | New Data Element | Conversion Requirements |
| --- | --- | --- | --- |
| transrequest (input/response) |  | CG_STATUS | No data to convert. |
| transstatus (query) |  | CG_STATUS | No data to convert. |
| transstatus (query) |  | CG_DEADLINE | No data to convert. |
| transstatus (query) |  | CONCOMITANT_EVAL_FLAG | No data to convert. |
| transcust (input/response) |  | CG_STATUS | No data to convert. |
| cgstatus (query) |  | CONTINUATION_FLAG | No data to convert. |
| cgstatus (query) |  | ASSIGNMENT_REF | No data to convert. |
| cgstatus (query) |  | CR_PRIMARY_PROVIDER_CODE | No data to convert. |
| cgstatus (query) |  | CR_ASSIGNMENT_REF | No data to convert. |
| cgstatus (query) |  | CR_SERVICE_INCREMENT | No data to convert. |
| cgstatus (query) |  | CR_TS_CLASS | No data to convert. |
| cgstatus (query) |  | CR_TS_TYPE | No data to convert. |
| cgstatus (query) |  | CR_DISPOSITION | No data to convert. |
| cgstatus (query) |  | CR_DISPOSITION_TIME | No data to convert. |
| cgstatus (query) |  | TIME_OF_LAST_UPDATE | No data to convert. |
| cgupdate (input/response) |  | CONTINUATION_FLAG | No data to convert. |
| cgupdate (input/response) |  | ASSIGNMENT_REF | No data to convert. |
| cgupdate (input/response) |  | CR_PRIMARY_PROVIDER_CODE | No data to convert. |
| cgupdate (input/response) |  | CR_ASSIGNMENT_REF | No data to convert. |
| cgupdate (input/response) |  | CR_SERVICE_INCREMENT | No data to convert. |
| cgupdate (input/response) |  | CR_TS_CLASS | No data to convert. |
| cgupdate (input/response) |  | CR_TS_TYPE | No data to convert. |
| cgupdate (input/response) |  | CR_DISPOSITION | No data to convert. |
| cgupdate (input/response) |  | CR_DISPOSITION_TIME | No data to convert. |

Additions to Existing Business Practice Standard WEQ-002

(OASIS S&CP)

002-101	NETWORK INTEGRATION TRANSMISSION SERVICE INTERFACE REQUIREMENTS

3/14/12 bold accepted not accepted nits spelled out

The following technical requirements apply to the posting and availability of information on OASIS related to the arrangement of NITS. The organization of this section of Business Practice Standard WEQ-002 is intentionally aligned with Business Practice Standard WEQ-002-4, general OASIS and PTP interface requirements, to facilitate comparisons between the requirements for these two OASIS interfaces.

3/14/12 accepted

002-101.1	Information Model Concepts

3/14/12 accepted

a. ASCII-Based NITS OASIS Templates

For providing NITS information to users, TSIPs shall use the specified NITS OASIS Templates. These NITS OASIS Templates define the information that must be presented to users, both in the form of graphical displays and as downloaded files. Users shall be able to request NITS OASIS Template information using query/response data flows. The NITS OASIS Templates are described in Business Practice Standards WEQ-002-101.3 through WEQ-002-101.5. The OASIS Data Dictionary, which defines the Data Elements in the OASIS Templates, is provided in Business Practice Standard WEQ-003. Data Elements must be used in the exact sequence and number as shown in the NITS OASIS Templates when file uploads and downloads are used. Although the contents of the graphical displays are precisely defined as the same information as required in the NITS OASIS Templates, the actual graphical display formats of the Transmission Service Information are beyond the scope of the OASIS standards requirements. Due to the nature of graphical displays, there may be more than one graphical display used to convey the information associated with a given NITS OASIS Template.

b. ASCII-Based NITS OASIS File Structures

For uploading requests from and downloading information to users, TSIPs shall use specific file structures that are defined for NITS OASIS Template information as specified in Business Practice Standard WEQ-002-101.2. These file structures are based on the use of combinations of template header and data records to indicate the action to be taken and the information associated with each request related to the arrangement of NITS on OASIS. The supported NITS OASIS Templates and Data Elements are defined in Business Practice Standards WEQ-002-101.3 through WEQ-002-101.5. The file structure used for NITS OASIS Template information shall provide for the capability to include multiple templates within a given file exchanged to/from OASIS, and to bind multiple templates into an autonomous transaction that will be treated as a single request.

3/14/12 accepted

002-101.2	Network Integration Transmission Service OASIS NODE CONVENTIONS AND STRUCTURES

3/14/12 bold accepted not accepted nits

002-101.2.1 	OASIS Node Naming Requirements

All OASIS Nodes shall comply with Business Practice Standard WEQ-002-4.2.1 in implementing NITS on OASIS.

002-101.2.1.1 	OASIS Node Names

3/14/12 accepted

All OASIS Nodes shall comply with Business Practice Standard WEQ-002-4.2.1.1 in implementing NITS on OASIS.

002-101.2.1.2 	OASIS Node and Transmission Provider Home Directory

3/14/12 accepted

All OASIS Nodes shall comply with Business Practice Standard WEQ-002-4.2.1.2 in implementing NITS on OASIS.

002-101.2.1.3 	Script Names

All OASIS Nodes shall comply with Business Practice Standard WEQ-002-4.2.1.3 in implementing NITS on OASIS. The NITS OASIS Template name as specified in the (CGI script name) field of the Transmission Provider’s OASIS URL shall designate one of the defined NITS OASIS INPUT or QUERY Template names as specified in Business Practice Standards WEQ-002-101.3 and WEQ-002-101.4.

3/14/12 accepted

002-101.2.2 	Data Element Dictionary

All OASIS Nodes shall comply with Business Practice Standard WEQ-002-4.2.2 in implementing NITS on OASIS.

002-101.2.3	Network Integration Transmission Service OASIS Template Constructs

3/14/12 not accepted

002-101.2.3.1	Network Integration Transmission Service OASIS Template Construction

3/14/12 bold accepted not accepted nits

Business Practice Standard WEQ-002-101.3 lists the set of NITS OASIS Templates that shall be supported by all OASIS Nodes. These NITS OASIS Templates are intended to be used precisely as shown for the transfer of data to/from OASIS Nodes, and identify, by Data Elements names, the information to be transferred. The construction of the NITS OASIS Templates shall follow the rules described below:

- Unique NITS OASIS Template Name

- 3/14/12 accepted

Each type of NITS OASIS Template shall be identified with a unique name which shall be displayed to the user whenever the NITS OASIS Template is accessed.

- Transfer Protocol

- 3/14/12 accepted

NITS OASIS Templates are transferred using the HTTP protocol.

Query/response (QUERY) NITS OASIS Templates shall support both the HTTP "GET" and "POST" methods for transferring a "query string" of name/value pairs to OASIS. The OASIS response returned in association with a query/response NITS OASIS Template invocation may be specified to be in browser-based format or as a NITS CSV Format structured record stream.

3/14/12 accepted

Input/response (INPUT) NITS OASIS Templates shall only support the “POST” method for delivery of a properly formatted NITS OASIS CSV Format structured record stream. The OASIS response returned in association with an input/response NITS OASIS Template invocation shall only be as a NITS CSV Format structured record stream.

3/14/12 accepted

- Source Information

- 3/14/12 accepted

Each NITS OASIS Template shall identify the source of its information by including or linking to the name of the Transmission Provider, the Eligible Customer, or the Transmission Customer who provided the information.

- Time Of Last Update

- 3/14/12 accepted

Each NITS OASIS Template shall include a time indicating when it was created or whenever the value of any Data Element was changed.

- Data Elements

- 3/14/12 accepted

NITS OASIS Templates shall define the elementary Data Element Dictionary names for the data values to be transferred or displayed for that NITS OASIS Template.

- Documentation

- 3/14/12 accepted

OASIS information shall be in non-cryptic English, with all mnemonics defined in Business Practice Standards WEQ-003 or WEQ-000. TSIPs shall provide on-line descriptions and help screens to assist users understanding the displayed information. Documentation of all formats, contents, and mnemonics shall be available both as displays and as files that can be downloaded electronically. In order to meet the "user-friendly" goal and permit the flexibility of the OASIS Nodes to expand to meet new requirements, the NITS OASIS Templates shall be as self-descriptive as possible.

3/14/12 accepted

002-101.2.3.2	Network Integration Transmission Service OASIS Template Categories

3/14/12 bold accepted not accepted nits

NITS OASIS Templates are grouped into the following two major categories:

a. Query/Response (QUERY)

These NITS OASIS Templates are used to query and display information posted on an OASIS Node. Each query/response NITS OASIS Template accepts a set of user specified Query Variables and returns the appropriate information from data posted on the OASIS Node based on those Query Variables. The valid Query Variables and information to be returned in response are identified by Data Element in Business Practice Standard WEQ-002-101.3.

b. Input/Response (INPUT)

These NITS OASIS Templates are used to upload/input information on an OASIS Node. The required input information and information to be returned in response are identified by Data Element in Business Practice Standard WEQ-002-101.3.

002-101.2.3.3	Network Integration Transmission Service OASIS Template User Interface Screens

3/14/12 bold accepted not accepted nits

All NITS OASIS Template browser-based screens provided as part of the OASIS user interface must adhere to the requirements specified in Business Practice Standard WEQ-002-4.2.3.3.

3/14/12 accepted

002-101.2.4	Query/Response Network Integration Transmission Service OASIS Template Requirements

3/14/12 not accepted

All query/response NITS OASIS Templates must adhere to the requirements specified in Business Practice Standards WEQ-002-4.2.4, 4.2.4.1 and 4.2.4.2 with the exception that all references to CSV Format shall mean the NITS CSV Format record structure as specified in Business Practice Standard WEQ-002-101.2.7.

3/14/12 accepted

002-101.2.5	Input/Response Network Integration Transmission Service OASIS Template Requirements

3/14/12 not accepted

Input/response NITS OASIS Templates support the posting of information on an OASIS Node related to the arrangement of NITS on OASIS. The "input" identifies the required data associated with an NITS OASIS Template(s) to be posted on the OASIS Node, and the "response" specifies the information returned to the user.

002-101.2.5.1	Input Requirements

Input information is transferred to an OASIS Node using the HTTP protocol through a browser-based forms user interface or programmatically via a structured CSV message in the form defined for the NITS CSV Format. CSV formatted messages are specified in the body of an HTTP message as a series of template Header and associated Data records as specified in Business Practice Standard WEQ-002-101.2.7.

3/14/12 accepted

OASIS Nodes shall support the following methods for users to transfer input data:

a. Browser-Based Forms

3/14/12 accepted Based needs to be lower case

Browser-based FORM input shall be provided to allow users to specify the necessary input data associated with each input/response NITS OASIS Templates through a defined user interface. This may be in the form of fill in blanks, buttons, pull-down selections, etc., and may use either the GET or POST methods. The exact nature and form of these screens are not specified, and may differ between OASIS Nodes.

b. GET Method

The HTTP GET method for specifying input information in the form of a query string appended to a standard OASIS URL is not supported for the input of input/response NITS OASIS Template information.

c. POST Method

The HTTP POST method for specifying input information in the form of a query string in the message body is not supported for the input of input/response NITS OASIS Template information.

d. CSV Format

CSV formatted input information transferred in the body of a user's HTTP message shall be supported for the input of input/response NITS OASIS Template information. The "Content-length:" HTTP header shall define the length in bytes of the INPUT, and the "Content-type: text/x-oasis-csv" HTTP header shall be used to identify the data type included in the message body.

002-101.2.5.2	Response to Input

In response to a validly formatted input for each input/response NITS OASIS Template, the OASIS Node shall return an indication as to the success/failure of the requested action. The OASIS Node shall respond to the input in one of two forms based on the method used to upload the information in association with the input/response NITS OASIS Template:

a. Browser-Based Forms

3/14/12 not accepted

If the user uses the browser-based form entry method to upload input/response NITS OASIS Template data to OASIS, the response from the OASIS Node shall be a web page using the browser-based format. This shall be the only method returned for all input/response NITS OASIS Templates invoked using the browser-based form methods of input via the OASIS user interface.

3/14/12 accepted

b. CSV Format

If the user submits input/response NITS OASIS Template information using the NITS CSV File format, then the response from the OASIS Node shall also be in CSV File format as specified in Business Practice Standard WEQ-002-101.2.7. OASIS returns the response information in the body of the HTTP response message. The "Content-length:" HTTP header shall define the length in bytes of the response, and the "Content-type: text/x-oasis-csv" HTTP header shall be used to identify the data type included in the message body. This shall be the only method returned for all input/response NITS OASIS Templates uploaded using the NITS CSV Format.

3/14/12 accepted

002-101.2.6	Query Variables

002-101.2.6.1	General

3/14/12 accepted

All query/response NITS OASIS Templates shall support the specification of a query string consisting of Query Variables formatted as name/value pairs. OASIS Nodes shall support the specification of Data Element names ("name" portion of name=value pair) in both the full name and alias forms defined in Business Practice Standard WEQ-003. OASIS Nodes shall support the specification of Query Variables from the user using either the HTTP GET or POST methods. On input, Data Element names and associated values shall be accepted and processed without regard to case. On output, Data Element names and associated values may not necessarily retain the input case, and could be returned in either upper or lower case.

Input/response NITS OASIS Templates shall only support the submission of input data to OASIS using the NITS CSV Format as specified in Business Practice Standard WEQ-002-101.2.7.

3/14/12 accepted

002-101.2.6.2	Standard Header Query Variables

3/14/12 accepted

The following standard Query Variable Data Elements shall be supported by the query/response NITS OASIS Templates:

VERSION

TEMPLATE

OUTPUT_FORMAT

PRIMARY_PROVIDER_CODE

PRIMARY_PROVIDER_DUNS

RETURN_TZ

Since these header Query Variables must be supported for all query/response NITS OASIS Templates, they are not listed explicitly in the Query Variable Data Element names in the query/response NITS OASIS Template descriptions in Business Practice Standard WEQ-002-101.4. If not supplied by the user, OASIS shall assume the specified default values listed below:

3/14/12 accepted

VERSION – defaults to the most current OASIS S&CP version required to be supported by regulatory order.

TEMPLATE – defaults to the value specified for (CGI script name) as defined in Business Practice Standard WEQ-002-4.2.1.3 for the OASIS Node Script Names for the Query template to be executed.

OUTPUT_FORMAT – defaults to the value of DATA.

PRIMARY_PROVIDER_CODE – defaults to the value specified in the OASIS URL for this Data Element.

PRIMARY_PROVIDER_DUNS – defaults to the Transmission Provider’s registered DUNS.

RETURN_TZ – defaults to the Transmission Provider’s specified default time zone.

3/14/12 accepted

002-101.2.6.3	Responses to Queries

3/14/12 accepted

Responses to queries will include the following information as a minimum:

TIME_STAMP

VERSION

TEMPLATE

OUTPUT_FORMAT

PRIMARY_PROVIDER_CODE

PRIMARY_PROVIDER_DUNS

RETURN_TZ

For the browser-based format (OUTPUT_FORMAT=HTML) response, there are no specific requirements dictating how or where such information shall be provided other than the above information must be identified and visible to the user from the browser-based formatted response. When the OUTPUT_FORMAT is designated as DATA, the above information shall be supplied as specified in the NITS CSV Format requirements of Business Practice Standard WEQ-002-101.2.7.3 in the first NITS OASIS QUERY Template header record of the response. Note that the value for the above TEMPLATE Data Element will be returned in the first field (column) of the first response record as QUERY=<QueryTemplateName>, e.g., QUERY=QueryDNRList.

3/14/12 accepted

The additional information returned shall include:

a	The requested information as defined by the NITS OASIS Template indicated in the query

b.	For CSV downloads, the requested information shall be returned in compliance with the NITS CSV Format as specified in Business Practice Standard WEQ-002-101.2.7.3 and the specific NITS OASIS QUERY Template executed.

3/14/12 accepted

002-101.2.6.4	Multiple Instances

3/14/12 accepted

All OASIS Nodes shall comply with Business Practice Standard WEQ-002-4.2.6.4 in implementing NITS on OASIS.

002-101.2.6.5	Logical Operations

3/14/12 accepted

All OASIS Nodes shall comply with Business Practice Standard WEQ-002-4.2.6.5 in implementing NITS on OASIS.

002-101.2.6.6	Handling of Time Data Elements

3/14/12 accepted

All OASIS Nodes shall comply with Business Practice Standard WEQ-002-4.2.6.6 in implementing NITS on OASIS.

002-101.2.6.7	Default Values

3/14/12 accepted

All OASIS Nodes shall comply with Business Practice Standard WEQ-002-4.2.6.7 in implementing NITS on OASIS.

002-101.2.6.8	Limitations on Queries

3/14/12 accepted

All OASIS Nodes shall comply with Business Practice Standard WEQ-002-4.2.6.8 in implementing NITS on OASIS.

002-101.2.7	Network Integration Transmission Service CSV Format

3/14/12 not accepted

002-101.2.7.1	General Record Format

3/14/12 accepted

The NITS CSV Format defines the structure for all HTTP (file) uploads to OASIS with respect to the information that must be conveyed in association with each input/response NITS OASIS Template necessary for the support of NITS transaction processing on OASIS. This file format is also used to convey the results of any query issued by the user where the response is specified to be returned with the OUTPUT_FORMAT=DATA query variable.

3/14/12 accepted

The NITS CSV Format is a record oriented, comma separated value file structure consisting of two types of records: 1) header records and 2) data records. Each record within the NITS CSV Format file structure shall be terminated by a fixed carriage return and line feed combination (denoted by the symbol ↵ in all examples). The fields within a record shall be delimited by commas (,). All data within a CSV formatted message shall use printable ASCII characters with no other special embedded codes, with the exception of the special encoding requirements associated with text fields. The information in each record shall be composed of named Data Elements as defined in Business Practice Standard WEQ-003. The values associated with each of those named Data Elements shall comply with the OASIS Data Dictionary Data Element definition.

3/14/12 accepted

The NITS CSV Format header and data records are organized in a hierarchical structure of templates. The following is the general format followed for each header record represented in the NITS CSV Format:

3/14/12 accepted

<HeaderRecordType>=<TemplateName>,<DataElementNameList>

where,

<HeaderRecordType>	- Data Element name identifying the general type of information contained in the header record and subsequent information records

<TemplateName>	- Name of the NITS OASIS Template identifying the specific form and content of information records or subordinate template header and information records associated with the named template.

<DataElementNameList>	- Comma separated list of additional template specific data element names identifying the type of information conveyed as corresponding data element values in each column of each subsequent data record up to the next header record.

The general format for each data record follows:

,<DataElementValueList>

where,

<DataElementValueList>	- Comma separated list of additional template specific values to be associated with the corresponding data element names specified in the preceding header record’s <DataElementNameList>.

The first record in the NITS CSV Format file must contain a header record identifying which of the two main template categories are being exchanged via the <HeaderRecordType> as follows:

- Input/response templates:
	INPUT=<TemplateName>,<DataElementNameList>

- Query/response templates:

QUERY=<TemplateName>,<DataElementNameList>

The OASIS Data Elements that must appear in the INPUT or QUERY header record’s <DataElementNameList> are specified in Business Practice Standards WEQ-002-101.2.7.2 and WEQ-002-101.2.7.3 respectively. Additional subordinate header and data records that may appear in the subsequent records in the NITS CSV Format input/response data uploads or the query/response downloads are also specified in Business Practice Standards WEQ-002-101.2.7.2 and WEQ-002-101.2.7.3.

3/14/12 accepted

OASIS nodes shall ignore any leading or trailing white-space (ASCII space characters) that is not encoded within explicit double quote characters within any value supplied for a template data element or that precedes or trails any data record. Extraneous comma delimiters that may be appended to any record beyond the defined limits of the data values (columns) being conveyed in a given data record shall also be ignored.

3/14/12 accepted

002-101.2.7.2	Input/Response Header Records

3/14/12 accepted

All input/response category template submissions (uploads) shall be identified by the first header record in the NITS CSV Format with the key string of “INPUT=<InputTemplateName>”. The specific template names that may appear in the INPUT header record are referred to as NITS OASIS INPUT Templates and are described in Business Practice Standard WEQ-002-101.3.1.

3/14/12 accepted

The general structure of the information conveyed in a given NITS OASIS INPUT Template using the NITS CSV Format is shown below:

The NITS OASIS INPUT Template consists of a leading INPUT header record and associated Data Elements followed by one or more subordinate NITS OASIS REQUEST Templates. Each NITS OASIS REQUEST Template consists of a REQUEST header record and associated Data Elements followed by zero or more NITS OASIS DATA Templates. Each NITS OASIS DATA Template consists of a DATA header record followed by one or more records of data values.

3/14/12 accepted

The general form of the NITS CSV Format for any input/response NITS OASIS Template shall conform to:

INPUT=<InputTemplateName>, <InputTemplateDataElementList>↵

,<InputTemplateDataElementValues>↵

REQUEST=<RequestTemplateName>, <RequestTemplateDataElementList>↵

,<RequestTemplateDataElementValues>↵

DATA=<DataTemplateName>, <DataTemplateDataElementList>↵

,<DataTemplateDataElementValues>↵

…

,<DataTemplateDataElementValues>↵

DATA=<DataTemplateName>, <DataTemplateDataElementList>↵

,<DataTemplateDataElementValues>↵

…

,<DataTemplateDataElementValues>↵

REQUEST=<RequestTemplateName>, <RequestTemplateDataElementList>↵

,<RequestTemplateDataElementValues>↵

DATA=<DataTemplateName>, <DataTemplateDataElementList>↵

,<DataTemplateDataElementValues>↵

REQUEST=<RequestTemplateName>, <RequestTemplateDataElementList>↵

,<RequestTemplateDataElementValues>↵

REQUEST=<RequestTemplateName>, <RequestTemplateDataElementList>↵

,<RequestTemplateDataElementValues>↵

DATA=<DataTemplateName>, <DataTemplateDataElementList>↵

,<DataTemplateDataElementValues>↵

…

3/14/12 not accepted

The valid NITS OASIS REQUEST Templates that may be submitted in association with a given NITS OASIS INPUT Template are specified in Business Practice Standard WEQ-002-101.3.2. There shall be no limit to the number of NITS OASIS REQUEST Templates that may be submitted in association with a given NITS OASIS INPUT Template. Multiple instances of the same NITS OASIS REQUEST Template may appear in association with a single NITS OASIS INPUT Template submission where the NITS OASIS REQUEST Template definition indicates that multiple instances are allowed.

3/14/12 accepted

The set of valid NITS OASIS DATA Templates that may be submitted in association with a given NITS OASIS REQUEST Template are specified in Business Practice Standard WEQ-002-101.3.4. There shall be only one instance of a given NITS OASIS DATA Template specified in association with a given NITS OASIS REQUEST Template.

3/14/12 accepted

Note that in contrast to the general OASIS and PTP OASIS Template constructs, a given NITS OASIS Template request may require the submission of several different types of data using different NITS OASIS DATA Templates. Therefore each specific OASIS request submission is made using separate distinct NITS OASIS REQUEST Templates that may be included in a given NITS OASIS INPUT Template upload. Multiple requests may be submitted in a single upload (NITS OASIS INPUT Template submission), but rather than being supplied as multiple data records within a single PTP OASIS Template upload, these multiple requests must be submitted as separate NITS OASIS REQUEST Templates followed by the associated NITS OASIS DATA Template(s) data records for each individual request being made.

3/14/12 accepted

The information returned to the user in response to submission of any input/response template shall be an echo of each header record present in the NITS CSV Format submission followed by the Data Element names and values as dictated for the defined “response” in each of the specific INPUT, REQUEST or DATA template records corresponding to the information submitted.

002-101.2.7.3	Query/Response Header Records

3/14/12 accepted

All query/response category templates are initiated by the submission of a specific set of OASIS URL Query Variables as a series of OASIS Data Element name/value pairs as specified in Business Practice Standard WEQ-002-101.2.6. The TEMPLATE Data Element value shall specify which of the supported NITS OASIS QUERY Templates is being invoked. Each of the supported NITS OASIS QUERY Templates is described in Business Practice Standard WEQ-002-101.4.

3/14/12 accepted

When the OUTPUT_FORMAT=DATA header query variable is specified, the response to the user will be in the NITS CSV Format as specified below and shall be identified by the first header record returned in the response with the key string of “QUERY=<QueryTemplateName>”. The general structure of the information returned in response to a given NITS OASIS QUERY Template using the NITS CSV Format is shown below:

3/14/12 accepted

The response to a NITS OASIS QUERY Template consists of a leading QUERY header record and associated Data Elements followed by one or more subordinate NITS OASIS OBJECT Templates dependent on the results being returned. Each NITS OASIS OBJECT Template consists of a OBJECT header record and associated Data Elements followed by zero or more NITS OASIS DATA Templates. Each NITS OASIS DATA Template consists of a DATA header record followed by one or more records of data values.

3/14/12 accepted

The general form of the NITS CSV Format received in response for any query/response NITS OASIS Template shall conform to:

QUERY=<QueryTemplateName>, <QueryTemplateDataElementList>↵

,<QueryTemplateDataElementValues>↵

OBJECT=<ObjectTemplateName>, <ObjectTemplateDataElementList>↵

,<ObjectTemplateDataElementValues>↵

DATA=<DataTemplateName>, <DataTemplateDataElementList>↵

,<DataTemplateDataElementValues>↵

…

,<DataTemplateDataElementValues>↵

DATA=<DataTemplateName>, <DataTemplateDataElementList>↵

,<DataTemplateDataElementValues>↵

…

,<DataTemplateDataElementValues>↵

OBJECT=<ObjectTemplateName>, <ObjectTemplateDataElementList>↵

,<ObjectTemplateDataElementValues>↵

DATA=<DataTemplateName>, <DataTemplateDataElementList>↵

,<DataTemplateDataElementValues>↵

…

,<DataTemplateDataElementValues>↵

OBJECT=<ObjectTemplateName>, <ObjectTemplateDataElementList>↵

,<ObjectTemplateDataElementValues>↵

…

3/14/12 not accepted

If an NITS OASIS OBJECT Template has no associated NITS OASIS DATA Templates defined, multiple instances of the same NITS OASIS OBJECT Template information to be returned in the query response shall be returned as multiple data records immediately following the Object Header record containing the NITS OASIS OBJECT Template Data Element values for each such instance:

QUERY=<QueryTemplateName>, <QueryTemplateDataElementList>↵

,<QueryTemplateDataElementValues>↵

OBJECT=<ObjectTemplateName>, <ObjectTemplateDataElementList>↵

,<ObjectTemplateDataElementValues>↵

,<ObjectTemplateDataElementValues>↵

…

,<ObjectTemplateDataElementValues>↵

3/14/12 not accepted

The valid NITS OASIS OBJECT Templates and associated NITS OASIS DATA Templates that may be returned in the response to the supported NITS OASIS QUERY Templates are specified in Business Practice Standard WEQ-002-101.4.

3/14/12 accepted

002-101.2.7.4	Data Records

3/14/12 accepted

Data Records immediately follow an associated Header Record in the NITS CSV Format input/response and query/response NITS OASIS Templates. Data Records are uniquely identified as always having the first data field (column) containing a null value. The remaining information contained in each Data Record specifies the value to be associated with each named Data Element that appears in the preceding Header Record.

3/14/12 accepted

If the NITS OASIS Template definition allows for multiple Data Records, there may be more than one Data Record in association with that NITS OASIS Template. Multiple Data Records for a given NITS OASIS Template appear in immediate succession up to the next Header Record or encountering the end of the input data stream.

3/14/12 accepted

The data values associated with each column’s Data Element are interpreted based on the Data Element type as defined in Business Practice Standard WEQ-003:

- Numeric Data Elements: All numeric Data Elements shall be represented by an ASCII string of numeric digits in base ten. Decimal point and sign are implied, or may be explicitly specified.

- 3/14/12 accepted

- Text Data Elements: Alphabetic and alphanumeric Data Elements shall be represented as ASCII strings and encoded using the following rules:

- Text strings that do not contain commas (,) or double quotes (") shall be accepted both with and without being enclosed by double quotes.

- Text fields with commas (,) or double quotes (") must be enclosed with double quotes. In addition double quotes within a text field shall be indicated by two double quotes ("").

- The Data Element field length specified in Data Dictionary does not include the additional double quotes necessary to encode text data.

- Null Data Elements: Null Data Elements shall be represented by two consecutive commas (,,) corresponding to the leading and trailing (if appropriate) Data Element comma separators. Null text strings may optionally be represented by two consecutive double quote characters within the leading and trailing comma separators (i.e., Y,"",Y).

002-101.2.7.5	Continuation Records

3/14/12 accepted

The NITS CSV Format does not require an explicit distinction of Continuation Records. The second and all subsequent Data Records appearing after a given Header Record and prior to the next Header Record always represent Continuation Records associated with the NITS OASIS Template identified in the preceding Header Record and first Data Record. Any required Data Elements that must be specified in the second or subsequent Data Records are described in each NITS OASIS Template definition in Business Practice Standards WEQ-002-101.3 and WEQ-002-101.4. On upload or input of template Data Records, any values supplied in the second or subsequent Data Records that correspond to NITS OASIS Template Data Elements other than those explicitly allowed to appear in multiple Data Records shall be ignored. However, commas must be included to properly align the fields (columns) of each Data Record with the corresponding Data Element names listed in the associated Header Record.

3/14/12 accepted

002-101.2.7.6	Error Handling in Network Integration Transmission Service CSV Formatted Responses

3/14/12 bold accepted not accepted nits

  - Validity of each record in the NITS CSV Format response to each NITS OASIS DATA Template input in association with a given NITS OASIS REQUEST Template shall be indicated through the use of the RECORD_STATUS and ERROR_MESSAGE Data Elements which are included in each Data Record (row) of the response:

    - If no error was encountered in an input Data Record, the RECORD_STATUS Data Element in the corresponding NITS OASIS DATA Template response Data Record shall be returned with a value of 200 (success), and the ERROR_MESSAGE shall be blank.

    - If any error is detected in processing an Input Data Record, it shall be indicated by a RECORD_STATUS Data Element value other than 200. The ERROR_MESSAGE shall be set to an appropriate text message to indicate the source of the error in that NITS OASIS DATA Template Data Record.

  - The overall validity of each NITS OASIS REQUEST Template submitted shall be indicated in the NITS CSV Format response to each NITS OASIS REQUEST Template through the use of the RECORD_STATUS and ERROR_MESSAGE Data Elements which are included in the NITS OASIS REQUEST Template Data Record (row) of the response:

    - If no error was encountered in any of the input Data Records submitted in association with the NITS OASIS REQUEST Template, the RECORD_STATUS Data Element in the corresponding NITS OASIS REQUEST Template response Data Record shall be returned with a value of 200 (success), and the ERROR_MESSAGE shall be blank.

    - If any error is detected in processing any of the input Data Records submitted in association with the NITS OASIS REQUEST Template, the error shall be indicated for the overall request by a RECORD_STATUS Data Element value other than 200. The ERROR_MESSAGE shall be set to an appropriate text message to indicate the source of the error.

  - The OASIS Node shall validate all NITS OASIS INPUT Template records submitted before returning a response to the user. The OASIS Node shall process all valid NITS OASIS REQUEST Templates, while invalid request shall be identified as erroneous through the use of RECORD_STATUS and ERROR_MESSAGE as noted above. The user must correct the invalid requests and resubmit only those NITS OASIS REQUEST Templates and associated NITS OASIS DATA Templates that were identified in error.

- 3/14/12 accepted

  - If an error is encountered validating the basic structure of the input data stream according to the defined Header and Data Record format specified in the NITS CSV Format, the entire data submission shall be rejected and a 4xx Client Error code shall be returned in the Status-line of the HTTP response. The message body should describe the nature of the validation error encountered.

- 3/14/12 accepted

002-101.2.8	Registration Information

3/14/12 accepted

All OASIS Nodes shall comply with Business Practice Standard WEQ-002-4.2.8 in implementing NITS on OASIS.

002-101.2.9	Representation of Time

3/14/12 accepted

All OASIS Nodes shall comply with Business Practice Standard WEQ-002-4.2.9 in implementing NITS on OASIS.

002-101.2.10	Transaction Process

3/14/12 accepted

OASIS shall implement the defined input/response NITS OASIS Templates to allow Eligible Customers or Transmission Customers to submit and update their initial application for NITS and to submit and update any modifications to that application for NITS after the initial application has been completed and NITS granted on OASIS. OASIS shall also provide a browser-based user interface that implements the same basic functionality provided by the NITS OASIS Template interface through one or more displays or forms.

3/14/12 accepted

The OASIS transaction process is managed through the submission of a NITS OASIS REQUEST Template. The NITS OASIS REQUEST Template specifies what action the Eligible Customer or Transmission Customer is requesting the Transmission Provider to review/evaluate and the associated data that may or must be submitted to describe the request. The current state associated with every NITS request is tracked as actions are taken by the Eligible Customer or Transmission Customer or Transmission Provider in the STATUS Data Element associated with each individual NITS OASIS REQUEST Template submitted. The Business Practice Standard WEQ-013-101 describes the NITS transaction process and the valid STATUS values and state transitions that the Eligible Customer or Transmission Customer and Transmission Provider may take in the course of processing each request. The Implementation Guide also provides specific requirements and recommendations related to the handling of particular requests from both a technical and business process perspective.

3/14/12 accepted

With the exception of the handling of the initial NITS Application request (REQUEST=NewNITSApplication) and the concomitant request process (INPUT=NewConcomitantRequest), each NITS request submitted via a NITS OASIS REQUEST Template is managed separately and independently. Multiple requests may be submitted in a single batch upload, or requests may be submitted separately using the NewNITSRequest NITS OASIS INPUT Template. After the initial submission, the request may be updated to change its STATUS and/or any other relevant information associated with the request as allowed for the specific NITS OASIS REQUEST Template submitted using the UpdateNITSRequest NITS OASIS INPUT Template.

3/14/12 accepted

Certain NITS requests support a full Eligible Customer/Transmission Customer-Transmission Provider interaction consisting of Eligible Customer or Transmission Customer request submittal, Transmission Provider approve/deny action, and final Eligible Customer or Transmission Customer confirmation. The final Eligible Customer or Transmission Customer confirmation requirement can be waived by the Eligible Customer or Transmission Customer using the PRECONFIRMED Data Element set to a value of YES. NITS requests that only require Transmission Provider approval to be finalized will be specified in the NITS OASIS REQUEST Template definition as requiring that the PRECONFIRMED Data Element be set to YES on initial submission.

3/14/12 accepted

Any NITS request may be queried to view the current STATUS and latest set of information associated with that request using the query/response NITS OASIS Templates.

002-101.2.10.1	Request Types

3/14/12 accepted

The NITS OASIS REQUEST Templates themselves define each type of NITS related request that may be submitted to OASIS. The recognized NITS OASIS REQUEST Templates are described in Business Practice Standard WEQ-002-101.3.2, and further discussed in Business Practice Standard WEQ-013.

3/14/12 accepted

There is no current requirement for use of an explicit REQUEST_TYPE data element specified within the data associated with a given NITS related request. The REQUEST_TYPE Data Element is included within any PTP Transmission Service request submitted as part of a NITS concomitant request for compatibility with the PTP transrequest OASIS Template.

3/14/12 accepted

002-101.2.10.2	Status Values

3/14/12 accepted

The STATUS Data Element is used in the OASIS transaction process to control the interaction between Eligible Customer or Transmission Customer and Transmission Provider and communicate information regarding the state of each requested transaction between the parties. The valid values for the STATUS Data Element are enumerated in the Business Practice Standard WEQ-003. Definitions for each of the STATUS values and a NITS request transaction state transition diagram for the STATUS Data Element is described in detail in the Business Practice Standard WEQ-013-101.

002-101.2.10.3	Dynamic Notification

3/14/12 accepted

Eligible Customer or Transmission Customers may request the delivery of dynamic notifications from OASIS when any request associated with a given NITS Application is updated by the Eligible Customer or Transmission Customer or the Transmission Provider. OASIS nodes shall support the delivery of dynamic notifications through either the HTTP protocol or via electronic mail. The selection of which mechanism is used and the contents of the messages delivered to the client program or e-mail address is defined by the content of the STATUS_NOTIFICATION Data Element as described in the next subsections. This STATUS_NOTIFICATION Data Element is established for a given NITS Application in association with the Eligible Customer or Transmission Customer information as specified in the applicable NITS OASIS REQUEST Templates.

3/14/12 accepted

OASIS Nodes shall monitor the information associated with every NITS OASIS REQUEST Template submitted in association with a NITS Application. OASIS Nodes shall issue a dynamic notification message to the Eligible Customer or Transmission Customer registered with that NITS Application whenever the STATUS or any other Data Element associated with any pending request is updated or changed either by the Eligible Customer or Transmission Customer or the Transmission Provider.

3/14/12 accepted

Regardless of whether this dynamic notification method is used or not, it shall still remain the user's responsibility to monitor and verify the desired request information. OASIS nodes shall not be obligated or liable to guarantee delivery/receipt of dynamic notification messages via the STATUS_NOTIFICATION mechanism other than on a "best effort" basis.

3/14/12 accepted

002-101.2.10.3.1	HTTP Notification

3/14/12 accepted

OASIS Nodes shall deliver dynamic notification to a client system based on HTTP OASIS URL information supplied in part by the STATUS_NOTIFICATION Data Element and by information supplied as part of the Eligible Customer or Transmission Customer's Company registration information. HTTP OASIS URL's are formed by the concatenation of a protocol field (i.e., http: ), a domain name (e.g.,//www.tsin.com), a port designation (e.g., :80), and resource location information in exactly the same fashion as specified for PTP OASIS Templates.

The STATUS_NOTIFICATION Data Element shall contain the protocol field "http:", which designates the notification method/protocol to be used, followed by all resource location information required; the target domain name and port designations shall be inserted into the notification OASIS URL based on the Eligible Customer or Transmission Customer's Company registration information. The resource location information may include directory information, cgi script identifiers and OASIS URL encoded query string name/value pairs as required by the Eligible Customer or Transmission Customer's application. An OASIS Node performs no processing on the resource location information other than to include it verbatim along with the protocol, domain name and port information when forming the OASIS URL that will be used to deliver the HTTP protocol notification message. Example usage of this information is specified in Business Practice Standard WEQ-002-4.2.10.3.1.

3/14/12 accepted

The contents of the HTTP protocol notification message delivered by an OASIS Node shall consist of the complete OASIS URL created by combining fields from the STATUS_NOTIFICATION Data Element and Company registration information as part of an HTTP POST method request. The standard HTTP header records for Content-type: text/x-oasis-csv and appropriate Content-length: value shall be used. OASIS Nodes shall append in the body of the HTTP request a NITS CSV Format output data stream of information associated with the specific pending NITS request that was submitted or updated and triggered the dynamic notification. The NITS CSV Format output data shall take the same form as that which would be returned in response to a Eligible Customer or Transmission Customer issued QueryNITSRequest NITS OASIS QUERY Template for the specific NITS request that was submitted or updated.  The Transmission Provider’s prevailing time zone will be used as the default value for RETURN_TZ when formatting the NITS CSV Format response.

3/14/12 accepted

In the event an error is encountered delivering the HTTP dynamic notification message to the target URL as indicated by a failure of the target system to respond, or return of HTTP response status of 408, 500, 503, or 504, OASIS Nodes shall retry up to two more times, once every 5 minutes.

002-101.2.10.3.2	E-mail Notification

3/14/12 accepted

OASIS Nodes shall deliver dynamic notification to an e-mail address based on Mailto: OASIS URL information specified in the STATUS_NOTIFICATION Data Element. Mailto: OASIS URL's consist of the "mailto:" protocol identifier and an Internet mail address to which the notification message should be sent. The STATUS_NOTIFICATION Data Element shall contain the protocol field "mailto:", which designates the notification method/protocol to be used, followed by an Internet mail address in conformance with RFC 822. OASIS Nodes shall send an e-mail message to that Internet mail address containing the following information:

3/14/12 accepted

- "To:" set to the mail address from the STATUS_NOTIFICATION Data Element

- "From:" set to an appropriate mail address of the OASIS Node

- "Subject:" shall be the NITS OASIS REQUEST Template name associated with the request submitted/updated followed by the value of the NITS Application APPLICATION_REF, the specific NITS request’s ASSIGNMENT_REF Data Element and the current value for the STATUS Data Element associated with the request formatted as data element name/value pairs, e.g., "Subject: REQUEST=AddNITSResource, APPLICATION_REF=718, ASSIGNMENT_REF=245, STATUS=ACCEPTED"),

The body of the mail message shall take the same form as that which would be returned in response to a Eligible Customer or Transmission Customer issued QueryNITSRequest NITS OASIS QUERY Template for the specific NITS request that was submitted or updated.  The Transmission Provider’s prevailing time zone will be used as the default value for RETURN_TZ when formatting the NITS CSV Format response.

3/14/12 accepted

002-101.2.10.4	Use of Comments

3/14/12 accepted

All OASIS Nodes shall comply with Business Practice Standard WEQ-002-4.2.10.4 in implementing NITS on OASIS.

002-101.2.11	Reference Identifiers

3/14/12 accepted

The TSIP shall assign a unique reference identifier in association with the following NITS OASIS Template Data Elements. These reference identifiers are returned in the OASIS response on initial submittal of the applicable information and must be specified by the Eligible Customer or Transmission Customer in any subsequent request to update or modify the associated information:

3/14/12 accepted

- APPLICATION_REF: Assigned on initial creation/submission of an application for NITS on OASIS, and referenced in all subsequent requests to add/update/modify any information in association with that application.

- ASSIGNMENT_REF: Assigned on initial submission of a NITS OASIS REQUEST Template and referenced in all subsequent requests to add/update/modify any information in association with that request. ASSIGNMENT_REF must be a unique value for each NITS request and coordinated with ASSIGNMENT_REF values associated with any PTP or Ancillary service requests submitted using the general OASIS and PTP OASIS Templates as specified in Business Practice Standard WEQ-002-4.

- 3/14/12 accepted

- TRANSACTION_REF: Assigned on initial creation/submission of a concomitant request and associated with each individual request submitted under that concomitant request. TRANSACTION_REF must be referenced in all subsequent requests to update any of individual requests within that concomitant request submittal.

- 3/14/12 accepted

The Eligible Customer or Transmission Customer shall assign unique identifiers for the following NITS Application information Data Elements to be used and supplied when referring to the corresponding portions of information defining key elements of the overall NITS Application:

- GENERATION_NAME: Assigned on initial creation/definition of a specific physical generation asset that may then be referenced in any Generation or Network Resource specific requests that are associated with that generation asset.

- LOAD_NAME: Assigned on initial creation/designation of a specific Network Load that may then be referenced in any load specific requests that are associated with that Network Load.

- RESOURCE_NAME: Assigned on initial creation/definition of a Network Resource that may then be referenced in any request to designate that resource as a DNR or eliminate that DNR in support of third party sales, etc.

002-101.2.12	Linking of Ancillary Services to Network Integration Transmission Service

3/14/12 accepted

The OASIS shall support the capability for the Eligible Customer or Transmission Customer to submit arrangements for Ancillary Services required in support of their NITS Application using the NITS OASIS REQUEST Templates. These submissions shall be functionally equivalent to the general OASIS and PTP OASIS Templates for ancrequest, and anccust. Third party Ancillary Service suppliers shall use the general OASIS and PTP OASIS Templates to receive and respond to any Eligible Customer or Transmission Customer requests for the arrangement of Ancillary Services.

3/14/12 accepted

002-101.3	INPUT/RESPONSE TEMPLATE DESCRIPTIONS

3/14/12 accepted

The following NITS OASIS Templates define the structure and Data Elements in fixed number and sequence which must be provided for all data transfers to and from the OASIS Nodes in association with the arrangement of NITS on OASIS. The definitions of the associated NITS OASIS Template Data Elements are listed in Business Practice Standard WEQ-003. TSIPs must provide a more detailed supplemental definition of any specific enumerated or restricted Data Element values required on submission of any of the defined NITS OASIS Templates. The definition and use of these enumerated or restricted Data Element values should be provided and accessible online via the general OASIS list NITS OASIS Template. Additional information on the use of each NITS OASIS Template and the associated Data Elements is provided in the detailed template descriptions below and in Business Practice Standard WEQ-013.

3/14/12 accepted

The following is a tabular summary of each input/response NITS OASIS Templates describing the valid INPUT, REQUEST and DATA NITS OASIS Templates that may be submitted on OASIS using the NITS CSV Format. The detailed description for each INPUT, REQUEST and DATA NITS OASIS Template are specified in Business Practice Standards WEQ-002-101.3.1 through 6.3.3 respectively.

3/14/12 accepted

Each NITS OASIS INPUT Template must contain one or more NITS OASIS REQUEST Templates defining the actions to be performed on OASIS. Each NITS OASIS REQUEST Template must contain zero or more NITS OASIS DATA Templates defining any required additional information that may be specified for a given REQUEST. For example, submission of the NITS OASIS INPUT Template NewNITSRequest for the NITS OASIS Request Template AddNITSDNR requires submission of the NITSResourceDesignation, NITSResourceCapacity, and AuxiliaryTransmission NITS OASIS DATA Templates. To update the STATUS of that AddNITSDNR NITS OASIS REQUEST Template requires only submission of the NITS OASIS INPUT Template UpdateNITSRequest with the NITS OASIS REQUEST Template AddNITSDNR and no DATA templates as the STATUS Data Element being updated is included in the NITS OASIS REQUEST Template header record.

3/14/12 accepted

INPUT/REQUEST Template Summary

| INPUT Template | INPUT Template Description | REQUEST Template(s) | REQUEST Template Description | DATA Template(s) | DATA Template Description |
| --- | --- | --- | --- | --- | --- |
| NewNITSRequest | The main NITS OASIS INPUT Template used to initiate new requests to define or modify key components of a NITS Application. The NewNITSRequest NITS OASIS INPUT Template must contain one or more specific NITS OASIS REQUEST Templates to be submitted in association with a given NITS Application. 3/14/12 accepted | CopyNITSApplication | Special request to copy one or more previously submitted NITS Requests into the OASIS Presubmittal Workspace under a newly assigned APPLICATION_REF for creating a new NITS Application for submittal. The copied REQUEST(s) under the new NITS Application will be assigned new ASSIGNMENT_REF identifiers and will be set to the STATUS of PRESUBMITTED. 3/14/12 accepted | Not Applicable; there are no DATA templates supported under the CopyNITSApplication REQUEST Template. |  |
|  |  | CopyNITSRequest | Special request to copy one or more previously submitted NITS Requests into the OASIS Presubmittal Workspace to be edited for submission as a new NITS request under an existing NITS Application. The copied REQUEST(s) will be assigned new ASSIGNMENT_REF identifiers and will be set to the STATUS of PRESUBMITTED. 3/14/12 accepted | Not Applicable; there are no DATA templates supported under the CopyNITSRequest REQUEST Template. |  |
|  |  | NewNITSApplication | Directs the OASIS to open a new NITS Application workspace with an initial set of information defining the basic characteristics of service being requested and assign/return a unique NITS Application reference to be used in all subsequent requests associated with the new application for NITS. The NewNITSApplication NITS OASIS Request Template must be submitted on its own with no other NITS OASIS REQUEST Templates present in the NewNITSRequest NITS OASIS INPUT Template submission. 3/14/12 accepted | NITSService | Defines the basic term of NITS service to be requested, or any additional term(s) to be renewed under the NITS Application. |
|  |  |  |  | NITSCustomer | Defines the basic information identifying the Eligible Customer that is responsible for the obligations under the NITS Application. |
|  |  |  |  | NITSAgent | Defines the basic information regarding and agency relationship(s) between the Eligible Customer and those entities authorized to act on their behalf. |
|  |  | ModifyNITSService | Allows for the modification of the term of service associated with a NITS Application, i.e., extension or renewal for a subsequent term. | NITSService | (see above) |
|  |  | ModifyNITSCustomer | Allows for the modification of the Customer associated with a NITS Application, i.e., transfer of obligation under NITS Application. | NITSCustomer | (see above) |
|  |  | AddNITSAgent | Request to add a new Designated Agent authorized to act on the Eligible Customer or Transmission Customer’s behalf. | NITSAgent | (see above) |
|  |  | ModifyNITSAgent | Request to modify specific information associated with an existing Designated Agent, including termination of agency relationship. | NITSAgent | (see above) |
|  |  | AddNITSLoad | Request to define or add a new Network Load to be served under the NITS Application. | NITSLoadDescription | Defines the basic attributes of a Network Load to be under the NITS Application. |
|  |  |  |  | NITSLoadForecast | Defines the required 10-year load forecast information associated with the Network Load under the NITS Application. This is an optional template; Transmission Provider’s Business Practices may require Eligible Customers or Transmission Customers to supply this information with the STATUS of OFF_OASIS. 3/14/12 accepted |
|  |  | ModifyNITSLoad | Request to modify the characteristics or information associated with an existing Network Load including the request for elimination of a specific Network Load from the NITS Application. | NITSLoadDescription | (see above) |
|  |  |  |  | NITSLoadForecast | (see above) |
|  |  | AddNITSGeneration | Request to define a new generation asset and related attributes that will be used in association with the application for NITS as a defined Network Resource. 3/14/12 accepted | NITSGenerationDescription | Defines the basic attributes of each physical generation asset owned, leased, etc., by the Eligible Customer or Transmission Customer and eligible to be a DNR under the NITS Application. 3/14/12 accepted |
|  |  |  |  | NITSGenerationDispatch | Defines the (re)dispatch cost information associated with a Eligible Customer or Transmission Customer’s generation asset for use by the Transmission Provider in operating the DNR. This is an optional template; Transmission Provider’s Business Practices may require Eligible Customer or Transmission Customers to supply this information with the STATUS of OFF_OASIS. 3/14/12 accepted |
|  |  | ModifyNITSGeneration | Request to modify specific information or attributes associated with an existing generation asset. 3/14/12 accepted | NITSGenerationDescription | (see above) |
|  |  |  |  | NITSGenerationDispatch | (see above) |
|  |  | AddNITSResource | Request to define/add a new Network Resource that may then be designated and called upon to serve any Network Load(s) served under the NITS Application. 3/14/12 accepted | NITSResourceDescription | Defines the basic attributes of an energy resource that is forecasted or designated to serve Network Load under the NITS Application. |
|  |  |  |  | NITSResourceForecast | Defines the required 10-year resource forecast information associated with the energy resource projected to be used to serve Network Load under the NITS Application. This is an optional template; Transmission Provider’s Business Practices may require Eligible Customer or Transmission Customers to supply this information with the STATUS of OFF_OASIS. 3/14/12 accepted |
|  |  | ModifyNITSResource | Request to modify the characteristics or information associated with an existing Network Resource under the NITS Application. | NITSResourceDescription | (see above) |
|  |  |  |  | NITSResourceForecast | (see above) |
|  |  | AddNITSDNR | Request to designate and commit a Network Resource for firm delivery to Network Load(s) served under the NITS Application. | NITSResourceDescription | (see above) |
|  |  |  |  | NITSResourceForecast | (see above) |
|  |  |  |  | NITSResourceDesignation | Defines the additional information required for a defined resource to be qualified as a DNR under the NITS Application. |
|  |  |  |  | NITSResourceCapacity | Defines the MW capacity over time that is being designated from a defined resource to serve Network Load under the NITS Application. |
|  |  |  |  | NITSSchedulingRights | Defines any limitations as to the deliverability of a given DNR to serve a given Network Load under the NITS Application. |
|  |  |  |  | AuxiliaryTransmission | Defines the nature of any Transmission Service rights acquired or eligible to be used by the Eligible Customer or Transmission Customer in delivering a DNR to load on any intervening Transmission Providers transmission system(s) and/or additional transmission requests for coordinated evaluation. |
|  |  | TerminateNITSDNR | Request to release on a temporary or indefinite basis a DNR from those resources committed for firm delivery to Network Load(s) served under the NITS Application. 3/14/12 accepted | NITSResourceDesignation | (see above – defines the DNR to be terminated) |
|  |  |  |  | NITSResourceCapacity | (see above – defines the capacity over time to be terminated from the DNR) |
|  |  | AddNITSSecondary | Request to serve Network Load on a secondary/non-firm basis from an non-designated Network Resource. | NITSSecondaryService | Defines the information required to request Transmission Service for delivery of a non-designated (secondary) resource to serve Network Load on a non-firm basis under the NITS Application. |
|  |  |  |  | NITSSecondaryCapacity | Defines the MW capacity over time that is being requested for delivery of a non-designated resource to serve Network Load under the NITS Application. |
|  |  |  |  | AuxiliaryTransmission | Optional information on Coordinated Requests submitted to applicable Transmission Providers that are to be evaluated with the NITSSecondaryService NITS OASIS REQUEST Template as a Coordinated Group. |
|  |  | TerminateNITSSecondary | Request to release capacity from secondary/non-firm Transmission Service used for delivery of a non-designated Network Resource to serve Network Load. 3/14/12 accepted | NITSSecondaryService | Defines the secondary service to be terminated (see above). |
|  |  |  |  | NITSSecondaryCapacity | Defines the capacity over time to be terminated from the secondary service (see above). |
|  |  | AddNITSAncillary | Request to arrange Ancillary Services in support of the NITS Application. | NITSAncillaryService | Defines the information required to request the arrangement of Ancillary Services, if required. |
|  |  |  |  | NITSAncillaryCapacity | Defines the MW capacity and price over time for the requested Ancillary Services. |
| UpdateNITSRequest | The UpdateNITSRequest NITS OASIS INPUT Template is used to update elements of an existing pending request submitted via the NewNITSRequest NITS OASIS INPUT Template. This template allows the user to change the STATUS of any pending REQUEST in accordance with the allowed template STATUS state transitions, or to add/replace component DATA template information associated with a given request to revise the NITS Application information to be submitted or resolve a deficiency. 3/14/12 accepted | (Any of the NITS OASIS REQUEST Templates allowed under the NewNITSRequest NITS OASIS INPUT Template, with the exception of CopyNITSApplication and CopyNITSRequest which may not be submitted in the UpdateNITSRequest INPUT Template.) 3/14/12 accepted | Dependent on the current STATUS of a given request identified by the type of NITS OASIS REQUEST Template and unique ASSIGNMENT_REF identifier, allows the user to alter the STATUS or associated DATA contained in that request as allowed within the context of that request and its current STATUS. The NewNITSApplication NITS OASIS REQUEST Template must be submitted on its own with no other NITS OASIS REQUEST Templates present in the UpdateNITSRequest NITS OASIS INPUT Template submission. 3/14/12 accepted | (Any of the NITS OASIS DATA Templates allowed under each NITS OASIS REQUEST Template) |  |
| CopyNITSRequest | The CopyNITSRequest NITS OASIS INPUT Template is used to copy any existing NITS OASIS REQUEST Template information into the OASIS Pre-submittal Workspace. The copied REQUEST(s) will be assigned new ASSIGNMENT_REF identifiers and will be set to the STATUS of PRESUBMITTED. 3/14/12 accepted | (Any of the NITS OASIS REQUEST Templates allowed under the NewNITSRequest I NITS OASIS NPUT Template) | The NITS OASIS REQUEST(s) Template to be copied into the Pre-submittal Workspace are identified by the NITS OASIS REQUEST Template name, APPLICATION_REF and ASSIGNMENT_REF 3/14/12 accepted |  |  |
| NewConcomitantRequest | This NITS OASIS INPUT Template is used to submit a new set of requests to modify service under a NITS Application that are to be evaluated concomitantly and acted upon as a single request. Current restrictions require the submission of a single TerminateNITSDNR NITS OASIS REQUEST Template in addition to only one of the other three requests for additional transmission service, AddNITSDNR, AddNITSSecondary, or AddPTPService NITS OASIS REQUEST Templates. 3/14/12 accepted | TerminateNITSDNR | Request to release on a temporary or indefinite basis a DNR from those Network Resources committed for firm delivery to Network Load(s) served under the NITS Application. 3/14/12 accepted | NITSResourceDesignation | Defines the DNR to be terminated (see above). |
|  |  |  |  | NITSResourceCapacity | Defines the capacity over time to be terminated from the DNR (see above). |
|  |  | AddNITSDNR | Request to designate and commit a Network Resource for firm delivery to Network Load(s) served under the NITS Application to be evaluated respecting the simultaneous release of a DNR and/or request for incremental PTP Transmission Service submitted as a concomitant request. | NITSResourceDescription | (see above). |
|  |  |  |  | NITSResourceForecast | (see above). |
|  |  |  |  | NITSResourceDesignation | (see above). |
|  |  |  |  | NITSResourceCapacity | (see above). |
|  |  |  |  | NITSSchedulingRights | (see above). |
|  |  |  |  | AuxiliaryTransmission | (see above). |
|  |  | AddNITSSecondary | Request to serve Network Load on a secondary/non-firm basis from a non-designated Network Resource. | NITSSecondaryService | Identifies the secondary service requested for delivery on non-designated Network Resource (see above). 3/14/12 accepted |
|  |  |  |  | NITSSecondaryCapacity | Defines the secondary network transmission capacity being requested over time .(see above). |
|  |  |  |  | AuxiliaryTransmission | Optional information on Coordinated Requests (see above). |
|  |  | AddPTPService | Request for new PTP Transmission Service to be evaluated respecting the simultaneous release of a DNR and/or request for designation of alternate Network Resource(s) submitted as a concomitant request. | PTPService | Identifies the PTP Transmission Service being requested for Concomitant Evaluation. |
|  |  |  |  | PTPCapacity | Defines the transmission capacity being requested over time. |
|  |  |  |  | AuxiliaryTransmission | Optional information on Coordinated Requests (see above). |
| UpdateConcomitantRequest | The UpdateConcomitantRequest NITS OASIS INPUT Template is used to update elements of an existing pending set of requests submitted for Concomitant Evaluation to the Transmission Provider. 3/14/12 accepted | (Any of the NITS OASIS REQUEST Templates allowed under the NewConcomitantRequest NITS OASIS INPUT Template) | Used to alter the STATUS on the group of requests submitted for Concomitant Evaluation or associated DATA contained in the request as allowed within the context of that request and its current STATUS. |  |  |

002-101.3.1	INPUT Template Definitions

3/14/12 accepted

The OASIS shall support the following NITS OASIS INPUT Templates for the uploading of NITS related request information. NITS OASIS INPUT Template information is conveyed via the first record in a NITS CSV Format submitted data message, the INPUT Header Record.

3/14/12 accepted

The following Data Elements shall be supported by all defined NITS OASIS INPUT Templates, and included in the INPUT Header Record’s Header Data Element List.

NITS OASIS Input Template Data Element List – Upload:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| VERSION | Optional | Defines the OASIS S&CP Protocol Version for all NITS OASIS INPUT Template information specified; if null, OASIS defaults to the current VERSION. |
| PRIMARY_PROVIDER_CODE | Required | Defines by registered entity code the Transmission Provider the NITS OASIS INPUT Template is being submitted to. |
| PRIMARY_PROVIDER_DUNS | Optional | Defines by registered entity code DUNS the Transmission Provider the NITS OASIS INPUT Template is being submitted to; if null, OASIS defaults to the DUNS registered under the PRIMARY_PROVIDER_CODE. |
| RETURN_TZ | Optional | Defines the time zone to be returned for all date/time related NITS OASIS INPUT Template Data Elements; if null, OASIS defaults to the prevailing time zone associated with the Transmission Provider. |

NITS OASIS Input Template Data Element List – Response:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| VERSION | Required | OASIS S&CP Protocol Version for all NITS OASIS INPUT Template information returned in the response. |
| PRIMARY_PROVIDER_CODE | Required | Registered entity code of the Transmission Provider generating the NITS OASIS INPUT Template response. |
| PRIMARY_PROVIDER_DUNS | Required | Registered entity DUNS of the Transmission Provider generating the NITS OASIS INPUT Template response. |
| RETURN_TZ | Required | Time zone used for return of all date/time related template Data Elements in NITS OASIS INPUT Template response. |
| TIME_STAMP | Required | Time stamp associated with the OASIS date/time the NITS OASIS INPUT Template response was generated. |
| INPUT_STATUS | Required | Value indicating success/failure of OASIS to process the submitted INPUT Template; 200 = Success. Failure indicates that some portion of the uploaded INPUT Template was malformed. 3/14/12 accepted |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in processing the submitted INPUT Template. |

Example:

INPUT=NewNITSRequest,VERSION,PRIMARY_PROVIDER_CODE,PRIMARY_PROVIDER_DUNS,RETURN_TZ,TIME_STAMP↵

,2.0,ATP,123456789,ES,↵

002-101.3.1.1	Submit New Network Integration Transmission Service Request (NewNITSRequest NITS OASIS INPUT Template)

3/14/12 bold accepted not accepted nits

The NewNITSRequest NITS OASIS INPUT Template is used to either 1) open a new OASIS workspace and begin the process for submitting a new NITS Application, or 2) enter one or more new NITS requests in association with a pending or completed NITS Application.

3/14/12 accepted

Two special REQUEST Templates, CopyNITSApplication and CopyNITSRequest, are used exclusively for copying existing NITS request information into the Pre-submittal Workspace that may then be updated prior to being submitted as new NITS request(s). These two REQUEST Templates must be submitted separately from any other requests and use a different REQUEST Template Header Record format than normal NITS request submissions as specified in Business Practice Standards WEQ-002-101.3.2.1.

3/14/12 accepted

To begin the NITS Application process on OASIS, the Eligible Customer must submit the NewNITSRequest NITS OASIS INPUT Template containing a single NewNITSApplication NITS OASIS REQUEST Template describing the basic attributes for the service being requested. Once a corresponding APPLICATION_REF has been assigned to the NITS Application, subsequent NewNITSRequest NITS OASIS INPUT Template submissions may be used to submit any of the supported NITS OASIS REQUEST Templates and their associated NITS OASIS DATA Templates as appropriate that will be associated with that new NITS Application.

3/14/12 accepted

Each NITS OASIS REQUEST Template submitted with the NewNITSRequest NITS OASIS INPUT Template shall be assigned a unique ASSIGNMENT_REF reference identifier by OASIS which is returned in the NITS OASIS REQUEST Template response.

The initial STATUS assigned to each new request submitted with the NewNITSRequest NITS OASIS INPUT Template shall be restricted to either PRESUBMITTED or QUEUED depending on the current STATUS of the associated NITS Application referenced in the APPLICATION_REF Data Element as follows.

If the NITS Application has a STATUS of PRESUBMITTED, all new NITS requests submitted referencing that NITS Application shall be restricted to the having the initial STATUS value of PRESUBMITTED.

3/14/12 accepted

If the NITS Application has a STATUS of CONFIRMED or any other intermediate pending state value (e.g., DEFICIENT), new NITS requests may be submitted as either PRESUBMITTED or QUEUED. Requests that are PRESUBMITTED will not be considered part of the NITS Application until they are completed and updated using the UpdateNITSRequest NITS OASIS INPUT Template to set the STATUS to QUEUED for evaluation by the Transmission Provider.

3/14/12 accepted

If the NITS Application STATUS is in any final state other than CONFIRMED, any attempts to submit a new request referencing that NITS Application shall be rejected with an error.

The following NITS OASIS REQUEST Templates shall be supported by the NewNITSRequest NITS OASIS INPUT Template:

NewNITSApplication (must appear alone)

3/14/12 accepted

ModifyNITSService

ModifyNITSCustomer

AddNITSAgent

ModifyNITSAgent

AddNITSLoad

ModifyNITSLoad

AddNITSGeneration

ModifyNITSGeneration

AddNITSResource

ModifyNITSResource

AddNITSDNR

TerminateNITSDNR

AddNITSSecondary

TerminateNITSSecondary

AddNITSAncillary

002-101.3.1.2	Update Network Integration Transmission Service Request (UpdateNITSRequest NITS OASIS INPUT Template)

3/14/12 bold accepted not accepted nits

The UpdateNITSRequest NITS OASIS INPUT Template is used to change the STATUS and/or information associated with one or more NITS OASIS REQUEST Templates submitted to OASIS using the NewNITSRequest NITS OASIS INPUT Template.

The specific requests to be updated using the UpdateNITSRequest NITS OASIS INPUT Template are identified by the name of each NITS OASIS REQUEST Template included in the NITS OASIS INPUT Template submission, the designated APPLICATION_REF identifying the specific NITS Application the request is a part of, and the unique OASIS assigned ASSIGNMENT_REF that was returned in response to the initial request submission. OASIS shall return an error if the supplied NITS OASIS REQUEST Template name, NITS Application, and associated ASSIGNMENT_REF do not match an existing NITS request.

3/14/12 accepted

The NewNITSApplication NITS OASIS REQUEST Template must be submitted as the only request in the UpdateNITSRequest NITS OASIS INPUT template submission. Additional special handling for updating the NewNITSApplication NITS OASIS REQUEST Template are described in the NITS OASIS REQUEST Template definition. There are no other restrictions with respect to the order or number of NITS OASIS REQUEST Template submissions that may be included and updated using the UpdateNITSRequest NITS OASIS INPUT Template.

3/14/12 accepted

All NITS OASIS REQUEST Templates shall support the updating of the STATUS and CUSTOMER_COMMENTS Data Elements in the REQUEST Header. The valid STATUS Data Element values that may be set by the Eligible Customer or Transmission Customer are specified in Business Practice Standard WEQ-013-101. Requests that are in a final state as established by the STATUS state transition diagram may not be updated, and any attempt to update these requests will be returned in error.

3/14/12 accepted

Each NITS OASIS REQUEST Template may permit the updating of information contained in one or more NITS OASIS DATA Templates associated with the given request. The NITS OASIS DATA Templates permitted in association with each NITS OASIS REQUEST Template are specified in the NITS OASIS REQUEST Template definition. Any NITS OASIS DATA Template(s) included in association with a NITS OASIS REQUEST Template referenced in the UpdateNITSRequest NITS OASIS INPUT Template shall be handled as follows.

3/14/12 accepted

If the NITS OASIS DATA Template referenced is not already part of the request as submitted in a preceding NewNITSRequest or UpdateNITSRequest NITS OASIS INPUT Template submission, the submitted information shall be incorporated into and treated as part of the updated request.  If the NITS OASIS DATA Template referenced has already been included in a prior NewNITSRequest or UpdateNITSRequest NITS OASIS INPUT Template submission, the information provided in the subsequent UpdateNITSRequest NITS OASIS INPUT Template shall replace the prior information in its entirety. An exception to this handling of updates, i.e., replacement of data in its entirety, must be implemented for submission of the AuxiliaryTransmission NITS OASIS DATA Template. The AuxiliaryTransmission NITS OASIS DATA Template shall support the incremental addition and deletion of information as specified for this template in Business Practice Standard WEQ-002-101.3.3.13. Submission of a given NITS OASIS DATA Template with no (0) corresponding data value records specified following the NITS OASIS DATA Template Header Record shall effectively remove that NITS OASIS DATA Template and all associated information from inclusion in the associated request.

3/14/12 accepted

The following NITS OASIS REQUEST Templates shall be supported by the UpdateNITSRequest NITS OASIS INPUT Template:

NewNITSApplication (must appear alone)

3/14/12 accepted

ModifyNITSService

ModifyNITSCustomer

AddNITSAgent

ModifyNITSAgent

AddNITSLoad

ModifyNITSLoad

AddNITSGeneration

ModifyNITSGeneration

AddNITSResource

ModifyNITSResource

AddNITSDNR

TerminateNITSDNR

AddNITSSecondary

TerminateNITSSecondary

AddNITSAncillary

002-101.3.1.3	Submit New Concomitant Request (NewConcomitantRequest NITS OASIS INPUT Template)

3/14/12 accepted

The NewConcomitantRequest NITS OASIS INPUT Template is used to submit a set of qualifying requests to be treated and acted on as a single action. Only a subset of valid NITS OASIS REQUEST Templates are allowed to be submitted as part of a concomitant request to OASIS.

3/14/12 accepted

The NewConcomitantRequest NITS OASIS INPUT Template submission must contain at least one TerminateNITSDNR NITS OASIS REQUEST Template, and at most one other request for AddNITSDNR, AddNITSSecondary, or AddPTPService NITS OASIS REQUEST Template.

The NITS Application specified in the APPLICATION_REF Data Element for each request included in the NewConcomitantRequest NITS OASIS INPUT Template must be identical and must reference a CONFIRMED NITS Application.

3/14/12 accepted

OASIS shall assign and return a unique ASSIGNMENT_REF identifier to each submitted request. OASIS shall also assign a single unique TRANSACTION_REF identifier that will be associated to each NewConcomitantRequest NITS OASIS REQUEST Template.

3/14/12 accepted

The individual NITS OASIS REQUEST Template submissions that comprise the set of request to be treated as concomitant requests may be set with an initial STATUS of PRESUBMITTED and updated or deleted as long as their request STATUS remains PRESUBMITTED. However, when ready for final submission to the Transmission Provider, all of the valid requests to be treated as concomitant requests (by TRANSACTION_REF) must be simultaneously updated to set each request STATUS to QUEUED using the UpdateConcomitantRequest NITS OASIS INPUT Template. Any attempt to set a given concomitant request to QUEUED when there are additional concomitant requests (by TRANSACTION_REF) set to PRESUBMITTED shall return an error.

3/14/12 accepted

The PRECONFIRMED Data Element value must be identical (YES or NO) for all requests at the time the concomitant requests are set to QUEUED.

OASIS shall validate all NITS OASIS REQUEST Template information submitted in the NewConcomitantRequest NITS OASIS INPUT Template. If any request is found to be in error, all requests in the template submission shall be returned with an error.

3/14/12 accepted

The following NITS OASIS REQUEST Templates shall be supported by the NewConcomitantRequest NITS OASIS INPUT Template:

TerminateNITSDNR (must be included)

AddNITSDNR

AddNITSSecondary

AddPTPService

002-101.3.1.4	Update Concomitant Request (UpdateConcomitantRequest NITS OASIS INPUT Template)

3/14/12 accepted

The UpdateConcomitantRequest NITS OASIS INPUT Template is primarily used to update the STATUS of a set of qualifying requests that were submitted for Concomitant Evaluation.  When attempting to update STATUS, all of the requests that were included in the original submission under the NewConcomitantRequest NITS OASIS INPUT Template must be included in the UpdateConcomitantRequest NITS OASIS INPUT Template submission.  Any new STATUS value to be set into each of the requests referenced in the UpdateConcomitantRequest NITS OASIS INPUT Template submission must be identical and reflect a valid STATUS state transition in accordance with Business Practice Standard WEQ-001-107 and WEQ-013.

3/14/12 accepted

The new request for service to be considered concomitantly with the termination of a DNR may be considered to be a Coordinated Request as allowed under Business Practice Standard WEQ-001-23 provided they are initially submitted with the CG_STATUS Data Element set to PROPOSED. Coordinated Requests must be updated to identify all associated requests and reservations that comprise the Coordinated Group of requests submitted across multiple Transmission Providers using this UpdateConcomitantRequest NITS OASIS INPUT Template with the appropriate supplied for the applicable AddNITSDNR, AddNITSecondary, or AddPTPService NITS OASIS REQUEST Template.

3/14/12 accepted

The following NITS OASIS REQUEST Templates shall be supported by the UpdateConcomitantRequest NITS OASIS INPUT Template:

TerminateNITSDNR

AddNITSDNR

AddNITSSecondary

AddPTPService

002-101.3.2	REQUEST Template Definitions

3/14/12 accepted

There are two distinct types of REQUEST Templates supported depending on the nature of the requested action. The CopyNITSApplication and CopyNITSRequest requests are used specifically to manipulate the Pre-submittal Workspace and are not requests directly related to the arrangement of NITS on OASIS. These two REQUEST Templates have their own unique REQUEST Header Record format as specified in Business Practice Standard WEQ-002-101.3.2.1. There are no associated DATA Templates for these two requests. All of the other NITS OASIS REQUEST Templates support the submission of information to the Transmission Provider for the arrangement of NITS on OASIS. The remainder of this section pertains to all of the other NITS requests supported by OASIS excluding the CopyNITSApplication and CopyNITSRequest requests.

3/14/12 accepted

The OASIS shall support the following NITS OASIS REQUEST Templates for the uploading of NITS related request information. NITS OASIS REQUEST Template information is conveyed via the REQUEST Header Record and the immediately following Header Data Element values record and any additional NITS OASIS DATA Templates supported by the request. Each NITS OASIS REQUEST Template submission shall support only a single instance of a given NITS OASIS DATA Template. However, multiple different NITS OASIS DATA Templates may be conveyed within a single NITS OASIS REQUEST Template submission depending on the nature of the request submitted.

3/14/12 accepted

Each successful NITS OASIS REQUEST Template submission shall be assigned an OASIS reference identifier returned in the ASSIGNMENT_REF header Data Element. ASSIGNMENT_REF must be a unique value for a given Transmission Provider at a minimum, and must also be unique from all other ASSIGNMENT_REF values assigned for that Transmission Provider in association with either transmission or ancillary service requests/reservations submitted using the general OASIS and PTP OASIS Templates specified under Business Practice Standard WEQ-002-4.

3/14/12 accepted

All requests received in association with a pending NITS Application which has been set to a STATUS of QUEUED, RECEIVED, or DEFICIENT shall have the TIME_QUEUED Data Element set to the time that the initial NITS Application request (NewNITSApplication NITS OASIS REQUEST Template) was initially submitted for evaluation with the STATUS of QUEUED. All requests submitted after the NITS Application has been marked as COMPLETED or ultimately confirmed by the Eligible Customer shall have the TIME_QUEUED data element set to the current time at the time of submittal of the request. Submission of any request referencing a NITS Application that is in a final state other than CONFIRMED shall be returned with an error.

3/14/12 accepted

The following Data Elements shall be supported by all defined NITS OASIS REQUEST Templates, and included in the REQUEST Header Record’s Header Data Element List.

NITS OASIS REQUEST Template Data Element List – Request Upload:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| TRANSACTION_REF | Required* | Required only on submission of UpdateConcomitantRequest NITS OASIS INPUT Template; must be null in all other instances. |
| APPLICATION_REF | Required* | Must be null in NewNITSApplication NITS OASIS REQUEST Template submission only; required in all other instances. |
| ASSIGNMENT_REF | Required* | Required only on submission of UpdateNITSRequest and UpdateConcomitantRequest NITS OASIS INPUT Templates; must be null in all other instances. 3/14/12 accepted |
| PRECONFIRMED | Optional | Specifies if request is pre-confirmed or not on initial submission with STATUS=QUEUED; if null, current value retained with default of ‘NO’. |
| STATUS | Optional | Specifies initial or any change in request STATUS value; if null, current value retained. |
| STATUS_COMMENTS | Optional | Optional comments that can be used to describe action taken in setting STATUS; if null, current value retained. |

NITS OASIS REQUEST Template Data Element List – Request/Query Response:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| TRANSACTION_REF | May be null | Returns unique OASIS assigned transaction reference identifier for concomitant requests only; null returned in all other instances. |
| APPLICATION_REF | Required | Returns the unique OASIS assigned application reference identifier required to associate request with given NITS Application. |
| ASSIGNMENT_REF | Required | Returns unique OASIS assigned request reference identifier for each request. |
| PRECONFIRMED | Required | Returns current value for pre-confirmation status of request. |
| STATUS | Required | Returns current request STATUS value. |
| STATUS_COMMENTS | May be null | Optional comments set by Eligible Customer or Transmission Customer or Transmission Provider (Seller) to describe action taken in setting STATUS; if null, current value retained. |
| CUSTOMER_COMMENTS | May be null | Returns current Eligible Customer or Transmission Customer comments associated with request. |
| PRIMARY_PROVIDER_COMMENTS | May be null | Returns any comments associated with request as set by the Transmission Provider; returns null value while request is PRESUBMITTED. |
| SELLER_COMMENTS | May be null | Returns any comments associated with the request as set by the Transmission Provider for all but the AddNITSAncillary NITS OASIS REQUEST Template request which is set by the ancillary service provider; returns null value while request is PRESUBMITTED. |
| SELLER_CODE | Required* | Set to PRIMARY_PROVIDER_CODE for all but the AddNITSAncillary NITS OASIS REQUEST Template request which is set to identify the ancillary service provider; *returns null value while request is PRESUBMITTED. |
| SELLER_DUNS | Required* | Set to PRIMARY_PROVIDER_DUNS for all but the AddNITSAncillary NITS OASIS REQUEST Template request which is set to identify the ancillary service provider; *returns null value while request is PRESUBMITTED. |
| CUSTOMER_CODE | Required* | Set to the Eligible Customer or Transmission Customer’s registered entity code at the time of initial submission of the REQUEST Template; *returns null value while request is PRESUBMITTED. |
| CUSTOMER_DUNS | Required* | Set to the Eligible Customer or Transmission Customer’s registered DUNS at the time of initial submission of the REQUEST Template; *returns null value while request is PRESUBMITTED. |
| AFFILIATE_FLAG | May be null | Identifies if the Eligible Customer or Transmission Customer submitting the request is affiliated with the Transmission Provider (Seller); returns null value while request is PRESUBMITTED. |
| TIME_QUEUED | May be null | Date/time that the request was submitted as QUEUED; returns null value while request is PRESUBMITTED. |
| RESPONSE_TIME_LIMIT | May be null | Once submitted and under evaluation by the Transmission Provider, this field may be set by the Transmission Provider with a deadline for Eligible Customer or Transmission Customer action dependent on the nature of the request and applicable Business Practice Standards; returns null value while request is PRESUBMITTED. 3/14/12 accepted |
| CREATEDBY_CODE | Required | Entity code associated with the user that originally queued the request for evaluation; returns null value while request is PRESUBMITTED. |
| MODIFIEDBY_CODE | Required | Entity code associated with the user that last updated the request status or any NITS OASIS DATA Templates associated with the request; returns null value while request is PRESUBMITTED. |
| TIME_OF_LAST_UPDATE | Required | Date/time the request status or any NITS OASIS DATA Templates associated with the request was last updated, i.e., time OASIS acted on request submission. |
| REQUEST_STATUS | Required | Value indicating success/failure of OASIS to process the submitted request; 200 = Success. Only returned in response to NITS OASIS INPUT Template upload; not included in NITS OASIS QUERY Template response. |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in processing the submitted request. Only returned in response to NITS OASIS INPUT Template upload; not included in NITS OASIS QUERY Template response. 3/14/12 accepted |

Example:

[NOTE: Whitespace (spaces) included after each comma delimiter to improve readability and would not be present in actual template data submissions.]

(Input)

REQUEST=NewNITSApplication, TRANSACTION_REF, APPLICATION_REF, ASSIGNMENT_REF, PRECONFIRMED, STATUS, STATUS_COMMENTS, CUSTOMER_COMMENTS ↵

, , , , NO, PRESUBMITTED, , ”Customer requesting NITS service to City of Abc”↵

DATA=…		<one or more DATA Templates associated with the REQUEST>

…

(Response)

REQUEST=NewNITSApplication, TRANSACTION_REF, APPLICATION_REF, ASSIGNMENT_REF, PRECONFIRMED, STATUS, STATUS_COMMENTS, CUSTOMER_COMMENTS, PRIMARY_PROVIDER_COMMENTS, SELLER_COMMENTS, SELLER_CODE, SELLER_DUNS, CUSTOMER_CODE, CUSTOMER_DUNS, AFFILIATE_FLAG, TIME_QUEUED, RESPONSE_TIME_LIMIT, CREATEDBY_CODE, MODIFIEDBY_CODE, TIME_OF_LAST_UPDATE, REQUEST_STATUS, ERROR_MESSAGE↵

, , 12, 78, NO, , ”Customer requesting NITS service to City of Abc”, , , ATP, 123456789, APSE, 987654321, NO, 20111108095702ES, , APSE, APSE, 20111108095702ES ,200, Success↵

3/14/12 not accepted

002-101.3.2.1	Copy Network Integration Transmission Service Application (CopyNITSApplication NITS OASIS REQUEST Template)

3/14/12 bold accepted not accepted nits

The CopyNITSApplication NITS OASIS REQUEST Template may be submitted to copy a specified set of NITS requests under an existing NITS Application to the OASIS Pre-submittal Workspace under a new NITS Application.

3/14/12 accepted

All NITS REQUEST Templates to be copied must refer to the same NITS Application by APPLICATION_REF. The entity submitting this request must be the Eligible Customer or Transmission Customer, defined in the NITS Application to be copied effective at the time of submission. If the Entity submitting the CopyNITSApplication request does not meet this requirement no requests will be copied.

3/14/12 accepted

Each NITS REQUEST Template that is to be copied to the Pre-submittal Workspace under a new NITS Application must be specified by its corresponding ASSIGNMENT_REF. Each NITS REQUEST Template to be copied must be specified in separate REQUEST Data records, one record per request, immediately following the REQUEST Header record. Each REQUEST Data record contains the appropriate data element values corresponding to each data element name listed in the REQUEST Header record.

3/14/12 accepted

If the request’s ASSIGNMENT_REF does not exist under the specified APPLICATION_REF, that request will not be copied and an error indication will be returned in the response data record corresponding with the invalid request.

If the NewNITSApplication REQUEST Template is included in the list of NITS requests to be copied, it will be ignored. OASIS will always create a NewNITSApplication REQUEST Template under a newly assigned APPLICATION_REF and ASSIGNMENT_REF in response to the CopyNITSApplication request and return the information associated with that NewNITSApplication REQUEST Template as the first record in the response. If the APPLICATION_REF specified in the input REQUEST Template is not CONFIRMED, the current contents from the existing NewNITSApplication REQUEST Template associated with the APPLICATION_REF will be copied to the NewNITSApplication REQUEST Template created. If the APPLICATION_REF specified in the input REQUEST Template is CONFIRMED, the current active information associated with the NITSService, NITSCustomer and NITSAgent information will be used to seed the NewNITSApplication REQUEST Template created.

3/14/12 accepted

The STATUS of the NewNITSApplication and all copied REQUEST Templates will be set to PRESUBMITTED. All DATA Templates associated with the copied REQUEST Templates will also be copied into the Pre-submittal Workspace.

3/14/12 accepted

The following Data Elements shall be supported by the CopyNITSApplication REQUEST Template, and included in the REQUEST Header Record’s Header Data Element List.

CopyNITSApplication REQUEST Template Data Element List – Request Upload:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| APPLICATION_REF | Required | Must identify the specific NITS Application associated which each of the associated NITS REQUEST Templates to be copied to the Pre-submittal Workspace. If multiple NITS REQUEST Templates are to be copied, they all must refer to the same APPLICATION_REF. 3/14/12 accepted |
| ASSIGNMENT_REF | Required | Identifies the specific NITS REQUEST Template(s) under the NITS Application that is to be copied to the Pre-submittal Workspace as part of a NewNITSApplication. 3/14/12 accepted |

CopyNITSApplication REQUEST Template Data Element List – Request Response:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| APPLICATION_REF | Required | Identifies the specific NITS Application associated which each of the associated NITS REQUEST Templates copied to the Pre-submittal Workspace. If multiple NITS REQUEST Templates are to be copied, they all must refer to the same APPLICATION_REF. 3/14/12 accepted |
| ASSIGNMENT_REF | May be null | Identifies the specific NITS REQUEST Template(s) under the original NITS Application that has been copied to the Pre-submittal Workspace. This value will be null only for the first record in the response corresponding to the NewNITSApplication REQUEST Template created by the CopyNITSApplication request. 3/14/12 accepted |
| NEW_APPLICATION_REF | Required | Identifies the new APPLICATION_REF assigned to the new NITS Application created and the associated REQUEST templates copied into the Pre-submittal Workspace. 3/14/12 accepted |
| NEW_ASSIGNMENT_REF | Required | Identifies the new ASSIGNMENT_REF assigned to the new NITS REQUEST Template copied into the Pre-submittal Workspace. 3/14/12 accepted |
| REQUEST_STATUS | Required | Value indicating success/failure of OASIS to copy the the specified NITS REQUEST Template to the Pre-submittal Workspace; 200 = Success. 3/14/12 accepted |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in copying the specified NITS REQUEST Template to the Pre-submittal Workspace. 3/14/12 accepted |

Example:

(Input)

REQUEST=CopyNITSApplication, APPLICATION_REF, ASSIGNMENT_REF↵

,12,47↵

,12,53↵

,12,79↵

,12,42↵

(Response)

REQUEST=CopyNITSApplication, APPLICATION_REF, ASSIGNMENT_REF, NEW_APPLICATION_REF, NEW_ASSIGNMENT_REF, REQUEST_STATUS, ERROR_MESSAGE↵

,12,,61,236,200,Success↵    (Identifies the NewNITSApplication REQUEST Template created)

,12,47,61,237,200,Success↵

,12,53,61,238,200,Success↵

,12,79,61,241,200,Success↵

,12,42,61,243,200,Success↵

3/14/12 accepted

002-101.3.2.2	Copy Network Integration Transmission Service Request (CopyNITSRequest NITS OASIS REQUEST Template)

3/14/12 bold accepted not accepted nits

The CopyNITSRequest NITS OASIS REQUEST Template may be submitted to copy a specified set of NITS requests under one or more existing NITS Applications into the OASIS Pre-submittal Workspace for subsequent editing and submission as new NITS requests.

3/14/12 accepted

Each NITS REQUEST Template to be copied must be specified in separate REQUEST Data records immediately following the REQUEST Header record. Each REQUEST Data record contains the appropriate data element values corresponding to each data element name listed in the REQUEST Header record.

The NITS REQUEST Template(s) to be copied to the Pre-submittal Workspace must be specified by its corresponding APPLICATION_REF and ASSIGNMENT_REF. The entity submitting the CopyNITSRequest must be the Eligible Customer or Transmission Customer, defined in the NITS Application associated with the existing REQUEST Template(s) to be copied effective at the time of submission.  The APPLICATION_REF to be used in the newly created request must be specified in the NEW_APPLICATION_REF Data Element and must reference an existing APPLICATION_REF that the submitting Entity is identified as being the Eligible Customer or Transmission Customer, in that NITS Application effective at the time of submission.

3/14/12 accepted

If the request’s APPLICATION_REF and/or ASSIGNMENT_REF specified does not exist or is not eligible to be copied by the submitting Entity, that request will not be copied and an error indication will be returned in the response data record corresponding with the invalid request.

The NewNITSApplication REQUEST Template may not be copied using the CopyNITSRequest REQUEST Template; the CopyNITSApplication REQUEST Template must be used to create a copy of an existing NITS Application.

The STATUS of all copied REQUEST Templates will be set to PRESUBMITTED. All DATA Templates associated with the copied REQUEST Template will also be copied into the Pre-submittal Workspace.

3/14/12 accepted

The following Data Elements shall be supported by the CopyNITSRequest REQUEST Template, and included in the REQUEST Header Record’s Header Data Element List.

CopyNITSRequest REQUEST Template Data Element List – Request Upload:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| APPLICATION_REF | Required | Must identify the specific NITS Application associated which each of the NITS REQUEST Templates to be copied to the Pre-submittal Workspace. 3/14/12 accepted |
| ASSIGNMENT_REF | Required | Identifies the specific NITS REQUEST Template(s) under the specified NITS Application that is to be copied to the Pre-submittal Workspace. 3/14/12 accepted |
| NEW_APPLICATION_REF | Required | Must refer to a valid existing NITS Application to which the copied REQUEST Template will be associated. |

CopyNITSApplication REQUEST Template Data Element List – Request Response:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| APPLICATION_REF | Required | Indentifies the specific NITS Application associated which each of the NITS REQUEST Templates copied to the Pre-submittal Workspace. 3/14/12 accepted |
| ASSIGNMENT_REF | Required | Identifies the specific NITS REQUEST Template(s) under the specified NITS Application that has been copied to the Pre-submittal Workspace. 3/14/12 accepted |
| NEW_APPLICATION_REF | Required | Identifies the APPLICATION_REF assigned to the copy of the NITS REQUEST Template created in the Pre-submittal Workspace. 3/14/12 accepted |
| NEW_ASSIGNMENT_REF | Required | Identifies the new ASSIGNMENT_REF assigned to the new NITS REQUEST Template copied into the Pre-submittal Workspace. 3/14/12 accepted |
| REQUEST_STATUS | Required | Value indicating success/failure of OASIS to copy the specified NITS REQUEST Template to the Pre-submittal Workspace; 200 = Success. 3/14/12 accepted |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in copying the specified NITS REQUEST Template to the Pre-submittal Workspace. 3/14/12 accepted |

Example:

(Input)

REQUEST=CopyNITSRequest, APPLICATION_REF, ASSIGNMENT_REF, NEW_APPLICATION_REF↵

,12,47,78↵

,61,237,61↵

,12,79,61↵

(Response)

REQUEST=CopyNITSRequest, APPLICATION_REF, ASSIGNMENT_REF, NEW_APPLICATION_REF, NEW_ASSIGNMENT_REF, REQUEST_STATUS, ERROR_MESSAGE↵

,12,47,78,319,200,Success↵

,61,237,61,322,200,Success↵

,12,79,61,323,200,Success↵

002-101.3.2.3	Request for New Network Integration Transmission Service Application (NewNITSApplication NITS OASIS REQUEST Template)

3/14/12 bold accepted not accepted nits

The NewNITSApplication NITS OASIS REQUEST Template must be submitted as the only request contained within the NewNITSRequest NITS OASIS INPUT Template and will result in OASIS opening a new workspace for creation of the full initial NITS Application and returning a unique APPLICATION_REF to be used in all subsequent requests associated with this NITS Application. The initial STATUS may be set to either PRESUBMITTED or QUEUED. The PRESUBMITTED STATUS shall allow the Eligible Customer to add and update additional NITS requests to form the final NITS Application before it is presented to the Transmission Provider for evaluation.

3/14/12 accepted

The initial term of service being requested and the identification of the Eligible Customer making the NITS Application must be supplied using the NITSService and NITSCustomer NITS OASIS DATA Templates. OASIS shall associate the new NITS Application with the registered entity that is submitting the request on OASIS. If the entity submitting the NewNITSAppliction NITS OASIS REQUEST Template request is not the Eligible Customer, that entity must be identified as a prospective Designated Agent of the Eligible Customer by inclusion of the NITSAgent NITS OASIS DATA Template as part of the NewNITSApplication request. If the entity submitting the NewNITSApplication request is not the Eligible Customer specified via the NITSCustomer template, or is not identified as a Designated Agent via the NITSAgent template, OASIS shall reject the NewNITSApplication request and return an error.

3/14/12 accepted

While the NewNITSApplication NITS OASIS REQUEST Template request is PRESUBMITTED, the information in any of the NITS OASIS DATA Templates associated with the request may be updated/replaced through the UpdateNITSRequest NITS OASIS INPUT Template. All related requests submitted with reference to a PRESUBMITTED NITS Application must also be set to the initial STATUS of PRESUBMITTED. Any request that is no longer to be considered part of the initial NITS Application may be updated by the Eligible Customer with the STATUS of DELETED to remove that request from further consideration prior to setting the NewNITSApplication NITS OASIS REQUEST Template STATUS from PRESUBMITTED to QUEUED.

3/14/12 accepted

Once all required information associated with the initial NITS Application has been established through one or more related NITS OASIS REQUEST Template submissions, the Eligible Customer shall submit the NITS Application for evaluation by the Transmission Provider by updating the NewNITSApplication NITS OASIS REQUEST Template request to QUEUED. OASIS shall simultaneously update all PRESUBMITTED requests referencing that NITS Application to QUEUED.

3/14/12 accepted

After the NewNITSApplication NITS OASIS REQUEST Template request is QUEUED, no further updates to the NITSService, NITSCustomer, or NITSAgent (if applicable) NITS OASIS DATA Templates shall be allowed. If changes are required and permitted by the Transmission Provider, the Eligible Customer must use the ModifyNITSService, ModifyNITSCustomer and Add/ModifyNITSAgent NITS OASIS REQUEST Templates to submit those modifications for Transmission Provider review and approval.

3/14/12 accepted

The UpdateNITSRequest NITS OASIS INPUT Template is used by the Eligible Customer to make any appropriate changes in the NewNITSApplication NITS OASIS REQUEST Template request STATUS or associated NITS OASIS DATA Template information as allowed based on the current STATUS of the request. The NewNITSApplication NITS OASIS REQUEST Template must be the only request specified in the UpdateNITSRequest NITS OASIS INPUT Template submission, and may not be included with any other REQUEST Template. Any attempts to update the NewNITSApplication NITS OASIS REQUEST Template or associated NITS OASIS DATA Template information by an entity that is not the Eligible Customer or Transmission Customer effective at the time the request update is submitted shall be blocked by OASIS.

3/14/12 accepted

The following NITS OASIS DATA Templates shall be supported by the NewNITSApplication NITS OASIS REQUEST Template:

NITSService (must be supplied)

NITSCustomer (must be supplied)

NITSAgent

The REQUEST Template Data Elements for the Input and Response shall be as specified in Business Practice Standard WEQ-002-101.3.2.

002-101.3.2.4	Request to Modify Network Integration Transmission Service Service (ModifyNITSService NITS OASIS REQUEST Template)

3/14/12 bold accepted not accepted nits

The ModifyNITSService NITS OASIS REQUEST Template is used to request a modification to the existing term of service under the NITS Application. The information regarding the modification being requested shall be included in the associated NITSService NITS OASIS DATA Template. Modifications to the NITS Application while it is still pending final confirmation may be rejected by the Transmission Provider as dictated by their Business Practices through setting of the request’s STATUS as appropriate.

3/14/12 accepted

While set to the STATUS of PRESUBMITTED, the information supplied in the NITSService NITS OASIS DATA Template may be updated using the UpdateNITSRequest NITS OASIS INPUT Template. Once QUEUED for evaluation, the UpdateNITSRequest NITS OASIS INPUT Template is limited to only making appropriate changes to the ModifyNITSService NITS OASIS REQUEST Template request STATUS.

3/14/12 accepted

Any attempts to submit or update the ModifyNITSService NITS OASIS REQUEST Template request by an entity that is not the current Eligible Customer or Transmission Customer effective at the time the request is submitted shall be blocked by OASIS.

3/14/12 accepted

The following NITS OASIS DATA Template shall be supported by the ModifyNITSService NITS OASIS REQUEST Template:

NITSService (must be supplied)

The REQUEST Template Data Elements for the Input and Response shall be as specified in Business Practice Standard WEQ-002-101.3.2.

002-101.3.2.5	Request to Modify Network Integration Transmission Service Customer (ModifyNITSCustomer NITS OASIS REQUEST Template)

3/14/12 bold accepted not accepted nits

The ModifyNITSCustomer NITS OASIS REQUEST Template is used to request a transfer of obligations under the NITS Application to another Eligible Customer or simply to update contact information or dynamic notification settings for the existing Eligible Customer or Transmission Customer. The information associated with the modification being requested shall be included in an associated NITSCustomer NITS OASIS DATA Template. Modifications to the Eligible Customer or Transmission Customer while the NITS Application is still pending final confirmation may be rejected by the Transmission Provider as dictated by their Business Practices through setting of the request’s STATUS as appropriate.

3/14/12 accepted

The ModifyNITSCustomer NITS OASIS REQUEST Template request must be submitted as PRECONFIRMED. There is no negotiation or final confirmation required for this request. Once verified with all affected parties and ACCEPTED by the Transmission Provider, the Eligible Customer or Transmission Customer obligations under the NITS Application shall be transferred to the new Eligible Customer or Transmission Customer if applicable.

3/14/12 accepted

While set to the STATUS of PRESUBMITTED, the information supplied in the NITSCustomer NITS OASIS DATA Template may be updated using the UpdateNITSRequest NITS OASIS INPUT Template. Once QUEUED for evaluation, the UpdateNITSRequest NITS OASIS INPUT Template is limited to only making appropriate changes to the ModifyNITSCustomer NITS OASIS REQUEST Template request STATUS.

3/14/12 accepted

Any attempts to submit or update the ModifyNITSCustomer NITS OASIS REQUEST Template request by an entity that is not the current Eligible Customer or Transmission Customer effective at the time the request is submitted shall be blocked by OASIS.

3/14/12 accepted

Upon final approval and confirmation of any change in the NITSCustomer NITS OASIS REQUEST Template information, OASIS shall set the EFFECTIVE_STOP_TIME of the currently active Eligible Customer or Transmission Customer information equal to the EFFECTIVE_START_TIME of the newly approved change in Eligible Customer or Transmission Customer. The EFFECTIVE_STOP_TIME of any new or modified Eligible Customer or Transmission Customer information shall be open ended and remain in effect until a subsequent request to ModifyNITSCustomer NITS OASIS REQUEST Template is received and approved. There shall be only one active Eligible Customer or Transmission Customer information record in effect at any point in time over the term of the NITS Application.

3/14/12 accepted

The following NITS OASIS DATA Template shall be supported by the ModifyNITSCustomer NITS OASIS REQUEST Template:

NITSCustomer (must be supplied)

The REQUEST Template Data Elements for the Input and Response shall be as specified in Business Practice Standard WEQ-002-101.3.2.

002-101.3.2.6	Request to Add an Agent (AddNITSAgent NITS OASIS REQUEST Template)

3/14/12 accepted

The AddNITSAgent NITS OASIS REQUEST Template is used to request the addition of a new entity to be recognized as an authorized Designated Agent for the Eligible Customer or Transmission Customer. This request may be submitted by the Eligible Customer or Transmission Customer, or any of the current Designated Agents effective at the time of submission.

3/14/12 accepted

If submitted with an initial STATUS of PRESUBMITTED, the information supplied in the NITSAgent NITS OASIS DATA Template may be updated using the UpdateNITSRequest NITS OASIS INPUT Template. Once QUEUED for evaluation, the UpdateNITSRequest NITS OASIS INPUT Template is limited to only making appropriate changes to the AddNITSAgent NITS OASIS REQUEST Template request STATUS. Any changes required to the Designated Agent information must then be made through submission of the ModifyNITSAgent NITS OASIS REQUEST Template.

3/14/12 accepted

Unless explicitly specified with an EFFECTIVE_STOP_TIME, Designated Agent information may be open ended until terminated through the ModifyNITSAgent NITS OASIS REQUEST Template request.

The following NITS OASIS DATA Templates shall be supported by the AddNITSAgent NITS OASIS REQUEST Template:

NITSAgent (must be supplied)

The REQUEST Template Data Elements for the Input and Response shall be as specified in Business Practice Standard WEQ-002-101.3.2.

002-101.3.2.7	Request to Modify an Agent (ModifyNITSAgent NITS OASIS REQUEST Template)

3/14/12 accepted

The ModifyNITSAgent NITS OASIS REQUEST Template is used to request a modification to an existing authorized Designated Agent for the Eligible Customer or Transmission Customer. This request may be submitted by the Eligible Customer or Transmission Customer, or any of the current Designated Agents effective at the time of submission.

3/14/12 accepted

Termination of an agency relationship with the Eligible Customer or Transmission Customer is effected by setting the EFFECTIVE_STOP_TIME in the NITSAgent NITS OASIS DATA Template using this ModifyNITSAgent request.

If submitted with an initial STATUS of PRESUBMITTED, the information supplied in the NITSAgent NITS OASIS DATA Template may be updated using the UpdateNITSRequest NITS OASIS INPUT Template. Once QUEUED for evaluation, the UpdateNITSRequest NITS OASIS INPUT Template is limited to only making appropriate changes to the ModifyNITSAgent NITS OASIS REQUEST Template request STATUS. Any changes required to the Designated Agent information must then be made through submission of another ModifyNITSAgent NITS OASIS REQUEST Template.

3/14/12 accepted

The following NITS OASIS DATA Templates shall be supported by the ModifyNITSAgent NITS OASIS REQUEST Template:

NITSAgent (must be supplied)

The REQUEST Template Data Elements for the Input and Response shall be as specified in Business Practice Standard WEQ-002-101.3.2.

002-101.3.2.8	Request to Add a Network Load (AddNITSLoad NITS OASIS REQUEST Template)

3/14/12 accepted

The AddNITSLoad NITS OASIS REQUEST Template is used to request the addition of a new load to be served under the NITS Application. This request may be submitted by the Eligible Customer or Transmission Customer, effective at the time of submission. The information describing the nature of the new Network Load is supplied in one or more related NITS OASIS DATA Templates that are supported for this request.

3/14/12 accepted

If submitted with an initial STATUS of PRESUBMITTED or when the request STATUS has been set to DEFICIENT by the Transmission Provider, any information supplied in any of the supported load NITS OASIS DATA Templates may be updated using the UpdateNITSRequest NITS OASIS INPUT Template. New data may be added to the AddNITSLoad NITS OASIS REQUEST Template request by submission of a previously omitted load NITS OASIS DATA Template. If information has already been submitted for a given NITS OASIS DATA Template, that information may be replaced in its entirety by re-submitting the correct information to replace the current information associated with the AddNITSLoad NITS OASIS REQUEST Template request.

Once QUEUED for evaluation or set to be reevaluated, the UpdateNITSRequest NITS OASIS INPUT Template is limited to only making appropriate changes to the AddNITSLoad NITS OASIS REQUEST Template request STATUS. Any changes required to the Network Load information must then be made through submission of the ModifyNITSLoad NITS OASIS REQUEST Template.

Each discreet load to be served under the Eligible Customer or Transmission Customer’s NITS Application must be assigned a Eligible Customer or Transmission Customer supplied, application unique LOAD_NAME to be used to reference that load in any associated NITSLoadDescription or NITSLoadForecast NITS OASIS DATA Templates information. Network Loads may be defined without being designated so that they may be included in the load forecast information supplied to the Transmission Provider under the NITS Application.

3/14/12 accepted

Submission of an AddNITSLoad NITS OASIS REQUEST Template request with reference to a LOAD_NAME that is currently defined under the NITS Application shall be returned with an error condition indicating that the named load already exists. The ModifyNITSLoad NITS OASIS REQUEST Template request shall be used to modify existing load designations and forecasts.

3/14/12 accepted

The Transmission Provider may specify the POD and/or Sink to be used in association with a given named Network Load. Multiple named Network Loads may be assigned the same POD and Sink if they are deemed electrically equivalent. The Transmission Provider may also require that a given named Network Load be disaggregated into more than one named Network Load and assigned different PODs and/or Sinks as required. The Eligible Customer or Transmission Customer may change the POD and/or Sink from that initially submitted using the UpdateNITSRequest INPUT Template.

3/14/12 accepted

All NITS OASIS DATA Templates included in a given AddNITSLoad NITS OASIS REQUEST Template request must reference the same unique LOAD_NAME; OASIS shall return an error on any attempt to submit a new or update an exsting AddNITSLoad NITS OASIS REQUEST Template request where the LOAD_NAME is not consistent across all associated NITS OASIS DATA Templates in the request.

The following NITS OASIS DATA Templates shall be supported by the AddNITSLoad NITS OASIS REQUEST Template:

NITSLoadDescription (must be supplied)

NITSLoadForecast (optional template)

The REQUEST Template Data Elements for the Input and Response shall be as specified in Business Practice Standard WEQ-002-101.3.2.

002-101.3.2.9	Request to Modify a Network Load (ModifyNITSLoad NITS OASIS REQUEST Template)

3/14/12 accepted

The ModifyNITSLoad NITS OASIS REQUEST Template is used to request the modification of the capacity designated or forecasted in association with an existing named Network Load under a NITS Application. This request may be submitted by the Eligible Customer or Transmission Customer, effective at the time of the request submission.

3/14/12 accepted

The information supplied in association with the ModifyNITSLoad NITS OASIS REQUEST Template request, if approved and confirmed, shall effectively replace any existing information associated with that named load under the NITS Application over the period of time specified in the NITS OASIS DATA Template(s) submitted as part of the ModifyNITSLoad NITS OASIS REQUEST Template request.

Removal of a named load from consideration under the NITS Application is accomplished by setting the EFFECTIVE_STOP_TIME in the NITSLoadDescription NITS OASIS DATA Template.  EFFECTIVE_STOP_TIME establishes the point in time after which all information that may have been provided in association with that Network Load. i.e., designations and forecasts, shall be considered terminated or no longer considered as part of the NITS Application.

3/14/12 accepted

Termination of only a portion of a designated Network Load or the incremental addition of capacity for the designated Network Load is accomplished by restating the NITSLoadForecast NITS OASIS DATA Template information over time with the new total MW capacity to be treated as designated Network Load under the NITS Application. If load forecast information is communicated to the Transmission Provider OFF_OASIS, the revised load information should reflect the new total load designated under the NITS Application.

3/14/12 accepted

New or revised load forecast information associated with the named Network Load are submitted with the NITSLoadForecast NITS OASIS DATA Template. Changes to the NITSLoadForecast NITS OASIS DATA Template may be submitted using separate ModifyNITSLoad NITS OASIS REQUEST Template requests.

3/14/12 accepted

If submitted with an initial STATUS of PRESUBMITTED or when the request STATUS has been set to DEFICIENT by the Transmission Provider, any information supplied in any of the supported load NITS OASIS DATA Templates may be updated using the UpdateNITSRequest NITS OASIS INPUT Template. New data may be added to the ModifyNITSLoad NITS OASIS REQUEST Template request by submission of a previously omitted load NITS OASIS DATA Template. If information has already been submitted for a given NITS OASIS DATA Template, that information may be replaced in its entirety by re-submitting the correct information to replace the current information associated with the ModifyNITSLoad NITS OASIS REQUEST Template request.

3/14/12 accepted

All NITS OASIS DATA Templates included in a given ModifyNITSLoad NITS OASIS REQUEST Template request must reference the same unique LOAD_NAME; OASIS shall return an error on any attempt to submit a new or update an existing ModifyNITSLoad NITS OASIS REQUEST Template request where the LOAD_NAME is not consistent across all associated NITS OASIS DATA Templates in the request.

On approval and final confirmation of any modification to the Network load, the information provided in the ModifyNITSLoad NITS OASIS REQUEST Template request shall replace any existing information previously associated with that Network Load over the timeframe(s) as specified in the associated NITS OASIS DATA Template(s) submitted.

The following NITS OASIS DATA Templates shall be supported by the ModifyNITSLoad NITS OASIS REQUEST Template:

NITSLoadDescription

NITSLoadForecast

The REQUEST Template Data Elements for the Input and Response shall be as specified in Business Practice Standard WEQ-002-101.3.2.

002-101.3.2.10	Request to Add a Generation Asset (AddNITSGeneration NITS OASIS REQUEST Template)

3/14/12 accepted

The AddNITSGeneration NITS OASIS REQUEST Template is used to request the addition of a new generation asset that may be used as a Network Resource. This request may be submitted by the Eligible Customer or Transmission Customer, effective at the time of submission. The information describing the nature of the generation asset is supplied in one or more related NITS OASIS DATA Templates that are supported for this request.

3/14/12 accepted

If submitted with an initial STATUS of PRESUBMITTED or when the request STATUS is set to DEFICIENT, any information supplied in any of the supported generation NITS OASIS DATA Templates may be updated using the UpdateNITSRequest NITS OASIS INPUT Template. New data may be added to the AddNITSGeneration NITS OASIS REQUEST Template request by submission of a previously omitted generation NITS OASIS DATA Template. If information has already been submitted for a given NITS OASIS DATA Template, that information may be replaced in its entirety by re-submitting the correct information to replace the current information associated with the AddNITSGeneration NITS OASIS REQUEST Template request.

3/14/12 accepted

Once QUEUED for evaluation or set to be reevaluated, the UpdateNITSRequest NITS OASIS INPUT Template is limited to only making appropriate changes to the AddNITSGeneration NITS OASIS REQUEST Template request STATUS. Any changes required to the generation information must then be made through submission of a separate ModifyNITSGeneration NITS OASIS REQUEST Template.

3/14/12 accepted

The AddNITSGeneration NITS OASIS REQUEST Template request must contain the NITSGenerationDescription NITS OASIS DATA Template defining the basic attributes of the generation asset. The NITSGenerationDispatch NITS OASIS DATA Template is optional.

3/14/12 accepted

Each generation asset that may be used to serve Network Load as a DNR under the Eligible Customer or Transmission Customer’s NITS Application must be assigned a Eligible Customer or Transmission Customer supplied, application unique GEN_NAME to be used to reference that generation asset in any associated NITSGenerationDispatch NITS OASIS DATA Template information, or subsequent AddNITSDNR NITS OASIS REQUEST Template request.

3/14/12 accepted

All NITS OASIS DATA Templates included in a given AddNITSGeneration NITS OASIS REQUEST Template request must reference the same unique GEN_NAME; OASIS shall return an error on any attempt to submit a new or update an exsting AddNITSGeneration NITS OASIS REQUEST Template request where the GEN_NAME is not consistent across all associated NITS OASIS DATA Templates in the request.

To facilitate the designation of multiple physical generation assets as a single DNR while providing for the ability to designate or terminate any of those resources on an aggregate basis or on an individual generation asset basis, a given generation asset may be assigned to a named aggregate generation group (GEN_GROUP) in the NITSGenerationDescription NITS OASIS DATA Template.

The following NITS OASIS DATA Templates shall be supported by the AddNITSGeneration NITS OASIS REQUEST Template:

NITSGenerationDescription (must be supplied)

NITSGenerationDispatch

The REQUEST Template Data Elements for the Input and Response shall be as specified in Business Practice Standard WEQ-002-101.3.2.

002-101.3.2.11	Request to Modify a Generation Asset (ModifyNITSGeneration NITS OASIS REQUEST Template)

3/14/12 accepted

The ModifyNITSGeneration NITS OASIS REQUEST Template is used to request the modification of information in association with an existing NITS generation asset.  This request may be submitted by the Eligible Customer or Transmission Customer effective at the time of submission.

3/14/12 accepted

Removing the relationship of a generation asset to the NITS Application is effected by setting the effective stop time in the NITSGenerationDescription NITS OASIS DATA Template using this ModifyNITSGeneration NITS OASIS REQUEST Template request. All Network Resource definitions and designations that reference a given generation asset being removed from the NITS Application must be terminated by the Eligible Customer or Transmission Customer effective at or prior to the time requested for the removal of the generation asset. The Transmission Provider may deny the request to remove a generation asset from a NITS Application if there are Network Resource definitions and designations that have not yet been terminated.

3/14/12 accepted

If submitted with an initial STATUS of PRESUBMITTED or when the request STATUS is set to DEFICIENT, any information supplied in any of the supported generation NITS OASIS DATA Templates may be updated using the UpdateNITSRequest NITS OASIS INPUT Template. New data may be added to the ModifyNITSGeneration NITS OASIS REQUEST Template request by submission of a previously omitted generation NITS OASIS DATA Template. If information has already been submitted for a given NITS OASIS DATA Template, that information may be replaced in its entirety by re-submitting the correct information to replace the current information associated with the ModifyNITSGeneration NITS OASIS REQUEST Template request.

3/14/12 accepted

On approval and final confirmation of any modification to the NITS generation asset, the information provided in the ModifyNITSGeneration NITS OASIS REQUEST Template request shall replace any existing information previously associated with the generation asset over that timeframe as specified in the associated generation NITS OASIS DATA Template(s) submitted.

All NITS OASIS DATA Templates included in a given ModifyNITSGeneration NITS OASIS REQUEST Template request must reference the same unique GEN_NAME; OASIS shall return an error on any attempt to submit a new or update an exsting ModifyNITSGeneration NITS OASIS REQUEST Template request where the GEN_NAME is not consistent across all associated NITS OASIS DATA Templates in the request.

The following NITS OASIS DATA Templates shall be supported by the ModifyNITSGeneration NITS OASIS REQUEST Template:

NITSGenerationDescription

NITSGenerationDispatch

The REQUEST Template Data Elements for the Input and Response shall be as specified in Business Practice Standard WEQ-002-101.3.2.

002-101.3.2.12	Request to Add a Network Resource (AddNITSResource NITS OASIS REQUEST Template)

3/14/12 accepted

The AddNITSResource NITS OASIS REQUEST Template is used to request the addition of a new resource to be used to serve load under the NITS Application. This request may be submitted by the Eligible Customer or Transmission Customer, effective at the time of submission. The information describing the nature of the new Network Resource is supplied in one or more related NITS OASIS DATA Templates that are supported for this request.

3/14/12 accepted

The AddNITSResource NITS OASIS REQUEST Template request is used to define a future resource to be forecasted but as yet not declared as a DNR, or establish a named Network Resource that may then be specified as a DNR in a subsequent AddNITSDNR NITS OASIS REQUEST Template request. Network Resources may be defined without being designated so that they may be included in the resource forecast information supplied to the Transmission Provider under the NITS Application.

3/14/12 accepted

If submitted with an initial STATUS of PRESUBMITTED or when the request STATUS has been set to DEFICIENT by the Transmission Provider, any information supplied in any of the supported resource NITS OASIS DATA Templates may be updated using the UpdateNITSRequest NITS OASIS INPUT Template. New data may be added to the AddNITSResource NITS OASIS REQUEST Template request by submission of a previously omitted resource NITS OASIS DATA Template. If information has already been submitted for a given NITS OASIS DATA Template, that information may be replaced in its entirety by re-submitting the correct information to replace the current information associated with the AddNITSResource NITS OASIS REQUEST Template request.

3/14/12 accepted

Once QUEUED for evaluation or set to be reevaluated, the UpdateNITSRequest NITS OASIS INPUT Template is limited to only making appropriate changes to the AddNITSResource NITS OASIS REQUEST Template request STATUS. Any changes required to the network resource information must then be made through submission of the ModifyNITSResource NITS OASIS REQUEST Template.

3/14/12 accepted

Each resource to be used to serve Network Load under the Eligible Customer or Transmission Customer’s NITS Application must be assigned a Eligible Customer or Transmission Customer supplied, application unique RESOURCE_NAME to be used to reference that resource in any associated NITSResourceForecast NITS OASIS DATA Template information, or subsequent AddNITSDNR NITS OASIS REQUEST Template request.

3/14/12 accepted

Submission of an AddNITSResource NITS OASIS REQUEST Template request with reference to a RESOURCE_NAME that is currently defined under the NITS Application shall be returned with an error condition indicating that the named resource already exists. The ModifyNITSResource NITS OASIS REQUEST Template request shall be used to modify existing named resource forecast information.

3/14/12 accepted

All NITS OASIS DATA Templates included in a given AddNITSResource NITS OASIS REQUEST Template request must reference the same unique RESOURCE_NAME; OASIS shall return an error on any attempt to submit a new or update an existing AddNITSResource NITS OASIS REQUEST Template request where the RESOURCE_NAME is not consistent across all associated NITS OASIS DATA Templates in the request.

3/14/12 accepted

New or revised resource forecast information associated with the named Network Resource is submitted with the NITSResourceForecast NITS OASIS DATA Template. Changes to the NITSResourceForecast NITS OASIS DATA Template may be submitted using separate ModifyNITSResource NITS OASIS REQUEST Template requests.

3/14/12 accepted

The following NITS OASIS DATA Templates shall be supported by the AddNITSResource NITS OASIS REQUEST Template:

NITSResourceDescription (must be supplied)

NITSResourceForecast

The REQUEST Template Data Elements for the Input and Response shall be as specified in Business Practice Standard WEQ-002-101.3.2.

002-101.3.2.13	Request to Modify a Network Resource (ModifyNITSResource NITS OASIS REQUEST Template)

3/14/12 accepted

The ModifyNITSResource NITS OASIS REQUEST Template is used to request the modification of the forecasted capacity in association with an existing named network resource under a NITS Application or to revise the effective date/time that the resource shall no longer be associated with the NITS Application. This request may be submitted by the Eligible Customer or Transmission Customer, effective at the time of the request submission.

3/14/12 accepted

The information supplied in association with the ModifyNITSResource NITS OASIS REQUEST Template request, if approved and confirmed, shall effectively replace any existing information associated with that named resource under the NITS Application over the period of time specified in the NITS OASIS DATA Template(s) submitted as part of the ModifyNITSResource NITS OASIS REQUEST Template request.

New or revised forecast information associated with the named Network Resource is submitted with the NITSResourceForecast NITS OASIS DATA Template

Removal of a named resource from consideration under the NITS Application is accomplished by setting the EFFECTIVE_STOP_TIME in the NITSResourceDescription NITS OASIS DATA Template.  EFFECTIVE_STOP_TIME is the only NITSResourceDescription NITS OASIS DATA Template Data Element that may be modified via the ModifyNITSResource NITS OASIS REQUEST Template request, and establishes the point in time after which all information that may have been provided in association with that Network Resource. i.e., designations and forecasts, shall no longer be considered part of the NITS Application. A Network Resource may not be removed from the NITS Application if it is currently declared by the Eligible Customer or Transmission Customer as a DNR beyond the proposed EFFECTIVE_STOP_TIME of that Network Resource.

3/14/12 accepted

If submitted with an initial STATUS of PRESUBMITTED or when the request STATUS has been set to DEFICIENT by the Transmission Provider, any information supplied in any of the supported resource NITS OASIS DATA Templates may be updated using the UpdateNITSRequest NITS OASIS INPUT Template. New data may be added to the ModifyNITSResource NITS OASIS REQUEST Template request by submission of a previously omitted resource NITS OASIS DATA Template. If information has already been submitted for a given NITS OASIS DATA Template, that information may be replaced in its entirety by re-submitting the correct information to replace the current information associated with the ModifyNITSResource NITS OASIS REQUEST Template request.

3/14/12 accepted

All NITS OASIS DATA Templates included in a given ModifyNITSResource NITS OASIS REQUEST Template request must reference the same existing unique RESOURCE_NAME; OASIS shall return an error on any attempt to submit a new or update an existing ModifyNITSResource NITS OASIS REQUEST Template request where the RESOURCE_NAME is not consistent across all associated NITS OASIS DATA Templates in the request, or where that RESOURCE_NAME does not currently exist.

On approval and final confirmation of any modification to the Network Resource, the information provided in the ModifyNITSResource NITS OASIS REQUEST Template request shall replace any existing information previously associated with that Network Resource over the timeframe(s) as specified in the associated NITS OASIS DATA Template(s) submitted.

The following NITS OASIS DATA Templates shall be supported by the ModifyNITSResource NITS OASIS REQUEST Template:

NITSResourceDescription

NITSResourceForecast

The REQUEST Template Data Elements for the Input and Response shall be as specified in Business Practice Standard WEQ-002-101.3.2.

002-101.3.2.14	Request to Add a Designated Network Resource (AddNITSDNR NITS OASIS REQUEST Template)

3/14/12 accepted

The AddNITSDNR NITS OASIS REQUEST Template is used to request the addition of a new DNR to those Network Resources committed to serve Network Load under the NITS Application. This request may be submitted by the Eligible Customer or Transmission Customer, effective at the time of submission. The information describing the nature of the new DNR is supplied in two or more related NITS OASIS DATA Templates that are supported for this request.

3/14/12 accepted

If the DNR to be added is already a defined named Network Resource under the NITS Application, only the NITSResourceDesignation and NITSResourceCapacity NITS OASIS DATA Templates are required to be submitted in the AddNITSDNR NITS OASIS REQUEST Template request. If the named resource to be designated is not defined under the NITS Application, the resource must be defined and optionally forecasted, if applicable, by submitting the NITSResourceDescription and NITSResourceForecast NITS OASIS DATA Templates as part of the AddNITSDNR NITS OASIS REQUEST Template request.

For Network Resources identified as OFF_SYSTEM, the AuxiliaryTransmission NITS OASIS DATA Template may be submitted to provide the information required for Transmission Service arrangements on other Transmission Providers’ systems to be utilized for delivery of energy from the DNR to the POR specified for the DNR on the Transmission Provider’s transmission system. If the new DNR being requested is to be considered as part of a coordinated service request across multiple transmission providers, the required information for each of the other Coordinated Requests in the Coordinated Group must be supplied in the AuxiliaryTransmission NITS OASIS DATA Template. The DNR request must also be flagged as ‘coordinated’ such that the request will be handled by the Transmission Provider in accordance with Business Practice Standard WEQ-001-23 established for handling such requests. The OASIS assigned ASSIGNMENT_REF returned in the AddNITSDNR NITS OASIS REQUEST Template response must be used as the ASSIGNMENT_REF for identifying the requested DNR as a Coordinated Request in the Coordinated Group information associated with each of the other Coordinated Request comprising the Coordinated Group.

3/14/12 accepted

If submitted with an initial STATUS of PRESUBMITTED or when the request STATUS has been set to DEFICIENT by the Transmission Provider, any information supplied in any of the supported resource NITS OASIS DATA Templates may be updated using the UpdateNITSRequest NITS OASIS INPUT Template. New data may be added to the AddNITSDNR NITS OASIS REQUEST Template request by submission of a previously omitted resource NITS OASIS DATA Template. If information has already been submitted for a given NITS OASIS DATA Template, that information may be replaced in its entirety by re-submitting the correct information to replace the current information associated with the AddNITSDNR NITS OASIS REQUEST Template request. Validation of the AddNITSDNR NITS OASIS REQUEST Template request information shall occur when the request is ultimately queued or marked for reevaluation.

3/14/12 accepted

All NITS OASIS DATA Templates submitted in association with a given AddNITSDNR NITS OASIS REQUEST Template request must reference the same Eligible Customer or Transmission Customer assigned RESOURCE_NAME. If the RESOURCE_NAME is not consistent in all NITS OASIS DATA Templates an error shall be returned when the AddNITSDNR request is attempted to be queued or set for reevaluation.

3/14/12 accepted

Submission of an AddNITSDNR NITS OASIS REQUEST Template request that includes the NITSResourceDescription or NITSResourceForecast NITS OASIS DATA Templates with reference to a RESOURCE_NAME that is currently defined under the NITS Application shall be returned with an error condition indicating that the named resource already exists. The ModifyNITSResource NITS OASIS REQUEST Template request must be used to modify that information associated with an existing named Network Resource.

3/14/12 accepted

The DNR’s capacity to be designated over time must be supplied in one or more data records specified in the NITSResourceCapacity NITS OASIS DATA Template. The capacity to be designated must be supplied as a positive valued integer MW quantity or zero (0) along with a corresponding time interval to which the capacity value applies. NITSResourceCapacity NITS OASIS DATA Template data records must be supplied such that a capacity value (or 0) is specified for each time point spanning the entire EFFECTIVE_START_TIME/EFFECTIVE_STOP_TIME time interval specified for the DNR in the AddNITSDNR NITS OASIS REQUEST Template request’s NITSResourceDesignation NITS OASIS DATA Template. On initial submission, the capacity specified at the beginning of the EFFECTIVE_START_TIME and at the end of the EFFECTIVE_STOP_TIME must be non-zero, i.e., no leading or trailing capacity segments will be permitted. If the NITSResourceDesignation NITS OASIS DATA Template identifies the resource as a GENERATION resource, the identity of a valid generation asset (GEN_NAME) or aggregate generation group (GEN_GROUP) associated with the NITS Application must be supplied in each capacity profile record submitted via the NITSResourceCapacity NITS OASIS DATA Template.

3/14/12 accepted

The incremental amount of transmission service to be associated with delivering the DNR to Network Load may be specified by the Eligible Customer or Transmission Customer in the NITSSchedulingRights NITS OASIS DATA Template. The Transmission Provider may require that the information in this NITS OASIS DATA Template must be specified by the Eligible Customer or Transmission Customer. If not required by the Transmission Provider, the Transmission Provider shall assume that the request to add the DNR is to be made fully available to be scheduled at the DNR’s designated capacity to all Network Loads sharing the same named POD and Sink. Any limitations to the Eligible Customer or Transmission Customer’s full deliverability to any specific POD and Sink, if applicable, must be identified by the Transmission Provider through the NITSSchedulingRights NITS OASIS DATA Template and the AddNITSDNR NITS OASIS REQUEST Template request must be set to COUNTEROFFER (or CR_COUNTEROFFER) to notify the Eligible Customer or Transmission Customer of such deliverability limitations and allow for the adjustment of the DNR capacity and incremental scheduling rights to be CONFIRMED.

3/14/12 accepted

Once QUEUED for evaluation or set to be reevaluated, the UpdateNITSRequest NITS OASIS INPUT Template is limited to only making appropriate changes to the AddNITSDNR NITS OASIS REQUEST Template request STATUS or updating the AuxiliaryTransmission NITS OASIS DATA Template information in accordance with the Business Practice Standards WEQ-001-23 for Coordinated Requests. If the AddNITSDNR NITS OASIS REQUEST Template request is set to COUNTEROFFER by the Transmission Provider, the Eligible Customer or Transmission Customer may adjust the requested DNR capacity by submitting an UpdateNITSRequest NITS OASIS INPUT Template for the AddNITSDNR NITS OASIS REQUEST Template request with the revised requested capacity stated in the NITSResourceCapacity NITS OASIS DATA Template. The Eligible Customer or Transmission Customer may also adjust the final amount of transmission scheduling rights that are to be confirmed in association with the DNR by updating the NITSSchedulingRights NITS OASIS DATA Template. Any other changes required to the DNR, assuming the DNR is ultimately CONFIRMED, must then be made through a subsequent TerminateNITSDNR and/or new AddNITSDNR NITS OASIS REQUEST Templates requests.

3/14/12 accepted

The CR_ACCEPTED and CR_COUNTEROFFER status values are supported for Coordinated Requests, and the negotiation of capacity via updates to the NITSResourceCapacity and NITSSchedulingRights NITS OASIS DATA Templates using the UpdateNITSRequest NITS OASIS INPUT Template must be supported based on the final disposition of the other requests in the Coordinated Group as specified in the AuxiliaryTransmission NITS OASIS DATA Template and Business Practice Standard WEQ-001-23.

3/14/12 accepted

Once CONFIRMED, the amount of capacity designated over time for the DNR shall be added to the total DNR capacity amount designated for the named Network Resource at the specified POR and Source under the NITS Application. The Transmission Provider shall also incorporate any incremental transmission scheduling rights assigned for delivery of the DNR to the total rights available to schedule that DNR to designated load.

The following NITS OASIS DATA Templates shall be supported by the AddNITSDNR NITS OASIS REQUEST Template:

NITSResourceDescription

NITSResourceForecast

NITSResourceDesignation (must be supplied)

NITSResourceCapacity (must be supplied)

NITSSchedulingRights

AuxiliaryTransmission

The REQUEST Template Data Elements for the Input and Response shall be as specified in Business Practice Standard WEQ-002-101.3.2.

002-101.3.2.15	Request to Terminate a Designated Network Resource (TerminateNITSDNR NITS OASIS REQUEST Template)

3/14/12 accepted

The TerminateNITSDNR NITS OASIS REQUEST Template is used to request the temporary or indefinite termination of a DNR under the NITS Application. This request may be submitted by the Eligible Customer or Transmission Customer, effective at the time of submission. The information describing the nature of the termination (temporary vs. indefinite) and the DNR capacity being terminated over time is supplied in the NITS OASIS DATA Templates that are supported for this request.

3/14/12 accepted

All NITS OASIS DATA Templates submitted in association with a given TerminateNITSDNR NITS OASIS REQUEST Template request must reference the same Eligible Customer or Transmission Customer assigned RESOURCE_NAME. If the RESOURCE_NAME is not consistent in all NITS OASIS DATA Templates an error shall be returned when the TerminateNITSDNR NITS OASIS REQUEST Template request is attempted to be queued or set for reevaluation.

3/14/12 accepted

Submission of a TerminateNITSDNR NITS OASIS REQUEST Template request that includes reference to a RESOURCE_NAME that is not currently defined or is not designated under the NITS Application shall be returned with an error condition indicating that the named designated resource does not exist.

If submitted with an initial STATUS of PRESUBMITTED or when the request STATUS has been set to DEFICIENT by the Transmission Provider, any information supplied in any of the supported NITS OASIS DATA Templates may be updated using the UpdateNITSRequest NITS OASIS INPUT Template. New data may be added to the TerminateNITSDNR NITS OASIS REQUEST Template request by submission of a previously omitted resource NITS OASIS DATA Template. If information has already been submitted for a given NITS OASIS DATA Template, that information may be replaced in its entirety by re-submitting the correct information to replace the current information associated with the TerminateNITSDNR NITS OASIS REQUEST Template request. Validation of the TerminateNITSDNR NITS OASIS REQUEST Template request information shall occur when the request is ultimately queued or marked for reevaluation.

3/14/12 accepted

If the TerminateNITSDNR NITS OASIS REQUEST Template request is for the temporary termination of the DNR, the Eligible Customer or Transmission Customer must supply the required attestation that the DNR still qualifies for treatment as a DNR at the end of the time interval the DNR is being terminated.

The DNR’s capacity to be terminated over time must be supplied in one or more data records specified in the NITSResourceCapacity NITS OASIS DATA Template. The capacity to be terminated must be supplied as a negative valued integer MW quantity or zero (0) along with a corresponding time interval to which the capacity value applies. NITSResourceCapacity NITS OASIS DATA Template data records must be supplied such that a capacity value (or 0) is specified for each time point spanning the entire EFFECTIVE_START_TIME/EFFECTIVE_STOP_TIME time interval specified for the DNR in the TerminateNITSDNR request’s NITSResourceDesignation NITS OASIS DATA Template. If the NITSResourceDesignation NITS OASIS DATA Template identifies the resource as a GENERATION resource, the identity of a valid generation asset (GEN_NAME) or aggregate generation group (GEN_GROUP) associated with the NITS Application must be supplied in each capacity profile record submitted via the NITSResourceCapacity NITS OASIS DATA Template.

3/14/12 accepted

If the TerminateNITSDNR NITS OASIS REQUEST Template request is for indefinite termination of all or a portion of the DNR, the EFFECTIVE_STOP_TIME of the termination request in the NITSResourceDesignation NITS OASIS DATA Template must be null, and the capacity to be terminated from the DNR must be for an indefinite period of time where STOP_TIME in the NITSResourceCapacity NITS OASIS DATA Template is also null.

Once CONFIRMED, the amount of capacity terminated over time for the DNR shall be subtracted from the total DNR capacity amount designated for the named Network Resource at the specified POR and Source under the NITS Application. The Transmission Provider shall also release any associated transmission scheduling rights assigned for delivery of the DNR to Network Load that are in excess of that required to schedule any remaining capacity associated with the DNR.

3/14/12 accepted

The following NITS OASIS DATA Templates shall be supported by the TerminateNITSDNR NITS OASIS REQUEST Template:

NITSResourceDesignation (must be supplied)

NITSResourceCapacity (must be supplied)

The REQUEST Template Data Elements for the Input and Response shall be as specified in Business Practice Standard WEQ-002-101.3.2.

002-101.3.2.16	Request for Service from a Non-Designated Network Resource (AddNITSSecondary NITS OASIS REQUEST Template)

3/14/12 accepted

The AddNITSSecondary NITS OASIS REQUEST Template is used to request Transmission Service from a non-designated Network Resource to serve Network Load under the NITS Application on a Non-Firm/Secondary Transmission Service basis. This request may be submitted by the Transmission Customer, effective at the time of submission. The information describing the nature of the Secondary Network Transmission Service being requested is supplied in the related NITS OASIS DATA Templates that are supported for this request. Requests for transmission service from a non-designated Network Resource may only be submitted in association with a CONFIRMED NITS Application.

3/14/12 accepted

If submitted with an initial STATUS of PRESUBMITTED, any information supplied in any of the supported resource NITS OASIS DATA Templates may be updated using the UpdateNITSRequest NITS OASIS INPUT Template. New data may be added to the AddNITSSecondary NITS OASIS REQUEST Template request by submission of a previously omitted resource NITS OASIS DATA Template. If information has already been submitted for a given NITS OASIS DATA Template, that information may be replaced in its entirety by re-submitting the correct information to replace the current information associated with the AddNITSSecondary NITS OASIS REQUEST Template request. The STATUS of DEFICIENT will not be used in connection with an AddNITSSecondary NITS OASIS REQUEST Template request. Validation of the AddNITSSecondary NITS OASIS REQUEST Template request information shall occur when the request is ultimately queued.

3/14/12 accepted

The Secondary Network Transmission Service capacity being requested must be supplied in one or more data records specified in the NITSSecondaryCapacity NITS OASIS DATA Template. The capacity being requested must be supplied as a positive valued integer MW quantity or zero (0) along with a corresponding time interval to which the capacity value applies. NITSSecondaryCapacity NITS OASIS DATA Template data records must be supplied such that a capacity value (or 0) is specified for each time point spanning the entire EFFECTIVE_START_TIME/EFFECTIVE_STOP_TIME time interval specified for the secondary service in the AddNITSSecondary NITS OASIS REQUEST Template request’s NITSSecondaryService NITS OASIS DATA Template. The time interval from EFFECTIVE_START_TIME to EFFECTIVE_STOP_TIME shall be used to establish the duration of the Secondary Network Transmission Service requested for use in determining confirmation time limits, etc. On initial submission, the capacity specified at the beginning of the EFFECTIVE_START_TIME and at the end of the EFFECTIVE_STOP_TIME must be non-zero, i.e., no leading or trailing capacity segments will be permitted.

3/14/12 accepted

If the Secondary Network Transmission Service being requested is to be considered as part of a Coordinated Group of requests across multiple Transmission Providers, the NITSSecondaryService NITS OASIS DATA Template must include the designation that this is part of a Coordinated Group prior to being set to QUEUED. The information identifying each of the other transmission requests and reservations that comprise the Coordinated Group must be supplied using the UpdateNITSRequest NITS OASIS INPUT Template and AuxiliaryTransmission NITS OASIS DATA Template within the timeline specified in Business Practice Standard WEQ-001-23 to remain a valid Coordinated Request.

3/14/12 accepted

Once QUEUED for evaluation, the UpdateNITSRequest NITS OASIS INPUT Template is limited to only making appropriate changes to the AddNITSSecondary NITS OASIS REQUEST Template request STATUS or adding/updating information to the AuxiliaryTransmssion NITS OASIS DATA Template, if applicable. If the AddNITSSecondary NITS OASIS REQUEST Template request is set to COUNTEROFFER by the Transmission Provider, the Transmission Customer may adjust the requested capacity by submitting an UpdateNITSRequest NITS OASIS INPUT Template for the AddNITSSecondary NITS OASIS REQUEST Template request with the revised requested capacity stated in the NITSSecondaryCapacity NITS OASIS DATA Template.

3/14/12 accepted

The CR_ACCEPTED and CR_COUNTEROFFER status values are supported for Coordinated Requests, and the negotiation of capacity via updates to the NITSSecondaryCapacity NITS OASIS DATA Template using the UpdateNITSRequest NITS OASIS INPUT Template must be supported based on the final disposition of the other requests in the Coordinated Group as specified in the AuxiliaryTransmission NITS OASIS DATA Template and Business Practice Standard WEQ-001-23.

Once CONFIRMED, the Transmission Customer may schedule deliveries from the non-designated Network Resource using the Secondary Network Transmission Service over the term reserved, or until the Secondary Network Transmission Service is released via the TerminateNITSSecondary NITS OASIS REQUEST Template request.

The following NITS OASIS DATA Templates shall be supported by the AddNITSSecondary NITS OASIS REQUEST Template:

NITSSecondaryService (must be supplied)

NITSSecondaryCapacity (must be supplied)

AuxiliaryTransmission

The REQUEST Template Data Elements for the Input and Response shall be as specified in Business Practice Standard WEQ-002-101.3.2.

002-101.3.2.17	Request to Release Service from a Non-Designated Network Resource (TerminateNITSSecondary NITS OASIS REQUEST Template)

3/14/12 accepted not accepted release

The TerminateNITSSecondary NITS OASIS REQUEST Template is used to request the release of Secondary Network Transmission Service arranged via the AddNITSSecondary NITS OASIS REQUEST Template request. This request may be submitted by the Transmission Customer, effective at the time of submission. The information identifying the Secondary Network Transmission Service and the capacity to be released from that service is supplied in the NITS OASIS DATA Templates that are supported for this request. Requests for releasing of Secondary Network Transmission Service may only be submitted in association with a CONFIRMED NITS Application.

3/14/12 accepted not accepted release

If submitted with an initial STATUS of PRESUBMITTED, any information supplied in any of the supported resource NITS OASIS DATA Templates may be updated using the UpdateNITSRequest NITS OASIS INPUT Template. New data may be added to the TerminateNITSSecondary NITS OASIS REQUEST Template request by submission of a previously omitted resource NITS OASIS DATA Template. If information has already been submitted for a given NITS OASIS DATA Template, that information may be replaced in its entirety by re-submitting the correct information to replace the current information associated with the TerminateNITSSecondary NITS OASIS REQUEST Template request. The STATUS of DEFICIENT will not be used in connection with an TerminateNITSSecondary NITS OASIS REQUEST Template request. Validation of the TerminateNITSSecondary NITS OASIS REQUEST Template request information shall occur when the request is ultimately queued.

3/14/12 accepted

The Secondary Network Transmission Service capacity to be released over time must be supplied in one or more data records specified in the NITSSecondaryCapacity NITS OASIS DATA Template. The capacity to be released must be supplied as a negative valued integer MW quantity or zero (0) along with a corresponding time interval to which the capacity value applies. NITSSecondaryCapacity NITS OASIS DATA Template data records must be supplied such that a capacity value (or 0) is specified for each time point spanning the entire EFFECTIVE_START_TIME/EFFECTIVE_STOP_TIME time interval specified in the TerminateNITSSecondary NITS OASIS REQUEST Template request’s NITSSecondaryService NITS OASIS DATA Template. The ASSIGNMENT_REF associated with the original request for Secondary Network Transmission Service must be supplied in the RELATED_REF Data Element when the Secondary Network Transmission Service TerminateNISTSecondary request is queued for evaluation.

3/14/12 accepted not accepted release

Once CONFIRMED, the amount of capacity released for the Secondary Network Transmission Service shall be subtracted from the current remaining amount of Secondary Network Transmission Service capacity reserved over the specified Source/POR, Sink/POD and Path.

3/14/12 not accepted

The following NITS OASIS DATA Templates shall be supported by the TerminateNITSSecondary NITS OASIS REQUEST Template:

NITSSecondaryService (must be supplied)

NITSSecondaryCapacity (must be supplied)

The REQUEST Template Data Elements for the Input and Response shall be as specified in Business Practice Standard WEQ-002-101.3.2.

002-101.3.2.18	Request for Ancillary Service (AddNITSAncillary NITS OASIS REQUEST Template) [Optional]

3/14/12 accepted

The AddNITSAncillary NITS OASIS REQUEST Template is used to request ancillary services in support of the NITS Application. This request may be submitted by the Eligible Customer or Transmission Customer, effective at the time of submission. The information describing the nature of the ancillary service being requested is supplied in the related NITS OASIS DATA Templates that are supported for this request. Support for and use of the AddNITSAncillary NITS OASIS REQUEST Template by the Transmission Provider is optional.

3/14/12 accepted

Requests for ancillary services using the AddNITSAncillary NITS OASIS REQUEST Template shall be processed on OASIS in exactly the same manner as an ancillary service request submitted or updated using the general OASIS ancrequest and anccust OASIS Templates as specified in Business Practice Standard WEQ-002-4. All AddNITSAncillary NITS OASIS REQUEST Template request submissions using the NewNITSRequest and UpdateNITSRequest NITS OASIS INPUT Templates shall be viewable by all OASIS users and may be acted upon by the ancillary service provider (SELLER) using the general OASIS ancstatus and ancsell OASIS Templates. That is, the AddNITSAncillary NITS OASIS REQUEST Template and related templates must be functionally equivalent to the general OASIS templates used to arrange ancillary service on OASIS in conjunction with PTP Transmission Service.

3/14/12 accepted

If submitted with an initial STATUS of PRESUBMITTED, any information supplied in any of the supported resource NITS OASIS DATA Templates may be updated using the UpdateNITSRequest NITS OASIS INPUT Template. New data may be added to the AddNITSAncillary NITS OASIS REQUEST Template request by submission of a previously omitted resource NITS OASIS DATA Template. If information has already been submitted for a given NITS OASIS DATA Template, that information may be replaced in its entirety by re-submitting the correct information to replace the current information associated with the AddNITSAncillary NITS OASIS REQUEST Template request. The STATUS of DEFICIENT will not be used in connection with an AddNITSAncillary NITS OASIS REQUEST Template request. Validation of the AddNITSAncillary NITS OASIS REQUEST Template request information shall occur when the request is ultimately queued.

3/14/12 accepted

The capacity and price, if applicable, of the ancillary service being requested may be supplied in one or more data records specified in the NITSAncillaryCapacity NITS OASIS DATA Template. If required, the capacity and price being requested must be supplied with capacity specified as a positive valued integer MW quantity or zero (0) along with a corresponding time interval to which the capacity and price value applies. NITSAncillaryCapacity NITS OASIS DATA Template data records must be supplied such that a capacity value (or 0) is specified for each time point spanning the entire EFFECTIVE_START_TIME/EFFECTIVE_STOP_TIME time interval specified for the term of service in the AddNITSAncillary NITS OASIS REQUEST Template request’s NITSAncillaryService NITS OASIS DATA Template.

3/14/12 accepted

Once QUEUED for evaluation, the UpdateNITSRequest NITS OASIS INPUT Template is limited to only making appropriate changes to the AddNITSAncillary request STATUS. If the AddNITSAncillary NITS OASIS REQUEST Template request is set to COUNTEROFFER by the ancillary service provider (SELLER), the Eligible Customer or Transmission Customer (CUSTOMER) and ancillary service provider (SELLER) may negotiate the price of service using the status value of COUNTEROFFER and REBID and the alternate updating of BID_PRICE (Eligible Customer or Transmission Customer) and OFFER_PRICE (Seller) as discussed in the applicable Business Practice Standards WEQ-002-4 and WEQ-013.

3/14/12 accepted

The following NITS OASIS DATA Templates shall be supported by the AddNITSAncillary NITS OASIS REQUEST Template:

NITSAncillaryService (must be supplied)

NITSAncillaryCapacity (must be supplied)

The REQUEST Template Data Elements for the Input and Response shall be as specified in Business Practice Standard WEQ-002-101.3.2.

002-101.3.2.19	Request for Point-to-Point Transmission Service (AddPTPService NITS OASIS REQUEST Template)

3/14/12 accepted

The AddPTPService NITS OASIS REQUEST Template is used to request PTP Transmission Services as part of a concomitant request to terminate a DNR and acquire additional PTP Transmission Service. This request may be submitted by the Transmission Customer, effective at the time of submission under the applicable NITS application. The information describing the nature of the PTP Transmission Service being requested is supplied in the related NITS OASIS DATA Templates that are supported for this request.

3/14/12 accepted

Requests for PTP Transmission Services using the AddPTPService NITS OASIS REQUEST Template shall be processed on OASIS in exactly the same manner as a Transmission Service request submitted or updated using the PTP transrequest and transcust OASIS Templates. All AddPTPService NITS OASIS REQUEST Template request submissions using the NewConcomitantRequest and UpdateConcomitantRequest NITS OASIS INPUT Templates shall be viewable by all and may be acted upon by the Transmission Provider (SELLER) using the PTP transstatus and transsell OASIS Templates. That is, the AddPTPService NITS OASIS REQUEST Template and related templates must be functionally equivalent to the OASIS Templates used to arrange PTP Transmission Service on OASIS as specified in Business Practice Standard WEQ-002-4.

3/14/12 accepted

If submitted with an initial STATUS of PRESUBMITTED, any information supplied in any of the supported request NITS OASIS DATA Templates may be updated using the UpdateConcomitantRequest NITS OASIS INPUT Template. New data may be added to the AddPTPService NITS OASIS REQUEST Template request by submission of a previously omitted NITS OASIS DATA Template. If information has already been submitted for a given NITS OASIS DATA Template, that information may be replaced in its entirety by re-submitting the correct information to replace the current information associated with the AddPTPService NITS OASIS REQUEST Template request. The STATUS of DEFICIENT will not be used in connection with an AddPTPService NITS OASIS REQUEST Template request. Validation of the AddPTPService NITS OASIS REQUEST Template request information shall occur when the request is ultimately queued.

3/14/12 accepted

The capacity and price of the Transmission Service being requested must be supplied in one or more data records specified in the PTPCapacity NITS OASIS DATA Template. The capacity and price being requested must be supplied with capacity specified as a positive valued integer MW quantity or zero (0) along with a corresponding time interval to which the capacity and price value applies. PTPCapacity NITS OASIS DATA Template data records must be supplied such that a capacity and price value (or 0) is specified for each time point spanning the entire EFFECTIVE_START_TIME/EFFECTIVE_STOP_TIME time interval specified for the term of service in the AddPTPService NITS OASIS REQUEST Template request’s PTPService NITS OASIS DATA Template. On initial submission, the capacity specified at the beginning of the EFFECTIVE_START_TIME and at the end of the EFFECTIVE_STOP_TIME must be non-zero, i.e., no leading or trailing capacity segments will be permitted.

3/14/12 accepted

If the PTP Transmission Service being requested is to be considered as part of a Coordinated Group of requests across multiple Transmission Providers, the PTPService NITS OASIS DATA Template must include the designation that this is part of a Coordinated Group prior to being set to QUEUED. The information identifying each of the other transmission requests and reservations that comprise the Coordinated Group must be supplied using the UpdateConcomitantRequest NITS OASIS INPUT Template and AuxiliaryTransmission NITS OASIS DATA Template within the timeline specified in Business Practice Standard WEQ-001-23 to remain a valid Coordinated Request.

3/14/12 accepted

Once QUEUED for evaluation, the UpdateConcomitantRequest NITS OASIS INPUT Template is limited to only making appropriate changes to the AddPTPService NITS OASIS REQUEST Template request STATUS. If the AddPTPService NITS OASIS REQUEST Template request is set to COUNTEROFFER by the Transmission Provider (SELLER), the Transmission Customer (CUSTOMER) and may negotiate the capacity and price of service using the status value of COUNTEROFFER and REBID and the alternate updating of CAPACITY_REQUESTED/BID_PRICE (Eligible Customer or Transmission Customer) and CAPACITY_GRANTED/OFFER_PRICE (Seller) as discussed in the applicable Business Practice Standards WEQ-002-4 and WEQ-013.

3/14/12 accepted

The CR_ACCEPTED and CR_COUNTEROFFER status values are supported for coordinated requests, and the negotiation of capacity via updates to the PTPCapacity NITS OASIS DATA Template using the UpdateConcomitantRequest NITS OASIS INPUT Template must be supported based on the final disposition of the other requests in the Coordinated Group as specified in the AuxiliaryTransmission NITS OASIS DATA Template and Business Practice Standard WEQ-001-23.

3/14/12 accepted

The following NITS OASIS DATA Templates shall be supported by the AddPTPService NITS OASIS REQUEST Template:

PTPService (must be supplied)

PTPCapacity (must be supplied)

Auxiliary Transmission

The REQUEST Template Data Elements for the Input and Response shall be as specified in Business Practice Standard WEQ-002-101.3.2.

002-101.3.3	DATA Template Definitions

3/14/12 accepted

The OASIS shall support the following NITS OASIS DATA Templates for the uploading of NITS related request information. NITS OASIS DATA Templates must be contained within a defined NITS OASIS REQUEST Template as specified in the NITS CSV Format. The contents of each NITS OASIS DATA Template is specific for that template and are identified by the NITS OASIS DATA Template Header Record Data Element Name list that immediately follows the key “DATA=<TemplateName>” header record identifier.

3/14/12 accepted

The data values to be associated with each of the named NITS OASIS DATA Template Data Elements are specified in one or more data records that immediately follow the NITS OASIS DATA Template Header Record up to the next NITS CSV Format Header Records or the end of the template input data stream.

When supplied within a new request submission, any NITS OASIS DATA Template that does not include one or more data records will return an error for that submitted request. When supplied within an update, e.g., UpdateNITSRequest, to an existing NITS request, any NITS OASIS DATA Template that does not include one or more data records will remove/delete all request information associated with that NITS OASIS DATA Template that may have been supplied in a prior new or update request submission.

3/14/12 accepted

002-101.3.3.1	Network Integration Transmission Service Service Description (NITSService NITS OASIS DATA Template)

3/14/12 bold accepted not accepted nits

The NITSService NITS OASIS DATA Template information defines the basic information associated with the term of NITS Service requested under a pending or confirmed NITS Application.

For a pending NITS Application, the data specified within this NITS OASIS DATA Template, with the exception of FILING_STATUS, may only be updated while the NITS Application has the STATUS of PRESUBMITTED. The Eligible Customer or Transmission Customer shall be allowed to submit an update to the FILING_STATUS Data Element using the ModifyNITSService NITS OASIS REQUEST Template to set the value to EXECUTED or UNEXECUTED to indicate the Eligible Customer or Transmission Customer’s intention with respect to the Transmission Provider’s filing of a NITS service agreement when the overall NITS Application (NewNITSApplication NITS OASIS REQUEST Template) request is set to CONFIRMED.

3/14/12 accepted

Any change to the NITSService NITS OASIS DATA Template information other than FILING_STATUS, once it has been queued for evaluation by the Transmission Provider or ultimately granted and confirmed must be submitted as a request to modify the NITS Application via the ModifyNITSService NITS OASIS REQUEST Template request.

When submitted in association with a new NITS Application, this template shall convey the initial term of service being requested (NewNITSApplication NITS OASIS REQUEST Template). When submitted as a modification to an existing NITS Application, this template may be used to request an extension to the original term of service or to request an early termination to the NITS Application.

3/14/12 accepted

To request an extension to the current term of service, the START_TIME in the request must be equal to the STOP_TIME of the current term of service, and the STOP_TIME must specify the end of the extension to the term of service being requested.

To request an early termination to the current term of service, the START_TIME in the request must be equal to the START_TIME of the current term of service, and the STOP_TIME must specify the new end to the current term of service being requested.

NITSService Data Element List – Request Upload:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| APPLICATION_NAME | Optional | Eligible Customer or Transmission Customer supplied descriptive name or other contract/agreement identifier that the Eligible Customer or Transmission Customer may use to assist management of their NITS Applications. |
| START_TIME | Required | Required on initial submission or on any request to modify the term of an existing NITS Application. Defines the start of NITS service being requested, or start of the extension to the current term of service being requested. 3/14/12 accepted |
| STOP_TIME | Required | Required on initial submission or on any request to modify the term of an existing NITS Application. Defines the stop/end of NITS service being requested. 3/14/12 accepted |
| SERVICE_DESCRIPTION | Optional | Eligible Customer or Transmission Customer provided summary description related to nature of the service being requested for new NITS Application or extension of an existing NITS Application. |
| FILING_STATUS | Optional | Set by Eligible Customer or Transmission Customer prior to or upon confirmation of service to indicate Eligible Customer or Transmission Customer’s intention with regard to the Transmission Provider’s filing of the NITS service agreement. One of: EXECUTED UNEXECUTED 3/14/12 accepted |

NITSService Data Element List – Request/Query Response:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| APPLICATION_NAME | Required | Eligible Customer or Transmission Customer supplied descriptive name or other contract/agreement identifier that the Eligible Customer or Transmission Customer may use to assist management of their NITS Applications. |
| START_TIME | Required | Defines the start of NITS service being requested, or start of the extension to the current term of service being requested. |
| STOP_TIME | Required | Defines the stop/end of NITS service being requested. |
| SERVICE_DESCRIPTION | Optional | Eligible Customer or Transmission Customer provided summary description related to nature of the service being requested for new NITS Application or extension of an existing NITS Application. |
| FILING_STATUS | May be null | Eligible Customer or Transmission Customer’s intention with regard to the Transmission Provider’s filing of the NIST service agreement; will be null until disposition of filing established by Eligible Customer or Transmission Customer update. |
| TIME_OF_LAST_UPDATE | Required | Date/time the information associated with this NITS information was last updated. |
| RECORD_STATUS | Required | Value indicating success/failure of OASIS to process the submitted request; 200 = Success. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in processing the submitted request. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. 3/14/12 accepted |

002-101.3.3.2	Network Integration Transmission Service Customer Information (NITSCustomer NITS OASIS DATA Template)

3/14/12 bold accepted not accepted nits

The NITSCustomer NITS OASIS DATA Template information defines the basic information associated with the current or prospective new Eligible Customer or Transmission Customer obligated to the terms of NITS requested under a pending or confirmed NITS Application. For a pending NITS Application, modifications to the data specified within this NITS OASIS DATA Template may only be updated while the NITS Application has the STATUS of PRESUBMITTED.

3/14/12 accepted

Any update to the NITSCustomer NITS OASIS DATA Template information once it has been queued for evaluation by the Transmission Provider or ultimately granted and confirmed must be submitted as a request to modify the NITS Application via the ModifyNITSCustomer NITS OASIS REQUEST Template request.

When submitted in association with a new NITS Application, this template shall identify the Eligible Customer or Transmission Customer to be obligated under the terms of the service being requested. When submitted as a modification to an existing NITS Application, this template shall convey updated information for the current Eligible Customer or Transmission Customer, or to request a transfer of obligation under the NITS Application to another Eligible Customer.

3/14/12 accepted

Every modification to the Eligible Customer or Transmission Customer information or transfer of obligation to a new Eligible Customer must specify the effective start date/time of such modification. The stop (end) date/time of each modification to the Eligible Customer or Transmission Customer shall be open-ended and set by OASIS to an appropriate value to indicate the information supplied is in effect until terminated by another modification to Eligible Customer or Transmission Customer information.

3/14/12 accepted

NITSCustomer Data Element List – Request Upload:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| CUSTOMER_CODE | Required | Registered entity code associated with the prospective Eligible Customer or Transmission Customer. |
| CUSTOMER_DUNS | Required | Registered DUNS associated with the prospective Eligible Customer or Transmission Customer. |
| CUSTOMER_NAME | Required | Primary contact information for the Eligible Customer or Transmission Customer associated with the NITS Application. |
| CUSTOMER_PHONE | Required | Primary contact information for the Eligible Customer or Transmission Customer associated with the NITS Application. |
| CUSTOMER_FAX | Required | Facsimile number associated with the prospective Eligible Customer or Transmission Customer. |
| CUSTOMER_EMAIL | Optional | Email address associated with the prospective Eligible Customer or Transmission Customer. |
| TRANSMISSION_OWNER | Required | Identifies (Y/N) if the Eligible Customer or Transmission Customer owns transmission assets to be considered as part of the Eligible Customer or Transmission Customer’s transmission system with respect to the NITS Application. 3/14/12 accepted |
| CUSTOMER_STATEMENT | Required* | Required if there is any change in CUSTOMER_CODE from the current Eligible Customer or Transmission Customer (i.e., transfer of obligation) associated with the NITS Application. Must be null in association with any request to simply update Eligible Customer or Transmission Customer contact or status notification information associated with the existing Eligible Customer or Transmission Customer. Provides a statement that Eligible Customer or Transmission Customer is an Eligible Customer for NITS. 3/14/12 accepted |
| ATTESTOR_NAME | Required* | Required with CUSTOMER_STATEMENT. Identifies the name and title of the individual responsible for attesting to the Eligible Customer or Transmission Customer’s eligibility for NITS. 3/14/12 accepted |
| ATTESTATION_SUBMITTER | Required* | Required with CUSTOMER_STATEMENT identifying the name and title of the individual submitting the statement of Eligible Customer or Transmission Customer’s eligibility for NITS. 3/14/12 accepted |
| STATUS_NOTIFICATION | Optional | If specified, identifies the protocol (http vs. email) and required attributes necessary to direct OASIS to deliver dynamic notification on any change in information associated with the Eligible Customer or Transmission Customer’s NITS Application. 3/14/12 accepted |
| CUSTOMER_COMMENTS | Optional | Optional comments provided by the Eligible Customer or Transmission Customer. 3/14/12 accepted |
| EFFECTIVE_START_TIME | Required | Defines the effective date/time that the obligation under the NITS Application is assumed by a new Eligible Customer or Transmission Customer (i.e., transfer), or the effective time of a modification to general Eligible Customer or Transmission Customer contact or dynamic notification settings. 3/14/12 accepted |
| EFFECTIVE_STOP_TIME | Must be null | Changes in Eligible Customer or Transmission Customer or Eligible Customer or Transmission Customer information is open ended and remains in effect until the end of the term of the NITS Application or terminated by supplying a subsequent update to the Eligible Customer or Transmission Customer information. 3/14/12 accepted |

NITSCustomer Data Element List – Request/Query Response:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| CUSTOMER_CODE | Required | Registered entity code associated with the prospective Eligible Customer or Transmission Customer. |
| CUSTOMER_DUNS | Required | Registered DUNS associated with the prospective Eligible Customer or Transmission Customer. |
| CUSTOMER_NAME | Required | Primary contact information for the Eligible Customer or Transmission Customer associated with the NITS Application. |
| CUSTOMER_PHONE | Required | Primary contact information for the Eligible Customer or Transmission Customer associated with the NITS Application. |
| CUSTOMER_FAX | Required | Facsimile number associated with the prospective Eligible Customer or Transmission Customer. |
| CUSTOMER_EMAIL | May be null | Email address associated with the prospective Eligible Customer or Transmission Customer. |
| TRANSMISSION_OWNER | Required | Identifies (Y/N) if the Eligible Customer or Transmission Customer owns transmission assets to be considered as part of the Eligible Customer or Transmission Customer’s transmission system with respect to the NITS Application. 3/14/12 accepted |
| CUSTOMER_STATEMENT | May be null | Required if there is any change in CUSTOMER_CODE from the current Eligible Customer or Transmission Customer (i.e., transfer of obligation) associated with the NITS Application. Must be null in association with any request to simply update Eligible Customer or Transmission Customer contact or status notification information associated with the existing Eligible Customer or Transmission Customer. Provides statement that Eligible Customer or Transmission Customer is an Eligible Customer for NITS. 3/14/12 accepted |
| ATTESTOR_NAME | May be null | Required with CUSTOMER_STATEMENT. Identifies the name and title of the individual responsible for attesting to the Eligible Customer or Transmission Customer’s eligibility for NITS. 3/14/12 accepted |
| ATTESTATION_SUBMITTER | May be null | Required with CUSTOMER_STATEMENT identifying the name and title of the individual submitting the statement of Eligible Customer or Transmission Customer’s eligibility for NITS. 3/14/12 accepted |
| STATUS_NOTIFICATION | May be null | If specified, identifies the protocol (http vs. email) and required attributes necessary to direct OASIS to deliver dynamic notification on any change in information associated with the Eligible Customer or Transmission Customer’s NITS Application. 3/14/12 accepted |
| CUSTOMER_COMMENTS | May be null | Optional comments provided by the Eligible Customer or Transmission Customer. 3/14/12 accepted |
| EFFECTIVE_START_TIME | Required | Defines the effective date/time that the obligation under the NITS Application is assumed by a new Eligible Customer or Transmission Customer (i.e., transfer), or the effective time of a modification to general Eligible Customer or Transmission Customer contact or dynamic notification settings. 3/14/12 accepted |
| EFFECTIVE_STOP_TIME | May be null | Last date/time the associate Eligible Customer or Transmission Customer information is deemed effective. OASIS returns null value for this field for current effective Eligible Customer or Transmission Customer record denoting that the information is in effect until further notice (open-ended). 3/14/12 accepted |
| AFFILIATE_FLAG | Required | Identifies (YES/NO) whether the Eligible Customer or Transmission Customer is affiliated with the Transmission Provider providing the service. |
| TIME_OF_LAST_UPDATE | Required | Date/time the information associated with this Eligible Customer or Transmission Customer information was last updated. |
| RECORD_STATUS | Required | Value indicating success/failure of OASIS to process the submitted request; 200 = Success. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in processing the submitted request. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. 3/14/12 accepted |

002-101.3.3.3	Network Integration Transmission Service Designated Agent Information (NITSAgent NITS OASIS DATA Template)

3/14/12 bold accepted not accepted nits

The NITSAgent NITS OASIS DATA Template information defines the basic information associated with a current or prospective new Designated Agent that may act on behalf of the Eligible Customer or Transmission Customer. For a pending NITS Application, modifications to the data specified within this NITS OASIS DATA Template may only be updated while the NITS Application has the STATUS of PRESUBMITTED.

Any update to the NITSAgent NITS OASIS DATA Template information once it has been queued for evaluation by the Transmission Provider or ultimately granted and confirmed must be submitted as a request to modify the NITS Application via the ModifyNITSAgent NITS OASIS REQUEST Template request.

The effective term of the agency relationship between Eligible Customer or Transmission Customer and Designated Agent may be open ended by specification of a null EFFECTIVE_STOP_TIME. OASIS shall limit any requests submitted by a NITS Designated Agent to be submitted (QUEUED_TIME) no earlier than the EFFECTIVE_START_TIME and no later than the EFFECTIVE_STOP_TIME in effect for that Designated Agent.

3/14/12 accepted

NITSAgent Data Element List – Request Upload:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| AGENT_CODE | Required | Registered entity code associated with the prospective NITS Designated Agent. |
| AGENT_DUNS | Required | Registered DUNS associated with the prospective NITS Designated Agent. |
| AGENT_NAME | Required | Primary contact information for the NITS Designated Agent. |
| AGENT_PHONE | Required | Primary contact information for the NITS Designated Agent. |
| AGENT_FAX | Required | Facsimile number associated with the NITS Designated Agent. 3/14/12 accepted |
| AGENT_EMAIL | Optional | Email address associated with the prospective NITS Designated Agent. |
| STATUS_NOTIFICATION | Optional | If specified, identifies the protocol (http vs. email) and required attributes necessary to direct OASIS to deliver dynamic notification on any change in information associated with the Eligible Customer or Transmission Customer’s NITS Application. 3/14/12 accepted |
| AGENT_COMMENTS | Optional | Optional comments provided by the NITS Designated Agent related to the nature of their association with the Eligible Customer or Transmission Customer. 3/14/12 accepted |
| EFFECTIVE_START_TIME | Required | Identifies the starting date/time that the agency relationship between Eligible Customer or Transmission Customer and Designated Agent begins, or the date/time that any updates to Designated Agent information becomes effective. |
| EFFECTIVE_STOP_TIME | Optional | Identifies the date/time that the agency relationship between Eligible Customer or Transmission Customer and Designated Agent ends, If null, the agency relationship is open ended and remains in effect until the end of the term of the NITS Application or terminated by supplying a non-null effective stop time. 3/14/12 accepted |

NITSAgent Data Element List – Request/Query Response:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| AGENT_CODE | Required | Registered entity code associated with the prospective NITS Designated Agent. |
| AGENT_DUNS | Required | Registered DUNS associated with the prospective NITS Designated Agent. |
| AGENT_NAME | Required | Primary contact information for the NITS Designated Agent. |
| AGENT_PHONE | Required | Primary contact information for the NITS Designated Agent. |
| AGENT_FAX | Required | Facsimile number associated with the NITS Designated Agent. 3/14/12 accepted |
| AGENT_EMAIL | May be null | Email address associated with the prospective NITS Designated Agent. |
| STATUS_NOTIFICATION | May be null | If specified, identifies the protocol (http vs. email) and required attributes necessary to direct OASIS to deliver dynamic notification on any change in information associated with the Eligible Customer or Transmission Customer’s NITS Application. 3/14/12 accepted |
| AGENT_COMMENTS | May be null | Optional comments provided by the NITS Designated Agent related to the nature of their association with the Eligible Customer or Transmission Customer. 3/14/12 accepted |
| EFFECTIVE_START_TIME | Required | Identifies the starting date/time that the agency relationship between Eligible Customer or Transmission Customer and Designated Agent begins, or the date/time that any updates to Designated Agent information becomes effective. |
| EFFECTIVE_STOP_TIME | May be null | Identifies the date/time that the agency relationship between Eligible Customer or Transmission Customer and Designated Agent ends, If null, the agency relationship is open ended and remains in effect until the end of the term if the NITS Application or terminated by supplying a non-null effective stop time. 3/14/12 accepted |
| TIME_OF_LAST_UPDATE | Required | Returned in response. |
| RECORD_STATUS | Required | Value indicating success/failure of OASIS to process the submitted request; 200 = Success. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in processing the submitted request. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. 3/14/12 accepted |

002-101.3.3.4	Network Load Description (NITSLoadDescription NITS OASIS DATA Template)

3/14/12 accepted

The NITSLoadDescription NITS OASIS DATA Template identifies the basic information related to a load to be served under a Eligible Customer or Transmission Customer’s NITS Application. Each load to be served must be uniquely identified by a Eligible Customer or Transmission Customer’s assigned LOAD_NAME and associated attributes.

3/14/12 accepted

Updates to the data specified within this NITS OASIS DATA Template may only be submitted while the associated AddNITSLoad NITS OASIS REQUEST Template request has the STATUS of PRESUBMITTED or DEFICIENT. Any update to the NITSLoadDescription NITS OASIS DATA Template information once it has been queued for evaluation by the Transmission Provider or ultimately granted and confirmed must be submitted as a request to modify the Network Load via the ModifyNITSLoad NITS OASIS REQUEST Template request.

As part of the evaluation of the information supplied for the Network Load, the Transmission Provider may require that the Eligible Customer or Transmission Customer disaggregate the identified load into multiple discreetly named loads due to transmission capability restrictions that may exist in serving the entire load at a single assigned POD and Sink (if applicable) on the Transmission Provider’s transmission system. The Transmission Provider may also direct the Eligible Customer or Transmission Customer to update the POD and/or SINK to specific values as required by the Provider to qualify for consideration as a designated load under the NITS Application.

3/14/12 accepted

For INTERRUPTIBLE loads, additional information regarding the nature of the circumstances, frequency, etc., of interruptions of the Network Load may be required as part of the Transmission Provider’s evaluation of the load being designated under the NITS Application. This information must be supplied by the Transmission Customer through OFF_OASIS communications if applicable.

3/14/12 accepted

Once established, any modification to the Network Load information must specify the effective start date/time of such modification. The stop (end) date/time of each modification to the Network Load information may be open-ended and set by OASIS to an appropriate value to indicate the information supplied is in effect until otherwise terminated by another modification to the Network Load. If approved, any Network Load information currently in effect will be terminated at the specified EFFECTIVE_START_TIME of the new modified Network Load information.

3/14/12 accepted

Elimination of the Network Load from the NITS Application is effected by setting the EFFECTIVE_STOP_TIME to that point in time after which all information that may have been provided in association with that Network Load (i.e., forecasts) shall be considered terminated or no longer considered as part of the NITS Application. If only a portion of the load is to be eliminated or undesignated, the Eligible Customer or Transmission Customer may submit revised load forecast information.

3/14/12 accepted

NITSLoadDescription Data Element List – Request Upload:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| LOAD_NAME | Required | Unique name to be associated with an Eligible Customer or Transmission Customer’s Network Load. Assigned by the Eligible Customer or Transmission Customer via the AddNITSLoad NITS OASIS REQUEST Template request, and supplied in any subsequent ModifyNITSLoad NITS OASIS REQUEST Template request to refer to that load. 3/14/12 accepted |
| LOAD_TYPE | Required | Identifies the Network Load to be served as either FIXED or INTERRUPTIBLE. |
| LOAD_AREA | Required | Identifies the BAA hosting the Network Load. |
| LOAD_SUBSTATION | Optional | For Network Load within the Transmission Provider’s host BAA, optionally identifies the primary point of interconnection with the Provider’s transmission system used to serve the majority of the Network Load by substation name. |
| LOAD_VOLTAGE | Optional | For Network Load within the Transmission Provider’s host BAA, optionally identifies the primary interconnection voltage level with the Provider’s transmission system used to serve the majority of the Network Load. |
| POINT_OF_DELIVERY | Required | Identifies the POD on the Transmission Provider’s transmission system that will be used to schedule deliveries to the Network Load. The Transmission Provider may require use of a specific POD dependent on the nature of Network Load. 3/14/12 accepted |
| SINK | Optional | Optional name of the SINK to be associated with the Network Load and used to schedule deliveries to the Network Load. Transmission Provider may require use of a specific SINK dependent on the nature of Network Load for loads directly connected to the Provider’s system. 3/14/12 accepted |
| LOAD_DESCRIPTION | Optional | Optional summary description of the nature of the load to be served under the NITS Application. |
| FORECAST_METHOD | Required | Identifies the method used to provide load forecast information to the Transmission Provider. Valid values are ON_OASIS and OFF_OASIS. When ON_OASIS is selected, the Eligible Customer or Transmission Customer shall use the NITSLoadForecast NITS OASIS DATA Template to submit the forecast information. 3/14/12 accepted |
| EFFECTIVE_START_TIME | Required | Identifies the earliest starting date/time that this Network Load may be referenced in forecasted load information or designated as a Network Load under the NITS Application. 3/14/12 accepted |
| EFFECTIVE_STOP_TIME | Optional | Identifies the date/time that the defined Network Load will no longer be considered as part of the NITS Application, If null, the Network Load’s association with the NITS Application is open ended and remains in effect until the end of the term of the NITS Application or until terminated by specifying a non-null value via ModifyNITSLoad NITS OASIS REQUEST Template request. |

NITSLoadDescription Data Element List – Request/Query Response:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| LOAD_NAME | Required | Unique name to be associated with a Customer’s Network Load. |
| LOAD_TYPE | Required | Identifies if the Network Load to be served is FIXED or INTERRUPTIBLE. |
| LOAD_AREA | Required | Identifies the BAA hosting the Network Load. |
| LOAD_SUBSTATION | May be null | For Network Load within the Transmission Provider’s host BAA, optionally identifies the primary point of interconnection with the Provider’s transmission system used to serve the majority of the Network Load by substation name. |
| LOAD_VOLTAGE | May be null | For Network Load within the Transmission Provider’s host BAA, optionally identifies the primary interconnection voltage level with the Provider’s transmission system used to serve the majority of the Network Load. |
| POINT_OF_DELIVERY | Required | Identifies the POD on the Transmission Provider’s transmission system that will be used to schedule deliveries to the Network Load. Transmission Provider may require use of a specific POD dependent on the nature of Network Load. 3/14/12 accepted |
| SINK | May be null | Optional name of the SINK to be associated with the Network Load and used to schedule deliveries to the Network Load. Transmission Provider may require use of a specific SINK dependent on the nature of Network Load for loads directly connected to the Provider’s system. 3/14/12 accepted |
| LOAD_DESCRIPTION | Optional | Optional summary description of the nature of the load to be served under the NITS Application. |
| FORECAST_METHOD | Required | Identifies the method used to provide load forecast information to the Transmission Provider. Valid values are ON_OASIS and OFF_OASIS. 3/14/12 accepted |
| EFFECTIVE_START_TIME | Required | Identifies the earliest starting date/time that the Network Load may be referenced in forecasted load information or designated as a Network Load under the NITS Application. 3/14/12 accepted |
| EFFECTIVE_STOP_TIME | May be null | Identifies the date/time that the defined Network Load will no longer be considered as part of the NITS Application, If null, the load’s association with the NITS Application is open ended. |
| TIME_OF_LAST_UPDATE | Required | Returned in response. |
| RECORD_STATUS | Required | Value indicating success/failure of OASIS to process the submitted request; 200 = Success. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in processing the submitted request. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. 3/14/12 accepted |

002-101.3.3.5	Network Load Forecast (NITSLoadForecast NITS OASIS DATA Template) [Optional]

3/14/12 accepted

The NITSLoadForecast NITS OASIS DATA Template provides information on the forecasted load to be considered when evaluating the NITS Application. The Network Load being forecasted must be uniquely identified as a named Load under the NITS Application via the NITSLoadDescription NITS OASIS DATA Template. The Transmission Provider’s use of this template is optional. The Eligible Customer or Transmission Customer should indicate their intent to use this template to submit load forecast information by setting the LOAD_METHOD element in the NITSLoadDescription NITS OASIS DATA Template to ON_OASIS.

3/14/12 accepted

Updates to the data specified within this NITS OASIS DATA Template may only be submitted while the associated AddNITSLoad or ModifyNITSLoad NITS OASIS REQUEST Templates request has the STATUS of PRESUBMITTED or DEFICIENT.

The Network Loads forecasted using this NITS OASIS DATA Template shall be incorporated into the Transmission Provider’s assessment of the on-going ability to provide NITS for the delivery of energy from the Eligible Customer or Transmission Customer’s designated resources to the designated loads under the NITS Application beyond the first year of service.

3/14/12 accepted per subcommittee wording

Submission of multiple data records associated with the NITSLoadForecast NITS OASIS DATA Template shall be supported to specify the capacity associated with the forecasted load profiled over time.

Once approved and confirmed as part of an AddNITSLoad NITS OASIS REQUEST Template request, the information supplied in this NITS OASIS DATA Template shall be added to the designated loads served under the NITS Application. Once approved and confirmed as part of a ModifyNITSLoad NITS OASIS REQUEST Template request, the information supplied in this NITS OASIS DATA Template shall replace any existing information associated with the named Network Load in its entirety over the timeframe(s) (START_TIME/STOP_TIME) specified.

3/14/12 accepted

NITSLoadForecast Data Element List – Request Upload:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| LOAD_NAME | Required | Unique name to be associated with an Eligible Customer or Transmission Customer’s Network Load to be forecasted. 3/14/12 accepted |
| SUMMER_FORECAST | Required* | Defines the Summer season peak MW capacity of the load forecasted over the specified start/stop interval. |
| WINTER_FORECAST | Required* | Defines the Winter season peak MW capacity of the load forecasted over the specified start/stop interval. |
| SUMMER_INTERRUPTIBLE | Optional | For Network Loads identified as LOAD_TYPE= INTERRUPTIBLE, the portion of the SUMMER_FORECAST that may be subject to interruption. |
| WINTER_INTERRUPTIBLE | Optional | For Network Loads identified as LOAD_TYPE= INTERRUPTIBLE, the portion of the WINTER_FORECAST that may be subject to interruption. |
| FORECAST_DESCRIPTION | Optional | Optional descriptive comments supplied by the Eligible Customer or Transmission Customer related to the load forecast information supplied. 3/14/12 accepted |
| START_TIME | Required | Defines the starting date/time for the period of time that the load is being forecasted. |
| STOP_TIME | Required | Defines the ending date/time for the period of time that the load is being forecasted. |

NITSLoadForecast Data Element List – Request/Query Response:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| LOAD_NAME | Required | Unique name to be associated with an Eligible Customer or Transmission Customer’s Network Load to be forecasted. 3/14/12 accepted |
| SUMMER_FORECAST | May be null | Defines the Summer season peak MW capacity of the load forecasted over the specified start/stop interval. |
| WINTER_FORECAST | May be null | Defines the Winter season peak MW capacity of the load forecasted over the specified start/stop interval. |
| SUMMER_INTERRUPTIBLE | May be null | For Network Loads identified as LOAD_TYPE= INTERRUPTIBLE, the portion of the SUMMER_FORECAST that may be subject to interruption. |
| WINTER_INTERRUPTIBLE | May be null | For Network Loads identified as LOAD_TYPE= INTERRUPTIBLE, the portion of the WINTER_FORECAST that may be subject to interruption. |
| FORECAST_DESCRIPTION | May be null | Optional descriptive comments supplied by the Eligible Customer or Transmission Customer related to the load forecast information supplied. 3/14/12 accepted |
| START_TIME | Required | Defines the starting date/time for the period of time that the load is being forecasted. |
| STOP_TIME | Required | Defines the ending date/time for the period of time that the load is being forecasted. |
| TIME_OF_LAST_UPDATE | Required | Time the information in the load forecast was last updated. |
| RECORD_STATUS | Required | Value indicating success/failure of OASIS to process the submitted request; 200 = Success. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in processing the submitted request. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. 3/14/12 accepted |

002-101.3.3.6	Network Integration Transmission Service Generation Description (NITSGenerationDescription NITS OASIS DATA Template)

3/14/12 bold accepted not accepted nits

The NITSGenerationDescription NITS OASIS DATA Template identifies the basic information related to a generation asset that may then be declared as a Network Resource and designated to serve load under an Eligible Customer or Transmission Customer’s NITS Application. Each generation asset must be uniquely identified by a Eligible Customer or Transmission Customer’s assigned GEN_NAME and associated attributes.

3/14/12 accepted

A given named generation asset may also be assigned to a single aggregate GEN_GROUP. The GEN_GROUP name assigned by the Eligible Customer or Transmission Customer must be unique from any other GEN_GROUP or GEN_NAME assignments under the NITS Application. The Eligible Customer or Transmission Customer may establish multiple generation groups, e.g., ‘base load’, ‘AGC’, etc., but a given physical generation asset may be a member of only one named aggregate GEN_GROUP. Resources that are backed by physical generation, as opposed to power purchase agreements, may be designated and terminated as a Network Resource (DNR) by reference to either the generation asset GEN_NAME or the aggregate non-asset specific GEN_GROUP as appropriate.

3/14/12 accepted

Updates to the data specified within this NITS OASIS DATA Template may only be submitted while the associated AddNITSGeneration NITS OASIS REQUEST Template request has the STATUS of PRESUBMITTED or DEFICIENT. Any update to the NITSGenerationDescription NITS OASIS DATA Template information once it has been queued for evaluation by the Transmission Provider or ultimately granted and confirmed must be submitted as a request to modify the generation asset via the ModifyNITSGeneration NITS OASIS REQUEST Template request.

The EFFECTIVE_START_TIME and EFFECTIVE_STOP_TIME Data Elements reflect the period of time all associated attributes defining the generation asset are to be considered in effect.  EFFECTIVE_STOP_TIME may be null to indicate the NITS generation information is open-ended and set by OASIS to an appropriate value to indicate the information supplied is in effect until otherwise terminated by another modification to the NITS generation.

3/14/12 accepted

Modifications to the basic generation attributes such as minimum/maximum operating limits, metered location, etc., shall be specified using the ModifyNITSGeneration NITS OASIS REQUEST Template request with the identification of the time interval over which those new attributes shall be in effect. Any such modifications shall replace any existing information associated with the generation asset from the specified EFFECTIVE_START_TIME forward.

3/14/12 accepted

Elimination of the NITS generation from the NITS Application is effected by setting the EFFECTIVE_STOP_TIME to that point in time after which all information that may have been provided in association with that NITS generation. Any Network Resource designations that refer to the NITS generation by GEN_NAME must be expired or indefinitely terminated on or before the specified NITS generation EFFECTIVE_STOP_TIME. If the named generation asset is the last asset active for a given defined aggregate GEN_GROUP, all Network Resource designations that refer to the NITS generation by GEN_GROUP must also be expired or indefinitely terminated on or before the specified NITS generation EFFECTIVE_STOP_TIME.

3/14/12 accepted

NITSGenerationDescription Data Element List – Request Upload:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| GEN_NAME | Required | Unique name to be associated with an Eligible Customer or Transmission Customer’s NITS generation asset. Assigned by the Eligible Customer or Transmission Customer via the AddNITSGeneration NITS OASIS REQUEST Template request, and supplied in any subsequent ModifyNITSGeneration NITS OASIS REQUEST Template request to refer to that asset. 3/14/12 accepted |
| GEN_GROUP | Optionat | Optional unique name to associate the named generation asset to an aggregate group of assets. |
| GEN_AREA | Required | Identifies the BAA where the generation asset is electrically located. |
| GEN_LOCATION | Required | Identifies the County, Parish or other locally recognized geographic area and State or Province where the generation asset is geographically located |
| GEN_OPERATOR | Required | Identifies the entity responsible for operation of the generation asset by name or registered entity code if available. 3/14/12 accepted |
| GEN_SHARE | Required | Identifies by percentage the Eligible Customer or Transmission Customer’s share if the asset is a jointly owned facility. Must be 100 for non-jointly owned assets. 3/14/12 accepted |
| GEN_MAX_CAPACITY | Required | Maximum total generating capacity of the asset. |
| GEN_MIN_CAPACITY | Required | Minimum operational generation output of the asset. |
| GEN_NORMAL_CAPACITY | Required | Normal or typical operating output of the generation asset. |
| GEN_ELIGIBLE_CAPACITY | Required | Maximum generation output that may be considered eligible to be designated by the Eligible Customer or Transmission Customer as a Network Resource under the NITS Application. 3/14/12 accepted |
| GEN_VAR_LEADING | Required | Generation asset’s maximum leading VAR capability. |
| GEN_VAR_LAGGING | Required | Generation asset’s maximum lagging VAR capability. |
| GEN_DESCRIPTION | May be null | Optional summary description of the nature of the generation asset being defined under the NITS Application, |
| EFFECTIVE_START_TIME | Required | Identifies the starting date/time that this NITS generation and associated attributes are eligible to be used as a Network Resource under the NITS Application. |
| EFFECTIVE_STOP_TIME | Optional | Identifies the date/time that the defined NITS generation and associated attributes will no longer be considered as part of the NITS Application, If null, the asset’s association with the NITS Application is open ended and remains in effect until the end of the term of the NITS Application or until terminated by specifying a non-null value via ModifyNITSLoad NITS OASIS REQUEST Template request. |

NITSGenerationDescription Data Element List – Request/Query Response:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| GEN_NAME | Required | Unique name to be associated with an Eligible Customer or Transmission Customer’s NITS generation asset. 3/14/12 accepted |
| GEN_GROUP | May be null | If defined, associates the named generation asset to an aggregate group of assets. |
| GEN_AREA | Required | Identifies the BAA where the NITS generation asset is located. |
| GEN_LOCATION | Required | Identifies the County, Parish or other locally recognized geographic area and State or Province where the generation asset is physically sited. |
| GEN_OPERATOR | Required | Identifies the entity responsible for operation of the generation asset. |
| GEN_SHARE | Required | Identifies by percentage share if the asset is a jointly owned facility. |
| GEN_MAX_CAPACITY | Required | Maximum total generating capacity of the asset. If a jointly owned facility, this represents the maximum share of the capacity of the generation asset. 3/14/12 accepted |
| GEN_MIN_CAPACITY | Required | Minimum operational generation output of the asset. |
| GEN_NORMAL_CAPACITY | Required | Normal or typical operating output of the generation asset. |
| GEN_ELIGIBLE_CAPACITY | Required | Maximum generation output that may be considered eligible to be designated as a Network Resource under the NITS Applciation. |
| GEN_VAR_LEADING | Required | Generation asset’s maximum leading VAR capability. |
| GEN_VAR_LAGGING | Required | Generation asset’s maximum lagging VAR capability. |
| GEN_DESCRIPTION | May be null | Optional summary description of the nature of the generation asset being defined under the NITS Application, |
| EFFECTIVE_START_TIME | Required* | Identifies the starting date/time that this NITS generation and associated attributes are eligible to be used as a Network Resource under the NITS Application. |
| EFFECTIVE_STOP_TIME | Optional | Identifies the date/time that the defined NITS generation and associated attributes will no longer be considered as part of the NITS Application, If null, the asset’s association with the NITS Application is open ended and remains in effect until the end of the term of the NITS Application or until terminated by specifying a non-null value via ModifyNITSLoad NITS OASIS REQUEST Template request. |
| TIME_OF_LAST_UPDATE | Required | Date/time that template information was last updated. . |
| RECORD_STATUS | Required | Value indicating success/failure of OASIS to process the submitted request; 200 = Success. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in processing the submitted request. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. 3/14/12 accepted |

002-101.3.3.7	Network Integration Transmission Service Generation Dispatch Cost (NITSGenerationDispatch NITS OASIS DATA Template) [Optional]

3/14/12 bold accepted not accepted nits

The NITSGenerationDispatch NITS OASIS DATA Template provides the Eligible Customer or Transmission Customer with the ability to upload generation dispatch cost information to the Transmission Provider as required for assessing any redispatch options or directives that may be issued to direct the redispatch of generation for reliability purposes.

Dispatch cost information is specified in blocks representing generating output capacity from a starting MW value to the next higher MW level with an associated cost. Each block submitted within a given effective start/end interval must be contiguous and monotonically increasing in capacity and cost over the operating range of the NITS generation asset.

3/14/12 accepted

Updates to the data specified within this NITS OASIS DATA Template may only be submitted while the associated AddNITSGeneration NITS OASIS REQUEST Template request has the STATUS of PRESUBMITTED or DEFICIENT. Any information specified in the update request will completely replace any previously specified information. Any update to the NITSGenerationDispatch NITS OASIS DATA Template information once it has been queued for evaluation by the Transmission Provider or ultimately granted and confirmed must be submitted as a request to modify the NITS generation via the ModifyNITSGeneration NITS OASIS REQUEST Template request.

3/14/12 accepted

The EFFECTIVE_START_TIME and EFFECTIVE_STOP_TIME data elements define the period of time over which the specified generation dispatch cost information is in effect. EFFECTIVE_STOP_TIME may be null to indicate the NITS generation dispatch information is open-ended and set by OASIS to an appropriate value to indicate the information supplied is in effect until otherwise terminated by another modification to the NITS generation.

3/14/12 accepted

Once approved and confirmed, modification of the NITS generation dispatch cost information as specified in this template will completely replace all existing dispatch cost information previously defined over the interval specified by EFFECTIVE_START_TIME/EFFECTIVE_STOP_TIME.

NITSGenerationDIspatch Data Element List – Request Upload:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| GEN_NAME | Required | Unique name to be associated with an Eligible Customer or Transmission Customer’s NITS generation asset. Assigned by the Customer via the AddNITSGeneration NITS OASIS REQUEST Template request, and supplied in any subsequent ModifyNITSGeneration NITS OASIS REQUEST Template request to refer to that asset. 3/14/12 accepted |
| DISPATCH_START (GEN_DISPATCH_START?) | Required | Identifies the starting generating output capacity associated with the dispatch profile and cost information specified. |
| DISPATCH_STOP | Required | Identifies the maximum generating output capacity that may be dispatched at the cost specified. |
| DISPATCH_COST | Required | Identifies the incremental cost of energy in $/MWh to dispatch the generation capacity from DISPATCH_START to DISPATCH_STOP. |
| DISPATCH_PRIORITY | Optional | Identifies the dispatch priority relative to other NITS generation assets supporting the NITS Application. Lower values represent higher priority for dispatch with priority=0 reserved for must-run generation commitments. 3/14/12 accepted |
| EFFECTIVE_START_TIME | Required | Identifies the starting date/time that this NITS generation dispatch cost information supplied is to be in effect. |
| EFFECTIVE_STOP_TIME | Optional | Identifies the ending date/time that this NITS generation dispatch cost information supplied is to be in effect. If null, the information supplied is open-ended and in effect until terminated\/replaced by a subsequent modification to the generation dispatch cost information. 3/14/12 accepted |

NITSGenerationDispatch Data Element List – Request/Query Response:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| GEN_NAME | Required | Unique name to be associated with an Eligible Customer or Transmission Customer’s NITS generation asset. 3/14/12 accepted |
| DISPATCH_START | Required | Identifies the starting generating output capacity associated with the dispatch profile and cost information specified. |
| DISPATCH_STOP | Required | Identifies the maximum generating output capacity that may be dispatched at the cost specified. |
| DISPATCH_COST | Required | Identifies the incremental cost of energy in $/MWh to dispatch the generation capacity from DISPATCH_START to DISPATCH_STOP. |
| DISPATCH_PRIORITY | May be null | Identifies the dispatch priority relative to other NITS generation assets supporting the NITS Application. Lower values represent higher priority for dispatch with priority=0 reserved for must-run generation commitments. 3/14/12 accepted |
| EFFECTIVE_START_TIME | Required | Identifies the starting date/time that this NITS generation dispatch cost information supplied is to be in effect. |
| EFFECTIVE_STOP_TIME | May be null | Identifies the ending date/time that this NITS generation dispatch cost information supplied is to be in effect. If null, the information supplied is open-ended and in effect until terminated\/replaced by a subsequent modification to the generation dispatch cost information. 3/14/12 accepted |
| TIME_OF_LAST_UPDATE | Required | Date/time the template information was last updated. |
| RECORD_STATUS | Required | Value indicating success/failure of OASIS to process the submitted request; 200 = Success. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in processing the submitted request. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. 3/14/12 accepted |

002-101.3.3.8	Network Resource Description (NITSResourceDescription NITS OASIS DATA Template)

3/14/12 accepted

The NITSResourceDescription NITS OASIS DATA Template identifies the basic information related to a Network Resource that may then be designated to serve load under a Eligible Customer or Transmission Customer’s NITS Application. Each Network Resource must be uniquely identified by a Eligible Customer or Transmission Customer’s assigned RESOURCE_NAME and associated attributes.

3/14/12 accepted

Updates to the data specified within this NITS OASIS DATA Template may only be submitted while the associated AddNITSResource or AddNITSDNR NITS OASIS REQUEST Templates request has the STATUS of PRESUBMITTED or DEFICIENT. Any update to the NITSResourceDescription NITS OASIS DATA Template information once it has been queued for evaluation by the Transmission Provider or ultimately granted and confirmed must be submitted as a request to modify the Network Resource via the ModifyNITSResource NITS OASIS REQUEST Template request.

EFFECTIVE_STOP_TIME is the only template Data Element that may be modified via the ModifyNITSResource NITS OASIS REQUEST Template request, and establishes the point in time after which all information that may have been provided in association with that Network Resource. i.e., designations and forecasts, shall be considered terminated or no longer considered as part of the NITS Application.

Network Resources may be defined and associated with a projected forecast of capacity to be used to serve load under the NITS Application without being declared as a DNR. The designation and termination of a DNR is handled independently from the basic definition and forecasting of Network Resources under the NITS Application.

3/14/12 accepted

NITSResourceDescription Data Element List – Request Upload:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| RESOURCE_NAME | Required | Unique name to be associated with an Eligible Customer or Transmission Customer’s Network Resource. 3/14/12 accepted |
| RESOURCE_CLASS | Required | Identifies the nature of the Network Resource as either ON_SYSTEM or OFF_SYSTEM. |
| RESOURCE_TYPE | Required | Identifies the nature of the Network Resource as either a physical generation asset (GENERATION) or power purchase agreement (EXECUTED_PPA or CONTINGENT_PPA). |
| SOURCE_AREA | Required | Identifies the BAA that is the source for energy deliveries from this defined resource. . |
| TITLE_AREA | Required | Identifies the BAA where the Eligible Customer or Transmission Customer assumes responsibility (title) to the energy delivered by this defined resource. . |
| RESOURCE_DESCRIPTION | Optional | Optional Eligible Customer or Transmission Customer supplied description of the nature of the Network Resource. 3/14/12 accepted |
| FORECAST_METHOD | Required | Identifies the method used to provide resource forecast (projection) information to the Transmission Provider. Valid values are ON_OASIS and OFF_OASIS. When ON_OASIS is selected, the Eligible Customer or Transmission Customer shall use the NITSResourceForecast NITS OASIS DATA Template to submit the forecast information. 3/14/12 accepted |
| EFFECTIVE_START_TIME | Required | Identifies the earliest starting date/time that the Network Resource may be referenced in forecasted load information or designated as a Network Resource under the NITS Application. |
| EFFECTIVE_STOP_TIME | Optional | Identifies the date/time that the defined Network Resource will no longer be considered as part of the NITS Application, If null, the resource’s association with the NITS Application is open ended. 3/14/12 accepted |

NITSResourceDescription Data Element List – Request/Query Response:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| RESOURCE_NAME | Required | Unique name to be associated with an Eligible Customer or Transmission Customer’s Network Resource. 3/14/12 accepted |
| RESOURCE_CLASS | Required | Identifies the nature of the Network Resource as either ON_SYSTEM or OFF_SYSTEM. |
| RESOURCE_TYPE | Required | Identifies the nature of the Network Resource as either GENERATION or EXECUTED_PPA or CONTINGENT_PPA. |
| SOURCE_AREA | Required | Identifies the BAA that is the source for energy deliveries from this defined resource. |
| TITLE_AREA | Required | Identifies the BAA where the Eligible Customer or Transmission Customer assumes responsibility (title) to the energy delivered by this defined resource. . |
| RESOURCE_DESCRIPTION | May be null | Optional Eligible Customer or Transmission Customer supplied description of the nature of the Network Resource. 3/14/12 accepted |
| FORECAST_METHOD | Required | Identifies the method used to provide load forecast information to the Transmission Provider. Valid values are ON_OASIS and OFF_OASIS. 3/14/12 accepted |
| EFFECTIVE_START_TIME | Required | Identifies the earliest starting date/time that the Network Resource may be referenced in forecasted load information or designated as a Network Resource under the NITS Application. |
| EFFECTIVE_STOP_TIME | May be null | Identifies the date/time that the defined Network Resource will no longer be considered as part of the NITS Application, If null, the resource’s association with the NITS Application is open ended. 3/14/12 accepted |
| TIME_OF_LAST_UPDATE | Required | Date/time the template information was last updated. |
| RECORD_STATUS | Required | Value indicating success/failure of OASIS to process the submitted request; 200 = Success. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in processing the submitted request. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. 3/14/12 accepted |

002-101.3.3.9	Network Resource Forecast (NITSResourceForecast NITS OASIS DATA Template) [Optional]

3/14/12 accepted

The NITSResourceForecast NITS OASIS DATA Template provides information on a Network Resource’s forecasted use to serve load under the NITS Application. The Network Resource being forecasted must be uniquely identified as a named Resource under the NITS Application via the NITSResourceDescription NITS OASIS DATA Template. The Transmission Provider’s use of this template is optional. The Eligible Customer or Transmission Customer should indicate their intent to use this template to submit load forecast information by setting the FORECAST_METHOD element in the NITSResourceDescription NITS OASIS DATA Template to ON_OASIS.

3/14/12 accepted

Updates to the data specified within this NITS OASIS DATA Template may only be submitted while the associated AddNITSResource, ModifyNITSResource, or AddNITSDNR NITS OASIS REQUEST Templates request has the STATUS of PRESUBMITTED or DEFICIENT.

The Network Resources’ forecasted using this NITS OASIS DATA Template shall be incorporated into the Transmission Provider’s assessment of the on-going ability to provide NITS for the delivery of energy from the Eligible Customer or Transmission Customer’s designated resources to the designated loads under the NITS Application beyond the first year of service.

3/14/12 accepted

Submission of multiple data records associated with the NITSResourceForecast NITS OASIS DATA Template shall be supported to specify the capacity associated with the forecasted resource use profiled over time.

Once approved and confirmed as part of an AddNITSResource or AddNITSDNR NITS OASIS REQUEST Templates request, the information supplied in this NITS OASIS DATA Template shall be added to the information associated with the resources designated/forecasted to serve load under the NITS Application. Once approved and confirmed as part of a ModifyNITSResource NITS OASIS REQUEST Template request, the information supplied in this NITS OASIS DATA Template shall replace any existing information associated with the named Network Resource in its entirety over the timeframe(s) specified.

3/14/12 accepted

NITSResourceForecast Data Element List – Request Upload:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| RESOURCE_NAME | Required | Unique name to be associated with an Eligible Customer or Transmission Customer’s Network Resource to be forecasted. 3/14/12 accepted |
| SUMMER_FORECAST | Required* | Defines the Summer season peak MW capacity of the resource forecasted over the specified start/stop interval. |
| WINTER_FORECAST | Required* | Defines the Winter season peak MW capacity of the resource forecasted over the specified start/stop interval. |
| FORECAST_DESCRIPTION | Optional | Optional descriptive comments supplied by the Eligible Customer or Transmission Customer related to the forecasted use of the Network Resource. 3/14/12 accepted |
| START_TIME | Required | Defines the starting date/time for the period of time that the resource is being forecasted. |
| STOP_TIME | Required | Defines the ending date/time for the period of time that the resource is being forecasted. |

NITSResourceForecast Data Element List – Request/Query Response:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| RESOURCE_NAME | Required | Unique name to be associated with an Eligible Customer or Transmission Customer’s Network Resource forecasted. 3/14/12 accepted |
| SUMMER_FORECAST | May be null | Defines the Summer season peak MW capacity of the resource forecasted over the specified start/stop interval. |
| WINTER_FORECAST | May be null | Defines the Winter season peak MW capacity of the resource forecasted over the specified start/stop interval. |
| FORECAST_DESCRIPTION | May be null | Optional descriptive comments supplied by the Eligible Customer or Transmission Customer related to the forecasted use of the Network Resource. 3/14/12 accepted |
| START_TIME | Required | Defines the starting date/time for the period of time that the resource is being forecasted. |
| STOP_TIME | Required | Defines the ending date/time for the period of time that the resource is being forecasted. |
| TIME_OF_LAST_UPDATE | Required | Time the information in the load forecast was last updated. |
| RECORD_STATUS | Required | Value indicating success/failure of OASIS to process the submitted request; 200 = Success. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in processing the submitted request. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. 3/14/12 accepted |

002-101.3.3.10	Network Resource Designation (NITSResourceDesignation NITS OASIS DATA Template)

3/14/12 accepted

The NITSResourceDesignation NITS OASIS DATA Template identifies the specific information required for an Eligible Customer or Transmission Customer to identify and attest to a named Network Resource as being qualified for consideration as a DNR under the NITS Application via the AddNITSDNR NITS OASIS REQUEST Template request. This template also provides the information necessary to terminate on a temporary or indefinite basis a named Network Resource via the TerminateNITSDNR NITS OASIS REQUEST Template request.

3/14/12 accepted

When used to designate a Network Resource (AddNITSDNR NITS OASIS REQUEST Template), the Eligible Customer or Transmission Customer must specify the POR and Source that the Transmission Provider will evaluate for the deliverability of the Network Resource to Network Load on a firm basis. The Transmission Provider may require a change to use a specific POR and/or Source names as part of their evaluation process. The Eligible Customer or Transmission Customer must also supply the necessary attestation that the resource being designated meets all the criteria required for treatment as a DNR. If the request to designate the Network Resource represents an extension to the term of an existing DNR that qualifies for on-going rollover rights as specified in the Transmission Provider’s Business Practices or the OATT, the DNR_ACTION must be set to DESIGNATION_EXTENSION rather than simple DESIGNATION.

3/14/12 accepted

If the Network Resource to be designated is identified as an OFF_SYSTEM resource, the Eligible Customer or Transmission Customer must supply the identification of all transmission arrangements available for use by the Eligible Customer or Transmission Customer for delivery of energy across external Transmission Providers’ systems via the AuxiliaryTransmission NITS OASIS DATA Template. The DNR designation may be identified as part of a Coordinated Group of transmission requests across multiple Transmission Providers’ systems by setting the CG_STATUS data element to PROPOSED. The external transmission requests that are to be considered as part of the Coordinated Group of requests must be identified in the AuxiliaryTransmission NITS OASIS DATA Template as dictated by Business Practice Standard WEQ-001-23.

3/14/12 accepted

When used for the termination of a DNR (TerminateNITSDNR NITS OASIS REQUEST Template) on a temporary basis, this template identifies the resource to be terminated by resource name and the specific POR and Source that was previously specified when the resource was designated. The attestation information supplied is required from the Eligible Customer or Transmission Customer to assert that the resource being temporarily terminated qualifies for continued treatment as a designated resource at the end of the time periods and in the capacity amounts that are being temporarily terminated. Indefinite termination of the resource does not require this attestation and the capacity over time indefinitely terminated may only be designated as a Network Resource in the future through submission and approval/confirmation of a new AddNITSDNR NITS OASIS REQUEST Template request.

3/14/12 accepted

Updates to the data specified within this NITS OASIS DATA Template may only be submitted while the associated AddNITSDNR or TerminateNITSDNR NITS OASIS REQUEST Templates request has the STATUS of PRESUBMITTED or DEFICIENT. An exception to this restriction is allowed for the DESIGNATION of a DNR with the AddNITSDNR NITS OASIS REQUEST Template request which has been flagged as a Coordinated Request (CG_STATUS=PROPOSED). The CG_STATUS Data Element may be updated using the UpdateNITSRequest NITS OASIS INPUT Template to attest that the Coordinated Group information has been supplied in the AuxiliaryTransmission NITS OASIS DATA Template information by setting CG_STATUS to ATTESTED. All other template Data Elements should be submitted with null values; any values submitted other than CG_STATUS in these cases will be ignored.

3/14/12 accepted

Once designated, any modification to the DNR must be submitted via the TerminateNITSDNR NITS OASIS REQUEST Template request. Once terminated on either temporary or indefinite basis, the Network Resource may only be restored as a DNR within the period and capacity of the termination through submission of a new AddNITSDNR NITS OASIS REQUEST Template request.

3/14/12 accepted

The amount of resource capacity to be designated or terminated must be specified using the NITSResourceCapacity NITS OASIS DATA Template. The EFFECTIVE_START_TIME and EFFECTIVE_STOP_TIME specified in the NITSResourceDesignation NITS OASIS DATA Template must span the full period of time of the designation or termination being requested. Indefinite terminations must specify a null EFFECTIVE_STOP_TIME indicating that the termination is open-ended for all time.

3/14/12 accepted

NITSResourceDesignation Data Element List – Request Upload:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| RESOURCE_NAME | Required | Unique name to be associated with an Eligible Customer or Transmission Customer’s Network Resource to be designated/terminated. (from NITSResourceDescription NITS OASIS DATA Template). 3/14/12 accepted |
| DNR_ACTION | Required | Identifies the nature of requested action to take with respect to the Network Resource to be one of: DESIGNATION, DESIGNATION_EXTENSION, TEMPORARY_TERMINATION, INDEFINITE_TERMINATION. |
| POINT_OF_RECEIPT | Required | Identifies the POR on the Transmission Provider’s system for delivering energy from the DNR to Network Load. |
| SOURCE | Optional | Identifies the source point name to be used in scheduling deliveries from the DNR. Transmission Provider may require use of a specific SOURCE in association with the DNR. 3/14/12 accepted |
| DNR_ATTESTATION | Required* | Required attestation that the Network Resource to be designated qualifies as a DNR, or in the case of TEMPORARY_UNDESIGNATION that the resource continues to qualify as a DNR at the end of the period of temporary termination. This field must contain the required Transmission Provider’s statement of attestation verbatim. This field must be null for an INDEFINITE_TERMINATION. 3/14/12 accepted |
| ATTESTOR_NAME | Required* | Required with DNR_ATTESTATION identifying the name and title of the individual responsible for attesting to the Network Resource’s qualification as a DNR. This field must be null for an INDEFINITE_TERMINATION. |
| ATTESTATION_SUBMITTER | Required* | Required with DNR_ATTESTATION identifying the name and title of the individual submitting the statement of the Network Resources qualification as a DNR. This field must be null for an INDEFINITE_TERMINATION. |
| POSTING_REF | Optional | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| SALE_REF | Optional | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| REQUEST_REF | Optional | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| DEAL_REF | Optional | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| CG_STATUS | Optional | Optional flag to indicate to the Transmission Provider that a DNR DESIGNATION request is to be considered and evaluated as a coordinated request with other external Transmission Provider requests identified in the AuxiliaryTransmission NITS OASIS DATA Template. One of: PROPOSED ATTESTED <null> 3/14/12 accepted |
| EFFECTIVE_START_TIME | Required | Identifies the starting date/time that the Network Resource is either designated or terminated. |
| EFFECTIVE_STOP_TIME | Required* | Identifies the ending date/time of resource’s designation or the time at which the resource is restored following a temporary termination. This field must be null for an INDEFINITE_TERMINATION. |

NITSResourceDesignation Data Element List – Request/Query Response:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| RESOURCE_NAME | Required | Unique name to be associated with an Eligible Customer or Transmission Customer’s Network Resource to be designated/terminated. (from NITSResourceDescription NITS OASIS DATA Template). 3/14/12 accepted |
| DNR_ACTION | Required | Identifies the nature of requested action to take with respect to the Network Resource to be one of: DESIGNATION, TEMPORARY_TERMINATION, INDEFINITE_TERMINATION. |
| POINT_OF_RECEIPT | Required | Identifies the POR on the Transmission Provider’s system for delivering energy from the DNR to Network Load. |
| SOURCE | May be null | Identifies the source point name to be used in scheduling deliveries from the DNR. Transmission Provider may require use of a specific SOURCE in association with the DNR. 3/14/12 accepted |
| DNR_ATTESTATION | May be null | Required attestation that the Network Resource to be designated qualifies as a DNR, or in the case of TEMPORARY_UNDESIGNATION that the resource continues to qualify as a DNR at the end of the period of temporary termination. This field must contain the required Transmission Provider’s statement of attestation verbatim. This field must be null for an INDEFINITE_TERMINATION. 3/14/12 accepted |
| ATTESTOR_NAME | May be null | Required with DNR_ATTESTATION identifying the name and title of the individual responsible for attesting to the Network Resource’s qualification as a DNR. This field must be null for an INDEFINITE_TERMINATION. |
| ATTESTATION_SUBMITTER | May be null | Required with DNR_ATTESTATION identifying the name and title of the individual submitting the attestation of the Network Resources qualification as a DNR. This field must be null for an INDEFINITE_TERMINATION. |
| POSTING_REF | Optional | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| SALE_REF | Optional | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| REQUEST_REF | Optional | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| DEAL_REF | Optional | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| CG_STATUS | May be null | Optional flag identifying if the DNR DESIGNATION request is to be considered and evaluated as a coordinated request. One of: PROPOSED ATTESTED <null> 3/14/12 accepted |
| EFFECTIVE_START_TIME | Required | Identifies the starting date/time that the Network Resource is either designated or terminated. |
| EFFECTIVE_STOP_TIME | May be null | Identifies the ending date/time of resource’s designation or the time at which the resource is restored following a temporary termination. This field must be null for an INDEFINITE_TERMINATION. |
| CG_DEADLINE | May be null | Set to non-null value on submission a Coordinated Request CG_STATUS=PROPOSED based on time the request is first set to QUEUED and time limit for Eligible Customer or Transmission Customer supply and attest to contiguity of the Coordinated Group. |
| CONCOMITANT_EVAL_FLAG | Required | Returned by OASIS as Y/N identifying if the request was submitted for Concomitant Evaluation by the Transmission Provider using the NewConcomitantRequest NITS OASIS INPUT Template. |
| TIME_OF_LAST_UPDATE | Required | Date/time the information associated with the template was last updated. |
| RECORD_STATUS | Required | Value indicating success/failure of OASIS to process the submitted request; 200 = Success. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in processing the submitted request. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. 3/14/12 accepted |

002-101.3.3.11	Designated Network Resource Capacity (NITSResourceCapacity NITS OASIS DATA Template)

3/14/12 accepted

The NITSResourceCapacity NITS OASIS DATA Template must be supplied in combination with the NITSResourceDesignation NITS OASIS DATA Template to specify the amount of capacity over time that is being designated or terminated from the named Network Resource via the AddNITSDNR or TerminateNITSDNR NITS OASIS REQUEST Templates request. Profiled capacity amounts over time and across multiple generation assets are specified using multiple Data Records following the NITSResourceCapacity NITS OASIS DATA Template Header Record and up to the next template Header Record or end of the submitted data.

3/14/12 accepted

When included in a request to designate a network resource (AddNITSDNR NITS OASIS REQUEST Template), the Eligible Customer or Transmission Customer must specify the amount of capacity requested to be designated from the named Network Resource. The capacity must be specified as positive integer MW values or zero (0). If the Transmission Provider cannot accommodate the resource designation in full, the amount of capacity that may be accommodated will be specified in CAPACITY_GRANTED with the AddNITSDNR NITS OASIS REQUEST Template status of COUNTEROFFER. The Eligible Customer or Transmission Customer may accept the CAPACITY_GRANTED or propose a lower amount to be designated using the status of REBID. CAPACITY_REQUESTED and CAPACITY_GRANTED must be equal to confirm the AddNITSDNR NITS OASIS REQUEST Template request.  Once confirmed the capacity granted to the DNR over time will be added to any capacity previously designated to that named Network Resource.

3/14/12 accepted

When included in the request to terminate a DNR (TerminateNITSDNR NITS OASIS REQUEST Template), the Eligible Customer or Transmission Customer must specify the amount of capacity to be terminated from the named Network Resource over time. The capacity must be specified as negative integer MW values or zero (0). Once confirmed, the capacity terminated from the DNR over time will be subtracted from the capacity designated for that named Network Resource. If the capacity specified to be terminated is greater than the actual capacity designated for the Network Resource, the Network Resource’s designated capacity will be set to 0.

The START_TIME/STOP_TIME intervals specified with this template must be within and span continuously over the EFFECTIVE_START_TIME and EFFECTIVE_END_TIME of the associated network resource designation or termination being requested. For indefinite termination of a DNR, only one MW value may be specified representing the MW capacity to be terminated from the designated resource beginning at the specified START_TIME through the entire term of the Network Resource’s designation; STOP_TIME must be specified as a null value.

3/14/12 accepted

If the Network Resource being designated or terminated is declared as a GENERATION backed resource, either the specific NITS generation asset(s), by GEN_NAME, or the associated aggregrate generation group, by GEN_GROUP, must be specified. Multiple generation assets may be assigned specific capacity amounts to be designated or terminated over time through submission of multiple Data Records in the NITSResourceCapacity NITS OASIS DATA Template. However, a given named generation asset (or aggregate group) must not be specified in more than one data record for any given point in time. An error will be returned if a required generation asset name (or aggregate group) is not supplied, or references an unknown asset, or is associated with more than one capacity value for a given point in time. There is no restriction that a Network Resource designated from an aggregate group of generation assets may be terminated as either an aggregate group or on an individual generation asset basis.

3/14/12 accepted

Updates to the data specified within this NITS OASIS DATA Template may only be submitted while the associated AddNITSDNR or TerminateNITSDNR NITS OASIS REQUEST Templates request has the STATUS of PRESUBMITTED or DEFICIENT or COUNTEROFFER.

NITSResourceCapacity Data Element List – Request Upload:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| RESOURCE_NAME | Required | Unique name to be associated with an Eligible Customer or Transmission Customer’s Network Resource to be designated/terminated. (from NITSResourceDescription NITS OASIS DATA Template). 3/14/12 accepted |
| GEN_NAME | Required* | Required if the Network Resource being designated or terminated has been identified with RESOURCE_TYPE=GENERATION. Identifies the generation asset by either the asset GEN_NAME or an associated aggregate GEN_GROUP that is to be designated or terminated. 3/14/12 accepted |
| CAPACITY_REQUESTED | Required | Identifies the amount of capacity to be designated (positive value) or terminated (negative value) from the named Network Resource. May be updated by the Eligible Customer or Transmission Customer when the AddNITSDNR NITS OASIS REQUEST Template request status is set to COUNTEROFFER (or CR_COUNTEROFFER) by the Transmission Provider. 3/14/12 accepted |
| START_TIME | Required | Identifies the beginning of the time interval for the capacity to be designated or terminated. |
| STOP_TIME | Required* | Identifies the end of the time interval for the capacity to be designated or terminated. Must be null for an indefinite termination. |
| CAPACITY_GRANTED | Must be null | Returned in response. |

NITSResourceCapacity Data Element List – Request/Query Response:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| RESOURCE_NAME | Required | Unique name to be associated with an Eligible Customer or Transmission Customer’s Network Resource to be designated/terminated. (from NITSResource NITS OASIS DATA Template). 3/14/12 accepted |
| GEN_NAME | May be null | Required if the Network Resource being designated or terminated has been identified with RESOURCE_TYPE=GENERATION. Identifies the generation asset by either the asset GEN_NAME or an associated aggregate GEN_GROUP that is to be designated or terminated. 3/14/12 accepted |
| CAPACITY_REQUESTED | Required | Identifies the amount of capacity to be designated (positive value) or terminated (negative value) from the named Network Resource. |
| START_TIME | Required | Identifies the beginning of the time interval for the capacity to be designated or terminated. |
| STOP_TIME | May be null | Identifies the end of the time interval for the capacity to be designated or terminated. Must be null for an indefinite termination. |
| CAPACITY_GRANTED | May be null | Specified by the Transmission Provider after evaluation with capacity amount that may be accommodated. |
| TIME_OF_LAST_UPDATE | Required | Date/time the information associated with the template was last updated. |
| RECORD_STATUS | Required | Value indicating success/failure of OASIS to process the submitted request; 200 = Success. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in processing the submitted request. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. 3/14/12 accepted |

002-101.3.3.12	Designated Network Resource Scheduling Rights (NITSSchedulingRights NITS OASIS DATA Template) [Optional]

3/14/12 accepted

Use of the NITSSchedulingRights NITS OASIS DATA Template is at the discretion of the Transmission Provider as established in their Business Practices.

The Transmission Provider may require the Eligible Customer or Transmission Customer to supply the information specified in this template as part of the AddNITSDNR request per their established Business Practices. In this case, the NITSSchedulingRights NITS OASIS DATA Template must be supplied by the Eligible Customer or Transmission Customer in combination with the NITSResourceDesignation NITS OASIS DATA Template to specify the amount of incremental Transmission Service that is being requested to support delivery of energy from the DNR to Network Load under the NITS Application.

3/14/12 accepted

If the Transmission Provider does not require submission of the NITSSchedulingRights NITS OASIS DATA Template by the Eligible Customer or Transmission Customer, but elects by Business Practice to use this template to convey information on the scheduling rights (to be) granted for delivery of energy from the DNR to Network Load, the Transmission Provider may supply the information associated with this DATA Template as part of the AddNITSDNR request at any time.

3/14/12 accepted

After evaluation by the Transmission Provider, the information specified in this template may be updated and made available for query and possible adjustment by the Eligible Customer or Transmission Customer to confirm the final scheduling rights granted for delivery of energy from the DNR if the full amount of the DNR is limited for delivery to one or more designated Network Loads.

Updates to the data specified within this NITS OASIS DATA Template may only be submitted by the Eligible Customer or Transmission Customer while the associated AddNITSDNR request has the STATUS of PRESUBMITTED or DEFICIENT or COUNTEROFFER.

3/14/12 accepted

Upon final confirmation of the AddNITSDNR request, the amount of capacity specified for CAPACITY_GRANTED over time shall be added to any existing scheduling rights available over the same transmission path for scheduling energy from the DNR to Network Load.

NITSSchedulingRights Data Element List – Request Upload:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| RESOURCE_NAME | Required | Unique name to be associated with an Eligible Customer or Transmission Customer’s Network Resource to be designated. (from NITSResourceDescription NITS OASIS DATA Template). 3/14/12 accepted |
| PATH_NAME | Optional | Identifies the OASIS path to be used for scheduling the DNR if required by the Transmission Provider in addition to POR/POD and Source/Sink. |
| POINT_OF_RECEIPT | Required | Identifies the POR on the Transmission Provider’s transmission system that will be scheduled for delivery of the DNR. Must match the POINT_OF_RECEIPT as specified in the NITSResourceDesignation NITS OASIS DATA Template. 3/14/12 accepted |
| POINT_OF_DELIVERY | Required | Identifies the POD on the Transmission Provider’s transmission system that will be scheduled for delivery of the DNR to the designated Network Load(s) under the NITS Application. Must match the POINT_OF_DELIVERY as specified in the NITSLoadDescription NITS OASIS DATA Template. 3/14/12 accepted |
| SOURCE | Optional | Identifies the named Source that has been assigned for use in scheduling the delivery of the DNR. Must match the Source as specified in the NITSResourceDesignation NITS OASIS DATA Template. |
| SINK | Optional | Identifies the named Sink on the Transmission Provider’s transmission system that will be scheduled for delivery of the DNR to the designated Network Load(s) under the NITS Application. Must match the SINK as specified in the NITSLoadDescription NITS OASIS DATA Template. 3/14/12 accepted |
| CAPACITY_REQUESTED | Required | Identifies the amount of incremental Transmission Service capacity requested for scheduling delivery of the DNR to designated Network Load(s) under the NITS Application. If not explicitly specified by the Eligible Customer or Transmission Customer, this value will be the sum of all capacity designated for the resource at each point in time over the term of the DNR. 3/14/12 accepted |
| START_TIME | Required | Identifies the beginning of the time interval for the corresponding scheduling rights being requested or confirmed to support the DNR. |
| STOP_TIME | Required | Identifies the end of the time interval for the corresponding scheduling rights being requested or confirmed to support the DNR. |

NITSSchedulingRights Data Element List – Request/Query Response:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| RESOURCE_NAME | Required | Unique name to be associated with an Eligible Customer or Transmission Customer’s Network Resource to be designated. (from NITSResourceDescription NITS OASIS DATA Template). 3/14/12 accepted |
| PATH_NAME | May be null | Identifies the OASIS path to be used for scheduling the DNR if required by the Transmission Provider in addition to POR/POD and Source/Sink. |
| POINT_OF_RECEIPT | Required | Identifies the POR on the Transmission Provider’s transmission system that will be scheduled for delivery of the DNR. Must match the POINT_OF_RECEIPT as specified in the NITSResourceDesignation NITS OASIS DATA Template. 3/14/12 accepted |
| POINT_OF_DELIVERY | Required | Identifies the POD on the Transmission Provider’s transmission system that will be scheduled for delivery of the DNR to the designated Network Load(s) under the NITS Application. Must match the POINT_OF_DELIVERY as specified in the NITSLoadDescription NITS OASIS DATA Template. 3/14/12 accepted |
| SOURCE | May be null | Identifies the named Source that has been assigned for use in scheduling the delivery of the DNR. Must match the Source as specified in the NITSResourceDesignation NITS OASIS DATA Template. |
| SINK | May be null | Identifies the named Sink on the Transmission Provider’s transmission system that will be scheduled for delivery of the DNR to the designated Network Load(s) under the NITS Application. Must match the SINK as specified in the NITSLoadDescription NITS OASIS DATA Template. 3/14/12 accepted |
| CAPACITY_REQUESTED | Required | Identifies the amount of incremental Transmission Service capacity requested for scheduling delivery of the DNR to designated Network Load(s) under the NITS Application. If not explicitly specified by the Eligible Customer or Transmission Customer, this value will be the sum of all capacity designated for the resource at each point in time over the term of the DNR. 3/14/12 accepted |
| START_TIME | Required | Identifies the beginning of the time interval for the corresponding scheduling rights being requested or confirmed to support the DNR. |
| STOP_TIME | Required | Identifies the end of the time interval for the corresponding scheduling rights being requested or confirmed to support the DNR. |
| CAPACITY_GRANTED | May be null | Specified by the Transmission Provider after evaluation with capacity amount that may be accommodated. |
| NERC_CURTAILMENT_PRIORITY | Required | Identifies the curtailment priority for delivery of the Network Resource as either a firm DNR (7) or as a non-firm (secondary) non-designated resource (6). |
| OTHER_CURTAILMENT_PRIORITY | May be null | Optional Transmission Provider specific curtailment priority assigned to the rights granted, |
| TIME_OF_LAST_UPDATE | Required | Date/time the information associated with the template was last updated. |
| RECORD_STATUS | Required | Value indicating success/failure of OASIS to process the submitted request; 200 = Success. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in processing the submitted request. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. 3/14/12 accepted |

002-101.3.3.13	Auxiliary Transmission Rights (AuxiliaryTransmission NITS OASIS DATA Template)

3/14/12 accepted

The AuxiliaryTransmission NITS OASIS DATA Template may be supplied in combination with the NITSResourceDesignation NITS OASIS DATA Template to identify the transmission arrangements on other parties’ transmission systems that are required to support the delivery of energy from the DNR to Network Load. The Transmission Provider may use this information to verify that the Eligible Customer or Transmission Customer has secured the necessary firm transmission rights in support of the DNR. Multiple Data Records may be submitted in association with this template as needed to document each external Transmission Provider and each external transmission service reservation that may be used in delivering energy from the DNR to Network Load.

3/14/12 accepted

This NITS OASIS DATA Template will also be supplied when a request to designate a Network Resource (AddNITSDNR NITS OASIS REQUEST Template), arrange Secondary Network Transmission Service (AddNITSSecondary NITS OASIS REQUEST Template), or request additional PTP Transmission Service as a concomitant request (AddPTPService NITS OASIS REQUEST Template) are to be considered and evaluated as part of a Coordinated Group of requests across multiple Transmission Providers. As a Coordinated Request with CG_STATUS set to PROPOSED, the set of requests that comprise the Coordinated Group must be supplied in this template with the information as dictated by Business Practice Standard WEQ-001-23. Existing reservations may be included as part of the Coordinated Group to meet the contiguity requirements and are submitted with the CR_DISPOSITION Data Element set to a value of CONFIRMED.

3/14/12 accepted

Updates to the data specified within this NITS OASIS DATA Template may be submitted at any time while the associated AddNITSDNR, AddNITSSecondary or AddPTPService NITS OASIS REQUEST Templates request STATUS is PRESUBMITTED or DEFICIENT, if applicable. Updates may also be supplied in association with any Coordinated Request where that request has the CG_STATUS Data Element set to a value of PROPOSED and the update is supplied prior to expiration of the CG_DEADLINE submission time limit.

3/14/12 accepted

The AuxiliaryTransmission NITS OASIS DATA Template information specified in any update may be supplied in an incremental fashion as follows. For any data record supplied where the CR_PRIMARY_PROVIDER_CODE and CR_ASSIGNMENT_REF Data Elements do not match with any existing data supplied with the AuxiliaryTransmission NITS OASIS DATA Template, the specified data record will be added to the information specified in this NITS OASIS DATA Template. If the CR_PRIMARY_PROVIDER_CODE and CR_ASSIGNMENT_REF Data Elements match an existing data record supplied with the AuxiliaryTransmission NITS OASIS DATA Template and the CR_DISPOSITION Data Element is set to DELETED, the information for that data record will be removed from the existing information associated with this NITS OASIS DATA Template. Finally, If the CR_PRIMARY_PROVIDER_CODE and CR_ASSIGNMENT_REF Data Elements match an existing data record supplied for the AuxiliaryTransmission NITS OASIS DATA Template and the CR_DISPOSITION Data Element is not set to DELETED, the information for that data record will be replaced in its entirety with the information supplied in the update request submission.

3/14/12 accepted

For Coordinated Requests when the associated CG_STATUS Data Element has been set to ATTESTED, any update that would remove or add any associated Coordinated Requests in the Coordinated Group that have been previously submitted in this template, i.e., attempt to add or delete any data record with CR_DISPOSITION other than CONFIRMED, shall be blocked and return an error. Once the CG_FLAG has been set to ATTESTED, updates to the existing Coordinated Requests are limited to modifying the CR_DISPOSITION and CR_DISPOSITION_TIME Data Elements.

3/14/12 accepted

In the event that existing confirmed transmission rights on an external Transmission Provider’s system are associated with network service, the CR_ASSIGNMENT_REF Data Element shall specify the value for the APPLICATION_REF associated with the CONFIRMED NITS Application.

AuxiliaryTransmission Data Element List – Request Upload:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| CR_PRIMARY_PROVIDER_CODE | Required | Identifies, by registered entity code, the external Transmission Provider whose transmission system is being used in delivery of the DNR or whose transmission system is part of the Coordinated Group of requests or reservations to be coordinated. |
| CR_ASSIGNMENT_REF | Required | OASIS unique identifier assigned to the Transmission Service reservation held on the external Transmission Provider’s system or that is associated with a reservation or request for service that is part of the Coordinated Group. If existing NITS service is being used (CR_DISPOSITION=CONFIRMED), specifies the APPLICATION_REF associated with that NITS service. 3/14/12 accepted |
| CR_SERVICE_INCREMENT | Optional | Must be provided if the external transmission rights represent a pending request that is to be considered part of a Coordinated Group of requests. If the Coordinated Request is for NITS, the service increment shall be set based on the duration of the requested DNR or Secondary Network Transmission Service. 3/14/12 accepted |
| CR_TS_CLASS | Optional | Must be provided if the external transmission rights represent a pending request that is to be considered part of a Coordinated Group of requests. |
| CR_TS_TYPE | Required | Indicates the type of Transmission Service reserved or requested; one of: POINT_TO_POINT NETWORK |
| CR_DISPOSITION | Required | Indicates that the Transmission Service represents existing confirmed service (CONFIRMED), or the current disposition of the Coordinated Request for service; one of: PENDING FULL PARTIAL NONE WITHDRAWN CONFIRMED DELETED |
| CR_DISPOSITION_TIME | Optional | Represents the time that the final disposition of a Coordinated Request was reached; must be null for all existing confirmed services identified in this template. |

AuxiliaryTransmission Data Element List – Request/Query Response:

3/14/12 accepted

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| CR_PRIMARY_PROVIDER_CODE | Required | Identifies by registered entity code the external Transmission Provider whose transmission system is being used in delivery of the DNR or whose transmission system is part of the Coordinated Group of requests or reservations to be coordinated. |
| CR_ASSIGNMENT_REF | Required | OASIS unique identifier assigned to the Transmission Service reservation held on the external Transmission Provider’s system or that is associated with a reservation or request for service that is part of the Coordinated Group. If existing NITS service is being used, specifies the APPLICATION_REF associated with that NITS service. 3/14/12 accepted |
| CR_SERVICE_INCREMENT | May be null | Specifies the increment of Transmission Service requested in the Coordinated Request. If the Coordinated Request is for NITS, the service increment shall be set based on the duration of the requested DNR or Secondary Network Transmission Service. 3/14/12 accepted |
| CR_TS_CLASS | May be null | Specifies the class of Transmission Service associated with a Coordinated Request. |
| CR_TS_TYPE | Required | Indicates the type of Transmission Service reserved or requested; one of: POINT_TO_POINT NETWORK |
| CR_DISPOSITION | Required | Indicates that the Transmission Service represents existing confirmed service (CONFIRMED), or the current disposition of the Coordinated Request for service; one of: PENDING FULL PARTIAL NONE WITHDRAWN CONFIRMED DELETED |
| CR_DISPOSITION_TIME | May be null | Represents the time that the final disposition of a Coordinated Request was reached; must be null for all existing confirmed services identified in this template. |
| TIME_OF_LAST_UPDATE | Required | Date/time the information associated with the template was last updated. |
| RECORD_STATUS | Required | Value indicating success/failure of OASIS to process the submitted request; 200 = Success. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in processing the submitted request. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. 3/14/12 accepted |

002-101.3.3.14	Secondary Network Transmission Service (NITSSecondaryService NITS OASIS DATA Template)

3/14/12 accepted

The NITSSecondaryService NITS OASIS DATA Template may be supplied to specify the nature of requests for network transmission service for non-firm delivery of energy from non-designated resources, i.e., Secondary Network Transmission Service.

The nature of the Secondary Network Transmission Service requested is specified in a single data record with the NITSSecondaryService NITS OASIS DATA Template. The transmission capacity to be supplied over time is conveyed in one or more NITSSecondaryCapacity NITS OASIS DATA Template data records. The EFFECTIVE_START_TIME and EFFECTIVE_STOP_TIME specified in the NITSSecondaryService NITS OASIS DATA Template must span the full period of time of the service being requested. The effective start/stop times will be used as the basis for determining service duration for calculation of Eligible Customer or Transmission Customer confirmation time limits.

3/14/12 accepted

The POINT_OF_DELIVERY and SINK information in this template must match a CONFIRMED designated Network Load as specified in the NITSLoadDescription NITS OASIS DATA Template under the NITS Application.

Updates to the data specified within this NITS OASIS DATA Template may only be submitted while the associated AddNITSSecondary NITS OASIS REQUEST Template request has the STATUS of PRESUBMITTED. The use of the STATUS of DEFICIENT is not supported for these requests.

NITSSecondaryService Data Element List – Request Upload:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| PATH_NAME | Required | Transmission Provider path for the Transmission Service to be provided. |
| POINT_OF_RECEIPT | Required | POR on Transmission Provider’s system for the Transmission Service to be provided. |
| POINT_OF_DELIVERY | Required | POD on Transmission Provider’s system for the Transmission Service to be provided. |
| SOURCE | Optional | Optionally identifies the location of the generation source. |
| SINK | Optional | Optionally identifies the location of the ultimate load. |
| POSTING_REF | Optional | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| SALE_REF | Optional | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| REQUEST_REF | Optional | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| DEAL_REF | Optional | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| RELATED_REF | Required | Must be the ASSIGNMENT_REF of the AddNITSSecondary request that is being released. 3/14/12 not accepted |
| CG_STATUS | May be null | Optional flag identifying if the DNR DESIGNATION request is to be considered and evaluated as a Coordinated Request. One of: PROPOSED ATTESTED <null> 3/14/12 accepted |
| EFFECTIVE_START_TIME | Required | Identifies the overall beginning of the time interval for the service being requested. |
| EFFFECTIVE_STOP_TIME | Required | Identifies the overall end of the time interval for the service being requested. |

NITSSecondaryService Data Element List – Request/Query Response:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| PATH_NAME | Required | Registered entity code associated with the Seller of the ancillary service. |
| POINT_OF_RECEIPT | Required | Registered DUNS associated with the Seller of the ancillary service. |
| POINT_OF_DELIVERY | Required | Identifies the BAA where the ancillary service is to be delivered/provided. |
| SOURCE | May be null | Optionally identifies the location of the generation source. |
| SINK | May be null | Optionally identifies the location of the ultimate load. |
| POSTING_REF | May be null | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| SALE_REF | May be null | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| REQUEST_REF | May be null | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| DEAL_REF | May be null | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| RELATED_REF | Required* | For the TerminateNITSSecondary request, must be the ASSIGNMENT_REF of the AddNITSSecondary request that is being released; *must be null in the AddNITSSecondary request. 3/14/12 not accepted |
| CG_STATUS | May be null | Optional identifier set to indicate the transmission request is a Coordinated Request. |
| EFFECTIVE_START_TIME | Required | Identifies the overall beginning of the time interval for the service being requested. |
| EFFFECTIVE_STOP_TIME | Required | Identifies the overall end of the time interval for the service being requested. |
| NERC_CURTAILMENT_PRIORITY | May be null | NERC TLR Curtailment Priority for service requested. |
| OTHER_CURTAILMENT_PRIORITY | May be null | Alternate curtailment priority specific to Transmission Provider defined procedures. |
| CG_DEADLINE | May be null | For Coordinated Requests only, deadline for providing all required Coordinated Group information via the AuxilaryTransmission NITS OASIS DATA Template. |
| CONCOMITANT_EVAL_FLAG | Required | Returned by OASIS as Y/N identifying if the request was submitted for Concomitant Evaluation by the Transmission Provider using the NewConcomitantRequest NITS OASIS INPUT Template. |
| TIME_OF_LAST_UPDATE | Must be null | Returned in response. |
| RECORD_STATUS | Required | Value indicating success/failure of OASIS to process the submitted request; 200 = Success. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in processing the submitted request. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. 3/14/12 accepted |

002-101.3.3.15	Secondary Network Transmission Capacity (NITSSecondaryCapacity NITS OASIS DATA Template)

3/14/12 accepted

The NITSSecondaryCapacity NITS OASIS DATA Template defines the capacity associated with a request for Secondary Network Transmission Service under the NITS Application.

The START_TIME/STOP_TIME intervals specified with this template must be within and span continuously over the EFFECTIVE_START_TIME and EFFECTIVE_END_TIME of the associated Secondary Network Transmission Service being requested as specified in the NITSSecondaryService NITS OASIS DATA Template. Intervals where no capacity is to be supplied must be indicated with a zero value for CAPACITY_REQUESTED. However, there may be no periods of a leading or trailing 0 MW capacity requested in the profile for Secondary Network Transmission Service with the initial submission. Multiple data records may be submitted with this template to represent a profile of capacity for the service being arranged over time.

3/14/12 accepted

When supplied in association with the AddNITSSecondary NITS OASIS REQUEST Template request, CAPACITY_REQUESTED must be stated as a positive MW integer value or 0 for the service requested. When the add secondary NITS request is confirmed, the additional capacity granted will be available to be scheduled.

When supplied in association with the TerminateNITSSecondary NITS OASIS REQUEST Template request, CAPACITY_REQUESTED must be stated as a negative MW integer value or 0. When the release secondary NITS request is confirmed, the capacity granted will be removed from the secondary service that may be scheduled on the designated Path/POR/POD. If the amount to be released exceeds the actual secondary NITS reserved, all capacity to deliver the non-designated resource(s) on that Path/POR/POD will be released.

3/14/12 space accepted not accepted release

Updates to the data specified within this NITS OASIS DATA Template may only be submitted while the associated AddNITSSecondary NITS OASIS REQUEST Template request has the STATUS of PRESUBMITTED or has been acted on by the Transmission Provider to set STATUS to ACCEPTED, CR_ACCEPTED, COUNTEROFFER, or CR_COUNTEROFFER. The use of the STATUS of DEFICIENT is not supported for these requests. On update, the full profiled capacity must be restated and will replace the existing profiled capacity in its entirety.

3/14/12 accepted

If the AddNITSSecondary NITS OASIS REQUEST Template request has been set to ACCEPTED, CR_ACCEPTED, COUNTEROFFER, or CR_COUNTEROFFER, the Transmission Customer may submit an update to the associated CAPACITY_REQUESTED and either REBID the request at a lower capacity, or confirm the requested service in the amount of capacity that may be accommodated by the Transmission Provider. OASIS will block any attempt to set the AddNITSSecondary NITS OASIS REQUEST Template request to CONFIRMED if the CAPACITY_REQUESTED and CAPACITY_GRANTED Data Elements are not equal over all points in time. There is no price information or negotiation of price related to the arrangement of and network transmission service.

3/14/12 accepted

Note that a pre-confirmed Coordinated Request set to the STATUS of CR_ACCEPTED may only REBID capacity to a lower value if one or more of the other requests in the Coordinated Group has been set to a CR_DISPOSITION value of PARTIAL or NONE.

NITSSecondaryCapacity Data Element List – Request Upload:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| CAPACITY_REQUESTED | Required | The volume of Secondary Network Transmission Service being requested over the start/stop interval. |
| START_TIME | Required | Identifies the start of the interval of service requested at a given capacity. |
| STOP_TIME | Required | Identifies the end of the interval of service requested at a given capacity. |

NITSSecondaryCapacity Data Element List – Request/Query Response:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| CAPACITY_REQUESTED | Required | The volume of Secondary Network Transmission Service being requested over the start/stop interval. |
| START_TIME | Required | Identifies the start of the interval of service requested at a given capacity. |
| STOP_TIME | Required | Identifies the end of the interval of service requested at a given capacity. |
| CAPACITY_GRANTED | May be null | Information on the amount of capacity that may be accommodated when the Secondary Network Transmission Service request is acted on by the Transmission Provider. |
| TIME_OF_LAST_UPDATE | Required | Date/time the information associated with the template was last updated. |
| RECORD_STATUS | Required | Value indicating success/failure of OASIS to process the submitted request; 200 = Success. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in processing the submitted request. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. 3/14/12 accepted |

002-101.3.3.16	Network Integration Transmission Service Ancillary Service (NITSAncillaryService NITS OASIS DATA Template) [Optional]

3/14/12 accepted bold not accepted nits

The NITSAncillaryService NITS OASIS DATA Template may be supplied to specify the nature of the ancillary service requested under the NITS Application. The information supplied in this NITS OASIS DATA Template must simultaneously be made available for query and update using either the NITS OASIS Templates defined here or the applicable general OASIS and PTP OASIS Templates as specified in Business Practice Standard WEQ-002-4.

3/14/12 accepted

The provider of the ancillary service may not necessarily be the Transmission Provider providing NITS, and must be designated in the SELLER_CODE and SELLER_DUNS Data Elements. Additional Data Elements are included in this template for compatibility with the ancstatus OASIS Template and are supplied either by the OASIS node software, the Transmission Provider, or the Seller of the ancillary service. The REASSIGNMENT_* Data Elements are not supported by the NITS OASIS Templates. Any arrangement of ancillary services on a secondary market between Eligible Customer or Transmission Customers must be conducted using the mechanisms provided for in Business Practice Standard WEQ-002-4. Third-party Sellers of ancillary services must also use the mechanisms provided for in Business Practice Standard WEQ-002-4 to view and update ancillary service requests submitted to them.

3/14/12 accepted

Updates to the data specified within this NITS OASIS DATA Template may only be submitted while the associated AddNITSAncillary NITS OASIS REQUEST Template request has the STATUS of PRESUBMITTED. The use of the STATUS of DEFICIENT is not supported for these requests.

The amount of ancillary service capacity being requested over time must be specified using the NITSAncillaryCapacity NITS OASIS DATA Template. The EFFECTIVE_START_TIME and EFFECTIVE_STOP_TIME specified in the NITSAncillaryService NITS OASIS DATA Template must span the full period of time of the service being requested.

3/14/12 accepted

NITSAncillaryService Data Element List – Request Upload:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| SELLER_CODE | Required | Registered entity code associated with the Seller of the ancillary service. 3/14/12 accepted |
| SELLER_DUNS | Required | Registered DUNS associated with the Seller of the ancillary service. |
| CONTROL_AREA | Required | Identifies the BAA where the ancillary service is to be delivered/provided. |
| ANC_SERVICE_POINT | Optional | Identifies the specific service point within the CONTROL_AREA where the ancillary service is to be delivered/provided. 3/14/12 accepted |
| SERVICE_INCREMENT | Required | Identifies the increment of service being requested. |
| AS_TYPE | Required | Identifies the specific type of ancillary service being requested. |
| POSTING_REF | Optional | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| SALE_REF | Optional | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| REQUEST_REF | Optional | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| DEAL_REF | Optional | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| EFFECTIVE_START_TIME | Required | Identifies the overall beginning of the time interval for the service being requested. |
| EFFFECTIVE_STOP_TIME | Required | Identifies the overall end of the time interval for the service being requested. |

NITSAncillaryService Data Element List – Request/Query Response:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| SELLER_CODE | Required | Registered entity code associated with the Seller of the ancillary service. 3/14/12 accepted |
| SELLER_DUNS | Required | Registered DUNS associated with the Seller of the ancillary service. |
| CONTROL_AREA | Required | Identifies the BAA where the ancillary service is to be delivered/provided. |
| ANC_SERVICE_POINT | May be null | Identifies the a specific service point within the CONTROL_AREA where the ancillary service is to be delivered/provided. |
| SERVICE_INCREMENT | Required | Identifies the increment of service being requested. |
| AS_TYPE | Required | Identifies the specific type of ancillary service being requested. |
| POSTING_REF | May be null | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| SALE_REF | May be null | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| REQUEST_REF | May be null | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| DEAL_REF | May be null | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| EFFECTIVE_START_TIME | Required | Identifies the overall beginning of the time interval for the service being requested. |
| EFFFECTIVE_STOP_TIME | Required | Identifies the overall end of the time interval for the service being requested. |
| SELLER_NAME | May be null | Contact name for the Seller. |
| SELLER_PHONE | May be null | Contact phone number for the Seller. |
| SELLER_FAX | May be null | Facsimile number for the Seller. |
| SELLER_EMAIL | May be null | Email address for the Seller. |
| CEILING_PRICE | Required | Identifies the Transmission Provider’s tariff price for the service. |
| PRICE_UNITS | Required | Identifies the units of price for the service. |
| NEGOTIATED_PRICE_FLAG | May be null | Set to “L” if the Seller accepted Price is lower than the posted offer price. Set to “H” if Seller accepted Price is higher than the posted offer price. Otherwise set to null. 3/14/12 accepted |
| TIME_OF_LAST_UPDATE | Required | Date/time the information associated with the template was last updated. |
| RECORD_STATUS | Required | Value indicating success/failure of OASIS to process the submitted request; 200 = Success. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in processing the submitted request. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. 3/14/12 accepted |

002-101.3.3.17	Network Integration Transmission Service Ancillary Service Capacity (NITSAncillaryCapacity NITS OASIS DATA Template) [Optional]

3/14/12 accepted bold not accepted nits

The NITSAncillaryCapacity NITS OASIS DATA Template defines the capacity and price associated with a request to procure an ancillary service under the NITS Application. The information supplied in this NITS OASIS DATA Template must simultaneously be made available for query and update using either the NITS OASIS Templates defined here or the applicable general OASIS and PTP OASIS Templates as specified in Business Practice Standard WEQ-002-4.

3/14/12 accepted

The START_TIME/STOP_TIME intervals specified with this template must be within and span continuously over the EFFECTIVE_START_TIME and EFFECTIVE_END_TIME of the associated network resource designation or termination being requested. Intervals where no capacity is to be supplied must be indicated with a zero value for CAPACITY. Multiple data records may be submitted with this template to represent a profile of capacity and/or price for the service being arranged.

3/14/12 accepted

Updates to the data specified within this NITS OASIS DATA Template may only be submitted while the associated AddNITSAncillary NITS OASIS REQUEST Template request has the STATUS of PRESUBMITTED or COUNTEROFFER. The use of the STATUS of DEFICIENT is not supported for these requests. On update, the full profiled capacity and price must be restated and will replace the existing profiled capacity in its entirety.

3/14/12 accepted

If the AddNITSAncillary NITS OASIS REQUEST Template request has been set to COUNTEROFFER, the Eligible Customer or Transmission Customer may submit an update to the associated BID_PRICE and either REBID the request, or confirm the requested service. OASIS will block any attempt to set the AddNITSAncillary NITS OASIS REQUEST Template request to CONFIRMED if the BID_PRICE and OFFER_PRICE are not equal over all points in time. Negotiation of capacity is not supported for ancillary services.

3/14/12 accepted

NITSAncillaryCapacity Data Element List – Request Upload:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| CAPACITY | Required | The volume of service being requested over the start/stop interval. |
| BID_PRICE | Required | The Eligible Customer or Transmission Customers price bid for purchase of the ancillary service. |
| START_TIME | Required | Identifies the start of the interval of service requested at a given capacity and price. |
| STOP_TIME | Required | Identifies the end of the interval of service requested at a given capacity and price. |

NITSAncillaryCapacity Data Element List – Request/Query Response:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| CAPACITY | Required | The volume of service being requested over the start/stop interval. |
| BID_PRICE | Required | The Eligible Customer or Transmission Customers price bid for purchase of the ancillary service. |
| START_TIME | Required | Identifies the start of the interval of service requested at a given capacity and price. |
| STOP_TIME | Required | Identifies the end of the interval of service requested at a given capacity and price. |
| OFFER_PRICE | May be null | The Seller’s offered price for service. |
| TIME_OF_LAST_UPDATE | Required | Date/time the information associated with the template was last updated. |
| RECORD_STATUS | Required | Value indicating success/failure of OASIS to process the submitted request; 200 = Success. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in processing the submitted request. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. 3/14/12 accepted |

002-101.3.3.18	Point-To-Point Transmission Service (PTPService NITS OASIS DATA Template)

3/14/12 accepted bold not accepted ptp

The PTPService NITS OASIS DATA Template may be supplied to specify the nature of the PTP Transmission Service requested for Concomitant Evaluation under the NITS Application. The information supplied in this NITS OASIS DATA Template must simultaneously be made available for query and update using either the NITS OASIS Templates defined here or the applicable general OASIS and PTP OASIS Templates as specified in Business Practice Standard WEQ-002-4.

3/14/12 accepted

Arrangements for PTP Transmission Service in a concomitant request are limited to services provided directly by the Transmission Provider. All SELLER_* related Data Elements shall be derived from basic information registered by the Transmission Provider on their OASIS node. The REQUEST_TYPE for all Concomitant requests for PTP Transmission Service is restricted to ORIGINAL or REDIRECT on a Firm basis. If the REQUEST_TYPE is specified as REDIRECT, the RELATED_REF must be specified with the ASSIGNMENT_REF of the Firm PTP reservation to be redirected.

3/14/12 accepted

The REASSIGNMENT_* Data Elements are not supported by the NITS OASIS Templates. Any arrangement of Transmission Services on the secondary market between Eligible Customer or Transmission Customers must be conducted using the mechanisms provided for in Business Practice Standard WEQ-002-4. The IMPACTED and COMPETING_REQUEST_FLAG Data Elements are only applicable to CONFIRMED reservations and are not supported in the NITS OASIS REQUEST Templates.

3/14/12 accepted

Updates to the data specified within this NITS OASIS DATA Template may only be submitted while the associated AddPTPService NITS OASIS REQUEST Template request has the STATUS of PRESUBMITTED. While the request remains PRESUBMITTED, all Data Elements marked as “Returned in response” will be null with the exception of TIME_OF_LAST_UPDATE. The use of the STATUS of DEFICIENT is not supported for the AddPTPService NITS OASIS REQUEST Template requests.

3/14/12 accepted

The amount of transmission capacity and price for service being requested over time must be specified by the Eligible Customer or Transmission Customer using the PTPCapacity NITS OASIS DATA Template. The EFFECTIVE_START_TIME and EFFECTIVE_STOP_TIME specified in this PTPService NITS OASIS DATA Template must span the full period of time service is being requested.

3/14/12 accepted

PTPService Data Element List – Request Upload:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| PATH_NAME | Required* | Transmission Provider path for the Transmission Service to be provided; if null, POR and POD must be specified. |
| POINT_OF_RECEIPT | Required* | POR on Transmission Provider’s system for Transmission Service to be provided; if null, PATH_NAME must be specified. |
| POINT_OF_DELIVERY | Required* | POD on Transmission Provider’s system for Transmission Service to be provided; if null, PATH_NAME must be specified. |
| SOURCE | Optional | Optionally identifies the location of the generation source. |
| SINK | Optional | Optionally identifies the location of the ultimate load. |
| SERVICE_INCREMENT | Required | Identifies the increment of service being requested as dictated by the Transmission Provider’s service definitions. |
| TS_CLASS | Requried | Identifies the class of Transmission Service being requested as dictated by the Transmission Provider’s service definitions. |
| TS_TYPE | Required | Identifies the type of Transmission Service being requested as dictated by the Transmission Provider’s service definitions. |
| TS_PERIOD | Required | Identifies the period of Transmission Service being requested as dictated by the Transmission Provider’s service definitions. |
| TS_WINDOW | Required | Identifies the window of Transmission Service being requested as dictated by the Transmission Provider’s service definitions. |
| TS_SUBCLASS | Optional | Optional sub-class of service as dictated by the Transmission Provider’s service definitions. |
| ANC_SVC_LINK | Optional | Identifies any associated ancillary service requests or reservations that should be requested or are supplied in support of the Transmission Service requested. |
| POSTING_REF | Optional | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| SALE_REF | Optional | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| REQUEST_REF | Optional | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| DEAL_REF | Optional | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| REQUEST_TYPE | Required | Must be either ORIGINAL or REDIRECT as applicable. |
| RELATED_REF | Required | If REQUEST_TYPE is REDIRECT, must be the ASSIGNMENT_REF of the firm PTP reservation that is being redirected. 3/14/12 accepted |
| ROLLOVER_WAIVED | Optional | Optional identifier set on long-term requests to indicate the Eligible Customer or Transmission Customer is voluntarily waiving any on-going rollover rights that may apply to the requested service. 3/14/12 accepted |
| CG_STATUS | Optional | Optional identifier set to indicate the transmission request is a Coordinated Request. |
| EFFECTIVE_START_TIME | Required | Identifies the overall beginning of the time interval for the service being requested. |
| EFFFECTIVE_STOP_TIME | Required | Identifies the overall end of the time interval for the service being requested. |

PTPService Data Element List – Request/Query Response:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| PATH_NAME | Required | Transmission Provider path for the Transmission Service to be provided. |
| POINT_OF_RECEIPT | Required | POR on Transmission Provider’s system for the Transmission Service to be provided. |
| POINT_OF_DELIVERY | Required | POD on Transmission Provider’s system for the Transmission Service to be provided. |
| SOURCE | May be null | Optionally identifies the location of the generation source. |
| SINK | May be null | Optionally identifies the location of the ultimate load. |
| SERVICE_INCREMENT | Required | Identifies the increment of service being requested as dictated by the Transmission Provider’s service definitions. |
| TS_CLASS | Requried | Identifies the class of Transmission Service being requested as dictated by the Transmission Provider’s service definitions. |
| TS_TYPE | Required | Identifies the type of Transmission Service being requested as dictated by the Transmission Provider’s service definitions. |
| TS_PERIOD | Required | Identifies the period of Transmission Service being requested as dictated by the Transmission Provider’s service definitions. |
| TS_WINDOW | Required | Identifies the window of Transmission Service being requested as dictated by the Transmission Provider’s service definitions. |
| TS_SUBCLASS | May be null | Optional sub-class of service as dictated by the Transmission Provider’s service definitions. |
| ANC_SVC_LINK | May be null | Identifies any associated ancillary service requests or reservations that should be requested or are supplied in support of the Transmission Service requested. |
| POSTING_REF | May be null | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| SALE_REF | May be null | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| REQUEST_REF | May be null | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| DEAL_REF | May be null | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| REQUEST_TYPE | Required | Must be either ORIGINAL or REDIRECT as applicable. 3/14/12 accepted |
| RELATED_REF | Required | If REQUEST_TYPE is REDIRECT, must be the ASSIGNMENT_REF of the firm PTP reservation that is being redirected. 3/14/12 accepted |
| ROLLOVER_WAIVED | May be null | Optional identifier set on long-term requests to indicate the Eligible Customer or Transmission Customer is voluntarily waiving any on-going rollover rights that may apply to the requested service. 3/14/12 accepted |
| CG_STATUS | May be null | Optional identifier set to indicate the transmission request is a Coordinated Request. |
| EFFECTIVE_START_TIME | Required | Identifies the overall beginning of the time interval for the service being requested. |
| EFFFECTIVE_STOP_TIME | Required | Identifies the overall end of the time interval for the service being requested. |
| NERC_CURTAILMENT_PRIORITY | May be null | NERC TLR Curtailment Priority for service requested. |
| OTHER_CURTAILMENT_PRIORITY | May be null | Alternate curtailment priority specific to Transmission Provider defined procedures. |
| CEILING_PRICE | May be null | Maximum tariff rate for service requested. |
| PRICE_UNITS | May be null | Units for price of service. |
| ANC_SVC_REQ | May be null | Ancillary service requirements for service requested. |
| NEGOTIATED_PRICE_FLAG | May be null | Returned in response. |
| SELLER_REF | May be null | Optional Transmission Provider (Seller) assigned reference identifier. |
| SELLER_NAME | May be null | Contact name for Transmission Provider (Seller). |
| SELLER_PHONE | May be null | Contact phone number for Transmission Provider (Seller). |
| SELLER_FAX | May be null | Contact facsimile number for Transmission Provider (Seller). |
| SELLER_EMAIL | May be null | Contact email address for Transmission Provider (Seller). |
| CUSTOMER_NAME | May be null | Contact name for Eligible Customer or Transmission Customer taken from NITS Application NITSCustomer NITS OASIS DATA Template information. |
| CUSTOMER_PHONE | May be null | Contact phone number for Eligible Customer or Transmission Customer taken from NITS Application NITSCustomer NITS OASIS DATA Template information. |
| CUSTOMER_FAX | May be null | Contact facsimile number for Eligible Customer or Transmission Customer taken from NITS Application NITSCustomer NITS OASIS DATA Template information. |
| CUSTOMER_EMAIL | May be null | Contact email address for Eligible Customer or Transmission Customer taken from NITS Application NITSCustomer NITS OASIS DATA Template information. |
| PRIMARY_PROVIDER_PROVISIONS | May be null | Set if request has specific additional provisions such as rollover rights. |
| CG_DEADLINE | May be null | For Coordinated Requests only, deadline for providing all required Coordinated Group information via the AuxilaryTransmission NITS OASIS DATA Template. |
| CONCOMITANT_EVAL_FLAG | Required | Returned by OASIS as Y/N identifying if the request was submitted for Concomitant Evaluation by the Transmission Provider. This data element will always be returned as ‘Y’ for PTP Transmission Services requested using the NewConcomitantRequest NITS OASIS INPUT Template. |
| TIME_OF_LAST_UPDATE | Required | Date/time the information associated with the template was last updated. |
| RECORD_STATUS | Required | Value indicating success/failure of OASIS to process the submitted request; 200 = Success. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in processing the submitted request. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. 3/14/12 accepted |

002-101.3.3.19	Point-To-Point Transmission Service Capacity (PTPCapacity NITS OASIS DATA Template)

3/14/12 accepted bold not accepted ptp

The PTPCapacity NITS OASIS DATA Template defines the capacity and price associated with a request to procure additional PTP Transmission Service as a concomitant request under the NITS Application. The information supplied in this NITS OASIS DATA Template must simultaneously be made available for query and update using either the NITS OASIS Templates defined here or the applicable general OASIS and PTP OASIS Templates as specified in Business Practice Standard WEQ-002-4.

3/14/12 accepted

The START_TIME/STOP_TIME intervals specified with this template must be within and span continuously over the EFFECTIVE_START_TIME and EFFECTIVE_END_TIME of the associated network resource designation or termination being requested. Intervals where no capacity is to be supplied must be indicated with a zero value for CAPACITY. Multiple data records may be submitted with this template to represent a profile of capacity and/or price for the service being arranged.

3/14/12 accepted

Updates to the data specified within this NITS OASIS DATA Template may only be submitted while the associated AddPTPService NITS OASIS REQUEST Template request has the STATUS of PRESUBMITTED, or has been acted on by the Transmission Provider to set STATUS to ACCEPTED, CR_ACCEPTED, COUNTEROFFER, or CR_COUNTEROFFER. The use of the STATUS of DEFICIENT is not supported for these requests. On update, the full profiled capacity and price must be restated and will replace the existing profiled capacity in its entirety.

3/14/12 accepted

If the AddPTPService NITS OASIS REQUEST Template request has been set to ACCEPTED, CR_ACCEPTED, COUNTEROFFER, or CR_COUNTEROFFER, the Eligible Customer or Transmission Customer may submit an update to the associated CAPACITY_REQUESTED and BID_PRICE and either REBID the request, or confirm the requested service. OASIS will block any attempt to set the AddPTPService NITS OASIS REQUEST Template request to CONFIRMED if the BID_PRICE and CAPACITY_REQUESTED are not equal to the OFFER_PRICE and CAPACITY_GRANTED over all points in time.

3/14/12 accepted

PTPCapacity Data Element List – Request Upload:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| CAPACITY_REQUESTED | Required | The transmission capacity being requested by the Eligible Customer or Transmission Customer over the start/stop interval. |
| BID_PRICE | Required | The Eligible Customer or Transmission Customer’s price bid for purchase of the Transmission Service. |
| START_TIME | Required | Identifies the start of the interval of service requested at a given capacity and price. |
| STOP_TIME | Required | Identifies the end of the interval of service requested at a given capacity and price. |

PTPCapacity Data Element List – Request/Query Response:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| CAPACITY_REQUESTED | Required | The transmission capacity being requested by the Eligible Customer or Transmission Customer over the start/stop interval. |
| BID_PRICE | Required | The Eligible Customer or Transmission Customer’s price bid for purchase of the Transmission Service. |
| START_TIME | Required | Identifies the start of the interval of service requested at a given capacity and price. |
| STOP_TIME | Required | Identifies the end of the interval of service requested at a given capacity and price. |
| CAPACITY_GRANTED | May be null | The transmission capacity that may be accommodated by the Transmission Provider over the start/stop interval. |
| OFFER_PRICE | May be null | The Transmission Provider’s price offered for sale of the Transmission Service. |
| TIME_OF_LAST_UPDATE | Required | Date/time the information associated with the template was last updated. |
| RECORD_STATUS | Required | Value indicating success/failure of OASIS to process the submitted request; 200 = Success. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in processing the submitted request. Only returned in response to NITS OASIS INPUT Template upload; not included in QUERY Template response. 3/14/12 accepted |

002-101.4	QUERY/RESPONSE TEMPLATE DESCRIPTIONS

3/14/12 accepted

The OASIS shall support the following NITS OASIS QUERY Templates for the downloading of NITS related request information and key information associated with all completed NITS Applications. The information to be returned in the query response is selected by user specified query variables passed as name/value pairs. The specific set of query variables that must be supported by each NITS OASIS QUERY Template is indicated below. The handling of query variables specified in the Query NITS OASIS Template invocation is dictated by Business Practice Standards WEQ-002-101.2.6.

3/14/12 accepted

Query/Response NITS OASIS Template information returned using the NITS CSV Format (OUTPUT_FORMAT=DATA) consists of a single QUERY record identifying the query template invoked followed by one or more OBJECT records and optional DATA records as applicable for the query executed. The following table summarizes the query/response NITS OASIS Templates that must be supported by OASIS. The OASIS browser-based user interface (OUTPUT_FORMAT=HTML) must provide the same functionality as the QUERY templates specified below. The nature of the presentation of the results of each query is not specified and may vary between OASIS Nodes, but the basic information returned via the query/response NITS OASIS Template interface must be clearly accessible and viewed through the browser-based OASIS interface.

3/14/12 accepted

Query/Response Template Summary

| QUERY Template | QUERY Template Description | OBJECT Template(s) | OBJECT Template Description | DATA Template(s) | DATA Template Description |
| --- | --- | --- | --- | --- | --- |
| QueryNITSRequest | This Query/Response Template lists the summary STATUS and related information for all NITS requests submitted on OASIS that meet the user supplied Query Variable selection criteria. | NewNITSApplication ModifyNITSService ModifyNITSCustomer AddNITSAgent ModifyNITSAgent AddNITSLoad ModifyNITSLoad AddNITSGeneration ModifyNITSGeneration AddNITSResource ModifyNITSResource AddNITSDNR TerminateNITSDNR AddNITSSecondary TerminateNITSSecondary AddNITSAncillary AddPTPService | For each of the specified REQUEST Template objects listed, the QueryNITSRequest will return a data record for each request submitted containing the current Data Elements as specified in the corresponding REQUEST Template Response which consists of request STATUS and additional summary information applicable to the request. | Not Applicable; there are no DATA templates supported for the QueryNITSRequest QUERY Template. |  |
| QueryNITSRequestDetails | This Query/Response Template lists the summary STATUS and related REQUEST Template information as well as the current information contained in all associated DATA Templates submitted on OASIS that meet the user supplied Query Variable selection criteria. | NewNITSApplication | For each of the specified REQUEST Template objects listed, the QueryNITSRequestDetails will return the current Data Elements as specified in the corresponding REQUEST Template Response followed by the detailed request information associated in each DATA Template submitted with the request. | NITSService NITSCustomer NITSAgent | Lists the current values for all DATA Templates associated with a given NITS request as identified in the OBJECT Template. |
|  |  | ModifyNITSService | (See above) | NITSService | (See above) |
|  |  | ModifyNITSCustomer | (See above) | NITSCustomer | (See above) |
|  |  | AddNITSAgent ModifyNITSAgent | (See above) | NITSAgent | (See above) |
|  |  | AddNITSLoad ModifyNITSLoad | (See above) | NITSLoadDescription NITSLoadForecast | (See above) |
|  |  | AddNITSGeneration ModifyNITSGeneration | (See above) | NITSGenerationDescription NITSGenerationDispatch | (See above) |
|  |  | AddNITSResource ModifyNITSResource | (See above) | NITSResourceDescription NITSResourceForecast | (See above) |
|  |  | AddNITSDNR | (See above) | NITSResourceDescription NITSResourceForecast NITSResourceDesignation NITSResourceCapacity NITSSchedulingRights AuxiliaryTransmission | (See above) |
|  |  | TerminateNITSDNR | (See above) | NITSResourceDesignation NITSResourceCapacity | (See above) |
|  |  | AddNITSSecondary | (See above) | NITSSecondaryService NITSSecondaryCapacity AuxiliaryTransmission | (See above) |
|  |  | TerminateNITSSecondary | (See above) | NITSSecondaryService NITSSecondaryCapacity | (See above) |
|  |  | AddNITSAncillary | (See above) | NITSAncillaryService NITSAncillaryCapacity | (See above) |
|  |  | AddPTPService | (See above) | PTPService PTPCapacity AuxiliaryTransmission | (See above) |
| QueryNITSApplication | Returns the current composite information associated with all or selected components of a CONFIRMED NITS Application. | NITSApplication | Returns summary information for the NITS Application followed by all or selected DATA Templates containing detailed information on components of that NITS Application. | NITSService NITSCustomer NITSAgent NITSLoadDescription NITSLoadForecast NITSGenerationDescription NITSGenerationDispatch NITSResourceDescription NITSResourceForecast NITSAncillaryReservation | Lists the current composite values for each of the components by DATA Template associated with a given NITS Application. |
| QueryNITSDNR | This Query/Response Template lists the current set of DNRs and their current capacity values over time that meet the user supplied Query Variable selection criteria. | NITSDNRList | Returns detailed information on all current Network Resources designated to serve Network Load under one or more CONFIRMED NITS Applications. | Not Applicable; there are no DATA templates supported for the QueryNITSRequest QUERY Template. |  |
| QueryNITSSchedulingRights | For those Transmission Provider’s that elect to support the NITSSchedulingRights template either for Eligible Customer or Transmission Customer input or Transmission Provider output, this template will return the current applicable NITS scheduliing rights available for delivering Designated or non-designated (Secondary) Network Resources to Network Load. 3/14/12 accepted | NITSSchedulingRightsList | Returns specific information on the set of network transmission scheduling rights held by current Network Resources, designated or non-designated, to serve Network Load under one or more CONFIRMED NITS Applications. 3/14/12 accepted | Not Applicable; there are no DATA templates supported for the QueryNITSRequest QUERY Template. |  |

002-101.4.1	QUERY Template Definitions

3/14/12 accepted

The OASIS shall support the following NITS OASIS QUERY Templates for the downloading/viewing of NITS related information. NITS OASIS QUERY Template information is conveyed via the first record in a NITS CSV Format response returned by OASIS, the QUERY Header Record.

3/14/12 accepted

The name of the NITS OASIS QUERY Template invoked shall be identified and returned in the QUERY=<QueryTemplateName> field of the NITS CSV Format Query Header Record, e.g., QUERY=QueryDNRList. The following Data Elements shall be supported by all defined NITS OASIS QUERY Templates, and included in the QUERY Header Record’s Header Data Element List.

3/14/12 accepted

Any errors encountered in validating the user supplied Query Variables associated with a given NITS OASIS QUERY Template or errors in actually performing the requested query shall be reported in the QUERY Template response QUERY_STATUS and ERROR_MESSAGE Data Elements.

Query Template Data Element List – Response:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| VERSION | Required | OASIS S&CP Protocol Version for all NITS OASIS Template information returned in the response. |
| PRIMARY_PROVIDER_CODE | Required | Registered entity code of the Transmission Provider generating the NITS OASIS Template response. |
| PRIMARY_PROVIDER_DUNS | Required | Registered entity DUNS of the Transmission Provider generating the NITS OASIS Template response. |
| RETURN_TZ | Required | Time zone used for return of all date/time related template Data Elements in NITS OASIS Template response. |
| TIME_STAMP | Required | Time stamp associated with the OASIS date/time the NITS OASIS Template response was generated. |
| QUERY_STATUS | Required | Value indicating success/failure of OASIS to process the submitted QUERY Template; 200 = Success. |
| ERROR_MESSAGE | May be null | Optional error message text describing error encountered in processing the submitted QUERY Template. |

Example Response:

QUERY=QueryNITSRequest,VERSION,PRIMARY_PROVIDER_CODE,PRIMARY_PROVIDER_DUNS,RETURN_TZ,TIME_STAMP↵

,2.0,ATP,123456789,ED, 20110531085302ED,↵

002-101.4.1.1	Query Network Integration Transmission Service Request Summary (QueryNITSRequest NITS OASIS QUERY Template)

3/14/12 accepted bold not accepted nits

The QueryNITSRequest NITS OASIS QUERY Template is used to query the summary status information related to all NITS Requests submitted that meet the selection criteria specified via the user supplied Query Variables. The response to the QueryNITSRequest NITS OASIS QUERY Template will consist of one or more NITS Request Object NITS OASIS Templates corresponding to each of the unique NITS OASIS REQUEST Templates that have been submitted on OASIS.

3/14/12 accepted

There are no NITS OASIS DATA Template records returned in response to the QueryNITSRequest NITS OASIS QUERY Template. The QueryNITSRequest NITS OASIS Query Template response returns only the current NITS OASIS REQUEST Template header information with request STATUS, request reference identifiers, etc., for each NITS OASIS REQUEST Template selected based on the user specified Query Variables. Since there are no NITS OASIS DATA Template records in the response, the Request Header information for all like NITS requests will be returned as multiple Object Header data rows, one for each submitted NITS OASIS REQUEST Template. That is, if there are twenty three AddNITSDNR NITS OASIS REQUEST Template requests that meet the Query Variable selection criteria, a single NITS Request Object NITS OASIS Template record of type OBJECT=AddNITSDNR followed by the Request Header Data Element name list would be returned in the response with twenty three data records immediately following the Object NITS OASIS Template Header containing the values corresponding to those Data Element names for each of those selected AddNITSDNR NITS OASIS REQUEST Template requests.

3/14/12 accepted

Information associated with any NITS OASIS REQUEST Templates that are PRESUBMITTED will only be returned to the user associated with the Eligible Customer or Transmission Customer identified in those requests.

Summary request information associated with each of the following supported NITS OASIS REQUEST Templates may be returned in the QueryNITSRequest NITS OASIS QUERY Template response:

NewNITSApplication

ModifyNITSService

ModifyNITSCustomer

AddNITSAgent

ModifyNITSAgent

AddNITSLoad

ModifyNITSLoad

AddNITSGeneration

ModifyNITSGeneration

AddNITSResource

ModifyNITSResource

AddNITSDNR

TerminateNITSDNR

AddNITSSecondary

TerminateNITSSecondary

AddNITSAncillary

AddPTPService

The following Query Variables may be specified to select which NITS OASIS REQUEST Templates are returned in the query response. The REQUEST Query Variable may be used to select only specific types of NITS Requests by one or more of the above NITS OASIS REQUEST Template names, e.g., the Query Variable name/value pair of REQUEST=AddNITSDNR returns summary information for only those AddNITSDNR requests that meet the criteria specified by any additional Query Variables.

3/14/12 accepted

QueryNITSRequest Query Variables:

REQUEST*

STATUS*

SELLER_CODE*

SELLER_DUNS*

CUSTOMER_CODE*

CUSTOMER_DUNS*

TRANSACTION_REF

APPLICATION_REF

ASSIGNMENT_REF

SALE_REF

REQUEST_REF

DEAL_REF

AFFILIATE_FLAG

CG_STATUS

CONCOMITANT_EVAL_FLAG

EFFECTIVE_START_TIME

EFFECTIVE_END_TIME

START_TIME_QUEUED

STOP_TIME_QUEUED

TIME_OF_LAST_UPDATE

002-101.4.1.2	Query Network Integration Transmission Service Request Details (QueryNITSRequestDetails NITS OASIS QUERY Template)

3/14/12 accepted bold not accepted nits

The QueryNITSRequestDetails NITS OASIS QUERY Template is used to query the details of each submitted NITS OASIS REQUEST Template and all associated NITS OASIS DATA Template information that meet the selection criteria specified via the user supplied Query Variables. The response to the QueryNITSRequestDetails NITS OASIS QUERY Template will consist of one or more Request Object Templates corresponding to each of the unique NITS OASIS REQUEST Templates that have been submitted on OASIS.

3/14/12 accepted

For each NITS OASIS REQUEST Template selected, OASIS will return a single Request Object Header record identifying the NITS OASIS REQUEST Template information being returned followed by a single data record containing the current values associated with each of the NITS Request Object Header Data Elements. The current contents of all NITS OASIS DATA Templates that have been associated with the NITS OASIS REQUEST Template will then be returned in the same DATA Template Response format as identified in Business Practice Standard WEQ-002-101.3.3, i.e., a DATA Template Header Record followed by one or more Data Records.

3/14/12 accepted

Information associated with each of the following supported NITS OASIS REQUEST Templates may be returned in the QueryNITSRequestDetails NITS OASIS QUERY Template response:

NewNITSApplication

ModifyNITSService

ModifyNITSCustomer

AddNITSAgent

ModifyNITSAgent

AddNITSLoad

ModifyNITSLoad

AddNITSGeneration

ModifyNITSGeneration

AddNITSResource

ModifyNITSResource

AddNITSDNR

TerminateNITSDNR

AddNITSSecondary

TerminateNITSSecondary

AddNITSAncillary

AddPTPService

The following Query Variables may be specified to select which NITS OASIS REQUEST Template are returned in the query response. The REQUEST Query Variable may be used to select only specific types of NITS Request Objects by above NITS OASIS REQUEST Template name, e.g., REQUEST=AddNITSDNR returns summary information for all selected AddNITSDNR NITS OASIS REQUEST Template requests.

3/14/12 accepted

QueryNITSRequestDetails Query Variables:

REQUEST*

STATUS*

SELLER_CODE*

SELLER_DUNS*

CUSTOMER_CODE*

CUSTOMER_DUNS*

TRANSACTION_REF

APPLICATION_REF

ASSIGNMENT_REF

SALE_REF

REQUEST_REF

DEAL_REF

AFFILIATE_FLAG

CG_STATUS

CONCOMITANT_EVAL_FLAG

EFFECTIVE_START_TIME

EFFECTIVE_END_TIME

START_TIME_QUEUED

STOP_TIME_QUEUED

TIME_OF_LAST_UPDATE

002-101.4.1.3	Query Network Integration Transmission Service Application Details (QueryNITSApplication NITS OASIS QUERY Template)

3/14/12 accepted bold not accepted nits

The QueryNITSApplication NITS OASIS QUERY Template returns composite information on all or selected components of a CONFIRMED NITS Application. This composite information represents the current data maintained for the NITS Application as a whole for the following NITS component NITS OASIS DATA Templates based on the set of all NITS OASIS REQUEST Templates that have been CONFIRMED as part of the Initial NITS Application and all subsequent CONFIRMED Modifications to Service applicable:

3/14/12 accepted

NITSService

NITSCustomer

NITSAgent

NITSLoadDescription

NITSLoadForecast*

NITSGenerationDescription

NITSGenerationDispatch*

NITSResourceDescription

NITSResourceForecast*

NITSAncillaryService

* Information associated with these NITS OASIS DATA Templates will only be returned in response to a Query executed by a user associated with the Eligible Customer or Transmission Customer under the NITS Application.

The response to the QueryNITSApplication NITS OASIS QUERY Template will consist of the NITSApplication NITS OASIS OBJECT Template corresponding to each CONFIRMED NITS Application that meets the selection criteria based on the user supplied Query Variables, followed by the specific NITS OASIS DATA Templates requested by name (or all) in association with that selected NITS Application.

Composite information on the current set of DNRs and the scheduling rights associated with delivery of those DNRs to Network Load or delivery of non-designated network resources to serve Network Load on a secondary (non-firm) basis is queried using the QueryNITSDNR and QueryNITSSchedulingRights NITS OASIS QUERY Templates.

3/14/12 accepted

The following Query Variables may be specified to select which NITS Applications and associated composite information views are returned in the query response. The DATA Query Variable may be used to select only specific types of NITS composite information to be returned in the response by the above NITS OASIS DATA Template names, e.g., DATA=NITSLoad returns all of the current information associated with each of the valid Network Loads based on the set of all AddNITSLoad and ModifyNITSLoad NITS OASIS REQUEST Templates requests that have been approved by the Transmission Provider and confirmed by the Eligible Customer or Transmission Customer.  If the DATA query variable is not specified, information associated with all of the above named NITS OASIS DATA Templates in association with each selected NITS Application will be returned.

3/14/12 accepted

If specified, the EFFECTIVE_START_TIME and EFFECTIVE_END_TIME Query Variables limit the data records returned in each NITS OASIS DATA Template to those records that overlap (EFFECTIVE_START_TIME/EFFECTIVE_STOP_TIME or START_TIME/STOP_TIME) any portion of time within that specified time range.

3/14/12 accepted

QueryNITSApplication Query Variables:

DATA*

SELLER_CODE*

SELLER_DUNS*

CUSTOMER_CODE*

CUSTOMER_DUNS*

APPLICATION_REF

AFFILIATE_FLAG

EFFECTIVE_START_TIME

EFFECTIVE_END_TIME

TIME_OF_LAST_UPDATE

002-101.4.1.4	Query Network Integration Transmission Service Designated Network Resources (QueryNITSDNR NITS OASIS QUERY Template)

3/14/12 accepted bold not accepted nits

The QueryNITSDNR NITS OASIS QUERY Template returns summary information on the current set of DNR and their corresponding designated capacity over time. This information is returned in a single NITSDNRList NITS OASIS OBJECT Template with no associated NITS OASIS DATA Templates. One record will be returned in the NITSDNRList NITS OASIS OBJECT Template for each DNR at given capacity for given interval in time that meets the selection criteria as specified in the user supplied Query Variables.  The capacity associated with a given DNR at each point in time shall reflect the composite value of applying all CONFIRMED AddNITSDNR and TerminateNITSDNR NITS OASIS REQUEST Templates requests to determine the current capacity available for the DNR.

3/14/12 accepted

QueryNITSDNR Query Variables:

SELLER_CODE*

SELLER_DUNS*

CUSTOMER_CODE*

CUSTOMER_DUNS*

RESOURCE_NAME*

RESOURCE_TYPE*

RESOURCE_CLASS*

POINT_OF_RECEIPT*

SOURCE*

GEN_NAME*

APPLICATION_REF

AFFILIATE_FLAG

START_TIME

END_TIME

002-101.4.1.5	Query Network Integration Transmission Service Scheduling Rights (QueryNITSSchedulingRights NITS OASIS QUERY Template) [Optional]

3/14/12 accepted bold not accepted nits

The QueryNITSSchedulingRights NITS OASIS QUERY Template returns summary information on the current set of NITS scheduling rights available for use in association with deliveries of both Designated and non-designated (Secondary) Network Resources to Network Loads under a CONFIRMED NITS Application. The use of the NITS scheduling rights related templates is at the discretion of the individual Transmission Provider’s Business Practices.

3/14/12 accepted

The summary information returned in response to this QUERY Template is a single NITSSchedulingRightsList NITS OASIS OBJECT Template with no associated NITS OASIS DATA Templates. One record will be returned in the NITSSchedulingRightsList NITS OASIS OBJECT Template for each Network Resource that may be scheduled to each Network Load at a given capacity for a given interval in time that meets the selection criteria as specified in the user supplied Query Variables.  The transmission scheduling rights associated with a given Designated or non-designated Network Resource for delivery to a given Network Load at each point in time shall reflect the composite value of applying all CONFIRMED AddNITSDNR, TerminateNITSDNR, AddNITSSecondary, and TerminateNITSSecondary NITS OASIS REQUEST Templates requests to determine the current capacity that may be scheduled from resource to load.

3/14/12 accepted

QueryNITSSchedulingRights Query Variables:

SELLER_CODE*

SELLER_DUNS*

CUSTOMER_CODE*

CUSTOMER_DUNS*

PATH*

RESOURCE_NAME*

RESOURCE_TYPE*

RESOURCE_CLASS*

POINT_OF_RECEIPT*

SOURCE*

GEN_NAME*

LOAD_NAME*

POINT_OF_DELIVERY*

SINK*

APPLICATION_REF

AFFILIATE_FLAG

START_TIME

END_TIME

002-101.4.2	OBJECT Template Definitions

3/14/12 accepted

The following sections describe the specific NITS OASIS OBJECT Templates and Object Header Data Elements associated with each Object that may be returned in response to the supported NITS OASIS QUERY Templates defined above.

002-101.4.2.1	Network Integration Transmission Service Request Object

3/14/12 accepted bold not accepted nits

The NITS Request OBJECT Template returns either summary status information or the full detailed information associated with any of the supported Input/Response NITS OASIS REQUEST Templates as specified in Business Practice Standard WEQ-002-101.3.2.

3/14/12 accepted

The QueryNITSRequest and QueryNITSRequestDetails NITS OASIS QUERY Template responses will include the NITS Request Object Header record identifying each of the NITS OASIS REQUEST Template submissions that match the specified Query Variable selection criteria by the corresponding NITS OASIS REQUEST Template name in the OBJECT=<ObjectTemplateName> field, e.g., OBJECT=AddNITSDNR returns the information associated with the AddNITSDNR NITS OASIS REQUEST Template submission. The following Object Header Data Elements will be returned in both the QueryNITSRequest summary and QueryNITSRequestDetails query response.

3/14/12 accepted

NITS Request Object Template Data Element List – Query Response:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| TRANSACTION_REF | May be null | Returns unique OASIS assigned transaction reference identifier for concomitant requests only; null returned in all other instances. |
| APPLICATION_REF | Required | Returns the unique OASIS assigned application reference identifier required to associate request with given NITS Application. |
| ASSIGNMENT_REF | Required | Returns unique OASIS assigned request reference identifier for each request. |
| PRECONFIRMED | Required | Returns current value for pre-confirmation status of request. |
| STATUS | Required | Returns current request STATUS value. |
| STATUS_COMMENTS | May be null | Optional comments set by Eligible Customer or Transmission Customer or Transmission Provider (Seller) to describe action taken in setting STATUS; if null, current value retained. |
| CUSTOMER_COMMENTS | May be null | Returns current Eligible Customer or Transmission Customer comments associated with request. 3/14/12 accepted |
| PRIMARY_PROVIDER_COMMENTS | May be null | Returns any comments associated with request as set by the Transmission Provider. |
| SELLER_COMMENTS | May be null | Returns any comments associated with the request as set by the Transmission Provider for all but the AddNITSAncillary NITS OASIS REQUEST Template request which is set by the ancillary service provider. |
| SELLER_CODE | Required | Set to PRIMARY_PROVIDER_CODE for all but the AddNITSAncillary NITS OASIS REQUEST Template request which is set to identify the ancillary service provider. |
| SELLER_DUNS | Required | Set to PRIMARY_PROVIDER_DUNS for all but the AddNITSAncillary NITS OASIS REQUEST Template request which is set to identify the ancillary service provider. |
| CUSTOMER_CODE | Required | Set to the Eligible Customer or Transmission Customer’s registered entity code at time of initial submission. |
| CUSTOMER_DUNS | Required | Set to the Eligible Customer or Transmission Customer’s registered DUNS at time of initial submission. |
| AFFILIATE_FLAG | May be null | Identifies if the Eligible Customer or Transmission Customer submitting the request is affiliated with the Transmission Provider (Seller); returns null value while request is PRESUBMITTED. |
| TIME_QUEUED | May be null | Date/time that the request was submitted as QUEUED; returns null value while request is PRESUBMITTED. |
| RESPONSE_TIME_LIMIT | May be null | Once submitted and under evaluation by the Transmission Provider, this field may be set by the Transmission Provider with a deadline for Eligible Customer or Transmission Customer action dependent on the nature of the request and applicable Business Practice Standards. 3/14/12 accepted |
| CREATEDBY_CODE | Required | Entity code associated with the user that originally queued the request for evaluation. |
| MODIFIEDBY_CODE | Required | Entity code associated with the user that last updated the request status or any NITS OASIS DATA Templates associated with the request. |
| TIME_OF_LAST_UPDATE | Required | Date/time the request was last updated, i.e., time OASIS acted on request submission. |

The QueryNITSRequest NITS OASIS QUERY Template response shall consist of a single NITS Request Object Template Header Record followed by one or more Data Records containing the above named Data Element values corresponding to each NITS Request of a given named NITS OASIS REQUEST Template matching the query criteria. There are no associated NITS OASIS DATA Templates returned in the query response. For example, if there are five TerminateNITSDNR NITS OASIS REQUEST Template requests that meet the selection criteria specified in the User supplied Query Variables, a single Object Header Record for OBJECT=TerminateNITSDNR will be returned followed by five data records containing the above Data Element values for each of the five selected NITS Requests submitted.

3/14/12 accepted

The QueryNITSRequestDetails NITS OASIS QUERY Template response shall consist of a single NITS Request Object Template Header Record followed by a single Data Record containing the above named Data Element values corresponding to one specific NITS Request. The NITS OASIS QUERY Template response will then return all the current NITS OASIS DATA Template information that has been submitted in association with that given NITS OASIS REQUEST Template request. If multiple NITS OASIS REQUEST Template requests of the same name, e.g., AddNITSDNR NITS OASIS REQUEST Template, are to be returned in the query response, each of those individual NITS OASIS REQUEST Template requests and the associated NITS OASIS DATA Template information will be returned in subsequent NITS Request Object Header records and subordinate NITS OASIS DATA Template records. The format for the NITS OASIS DATA Template response information shall be identical to the NITS OASIS INPUT Request NITS OASIS DATA Template input/query response information as specified in Business Practice Standard WEQ-002-101.3.3, with the exception that the RECORD_STATUS and ERROR_MESSAGE Data Elements will not be included in the NITS OASIS QUERY Template response.

3/14/12 accepted

The list of valid NITS OASIS OBJECT Template request names are listed below with the corresponding NITS OASIS DATA Templates that may be returned in association with each NITS OASIS OBJECT Template request. The Header Data Elements returned in association with each NITS OASIS OBJECT Template request in response to the QueryNITSRequest or QueryNITSRequestDetails NITS OASIS QUERY Templates are specified in Business Practice Standard WEQ-002-101.3.2. The NITS OASIS DATA Template Data Elements returned in the response to the QueryNITSRequestDetails NITS OASIS QUERY Template are specified in Business Practice Standard WEQ-002-101.3.3.

3/14/12 accepted

NewNITSApplication

NITSService

NITSCustomer

NITSAgent

ModifyNITSService

NITSService

ModifyNITSCustomer

NITSCustomer

AddNITSAgent

NITSAgent

ModifyNITSAgent

NITSAgent

AddNITSLoad

NITSLoadDescription

NITSLoadForecast*

ModifyNITSLoad

NITSLoadDescription

NITSLoadForecast*

AddNITSGeneration

NITSGenerationDescription

NITSGenerationDispatch*

ModifyNITSGeneration

NITSGenerationDescription

NITSGenerationDispatch*

AddNITSResource

NITSResourceDescription

NITSResourceForecast*

ModifyNITSResource

NITSResourceDescription

NITSResourceForecast*

AddNITSDNR

NITSResourceDescription

NITSResourceForecast

NITSResourceDesignation

NITSResourceCapacity

NITSSchedulingRights

AuxiliaryTransmission

TerminateNITSDNR

NITSResourceDesignation

NITSResourceCapacity

AddNITSSecondary

NITSSecondaryService

NITSSecondaryCapacity

AuxiliaryTransmission

TerminateNITSSecondary

NITSSecondaryService

NITSSecondaryCapacity

AddNITSAncillary

NITSAncillaryService

NITSAncillaryCapacity

AddPTPService

PTPService

PTPCapacity

AuxiliaryTransmission

* Information associated with these NITS OASIS DATA Templates will only be returned in response to a query executed by a user associated with the Eligible Customer or Transmission Customer under the NITS Application.

002-101.4.2.2	Network Integration Transmission Service Application Object (NITSApplication NITS OASIS OBJECT Template)

3/14/12 accepted bold not accepted nits

The NITS Application OBJECT Template returns the current composite information associated with each CONFIRMED NITS Application that meets the user’s selection criteria. One OBJECT=NITSApplication Object Template data record will be returned for a given NITS Application with the following Data Elements:

3/14/12 accepted

NITSApplication Object Template Data Element List – Query Response:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| APPLICATION_REF | Required | Returns the unique OASIS assigned application reference identifier for the NITS Application. |
| SELLER_CODE | Required | Set to registered PRIMARY_PROVIDER_CODE for the Transmission Provider. |
| SELLER_DUNS | Required | Set to registered PRIMARY_PROVIDER_DUNS for the Transmission Provider |
| CUSTOMER_CODE | Required | Set to the Eligible Customer or Transmission Customer’s registered entity code effective at the time the QUERY Template was submitted. |
| CUSTOMER_DUNS | Required | Set to the Eligible Customer or Transmission Customer’s registered DUNS effective at the time the QUERY Template was submitted.. |
| AFFILIATE_FLAG | May be null | Identifies if the Eligible Customer or Transmission Customer effective at the time the QUERY Template was submitted is affiliated with the Transmission Provider (Seller). |

Immediately following the above NITSApplication NITS OASIS OBJECT Template header data record with the values for those Data Elements, the query response will consist of one or more NITS OASIS DATA Template responses based on the information associated with the NITS Application and any specific NITS OASIS DATA Template information selected with the DATA=<Data TemplateName> Query Variable. The following are the NITS OASIS DATA Templates that are supported in the response to the QueryNITSApplication NITS OASIS QUERY Template which are described in more detail in Business Practice Standard WEQ-002-101.4.3:

3/14/12 accepted

NITSService

NITSCustomer

NITSAgent

NITSLoadDescription

NITSLoadForecast

NITSGenerationDescription

NITSGenerationDispatch

NITSResourceDescription

NITSResourceForecast

NITSAncillaryReservation

002-101.4.2.3	List of Designated Network Resources (NITSDNRList NITS OASIS OBJECT Template)

3/14/12 accepted

The NITSDNRList NITS OASIS OBJECT Template returns information on all of the current DNR and their designated capacity meeting the selection criteria specified by user supplied Query Variables over the time frame selected, if applicable. There are no NITS OASIS DATA Templates defined in association with the NITSDNRList NITS OASIS OBJECT Template.

3/14/12 accepted

The following Object Header Data Elements will be returned for each DNR meeting the query selection criteria:

NITSDNRList Object Template Data Element List – Query Response:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| APPLICATION_REF | Required | Returns the unique OASIS assigned application reference identifier for the NITS Application. |
| SELLER_CODE | Required | Set to registered PRIMARY_PROVIDER_CODE for the Transmission Provider. |
| SELLER_DUNS | Required | Set to registered PRIMARY_PROVIDER_DUNS for the Transmission Provider |
| CUSTOMER_CODE | Required | Set to the Eligible Customer or Transmission Customer’s registered entity code responsible under the NITS Application at the time of the resource designation. |
| CUSTOMER_DUNS | Required | Set to the Eligible Customer or Transmission Customer’s registered DUNS responsible under the NITS Application at the time of the resource designation. |
| AFFILIATE_FLAG | Required | Identifies if the responsible Eligible Customer or Transmission Customer is affiliated with the Transmission Provider (Seller). |
| RESOURCE_NAME | Required | Unique name to be associated with an Eligible Customer or Transmission Customer’s Network Resource that has been designated. 3/14/12 accepted |
| RESOURCE_CLASS | Required | Identifies the nature of the DNR as either ON_SYSTEM or OFF_SYSTEM. |
| RESOURCE_TYPE | Required | Identifies the nature of the DNR as either GENERATION or EXECUTED_PPA or CONTINGENT_PPA. |
| SOURCE_AREA | Required | Identifies the BAA that is the source for energy deliveries from this defined resource, i.e., the electrical location of the resource. |
| TITLE_AREA | Required | Identifies the BAA where the Eligible Customer or Transmission Customer assumes responsibility (title) to the energy delivered by this defined resource. |
| GEN_NAME | Required* | Required if the DNR has been identified with RESOURCE_TYPE=GENERATION. Identifies the generation asset by either the asset GEN_NAME or an associated aggregate GEN_GROUP that has been designated. 3/14/12 accepted |
| GEN_LOCATION | Required* | Required if the DNR has been identified with RESOURCE_TYPE=GENERATION. Identifies the geographic location of the generation asset if the DNR is designated from a specific generation asset, or “AGGREGATE” if the DNR is associated with an aggregate GEN_GROUP. 3/14/12 accepted |
| GEN_MAX_CAPACITY | Required | Maximum total generating capacity of the asset. If a jointly owned facility, this represents the maximum share of the capacity of the generation asset. If the DNR is associated with an aggregate GEN_GROUP, this represents the sum of all generation capacity for all generation assets that are in the GEN_GROUP. 3/14/12 accepted |
| POINT_OF_RECEIPT | Required | Identifies the POR on the Transmission Provider’s system for delivering energy from the DNR to Network Load. |
| SOURCE | May be null | Identifies the source point name to be used in scheduling deliveries from the DNR. Transmission Provider may require use of a specific SOURCE in association with the DNR. 3/14/12 accepted |
| CAPACITY_DESIGNATED | Required | Total MW capacity associated with the DNR that may be used at the POINT_OF_RECEIPT over the START/STOP time interval. |
| START_TIME | Required | Identifies the beginning of the time interval for the capacity designated. |
| STOP_TIME | May be null | Identifies the end of the time interval for the capacity to designated. |

002-101.4.2.4	List of Network Integration Transmission Service Scheduling Rights (NITSSchedulingRightsList NITS OASIS OBJECT Template)

3/14/12 accepted bold not accepted nits

The NITSSchedulingRightsList NITS OASIS OBJECT Template returns information on all of transmission scheduling rights that have been granted to the Eligible Customer or Transmission Customer under a NITS Application meeting the selection criteria specified by user supplied Query Variables over the time frame selected, if applicable. There are no NITS OASIS DATA Templates defined in association with the NITSSchedulingRightList NITS OASIS OBJECT Template. This OBJECT Template is only required for Transmission Provider’s that support the NITS scheduling rights related templates and the QueryNITSSchedulingRights QUERY Template.

3/14/12 accepted

The following Object Header Data Elements will be returned for each set of scheduling rights granted by the Transmission Provider for delivery of DNR and non-designated Network Resources meeting the QueryNITSSchedulingRights NITS OASIS QUERY Template selection criteria:

The scheduling rights managed under the NITS Application may be resource specific in which case the RESOURCE_NAME will be specified with the transmission rights from SOURCE/POINT_OF_RECEIPT to POINT_OF_DELIVERY/SINK and PATH. If the scheduling rights are an aggregate of the rights at a specific Transmission Provider Interface (POINT_OF_RECEIPT) and may be used to schedule any DNR for delivery at the POR, the RESOURCE_NAME may be null. Deliveries from non-designated Network Resources are not associated with a named Network Resource under the NITS Application.

3/14/12 accepted

NITSSchedulingRightsList Object Template Data Element List – Query Response:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| APPLICATION_REF | Required | Returns the unique OASIS assigned application reference identifier for the NITS Application. |
| SELLER_CODE | Required | Set to registered PRIMARY_PROVIDER_CODE for the Transmission Provider. |
| SELLER_DUNS | Required | Set to registered PRIMARY_PROVIDER_DUNS for the Transmission Provider |
| CUSTOMER_CODE | Required | Set to the Eligible Customer or Transmission Customer’s registered entity code responsible under the NITS Application at the time of the resource designation. |
| CUSTOMER_DUNS | Required | Set to the Eligible Customer or Transmission Customer’s registered DUNS responsible under the NITS Application at the time of the resource designation. |
| AFFILIATE_FLAG | Required | Identifies if the responsible Eligible Customer or Transmission Customer is affiliated with the Transmission Provider (Seller). |
| RESOURCE_NAME | May be null | Unique name to be associated with an Eligible Customer or Transmission Customer’s DNR (from NITSResourceDescription NITS OASIS DATA Template). May be null if the transmission scheduling rights may be used by any Network Resource designated for delivery at the POR or for non-designated Network Resources delivered at the POR. 3/14/12 accepted |
| PATH_NAME | May be null | Identifies the OASIS path to be used for scheduling the DNR if required by the Transmission Provider in addition to POR/POD and Source/Sink. |
| POINT_OF_RECEIPT | Required | Identifies the POR on the Transmission Provider’s transmission system that will be scheduled for delivery of the DNR. Must match the POINT_OF_RECEIPT as specified in the NITSResourceDesignation NITS OASIS DATA Template. 3/14/12 accepted |
| POINT_OF_DELIVERY | Required | Identifies the POD on the Transmission Provider’s transmission system that will be scheduled for delivery of the DNR to the designated load(s) under the NITS Application. Must match the POINT_OF_DELIVERY as specified in the NITSLoadDescription NITS OASIS DATA Template. |
| SOURCE | May be null | Identifies the named Source that has been assigned for use in scheduling the delivery of the Network Resource |
| SINK | May be null | Identifies the named Sink on the Transmission Provider’s transmission system that will be scheduled for delivery of the DNR to the designated load(s) under the NITS Application. Must match the SINK as specified in the NITSLoadDescription NITS OASIS DATA Template. |
| CAPACITY_GRANTED | Required | Identifies the amount of Transmission Service capacity granted for scheduling delivery of the Network Resource to designated load(s) under the NITS Application. |
| NERC_CURTAILMENT_PRIORITY | Required | Identifies the curtailment priority for delivery of the Network Resource as either a firm DNR (7) or as a non-firm (secondary) non-designated resource (6). |
| OTHER_CURTAILMENT_PRIORITY | May be null | Optional Transmission Provider specific curtailment priority assigned to the rights granted, |
| START_TIME | Required | Identifies the beginning of the time interval for the corresponding scheduling rights being requested or confirmed to support the DNR. |
| STOP_TIME | Required | Identifies the end of the time interval for the corresponding scheduling rights being requested or confirmed to support the DNR. |

002-101.4.3	DATA Template Definitions

3/14/12 accepted

The OASIS shall support the following NITS OASIS DATA Templates for the querying of NITS related request information. For the most part, each NITS OASIS DATA Template returned in a response to a NITS OASIS QUERY Template will contain the same information as defined for the NITS OASIS DATA Templates supported for the Input/Response upload templates defined in Business Practice Standard WEQ-002-101.3.3.

3/14/12 accepted

The contents of each NITS OASIS DATA Template is specific for that template and are identified by the NITS OASIS DATA Template Header Record Data Element Name list that immediately follows the key “DATA=<DataTemplateName>” header record identifier.

The data values to be associated with each of the named NITS OASIS DATA Template Data Elements are specified in one or more data records that immediately follow the NITS OASIS DATA Template Header Record up to the next NITS CSV Format Header Records or the end of the template input data stream.

002-101.4.3.1	Network Integration Transmission Service Service Description (NITSService NITS OASIS DATA Template)

3/14/12 accepted bold not accepted nits

The NITSService NITS OASIS DATA Template is returned in response to either the QueryNITSRequestDetails or the QueryNITSApplication NITS OASIS QUERY Templates. When returned in the QueryNITSRequestDetails NITS OASIS QUERY Template response, a single data record will be returned with the information as currently present in the corresponding NITS Request. When returned in the QueryNITSApplication NITS OASIS QUERY Template response, multiple data records may be returned corresponding to the initial information confirmed in association with the NITS Application and any subsequent modifications to the NITSService NITS OASIS DATA Template information applied to the NITS Application. With the exception of the RECORD_STATUS and ERROR_MESSAGE Data Elements that are not returned in the query response, the Data Elements to be returned are specified in Business Practice Standard WEQ-002-101.3.3.1.

3/14/12 accepted

002-101.4.3.2	Network Integration Transmission Service Customer Information (NITSCustomer NITS OASIS DATA Template)

3/14/12 accepted bold not accepted nits

The NITSCustomer NITS OASIS DATA Template is returned in response to either the QueryNITSRequestDetails or the QueryNITSApplication NITS OASIS Query Templates. When returned in the QueryNITSRequestDetails NITS OASIS QUERY Template response, a single data record will be returned with the information as currently present in the corresponding NITS Request. When returned in the QueryNITSApplication NITS OASIS QUERY Template response, multiple data records may be returned corresponding to the initial information confirmed in association with the NITS Application and any subsequent modifications to the NITSCustomer NITS OASIS DATA Template information applied to the NITS Application. With the exception of the RECORD_STATUS and ERROR_MESSAGE Data Elements that are not returned in the query response, the Data Elements to be returned are specified in Business Practice Standard WEQ-002-101.3.3.2.

3/14/12 accepted

002-101.4.3.3	Network Integration Transmission Service Designated Agent Information (NITSAgent NITS OASIS DATA Template)

3/14/12 accepted bold not accepted nits

The NITSAgent NITS OASIS DATA Template is returned in response to either the QueryNITSRequestDetails or the QueryNITSApplication NITS OASIS Query Templates. When returned in the QueryNITSRequestDetails NITS OASIS QUERY Template response, a single data record will be returned with the information as currently present in the corresponding NITS Request. When returned in the QueryNITSApplication NITS OASIS QUERY Template response, multiple data records may be returned corresponding to the initial information confirmed in association with the NITS Application and any subsequent modifications to the NITSAgent NITS OASIS DATA Template information applied to the NITS Application. With the exception of the RECORD_STATUS and ERROR_MESSAGE Data Elements that are not returned in the query response, the Data Elements to be returned are specified in Business Practice Standard WEQ-002-101.3.3.3.

3/14/12 accepted

002-101.4.3.4	Network Load Description (NITSLoadDescription NITS OASIS DATA Template)

3/14/12 accepted

The NITSLoadDescription NITS OASIS DATA Template is returned in response to either the QueryNITSRequestDetails or the QueryNITSApplication NITS OASIS Query Templates. When returned in the QueryNITSRequestDetails NITS OASIS QUERY Template response, a single data record will be returned with the information as currently present in the corresponding NITS Request. When returned in the QueryNITSApplication NITS OASIS QUERY Template response, multiple data records may be returned corresponding to the initial information confirmed in association with the NITS Application and any subsequent modifications to the NITSLoadDescription NITS OASIS DATA Template information applied to the NITS Application. With the exception of the RECORD_STATUS and ERROR_MESSAGE Data Elements that are not returned in the query response, the Data Elements to be returned are specified in Business Practice Standard WEQ-002-101.3.3.4.

3/14/12 accepted

002-101.4.3.5	Network Load Forecast (NITSLoadForecast NITS OASIS DATA Template)

3/14/12 accepted

The NITSLoadForecast NITS OASIS DATA Template is returned in response to either the QueryNITSRequestDetails or the QueryNITSApplication NITS OASIS Query Templates. When returned in the QueryNITSRequestDetails NITS OASIS QUERY Template response, a multiple data records may be returned with the information over time as currently present in the corresponding NITS Request. When returned in the QueryNITSApplication NITS OASIS QUERY Template response, multiple data records may be returned corresponding to the initial information confirmed in association with the NITS Application and any subsequent modifications to the NITSLoadForecast NITS OASIS DATA Template information applied to the NITS Application. With the exception of the RECORD_STATUS and ERROR_MESSAGE Data Elements that are not returned in the query response, the Data Elements to be returned are specified in Business Practice Standard WEQ-002-101.3.3.5.

3/14/12 accepted

002-101.4.3.6	Network Integration Transmission Service Generation Description (NITSGenerationDescription NITS OASIS DATA Template)

3/14/12 accepted bold not accepted nits

The NITSGenerationDescription NITS OASIS DATA Template is returned in response to either the QueryNITSRequestDetails or the QueryNITSApplication NITS OASIS Query Templates. When returned in the QueryNITSRequestDetails NITS OASIS QUERY Template response, a single data record will be returned with the information as currently present in the corresponding NITS Request. When returned in the QueryNITSApplication NITS OASIS QUERY Template response, multiple data records may be returned corresponding to the initial information confirmed in association with the NITS Application and any subsequent modifications to the NITSGenerationDescription NITS OASIS DATA Template information applied to the NITS Application. With the exception of the RECORD_STATUS and ERROR_MESSAGE Data Elements that are not returned in the query response, the Data Elements to be returned are specified in Business Practice Standard WEQ-002-101.3.3.6.

3/14/12 accepted

002-101.4.3.7	Network Integration Transmission Service Generation Dispatch Cost (NITSGenerationDispatch NITS OASIS DATA Template)

3/14/12 accepted bold not accepted nits

The NITSGenerationDispatch NITS OASIS DATA Template is returned in response to either the QueryNITSRequestDetails or the QueryNITSApplication NITS OASIS Query Templates. When returned in the QueryNITSRequestDetails NITS OASIS QUERY Template response, multiple data records may be returned with the information as currently present in the corresponding NITS Request. When returned in the QueryNITSApplication NITS OASIS QUERY Template response, multiple data records may be returned corresponding to the initial information confirmed in association with the NITS Application and any subsequent modifications to the NITSGenerationDispatch NITS OASIS DATA Template information applied to the NITS Application. With the exception of the RECORD_STATUS and ERROR_MESSAGE Data Elements that are not returned in the query response, the Data Elements to be returned are specified in Business Practice Standard WEQ-002-101.3.3.7.

3/14/12 accepted

002-101.4.3.8	Network Resource Description (NITSResourceDescription NITS OASIS DATA Template)

3/14/12 accepted

The NITSResourceDescription NITS OASIS DATA Template is returned in response to either the QueryNITSRequestDetails or the QueryNITSApplication NITS OASIS Query Templates. When returned in the QueryNITSRequestDetails NITS OASIS QUERY Template response, a single data record will be returned with the information as currently present in the corresponding NITS Request. When returned in the QueryNITSApplication NITS OASIS QUERY Template response, multiple data records may be returned corresponding to the initial information confirmed in association with the NITS Application and any subsequent modifications to the NITSResourceDescription NITS OASIS DATA Template information applied to the NITS Application. With the exception of the RECORD_STATUS and ERROR_MESSAGE Data Elements that are not returned in the query response, the Data Elements to be returned are specified in Business Practice Standard WEQ-002-101.3.3.8.

3/14/12 accepted

002-101.4.3.9	Network Resource Forecast (NITSResourceForecast NITS OASIS DATA Template)

3/14/12 accepted

The NITSResourceForecast NITS OASIS DATA Template is returned in response to either the QueryNITSRequestDetails or the QueryNITSApplication NITS OASIS Query Templates. When returned in the QueryNITSRequestDetails NITS OASIS QUERY Template response, multiple data records may be returned with the information over time as currently present in the corresponding NITS Request. When returned in the QueryNITSApplication NITS OASIS QUERY Template response, multiple data records may be returned corresponding to the initial information confirmed in association with the NITS Application and any subsequent modifications to the NITSResourceForecast NITS OASIS DATA Template information applied to the NITS Application. With the exception of the RECORD_STATUS and ERROR_MESSAGE Data Elements that are not returned in the query response, the Data Elements to be returned are specified in Business Practice Standard WEQ-002-101.3.3.9.

3/14/12 accepted

002-101.4.3.10	Network Resource Designation (NITSResourceDesignation NITS OASIS DATA Template)

3/14/12 accepted

The NITSResourceDesignation NITS OASIS DATA Template is returned in response to the QueryNITSRequestDetails NITS OASIS Query Template. A single data record will be returned with the information as currently present in the corresponding NITS Request. With the exception of the RECORD_STATUS and ERROR_MESSAGE Data Elements that are not returned in the query response, the Data Elements to be returned are specified in Business Practice Standard WEQ-002-101.3.3.10.

3/14/12 accepted

002-101.4.3.11	Designated Network Resource Capacity (NITSResourceCapacity NITS OASIS DATA Template)

3/14/12 accepted

The NITSResourceCapacity NITS OASIS DATA Template is returned in response to the QueryNITSRequestDetails NITS OASIS Query Template. Multiple data records may be returned with the information over time as currently present in the corresponding NITS Request. With the exception of the RECORD_STATUS and ERROR_MESSAGE Data Elements that are not returned in the query response, the Data Elements to be returned are specified in Business Practice Standard WEQ-002-101.3.3.11.

3/14/12 accepted

002-101.4.3.12	Designated Network Resource Scheduling Rights (NITSSchedulingRights NITS OASIS DATA Template)

3/14/12 accepted

The NITSSchedulingRights NITS OASIS DATA Template is returned in response to the QueryNITSRequestDetails NITS OASIS Query Template. Multiple data records may be returned with the information over time as currently present in the corresponding NITS Request. With the exception of the RECORD_STATUS and ERROR_MESSAGE Data Elements that are not returned in the query response, the Data Elements to be returned are specified in Business Practice Standard WEQ-002-101.3.3.12.

3/14/12 accepted

002-101.4.3.13	Auxiliary Transmission Rights (AuxiliaryTransmission NITS OASIS DATA Template)

3/14/12 accepted

The AuxilliaryTransmission NITS OASIS DATA Template is returned in response to the QueryNITSRequestDetails NITS OASIS Query Template. Multiple data records may be returned with the information on each request in a Coordinated Group as currently present in the corresponding NITS Request. With the exception of the RECORD_STATUS and ERROR_MESSAGE Data Elements that are not returned in the query response, the Data Elements to be returned are specified in Business Practice Standard WEQ-002-101.3.3.13.

3/14/12 accepted

002-101.4.3.14	Secondary Network Transmission Service (NITSSecondaryService NITS OASIS DATA Template)

3/14/12 accepted

The NITSSecondaryService NITS OASIS DATA Template is returned in response to the QueryNITSRequestDetails NITS OASIS Query Template. A single data record will be returned with the information as currently present in the corresponding NITS Request. With the exception of the RECORD_STATUS and ERROR_MESSAGE Data Elements that are not returned in the query response, the Data Elements to be returned are specified in Business Practice Standard WEQ-002-101.3.3.14.

3/14/12 accepted

002-101.4.3.15	Secondary Network Transmission Capacity (NITSSecondaryCapacity NITS OASIS DATA Template)

3/14/12 accepted

The NITSSecondaryCapacity NITS OASIS DATA Template is returned in response to the QueryNITSRequestDetails NITS OASIS Query Template. Multiple data records may be returned with the information over time as currently present in the corresponding NITS Request. With the exception of the RECORD_STATUS and ERROR_MESSAGE Data Elements that are not returned in the query response, the Data Elements to be returned are specified in Business Practice Standard WEQ-002-101.3.3.15.

3/14/12 accepted

002-101.4.3.16	Network Integration Transmission Service Ancillary Service (NITSAncillaryService NITS OASIS DATA Template)

3/14/12 accepted bold not accepted nits

The NITSAncillaryService NITS OASIS DATA Template is returned in response to the QueryNITSRequestDetails NITS OASIS Query Template. A single data record will be returned with the information as currently present in the corresponding NITS Request. With the exception of the RECORD_STATUS and ERROR_MESSAGE Data Elements that are not returned in the query response, the Data Elements to be returned are specified in Business Practice Standard WEQ-002-101.3.3.16.

3/14/12 accepted

002-101.4.3.17	Network Integration Transmission Service Ancillary Service Capacity (NITSAncillaryCapacity NITS OASIS DATA Template)

3/14/12 accepted bold not accepted nits

The NITSAncillaryCapacity NITS OASIS DATA Template is returned in response to the QueryNITSRequestDetails NITS OASIS Query Template. Multiple data records may be returned with the information over time as currently present in the corresponding NITS Request. With the exception of the RECORD_STATUS and ERROR_MESSAGE Data Elements that are not returned in the query response, the Data Elements to be returned are specified in Business Practice Standard WEQ-002-101.3.3.17.

3/14/12 accepted

002-101.4.3.18	Point-To-Point Transmission Service (PTPService NITS OASIS DATA Template)

3/14/12 accepted bold not accepted ptp

The PTPService NITS OASIS DATA Template is returned in response to the QueryNITSRequestDetails NITS OASIS Query Template. A single data record will be returned with the information as currently present in the corresponding NITS Request. With the exception of the RECORD_STATUS and ERROR_MESSAGE Data Elements that are not returned in the query response, the Data Elements to be returned are specified in Business Practice Standard WEQ-002-101.3.3.18.

3/14/12 accepted

002-101.4.3.19	Point-To-Point Transmission Service Capacity (PTPCapacity NITS OASIS DATA Template)

3/14/12 accepted bold not accepted ptp

The PTPCapacity NITS OASIS DATA Template is returned in response to the QueryNITSRequestDetails NITS OASIS Query Template. Multiple data records may be returned with the information over time as currently present in the corresponding NITS Request. With the exception of the RECORD_STATUS and ERROR_MESSAGE Data Elements that are not returned in the query response, the Data Elements to be returned are specified in Business Practice Standard WEQ-002-101.3.3.19.

3/14/12 accepted

002-101.4.3.20	Network Integration Transmission Service Ancillary Reservation (NITSAncillaryReservation NITS OASIS DATA Template)

3/14/12 accepted bold not accepted nits

The NITSAncillaryReservation NITS OASIS DATA Template is returned in response to the QueryNITSApplication NITS OASIS Query Template. Multiple data records may be returned corresponding to the initial information confirmed in association with the NITS Application and any subsequent additions to the Ancillary Services arranged in association with the NITS Application. The Data Elements in this template are effectively a join of the information over time from the NITSAncillaryService and NITSAncillaryCapacity NITS OASIS DATA Template submitted and confirmed with the AddNITSAncillary NITS OASIS REQUEST Template.

3/14/12 accepted

NITSAncillaryReservation Data Element List – Query Response:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| SELLER_CODE | Required | Registered entity code associated with the Seller of the ancillary service. |
| SELLER_DUNS | Required | Registered DUNS associated with the Seller of the ancillary service. |
| CONTROL_AREA | Required | Identifies the BAA where the ancillary service is to be delivered/provided. |
| ANC_SERVICE_POINT | May be null | Identifies the specific service point within the CONTROL_AREA where the ancillary service is to be delivered/provided. 3/14/12 accepted |
| SERVICE_INCREMENT | Required | Identifies the increment of service being requested. |
| AS_TYPE | Required | Identifies the specific type of ancillary service being requested. |
| POSTING_REF | May be null | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| SALE_REF | May be null | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| REQUEST_REF | May be null | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| DEAL_REF | May be null | Optional identifier set by the Eligible Customer or Transmission Customer to track requests. |
| SELLER_NAME | May be null | Contact name for the Seller. |
| SELLER_PHONE | May be null | Contact phone number for the Seller. |
| SELLER_FAX | May be null | Facsimile number for the Seller. |
| SELLER_EMAIL | May be null | Email address for the Seller. |
| CEILING_PRICE | Required | Identifies the Transmission Provider’s tariff price for the service. |
| PRICE_UNITS | Required | Identifies the units of price for the service. |
| NEGOTIATED_PRICE_FLAG | May be null | Set to “L” if the Seller accepted Price is lower than the posted offer price. Set to “H” if Seller accepted Price is higher than the posted offer price. Otherwise set to null. 3/14/12 accepted |
| CAPACITY | Required | The volume of service granted over the start/stop interval. |
| BID_PRICE | Required | The Eligible Customer or Transmission Customers price bid for purchase of the ancillary service. |
| OFFER_PRICE | May be null | The Seller’s offered price for service. |
| START_TIME | Required | Identifies the start of the interval of service requested at a given capacity and price. |
| STOP_TIME | Required | Identifies the end of the interval of service requested at a given capacity and price. |

002-101.5	AUDIT TEMPLATE DESCRIPTIONS

3/14/12 accepted

Auditing of information on OASIS associated with the arrangement of NITS shall be supported through the implementation of audit Templates that are an extension to the basic OASIS Query/Response Templates. Every request submitted or updated on OASIS related to NITS, either through Input/Response template submissions or the corresponding browser-based forms interface, and every Transmission Provider action or update to these NITS requests and associated data elements must be auditable using the following audit QUERY Templates:

3/14/12 accepted

AuditNITSRequest		- audit counterpart to QueryNITSRequest

AuditNITSRequestDetails	- audit counterpart to QueryNITSRequestDetails

OASIS auditing of changes made to requests in the Pre-submittal Workspace is not required.

3/14/12 accepted

The audit QUERY Templates follow the same basic Query/Response Template interactions and processing requirements as specified in Business Practice Standard WEQ-002-101.4 for their non-audit counterparts. The OBJECT and DATA Templates returned in the response to an audit QUERY Template also follow the same basic structure as their non-audit query counterparts, but the Header Record Data Element List is extended to return information on each change made to the associated request over time. These extensions are specified in the following Data Elements:

3/14/12 accepted

REVISION

RECORD_TYPE

TIME_OF_LAST_UPDATE

MODIFYING_COMPANY_CODE

MODIFYING_NAME

The REVISION Data Element shall be assigned as an incrementing integer value starting at 1 on the initial submission of each new NITS request, and incremented with each change in the REQUEST Template Data Elements or any of the request’s associated DATA Template Data Elements.

The RECORD_TYPE Data Element shall be set to one of the following values depending on the nature of the change made to the NITS REQUEST or DATA Template information:

I -	denotes a record of information as it appeared on its initial Insertion (posting) on OASIS

U -	denotes a record of information as it appeared immediately following an Update to the posted information

D -	denotes a record of any Deleted information as it last appeared on OASIS

The TIME_OF_LAST_UPDATE Data Element shall specify the time that the REQUEST or DATA Template Data Elements were inserted, updated or deleted. The MODIFYING_COMPANY_CODE and MODIFYING_NAME shall specify the identity of the entity and OASIS user account responsible for the insert, update or deletion of the corresponding information. MODIFYING_NAME may be null if the change in the Data Element values was the result of automated back-end processes that are not attributable to a given OASIS user.

3/14/12 accepted

Every change on OASIS in association with the REQUEST Template data or any of the subordinate DATA Template information must be captured with the incrementing REVISION data element as a change to the REQUEST Template in the OASIS audit log. For example, if the NewNITSRequest template submission contains the AddNITSLoad request with NITSLoadDescription information, that action would be logged as REVISION=1 and RECORD_TYPE=I for an insert into OASIS on both the AddNITSLoad REQUEST and NITSLoadDescription DATA Templates. If a subsequent UpdateNITSRequest is submitted for the same AddNITSLoad request to add NITSLoadForecast information, that action would be captured as REVISION=2 with RECORD_TYPE=U for an update to the AddNITSLoad REQUEST Template and as REVISION=2 and RECORD_TYPE=I for the insert of the NITSLoadForecast DATA Template. If the Transmission Provider then updates the STATUS of the AddNITSLoad request to COMPLETED, that change would be indicated as REVISION=3 with RECORD_TYPE=U associated with the AddNITSLoad REQUEST Template, and no DATA Template updates as no information was added or changed.

3/14/12 accepted

002-101.5.1	Audit QUERY Template Definitions

3/14/12 accepted

The QUERY Template definitions for the AuditNITSRequest and AuditNITSRequestDetails shall be identical to that supplied for their non-audit counterparts in Business Practice Standards WEQ-002-101.4.1.1 and WEQ-002-101.4.1.2. The audit QUERY Templates must support the same basic Query Variables as specified for the non-audit counterpart QUERY Templates. These Query Variables control the set of applicable NITS requests that will be selected and returned via the OBJECT and associated DATA (if applicable) Templates in the query response.

3/14/12 accepted

The name of the audit QUERY Template invoked shall be identified and returned in the QUERY=<QueryTemplateName> field of the NITS CSV Format Query Header Record, e.g., QUERY=AuditNITSRequest. The NITS OASIS QUERY Template response Data Element list shall be identical to that specified in Business Practice Standard WEQ-002-101.4.1.

3/14/12 accepted

002-101.5.2	Audit OBJECT Template Definitions

3/14/12 accepted

The OBJECT Template definition associated with the AuditNITSRequest and AuditNITSRequestDetails QUERY Templates follows the same basic format as specified for the NITS Request Object in Business Practice Standard WEQ-002-101.4.2.1. The OBJECT=<ObjectTemplateName> field in the NITS CSV Format Object Header Record will contain the name of the NITS OASIS REQUEST Template information being returned in the query response, e.g., OBJECT=AddNITSDNR. The Object Header Data Element list returned shall be extended to include the following key audit Data Elements appended to the Request Object Template Data Element List specified in Business Practice Standard WEQ-002-101.4.2.1.

3/14/12 accepted per subcommittee wording

Audit NITS Request Object Template Data Element List – Query Response:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| REVISION | Required | Returns incrementing revision number identifying each change made to the corresponding NITS REQUEST Template information. |
| RECORD_TYPE | Required | Identifies the nature of change made to the corresponding NITS REQUEST Template information; restricted values: I – Insert U – Update D – Delete |
| TIME_OF_LAST_UPDATE | Required | Specifies the date/time when the change in REQUEST or DATA Template information was performed, |
| MODIFYING_COMPANY_CODE | Required | Identifies the entity by registered entity code that submitted or updated the REQUEST or DATA Template information. |
| MODIFYING_NAME | May be null | Identifies the OASIS user account that was used to submit or update the REQUEST or DATA Template information. |
| […] |  | All of the NITS Request OBJECT Template Data Elements as specified in Business Practice Standard WEQ-002-101.4.2.1 shall be included after the above audit specific OBJECT Header Data Elements. 3/14/12 accepted |

002-101.5.3	Audit DATA Template Definitions

3/14/12 accepted

The DATA Template definitions associated with the AuditNITSRequestDetails QUERY Template follows the same basic format as specified for the NITS OASIS DATA Templates returned for the QueryNITSRequestDetails QUERY Template as specified in Business Practice Standard WEQ-002-101.4.3. The DATA=<DataTemplateName> field in the NITS CSV Format Data Header Record will contain the name of the NITS OASIS DATA Template information being returned in the query response, e.g., DATA=NITSResourceDesignation. The Data Header Data Element list returned shall be extended to include the following key audit Data Elements appended to the DATA Template Data Element List specified in Business Practice Standards WEQ-002-101.3.3.1 through WEQ-002-101.3.3.19.

3/14/12 accepted per subcommittee wording

Audit NITS Request Data Template Data Element List – Query Response:

| Data Element Name | Requirement | Usage |
| --- | --- | --- |
| REVISION | Required | Returns the revision number identifying each change made to the corresponding NITS REQUEST Template information where that change was accompanied by or the result of a change in the DATA Template information associated with the request. |
| RECORD_TYPE | Required | Identifies the nature of change made to the corresponding NITS DATA Template information; restricted values: I – Insert U – Update D – Delete |
| TIME_OF_LAST_UPDATE | Required | Specifies the date/time when the change in the DATA Template information was performed, |
| MODIFYING_COMPANY_CODE | Required | Identifies the entity by registered entity code that submitted, updated, or deleted the DATA Template information. |
| MODIFYING_NAME | May be null | Identifies the OASIS user account that was used to submit, update, or delete the DATA Template information. |
| […] |  | All of the NITS Request DATA Template Data Elements as specified in Business Practice Standard WEQ-002-101.3.3.1 through WEQ-002-101.3.3.19 included after the above audit specific DATA Header Data Elements. 3/14/12 accepted |

002-101.5.4	Audit Network Integration Transmission Service Request Summary (AuditNITSRequest NITS OASIS QUERY Template)

3/14/12 accepted bold not accepted nits

The AuditNITSRequest NITS OASIS QUERY Template is used to query the audit log of the summary status information related to all NITS Requests submitted that meet the selection criteria specified via the user supplied Query Variables.

3/14/12 accepted

In response to the AuditNITSRequest query, each selected NITS REQUEST Template submission that matches the selection criteria will be returned by REQUEST Template name in the same fashion as described for the QueryNITSRequest QUERY Template. For each named REQUEST Template selected, a single OBJECT Header Record will be returned with the REQUEST Template name identified in the OBJECT=<ObjectTemplateName> field, e.g., OBJECT=AddNITSDNR, followed by the Data Element names as specified in Business Practice Standard WEQ-002-101.5.2. The subsequent Data Records that will be returned for the named REQUEST Template will be grouped by request ASSIGNMENT_REF value and contain a single Data Record for each Insert or Update action ordered in time, i.e., by increasing REVISION number. Delete is not an option for a REQUEST Template. The audit information for each ASSIGNMENT_REF will follow in turn for all REQUEST Template submissions and updates for that given named request. If additional named REQUEST Templates are included in the requests selected to be returned, a new OBJECT Header Record will be written for each of those selected REQUEST Templates by name followed by the ordered set of all actions taken on each of the submitted/updated requests grouped by ASSIGNMENT_REF.

3/14/12 accepted

002-101.5.5	Audit Network Integration Transmission Service Request Details (AuditNITSRequestDetails NITS OASIS QUERY Template)

3/14/12 accepted bold not accepted nits

The AuditNITSRequestDetails QUERY Template response is similar in form to that returned via the QueryNITSRequestDetails QUERY Template. For each separate REQUEST Template submission that meets the users Query Variable selection criteria, the NITS OASIS QUERY Template response shall first return the audit information for the REQUEST Template itself, e.g., STATUS, etc. consisting of the OBJECT Header Record identifying the REQUEST Template name followed by the Header Data Element list as specified above. The corresponding REQUEST Template Data Element Values will follow the OBJECT Header Record, one record for each insert and subsequent update made to the REQUEST or its associated DATA Template information.

3/14/12 accepted

Following the REQUEST Template audit information, each DATA Template associated with that given NITS request will be returned. The DATA Template information will be identified with the leading DATA Header Record with the DATA=<DataTemplateName> field set to the name of the DATA Template whose audit information is being returned followed by the Data Element list as specified in Business Practice Standard WEQ-002-101.5.3. The records returned following the DATA Header Record will return the Data Element Values as specified in the corresponding Data Element list for the initial submission of the associated DATA Template information followed by each subsequent update or delete action taken on the corresponding DATA Template information. For DATA Templates that consist of multiple Data Records within a single submission, e.g., capacity profile information over time, the full set of Data Records submitted, updated or deleted shall be listed as a group under the same REVISION number corresponding to the submit/update action taken for that specific NITS request.

3/14/12 accepted

The above sequence of returning the audit information associated with a given NITS OASIS REQUEST Template followed by the audit information for all associated NITS OASIS DATA Templates under that REQUEST Template shall be repeated for each NITS OASIS REQUEST Template that meets the users Query Variable selection criteria.

002-101.6	NETWORK INTEGRATION TRANSMISSION SERVICE INTEGRATION WITH OASIS POINT-TO-POINT TEMPLATE IMPLEMENTATION

3/14/12 accepted bold not accepted nits or ptp

OASIS implementation of the PTP transstatus template shall include integration of information from the NITS OASIS REQUEST and DATA Template submissions with the information currently returned for PTP Transmission Service requests and reservations for the following NITS REQUEST Templates:

AddNITSDNR

TerminateNITSDNR

AddNITSSecondary

TerminateNITSSecondary

AddPTPService

Each request for the above NITS REQUEST Template submissions must be included in the OASIS transstatus Template response so that the relative queue position of PTP and NITS requests can be easily viewed on OASIS. The correspondence between the NITS REQUEST and DATA Template Data Elements and the transstatus Data Elements that must be integrated between the OASIS template definitions and returned in the transstatus Template response are specified in the following subsections.

3/14/12 accepted

There shall be no requirement for OASIS to support the return of NITS related requests submitted using the NITS OASIS Templates in the response to the transstatusaudit template. For Transmission Providers that used the OASIS PTP Templates to document NITS related requests prior to implementation of OASIS Version 2.0 will still be required to provide audit data for these requests via the transstatusaudit Template.

3/14/12 accepted

002-101.6.1	AddNITSDNR/TerminateNITSDNR Integration

3/14/12 accepted

The following table shows the correspondence between the NITS Template Data Elements that will be used by OASIS to populate the transstatus template response to convey information related to the designation and termination of Network Resources. Template Data Elements that must be implied by the nature of the NITS request being returned are noted.

3/14/12 accepted

| transstatus Data Element Name | NITS Template | NITS Template Data Element Name |
| --- | --- | --- |
| CONTINUATION_FLAG | Not Applicable | Not Applicable; this data element used to convey profile information in multi-row datasets in the transstatus response. |
| ASSIGNMENT_REF | AddNITSDNR TerminateNITSDNR | ASSIGNMENT_REF |
| SELLER_CODE | AddNITSDNR TerminateNITSDNR | SELLER_CODE |
| SELLER_DUNS | AddNITSDNR TerminateNITSDNR | SELLER_DUNS |
| CUSTOMER_CODE | AddNITSDNR TerminateNITSDNR | CUSTOMER_CODE |
| CUSTOMER_DUNS | AddNITSDNR TerminateNITSDNR | CUSTOMER_DUNS |
| PATH_NAME | Not Applicable | Population of this data element is optional according to Transmission Provider Business Practice. |
| POINT_OF_RECEIPT | NITSResourceDesignation | POINT_OF_RECEIPT |
| POINT_OF_DELIVERY | NITSLoadDescription | POINT_OF_DELIVERY; population of this data element is optional according to Transmission Provider Business Practice. If multiple PODs are assigned to the Network Loads under the NITS Application, this Data Element may be null. 3/14/12 accepted |
| SOURCE | NITSResourceDesignation | SOURCE |
| SINK | NITSLoadDescription | SINK; population of this data element is optional according to Transmission Provider Business Practice. If multiple Sinks are assigned to the Network Loads under the NITS Application, this Data Element may be null. 3/14/12 accepted |
| CAPACITY_REQUESTED | NITSResourceCapacity | CAPACITY_REQUESTED |
| CAPACITY_GRANTED | NITSResourceCapacity | CAPACITY_GRANTED |
| SERVICE_INCREMENT | Not Applicable | Implied with value based on the duration of the DNR requested as specified for “Request Interval” in Business Practice Standard WEQ-001-105.1.5 and Table 105-A. . |
| TS_CLASS | Not Applicable | Implied with value of FIRM |
| TS_TYPE | Not Applicable | Implied with value of NETWORK |
| TS_PERIOD | Not Applicable | Implied with value of FULL_PERIOD |
| TS_WINDOW | Not Applicable | Implied with value of EXTENDED |
| TS_SUBCLASS | Not Applicable | Returned as null value |
| NERC_CURTAILMENT_PRIORITY | Not Applicable | Implied with value of 7 |
| OTHER_CURTAILMENT_PRIORITY | Not Applicable | Returned as null value |
| START_TIME | NITSResourceCapacity | START_TIME |
| STOP_TIME | NITSResourceCapacity | STOP_TIME |
| CEILING_PRICE | Not Applicable | Returned as null value |
| OFFER_PRICE | Not Applicable | Returned as null value |
| BID_PRICE | Not Applicable | Returned as null value |
| PRICE_UNITS | Not Applicable | Returned as null value |
| PRECONFIRMED | AddNITSDNR TerminateNITSDNR | PRECONFIRMED |
| ANC_SVC_LINK | Not Applicable | Returned as null value |
| ANC_SVC_REQ | Not Applicable | Returned as null value |
| POSTING_REF | NITSResourceDesignation | POSTING_REF |
| SALE_REF | NITSResourceDesignation | SALE_REF |
| RQUEST_REF | NITSResourceDesignation | RQUEST_REF |
| DEAL_REF | NITSResourceDesignation | DEAL_REF |
| IMPACTED | Not Applicable | Returned as null value |
| COMPETING_REQUEST_FLAG | Not Applicable | Returned as null value |
| REQUEST_TYPE | Not Applicable | Implied with value of: ORIGINAL for AddNITSDNR RELINQUISH for TerminateNITSDNR request. |
| RELATED_REF | Not Applicable | Returned as null value |
| NEGOTIATED_PRICE_FLAG | Not Applicable | Returned as null value |
| STATUS | AddNITSDNR TerminateNITSDNR | STATUS |
| STATUS_NOTIFICATION | Not Applicable | Returned as null value |
| STATUS_COMMENTS | AddNITSDNR TerminateNITSDNR | STATUS_COMMENTS |
| TIME_QUEUED | AddNITSDNR TerminateNITSDNR | TIME_QUEUED |
| RESPONSE_TIME_LIMIT | AddNITSDNR TerminateNITSDNR | RESPONSE_TIME_LIMIT |
| TIME_OF_LAST_UPDATE | AddNITSDNR TerminateNITSDNR | TIME_OF_LAST_UPDATE |
| PRIMARY_PROVIDER_COMMENTS | AddNITSDNR TerminateNITSDNR | PRIMARY_PROVIDER_COMMENTS |
| SELLER_REF | Not Applicable | Returned as null value |
| SELLER_COMMENTS | AddNITSDNR TerminateNITSDNR | SELLER_COMMENTS |
| CUSTOMER_COMMENTS | AddNITSDNR TerminateNITSDNR | CUSTOMER_COMMENTS |
| SELLER_NAME | Not Applicable | Optional at Transmission Providers discretion. |
| SELLER_PHONE | Not Applicable | Optional at Transmission Providers discretion. |
| SELLER_FAX | Not Applicable | Optional at Transmission Providers discretion. |
| SELLER_EMAIL | Not Applicable | Optional at Transmission Providers discretion. |
| CUSTOMER_NAME | NITSCustomer | CUSTOMER_NAME; optional at Transmission Providers discretion. |
| CUSTOMER_PHONE | NITSCustomer | CUSTOMER_PHONE; optional at Transmission Providers discretion |
| CUSTOMER_FAX | NITSCustomer | CUSTOMER_FAX; optional at Transmission Providers discretion |
| CUSTOMER_EMAIL | NITSCustomer | CUSTOMER_EMAIL; optional at Transmission Providers discretion |
| REASSIGNED_REF | Not Applicable | Returned as null value |
| REASSIGNED_CAPACITY | Not Applicable | Returned as null value |
| REASSIGNED_START_TIME | Not Applicable | Returned as null value |
| REASSIGNED_STOP_TIME | Not Applicable | Returned as null value |
| PRIMARY_PROVIDER_APPROVAL | Not Applicable | Returned as null value |
| PRIMARY_PROVIDER_PROVISIONS | Not Applicable | Returned as null value |
| ROLLOVER_WAIVED | Not Applicable | Returned as null value |
| CG_STATUS | NITSResourceDesignation | CG_STATUS |
| CG_DEADLINE | NITSResourceDesignation | CG_DEADLINE |
| CONCOMITANT_EVAL_FLAG | NITSResourceDesignation | CONCOMITANT_EVAL_FLAG |

002-101.6.2	AddNITSSecondary/TerminateNITSSecondary Integration

3/14/12 accepted

The following table shows the correspondence between the NITS Template Data Elements that will be used by OASIS to populate the transstatus template response to convey information related to the arrangement or termination of Network Service from Non-Designated Resources. Template Data Elements that must be implied by the nature of the NITS request being returned are noted.

3/14/12 accepted

| transstatus Data Element Name | NITS Template | NITS Template Data Element Name |
| --- | --- | --- |
| CONTINUATION_FLAG | Not Applicable | Not Applicable; this data element used to convey profile information in multi-row datasets in the transstatus response. |
| ASSIGNMENT_REF | AddNITSSecondary TerminateNITSSecondary | ASSIGNMENT_REF |
| SELLER_CODE | AddNITSSecondary TerminateNITSSecondary | SELLER_CODE |
| SELLER_DUNS | AddNITSSecondary TerminateNITSSecondary | SELLER_DUNS |
| CUSTOMER_CODE | AddNITSSecondary TerminateNITSSecondary | CUSTOMER_CODE |
| CUSTOMER_DUNS | AddNITSSecondary TerminateNITSSecondary | CUSTOMER_DUNS |
| PATH_NAME | NITSSecondaryService | PATH_NAME |
| POINT_OF_RECEIPT | NITSSecondaryService | POINT_OF_RECEIPT |
| POINT_OF_DELIVERY | NITSSecondaryService | POINT_OF_DELIVERY; population of this data element is optional according to Transmission Provider Business Practice. If multiple PODs are assigned to the Network Loads under the NITS Application, this Data Element may be null. 3/14/12 accepted |
| SOURCE | NITSSecondaryService | SOURCE |
| SINK | NITSSecondaryService | SINK; population of this data element is optional according to Transmission Provider Business Practice. If multiple Sinks are assigned to the Network Loads under the NITS Application, this Data Element may be null. 3/14/12 accepted |
| CAPACITY_REQUESTED | NITSSecondaryCapacity | CAPACITY_REQUESTED |
| CAPACITY_GRANTED | NITSSecondaryCapacity | CAPACITY_GRANTED |
| SERVICE_INCREMENT | Not Applicable | Implied with value based on the duration of the Secondary Network Service requested as specified for “Request Interval” in Business Practice Standard WEQ-001-105.1.5 and Table 105-A. |
| TS_CLASS | Not Applicable | Implied with value of SECONDARY |
| TS_TYPE | Not Applicable | Implied with value of NETWORK |
| TS_PERIOD | Not Applicable | Implied with value of FULL_PERIOD |
| TS_WINDOW | Not Applicable | Implied with value of EXTENDED |
| TS_SUBCLASS | Not Applicable | Returned as null value |
| NERC_CURTAILMENT_PRIORITY | Not Applicable | Implied with value of 6 |
| OTHER_CURTAILMENT_PRIORITY | Not Applicable | Returned as null value |
| START_TIME | NITSSecondaryCapacity | START_TIME |
| STOP_TIME | NITSSecondaryCapacity | STOP_TIME |
| CEILING_PRICE | Not Applicable | Returned as null value |
| OFFER_PRICE | Not Applicable | Returned as null value |
| BID_PRICE | Not Applicable | Returned as null value |
| PRICE_UNITS | Not Applicable | Returned as null value |
| PRECONFIRMED | AddNITSSecondary TerminateNITSSecondary | PRECONFIRMED |
| ANC_SVC_LINK | Not Applicable | Returned as null value |
| ANC_SVC_REQ | Not Applicable | Returned as null value |
| POSTING_REF | NITSSecondaryService | POSTING_REF |
| SALE_REF | NITSSecondaryService | SALE_REF |
| RQUEST_REF | NITSSecondaryService | RQUEST_REF |
| DEAL_REF | NITSSecondaryService | DEAL_REF |
| IMPACTED | Not Applicable | Returned as null value |
| COMPETING_REQUEST_FLAG | Not Applicable | Returned as null value |
| REQUEST_TYPE | Not Applicable | Implied with value of ORIGINAL for AddNITSSecondary request, or RELINQUISH for TerminateNITSSecondary request. |
| RELATED_REF | NITSSecondaryService | RELATED_REF |
| NEGOTIATED_PRICE_FLAG | Not Applicable | Returned as null value |
| STATUS | AddNITSSecondary TerminateNITSSecondary | STATUS |
| STATUS_NOTIFICATION | Not Applicable | Returned as null value |
| STATUS_COMMENTS | AddNITSSecondary TerminateNITSSecondary | STATUS_COMMENTS |
| TIME_QUEUED | AddNITSSecondary TerminateNITSSecondary | TIME_QUEUED |
| RESPONSE_TIME_LIMIT | AddNITSSecondary TerminateNITSSecondary | RESPONSE_TIME_LIMIT |
| TIME_OF_LAST_UPDATE | AddNITSSecondary TerminateNITSSecondary | TIME_OF_LAST_UPDATE |
| PRIMARY_PROVIDER_COMMENTS | AddNITSSecondary TerminateNITSSecondary | PRIMARY_PROVIDER_COMMENTS |
| SELLER_REF | Not Applicable | Returned as null value |
| SELLER_COMMENTS | AddNITSSecondary TerminateNITSSecondary | SELLER_COMMENTS |
| CUSTOMER_COMMENTS | AddNITSSecondary TerminateNITSSecondary | CUSTOMER_COMMENTS |
| SELLER_NAME | Not Applicable | Optional at Transmission Providers discretion. |
| SELLER_PHONE | Not Applicable | Optional at Transmission Providers discretion. |
| SELLER_FAX | Not Applicable | Optional at Transmission Providers discretion. |
| SELLER_EMAIL | Not Applicable | Optional at Transmission Providers discretion. |
| CUSTOMER_NAME | NITSCustomer | CUSTOMER_NAME; optional at Transmission Providers discretion. |
| CUSTOMER_PHONE | NITSCustomer | CUSTOMER_PHONE; optional at Transmission Providers discretion |
| CUSTOMER_FAX | NITSCustomer | CUSTOMER_FAX; optional at Transmission Providers discretion |
| CUSTOMER_EMAIL | NITSCustomer | CUSTOMER_EMAIL; optional at Transmission Providers discretion |
| REASSIGNED_REF | Not Applicable | Returned as null value |
| REASSIGNED_CAPACITY | Not Applicable | Returned as null value |
| REASSIGNED_START_TIME | Not Applicable | Returned as null value |
| REASSIGNED_STOP_TIME | Not Applicable | Returned as null value |
| PRIMARY_PROVIDER_APPROVAL | Not Applicable | Returned as null value |
| PRIMARY_PROVIDER_PROVISIONS | Not Applicable | Returned as null value |
| ROLLOVER_WAIVED | Not Applicable | Returned as null value |
| CG_STATUS | NITSSecondaryService | CG_STATUS |
| CG_DEADLINE | NITSSecondaryService | CG_DEADLINE |
| CONCOMITANT_EVAL_FLAG | NITSSecondaryService | CONCOMITANT_EVAL_FLAG |

002-101.6.3	AddPTPService Integration

3/14/12 accepted

The AddPTPService template Data Elements have a one-to-one correspondence with the Data Elements required in the response to the transstatus template. OASIS shall process the submission of the AddPTPService request in the same manner that it processes PTP Transmission Service requests submitted using the transrequest and transcust OASIS PTP Templates.

3/14/12 accepted